3GPP TSG RAN#93e	RP-21xxxx

e-Meeting, September 13th – 17th, 2021

Agenda item:

Source: 	3GPP TSG RAN Chair

Title: 	Email Summary on RAN Rel-18 discussion

Document for:	Discussion/Decision

# Introduction

In this document, we will provide a summary on RAN Rel-18 discussion, using RP-211679 as a starting point

# Discussion

## Initial Round

Questions:

- Any questions/comments/suggestions to the list of topics led by each WG as listed in RP-211679?

| Company | Views |
| --- | --- |
| vivo | On ultra-low power WUR/WUS (Page 13) Thanks for highlighting the ultra-low power WUR/WUS topic which has received strong interest in both discussion of eRedCap and additional topics set#1 (UE power savings) threads, we fully support to continue discussion on this topic in Q4. Regarding details on how to manage this discussion, given the fact that 1) this feature is not intended to be limited to eRedCap, 2) this feature should start with a study item (as summarized in RP-211664, the moderator summary of additional topic set#1) while other eRedCap objectives are more likely to be a work item, We think a separate email thread until RAN#94e would be more beneficial for developing potential study objectives for the ultra-low power WUR/WUS, and the eRedCap email thread can focus on the potential WI objectives on the areas listed in Page 23 as the starting point. We also would like to better understand what would be the basis for futhre decision regarding “Whether or not/how to have such a project is to be handled after further discussion” until RAN#94. Finally, we would suggest to update the sub-bullet as highlighted below since the applicability to all device types should be considered not only in the normative work, but also in the study item phase when we evaluate and select among different design options, and furthermore depending on the further discussion, we may end up with study item only for this feature in Rel-18 timeframe. - Primarily target ultra-low power WUS/WUR required by RedCap use cases, with studied/specified solutions not be limited to RedCap UEs only On UE aggregation (Page 14) We agree with online comment from CMCC to include “UE back-up” to the topic of “UE aggregation” as they are technically relevant. We propose to change “UE aggregation” to “UE collaboration” to be more inclusive for now, and details can be further developed in Q4. |
| Ericsson | The list of “Potential items led per WG for subsequent discussion” looks reasonable to us. Of course, it does not (and cannot) reflect the wish list of each individual company, but it represents a balanced mix of topics that will bring the industry forward. A continued discussion of controversial topics or topics lacking support will not be constructive and will basically be a waste of time. We even think that some of the topics aggregated under [17] lack consensus, especially “UE aggregation” and “High-speed packetization”. Whether the workload will finally be manageable will depend on the actual content of these items. Some bare the risk of becoming far too large. So even if we limit the number of items during RAN#93e, we have to be mindful in the upcoming discussions. |
| Verzion | Thanks for the great effort Wanshi. Overall it looks reasonable to us too. For us, improve basic network performance is still the most important – it is the foundation of everything, including all the vertical segments. We support the proposal to combing UL MIMO with DL MIMO, just that we hope the scope is not diminished. We also think DC/CA is important and combining it with mobility into a Mobility and DC/CA WI seems reasonable, if DC/CA isn’t going to be a separate WI. We have some specific deployment scenarios about DC/CA that we hope R18 can address. |
| Samsung | For MIMO, we support merging DL and UL MIMO into one topic/work item. In this case the group needs to be selective and careful in defining the scope for each area of enhancements. |
| Futurewei | Overall, the decision of continuation, dropping, merging, promoting of certain topics or sub-topics should be based on the companies’ inputs and outcomes of the email discussions, in a case by case manner. Also, the decision should not be solely based on whether a topic/sub-topic is listed as controversial or not in the email discussion summary as the criteria is quite in-consistent across different email threads. Specifically, on the handling of uplink enhancements, it is fine for us to merge the uplink MIMO related discussion with downlink MIMO to move forward. However, during the email discussion, many showed concern that these uplink MIMO related proposals are really only relevant to CPE-like devices and not much for normal devices like smartphones. So these proposals should be further discussed (under MIMO item). For the rest of the proposals, enhancement for multi-carrier UL operation has very good support from the companies and can be handled under an uplink enhancement item together with uplink coverage enhancements. Therefore, we’d like to suggest to change the title of the 2nd RAN1 item to “Uplink enhancements (excluding uplink MIMO)” to better reflect the situation. On enhancements for XR, it is ok to leave the decision of leading WG (RAN1 vs RAN2) later after the scope is stabilized. One issue we see is that the ongoing XR SI outcomes are not available and the scoping decision should be based on the SI outcomes instead of bypassing it and hence waste the efforts the companies put in. We also agree with some companies’ online comments that a discussion on the set of other RAN1 items at this stage should be kept to include items with a good amount of support such as flexible spectrum integration and passive IoT. We understand the need to control the overall scope of R18. Based on further discussion, some of these other items can be part of the R18 work in the place of some/part of the separately listed items. Also about TU estimation, we think it should be left for later discussion. |

Questions:

- Any questions/comments/suggestion to other aspects related to RP-211679?

| Company | Views |
| --- | --- |
| vivo | On handling of “controversial” areas We understand the example areas for each email threads are mostly based on the claimed “non-controversial” part from the moderator’s summary. However, since the listed example areas are “for information only”, can the “a bit more controversial” areas still be discussed in the next step for potential convergence? We think this question applies to many topics, as one particular example, we see some additional cost/complexity reduction features received good support in previous round of discussion eRedCap but currently not listed in Page 23, as they were not stable enough to be marked as “non-controversial” for now. |
| Samsung | MIMO: On slide 19 (list of areas) - On DL aspects: - Enhancement on coherent JT (CJT) is supported by a number of companies especially operators (cf. RP-212524). This should be included. - Multi-beam enhancement to reduce overhead/latency (especially UE-initiated beam management) is also supported by many companies. It is unclear why this is not included. - In general, if the list is based only on the non-controversial parts, “Other CPE-specific considerations” should also be removed to be consistent (it was labeled controversial and low priority by the moderator). - On UL aspects: - >4Tx UL: While we don’t oppose this area of enhancement, the scope of this enhancement can be excessive if the WID is not specific about the areas of enhancements. - “Potentially other UL enhancements, e.g. ...” should either be removed or replaced by a more specific task description. As of now this looks like a container for a handful of different proposals supported by only 1-2 companies. |
| Futurewei | The overlapping between the RAN1 meeting and email discussion should be avoided. Towards the end of the meeting, it is really hard for the delegate to handle this extra work. Competing Rel-17 should take highest priority at this stage. About the detailed scope of each item, our understanding is that the email discussion summary should server as the starting point for further discussion. |

## Intermediate Round

## Final Round

# Conclusion

Based on the email discussion, the following are proposed:

TBD

# References

RP-211679	RAN Chair’s Summary for RAN Release 18	RAN Chair (Qualcomm)