3GPP TSG RAN#93e	RP-21xxxx

e-Meeting, September 13th – 17th, 2021

Agenda item:

Source: 	3GPP TSG RAN Chair

Title: 	Email Summary on RAN Rel-18 discussion

Document for:	Discussion/Decision

# Introduction

In this document, we will provide a summary on RAN Rel-18 discussion, using RP-211679 as a starting point

# Discussion

## Initial Round

Questions:

- Any questions/comments/suggestions to the list of topics led by each WG as listed in RP-211679?

| Company | Views |
| --- | --- |
| vivo | On ultra-low power WUR/WUS (Page 13) Thanks for highlighting the ultra-low power WUR/WUS topic which has received strong interest in both discussion of eRedCap and additional topics set#1 (UE power savings) threads, we fully support to continue discussion on this topic in Q4. Regarding details on how to manage this discussion, given the fact that 1) this feature is not intended to be limited to eRedCap, 2) this feature should start with a study item (as summarized in RP-211664, the moderator summary of additional topic set#1) while other eRedCap objectives are more likely to be a work item, We think a separate email thread until RAN#94e would be more beneficial for developing potential study objectives for the ultra-low power WUR/WUS, and the eRedCap email thread can focus on the potential WI objectives on the areas listed in Page 23 as the starting point. We also would like to better understand what would be the basis for futhre decision regarding “Whether or not/how to have such a project is to be handled after further discussion” until RAN#94. Finally, we would suggest to update the sub-bullet as highlighted below since the applicability to all device types should be considered not only in the normative work, but also in the study item phase when we evaluate and select among different design options, and furthermore depending on the further discussion, we may end up with study item only for this feature in Rel-18 timeframe. - Primarily target ultra-low power WUS/WUR required by RedCap use cases, with studied/specified solutions not be limited to RedCap UEs only On UE aggregation (Page 14) We agree with online comment from CMCC to include “UE back-up” to the topic of “UE aggregation” as they are technically relevant. We propose to change “UE aggregation” to “UE collaboration” to be more inclusive for now, and details can be further developed in Q4. |
| Ericsson | The list of “Potential items led per WG for subsequent discussion” looks reasonable to us. Of course, it does not (and cannot) reflect the wish list of each individual company, but it represents a balanced mix of topics that will bring the industry forward. A continued discussion of controversial topics or topics lacking support will not be constructive and will basically be a waste of time. We even think that some of the topics aggregated under [17] lack consensus, especially “UE aggregation” and “High-speed packetization”. Whether the workload will finally be manageable will depend on the actual content of these items. Some bare the risk of becoming far too large. So even if we limit the number of items during RAN#93e, we have to be mindful in the upcoming discussions. |
| Verzion | Thanks for the great effort Wanshi. Overall it looks reasonable to us too. For us, improve basic network performance is still the most important – it is the foundation of everything, including all the vertical segments. We support the proposal to combing UL MIMO with DL MIMO, just that we hope the scope is not diminished. We also think DC/CA is important and combining it with mobility into a Mobility and DC/CA WI seems reasonable, if DC/CA isn’t going to be a separate WI. We have some specific deployment scenarios about DC/CA that we hope R18 can address. |
| Samsung | For MIMO, we support merging DL and UL MIMO into one topic/work item. In this case the group needs to be selective and careful in defining the scope for each area of enhancements. |
| Futurewei | Overall, the decision of continuation, dropping, merging, promoting of certain topics or sub-topics should be based on the companies’ inputs and outcomes of the email discussions, in a case by case manner. Also, the decision should not be solely based on whether a topic/sub-topic is listed as controversial or not in the email discussion summary as the criteria is quite in-consistent across different email threads. Specifically, on the handling of uplink enhancements, it is fine for us to merge the uplink MIMO related discussion with downlink MIMO to move forward. However, during the email discussion, many showed concern that these uplink MIMO related proposals are really only relevant to CPE-like devices and not much for normal devices like smartphones. So these proposals should be further discussed (under MIMO item). For the rest of the proposals, enhancement for multi-carrier UL operation has very good support from the companies and can be handled under an uplink enhancement item together with uplink coverage enhancements. Therefore, we’d like to suggest to change the title of the 2nd RAN1 item to “Uplink enhancements (excluding uplink MIMO)” to better reflect the situation. On enhancements for XR, it is ok to leave the decision of leading WG (RAN1 vs RAN2) later after the scope is stabilized. One issue we see is that the ongoing XR SI outcomes are not available and the scoping decision should be based on the SI outcomes instead of bypassing it and hence waste the efforts the companies put in. We also agree with some companies’ online comments that a discussion on the set of other RAN1 items at this stage should be kept to include items with a good amount of support such as flexible spectrum integration and passive IoT. We understand the need to control the overall scope of R18. Based on further discussion, some of these other items can be part of the R18 work in the place of some/part of the separately listed items. Also about TU estimation, we think it should be left for later discussion. |
| CMCC | Firstly, we think the features introduced at this stage, it is important to see if the feature itself is helpful for improving the service availability, for exploiting the new market or new service, in this sense, we think the list should be inclusive by considering more topics of “from 0 to 1” rather than emphasizing more of the enhancements of “from 1 to n”. Secondly, we should consider the history of those features proposed to continue evolution, if the proposal whose preceding project has confronted rescoping or more troubles should be suspended for a while, from CMCC point of view, we are conservative toward these proposals. Including DSS, Sidelink, IAB, etc. In addition, if we recall the procedure of developing SI for XR, we see a lot of challenge and suspect, but finally, the group made it, which leads to today’s seemingly unanimous proposal for WI for XR. Today, Passive IoT is facing similar situation, we hope this group could also look into it and initiate some activity to the extend 3GPP’s coverage. For the item of flexible spectrum, we know a lot of companies are favor of existing frame work of CA, but we should just simply single out the proposal by simple preference, if go with way, each company will have his own preference, we propose to have a study to identify if there is any advantage of Framework 1 in the project involving CA/DC as below  Study and evaluate the potential gain/benefit of Framework 1 over Framework 2 with multiple carriers within one or more bands, including - Flexible DL and UL carriers selection for initial access procedure and carrier selection/aggregation for subsequent data transmission - The study shall consider RRC_IDLE, RRC_INACTIVE, and RRC_CONNECTED states Note: Framework 1: The multiple carriers within one or more bands are modeled as one serving cell; Framework 2: The multiple carriers within the one or more bands are modeled as multiple serving cells (current CA framework) A bit more specific, we hope the group could seriously consider some of proposals so called controversial or lack of support, do not just simply say it can be done by this way or that way. Just as Chair and a lot of companies claimed, we should consider the commercial deployment. From CMCC point of view, we have deployed the more than 400,000 5G base stations and we have visited a lot of customers, we believe some of the commercial interests and challenge identified in our network should be respected. A bit more specific, UE aggregation and Dual-UE backup. For the UE aggregation, we are very disappointed at some companies repeated comments, saying it can be left to application, to be transparent to network, which has ever been adopted for some urgent service and request customizing the application and server, do those opponent companies believe it is a right way to advertise 5G capability? For Dual-UE backup, it is really a missing part of URLCC considering the whole end-to-end URLLC solution, since till now, the group did not take the UE reliability as potential risk. Some companies may claim it can be resolved by implementation by duplicated resources, if thus, please these companies have a check if any features proposed in this release can improve the spectrum or resource utilization by 100%? |
| Spreadtrum | Thank Wanshi for great efforts. For slide 14, we share online comment from CMCC: “UE back-up” is an interested topic to industry and should be included in R18, e.g. in “UE aggregation” item. |
| LG Electronics | We think slide 14 does not capture the outcome of [17] correctly. The first three bullets, i.e. inter-gNB coordination, UE aggregation, and High-speed packetization are lack of consensus, and it may not be worth to continue the discussion. Companies in each camp would bring same argument again, and situation will be pretty much same even after another round of discussion. The remaining three bullets, i.e. SDT, Network slicing enh., and Security enh. have enough consensus to keep working in Rel-18. Regarding SDT, there are clear consensus on MT-triggered SDT, and we don’t see any contentious issue. For other SDT objectives, further discussions are needed to make reasonable and acceptable scope. Regarding WUR/WUS for RedCap in slide 13, we are not very supportive for that item at the moment, and at least the practical benefit/feasibility should be studied first if we are to include such item. Regarding UAV in slide 34, we think drone identification over PC5 is better to be discussed in sidelink enhancement. Since PC5 topic has been discussed separately from Uu topic, it would be better to keep this principle. |
| Nokia | The RAN1 part looks quite full, thus adding new items for discussion may not be beneficial for convergence. The current list presents well the most popular items from industry, especially when in reality the main objectives for XR for NR also fall under RAN1 leadership (understood from the discussion the leading working group to be discussed). For the RAN2 part, CA/DC enhancements have strong support both from vendors and operators and therefore, it should be explicitly included to the List of Potential RAN2-led Items for Subsequent Discussion till RAN#94-e either as a dedicated item or combined to Mobility e.g. as Mobility and CA/DC Enhancements. Dedicated discussion on the scope of CA/DC enhancements will also facilitate the fast alignment on the objectives. SDT enhancements have also good support and even stronger support than some of the other items currently explicitly listed as potential RAN2-led items. For some of the SDT enhancements there was also rather good alignment between the companies. It would be good to mention more explicitly that there are topics coming from SA, which will require also RAN work e.g. resilient timing. |
| Lenovo, Motorola Mobility | Extension/Enhancements beyond 52.6 Based on the non-controversial recommendation of the moderator from NWM for B52.6 enhancements/extension, our understanding is that we need to continue the discuss on the two topics related to further enhancements and extension beyond 52.6GHz. In our view, there is reasonable support/interest from companies to consider one or the other topic and we suggest having continued discussion to prioritize one topic among the two: - Topic 1: Further enhancements from 52.6-71GHz - Topic 2: Extension beyond 71GHz Further clarification and guidance is needed on any potential leftover items for 52.6-71GHz. Currently, there is some discussion in RAN#93-e, to down-scope or down prioritize some of the aspects related to channel access in Rel-17. Will these leftover aspects be dropped, or would there be a potential discussion during RAN plenary in December to consider them for Rel-18? Coverage Enhancements Based on the currently listed example areas for coverage enhancements, is it that only UL related coverage enhancements will be considered under this potential SI/WI in Rel-18? Or there can be discussion on the DL aspects as well? We think that DL coverage enhancements are also crucial for especially unlicensed channel access in FR2-2 due to regulatory limitations on EIRP. IIoT/URLLC: Based on the NWM for IIoT/URLLC, although there are diverse views on the topics to be studied for IIoT/URLLC in Rel18, relatively wide support is present even from vertical industry to study the feasibility of using Sidelink for IIoT/URLLC. We prefer at this stage to focus on the market needs from vertical partners, and the technical areas to be studied with deciding in RAN#94-e on how/where to handle this Study item. AI/ML for NG-RAN: Regarding AI for NG-RAN, the work should include two parts: - A normative work for the study outcomes including the prioritized use cases (e.g., energy saving, load balance, and mobility optimization) following the conclusions of 3GPP RAN3 R17 study item. - A further study on potential RAN enhancements to support distributed intelligence to fully utilize the AI/ML capability among dense deployed RAN nodes. For example, RAN can study mechanisms to facilitate AI management procedures, AI/ML model distributing. As discussed in the email discussion, many companies support to have a WI fo the normative work and a separate SI for the further study part. CA/DC enhancement: During June workshop and August NWM discussion, many companies support CA/DC enhancement in Rel-18 with focus on multi-carrier scheduling, FR2 SCell scheduling PUSCH on PCell and MR-MC. From our side, we think CA/DC enhancement is quite important to improve eMBB experience and would like to propose further discussion with focus on detailed scope for CA and DC. |
| Apple | Thanks Wanshi for your efforts and guidance. 1) Regarding the RedCap / LP WUR: Even though we understand the intention is to decide later whether to have a separate item for LP WUR/WUS, we think it is better to discuss the topic in a separate email thread, not as part of eRedCap email. As commented by a lot of companies during the prep email discussion, this is a new topic that requires investigation on many aspects, most of which are orthogonal to the other eRedCap enhancements. Therefore, we should initially be open to consider all the UEs categories, and the final applicability should be the outcome of the study. A separate email thread for LP WUR/WUS would allow a more focused discussion to identify the proper objectives, and it would also allow a more focused discussion for RedCap enhancements. 2) Regarding MIMO We are supportive of merging DL and UL MIMO related aspects, which makes more sense logically. However, for the detailed objectives in slide 19, we think a dedicated bullet on "latency reduction for beam management, e.g. UE initiated beam management” should be added, for which there were a number of proponents during the email discussion. We also think the CPE related enhancement should be removed/de-prioritized, as it was labelled as controversial and low priority in moderator’s summary. 3) Regarding the additional email thread for potential RAN2/3-led topics on slide 14 We would also like to include a couple of user plane enhancement proposals submitted by us for this meeting (2354 on SDAP enhancement, and 2357 on sub-DRB differentiation). 4) on SL relay: We think it is better to clarify that the exact scope of SL Relay WI in R18 shall still be open for subsequent discussion, and not limited to the items listed in Slide 31. There are valid use cases and business interests in support multi-hop work for further coverage extension with UE-to-NW relay and UE-to-UE relay, as discussed in Apple paper RP-212356. 5) RAN4 related topics: It seems only high level guidance is provided for RAN4-led WI/SI and no candidate list is included in RP-211679. If RAN4-led “new areas” are targeted to be approved in Dec., we should at least have a stable candidate list in this meeting as how RAN1/2/3 proposals are handled. Also, we need to clarify the objective and agenda of the planned email discussion for RAN4-led topics during Oct. 18th – 27th since there is no candidate list. On slide 12, what does the following bullet mean? “A certain amount of RAN1/2/3 WG capacity is to be reserved when approving the package in December" … Is RAN1/2/3 capacity/TU reserved for RAN4-led items which impact other WG? We are OK with single email thread approach for RAN4 proposals during Dec. plenary. However, if there is pre-plenary email discussion, email thread dedicated to specific topics can be more efficient to make the progress. |

Questions:

- Any questions/comments/suggestion to other aspects related to RP-211679?

| Company | Views |
| --- | --- |
| vivo | On handling of “controversial” areas We understand the example areas for each email threads are mostly based on the claimed “non-controversial” part from the moderator’s summary. However, since the listed example areas are “for information only”, can the “a bit more controversial” areas still be discussed in the next step for potential convergence? We think this question applies to many topics, as one particular example, we see some additional cost/complexity reduction features received good support in previous round of discussion eRedCap but currently not listed in Page 23, as they were not stable enough to be marked as “non-controversial” for now. |
| Samsung | MIMO: On slide 19 (list of areas) - On DL aspects: - Enhancement on coherent JT (CJT) is supported by a number of companies especially operators (cf. RP-212524). This should be included. - Multi-beam enhancement to reduce overhead/latency (especially UE-initiated beam management) is also supported by many companies. It is unclear why this is not included. - In general, if the list is based only on the non-controversial parts, “Other CPE-specific considerations” should also be removed to be consistent (it was labeled controversial and low priority by the moderator). - On UL aspects: - >4Tx UL: While we don’t oppose this area of enhancement, the scope of this enhancement can be excessive if the WID is not specific about the areas of enhancements. - “Potentially other UL enhancements, e.g. ...” should either be removed or replaced by a more specific task description. As of now this looks like a container for a handful of different proposals supported by only 1-2 companies. |
| Futurewei | The overlapping between the RAN1 meeting and email discussion should be avoided. Towards the end of the meeting, it is really hard for the delegate to handle this extra work. Competing Rel-17 should take highest priority at this stage. About the detailed scope of each item, our understanding is that the email discussion summary should server as the starting point for further discussion. |
| Spreadtrum | For slide 6, we share online comment from Samsung: the arrangement for October R18 email discussion should be avoided overlapping with RAN1 e-meeting. For slide 23 RedCap Evolution: - As quite some companies proposed the following topics during NWM discussion, we suggest to include those topics as well for next step discussion: - reduced number of HARQ processes - relaxed UE processing time - coverage recovery |
| LG Electronics | As commented by Samsung in Monday GTW session, the next e-mail discussion schedule is partly overlapped with October RAN1 meeting (the RAN1 meeting lasts until October 19 while the e-mail discussion starts on October 18). As same delegates are involved with both RAN1 discussion and R18 discussion, the delegates overhead are extremely large in this overlapped period. We hope the e-mail discussion schedule is a bit adjusted to avoid overlap with WG meetings. Regarding RAN4 Rel-18 packages, we are ok to approve spectrum related packages in December. However, for non-spectrum related packages, we think it would be better to approve them in March next year, since it would be more efficient to have a single-step approval and also it would be difficult to know what is new area or not when approving non-spectrum WI in December. |
| Apple | It would be good to list on slides 9, 10, and 11 also the secondary working groups for each topic, potentially with TU allocations. Regarding “operator support” on slide 12, we agree with operators’ views can largely represent commercial interests. Operators’ inputs are always welcome. Meanwhile, NW and UE vendors’ commercial interests and limitation, especially implementation limitation, should be equally considered as well. |

## Intermediate Round

## Final Round

# Conclusion

Based on the email discussion, the following are proposed:

TBD

# References

RP-211679	RAN Chair’s Summary for RAN Release 18	RAN Chair (Qualcomm)