3GPP TSG RAN#93	RP-21xxxx

- Electronic Meeting, September 13 – September 17, 2021

Source:	Moderator (Huawei)

Title:	Moderator’s Summary for [93e-06-pi2-BPSK]

Agenda item:	9.1

Document for:	Discussion and decision

# Introduction

Document RP-211761 [1] was submitted to RAN#93e with the following proposal:

Proposal 1: Rel-16 pi/2 BPSK with 2-tap filter (1+D) to be specified in 3GPP specifications as a mandatory feature for Rel-17 UEs. Note that the UE TX power should be delivered by the existing MPR tables in Rel-16 specifications i.e., no new requirements are necessary for Rel-17 UEs.

Proposal 2: Increased UE Tx power (with a single PA up to 32 dBm) should be targeted by using the existing RAN4 study item in Rel-17 which may possibly necessitate the definition of a new power class.

The present document summarizes the RAN#93e discussions on the proposals, using email thread [93e-06-pi2-BPSK].

# Initial Round

Companies are invited to provide their views using the tables below.

Proposal 1: Rel-16 pi/2 BPSK with 2-tap filter (1+D) to be specified in 3GPP specifications as a mandatory feature for Rel-17 UEs. Note that the UE TX power should be delivered by the existing MPR tables in Rel-16 specifications i.e., no new requirements are necessary for Rel-17 UEs.

| Company | Comments on proposal 1 |
| --- | --- |
| Qualcomm | We are open to discussion in conjunction with a generalized merger proposal based on the suggestion in RP-211761: ‘The proposals if incorporated in the current from would enable a “merger” between the 5Gi pi/2 BPSK waveform with that of the 3GPP 5G NR version.’ |
| T-Mobile USA | Question 1) This proposal is only for bands n40, n41, n77, n78 and n79 as indicated in Table 6.2.2-1, correct? Question 2) Does this mean that support for powerBoosting-pi2BPSK would also be mandatory? For Rel-17 we think powerBoosting-pi2BPSK can only be mandatory for PC3 UEs. Since MPR is not defined yet for pi/2 BPSK with spectral shaping and power boosting for power class 2, it doesn’t make sense to us to require pi/2 BPSK with 2-tap filter (1+D) for Power Class 2 UEs. |
| Huawei, HiSilicon | In our understanding, Pi/2 BPSK is an optional feature for FR1 from Rel-15. And from the very beginning, it was agreed in both RAN1 and RAN4 that the specific filter type is not mandated, it is up to UE implementation. We think the proposal is not aligned with the previous agreements. Very strong justification is needed to take this proposal 1. |
| Ericsson | We welcome any proposal to align 5Gi with 3GPP 5G NR. However, any proposal should also be technically justified. Mandating a UE feature will have to be motivated by a solid performance advantage. So far, a significant advantage of the proposed 2-tap filter as compared to the (transparent) filtering possible from Rel-15 has not been identified. |
| Skyworks | It should be clarified which baseline power class are mandated for power boosting and associated duty cycle |
| Nokia | We are open to discuss a compromise way forward for aligning 5Gi with 3GPP 5G NR. However, we should continue defining UE requirements like in the current specifications and as discussed in the Rel-17 study item, instead of trying to specify a certain shaping filter. |
| Intel | In our understanding 3GPP does not explicitly define specific type of pulse shaping filter and “2-tap filter (1+D)” is transparent to 3GPP specs. So, we would like to confirm with the proponents that the proposal is limited to “powerBoosting-pi2BPSK” UE feature and the intention is to make respective requirements in Rel-16 FR1 RF specifications mandatory for Rel-17 PC3 UEs. From our point of view, we are open to further discuss a compromise solution and consider a mandatory capability if it can help to harmonize with 5Gi and avoid technology fragmentation. |

Proposal 2: Increased UE Tx power (with a single PA up to 32 dBm) should be targeted by using the existing RAN4 study item in Rel-17 which may possibly necessitate the definition of a new power class.

| Company | Comments on proposal 2 |
| --- | --- |
| Qualcomm | We are open to discussion in conjunction with a generalized merger proposal based on the suggestion in RP-211761: ‘The proposals if incorporated in the current from would enable a “merger” between the 5Gi pi/2 BPSK waveform with that of the 3GPP 5G NR version.’ |
| T-Mobile USA | We support pi/2 BPSK with spectral shaping and power boosting for PC2 and PC1.5 and even PC1 (for FWA), but we would be hesitant about defining a new power class. |
| Huawei, HiSilicon | It’s not clear what the targeted UE type is for this proposal. The max output power supported for handheld UE is PC1.5, i.e. 29dBm. And the UE architecture for this power class is 2x26dBm PA configuration. In addition, the targeted output power is still under discussion in RAN4 without conclusion. We think it would be better to continue the discussion and make decision in RAN4 based on further technical analysis. |
| Ericsson | We welcome any proposal to align 5Gi with 3GPP 5G NR. We are supportive to continue studying an increased UE Tx power as per agreed SID. |
| Skyworks | The target power for single PA should be based on power boosting of an existing 1Tx power class. Currently this PC2 and based on the characteristics of real PC2 PA2 targeting high PAE, the Psat of such PAs is lower than 32dBm thus we cannot agree to a target that would require a dedicated PA design only for shaped Pi/2 BPSK. We urge companies to be very careful with simulated results because the PA models are very poor in predicting the behaviour in the Psat region where state of the art linear PAs rather act like a limiter. Without measurement it is premature to set a target power and we can accept an objective >29dBm with max boosting to be further evaluated. |
| Nokia | We are open to discuss a compromise way forward for aligning 5Gi with 3GPP 5G NR. However, the compromise proposal should be clear so that there is no space for different interpretations. The current proposals are not sufficiently clear. We are also supportive of continuing studies for increased UE Tx power under the ongoing study item. |
| Intel | Increased UE TX power class can be discussed in the ongoing SI and it is not precluded based on current SID. We are supportive of the studies and prefer to have a discussion in RAN4 first as a part of SI. |

Moderator’s summary of initial round

# Intermediate Round

# Final Round

# Conclusions

# References

- RP-211761, Way forward on pi/2 BPSK, IITH, CEWiT, IITM, Tejas Networks, Reliance Jio