3GPP TSG RAN#93	RP-21xxxx

- Electronic Meeting, September 13 – September 17, 2021

Source:	Moderator (Huawei)

Title:	Moderator’s Summary for [93e-06-pi2-BPSK]

Agenda item:	9.1

Document for:	Discussion and decision

# Introduction

Document RP-211761 [1] was submitted to RAN#93e with the following proposal:

Proposal 1: Rel-16 pi/2 BPSK with 2-tap filter (1+D) to be specified in 3GPP specifications as a mandatory feature for Rel-17 UEs. Note that the UE TX power should be delivered by the existing MPR tables in Rel-16 specifications i.e., no new requirements are necessary for Rel-17 UEs.

Proposal 2: Increased UE Tx power (with a single PA up to 32 dBm) should be targeted by using the existing RAN4 study item in Rel-17 which may possibly necessitate the definition of a new power class.

The present document summarizes the RAN#93e discussions on the proposals, using email thread [93e-06-pi2-BPSK].

# Initial Round

Companies are invited to provide their views using the tables below.

Proposal 1: Rel-16 pi/2 BPSK with 2-tap filter (1+D) to be specified in 3GPP specifications as a mandatory feature for Rel-17 UEs. Note that the UE TX power should be delivered by the existing MPR tables in Rel-16 specifications i.e., no new requirements are necessary for Rel-17 UEs.

| Company | Comments on proposal 1 |
| --- | --- |
| Qualcomm | We are open to discussion in conjunction with a generalized merger proposal based on the suggestion in RP-211761: ‘The proposals if incorporated in the current from would enable a “merger” between the 5Gi pi/2 BPSK waveform with that of the 3GPP 5G NR version.’ |
| T-Mobile USA | Question 1) This proposal is only for bands n40, n41, n77, n78 and n79 as indicated in Table 6.2.2-1, correct? Question 2) Does this mean that support for powerBoosting-pi2BPSK would also be mandatory? For Rel-17 we think powerBoosting-pi2BPSK can only be mandatory for PC3 UEs. Since MPR is not defined yet for pi/2 BPSK with spectral shaping and power boosting for power class 2, it doesn’t make sense to us to require pi/2 BPSK with 2-tap filter (1+D) for Power Class 2 UEs. |
| Huawei, HiSilicon | In our understanding, Pi/2 BPSK is an optional feature for FR1 from Rel-15. And from the very beginning, it was agreed in both RAN1 and RAN4 that the specific filter type is not mandated, it is up to UE implementation. We think the proposal is not aligned with the previous agreements. Very strong justification is needed to take this proposal 1. |
| Ericsson | We welcome any proposal to align 5Gi with 3GPP 5G NR. However, any proposal should also be technically justified. Mandating a UE feature will have to be motivated by a solid performance advantage. So far, a significant advantage of the proposed 2-tap filter as compared to the (transparent) filtering possible from Rel-15 has not been identified. |
| Skyworks | It should be clarified which baseline power class are mandated for power boosting and associated duty cycle |
| Nokia | We are open to discuss a compromise way forward for aligning 5Gi with 3GPP 5G NR. However, we should continue defining UE requirements like in the current specifications and as discussed in the Rel-17 study item, instead of trying to specify a certain shaping filter. |
| Intel | In our understanding 3GPP does not explicitly define specific type of pulse shaping filter and “2-tap filter (1+D)” is transparent to 3GPP specs. So, we would like to confirm with the proponents that the proposal is limited to “powerBoosting-pi2BPSK” UE feature and the intention is to make respective requirements in Rel-16 FR1 RF specifications mandatory for Rel-17 PC3 UEs. From our point of view, we are open to further discuss a compromise solution and consider a mandatory capability if it can help to harmonize with 5Gi and avoid technology fragmentation. |
| IITH, IITM, CEWIT, Reliance Jio, Tejas Networks | Since Rel-15, 5G NR has undergone multiple releases and yet pi/2 BPSK with spectrum shaping is not optimally exploited for deployments. The key elements in enabling this feature are the a) specification of spectrum shaping, b) MPR tables/UE power classes that enable optimum exploitation of this spectrum shaping and c) making this a mandatory feature. The benefits of 1+D filter over any other filter has been demonstrated since Rel-15 days. The benefits are: - pi/2 BPSK DFT-s-OFDM with 1+D filter is shown to reach 32dBm with a PC2 device which is close to the saturation level of the PA. This benefit is purely because of the strong spectrum shaping using the 1+D filter. This large performance benefit can be obtained using the existing PA technology. The ability to reach up to 32 dBm with single PA is a remarkable feature for networks that aim for large coverage. - Regarding, the filter choice, 1+D filter has minimal implementation complexity for the UE compared to any other filter. - The dispersion caused by 1+D filter is least compared to any other filter. This has obvious benefits for channel estimation. - Not specifying the filter and leaving it open for implementation does not guarantee the above benefits. - Given that it is imperative to define the filter to guarantee the performance, 1+D filter is the optimum choice and therefore deserves introduction in the specification as a mandatory feature. |
| ZTE | Usually shaping filter related issue should be left up to the implementation in past, therefore we don't see its necessity of putting that filter type explicitly in that proposal. In addition, there were similar discussion in RAN4 leading SID "Optimization of pi/2 BPSK uplink power in NR". However we are also open to further discuss it to harmonize with 5Gi and 3GPP tech. |
| Vodafone | We support the objective to have a single 5G standard. Ideally this should be based on the technical results from 3GPP studies (which seem to be still ongoing in RAN 1?) and take into account the technical comments from e.g. Intel, above, and practical aspects, e.g. from Skyworks, below. |
| MediaTek | Just some question for clarification. As this is a study item, we are not 100% sure if we can mandate a certain implementation at this stage. It seems more like a WI discussion? |
| Samsung | It has been our view that new features in Rel-16 and later releases have to be UE-optional as a general principle. It is not clear to us why pi/2-BPSK has to be handled differently. We don’t see a good reason why the (1+D) filter has to be specified. As discussed before in 3GPP, the UE tx filter can be left as an implementation choice. |
| Apple | We appreciate and support the efforts to align 5Gi with 3GPP NR. We prefer to leave it as UE’s implementation. Meanwhile, to ensure the harmonization between 5Gi and 3GPP NR, we are open to further discuss the other options in the SI. Regarding MRP requirements, existing requirements for PC2 MPR cannot be met by pi/2 BPSK with 26 dBm + 3 dB power boost. The following mitigations should be considered: MPR relaxation, RB allocation restrictions, or a different shaping filter. |

Proposal 2: Increased UE Tx power (with a single PA up to 32 dBm) should be targeted by using the existing RAN4 study item in Rel-17 which may possibly necessitate the definition of a new power class.

| Company | Comments on proposal 2 |
| --- | --- |
| Qualcomm | We are open to discussion in conjunction with a generalized merger proposal based on the suggestion in RP-211761: ‘The proposals if incorporated in the current from would enable a “merger” between the 5Gi pi/2 BPSK waveform with that of the 3GPP 5G NR version.’ |
| T-Mobile USA | We support pi/2 BPSK with spectral shaping and power boosting for PC2 and PC1.5 and even PC1 (for FWA), but we would be hesitant about defining a new power class. |
| Huawei, HiSilicon | It’s not clear what the targeted UE type is for this proposal. The max output power supported for handheld UE is PC1.5, i.e. 29dBm. And the UE architecture for this power class is 2x26dBm PA configuration. In addition, the targeted output power is still under discussion in RAN4 without conclusion. We think it would be better to continue the discussion and make decision in RAN4 based on further technical analysis. |
| Ericsson | We welcome any proposal to align 5Gi with 3GPP 5G NR. We are supportive to continue studying an increased UE Tx power as per agreed SID. |
| Skyworks | The target power for single PA should be based on power boosting of an existing 1Tx power class. Currently this PC2 and based on the characteristics of real PC2 PA2 targeting high PAE, the Psat of such PAs is lower than 32dBm thus we cannot agree to a target that would require a dedicated PA design only for shaped Pi/2 BPSK. We urge companies to be very careful with simulated results because the PA models are very poor in predicting the behaviour in the Psat region where state of the art linear PAs rather act like a limiter. Without measurement it is premature to set a target power and we can accept an objective >29dBm with max boosting to be further evaluated. |
| Nokia | We are open to discuss a compromise way forward for aligning 5Gi with 3GPP 5G NR. However, the compromise proposal should be clear so that there is no space for different interpretations. The current proposals are not sufficiently clear. We are also supportive of continuing studies for increased UE Tx power under the ongoing study item. |
| Intel | Increased UE TX power class can be discussed in the ongoing SI and it is not precluded based on current SID. We are supportive of the studies and prefer to have a discussion in RAN4 first as a part of SI. |
| IITH, IITM, CEWIT, Reliance Jio, Tejas Networks | 5Gi is expected to enable LMLC deployments with large rural cells. Therefore, a UE that achieves up to 32dBm with single PA is our target to enable this large rural coverage. As mentioned before, our measurements show that we can indeed achieve 32dBm using a single commercial PC2 PA. This can be enabled as a power boost to an existing power class or a new power class definition. We are open for either approach. However, we are keen to see 5G deployments in India obtain the benefits of this feature in a reasonable time frame. |
| ZTE | This was still under the discussion in RAN4, the initial agreement is to have PC2 UE as baseline for the power enhancement. |
| MediaTek | We are open to continue the discussion in RAN4. |
| Samsung | Increased UE TX power can be discussed in the SI according to the usual practice in 3GPP. We would not feel comfortable to set a kind of target value (up to 32 dBm with a single PA) already now |
| Apple | Feasibility to increase UE Tx power with single PA up to 32dBm is questionable. 29dBm can be considered as a compromised alternative for this release. We are open for further discussion in this SI. |

Moderator’s summary of initial round

Questions for clarification on proposal 1 were received from several companies. It was asked whether the proposal should be limited to mandating “powerBoosting-pi2BPSK” rather than a specific filter implementation, which many companies prefer to leave to implementation. One company asked whether the proposal is only for bands n40, n41, n77, n78 and n79.

While some companies expressed a preference to leave the feature optional, some other companies indicated that the proposal should be discussed along with the possibility to “merge” the pi/2 BPSK versions of 5Gi and 3GPP 5G. How such merge is defined or implemented would require further discussions.

On proposal 2, similar to past discussions in RAN plenary, most companies prefer continuing the discussion on proposal 2 (as well as proposal 1) under the on-going RAN4 study, including the target/feasible output power (29 dBm, 32 dBm, or something else), the targeted UE types, and whether or not this would require a new UE power class.

In conclusion of the initial round, there seems little chances to agree on the proposals exactly as formulated in RP-211761. Questions for clarifications on the targeted bands, targeted UE types, targeted and feasible output powers, whether to define new UE power classes, can continue being discussed in the intermediate round of discussions.

# Intermediate round

For this round of discussion, the proponents and other companies are encouraged to continue the discussion and possibly propose revisions to the proposals to address the questions received in the first round on shaping filter, powerBoosting-pi2BPSK, targeted bands, targeted UE types, targeted output power, and UE power classes.

| Company | Further comments on points raised in the initial round |
| --- | --- |
| Qualcomm | Following up on offline phone call, we propose to use this round to fine tune wording of counter proposals that may be acceptable to 3GPP towards standards merge: Would companies please use dedicated tables below for comments? |

Companies are invited to comment on the updated proposals 1A and 1B using the table below.

| Company | Proposal 1A: Rel-16 pi/2 BPSK to be specified in 3GPP specifications as a mandatory feature for Rel-17 UEs. Note that the UE TX power should be delivered by the existing MPR tables in Rel-16 specifications i.e., no new requirements are necessary for Rel-17 UEs. Vs Proposal 1B: Rel-16 pi/2 BPSK with pulse shaping filter (with unspecified coefficients) to be specified in 3GPP specifications as a mandatory feature for Rel-17 UEs. Note that the UE TX power should be delivered by the existing MPR tables in Rel-16 specifications i.e., no new requirements are necessary for Rel-17 UEs. |
| --- | --- |
| Qualcomm | We support Proposal 1A: We think 1A already captures pulse shaping filter parameters, and there is no further need to specify use of filter as proposed in 1B. Perhaps proponents of 1B can reword 1B better. |
| T-Mobile USA | Our comments and questions in the first round haven’t been answered: Question 1) This proposal is only for bands n40, n41, n77, n78 and n79 as indicated in Table 6.2.2-1, correct? Question 2) Does this mean that support for powerBoosting-pi2BPSK would also be mandatory? We are opposed to Proposal 1A or 1B being mandatory for PC2 Rel-17 UEs. It should also be clarified what is mean by “Rel-16 pi/2 BPSK to be specified in 3GPP specifications as a mandatory.” There are several capabilities with “pi/2 BPSK listed in 38.306, so are these proposals for all of them? lowPAPR-DMRS-PUCCH-r16 Indicates whether the UE supports low PAPR DMRS for PUCCH format 3 and format 4 with transform precoding and with pi/2 BPSK modulation. UE indicates support of this feature shall indicate support of pucch-F3-4-HalfPi-BPSK and any combination of support of pucch-F3-WithFH, pucch-F4-WithFH and pucch-F1-3-4WithoutFH.; Band; No; N/A; N/A. lowPAPR-DMRS-PUSCHwithPrecoding-r16 Indicates whether the UE supports low PAPR DMRS for PUSCH with transform precoding and with pi/2 BPSK modulation. UE indicates support of this feature shall indicate support of pusch-HalfPi-BPSK.; Band; No; N/A; N/A. powerBoosting-pi2BPSK Indicates whether UE supports power boosting for pi/2 BPSK, when applicable as defined in 6.2 of TS 38.101-1 [2]. This capability is not applicable to IAB-MT.; Band; No; TDD only; FR1 only. pucch-F3-4-HalfPi-BPSK Indicates whether the UE supports pi/2-BPSK for PUCCH format 3/4 as defined in 6.3.2.6 of TS 38.211 [6]. It is optional for FR1 and mandatory with capability signalling for FR2. This capability is not applicable to IAB-MT.; UE; CY; No; Yes. pusch-HalfPi-BPSK Indicates whether the UE supports pi/2-BPSK modulation scheme for PUSCH as defined in 6.3.1.2 of TS 38.211 [6]. It is optional for FR1 and mandatory with capability signalling for FR2. This capability is not applicable to IAB-MT.; UE; CY; No; Yes. |
| Apple | UE Tx power for pi/2 BPSK is still under discussion. If increased Tx power from the existing 23dBm+3dB power boosting is introduced, it should be discussed and decided in RAN4 how the existing requirements can apply and if new requirements and/or new applicability rules should be further specified. The following wording “Note that the UE TX power should be delivered by the existing MPR tables in Rel-16 specifications i.e., no new requirements are necessary for Rel-17 UEs.” is premature at this stage and should be discussed in RAN4. |

Companies are invited to comment on the updated proposal 2A using the table below.

| Company | Proposal 2A: Increased UE Tx power should be targeted by using the existing RAN4 study item in Rel-17 which may possibly necessitate the definition of a new power class. |
| --- | --- |
| Qualcomm | We are ok to support this proposal |
| T-Mobile USA | The current SID says: Note: whether or not a new UE power class will be introduced for the identified achievable UE Tx power for pi/2 BPSK will be decided at the drafting stage of the following WI. We are opposed to Proposal 2A. Increased UE Tx power is already targeted by the existing RAN4 SI, and it was already decided that the need for a new power class would be decided at the drafting stage of the following WI. |
| Apple | We agree with TMUS’s observation. However, we can compromise with the proposal to discuss power class issue during SI. |

# Conclusion

# References

- RP-211761, Way forward on pi/2 BPSK, IITH, CEWiT, IITM, Tejas Networks, Reliance Jio