3GPP TSG RAN Meeting #93e	Draft-RP-212633

Electronic Meeting, September 13 - 17, 2021	(revision of RP-211903)

- Source:	China Unicom

- Title:	New WID on high power UE (power class 2) for NR FDD band

- Document for:	Approval

Agenda Item:	9.1.4

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

# Title: High power UE (power class 2) for NR FDD band

## Acronym: NR_PC2_UE_FDD

## Unique identifier: 	{A number to be provided by MCC at the plenary}

## 1	Impacts 	{ For Normative work, identify the anticipated impacts. For a Study, identify the scope of the study.}

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  | X |  |  |  |
| No | X |  | X | X |  |
| Don't know |  |  |  |  |  |

## 2	Classification of the Work Item and linked work items

### 2.1	Primary classification

This work item is a … {Tick one box. "Feature / Building Block / Work Task" form a hierarchical structure. E.g. no Building Block can be proposed without a corresponding parent Feature. The full structure of all existing Work Items is shown in the 3GPP Work Plan in ftp://ftp.3gpp.org/Information/WORK_PLAN }

|  | Feature |
| --- | --- |
| X | Building Block |
|  | Work Task |
|  | Study Item |

### 2.2	Parent Work Item

{"Parent" Work Item refers to the related, earlier Stage, Work Item, e.g. the related Stage 1 Work Item shall be indicated here when a Stage 2 Work Item is presented or e.g. the related Study Item shall be indicated here when a normative-work Work Items is started. List here all parent Work Items of which requirements are either fully or partially covered by the proposed Item. List previous Work Items of earlier releases if relevant.}

{This section is mandatory to be filled out by the rapporteur.}

{Not applicable for a Study Item}

{For a Feature: list here the parent Study Item if applicable}

{For a Building Block: list here the parent Feature }

{For a Work Task: list here the parent Building Block }

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |

### 2.3	Other related Work Items and dependencies

{List here other Work Items which relate to the proposed one, such as preceding SI or a preceding WI (e.g. if further enhancing a feature).}

| Other related Work Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |
|  |  | {optional free text} |

Dependency on non-3GPP (draft) specification:

{This section is to be typically used to identify the IETF dependencies. Delete the header "Dependency on non-3GPP (draft) specification:" if no such dependency.}

## 3	Justification

Increasing the transmit power of UE has significant benefits on extending cell coverage area and improving the experience of cell edge users. In Rel-16 several study/work items related to HPUE have been proposed and worked on to standardize the requirements for EN-DC scenarios. In Rel-17, HPUE for NR CA and SUL is also specified.

However, extension of cell coverage for lower-frequency NR SA FDD band is also important for operators, because uplink data rate is not satisfactory even in FDD bands. Therefore increment of UE transmit power would be beneficial for improving the uplink performance for all users.

In the SI on FS_NR_PC2_UE_FDD, several SAR compliance solutions for NR FDD PC2 were studied, companies reached on consensus that UE-implementation based method would be used to make sure SAR regulation is not violated. System performance gain by introducing FDD PC2 was verified by dynamic system level simulations and Monte Carlo simulations. It was also agreed by companies that MSD method will be adopted for n1 and n3 PC2 to handle REFSENS degradations, the results captured in TR38.861 could be considered as starting point to determine the MSD values for FDD PC2 UE in WI phase.

This Work Item is proposed to work on specifying MSD requirements for NR FDD band n1 and n3 PC2.

## 4	Objective

### 4.1	Objective of SI or Core part WI or Testing part WI

The objectives of the core part are as follows, taking all the outcome from SI captured in TR 38.861 into account,

- Specify RF requirements for n1 and n3 to support both 1Tx and 2Tx architecture

- Specify UE maximum output power, Tx power tolerance for band n1 and n3.

- Specify A-MPR requirements for band n1 and n3 if needed

- Specify PC2 MSD requirements for NR band n1 and n3.

- Investigate HD-duplex solution which targets at reduce MSD.

Note 1: The ability to define 1Tx and 2Tx requirements is subject to data availability.  If sufficient data is not available and/or agreement cannot be reached, the work item can be closed as long as one set of requirements is defined.

Note 2: The applicability of requirements and optional or mandatory for features can be further discussed.

Note 3: Ensure that the UE RF requirements of power class 2 UEs shall comply with those of power class 3 when the maximum transmit power is limited to 23dBm by gNB configuration.

### 4.2	Objective of Performance part WI

Specify the necessary performance requirements such as release independence in TS 38.307.

## 5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Rapporteur |

{Note 1: Only TSs may contain normative provisions. Study Items shall create or impact only TRs.
"Internal TR" is intended for 3GPP internal use only whereas "External TR" may be transposed by OPs.}

{Note 2: The first listed Rapporteur is the specification primary Rapporteur. Secondary Rapporteur(s) are possible for particular aspect(s) of the TS/TR. In this case, their responsibility has to be provided as "Remarks".}

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| 38.101-1 | Add PC2 FDD to User Equipment (UE) radio transmission and reception | TSG#95 | Core part |
| 38.307 | Add PC2 EN-DC Requirements on User Equipment (UEs) supporting a release-independent frequency band | RAN#95 | Perf. part |

## 6	Work item Rapporteur(s)

Basaier Jialade

Company:	China Unicom

Email:	basejld@chinaunicom.cn

## 7	Work item leadership

RAN WG4

## 8	Aspects that involve other WGs

{Specify all the other WG(s) to be involved and, if specific, their task. E.g.: "SA2, SA3, SA5. CT6 for storage, and potentially SA4". If not applicable, indicate "None" or "None identified yet".}

## 9	Supporting Individual Members

{At least 4 supporting Individual Members are needed. There is an expectation that these companies will provide resources to progress the work. Note that having 4 supporting companies is a necessary but not sufficient condition: the usual TSG approval process by consensus is needed for the WID approval.}

Supporting IM name

China Unicom

China Telecom

Huawei

HiSilicon

ZTE

Sanechips

OPPO

CITC

Telecom Italia

Xiaomi

CATT