3GPP TSG RAN Meeting #93-e						RP-21xxxx

Electronic Meeting, September 13 - 17, 2021

Agenda Item:		9.3.2.3

Source:		Moderator (CMCC)

Title:				Email discussion on [93e-18-PowSav-WI]

Document for:		Discussion & Decision

# Introduction

In RAN#93-e, Status report [1] was flagged due to different understanding on the completion level and remaining issues, and other 7 contributions are brought up for seeking guidance from RAN on how to proceed the paging early indication (PEI) functionality, which was stuck in RAN1 discussion. From these contributions, there are 3 open issues to be settled at RAN#93-e:

Issue 1: How to proceed PEI in R17

Issue 2: Whether or how to involve RAN3 for supporting paging sub-grouping

Issue 3: Whether or what to modify the Status report [1]

# Initial Round Discussion

2.1	Issue 1 – How to proceed PEI in Rel-17

Individual company’s contribution [2] and [4] propose to Down-scope PEI from PowSav WI based on the concern on residual specification effort in left 2 two meetings. To the contrary, with confidence, a joint contribution [6] co-sourced by 34 companies and individual company’s contribution [3] and [8] propose to select PDCCH based PEI as the only solution for improving idle mode UE power saving in RAN#93-e. Individual company’s contribution [5] provides proposals on procedures for selecting the PEI candidate, which will not be easy in RAN1 to converge based on reality reflected in [2-8], and lead to either no consensus/agreement or no enough time to complete it in left two meetings, finally lead to without PEI in Rel-17 anyway. Based on the review of these contributions, Moderator would like to check companies’ view:

Is it acceptable to you to select PDCCH based PEI as the only option in RAN#93-e? Or

Is it acceptable to you to select PDCCH based PEI as the only option in RAN#93-e by adding some restriction, e.g., limited specification effort?

Otherwise, PEI seems to be down-scoped from Rel-17 automatically in large sense.

Table 1. Is it acceptable to you to select PDCCH based PEI as the only option in RAN#93-e?

| Item | Company | Comments on PEI decision (If any) |
| --- | --- | --- |
| 1 | SoftBank | Yes, PDCCH based PEI should be the only option. Given the large number of supporting companies of RP-212308, we don’t see the necessity of down-scoping of PEI. |
| 2 | MediaTek | Yes, we support approval of PDCCH-based PEI as the only option. RAN1 agreed to support PEI in RAN1#103-e, and other WGs already made progress based on PEI assumption. |
| 3 | NTT DOCOMO | Yes, PDCCH based PEI should be selected. |
| 4 | OPPO | Yes, all technical discussion happened in RAN1. Now we need to select PDCCH based PEI to help the process. The further restriction are technical details to be decided in RAN1. |
| 5 | Spreadtrum | Yes, PDCCH based PEI should be supported. As mentioned by some companies, DCI format can be reused. We don’t see risk for RAN1 progress. |
| 6 | CATT | No. PDCCH based PEI only provide limited power saving gain. The power saving gain with PDCCH based PEI is not clear due to assumption of number of SSBs used to decode PEI. The observation of power saving gains from different PEI candidates agreed in RAN1#105-e showed that the higher power saving gain from PDCCH based PEI is based on the assumption of 1 SSB used for PEI decoding. RAN4 reply LS in R1-2104170/R4-2105799 indicated that RS needs to be separated apart of 2 ms or 2 slots for AGC and channel tracking for fast SCell activation of CONNECTED mode UEs without out-of-sync to network. For IDLE/Inactive UEs in out-of-sync with the network, it implies that more than 2 SSBs are needed for UE to perform AGC and channel tracking. Thus, the power saving gain of PDCCH based PEI is over estimated due to the assumptions of 1 SSB used for PEI decoding by most companies. |
| 7 | MediaTek 2 | We would like to response to CATT concerns with the following comments: - RAN4 LS is for connected-mode UE with CA operation, there is no RAN4 agreement/observation that the same assumption applies to idle-mode UE with narrow-band operation - RAN4 LS suggests 2 TRS bursts for UE to be ready for PDSCH scheduling (as no further RS guaranteed before PDSCH), how this implies 2 SSB should be required for PDCCH PEI is not clear. Note that companies’ evaluations already show PDCCH can tolerate 0.5 – 1 ppm CFO which is much robust than PDSCH requiring <= 0.1 ppm. While we think UE implementation can always be updated/improved to exploit the robustness of PDCCH-based PEI and realize UE power saving gain, it can be a reasonable WF that RAN1 decides PDCCH-based PEI and LS to request RAN4 to take into account trade-off between configuring robust PDCCH-based PEI and minimum UE synchronization effort in RAN4 definition of performance test(s). Debating UE implementation in RAN1 is not efficient and causes pending decision. |
| 8 | Apple | Yes. We think PEI is a useful feature for UE power saving and it should not be down-scoped. The down-selection is necessary now in order to complete the work in Rel-17. |
| 9 | Lenovo/Motorola Mobility | Yes, PDDCH based PEI should be agreed without further delay. If PEI indicates a UE to skip paging DCI monitoring for more than one paging cycle (e.g. for UEs with delay tolerant applications), power saving gain from PEI is obvious, irrespective of number of SSBs UE has to detect before PEI detection. Further, TRS provision to idle/inactive UEs can further reduce the number of SSBs required for PEI detection. By reusing existing search space/DCI format designs as much as possible, PDCCH based PEI can be specified quickly during the remaining RAN1 meetings. |
| 10 | ITRI | Yes, PDCCH based PEI should be selected for further progress. |
| 11 | TCL | Yes, we support PDCCH based PEI for further progress. In our view, adding some restrictions to PDCCH based PEI may affect the PEI use for TRS/CSI-RS availability indication. Therefore, we would like to select the PDCCH based PEI option 1 without adding some restrictions. |
| 12 | Transsion Holdings | Yes,PDCCH based PEI should be supported . |
| 13 | ChinaTelecom | Yes, PDCCH based PEI should be selected as the only option. |
| 14 | Xiaomi | Yes, PDCCH based PEI should be selected as the only option. |
| 15 | InterDigital | Yes, we support PDCCH based PEI as the only option to be supported. Based on our evaluation, both sequence-based PEI and PDCCH-based PEI perform similar, therefore supporting one of the schemes shouldn’t be an issue. |
| 16 | FGI, Asia Pacific Telecom | Yes, given that there are only two remaining meetings, it’s better to down select one of the options in RAN#93. We support PDCCH-based PEI as the only option. Apart from power saving gain, another reason is more flexible to carry other information to increase the efficiency of system. Thus, it should be opened to further discuss whether other information can be included (e.g., TRS availability, SI/ETWS updates which have been proposed by several companies in RAN1). |
| 17 | Samsung | We have two main concerns on PDCCH-based PEI: 1) power saving gain of PDCCH-based PEI and 2) remaining works to do. Regarding power saving gain, even though proponents of PDCCH-based PEI showed that its gain is compatible with the other alternative, we are still questionable whether its power saving gain is meaningful as compared to legacy paging, especially considering typical/real/practical UE implementation. This is because the power saving gains are quite dependent on the assumptions about the number of SSBs that UE has to detect, UE’s SNR range, location of PEI w.r.t PO and so on. Also, PDCCH-based PEI has lots of works to do such as design of a new DCI format to support multiple functionalities such as TRS availability, SI update or ETWS. Having said that, we would like to propose that no PEI is supported in Rel-17. |
| 18 | LG | Yes, PDCCH based PEI should be selected as the only option in this plenary meeting. Also we believe that details of PEI functionality and designs can be discussed in WG level and can be finalized within remaining WG meetings. So there is no need for restrictions on standard work. |

2.2	Issue 2 - Whether or how to involve RAN3 for supporting paging sub-grouping

Contributions [8] proposes to update the WID to involve RAN3 according to the LS sent from RAN3 for requesting TU TU for discussion sub-grouping that has been agreed in RAN2 to support. Contribution [5] also mentions the work in RAN3. It is not so complicated given that status of paging sub-grouping in RAN2, RAN3 will be involved anyway unless additional decision on paging sub-grouping is made, e.g., no support of paging sub-grouping in Rel-17. Moderator would like to check companies’ view on whether or how to involve RAN3 for supporting paging sub-grouping, e.g., update the WID to allocate proper TU(s) to RAN3 for paging sub-grouping conveyed by PDCCH based PEI, etc.

Table 2. Whether or how to involve RAN3 for supporting paging sub-grouping

| Item | Company | Comments on RAN3 work in WID (If any) |
| --- | --- | --- |
| 1 | MediaTek | Per on-going LS between RAN2, RAN3 and other WGs (R2-2106552 and R2- 2108917), RAN3 work is necessary. We will propose WID update based on RAN3 chair’s recommendation in the email discussion, [93e-03-RAN3-TUs], in this RAN Plenary meeting. |
| 2 | CATT | RAN2 agreements for network-controlled paging subgrouping is decided by CN with signaling from CN to RAN specified by RAN3. We support the update of WID to include RAN3 in the objective of unnecessary paging reception. |
| 3 | Apple | As per the current agreements reached on RAN2#115-e on this topic and the follow-on LS R2-2108917, it is clear that RAN3 work is necessary, specifically for the following requirements - Signalling between AMF and gNB(s) to inform gNB(s) about the related subgroup information for paging a UE in RRC_IDLE/RRC_INACTIVE. To this effect, we propose a WID update to include RAN3 as one of the involved working groups. |
| 4 | Qualcomm | Following the LS from RAN2, it was identified that some work is needed in RAN3. We think it is reasonable to update the WID by adding RAN3. |
| 5 | Lenovo/Motorola Mobility | According to the discussion in RAN2, RAN3 should be involved. We support to update the WID to include RAN3 related objectives. The TU update can be discussed in [93e-03-RAN3-TUs]. |
| 6 | TCL | We support to update WID by adding RAN3. |
| 7 | FGI, Asia Pacific Telecom | Based on the current progress of RAN2, at least some gNB-related signals/procedures of paging sub-grouping are relevant to RAN3. We support to update the WID to involve RAN3. |
| 8 | Samsung | We support WID update according to RAN2 discussion and the LS. |
| 9 | LG | We share similar view with other companies that the updating the WID is required to include RAN3 work. |

2.3	Issue 3 – Whether or what to modify the Status report [1]

In the reflector, CATT commented that “subgroup function also relies on the outcome and design from SA2/CT1 discussion in order to get the related works going in RAN1/RAN2/RAN3.  In addition, there are several remaining issues not captured.   The evaluation assumption on the number of RS used for the coherent detection made in RAN1 is different to that in RAN4’s reply LS to RAN1 (R4-2105799/R1-2104170).  The issue should be captured an open issue in RAN1 in order to complete the design.  There are works in RAN3 on the paging subgroups information exchange between CN and RAN.  They should be captured in RAN3 open issue. The estimated completion level is well optimistic.  The status of the work are behind schedule with 1 quarter left in RAN1 and 2 quarters left in RAN2/3/4.” Moderator would like to check if any more companies have different views on the Status Report [1] of PowSav WID, e.g., different estimation of the progress of the WID, different observations on the remaining issues, etc, and please fill you views in Table 3 on whether or what to modify the SR[1].

Table 3. Whether or what to modify the SR [1]?

| Item | Company | Comments on Status report (If any) |
| --- | --- | --- |
| 1 | MediaTek | - RAN3 issue can be addressed together in issue 2. We can update SR to reflect RAN3 work needed. - This RAN4 issue is dependent on PDCCH-based PEI decision. We can update SR after the decision is made. - Current 70% overall completion level is reasonable, compared with previous completion level of 55% (RAN#92-e). |
| 2 | Spreadtrum | There is no consensus that connected mode assumption applies to idle mode UE. Some UE vendors have different designs for SCell operation in connected mode and PCell PEI reception in idle mode respectively. |
| 3 | CATT | - The open issue also needs to capture the open issue of number of SSBs used for coherent detection of PDCCH-based PEI since RAN4 reply LS in R1-2104170/R4-2105799 implies that at least 2 SSBs are needed for AGC and channel tracking. - The signaling between CN and RAN should be captured as an open issue in RAN3. - The completion level of objective 1 in reduction of unnecessary paging reception in RAN1 is significantly behind since very little discussions on the procedures of paging subgroups and PEI configurations. The additional tasks of paging subgroup assigned by CN gets the extra complexity in the feature with dependency on the CN/SA2 works. Thus, the overall progress in less than 60% with “yellow” mark |
| 4 | Apple | Based on our response to issue #2 above, we feel that the WID has to be updated to include RAN3 as impacted WG. In addition, we think it is reasonable to include the RAN3 open issues list as well. The evaluation assumption is not part of specification work, and it does not need to be listed as part of open issues as far as we understand. The 70% overall completion level is fine with us, especially if we are able to do the PEI down-selection in RAN#93-e. We are also fine with adjusting the completion level slightly lower (e.g. 65%) given that the PEI down-selection is not done yet. |
| 5 | Qualcomm | The SR should be updated to capture adding RAN3 as discussed as part of the previous topic. We don’t think RAN3 TUs need to be added, the time can be allocated from what is reserved for handling unplanned work arising from other WIs. |
| 6 | Lenovo/Motorola Mobility | As commented in Section 2.1, we don’t think the number of SSBs assumed for PDCCH-based PEI is an open issue. We agree to the 70% overall completion level. |
| 7 | Samsung | Physical channel/signal for PEI is not decided yet and there are still many works to do as mentioned in Issue 1. Also, based on the discussion in Issue 2, RAN3 works are necessary. So, we think that 70% of overall progress is overestimated and either 60 or 65% seems reasonable. |
| 8 | LG | Updating SR to add RAN3 issue seems to be required. Regarding the evaluation assumption on the number of RS, we think it would not be required when we decide selecting PDCCH based PEI as the only option. For the estimated completion level, we share similar view with MediaTek. Current estimates seems reasonable. |

# Initial Round Summary

3.1 	Proposal for Intermediate Round discussion on Issue 1

3.2 	Proposal for Intermediate Round discussion on Issue 2

3.3 	Proposal for Intermediate Round discussion on Issue 3

# Intermediate Round Summary

4.1 	Proposal for Final Round discussion on Issue 1

4.2 	Proposal for Final Round discussion on Issue 2

4.3 	Proposal for Final Round discussion on Issue 3

# Final Round Summary

5.1 	Final Proposal on Issue 1

5.2 	Final Proposal on Issue 2

5.3 	Final Proposal on Issue 3

    - Summary

# Reference

[1] RP-212307, “Status report for WI_UE Power Saving Enhancements for NR”, Rapporteur (MediaTek Inc.), RAN#93-e

[2] RP-211791, “On the potential paging enhancement for UE power savings”, Samsung, RAN3#93-e

[3] RP-211813, “Issue on paging early indication in Rel-17 power saving WI”, LG Electronics, RAN#93-e

[4] RP-212003, “Discussion on potential down scoping for paging enhancement in Rel-17 UE Power Saving”, Intel, RAN#93-e

[5] RP-212234, “The scope and progress of UE power saving enhancements”, CATT, RAN#93-e

[6] RP-212308, “PDCCH-based PEI decision”, MediaTek Inc., CMCC, China Telecom, China Unicom, CHTTL, 
LG Uplus, KT Corp., NTT DoCoMo, SK telecom, Softbank, T-Mobile USA, Vodafone, Huawei, HiSilicon, Nokia Corporation, ZTE, Sanechips, Apple, Asus, Asia Pacific Telecom, FGI, Lenovo, Motorola Mobility, LG Electronics, III, InterDigital, ITRI, Nordic Semiconductors, OPPO, Panasonic, Spreadtrum, TCL, Transsion holdings, Xiaomi, RAN#93-e

[7] RP-212309, “On PDCCH-Based PEI for Idle/Inactive-Mode Power Saving”, MediaTek Inc., RAN#93-e

[8] RP-212413, “Discussion on Rel-17 UE power saving enhancement”, ZTE, Sanechips, RAN#93-e