|  | ASIA-PACIFIC TELECOMMUNITY |  |
| --- | --- | --- |
|  | The 28th Meeting of the APT Wireless Group (AWG-28) |  |
|  | 6 – 14 September 2021, Virtual/Online Meeting | 14 September 2021 |

Source: AWG-28/OUT-17

APT Wireless Group

Reply liaison statement to 3GPP RAN, 3GPP RAN4

| Title of the Liaison Statement: | FREQUENCY ARRANGEMENTs FOR IMT in the BAND 470 – 703 MHz |
| --- | --- |
| Objective: | For Information and comments |
| Origin: | APT Wireless Group |
| Contacts: | Mr. Le Van Tuan, Chairman, AWG |

The AWG is pleased to acknowledge liaison statements (R4-2114750) from TSG RAN4 and is pleased to know that the 3GPP has completed the 3GPP Rel-17 Study Item (SI) on the extended 600MHz NR band for APT region

AWG invites 3GPP to start a work item to include giving a band number and appropriate modifications of relevant 3GPP specifications for the options of 600 MHz band plans.

AWG -28 has considered the information and issues from the 3GPP liaison statement regarding the development of the APT 600 MHz band plan. AWG would like to provide the following views and information to 3GPP:

- Band Name

It is the AWG preference to call this band as: “APT 600 MHz band”

- Preference for Options B1 and B2

AWG is still considering Options B1 and B2 at this stage and will continue keeping these options under review with the objective to decide on a single option at our next AWG-29 meeting as more information becomes available.

- Option B2a

As previously indicated option B2a is not a priority for the AWG.

- Time frame of work completion

AWG kindly invites 3GPP to finalize the relevant specifications by September 2022, and requests 3GPP to respond upon the feasibility of this request.

- Other existing users in the 600 MHz band and in adjacent bands

Different APT administrations will have chosen to implement different systems in different portions of the band to suit their national requirements. This may be radio systems under the Fixed, Mobile and Broadcasting Services. Typical existing systems include Digital Terrestrial Television and Radio Microphones / Program Making and Special Events. Some countries allow Television White space devices. The 606 – 614 MHz may also be used by stations of the Radio Astronomy Service (see ITU Radio Regulations footnote No. 5.149), some countries are listed in ITU Radio Regulations footnotes as having an additional allocation in the 606-614 MHz and 608-614 MHz bands (see RR No. 5.305 and No. 5.307). An overview of spectrum usage is given in Figure 1 below. It should be noted that many countries may still use DTT channels throughout the 600 MHz band but it is assumed that APT 600 and DTT will not be able to share in the same area. Radio Microphones / Program Making and Special Events and Television White space devices operate in the spaces / gaps in the spectrum (spectrum that is unused in an area) and are shown as an underlay / secondary user. Some APT counties are listed in a footnote in the ITU Radio Regulations as having an International Mobile Telecommunications (IMT) identification in the 610-698 MHz frequency range (see RR No. 5.296A).

Figure 1 - Likely spectrum use cases compared with the proposed APT 600 MHz plan

5.1 Digital Terrestrial Television (DTT) characteristics

Digital Terrestrial Television (DTT) may be used throughout the 470 to 698 MHz band in many APT countries. The amount of usage will vary from country to country. Some countries may have relatively light usage of DTT or not have many / any stations in the 600 MHz band while other countries could have dense usage throughout both the 500 and 600 MHz bands.

Different technologies are used in different countries which may use different channel sizes in the APT region, this includes 6, 7 and 8 MHz (see Recommendation ITU-R BT.798-1). The common technologies used by APT countries are:

- DVB-T

- ATSC

- ISDB

- DTMB

- DMB

Typical technical characteristics can be found in Report ITU-R BT.2383-3 “Typical frequency sharing characteristics for digital terrestrial television broadcasting systems in the frequency band 470-862 MHz” A summary of the reference broadcasting networks for fixed reception can be found in Annex 1.

5.2 Radio microphone / audio Program Making and Special Event (PMSE) characteristics

Radio microphones / audio Program Making and Special Events utilise frequencies that are not used by other transmissions in that area (e.g. DTT). Some countries allow radio microphones under general authorisation (e.g. licence exemption / unlicenced) while others have a licensing scheme (particularly for professional users). Radio microphones usage is widespread with major events requiring a large number of channels. Information on radio microphones can be found in ECC Report 323 ECC Report 204, EN 300 422-1 V2.1.2, EN 300 422-2 V2.1.1, EN 300 422-3 V2.1.1 and AS/NZS 4268. A summary of the characteristics can be found in Annex 1.

5.3 Television White Space (TVWS) users

Some countries may permit the use of Television White Space (TVWS) devices that operate in the UHF television spectrum as a secondary user, working around other primary users to avoid interference. Many implementations utilise a database while some implementations are manually configurable (may be licenced instead of a database). Technical characteristics for these devices can be found:

- FCC CFR Title 47, Part 15, Subpart H - Television Band Devices 15.701 – 15.717; or

- ETSI EN 301 598 V1.1.1 and EN 301 598 V2.1.1 ‘White Space Devices (WSD); Wireless Access Systems operating in the 470 MHz to 790 MHz TV broadcast band

5.4 Radio astronomy within 608 -614 MHz

Some countries may be operating Stations under the Radio Astronomy Service within the 608 – 614 MHz band at specific locations. These receivers are very sensitive as they are trying to detect weak / distant signals. They may be susceptible to both in band and unwanted emissions. Typically Radio Astronomy Stations are located away from populated areas (often located far away from urban centres) and are protected through the implementation of a co-ordination zone around those specific locations on a case by case basis. The threshold levels detrimental to radio astronomy observations for the relevant frequency bands can be found in Recommendation ITU-R RA.769.

The clearing of the band for mobile/IMT use is a national issue in the Administrations

- 5G NR band combinations used in APT countries

5G NR deployments in APT countries are based on band combinations that are available in those individual countries. It is common for many APT countries to use a combination of bands from different regions (e.g. combinations of bands commonly used in North America and Europe). It is important that APT band combinations are supported to maximise the utility of APT 600 MHz. 5G NR deployments could be either Non-Stand-Alone (NSA) or Stand Alone (SA) modes and could be used in Carrier Aggregation (CA) or Dual Connectivity (DC) configurations between Frequency Range 1 (FR1) and Frequency Range 2 (FR2) configurations.

- Compatibility of options for 600 MHz with the APT 700 MHz band plan

APT 700 MHz (Band 28) has widespread deployment many APT countries with many thousands of base stations and numbers are continuing to grow. Other countries planning are also planning to implement APT 700 MHz (B28 or n28) in the future. It is essential that any 600 MHz APT band plan is compatible with APT 700 MHz (B28 or n28). Any country considering APT 600 MHz is likely to have implemented APT 700 MHz.

AWG would appreciate 3GPP to provide any feedback before the next 29th APT Wireless Group (AWG-29) meeting, scheduled in March/April 2022, if any.

For technical queries please contact:

Dr. Mansoor Shafi

Spark Fellow Wireless

Spark New Zealand Ltd

+64 274515186

Email: Mansoor.shafi@spark.co.nz

AWG will keep 3GPP informed of the progress and outcomes of relevant studies and decisions in future AWG meetings.

Mr. Le Van Tuan

Chairman

APT Wireless Group

Annex 1

Table 1: Summary of the reference broadcasting networks from Report ITU-R BT.2383-3 for fixed reception

| Parameter | DVB-T / DVB-T2 | ATSC | ISDB | DTMB |
| --- | --- | --- | --- | --- |
| Channel bandwidths | 8 MHz & 7 MHz | 6 MHz | 8 MHz & 6 MHz | 8 MHz |
| High power transmitters / main stations |  |  |  |  |
| ERP (W) | 200 000 | 1000000 | 30000 | 200 000 |
| EIRP (dBW) | 55.16 | 62.15 | 46.92 | 55.16 |
| Effective antenna height (m) | 300 |  |  |  |
| Antenna height AGL (m) | 200 |  |  |  |
| Antenna Height Above Average Terrain – HAAT (m) |  | 365 | 100 |  |
| Medium Power transmitters / relay station |  |  |  |  |
| ERP (W) | 5000 | 400000 | 50 | 5000 |
| EIRP (dBW) | 39.14 | 58.17 | 19.14 | 39.14 |
| Effective antenna height (m) | 150 |  |  | 150 |
| Antenna height AGL (m) | 75 |  |  |  |
| Antenna Height Above Average Terrain – HAAT (m) |  | 550 | 20 |  |
| Low Power transmitters / repeater |  |  |  |  |
| ERP (W) | 250 | 50000 | 0.05 | 250 |
| EIRP (dBW) | 26.13 | 49.14 |  | 26.13 |
| Effective antenna height (m) | 75 |  |  | 75 |
| Antenna height AGL (m) | 30 |  |  | 30 |
| Antenna Height Above Average Terrain – HAAT (m) |  | 1800 | 10 |  |
| Receivers |  |  |  |  |
| Antenna Gain (dBi) | 13.3 | 13.3 | 13.3 | 13.3 |
| Antenna height above ground (m) | 10 | 10 | 10 | 10 |
| Feeder loss (dB) | 4 | 4 | 4 | 4 |
| Polarisation discrimination (dB) | 0 | 0 | 0 | 0 |
| Receiver signal bandwidth (MHz) | 7.6 /7.77 | 6 | 5.6 /7.4 | 7.56 |
| Thermal noise (kT0) (dBm/Hz) | -173.98 | -173.20 | -173.98 | -173.98 |
| Receiver noise figure (dB) | 7 / 6 | 7 | 7 | 7 |
| Receiver noise floor (dBm/Hz) | -166.98 / -167.98 | -166.2 | -166.98 | -166.98 |
| Carrier to noise ratio (dB) | 20 | 15 | 21 | 19 |
| Minimum field strength dBuV/m | 55.2 /54.3 | 41 | 55 / 56.2 | 50 |

Further technical found in technical characteristics can be found in Report ITU-R BT.2383-3. Further detail on DVB-T / DVB-T2 transmitters can be found in EN 302 296 V2.2.1 and further detail on DVB-T / DVB-T2 EN 303 340 V1.2.1

Table 2: Summary of Radio Microphone Characteristics

| Parameter | Wireless microphone | In Ear Monitor (IEM) | Audio / RF link |
| --- | --- | --- | --- |
| Transmitter characteristics |  |  |  |
| Modulation | Analogue FM wideband or Digital modulation | Analogue FM wideband or Digital modulation | Analogue FM wideband or Digital modulation |
| RF output power | 10 mW to 100 mW (Typical – periodically some higher power levels may be used up to 500mW) | 10 mW to 100 mW (Typical – periodically some higher power levels may be used up to 500mW) | 10 mW to 100 mW (Typical – periodically some higher power levels may be used up to 500mW) |
| Transmitter bandwidth | 200 kHz (Typical) | 200 kHz (Typical) | 2 x 200 kHz (Typical) |
| Receiver characteristics |  |  |  |
| Receiver bandwidth | 200 kHz (Typical) | 200 kHz (Typical) | 2 x 200 kHz (Typical) |
| Thermal noise | -121 dBm / 200 kHz | -121 dBm / 200 kHz | -121 dBm / 200 kHz |
| Noise figure | 6 dB (Typical) | 6 dB (Typical) | 6 dB (Typical) |
| Receiver noise floor | -115 dBm / 200 kHz | -115 dBm / 200 kHz | -115 dBm / 200 kHz |

Further technical details on radio microphones can be found in ECC Report 323 ECC Report 204, EN 300 422-1 V2.1.2, EN 300 422-2 V2.1.1, EN 300 422-3 V2.1.1, and AS/NZS 4268

AWG is in the process of updating APT Report 79 with a new option, to be added to the current published version which contains option A.

These figures are taken from tables 6 and 7 in Report ITU-R BT.2383-3 with the correction term Corr = 10LOG(FA/FR) being applied for an actual working frequency of  650 MHz.

These figures are taken from tables 8 in Report ITU-R BT.2383-3 with the correction term Corr = 10LOG(FA/FR) being applied for an actual working frequency of  650 MHz.

Polarisation discrimination should not be taken into account in frequency planning for fixed reception due to the possibility of multipath interference

This value is for 64QAM 3/4 which is the typical parameter used in many countries