3GPP TSG-RAN5 Meeting #93-e	R5-216493

# Online, , 8th Nov 2021 - 19th Nov 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 2477 | rev | - | Current version: | 16.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update to 5GC test case 9.1.5.1.14 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT, TDIA |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-10-21 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | "Non-suitable cell" may cause calibration issue on FR2, which can be avoided by replacing with "Non-Suitable “Off” cell". |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1. Replacing "Non-suitable cell" with "Non-Suitable “Off” cell" 2. Editorial updating |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.1.5.1.14 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

9.1.5.1.14	Initial registration / Rejected / Congestion / Abnormal / T3346

<< Skip unchanged sections >>

Table 9.1.5.1.14.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 0 | The SS configures: - NGC Cell A as the "Non-suitable "Off" cell". - NGC Cell B as the "Non-suitable "Off" cell". - NGC Cell E as the "Serving cell". | - |  | - |  |
| 0A | The UE is Switched/Powerd ON | - | - | - | - |
| 1-12 | Steps 2-13 of Table 4.5.2.2-2 of the generic procedure in TS 38.508-1 [4] are performed. | - |  | - | - |
| 13 | SS transmits a REGISTRATION REJECT message with cause #22 (Congestion) and T3346 set to 3 minutes. (Note 1) | <-- | REGISTRATION REJECT | - | - |
| 14 | The SS releases the RRC connection | - |  | - | - |
| 15 | Check: Does the UE transmit a RRCSetupRequest on NGC Cell E within T3346 of Step 13? (Note 1) | --> | NR RRC: RRCSetupRequest | 1 | F |
| 16 | Check: Does the UE transmit a REGISTRATION REQUEST on NGC Cell E | --> | 5GMM: REGISTRATION REQUEST | 2 | P |
| 17-25 | Void |  |  |  |  |
| 26 | SS transmits a REGISTRATION REJECT message with cause #22 (Congestion) and T3346 set to 3 minutes. (Note 1) | <-- | REGISTRATION REJECT | - | - |
| 27 | The SS releases the RRC connection | - |  |  |  |
| 28 | The SS configures: - NGC Cell A as the "Non-suitable "Off" cell". - NGC Cell B as the "Serving cell". - NGC Cell E as the "Non-suitable "Off" cell". |  |  |  |  |
| 29 | Check: Does the UE transmit a REGISTRATION REQUEST on NGC Cell B within T3346 of Step 26? (Note 1) | --> | 5GMM: REGISTRATION REQUEST | 4 | P |
| 30-31 | Void | - | - | - | - |
| 32-38 | Steps 5-11 of Table 4.5.2.2-2 of the generic procedure in TS 38.508-1 [4] are performed. |  |  |  |  |
| 39 | SS transmits a REGISTRATION REJECT message with cause #22 (Congestion) and T3346 set to 3 minutes. (Note 1) | <-- | REGISTRATION REJECT | - | - |
| 40 | The SS releases the RRC connection | - |  | - | - |
| 41 | The SS configures: - NGC Cell A as the "Serving cell". - NGC Cell B as the "Non-suitable "Off" cell". - NGC Cell E as the "Non-suitable "Off" cell". |  |  |  |  |
| 42 | Check: Does the UE transmit a RRCSetupRequest on NGC Cell A within T3346 of Step 39? (Note 1) | --> | NR RRC: RRCSetupRequest | 5 | F |
| 43 | Check: Does the UE transmit a REGISTRATION REQUEST on NGC Cell A | --> | 5GMM: REGISTRATION REQUEST | 5 | P |
| 44-74b1 | Void | - | - | - | - |
| 75-90a1 | Steps 5-20a1 of Table 4.5.2.2-2 of the generic procedure in TS 38.508-1 [4] are performed. | - | - | - | - |
| Note 1: T3346 is set to 3 minutes. This is checked for 3 minutes less tolerance. |  |  |  |  |  |