3GPP TSG-RAN WG5 Meeting #93-e	R5-217810

# Electronic Meeting, 8th Nov 2021 – 19th Nov 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 2517 | rev | 1 | Current version: | 16.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of NR5G RRC TC 8.1.1.3.7b |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm CDMA Technologies |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-11-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Addition of NR5G RRC Test Case 8.1.1.3.7b. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding NR5G RRC Test Case 8.1.1.3.7b |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | RRC WP will not reach 100% completion. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.1.3.7b (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | TTCN Impact |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R5-216815R5-216815r1 - Reference to non existant NRcell 2 is removed from Table 8.1.1.3.7b.3.2-1 - Test procedure is updated to make sure non conformant UEs don’t pass the TC. R5-217810: This is the final version of R5-216815r1 |  |  |  |  |  |  |  |  |

#### 8.1.1.3	RRC release

<<Start of Change>>

##### 8.1.1.3.7b	RRC connection release / Success / Deprioritisation / Deletion of Stored deprioritisation request

8.1.1.3.7b.1	Test Purpose (TP)

(1)

with { UE in NR RRC_CONNECTED state and T325 is running}

ensure that {

when { UE is requested to perform manual PLMN selection }

then { UE deletes the stored deprioritisation request(s) and proceeds with the PLMN selection }

}

8.1.1.3.7b.2	Conformance requirements

References: The conformance requirements covered in the current TC is specified in: TS 38.331 clause 5.3.8.3 and TS 38.304 clause 5.2.4.1.

[TS 38.331, clause 5.3.8.3]

…

The UE shall:

1>	if deprioritisationReq is included and the UE supports RRC connection release with deprioritisation:

2>	start or restart timer T325 with the timer value set to the deprioritisationTimer signalled;

2>	store the deprioritisationReq until T325 expiry;

[TS 38.304, clause 5.2.4.1]

…

In case UE receives RRCRelease with deprioritisationReq, UE shall consider current frequency and stored frequencies due to the previously received RRCRelease with deprioritisationReq or all the frequencies of NR to be the lowest priority frequency (i.e. lower than any of the network configured values) while T325 is running irrespective of camped RAT. The UE shall delete the stored deprioritisation request(s) when a PLMN selection or SNPN selection is performed on request by NAS (TS 23.122 [9]).

NOTE:	UE should search for a higher priority layer for cell reselection as soon as possible after the change of priority. The minimum related performance requirements specified in TS 38.133 [8] are still applicable.

8.1.1.3.7b.3	Test Description

8.1.1.3.7b.3.1	Pre-test conditions

System Simulator:

-	2 NR cells: NR Cells 1 and 3 as specified in TS 38.508-1 [4] table 6.3.2.2-1 are configured as shown in Table 8.1.1.3.7b.3.1–1. PLMN settings are defined in TS 36.523-1 [13] table 6.0.1-1.

-	System information combination NR-4 as defined in TS 38.508-1 [4] clause 4.4.3.1.2 is used in NR cells.

Table 8.1.1.3.7b.3.1-1: PLMN identifiers

| NR Cell | PLMN identifiers |
| --- | --- |
| NR Cell 1 | PLMN1 |
| NR Cell 3 | PLMN2 |

UE:

-	None.

Preamble:

-	The UE is in 5GS state 3N-A on NR Cell 1 according to TS 38.508-1 [4], clause 4.4A.2 Table 4.4A.2-3.

8.1.1.3.7b.3.2	Test procedure sequence

Table 8.1.1.3.7b.3.2-1 illustrates the downlink power levels to be applied for NR Cell 1, Cell 2 and Cell 3 at various time instants of the test execution. Row marked "T0" denotes the conditions after the preamble, while rows marked "T1" and "T2" are to be applied subsequently. The exact instants on which these values shall be applied are described in the texts in this clause.

Table 8.1.1.3.7b.3.2-1: Time instances of cell power level and parameter changes for FR1

|  | Parameter | Unit | NR Cell 1 | NR Cell 3 | Remark |
| --- | --- | --- | --- | --- | --- |
| T0 | SS/PBCH SSS EPRE | dBm/SCS | -88 | Off |  |
| T1 | SS/PBCH SSS EPRE | dBm/SCS | -85 | -91 | The power level values are set so that RNRCell 1 < R NRCell 3. |

Table 8.1.1.3.7b.3.2-2: Time instances of cell power level and parameter changes for FR2

|  | Parameter | Unit | NR Cell 1 | NR Cell 3 | Remark |
| --- | --- | --- | --- | --- | --- |
| T0 | SS/PBCH SSS EPRE | dBm/SCS | FFS | FFS |  |
| T1 | SS/PBCH SSS EPRE | dBm/SCS | FFS | FFS |  |

Table 8.1.1.3.7b.3.2-3: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS adjusts the SS/PBCH EPRE levels according to row "T1" in table 8.1.1.3.7b.2-1/2. | - | - | - | - |
| 2 | The SS transmits an RRCRelease message on NR Cell 1 including IE deprioritisationReq. | <-- | NR RRC: RRCRelease | - | - |
| 3 | SS starts timer T325. | - | - | - | - |
| 4 | PLMN2 is manually selected. |  |  |  |  |
| 5 | Check: Does the test result of generic test procedure in TS 38.508-1 [4] Table 4.9.5.2.2-1 indicate that the UE is camped on NR Cell 3? | --> | NR RRC: RRCSetupRequest | - | - |
| 6 | PLMN1 is manually selected before T325 expiry. | - | - | - | - |
| 7 | Check: Does the test result of generic test procedure in TS 38.508-1 [4] Table 4.9.5.2.2-1 indicate that the UE is camped on NR Cell 1 before T325 expiry? | --> | NR RRC: RRCSetupRequest | 1 | P |

8.1.1.3.7b.3.3	Specific message contents

Table 8.1.1.3.7b.3.3-1: RRCRelease (step 2, Table 8.1.1.3.7b.3.2-3)

| Derivation Path: TS 38.508-1 [4] Table 4.6.1-16 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCRelease ::= SEQUENCE { |  |  |  |
| rrc-TransactionIdentifier | RRC-TransactionIdentifier |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcRelease SEQUENCE { |  |  |  |
| deprioritisationReq SEQUENCE { |  |  |  |
| deprioritisationType | frequency |  |  |
| deprioritisationTimer | min5 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

<<End of Change>>