3GPP TSG-RAN5 Meeting #93-e	R5-218263

# Online, , 8th Nov 2021 - 19th Nov 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.521-1 | CR | 5385 | rev | 1 | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of new LTE_CA_R15 test case - SEM for 4UL CA |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CETECOM GmbH |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_CA_R15-UEConTest |  |  |  |  |  | Date: |  |  | 2021-11-18 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | According to LTE_CA_R15-UEConTest work plan, a new test case "Spectrum Emission Mask for CA (4UL)" is missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | New test case 6.6.2.1A.5 "Spectrum Emission Mask for CA (4UL)" has been introduced. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | LTE_CA_R15-UEConTest work plan will remain incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.6.2.1A.5 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 36.521-2 CR 0972 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | This is an update of R5-217461 and the outcome of 1 revision to this document. |  |  |  |  |  |  |  |  |

< begin of modification >

6.6.2.1A.4.5	Test Requirements

The measured UE mean power with the aggregated channel bandwidth as specified in clause 5.4.2A, derived in step 6, shall fulfil requirements in Table 6.2.3A.1.5 as appropriate, and the power of any UE emission shall fulfil requirements in Table.6.6.2.1A.4.5.

Table 6.6.2.1A.4.5-1: :General E-UTRA spectrum emission mask for CA Bandwidth Class C, E-UTRA bands ≤ 3GHz– intra-band contiguous CA

|  | Spectrum emission limit [dBm]/BWChannel_CA |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| ΔfOOB (MHz) | 25RB+100RB (24.95 MHz) | 50RB+100RB (29.9 MHz) | 75RB+75RB (30 MHz) | 75RB+100RB (34.85 MHz) | 100RB+100RB (39.8 MHz) | Measurement bandwidth |
| 0-1 | -20.5 | -21 | -21 | -22 | -22.5 | 30 kHz |
| 1-5 | -8.5 | -8.5 | -8.5 | -8.5 | -8.5 | 1 MHz |
| 5-24.95 | -11.5 | -11.5 | -11.5 | -11.5 | -11.5 | 1 MHz |
| 24.95-29.9 | -23.5 |  |  |  |  | 1 MHz |
| 29.9-29.95 |  | -23.5 |  |  |  | 1 MHz |
| 29.95-30 |  |  |  |  |  | 1 MHz |
| 30-34.85 |  |  | -23.5 |  |  | 1 MHz |
| 34.85-34.9 |  |  |  | -23.5 |  | 1 MHz |
| 34.9-35 |  |  |  |  |  | 1 MHz |
| 35-39.8 |  |  |  |  |  | 1 MHz |
| 39.8-39.85 |  |  |  |  | -23.5 | 1 MHz |
| 39.85-44.8 |  |  |  |  |  | 1 MHz |
| Note 1: The first and last measurement position with a 30 kHz filter is at ΔfOOB equals to 0.015 MHz and 0.985 MHz. Note 2: At the boundary of spectrum emission limit, the first and last measurement position with a 1 MHz filter is the inside of +0.5MHz and -0.5MHz, respectively. Note 3: The measurements are to be performed above the upper edge of the aggregated channel bandwidth and below the lower edge of the aggregated channel bandwidth. |  |  |  |  |  |  |

Table 6.6.2.1A.4.5-2: General E-UTRA spectrum emission mask for CA Bandwidth Class C, 3GHz < E-UTRA bands ≤ 4.2GHz – intra-band contiguous CA

|  |  | Spectrum emission limit [dBm]/BWChannel_CA |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ΔfOOB (MHz) |  | 25RB+100RB (24.95 MHz) |  | 50RB+100RB (29.9 MHz) | 75RB+75RB (30 MHz) | 75RB+100RB (34.85 MHz) | 100RB+100RB (39.8 MHz) | Measurement bandwidth |
| 0-1 |  | -20.2 |  | -20.7 | -20.7 | -21.7 | -22.2 | 30 kHz |
| 1-5 |  | -8.2 |  | -8.2 | -8.2 | -8.2 | -8.2 | 1 MHz |
| 5-24.95 |  | -11.2 |  | -11.2 | -11.2 | -11.2 | -11.2 | 1 MHz |
| 24.95-29.9 |  | -23.2 |  |  |  |  |  | 1 MHz |
| 29.9-29.95 |  |  |  | -23.2 |  |  |  | 1 MHz |
| 29.95-30 |  |  |  |  |  |  |  | 1 MHz |
| 30-34.85 |  |  |  |  | -23.2 |  |  | 1 MHz |
| 34.85-34.9 |  |  |  |  |  | -23.2 |  | 1 MHz |
| 34.9-35 |  |  |  |  |  |  |  | 1 MHz |
| 35-39.8 |  |  |  |  |  |  |  | 1 MHz |
| 39.8-39.85 |  |  |  |  |  |  | -23.2 | 1 MHz |
| 39.85-44.8 |  |  |  |  |  |  |  | 1 MHz |
| Note 1: The first and last measurement position with a 30 kHz filter is at ΔfOOB equals to 0.015 MHz and 0.985 MHz. Note 2: At the boundary of spectrum emission limit, the first and last measurement position with a 1 MHz filter is the inside of +0.5MHz and -0.5MHz, respectively. Note 3: The measurements are to be performed above the upper edge of the aggregated channel bandwidth and below the lower edge of the aggregated channel bandwidth. |  |  |  |  |  |  |  |  |

Table 6.6.2.1A.4.5-3: General E-UTRA spectrum emission mask for CA Bandwidth Class B, E-UTRA bands ≤ 3GHz – intra-band contiguous CA

| Spectrum emission limit [dBm]/BWChannel_CA |  |  |  |
| --- | --- | --- | --- |
| ΔfOOB (MHz) | 25RB+50RB (14.95 MHz) | 50RB+50RB (19.9 MHz) | Measurement bandwidth |
| 0-1 | -20 | -21 | 30 kHz |
| 1-5 | -10 | -10 | 1 MHz |
| 5-14.95 | -13 | -13 | 1 MHz |
| 14.95-19.90 | -25 | -13 | 1 MHz |
| 19.90-19.95 | -25 | -25 | 1 MHz |
| 19.95-24.90 |  | -25 | 1 MHz |

Table 6.6.2.1A.4.5-4: General E-UTRA spectrum emission mask for CA Bandwidth Class B, 3GHz < E-UTRA bands ≤ 4.2GHz – intra-band contiguous CA

| Spectrum emission limit [dBm]/BWChannel_CA |  |  |  |
| --- | --- | --- | --- |
| ΔfOOB (MHz) | 25RB+50RB (14.95 MHz) | 50RB+50RB (19.9 MHz) | Measurement bandwidth |
| 0-1 | -20 | -21 | 30 kHz |
| 1-5 | -10 | -10 | 1 MHz |
| 5-14.95 | -13 | -13 | 1 MHz |
| 14.95-19.90 | -25 | -13 | 1 MHz |
| 19.90-19.95 | -25 | -25 | 1 MHz |
| 19.95-24.90 |  | -25 | 1 MHz |

Table 6.6.2.1A.4.5-5: General E-UTRA spectrum emission mask, E-UTRA bands ≤ 3GHz – inter-band CA

|  | Spectrum emission limit (dBm)/ Channel bandwidth |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| ΔfOOB (MHz) | 1.4 MHz | 3.0 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Measurement bandwidth |
| 0-1 | -8.5 | -11.5 | -13.5 | -16.5 | -18.5 | -19.5 | 30 kHz |
| 1-2.5 | -8.5 | -8.5 | -8.5 | -8.5 | -8.5 | -8.5 | 1 MHz |
| 2.5-2.8 | -23.5 |  |  |  |  |  | 1 MHz |
| 2.8-5 |  |  |  |  |  |  | 1 MHz |
| 5-6 |  | -23.5 | -11.5 | -11.5 | -11.5 | -11.5 | 1 MHz |
| 6-10 |  |  | -23.5 |  |  |  | 1 MHz |
| 10-15 |  |  |  | -23.5 |  |  | 1 MHz |
| 15-20 |  |  |  |  | -23.5 |  | 1 MHz |
| 20-25 |  |  |  |  |  | -23.5 | 1 MHz |
| Note 1: The first and last measurement position with a 30 kHz filter is at ΔfOOB equals to 0.015 MHz and 0.985 MHz. Note 2: At the boundary of spectrum emission limit, the first and last measurement position with a 1 MHz filter is the inside of +0.5MHz and -0.5MHz, respectively. Note 3: The measurements are to be performed above the upper edge of the channel and below the lower edge of the channel Note 4: For the 2.5-2.8 MHz offset range with 1.4 MHz channel bandwidth, the measurement position is at ΔfOOB equals to 3 MHz. |  |  |  |  |  |  |  |

Table 6.6.2.1A.4.5-6: General E-UTRA spectrum emission mask, 3GHz < E-UTRA bands ≤ 4.2GHz – inter-band CA

|  | Spectrum emission limit (dBm)/ Channel bandwidth |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| ΔfOOB (MHz) | 1.4 MHz | 3.0 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Measurement bandwidth |
| 0-1 | -8.2 | -11.2 | -13.2 | -16.2 | -18.2 | -19.2 | 30 kHz |
| 1-2.5 | -8.2 | -8.2 | -8.2 | -8.2 | -8.2 | -8.2 | 1 MHz |
| 2.5-2.8 | -23.2 |  |  |  |  |  | 1 MHz |
| 2.8-5 |  |  |  |  |  |  | 1 MHz |
| 5-6 |  | -23.2 | -11.2 | -11.2 | -11.2 | -11.2 | 1 MHz |
| 6-10 |  |  | -23.2 |  |  |  | 1 MHz |
| 10-15 |  |  |  | -23.2 |  |  | 1 MHz |
| 15-20 |  |  |  |  | -23.2 |  | 1 MHz |
| 20-25 |  |  |  |  |  | -23.2 | 1 MHz |
| Note 1: The first and last measurement position with a 30 kHz filter is at ΔfOOB equals to 0.015 MHz and 0.985 MHz. Note 2: At the boundary of spectrum emission limit, the first and last measurement position with a 1 MHz filter is the inside of +0.5MHz and -0.5MHz, respectively. Note 3: The measurements are to be performed above the upper edge of the channel and below the lower edge of the channel Note 4: For the 2.5-2.8 MHz offset range with 1.4 MHz channel bandwidth, the measurement position is at ΔfOOB equals to 3 MHz. |  |  |  |  |  |  |  |

NOTE:	As a general rule, the resolution bandwidth of the measuring equipment should be equal to the measurement bandwidth. However, to improve measurement accuracy, sensitivity and efficiency, the resolution bandwidth may be smaller than the measurement bandwidth. When the resolution bandwidth is smaller than the measurement bandwidth, the result should be integrated over the measurement bandwidth in order to obtain the equivalent noise bandwidth of the measurement bandwidth.

##### 6.6.2.1A.5	Spectrum Emission Mask for CA (4UL)

6.6.2.1A.5.1	Test purpose

To verify that the power of any UE emission shall not exceed specified level for the specified channel bandwidth for CA.

6.6.2.1A.5.2	Test applicability

This test case applies to all types of E-UTRA UE release 11 and forward that support inter-band and 4UL CA.

6.6.2.1A.5.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 6.6.2.1A.0.

6.6.2.1A.5.4	Test description

6.6.2.1A.5.4.1	Initial condition

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA CA configurations specified in table 5.4.2A.1-2. All of these configurations shall be tested with applicable test parameters for each channel bandwidth and are shown in table 6.6.2.1A.5.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annex A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.6.2.1A.5.4.1-1: inter-band CA Test Configuration Table

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [7] subclause 4.1 |  |  |  |  |  |  | NC |  |  |  |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 for different CA bandwidth classes, and PCC and SCCs are mapped onto physical frequencies according to Table 6.1-2. |  |  |  |  |  |  | Low and High range |  |  |  |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration across bandwidth combination sets supported by the UE (Note 1). |  |  |  |  |  |  | Lowest NRB_agg Highest NRB_agg (Note 2) |  |  |  |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |  |  |  |  |
| CA Configuration / NRB_agg DL Allocation |  |  |  |  | DL Allocation | CC MOD | UL Allocation |  |  |  |  |
| ID | PCC NRB | SCC1 NRB | SCC2 NRB | SCC3 NRB | PCC & SCC RB allocation |  | NRB_ alloc | PCC & SCC1 & SCC2 & SCC3 RB allocations (LCRB @ RBstart) |  |  |  |
| 1 | 100 | 50 | 100 | 50 |  | QPSK | 24 | P_12@0 | S_0@0 | S_12@0 | S_0@0 |
| 2 | 100 | 50 | 100 | 50 |  | QPSK | 100 | P_50@0 | S_0@0 | S_50@0 | S_0@0 |
| 3 | 100 | 50 | 100 | 50 |  | QPSK | 300 | P_100@0 | S_50@0 | S_100@0 | S_50@0 |
| 4 | 100 | 50 | 100 | 50 |  | 16 QAM | 24 | P_12@0 | S_0@0 | S_12@0 | S_0@0 |
| 5 | 100 | 50 | 100 | 50 |  | 16 QAM | 100 | P_50@0 | S_0@0 | S_50@0 | S_0@0 |
| 6 | 100 | 50 | 100 | 50 |  | 16 QAM | 300 | P_100@0 | S_50@0 | S_100@0 | S_50@0 |
| 7 | 100 | 50 | 100 | 50 |  | 64 QAM | 24 | P_12@0 | S_0@0 | S_12@0 | S_0@0 |
| 8 | 100 | 50 | 100 | 50 |  | 64 QAM | 300 | P_100@0 | S_50@0 | S_100@0 | S_50@0 |
| 9 | 100 | 50 | 100 | 50 | N/A for this | 256 QAM | 300 | P_100@0 | S_50@0 | S_100@0 | S_50@0 |
| 10 | 100 | 100 | 100 | 100 | test | QPSK | 24 | P_12@0 | S_0@0 | S_12@0 | S_0@0 |
| 11 | 100 | 100 | 100 | 100 |  | QPSK | 100 | P_50@0 | S_0@0 | S_50@0 | S_0@0 |
| 12 | 100 | 100 | 100 | 100 |  | QPSK | 400 | P_100@0 | S_100@0 | S_100@0 | S_100@0 |
| 13 | 100 | 100 | 100 | 100 |  | 16 QAM | 24 | P_12@0 | S_0@0 | S_12@0 | S_0@0 |
| 14 | 100 | 100 | 100 | 100 |  | 16 QAM | 100 | P_50@0 | S_0@0 | S_50@0 | S_0@0 |
| 15 | 100 | 100 | 100 | 100 |  | 16 QAM | 400 | P_100@0 | S_100@0 | S_100@0 | S_100@0 |
| 16 | 100 | 100 | 100 | 100 |  | 64 QAM | 24 | P_12@0 | S_0@0 | S_12@0 | S_0@0 |
| 17 | 100 | 100 | 100 | 100 |  | 64 QAM | 300 | P_100@0 | S_50@0 | S_100@0 | S_50@0 |
| 18 | 100 | 100 | 100 | 100 |  | 256 QAM | 400 | P_100@0 | S_100@0 | S_100@0 | S_100@0 |
| NOTE 1: CA Configuration Test CC Combination settings are checked separately for each CA Configuration, which applicable aggregated channel bandwidths are specified in Table 5.4.2A.1-2. NOTE 2: If the UE supports multiple CC Combinations in the CA Configuration with the same NRB_agg , only the combination with the highest NRB_PCC is tested. |  |  |  |  |  |  |  |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in Figure TS 36.508 [7] Annex A, Figure group A.55 as appropriate.

2.	The parameter settings for the cell are set up according to TS 36.508 [7] clause 4.4.3.

3.	Downlink signals for PCC are initially set up according to Annex C0, C.1 and C.3.0, and uplink signals according to Annex H.1 and H.3.0.

4.	The UL Reference Measurement channels are set according to Table 6.6.2.1A.5.4.1-1.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in State 3A-RF according to TS 36.508 [7] clause 5.2A.2. Message contents are defined in clause 6.6.2.1A.5.4.3.

6.6.2.1A.5.4.2	Test procedure

1.	Configure SCCs according to Annex C.0, C.1 and Annex C.3.0 for all downlink physical channels except PHICH.

2.	The SS shall configure SCCs as per TS 36.508 [7] clause 5.2A.4. Message contents are defined in clause 6.6.2.1A.5.4.3.

3.	SS activates SCCs by sending the activation MAC-CE (Refer TS 36.321 [13], clauses 5.13, 6.1.3.8). Wait for at least 2 seconds (Refer TS 36.133, clauses 8.3.3.2).

4.	SS sends uplink scheduling information via PDCCH DCI format 0 for C_RNTI to schedule the UL RMC according to Table 6.6.2.1.5.1-1 on PCC and SCCs. Since the UE has no payload data to send, the UE transmits uplink MAC padding bits on the UL RMC.

5.	Send continuously uplink power control "up" commands in the uplink scheduling information to the UE until the UE transmits at PUMAX level.

6.	Measure the mean power over all component carriers in the CA configuration of the radio access mode, which shall meet the requirements described in Table 6.2.3A.5.5-1. The period of the measurement shall be at least the continuous duration of one sub-frame (1ms). For TDD slots with transient periods are not under test.

7.	Measure the power of the transmitted signal with a measurement filter of bandwidths according to table 6.6.2.1A.5.5-1 or 6.6.2.1A.5.5-2, as applicable. The centre frequency of the filter shall be stepped in continuous steps according to the same table. The measured power shall be recorded for each step. The measurement period shall capture the active TSs.

6.6.2.1A.5.4.3	Message contents

Message contents are according to TS 36.508 [7] clause 4.6. In test procedure step 2, for SCC configuration there are no additional message contents.

6.6.2.1A.5.5	Test Requirements

The measured UE mean power with the aggregated channel bandwidth as specified in clause 5.4.2A, derived in step 6, shall fulfil requirements in Table 6.2.3A.5.5-1 as appropriate, and the power of any UE emission shall fulfil requirements in Table.6.6.2.1A.1.5-1 or Table.6.6.2.1A.1.5-2 for Bandwidth Class C, as applicable.

Table 6.6.2.1A.5.5-1: General E-UTRA spectrum emission mask for CA Bandwidth Class C, E-UTRA bands ≤ 3GHz

|  | Spectrum emission limit [dBm]/BWChannel_CA |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| ΔfOOB (MHz) | 25RB+100RB (24.95MHz) | 50RB+75RB (24.75 MHz) | 50RB+100RB (29.9 MHz) | 75RB+75RB (30 MHz) | 75RB+100RB (34.85 MHz) | 100RB+100RB (39.8 MHz) | Measurement bandwidth |
| 0-1 | -20.5 | -20.5 | -21 | -21 | -22 | -22.5 | 30 kHz |
| 1-5 | -8.5 | -8.5 | -8.5 | -8.5 | -8.5 | -8.5 | 1 MHz |
| 5-24.75 | -11.5 | -11.5 | -11.5 | -11.5 | -11.5 | -11.5 | 1 MHz |
| 24.75-24.95 |  | -23.5 |  |  |  |  | 1 MHz |
| 24.95-29.75 | -23.5 |  |  |  |  |  | 1 MHz |
| 29.75-29.9 |  |  |  |  |  |  | 1 MHz |
| 29.9-29.95 |  |  | -23.5 |  |  |  | 1 MHz |
| 29.95-30 |  |  |  |  |  |  | 1 MHz |
| 30-34.85 |  |  |  | -23.5 |  |  | 1 MHz |
| 34.85-34.9 |  |  |  |  | -23.5 |  | 1 MHz |
| 34.9-35 |  |  |  |  |  |  | 1 MHz |
| 35-39.8 |  |  |  |  |  |  | 1 MHz |
| 39.8-39.85 |  |  |  |  |  | -23.5 | 1 MHz |
| 39.85-44.8 |  |  |  |  |  |  | 1 MHz |
| Note 1: The first and last measurement position with a 30 kHz filter is at ΔfOOB equals to 0.015 MHz and 0.985 MHz. Note 2: At the boundary of spectrum emission limit, the first and last measurement position with a 1 MHz filter is the inside of +0.5MHz and -0.5MHz, respectively. Note 3: The measurements are to be performed above the upper edge of the aggregated channel bandwidth and below the lower edge of the aggregated channel bandwidth. |  |  |  |  |  |  |  |

Table 6.6.2.1A.5.5-2: General E-UTRA spectrum emission mask for CA Bandwidth Class C, 3GHz < E-UTRA bands ≤ 4.2GHz

|  | Spectrum emission limit [dBm]/BWChannel_CA |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| ΔfOOB (MHz) | 25RB+100RB (24.95MHz) | 50RB+75RB (24.75 MHz) | 50RB+100RB (29.9 MHz) | 75RB+75RB (30 MHz) | 75RB+100RB (34.85 MHz) | 100RB+100RB (39.8 MHz) | Measurement bandwidth |
| 0-1 | -20.2 | -20.2 | -20.7 | -20.7 | -21.7 | -22.2 | 30 kHz |
| 1-5 | -8.2 | -8.2 | -8.2 | -8.2 | -8.2 | -8.2 | 1 MHz |
| 5-24.75 | -11.2 | -11.2 | -11.2 | -11.2 | -11.2 | -11.2 | 1 MHz |
| 24.75-24.95 |  | -23.2 |  |  |  |  | 1 MHz |
| 24.95-29.75 | -23.2 |  |  |  |  |  | 1 MHz |
| 29.75-29.9 |  |  |  |  |  |  | 1 MHz |
| 29.9-29.95 |  |  | -23.2 |  |  |  | 1 MHz |
| 29.95-30 |  |  |  |  |  |  | 1 MHz |
| 30-34.85 |  |  |  | -23.2 |  |  | 1 MHz |
| 34.85-34.9 |  |  |  |  | -23.2 |  | 1 MHz |
| 34.9-35 |  |  |  |  |  |  | 1 MHz |
| 35-39.8 |  |  |  |  |  |  | 1 MHz |
| 39.8-39.85 |  |  |  |  |  | -23.2 | 1 MHz |
| 39.85-44.8 |  |  |  |  |  |  | 1 MHz |
| Note 1: The first and last measurement position with a 30 kHz filter is at ΔfOOB equals to 0.015 MHz and 0.985 MHz. Note 2: At the boundary of spectrum emission limit, the first and last measurement position with a 1 MHz filter is the inside of +0.5MHz and -0.5MHz, respectively. Note 3: The measurements are to be performed above the upper edge of the aggregated channel bandwidth and below the lower edge of the aggregated channel bandwidth. |  |  |  |  |  |  |  |

NOTE:	As a general rule, the resolution bandwidth of the measuring equipment should be equal to the measurement bandwidth. However, to improve measurement accuracy, sensitivity and efficiency, the resolution bandwidth may be smaller than the measurement bandwidth. When the resolution bandwidth is smaller than the measurement bandwidth, the result should be integrated over the measurement bandwidth in order to obtain the equivalent noise bandwidth of the measurement bandwidth.

#### 6.6.2.1B	Spectrum Emission Mask for UL-MIMO

< end of modification >