3GPP TSG-RAN5 Meeting #93-e	R5-217905

# Online, , 8th Nov 2021 - 19th Nov 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.579-1 | CR | 0186 | rev | 1 | Current version: | 15.3.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | 5.5.7.3 MCDATA Group Configuration Updates |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NIST |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | MCenhUEConTest |  |  |  |  |  | Date: |  |  | 2021-11-18 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The MCData Group Configuration document did not include aspects for Enahnced Status |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1) Removed "MCPTT" from the title of clause 5.5.7 because it also includes MCVideo and MCData 2) Added parameters for Enhanced Status in the MCData Group Configuration document 3) Changed the title of the clause. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | MCData test cases involving the Enahnced Status feature will not be properly tested |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.5.7, 5.5.7.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | This is the final version of R5-216588r1. |  |  |  |  |  |  |  |  |

<START OF CHANGES>

### 5.5.7	Default MCX group management messages and other information elements

…

#### 5.5.7.3	MCDATA Group Configuration

The structure of a group configuration document is specified in TS 24.481 [11] clause 7.

Single MCDATA group configuration parameters are defined in TS 24.483 [13] clause 6.3.

Table 5.5.7.3-1: MCDATA Group Configuration Defaults

| Derivation Path: TS 24.481 [11] clause 7.2.2 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| list-service[1] |  | Group 1 |  |  |
| uri attribute | px_MCDATA_Group_A_ID | Value is a "uri" attribute specified in OMA OMA-TS-XDM_Group-V1_1 | TS 24.483 [13] clause 6.2.7 |  |
| display-name | px_MCData _Group_A_Name | Value is a <display-name> element specified in OMA OMA-TS-XDM_Group-V1_1 | TS 24.483 [13] clause 6.2.8 |  |
| list |  |  |  |  |
| entry[1] |  | group member 1 |  |  |
| uri attribute | px_MCData_ID_User_A | Indicates an MCData user identity (MCData ID) which is a globally unique identifier within the MCData service that represents the MCData user | TS 24.483 [13] clause 6.2.11 |  |
| display-name | Not present |  |  |  |
| mcpttgi:user-priority | "3" | Indicates the user priority of the MCData group member | TS 24.483 [13] clause 6.2.12 |  |
| mcpttgi:participant-type | px_MCData _User_A_ParticipantType | Participant type of the MCData group | TS 24.483 [13] clause 6.2.13 |  |
| rl:mcdata-mcdata-id |  |  |  |  |
| uri attribute | px_MCData_ID_User_A |  |  |  |
| entry[2] |  | Group member 2 |  |  |
| uri attribute | px_MCData_ID_User_B | Indicates an MCData user identity (MCData ID) which is a globally unique identifier within the MCData service that represents the MCData user | TS 24.483 [13] clause 6.2.11 |  |
| display-name | Not present |  |  |  |
| mcpttgi:user-priority | "2" | Indicates the user priority of the MCData group member | TS 24.483 [13] clause 6.2.12 |  |
| mcpttgi:participant-type | px_MCData _User_B_ParticipantType | Participant type of the MCData group | TS 24.483 [13] clause 6.2.13 |  |
| rl:mcdata-mcdata-id |  |  |  |  |
| uri attribute | px_MCData_ID_User_B |  | TS 24.483 [13] clause 6.2.11 |  |
| entry[3] |  | Group member 3 |  |  |
| uri attribute | px_MCData_ID_User_C | Indicates an MCData user identity (MCData ID) which is a globally unique identifier within the MCData service that represents the MCData user | TS 24.483 [13] clause 6.2.11 |  |
| display-name | Not present |  |  |  |
| mcpttgi:user-priority | "1" | Indicates the user priority of the MCData group member | TS 24.483 [13] clause 6.2.12 |  |
| mcpttgi:participant-type | px_MCData _User_C_ParticipantType | Participant type of the MCData group | TS 24.483 [13] clause 6.2.13 |  |
| rl:mcdata-mcdata-id |  |  |  |  |
| uri attribute | px_MCData_ID_User_C |  | TS 24.483 [13] clause 6.2.11 |  |
| cp:ruleset |  |  |  |  |
| cp:rule |  |  |  |  |
| cp:id attribute | "rule1" |  |  |  |
| cp:actions |  |  |  |  |
| mcpttgi:on-network-allow-getting-member-list | "true" | Indicates that the identity is allowed to get the MCS group member list of the MCS group in on-network procedures. |  |  |
| mcpttgi:mcdata-on-network-allow-getting-affiliation-list | "true" | Indicates that the identity is allowed to get the list of MCData users affiliated to the MCData group in on-network MCData procedures |  |  |
| mcpttgi:mcdata-allow-transmit-data-in-this-group | "true" | Indicates that the identity is allowed to transmit data in this group |  |  |
| oxe:supported-services |  |  |  |  |
| oxe:service |  |  |  |  |
| oxe:enabler | "urn:urn-7:3gpp-service.ims.icsi.mcdata.sds" | String defining an enabler |  |  |
| mcpttgi:off-network-ProSe-layer-2-group-id | tsc_MCPTT_Group_A_ProSeLayer2GroupID | Indicates the Prose layer-2 group ID | TS 23.303 [68] TS 24.483 [13] clause 6.2.27 |  |
| mcpttgi:off-network-IP-multicast-address | "0.0.0.0" | Indicates the ProSe group IP multicast address;the IP version is implicitly given by the notation of the IP address | TS 23.303 [68] TS 24.483 [13] clause 6.2.28 |  |
| mcpttgi:off-network-ProSe-relay-service-code | "123456" | Indicates the connectivity service that the ProSe UE-to-network relay provides to public safety applications | TS 23.303 [68] TS 24.483 [13] clause 6.2.29 |  |
| mcpttgi:owner | px_MCData_Group_A_Owner_Organization | Group’s owner (Mission Critical Organisation). | TS 24.483 [13] clause 6.2.15 |  |
| mcpttgi:level-within-group-hierarchy | "0" | Indicates the level within a group hierarchy (only applicable for group-broadcast group). | TS 24.483 [13] clause 6.2.17 |  |
| mcpttgi:mcdata-enhanced-status-operational-values |  | A list of operational values used for the enhanced status service and two text strings used to display a meaningful message to the user. |  |  |
| mcpttgi:status |  |  |  |  |
| id | "0" |  |  |  |
| mcpttgi:shortText |  |  |  |  |
| langType | "English" |  |  |  |
| langText | "going" |  |  |  |
| mcpttgi:description |  |  |  |  |
| langType | "English" |  |  |  |
| langText | "going to the operation site" |  |  |  |
| mcpttgi:status |  |  |  |  |
| id | "1" |  |  |  |
| mcpttgi:shortText |  |  |  |  |
| langType | "English" |  |  |  |
| langText | "arrived" |  |  |  |
| mcpttgi:description |  |  |  |  |
| langType | "English" |  |  |  |
| langText | "just arrived at the operation site" |  |  |  |
| mcpttgi:level-within-user-hierarchy | "0" | Indicates the level within user hierarchy (only applicable for user-broadcast group). | TS 24.483 [13] clause 6.2.18 |  |
| mcpttgi:mcpttgi:mcdata-on-network-group-priority | "1" | Indicates the priority level of the group in on-network MCData procedures. Higher value indicates higher priority |  |  |
| mcpttgi:mcdata-on-network-max-data-size-for-SDS | "10000" | Indicates the maximum size of data (in bytes) that the originating MCData client is allowed to send to the MCData server for on-network SDS communications |  |  |
| mcpttgi:mcdata-on-network-max-data-size-for-FD | "10000" | Indicates the maximum size of data (in bytes) that the originating MCData client is allowed to send to the MCData server for on-network FD communications |  |  |
| mcpttgi:mcdata-on-network-max-data-size-auto-recv | "2000" | Indicates the maximum size of data (in bytes) which the MCData server always requests the terminating MCData client to automatically download for on-network FD communications using HTTP |  |  |
| mcpttgi:mcdata-off-network-ProSe-signalling-PPPP | "1" | Indicates the ProSe Per-Packet Priority value to be used when transmitting IP packets carrying signalling for a call on the MCData group in off-network MCData procedures |  |  |
| mcpttgi:mcdata-off-network-ProSe-media-PPPP | "1" | Indicates the ProSe Per-Packet Priority value to be used when transmitting IP packets carrying media for a call on the MCData group in off-network MCData procedures |  |  |
| list-service[2] |  | Group 2 |  |  |
| uri attribute | px_MCDATA_Group_D_ID | Value is a "uri" attribute specified in OMA OMA-TS-XDM_Group-V1_1 | TS 24.483 [13] clause 6.2.7 |  |
| display-name | px_MCData _Group_D_Name | Value is a <display-name> element specified in OMA OMA-TS-XDM_Group-V1_1 | TS 24.483 [13] clause 6.2.8 |  |
| list |  |  |  |  |
| entry[1] |  | group member 1 |  |  |
| uri attribute | px_MCData_ID_User_A | Indicates an MCData user identity (MCData ID) which is a globally unique identifier within the MCData service that represents the MCData user | TS 24.483 [13] clause 6.2.11 |  |
| display-name | Not present |  |  |  |
| mcpttgi:user-priority | "3" | Indicates the user priority of the MCData group member | TS 24.483 [13] clause 6.2.12 |  |
| mcpttgi:participant-type | px_MCData _User_A_ParticipantType | Participant type of the MCData group | TS 24.483 [13] clause 6.2.13 |  |
| rl:mcdata-mcdata-id |  |  |  |  |
| uri attribute | px_MCData_ID_User_A |  |  |  |
| entry[2] |  | Group member 2 |  |  |
| uri attribute | px_MCData_ID_User_B | Indicates an MCData user identity (MCData ID) which is a globally unique identifier within the MCData service that represents the MCData user | TS 24.483 [13] clause 6.2.11 |  |
| display-name | Not present |  |  |  |
| mcpttgi:user-priority | "2" | Indicates the user priority of the MCData group member | TS 24.483 [13] clause 6.2.12 |  |
| mcpttgi:participant-type | px_MCData _User_B_ParticipantType | Participant type of the MCData group | TS 24.483 [13] clause 6.2.13 |  |
| rl:mcdata-mcdata-id |  |  |  |  |
| uri attribute | px_MCData_ID_User_B |  | TS 24.483 [13] clause 6.2.11 |  |
| cp:ruleset |  |  |  |  |
| cp:rule |  |  |  |  |
| cp:id attribute | "rule2" |  |  |  |
| cp:actions |  |  |  |  |
| mcpttgi:on-network-allow-getting-member-list | "true" | Indicates that the identity is allowed to get the MCS group member list of the MCS group in on-network procedures. |  |  |
| mcpttgi:mcdata-on-network-allow-getting-affiliation-list | "false" | Indicates that the identity is allowed to get the list of MCData users affiliated to the MCData group in on-network MCData procedures |  |  |
| mcpttgi:mcdata-allow-transmit-data-in-this-group | "true" | Indicates that the identity is allowed to transmit data in this group |  |  |
| oxe:supported-services |  |  |  |  |
| oxe:service |  |  |  |  |
| oxe:enabler | "urn:urn-7:3gpp-service.ims.icsi.mcdata.sds" | String defining an enabler |  |  |
| mcpttgi:off-network-ProSe-layer-2-group-id | tsc_MCPTT_Group_D_ProSeLayer2GroupID | Indicates the Prose layer-2 group ID | TS 23.303 [68] TS 24.483 [13] clause 6.2.27 |  |
| mcpttgi:off-network-IP-multicast-address | "0.0.0.0" | Indicates the ProSe group IP multicast address;the IP version is implicitly given by the notation of the IP address | TS 23.303 [68] TS 24.483 [13] clause 6.2.28 |  |
| mcpttgi:off-network-ProSe-relay-service-code | "123456" | Indicates the connectivity service that the ProSe UE-to-network relay provides to public safety applications | TS 23.303 [68] TS 24.483 [13] clause 6.2.29 |  |
| mcpttgi:owner | px_MCVideo_Group_D_Owner_Organization | Group’s owner (Mission Critical Organisation). | TS 24.483 [13] clause 6.2.15 |  |
| mcpttgi:level-within-group-hierarchy | "0" | Indicates the level within a group hierarchy (only applicable for group-broadcast group). | TS 24.483 [13] clause 6.2.17 |  |
| mcpttgi:level-within-user-hierarchy | "0" | Indicates the level within user hierarchy (only applicable for user-broadcast group). | TS 24.483 [13] clause 6.2.18 |  |
| mcpttgi:mcdata-on-network-group-priority | "1" | Indicates the priority level of the group in on-network MCData procedures. Higher value indicates higher priority |  |  |
| mcpttgi:mcdata-on-network-max-data-size-for-SDS | "10000" | Indicates the maximum size of data (in bytes) that the originating MCData client is allowed to send to the MCData server for on-network SDS communications |  |  |
| mcpttgi:mcdata-on-network-max-data-size-for-FD | "10000" | Indicates the maximum size of data (in bytes) that the originating MCData client is allowed to send to the MCData server for on-network FD communications |  |  |
| mcpttgi:mcdata-on-network-max-data-size-auto-recv | "2000" | Indicates the maximum size of data (in bytes) which the MCData server always requests the terminating MCData client to automatically download for on-network FD communications using HTTP |  |  |
| mcpttgi:mcdata-off-network-ProSe-signalling-PPPP | "1" | Indicates the ProSe Per-Packet Priority value to be used when transmitting IP packets carrying signalling for a call on the MCData group in off-network MCData procedures |  |  |
| mcpttgi:mcdata-off-network-ProSe-media-PPPP | "1" | Indicates the ProSe Per-Packet Priority value to be used when transmitting IP packets carrying media for a call on the MCData group in off-network MCData procedures |  |  |

<END OF CHANGES>