3GPP TSG-RAN5 Meeting #93-e	R5-217999

# Electronic Meeting, 8th Nov 2021 - 19th Nov 2021

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 2621 | rev | 1 | Current version: | 16.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of new CVX TC 12.1.3.1-PC5-only operation / Measurement configuration and reporting via PC5 RRC / PSBCH-RSRP measurement configuration |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | TDIA,CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_V2X_NRSL_eV2XARC-UEConTest |  |  |  |  |  | Date: |  |  | 2021-10-19 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | - Addition of new CVX test case 12.1.3.1 in accordance with work plan. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Adding CVX test case 12.1.3.1- PC5-only operation / Measurement configuration and reporting via PC5 RRC / PSBCH-RSRP measurement configuration. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Work plan cannot be completed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 12.1.3.1(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS/TR 38.523-2 CR 0184 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Changed Section>

### 12.1.3	PC5-only operation / Measurement configuration and reporting via PC5 RRC

12.1.3.1	PC5-only operation / Measurement configuration and reporting via PC5 RRC / PSBCH-RSRP measurement configuration

12.1.3.1.1	Test Purpose (TP)

(1)

with {  UE having established PC5 RRC connection with peer UE on unicast sidelink }

ensure that {

when { UE is configured by upper layer to configure periodical PSBCH-RSRP measurement}

then { UE sends a RRCReconfigurationSidelink message to peer UE }

}

12.1.3.1.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 38.331 [22], subclause 5.8.1, 5.8.9.1.3, 5.8.9.1.9. Unless otherwise stated these are Rel-16 requirements.

[TS 38.331, clause 5.8.1]

The PC5-RRC signalling, as specified in sub-clause 5.8.9, can be initiated after its corresponding PC5 unicast link establishment (TS 23.287 [55]).

[TS 38.331, clause 5.8.9.1.2]

The UE shall set the contents of RRCReconfigurationSidelink message as follows:

1>	for each sidelink DRB that is to be released, according to sub-clause 5.8.9.1a.1.1, due to configuration by sl-ConfigDedicatedNR, SIB12, SidelinkPreconfigNR or by upper layers:

2>	set the SLRB-PC5-ConfigIndex included in the slrb-ConfigToReleaseList corresponding to the sidelink DRB;

1>	for each sidelink DRB that is to be established or modified, according to sub-clause 5.8.9.1a.2.1, due to receiving sl-ConfigDedicatedNR, SIB12 or SidelinkPreconfigNR:

2>	set the SLRB-Config included in the slrb-ConfigToAddModList, according to the received sl-RadioBearerConfig and sl-RLC-BearerConfig corresponding to the sidelink DRB;

1>	set the sl-MeasConfig as follows:

2>	If the frequency used for NR sidelink communication is included in sl-FreqInfoToAddModList in sl-ConfigDedicatedNR within RRCReconfiguration message or included in sl-ConfigCommonNR within SIB12:

3>	if UE is in RRC_CONNECTED:

4>	set the sl-MeasConfig according to stored NR sidelink measurement configuration information for this destination;

3>	if UE is in RRC_IDLE or RRC_INACTIVE:

4>	set the sl-MeasConfig according to stored NR sidelink measurement configuration received from SIB12;

2>	else:

3>	set the sl-MeasConfig according to the sl-MeasPreconfig in SidelinkPreconfigNR;

1>	start timer T400 for the destination associated with the sidelink DRB;

1>	set the sl-CSI-RS-Config;

1>	set the sl-LatencyBoundCSI-Report,

NOTE 1:	How to set the parameters included in sl-CSI-RS-Config and sl-LatencyBoundCSI-Report is up to UE implementation.

The UE shall submit the RRCReconfigurationSidelink message to lower layers for transmission.

[TS 38.331, clause 5.8.9.1.3]

The UE shall perform the following actions upon reception of the RRCReconfigurationSidelink:

1>	if the RRCReconfigurationSidelink includes the sl-ResetConfig:

2>	perform the sidelink reset configuration procedure as specified in 5.8.9.1.10;

1>	if the RRCReconfigurationSidelink includes the slrb-ConfigToReleaseList:

2>	for each SLRB-PC5-ConfigIndex value included in the slrb-ConfigToReleaseList that is part of the current UE sidelink configuration;

3>	perform the sidelink DRB release procedure, according to sub-clause 5.8.9.1a.1;

1>	if the RRCReconfigurationSidelink includes the slrb-ConfigToAddModList:

2>	for each slrb-PC5-ConfigIndex value included in the slrb-ConfigToAddModList that is not part of the current UE sidelink configuration:

3>	if sl-MappedQoS-FlowsToAddList is included:

4>	apply the SL-PQFI included in sl-MappedQoS-FlowsToAddList;

3>	perform the sidelink DRB addition procedure, according to sub-clause 5.8.9.1a.2;

2>	for each slrb-PC5-ConfigIndex value included in the slrb-ConfigToAddModList that is part of the current UE sidelink configuration:

3>	if sl-MappedQoS-FlowsToAddList is included:

4>	add the SL-PQFI included in sl-MappedQoS-FlowsToAddList to the corresponding sidelink DRB;

3>	if sl-MappedQoS-FlowsToReleaseList is included:

4>	remove the SL-PQFI included in sl-MappedQoS-FlowsToReleaseList from the corresponding sidelink DRB;

3>	if the sidelink DRB release conditions as described in sub-clause 5.8.9.1a.1.1 are met:

4>	perform the sidelink DRB release procedure according to sub-clause 5.8.9.1a.1.2;

3>	else if the sidelink DRB modification conditions as described in sub-clause 5.8.9.1a.2.1 are met:

4>	perform the sidelink DRB modification procedure according to sub-clause 5.8.9.1a.2.2;

1>	if the RRCReconfigurationSidelink message includes the sl-MeasConfig:

2>	perform the sidelink measurement configuration procedure as specified in 5.8.10;

1>	if the RRCReconfigurationSidelink message includes the sl-CSI-RS-Config:

2>	apply the sidelink CSI-RS configuration;

1>	if the RRCReconfigurationSidelink message includes the sl-LatencyBoundCSI-Report:

2>	apply the configured sidelink CSI report latency bound;

1>	if the UE is unable to comply with (part of) the configuration included in the RRCReconfigurationSidelink (i.e. sidelink RRC reconfiguration failure):

2>	continue using the configuration used prior to the reception of the RRCReconfigurationSidelink message;

2>	set the content of the RRCReconfigurationFailureSidelink message;

3>	submit the RRCReconfigurationFailureSidelink message to lower layers for transmission;

1>	else:

2>	set the content of the RRCReconfigurationCompleteSidelink message;

3>	submit the RRCReconfigurationCompleteSidelink message to lower layers for transmission;

NOTE 1:	When the same logical channel is configured with different RLC mode by another UE, the UE handles the case as sidelink RRC reconfiguration failure.

[TS 38.331, clause 5.8.9.1.9]

The UE shall perform the following actions upon reception of the RRCReconfigurationCompleteSidelink:

1>	stop timer T400 for the destination, if running;

1>	consider the configurations in the corresponding RRCReconfigurationSidelink message to be applied.

12.1.3.1.3	Test description

12.1.3.1.3.1	Pre-test conditions

System Simulator:

-	NR-SS-UE

-	NR-SS-UE1 operating as NR sidelink communication device on the resources (i.e. the frequency included in pre-configuration) that UE is expected to use for transmission and reception via PC5 interface.

-	GNSS simulator

- The GNSS simulator is started and configured for Scenario #1.

UE:

-	UE is authorised to perform NR sidelink communication.

-	The UE is equipped with below information in UE or in a USIM containing default values (as per TS 38.508-1 [4]) except for those listed in Table 12.1.3.1.3.1-1.

Table 12.1.3.1.3.1-1: UE/ USIM configuration

| USIM field | Priority | Value | Access Technology Identifier |
| --- | --- | --- | --- |
| EFUST |  | As per TS 36.508 [18] clause 4.9.3.4 |  |
| EFVST |  | Service n°119 is "available" |  |
| EFV2XP_PC5 |  | As per TS 38.508-1[4] clause 4.8.3.3.3 SL-PreconfigurationNR included in V2X data policy over PC5 is defined in Table 12.1.3.1.3.3-1 |  |

Preamble:

-	The UE is in state 4-A as defined in TS 38.508-1 [4], subclause 4.4A, using generic procedure parameter Sidelink (On), Cast Type (Unicast), GNSS Sync (On) as defined in TS 38.508-1 [4], subclause 4.5.1.

12.1.3.1.3.2	Test procedure sequence

Table 12.1.3.1.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | UE is configured by upper layer to initiate the sidelink RRC reconfiguration procedure to configure periodical PSBCH-RSRP measurement Note: This step is triggered by MMI or AT command. | -- |  |  |  |
| 2 | Check: Does UE send a RRCReconfigurationSidelink message to NR-SS-UE1? | --> | PC5 RRC: RRCReconfigurationSidelink | 1 | P |
| 3 | NR-SS-UE1 sends a RRCReconfigurationCompleteSidelink message | <-- | PC5 RRC: RRCReconfigurationCompleteSidelink |  |  |

12.1.3.1.3.3	Specific message contents

Table 12.1.3.1.3.3-1: SL-PreconfigurationNR

| Derivation path: TS 38.508-1 [4], Table 4.10.1-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| SL-PreconfigurationNR-r16 ::= SEQUENCE { |  |  |  |
| sidelinkPreconfigNR-r16 SEQUENCE { |  |  |  |
| sl-MeasPreConfig-r16 | SL-MeasConfigCommon |  |  |
| } |  |  |  |
| } |  |  |  |

Table 12.1.3.1.3.3-2: SL-MeasConfigCommon (Table 12.1.3.1.3.3-1)

| Derivation path: TS 38.508-1 [4], Table 4.6.6-13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| SL-MeasConfigCommon-r16 ::= SEQUENCE { |  |  |  |
| sl-MeasObjectListCommon-r16 ::= SEQUENCE (SIZE (1..maxNrofSL-ObjectId-r16)) OF SL-MeasObjectInfo-r16{ | 1 entry |  |  |
| SL-MeasObjectInfo-r16[1] SEQUENCE { |  | entry 1 |  |
| sl-MeasObjectId-r16 | 1 |  |  |
| sl-MeasObject-r16 SEQUENCE { |  |  |  |
| frequencyInfoSL-r16 | ARFCN-ValueNR as defined in TS 38.508-1 [4], Table 4.6.3-5 with condition SL_SSB of NRf1 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 12.1.3.1.3.3-3: RRCReconfigurationSidelink (step 2, Table 12.1.3.1.3.2-1)

| Derivation path: TS 38.508-1 [4], Table 4.6.1A-3 with condition SL_MEAS |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| RRCReconfigurationSidelink ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReconfigurationSidelink-r16 SEQUENCE { |  |  |  |
| sl-MeasConfig-r16 CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| sl-ReportConfigToAddModList-r16 | SL-ReportConfigList-r16 as defined in TS 38.508-1 [4], Table 4.6.6-24 with condition PERIODICAL |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 12.1.3.1.3.3-4: RRCReconfigurationCompleteSidelink (step 3, Table 12.1.3.1.3.2-1)

FFS

<End of Changed Section>