3GPP TSG-RAN5 Meeting #92-e	R5-217127

# Electronic Meeting, 16th – 27th Aug 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.571-2 | CR | 0155 | rev | - | Current version: | 16.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to pre-test conditions for NR DL-TDOA UE-Based test cases |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Rohde & Schwarz, |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos-UEConTest |  |  |  |  |  | Date: |  |  | 2021-10-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The current scenario used for UE-Based NR DL-TDOA positioning requires simulating a total of 4 cells in order to get a position fix. However, 3 cells are enough to get a 2D potition fix. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Modify the pre-test conditions to simulate 3 NR cells for UE-Based DL-TDOA test cases |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The current NR DL-TDOA scenario overestimates the amount of cell to get a 2D fix, requiring more resources than needed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.3.4.1, 9.3.4.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# < Unchanged sections omitted >

#### 9.3.4.1	E-SMLC Initiated Assistance Data Delivery followed by Location Information Transfer: UE-Based

Editor's note: Test configuration D is incomplete:

- The corresponding attach procedure for NG-RAN E-UTRA has not yet been defined.

- The message contents need to be revised for Test Configuration D.

9.3.4.1.1	Test Purpose (TP)

(1)

with { a NAS signalling connection existing }

ensure that {

when { UE receives assistance data and a location request for UE-based  }

then { UE sends a PROVIDE LOCATION INFORMATION  message containing a location estimate }

}

9.3.4.1.2	Conformance requirements

As defined in clause 7.3.4.1.2.

9.3.4.1.3	Test description

9.3.4.1.3.1	Pre-test conditions

System Simulator:

For Test Configuration B (Table 9.3.4.1.3.2-1):NR Cell 1.

-	Satellite signals (sub-test case 15): as specified in 8.2.1.

-	MBS signals (Sub-test 16): as specified in 8.2.4.

-	WLAN signals (Sub-test 17): as specified in 8.2.5.

-	Sub-test 20: NR Cell 1 and NR Cell 2, as specified in 8.2.10.

-	Sub-test 21: NR Cell 1, NR Cell 2 and NR Cell 3, as specified in 8.2.11.

For Test Configuration D (Table 9.3.4.1.3.2-1): LTE Cell 1.

-	Satellite signals (sub-test case 15): as specified in 8.2.1.

-	MBS signals (Sub-test 16): as specified in 8.2.4.

-	WLAN signals (Sub-test 17): as specified in 8.2.5.

UE:

The UE shall begin the test with no assistance data stored.

Preamble:

-	For Test Configuration B (Table 9.3.4.1.3.2-1): The UE is in state 3N-A as defined in TS 38.508-1 [30], subclause 4.4A on NR Cell 1.

-	For Test Configuration D (Table 9.3.4.1.3.2-1): FFS.

Related PICS/PIXIT Statements:

-

< End of changes >

# < Unchanged sections omitted >

#### 9.3.4.3	E-SMLC Initiated Position Measurement without assistance data: UE-Based

Editor's note: Test configuration D is incomplete:

- The corresponding attach procedure for NG-RAN E-UTRA has not yet been defined.

- The message contents need to be revised for Test Configuration D.

9.3.4.3.1	Test Purpose (TP)

(1)

with { a NAS signalling connection existing }

ensure that {

when { UE has no assistance data stored and receives a location request for UE-based and the UE

requires assistance data in order to fulfil the location request }

then {    UE sends a REQUEST ASSISTANCE DATA message followed by a PROVIDE LOCATION INFORMATION

message containing a location estimate }

}

9.3.4.3.2	Conformance requirements

As defined in clause 7.3.4.3.2.

9.3.4.3.3	Test description

9.3.4.3.3.1	Pre-test conditions

System Simulator:

For Test Configuration B (Table 9.3.4.3.3.2-1): NR Cell 1.

-	Satellite signals (sub-test 15): as specified in 8.2.1.

-	MBS signals (sub-test 16): as specified in 8.2.4.

-	WLAN signals (sub-test 17): as specified in 8.2.5.

-	Sub-test 20: NR Cell 1 and NR Cell 2, as specified in 8.2.10.

-	Sub-test 21: NR Cell 1, NR Cell 2 and NR Cell 3, as specified in 8.2.11.

For Test Configuration D (Table 9.3.4.3.3.2-1): LTE Cell 1.

-	Satellite signals (sub-test 15): as specified in 8.2.1.

-	MBS signals (sub-test 16): as specified in 8.2.4.

-	WLAN signals (sub-test 17): as specified in 8.2.5.

UE:

The UE shall begin the test with no assistance data stored.

Preamble:

-	For Test Configuration B (Table 9.3.4.3.3.2-1): The UE is in state 3N-A as defined in TS 38.508-1 [30], subclause 4.4A on NR Cell 1.

-	For Test Configuration D (Table 9.3.4.3.3.2-1): FFS.

Related PICS/PIXIT Statements:

Method of triggering an LPP Request Assistance Data message.

< End of changes >