3GPP TSG-RAN5 Meeting #93-e	R5-217069

# Online, , 8th Nov 2021 - 19th Nov 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 2582 | rev | - | Current version: | 16.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update of NR 2-step RACH test case 7.1.1.1.10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_2step_RACH-UEConTest |  |  |  |  |  | Date: |  |  | 2021-10-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Specific message contents in test case 7.1.1.1.10 are FFS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added specific message contents of the test case. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The test case will be incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.1.1.10 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

7.1.1.1.10	Random access procedure / 2-step RACH/not complete/ RA_TYPE to 4-stepRA

7.1.1.1.10.1	Test Purpose (TP)

(1)

with { UE in RRC_Connected state after transmission of a MSGA on NR SpCell }

ensure that {

when { UE does not receive a matching MSGB in msgB-ResponseWindow and PREAMBLE_TRANSMISSION_COUNTER is equal to msgA-TransMax+1 }

then { UE triggers 4-step RACH procedue on NR SpCell }

}

7.1.1.1.10.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 38.321, clauses 5.1.1, and 5.1.4a.

[TS 38.321, clause5.1.1]

The Random Access procedure described in this clause is initiated by a PDCCH order, by the MAC entity itself, or by RRC for the events in accordance with TS 38.300 [2]. There is only one Random Access procedure ongoing at any point in time in a MAC entity. The Random Access procedure on an SCell shall only be initiated by a PDCCH order with ra-PreambleIndex different from 0b000000.

…

1>	if the Random Access procedure is initiated by PDCCH order and if the ra-PreambleIndex explicitly provided by PDCCH is not 0b000000; or

1>	if the Random Access procedure was initiated for SI request (as specified in TS 38.331 [5]) and the Random Access Resources for SI request have been explicitly provided by RRC; or

1>	if the Random Access procedure was initiated for SpCell beam failure recovery (as specified in clause 5.17) and if the contention-free Random Access Resources for beam failure recovery request for 4-step RA type have been explicitly provided by RRC for the BWP selected for Random Access procedure; or

1>	if the Random Access procedure was initiated for reconfiguration with sync and if the contention-free Random Access Resources for 4-step RA type have been explicitly provided in rach-ConfigDedicated for the BWP selected for Random Access procedure:

2>	set the RA_TYPE to 4-stepRA.

1>	else if the BWP selected for Random Access procedure is configured with both 2-step and 4-step RA type Random Access Resources and the RSRP of the downlink pathloss reference is above msgA-RSRP-Threshold; or

1>	if the BWP selected for Random Access procedure is only configured with 2-step RA type Random Access resources (i.e. no 4-step RACH RA type resources configured); or

1>	if the Random Access procedure was initiated for reconfiguration with sync and if the contention-free Random Access Resources for 2-step RA type have been explicitly provided in rach-ConfigDedicated for the BWP selected for Random Access procedure:

2>	set the RA_TYPE to 2-stepRA.

1>	else:

2>	set the RA_TYPE to 4-stepRA.

1>	perform initialization of variables specific to Random Access type as specified in clause 5.1.1a;

1>	if RA_TYPE is set to 2-stepRA:

2>	perform the Random Access Resource selection procedure for 2-step RA type (see clause 5.1.2a).

1>	else:

2>	perform the Random Access Resource selection procedure (see clause 5.1.2).

[TS 38.321, clause 5.1.4a]

Once the MSGA preamble is transmitted, regardless of the possible occurrence of a measurement gap, the MAC entity shall:

…

1>	if msgB-ResponseWindow expires, and the Random Access Response Reception has not been considered as successful based on descriptions above:

2>	increment PREAMBLE_TRANSMISSION_COUNTER by 1;

2>	if PREAMBLE_TRANSMISSION_COUNTER = preambleTransMax + 1:

3>	indicate a Random Access problem to upper layers;

3>	if this Random Access procedure was triggered for SI request:

4>	consider this Random Access procedure unsuccessfully completed.

2>	if the Random Access procedure is not completed:

3>	if msgA-TransMax is applied (see clause 5.1.1a) and PREAMBLE_TRANSMISSION_COUNTER = msgA-TransMax + 1:

4>	set the RA_TYPE to 4-stepRA;

4>	perform initialization of variables specific to Random Access type as specified in clause 5.1.1a;

4>	if the Msg3 buffer is empty:

5>	obtain the MAC PDU to transmit from the MSGA buffer and store it in the Msg3 buffer;

4>	flush HARQ buffer used for the transmission of MAC PDU in the MSGA buffer;

4>	discard explicitly signalled contention-free 2-step RA type Random Access Resources, if any;

4>	perform the Random Access Resource selection procedure as specified in clause 5.1.2.

3>	else:

4>	select a random backoff time according to a uniform distribution between 0 and the PREAMBLE_BACKOFF;

4>	if the criteria (as defined in clause 5.1.2a) to select contention-free Random Access Resources is met during the backoff time:

5>	perform the Random Access Resource selection procedure for 2-step RA type Random Access (see clause 5.1.2a).

4>	else:

5>	perform the Random Access Resource selection procedure for 2-step RA type Random Access (see clause 5.1.2a) after the backoff time.

Upon receiving a fallbackRAR, the MAC entity may stop msgB-ResponseWindow once the Random Access Response reception is considered as successful.

7.1.1.1.10.3	Test description

7.1.1.1.10.3.1	Pre-test conditions

Same Pre-test conditions as in clause 7.1.1.0.

7.1.1.1.10.3.2	Test procedure sequence

Table 7.1.1.1.10.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | SS transmits an RRCReconfiguration message to configure both 2-Step and 4-Step RA type Random Access Resources. Note 1 | <-- | RRCReconfiguration | - | - |
| 2 | The UE transmits RRCReconfigurationComplete message. Note 2 | --> | RRCReconfigurationComplete | - | - |
| 3 | SS transmits Timing Advance command to SpCell. SS does not send any subsequent timing alignments. Start Timer_T1 = Time Alignment timer value on SS. | <-- | MAC PDU (Timing Advance Command MAC Control Element) | - | - |
| 4 | 40 to 50 TTI before Timer_T1 expires the SS transmits a MAC PDU containing a PDCP SDU | <-- | MAC PDU | - | - |
| 5 | The SS ignores scheduling requests and does not allocate any uplink grant. | - | - | - | - |
| - | Exception: Step 6 will be repeated preambleTransMax times and SS does not response the MSGA in STEP 6, to make PREAMBLE_TRANSMISSION_COUNTER = msgA-TransMax+1. | - | - | - | - |
| 6 | The UE transmites MSGA using the selected PRACH occasion and the associated PUSCH resource of MSGA | --> | MAC PDU (including C-RNTI MAC CE) | - | - |
| 7 | Check: Does the UE transmit preamble on PRACH? | --> | PRACH Preamble | 1 | P |
| 8 | The SS transmits Random Access Response and RAPID corresponding to the transmitted Preamble in step 7. | <-- | Random Access Response | - | - |
| 9 | UE sends a msg3 using the grant associated to the Random Access Response received in step 8 | --> | msg3 (C-RNTI MAC CONTROL ELEMENT) | - | - |
| 10 | SS schedules PDCCH transmission for UE C_RNTI and allocate uplink grant. | <-- | Contention Resolution | - | - |
| 11 | The UE transmits a MAC PDU with C-RNTI containing looped back PDCP SDU | --> | MAC PDU | - | - |
| Note 1: for EN-DC the NR RRCReconfiguration message is contained in RRCConnectionReconfiguration. Note 2: for EN-DC the NR RRCReconfigurationComplete message is contained in RRCConnectionReconfigurationComplete. |  |  |  |  |  |

7.1.1.1.10.3.3	Specific message contents

Table 7.1.1.1.10.3.3-1: RRCReconfiguration for EN-DC (step 1, Table 7.1.1.1.10.3.2-1)

| Derivation Path: 38.508-1 [4], Table 4.6.1-13 with condition EN-DC_HO. |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReconfiguration ::= SEQUENCE { |  |  |  |
| secondaryCellGroup | CellGroupConfig |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.1.10.3.3-1A: RRCReconfiguration for NR/5GC (step 1, Table 7.1.1.1.10.3.2-1)

| Derivation Path: 38.508-1 [4], Table 4.6.1-13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| radioBearerConfig | RadioBearerConfig as per TS 38.508-1[4] Table 4.6.3-132 with conditions DRBn and Recover_PDCP | n set to the default DRB of the first PDU session | NR |
| rrcReconfiguration ::= SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| masterCellGroup | CellGroupConfig |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.1.10.3.3-2: CellGroupConfig for EN-DC (Table 7.1.1.1.10.3.3-1)

| Derivation Path: 38.508-1 [4], Table 4.6.3-19 with condition PSCell_change |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CellGroupConfig ::= SEQUENCE { |  |  |  |
| spCellConfig SEQUENCE { |  |  |  |
| spCellConfigCommon | ServingCellConfigCommon |  |  |
| reconfigurationWithSync SEQUENCE { |  |  |  |
| rach-ConfigDedicated CHOICE { |  |  |  |
| uplink | RACH-ConfigDedicated |  |  |
| } |  |  |  |
| newUE-Identity | UE identity different from NR cell 1 UE identity |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.1.10.3.3-2A: CellGroupConfig for NR/5GC (Table 7.1.1.1.10.3.3-1A)

| Derivation Path: 38.508-1 [4], Table 4.6.3-19 with condition PCell_change |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CellGroupConfig ::= SEQUENCE { |  |  |  |
| spCellConfig SEQUENCE { |  |  |  |
| reconfigurationWithSync SEQUENCE { |  |  |  |
| spCellConfigCommon | ServingCellConfigCommon |  |  |
| rach-ConfigDedicated CHOICE { |  |  |  |
| uplink | RACH-ConfigDedicated |  |  |
| } |  |  |  |
| newUE-Identity | UE identity different from NR cell 1 UE identity |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.1.10.3.3-3: RACH-ConfigDedicated (Table 7.1.1.1.10.3.3-2 and Table 7.1.1.1.10.3.3-2A)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-129 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RACH-ConfigDedicated::= SEQUENCE { |  |  |  |
| cfra SEQUENCE { |  |  |  |
| occasions SEQUENCE { |  |  |  |
| rach-ConfigGeneric | RACH-ConfigGeneric |  | FR1, PRACH Preamble format 0 used |
| } |  |  |  |
| resources CHOICE { |  |  |  |
| ssb SEQUENCE { |  |  |  |
| ssb-ResourceList SEQUENCE (SIZE(1..maxRA-SSB-Resources)) OF CFRA-SSB-Resource { | 1 entry |  |  |
| CFRA-SSB-Resource[1] SEQUENCE { |  | entry 1 |  |
| ssb | 0 |  |  |
| ra-PreambleIndex | 52 | Randomly selected |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| cfra-TwoStep-r16 SEQUENCE { |  |  |  |
| occasionsTwoStepRA-r16 SEQUENCE { |  |  |  |
| rach-ConfigGenericTwoStepRA-r16 | RACH-ConfigGenericTwoStepRA |  |  |
| ssb-PerRACH-OccasionTwoStepRA-r16 |  |  |  |
| } |  |  |  |
| msgA-CFRA-PUSCH-r16 | MsgA-PUSCH-Resource |  |  |
| msgA-TransMax-r16 | N10 |  |  |
| resourcesTwoStep-r16 SEQUENCE { |  |  |  |
| ssb-ResourceList SEQUENCE (SIZE(1..maxRA-SSB-Resources)) OF CFRA-SSB-Resource { |  |  |  |
| ssb | 0 |  |  |
| ra-PreambleIndex | 52 | Randomly selected |  |
| msgA-PUSCH-Resource-Index-r16 | Not present |  |  |
| } |  |  |  |
| ra-ssb-OccasionMaskIndex | 0 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.1.10.3.3-4: RACH-ConfigGeneric (Table 7.1.1.1.10.3.3-3)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-129 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RACH-ConfigDedicated::= SEQUENCE { |  |  |  |
| prach-ConfigurationIndex | 14 |  | FR1 |
|  | 149 |  | FR2 |
| zeroCorrelationZoneConfig | 12 |  | FR1 |
|  | 15 |  | FR2 |
| } |  |  |  |

Table 7.1.1.1.10.3.3-5: RACH-ConfigGenericTwoStepRA (Table 7.1.1.1.10.3.3-3)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-130A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RACH-ConfigDedicated::= SEQUENCE { |  |  |  |
| msgA-PRACH-ConfigurationIndex-r16 | Not present |  |  |
| msgA-RO-FDM-r16 | Not present |  |  |
| msgA-RO-FrequencyStart-r16 | Not present |  |  |
| msgA-ZeroCorrelationZoneConfig-r16 | Not present |  |  |
| msgA-PreamblePowerRampingStep-r16 | Not present |  |  |
| msgA-PreambleReceivedTargetPower-r16 | Not present |  |  |
| } |  |  |  |

Table 7.1.1.1.10.3.3-6: ServingCellConfigCommon (Table 7.1.1.1.10.3.3-2 and Table 7.1.1.1.10.3.3-2A)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-168 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfigCommon ::= SEQUENCE { |  |  |  |
| uplinkConfigCommon SEQUENCE { |  |  |  |
| initialUplinkBWP | BWP-UplinkCommon |  |  |
| } |  |  |  |
| tdd-UL-DL-ConfigurationCommon | TDD-UL-DL-ConfigCommon |  |  |
| } |  |  |  |

Table 7.1.1.1.10.3.3-7: BWP-UplinkCommon (Table 7.1.1.1.10.3.3-6)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-10 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| BWP-UplinkCommon ::= SEQUENCE { |  |  |  |
| rach-ConfigCommon CHOICE { |  |  |  |
| setup | RACH-ConfigCommon |  |  |
| } |  |  |  |
| msgA-ConfigCommon-r16 CHOICE { |  |  |  |
| setup | MsgA-ConfigCommon |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.1.10.3.3-8: RACH-ConfigCommon (Table 7.1.1.1.10.3.3-7)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-128 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RACH-ConfigCommon::= SEQUENCE { |  |  |  |
| rach-ConfigGeneric | RACH-ConfigGeneric |  |  |
| ssb_perRACH_OccasionAndCB_PreamblesPerSSB CHOICE { |  |  |  |
| one | n36 |  |  |
| } |  |  |  |
| prach-RootSequenceIndex CHOICE { |  |  |  |
| l139 | Set according to table 4.4.2-2 in TS 38.508-1 [4] for the NR Cell. |  |  |
| l839 | Set according to table 4.4.2-2 in TS 38.508-1 [4] for the NR Cell. | PRACH Preamble format 0 used | FR1, |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.1.10.3.3-9: MsgA-ConfigCommon (Table 7.1.1.1.10.3.3-7)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-81A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MsgA-ConfigCommonL-r16 ::= SEQUENCE { |  |  |  |
| rach-ConfigCommonTwoStepRA-r16 | RACH-ConfigCommonTwoStepRA |  |  |
| msgA-PUSCH-Config-r16 | MsgA-PUSCH-Config |  |  |
| } |  |  |  |

Table 7.1.1.1.10.3.3-10: TDD-UL-DL-ConfigCommon (Table 7.1.1.1.10.3.3-6)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-192 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| TDD-UL-DL-ConfigCommon ::= SEQUENCE { |  |  |  |
| referenceSubcarrierSpacing | SubcarrierSpacing |  |  |
| pattern1 SEQUENCE { |  |  |  |
| dl-UL-TransmissionPeriodicity | ms5 |  | FR1 |
|  | ms0p625 |  | FR2 |
| nrofDownlinkSlots | 3 |  | FR1 |
|  | 3 |  | FR2 |
| nrofDownlinkSymbols | 6 |  | FR1 |
|  | 10 |  | FR2 |
| nrofUplinkSlots | 2 |  | FR1 |
|  | 1 |  | FR2 |
| nrofUplinkSymbols | 4 |  | FR1 |
|  | 2 |  | FR2 |
| dl-UL-TransmissionPeriodicity-v1530 | ms3 |  | FR1 |
| } |  |  |  |
| pattern2 | Not present |  |  |
| pattern2 SEQUENCE { |  |  | FR1 |
| dl-UL-TransmissionPeriodicity | ms2 |  |  |
| nrofDownlinkSlots | 4 |  |  |
| nrofDownlinkSymbols | 0 |  |  |
| nrofUplinkSlots | 0 |  |  |
| nrofUplinkSymbols | 0 |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.1.10.3.3-11: RACH-ConfigCommonTwoStepRA (Table 7.1.1.1.10.3.3-7)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-128A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RACH-ConfigCommonTwoStepRA-r16 ::= SEQUENCE { |  |  |  |
| msgA-RSRP-Threshold-r16 | 57 | -100 dBm |  |

Table 7.1.1.1.10.3.3-12: MsgA-PUSCH-Config (Table 7.1.1.1.10.3.3-7)

Derivation Path: TS 38.508-1 [4], Table 4.6.3-81B