3GPP TSG-RAN5 Meeting #93-e	R5-217955

# Online, , 8th Nov 2021 - 19th Nov 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-1 | CR | 5069 | rev | 1 | Current version: | 16.10.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Additional updates to protocol test configurations to reflect the Band 24 condition |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ligado Networks |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_B24_mod-UEConTest |  |  |  |  |  | Date: |  |  | 2021-10-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | A new condition for Band 24 was specified for inclusion of ul-carrierfreq IE for certain test cases due to the high frequency range having a Tx-Rx spacing that is different from default |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update message contents to include condition for all test cases irrespective of the target cell to include ul-carrierfreq IE if target cell is a Band 24 cell configured to operate on the high freq range |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Future updates to the mapping for the Cell and the test frequency can lead to failed test case(s) |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3.5.6, 8.2.4.9, 8.2.4.10, 8.2.4.13, 8.2.4.13a, 8.2.4.14, 8.2.4.14a, 8.2.4.15, 8.2.4.30.1, 8.2.4.30.2, 8.2.4.30.3, 8.2.4.30.5, 8.2.4.31, 8.3.1.11, 8.3.1.11a, 8.3.1.15, 8.4.2.2, 8.4.2.4, 8.6.5.1, 8.6.5.2, 13.4.1.3, 13.4.1.4, 13.4.1.5, 13.4.2.6, 13.4.2.7, 13.4.2.8, 21.1.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  | Rel-17 to trigger a spec. upgrade Band 24 is unique in that after regulatory update, only 10 MHz of DL is usable while 20 MHz (2 10 MHz non-contiguous blocks) of UL is usable. Two Tx-Rx spacing have been specified so that 10 MHz of DL can be paired with either of the 2 10 MHz UL blocks. To ensure that both the 10 MHz UL blocks are fully tested, the test frequencies have specified such that the Low and Mid test frequencies are based on Tx-Rx spacing of -101.5 MHz while High test frequency is based on Tx-Rx spacing of -120.5 MHz. Cells associated with high test frequency will require inclusion of the UL EARFCN in several test cases that involve Band 24 high frequency range. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev1: i) Removed changes to clauses 7.1.2.1, 7.1.2.1a, 7.1.2.8, 7.1.4.3, 7.1.4.12, 7.2.2.11, 7.2.3.21, 7.3.5.2, 7.3.5.3, 7.3.5.4, 7.3.5.5, 7.3.7.4, 8.3.1.8, 8.3.1.9, 8.3.1.9a, 8.3.1.16, 8.3.1.21, 8.6.4.5, 8.6.6.1, 8.6.6.5, 8.6.7.1, 8.6.7.2, 8.6.7.3, 13.2.1, 17.1.3, 17.3.1, 17.4.4, 17.4.5, 21.3.4, 21.3.5 as no changes are necessary since the carrierFreq IE is not present; ii) Added changes to clauses 8.2.4.9, 8.2.4.10, 8.2.4.13, 8.2.4.13a, 8.2.4.14, 8.2.4.14a, 8.2.4.15, 8.2.4.30.1, 8.2.4.30.2, 8.2.4.30.3, 8.2.4.30.5 and 8.2.4.31 as section 8.2.4 changes were missed; iii) Performed changes to Table 8.2.4.14.3.3-6 at the request of MCC TF160 – correcting existing IE location by moving them from the top to under MobilityControlInfo IE; iv) Performed changes to Table 8.2.4.14.3.3-6 and 8.2.4.14a.3.3-12 at the requrest of MCC TF160 – correcting the order of IE to conform with 36.331. |  |  |  |  |  |  |  |  |

#### 7.3.5.6	PDCP handover / DAPS handover with key change / Status reporting

<<Unchanged Text Skipped>>

7.3.5.6.3.3	Specific message contents

Table 7.3.5.6.3.3-0: Conditions for specific message contents
in Table 7.3.5.6.3.3-2

| Condition | Explanation |
| --- | --- |
| Band 24 High range | If Band 24 high frequency range is selected for the target cell |

<<Unchanged Text Skipped>>

Table 7.3.5.6.3.3-2: MobilityControlInfo (Table 7.3.5.6.3.3-1)

| Derivation Path: 36.508, Table 4.6.5-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MobilityControlInfo ::= SEQUENCE { |  |  |  |
| targetPhysCellId | PhysicalCellIdentity of Cell 2 |  |  |
| carrierFreq SEQUENCE { |  |  |  |
| dl-CarrierFreq | Same downlink EARFCN as used for Cell 2 |  |  |
| ul-CarrierFreq | Not present |  |  |
|  | Same uplink EARFCN as used for Cell 2 |  | Band 24 High range |
| } |  |  |  |
| radioResourceConfigCommon | RadioResourceConfigCommon-DEFAULT |  |  |
| rach-ConfigDedicated | Rach-ConfigDedicated-DEFAULT |  |  |
| } |  |  |  |

<<Unchanged Text Skipped>>

------------------------------------------------------------- NEXT CHANGE ----------------------------------------------------------

#### 8.2.4.9	RRC connection reconfiguration / Handover / Inter-band blind handover / Success

<<Unchanged Text Skipped>>

8.2.4.9.3.3	Specific message contents

Table 8.2.4.9.3.3-0: Conditions for specific message contents
in Table 8.2.4.9.3.3-2

| Condition | Explanation |
| --- | --- |
| Band > 64 | If band > 64 is selected |
| Band 24 High range | If Band 24 high frequency range is selected for the target cell |

<<Unchanged Text Skipped>>

Table 8.2.4.9.3.3-2: MobilityControlInfo (Table 8.2.4.9.3.3-1)

| Derivation Path: 36.508, Table 4.6.5-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MobilityControlInfo ::= SEQUENCE { |  |  |  |
| targetPhysCellId | PhysicalCellIdentity of Cell 10 |  |  |
| carrierFreq SEQUENCE { |  |  |  |
| dl-CarrierFreq | Same downlink EARFCN as used for Cell 10 |  |  |
| ul-CarrierFreq | Not present |  |  |
|  | Same uplink EARFCN as used for Cell 10 |  | Band 24 High range |
| } |  |  |  |
| carrierFreq SEQUENCE {} | Not present |  | Band > 64 |
| carrierFreq-v9e0 SEQUENCE { |  |  | Band > 64 |
| dl-CarrierFreq-v9e0 | Same downlink EARFCN as used for Cell 10 |  |  |
| ul-CarrierFreq-v9e0 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |

<<Unchanged Text Skipped>>

------------------------------------------------------------- NEXT CHANGE ----------------------------------------------------------

#### 8.2.4.10	RRC connection reconfiguration / Handover (between FDD and TDD)

<<Unchanged Text Skipped>>

8.2.4.10.3.3	Specific message content

Table 8.2.4.10.3.3-0: Conditions for specific message contents
in Tables 8.2.4.10.3.3-2 and 8.2.4.10.3.3-4

| Condition | Explanation |
| --- | --- |
| Band > 64 | If band > 64 is selected |
| Band 24 High range | If Band 24 high frequency range is selected for the target cell |

<<Unchanged Text Skipped>>

Table 8.2.4.10.3.3-2: MobilityControlInfo (Table 8.2.4.10.3.3-1)

| Derivation Path: 36.508, Table 4.6.5-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MobilityControlInfo-HO ::= SEQUENCE { |  |  |  |
| targetPhysCellId | PhysicalCellIdentity of Cell 10 |  |  |
| carrierFreq SEQUENCE { |  |  |  |
| dl-CarrierFreq | Same downlink EARFCN as used for Cell 10 |  |  |
| ul-CarrierFreq | Not present |  |  |
|  | Same uplink EARFCN as used for Cell 10 |  | Band 24 High range |
| } |  |  |  |
| carrierFreq SEQUENCE {} | Not present |  | Band > 64 |
| carrierFreq-v9e0 SEQUENCE { |  |  | Band > 64 |
| dl-CarrierFreq-v9e0 | Same downlink EARFCN as used for Cell 10 |  |  |
| ul-CarrierFreq-v9e0 | Not present |  |  |
| } |  |  |  |
| carrierBandwidth SEQUENCE { |  |  |  |
| dl-Bandwidth | Same downlink system bandwidth as used for Cell 10 |  |  |
| ul-Bandwidth | Not present |  |  |
| } |  |  |  |
| additionalSpectrumEmission | Same additionalSpectrumEmission as used for Cell 10 |  |  |
| radioResourceConfigCommon | RadioResourceConfigCommon-withTDDconfig |  |  |
| } |  |  |  |

<<Unchanged Text Skipped>>

------------------------------------------------------------- NEXT CHANGE ----------------------------------------------------------

#### 8.2.4.13	RRC connection reconfiguration / Handover / Success (with measurement) / Inter-band

<<Unchanged Text Skipped>>

8.2.4.13.3.3	Specific message contents

Table 8.2.4.13.3.3-0: Conditions for specific message contents
in Tables 8.2.4.13.3.3-2 and 8.2.4.13.3.3-5

| Condition | Explanation |
| --- | --- |
| Band > 64 | If band > 64 is selected |
| Band 24 High range | If Band 24 high frequency range is selected for the target cell |

<<Unchanged Text Skipped>>

Table 8.2.4.13.3.3-5: MobilityControlInfo (Table 8.2.4.13.3.3-4)

| Derivation Path: 36.508, Table 4.6.5-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MobilityControlInfo ::= SEQUENCE { |  |  |  |
| targetPhysCellId | PhysicalCellIdentity of Cell 10 |  |  |
| carrierFreq SEQUENCE { |  |  |  |
| dl-CarrierFreq | Same downlink EARFCN as used for Cell 10 |  |  |
| ul-CarrierFreq | Not present |  |  |
|  | Same uplink EARFCN as used for Cell 10 |  | Band 24 High range |
| } |  |  |  |
| carrierFreq SEQUENCE {} | Not present |  | Band > 64 |
| carrierFreq-v9e0 SEQUENCE { |  |  | Band > 64 |
| dl-CarrierFreq-v9e0 | Same downlink EARFCN as used for Cell 10 |  |  |
| ul-CarrierFreq-v9e0 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |

<<Unchanged Text Skipped>>

------------------------------------------------------------- NEXT CHANGE ----------------------------------------------------------

#### 8.2.4.13a	RRC connection reconfiguration / Handover / Success (with measurement) / Inter-band / Between FDD and TDD

<<Unchanged Text Skipped>>

8.2.4.13a.3.3	Specific message contents

Table 8.2.4.13a.3.3-0: Conditions for specific message contents
in Tables 8.2.4.13a.3.3-2, 8.2.4.13a.3.3-5, 8.2.4.13a.3.3-6 and 8.2.4.13a.3.3-8

| Condition | Explanation |
| --- | --- |
| Band > 64 | If band > 64 is selected |
| Band 24 High range | If Band 24 high frequency range is selected for the target cell |

<<Unchanged Text Skipped>>

Table 8.2.4.13a.3.3-8: MobilityControlInfo (step 12, Table 8.2.4.13a.3.3-4)

| Derivation Path: 36.508, Table 4.6.5-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MobilityControlInfo ::= SEQUENCE { |  |  |  |
| targetPhysCellId | PhysicalCellIdentity of Cell 1 |  |  |
| carrierFreq SEQUENCE { |  |  |  |
| dl-CarrierFreq | Same downlink EARFCN as used for Cell 1 |  |  |
| ul-CarrierFreq | Not present |  |  |
|  | Same uplink EARFCN as used for Cell 1 |  | Band 24 High range |
| } |  |  |  |
| carrierFreq SEQUENCE {} | Not present |  | Band > 64 |
| carrierFreq-v9e0 SEQUENCE { |  |  | Band > 64 |
| dl-CarrierFreq-v9e0 | Same downlink EARFCN as used for Cell 1 |  |  |
| ul-CarrierFreq-v9e0 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |

<<Unchanged Text Skipped>>

------------------------------------------------------------- NEXT CHANGE ----------------------------------------------------------

#### 8.2.4.14	RRC connection reconfiguration / Handover / Failure / Re-establishment successful / Inter-band

<<Unchanged Text Skipped>>

8.2.4.14.3.3	Specific message contents

Table 8.2.4.14.3.3-0: Conditions for specific message contents
in Tables 8.2.4.14.3.3-3 and 8.2.4.14.3.3-6

| Condition | Explanation |
| --- | --- |
| Band > 64 | If band > 64 is selected |
| Band 24 High range | If Band 24 high frequency range is selected for the target cell |

<<Unchanged Text Skipped>>

Table 8.2.4.14.3.3-6: MobilityControlInfo (Table 8.2.4.14.3.3-5)

| Derivation Path: 36.308, Table 4.6.5-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment |  | Condition |
| MobilityControlInfo ::= SEQUENCE { |  |  |  |  |
| targetPhysCellId | PhysicalCellIdentity of Cell 10 |  |  |  |
| carrierFreq SEQUENCE {} | Not present |  |  | Band > 64 |
| carrierFreq SEQUENCE { |  |  |  |  |
| dl-CarrierFreq | Same downlink EARFCN as used for Cell 10 |  |  |  |
| ul-CarrierFreq | Not present |  |  |  |
|  | Same uplink EARFCN as used for Cell 10 |  |  | Band 24 High range |
| } |  |  |  |  |
| rach-ConfigDedicated SEQUENCE { |  |  |  |  |
| ra-PreambleIndex | 63 |  |  |  |
| ra-PRACH-MaskIndex | 0 |  |  |  |
| } |  |  |  |  |
| carrierFreq-v9e0 SEQUENCE { |  |  |  | Band > 64 |
| dl-CarrierFreq-v9e0 | Same downlink EARFCN as used for Cell 10 |  |  |  |
| ul-CarrierFreq-v9e0 | Not present |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

<<Unchanged Text Skipped>>

------------------------------------------------------------- NEXT CHANGE ----------------------------------------------------------

#### 8.2.4.14a	RRC connection reconfiguration / Handover / Failure / Re-establishment successful / Inter-band / Between FDD and TDD

<<Unchanged Text Skipped>>

8.2.4.14a.3.3	Specific message contents

Table 8.2.4.14a.3.3-00: Conditions for specific message contents
in Tables 8.2.4.14a.3.3-2, 8.2.4.14a.3.3-5 and 8.2.4.14a.3.3-12

| Condition | Explanation |
| --- | --- |
| Band > 64 | If band > 64 is selected |
| Band 24 High range | If Band 24 high frequency range is selected for the target cell |

<<Unchanged Text Skipped>>

Table 8.2.4.14a.3.3-12: MobilityControlInfo (Table 8.2.4.14a.3.3-11)

| Derivation Path: 36.308, Table 4.6.5-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MobilityControlInfo ::= SEQUENCE { |  |  |  |
| targetPhysCellId | PhysicalCellIdentity of Cell 1 |  |  |
| carrierFreq SEQUENCE { |  |  |  |
| dl-CarrierFreq | Same downlink EARFCN as used for Cell 1 |  |  |
| ul-CarrierFreq | Not present |  |  |
|  | Same uplink EARFCN as used for Cell 1 |  | Band 24 High range |
| } |  |  |  |
| carrierFreq SEQUENCE {} | Not present |  | Band > 64 |
| rach-ConfigDedicated SEQUENCE { |  |  |  |
| ra-PreambleIndex | 63 |  |  |
| ra-PRACH-MaskIndex | 0 |  |  |
| } |  |  |  |
| carrierFreq-v9e0 SEQUENCE { |  |  | Band > 64 |
| dl-CarrierFreq-v9e0 | Same downlink EARFCN as used for Cell 1 |  |  |
| ul-CarrierFreq-v9e0 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |

<<Unchanged Text Skipped>>

------------------------------------------------------------- NEXT CHANGE ----------------------------------------------------------

#### 8.2.4.15	RRC connection reconfiguration / Handover / Failure / Re-establishment failure / Inter-band

<<Unchanged Text Skipped>>

8.2.4.15.3.3	Specific message contents

Table 8.2.4.15.3.3-0: Conditions for specific message contents
in Tables 8.2.4.15.3.3-2 and 8.2.4.15.3.3-5

| Condition | Explanation |
| --- | --- |
| Band > 64 | If band > 64 is selected |
| Band 24 High range | If Band 24 high frequency range is selected for the target cell |

<<Unchanged Text Skipped>>

Table 8.2.4.15.3.3-5: MobilityControlInfo (Table 8.2.4.15.3.3-4)

| Derivation Path: 36.508 Table 4.6.5-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment |  | Condition |
| MobilityControlInfo ::= SEQUENCE { |  |  |  |  |
| targetPhysCellId | PhysicalCellIdentity of Cell 10 |  |  |  |
| carrierFreq SEQUENCE { |  |  |  |  |
| dl-CarrierFreq | Same downlink EARFCN as used for Cell 10 |  |  |  |
| ul-CarrierFreq | Not present |  |  |  |
|  | Same uplink EARFCN as used for Cell 10 |  |  | Band 24 High range |
| } |  |  |  |  |
| carrierFreq SEQUENCE {} | Not present |  |  | Band > 64 |
| carrierFreq-v9e0 SEQUENCE { |  |  |  | Band > 64 |
| dl-CarrierFreq-v9e0 | Same downlink EARFCN as used for Cell 10 |  |  |  |
| ul-CarrierFreq-v9e0 | Not present |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

<<Unchanged Text Skipped>>

------------------------------------------------------------- NEXT CHANGE ----------------------------------------------------------

##### 8.2.4.30.1	RRC connection reconfiguration / Handover / DAPS Handover / Success / Intra-Frequency

<<Unchanged Text Skipped>>

Table 8.2.4.30.1.3.3-2: MobilityControlInfo (Table 8.2.4.30.1.3.3-1)

| Derivation Path: 36.508, Table 4.6.5-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MobilityControlInfo ::= SEQUENCE { |  |  |  |
| targetPhysCellId | PhysicalCellIdentity of Cell 2 |  |  |
| carrierFreq SEQUENCE { |  |  |  |
| dl-CarrierFreq | Same downlink EARFCN as used for Cell 2 |  |  |
| ul-CarrierFreq | Not present |  |  |
|  | Same uplink EARFCN as used for Cell 2 |  | Band 24 High range |
| } |  |  |  |
| radioResourceConfigCommon | RadioResourceConfigCommon-DEFAULT |  |  |
| rach-ConfigDedicated | Rach-ConfigDedicated-DEFAULT |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| Band 24 High range | If Band 24 high frequency range is selected for the target cell |

<<Unchanged Text Skipped>>

------------------------------------------------------------- NEXT CHANGE ----------------------------------------------------------

##### 8.2.4.30.2	DAPS handover / Failure / source link available / Radio Link Failure in source / Intra-Frequency

<<Unchanged Text Skipped>>

Table 8.2.4.30.2.3.3-3: MobilityControlInfo (Table 8.2.4.30.2.3.3-2)

| Derivation Path: 36.508, Table 4.6.5-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MobilityControlInfo ::= SEQUENCE { |  |  |  |
| targetPhysCellId | PhysicalCellIdentity of Cell 2 |  | Step 2 and 18 |
|  | PhysicalCellIdentity of Cell 1 |  | Step 11 |
| carrierFreq SEQUENCE { |  |  |  |
| dl-CarrierFreq | Same downlink EARFCN as used for Cell 2 |  | Step 2 and 18 |
|  | Same downlink EARFCN as used for Cell 1 |  | Step 11 |
| ul-CarrierFreq | Not present |  |  |
|  | Same uplink EARFCN as used for Cell 2 |  | Band 24 High range, Step 2 and 18 |
|  | Same uplink EARFCN as used for Cell 1 |  | Band 24 High range, Step 11 |
| } |  |  |  |
| t304 | ms2000 |  |  |
| radioResourceConfigCommon | RadioResourceConfigCommon-DEFAULT |  |  |
| rach-ConfigDedicated | Rach-ConfigDedicated-DEFAULT |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| Band 24 High range | If Band 24 high frequency range is selected for the target cell |

<<Unchanged Text Skipped>>

------------------------------------------------------------- NEXT CHANGE ----------------------------------------------------------

##### 8.2.4.30.3	DAPS handover / Failure / source link available / Radio Link Failure in source / Intra-Frequency

<<Unchanged Text Skipped>>

Table 8.2.4.30.3.3.3-2: MobilityControlInfo (Table 8.2.4.30.3.3.3-1)

| Derivation Path: 36.508, Table 4.6.5-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MobilityControlInfo ::= SEQUENCE { |  |  |  |
| targetPhysCellId | PhysicalCellIdentity of Cell 2 |  | Step 2 and 6 |
|  | PhysicalCellIdentity of Cell 1 |  | Step 14 |
| carrierFreq SEQUENCE { |  |  |  |
| dl-CarrierFreq | Same downlink EARFCN as used for Cell 2 |  | Step 2 and 6 |
|  | Same downlink EARFCN as used for Cell 1 |  | Step 14 |
| ul-CarrierFreq | Not present |  |  |
|  | Same uplink EARFCN as used for Cell 2 |  | Band 24 High range, Step 2 and 6 |
|  | Same uplink EARFCN as used for Cell 1 |  | Band 24 High range, Step 14 |
| } |  |  |  |
| radioResourceConfigCommon | RadioResourceConfigCommon-DEFAULT |  |  |
| rach-ConfigDedicated | Rach-ConfigDedicated-DEFAULT |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| Band 24 High range | If Band 24 high frequency range is selected for the target cell |

<<Unchanged Text Skipped>>

------------------------------------------------------------- NEXT CHANGE ----------------------------------------------------------

##### 8.2.4.30.5	DAPS handover / Failure / source link available / Radio Link Failure in source / Inter-Frequency

<<Unchanged Text Skipped>>

Table 8.2.4.30.5.3.3-3: MobilityControlInfo (Table 8.2.4.30.5.3.3-2)

| Derivation Path: 36.508, Table 4.6.5-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MobilityControlInfo ::= SEQUENCE { |  |  |  |
| targetPhysCellId | PhysicalCellIdentity of Cell 2 |  | Step 2 and 18 |
|  | PhysicalCellIdentity of Cell 1 |  | Step 11 |
| carrierFreq SEQUENCE { |  |  |  |
| dl-CarrierFreq | Same downlink EARFCN as used for Cell 2 |  | Step 2 and 18 |
|  | Same downlink EARFCN as used for Cell 1 |  | Step 11 |
| ul-CarrierFreq | Not present |  |  |
|  | Same uplink EARFCN as used for Cell 2 |  | Band 24 High range, Step 2 and 18 |
|  | Same uplink EARFCN as used for Cell 1 |  | Band 24 High range, Step 11 |
| } |  |  |  |
| t304 | ms2000 |  |  |
| radioResourceConfigCommon | RadioResourceConfigCommon-DEFAULT |  |  |
| rach-ConfigDedicated | Rach-ConfigDedicated-DEFAULT |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| Band 24 High range | If Band 24 high frequency range is selected for the target cell |

<<Unchanged Text Skipped>>

<<Unchanged Text Skipped>>

------------------------------------------------------------- NEXT CHANGE ----------------------------------------------------------

#### 8.2.4.31	RRC connection reconfiguration / Handover / Conditional Handover

<<Unchanged Text Skipped>>

Table 8.2.4.31.1.3.3-8: MobilityControlInfo (Table 8.2.4.31.1.3.3-7)

| Derivation Path: 36.508, Table 4.6.5-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MobilityControlInfo ::= SEQUENCE { |  |  |  |
| targetPhysCellId | PhysicalCellIdentity of Cell 2 |  | HO Cell 2 |
|  | PhysicalCellIdentity of Cell 1 |  | HO Cell 1 |
|  | PhysicalCellIdentity of Cell 4 |  | HO Cell 4 |
| carrierFreq SEQUENCE { |  |  |  |
| dl-CarrierFreq | Same downlink EARFCN as used for Cell 2 |  | HO Cell 2 |
|  | Same downlink EARFCN as used for Cell 1 |  | HO Cell 1 |
|  | Same downlink EARFCN as used for Cell 4 |  | HO Cell 4 |
| ul-CarrierFreq | Not present |  |  |
|  | Same uplink EARFCN as used for Cell 2 |  | Band 24 High range, HO Cell 2 |
|  | Same uplink EARFCN as used for Cell 1 |  | Band 24 High range, HO Cell 1 |
|  | Same uplink EARFCN as used for Cell 4 |  | Band 24 High range, HO Cell 4 |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| Band 24 High range | If Band 24 high frequency range is selected for the target cell |

<<Unchanged Text Skipped>>

------------------------------------------------------------- NEXT CHANGE ----------------------------------------------------------

#### 8.3.1.11	Measurement configuration control and reporting / Intra E-UTRAN measurements / Continuation of the measurements after RRC connection re-establishment

<<Unchanged Text Skipped>>

8.3.1.11.3.3	Specific message contents

Table 8.3.1.11.3.3-0: Conditions for specific message contents
in Tables 8.3.1.11.3.3-2 and 8.3.1.11.3.3-7

| Condition | Explanation |
| --- | --- |
| Band > 64 | If band > 64 is selected |
| Band 24 High range | If Band 24 high frequency range is selected for the target cell |

<<Unchanged Text Skipped>>

Table 8.3.1.11.3.3-7: MobilityControlInfo (step 6, Table 8.1.3.11.3.2-2)

| Derivation Path: 36.508, Table 4.6.5-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MobilityControlInfo ::= SEQUENCE { |  |  |  |
| targetPhysCellId | PhysicalCellIdentity of Cell 2 |  |  |
| carrierFreq | Not present |  | Band > 64 |
| carrierFreq SEQUENCE { |  |  |  |
| dl-CarrierFreq | Same downlink EARFCN as used for Cell 2 |  |  |
| ul-CarrierFreq | Not present |  |  |
|  | Same uplink EARFCN as used for Cell 2 |  | Band 24 High range |
| } |  |  |  |
| carrierFreq-v9e0 SEQUENCE { |  |  | Band > 64 |
| dl-CarrierFreq-v9e0 | Same downlink EARFCN as used for Cell 2. |  |  |
| } |  |  |  |
| } |  |  |  |

<<Unchanged Text Skipped>>

------------------------------------------------------------- NEXT CHANGE ----------------------------------------------------------

#### 8.3.1.11a	Measurement configuration control and reporting / Intra Frequency measurements / Continuation of the measurements after RRC connection re-establishment / Single Frequency operation

<<Unchanged Text Skipped>>

8.3.1.11a.3.3	Specific message contents

Table 8.3.1.11a.3.3-0: Conditions for specific message contents
in Tables 8.3.1.11a.3.3-2 and 8.3.1.11a.3.3-5

| Condition | Explanation |
| --- | --- |
| Band > 64 | If band > 64 is selected |
| Band 24 High range | If Band 24 high frequency range is selected for the target cell |

<<Unchanged Text Skipped>>

Table 8.3.1.11a.3.3-5: MobilityControlInfo (step 6, Table 8.1.3.11.3.2-2)

| Derivation Path: 36.508, Table 4.6.5-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MobilityControlInfo ::= SEQUENCE { |  |  |  |
| targetPhysCellId | PhysicalCellIdentity of Cell 2 |  |  |
| carrierFreq | Not present |  | Band > 64 |
| carrierFreq SEQUENCE { |  |  |  |
| dl-CarrierFreq | Same downlink EARFCN as used for Cell 2. |  |  |
| ul-CarrierFreq | Not present |  |  |
|  | Same uplink EARFCN as used for Cell 2. |  | Band 24 High range |
| } |  |  |  |
| carrierFreq-v9e0 SEQUENCE { |  |  | Band > 64 |
| dl-CarrierFreq-v9e0 | Same downlink EARFCN as used for Cell 2. |  |  |
| } |  |  |  |
| } |  |  |  |

<<Unchanged Text Skipped>>

------------------------------------------------------------- NEXT CHANGE ----------------------------------------------------------

#### 8.3.1.15	Measurement configuration control and reporting / Intra E-UTRAN measurements / Inter-band handover / IE measurement configuration not present

<<Unchanged Text Skipped>>

8.3.1.15.3.3	Specific message contents

Table 8.3.1.15.3.3-0: Conditions for specific message contents
in Tables 8.3.1.15.3.3-2 and 8.3.1.15.3.3-6

| Condition | Explanation |
| --- | --- |
| Band > 64 | If band > 64 is selected |
| Band 24 High range | If Band 24 high frequency range is selected for the target cell |

<<Unchanged Text Skipped>>

Table 8.3.1.15.3.3-6: MobilityControlInfo (Table 8.3.1.15.3.3-5)

| Derivation Path: 36.508 Table 4.6.5-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MobilityControlInfo ::= SEQUENCE { |  |  |  |
| targetPhysCellId | PhysicalCellIdentity of Cell 10 |  |  |
| carrierFreq | Not present |  | Band > 64 |
| carrierFreq SEQUENCE { |  |  |  |
| dl-CarrierFreq | Same downlink EARFCN as used for Cell 10 |  |  |
| ul-CarrierFreq | Not present |  |  |
|  | Same uplinklink EARFCN as used for Cell 10 |  | Band 24 High range |
| } |  |  |  |
| carrierFreq-v9e0 SEQUENCE { |  |  | Band > 64 |
| dl-CarrierFreq-v9e0 | Same downlink EARFCN as used for Cell 10. |  |  |
| } |  |  |  |
| } |  |  |  |

<<Unchanged Text Skipped>>

------------------------------------------------------------- NEXT CHANGE ----------------------------------------------------------

#### 8.4.2.2	Inter-RAT handover / From UTRA PS to E-UTRA / Data

<<Unchanged Text Skipped>>

8.4.2.2.3.3	Specific message contents

Table 8.4.2.2.3.3-1: Void

<<Unchanged Text Skipped>>

Table 8.4.2.2.3.3-7: MobilityControlInfo (Table 8.4.2.2.3.3-5)

| Derivation Path: 36.508, Table 4.6.5-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MobilityControlInfo ::= SEQUENCE { |  |  |  |
| targetPhysCellId | PhysicalCellIdentity of Cell 1. |  |  |
| carrierFreq SEQUENCE { |  |  |  |
| dl-CarrierFreq | Same downlink EARFCN as used for Cell 1. |  |  |
| ul-CarrierFreq | Not present |  |  |
|  | Same uplink EARFCN as used for Cell 1. |  | Band 24 High range |
| } |  |  |  |
| carrierFreq | Not present |  | Band > 64 |
| carrierBandwidth SEQUENCE { |  |  |  |
| dl-Bandwidth | Downlink system bandwidth under test. |  |  |
| ul-Bandwidth | Uplink Bandwidth under test. |  | FDD |
| ul-Bandwidth | Not present |  | TDD |
| } |  |  |  |
| additionalSpectrumEmission | 1 |  |  |
| carrierFreq-v9e0 SEQUENCE { |  |  | Band > 64 |
| dl-CarrierFreq-v9e0 | Same downlink EARFCN as used for Cell 1 |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| FDD | FDD cell environment |
| TDD | TDD cell environment |
| Band > 64 | If band > 64 is selected |
| Band 24 High range | If Band 24 high frequency range is selected for the target cell |

<<Unchanged Text Skipped>>

------------------------------------------------------------- NEXT CHANGE ----------------------------------------------------------

#### 8.4.2.4	Inter-RAT handover / From UTRA HSPA to E-UTRA / Data

<<Unchanged Text Skipped>>

8.4.2.4.3.3	Specific message contents

Table 8.4.2.4.3.3-1: Void

<<Unchanged Text Skipped>>

Table 8.4.2.4.3.3-7: MobilityControlInfo (Table 8.4.2.4.3.3-5)

| Derivation Path: 36.508, Table 4.6.5-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MobilityControlInfo ::= SEQUENCE { |  |  |  |
| targetPhysCellId | PhysicalCellIdentity of Cell 1. |  |  |
| carrierFreq SEQUENCE { |  |  |  |
| dl-CarrierFreq | Same downlink EARFCN as used for Cell 1. |  |  |
| ul-CarrierFreq | Not present |  |  |
|  | Same uplink EARFCN as used for Cell 1. |  | Band 24 High range |
| } |  |  |  |
| carrierFreq | Not present |  | Band > 64 |
| carrierBandwidth SEQUENCE { |  |  |  |
| dl-Bandwidth | Downlink system bandwidth under test. |  |  |
| ul-Bandwidth | Uplink Bandwidth under test. |  | FDD |
| ul-Bandwidth | Not present |  | TDD |
| } |  |  |  |
| additionalSpectrumEmission | 1 |  |  |
| carrierFreq-v9e0 SEQUENCE { |  |  | Band > 64 |
| dl-CarrierFreq-v9e0 | Same downlink EARFCN as used for Cell 1 |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| FDD | FDD cell environment |
| TDD | TDD cell environment |
| Band > 64 | If band > 64 is selected |
| Band 24 High range | If Band 24 high frequency range is selected for the target cell |

<<Unchanged Text Skipped>>

------------------------------------------------------------- NEXT CHANGE ----------------------------------------------------------

#### 8.6.5.1	Radio Link Failure logging / Reporting at UTRAN Inter-RAT handover

<<Unchanged Text Skipped>>

8.6.5.1.3.3	Specific message contents

Table 8.6.5.1.3.3-0: Conditions for specific message contents
in Tables 8.6.5.1.3.3-2, 8.6.5.1.3.3-8 and 8.6.5.1.3.3-12

| Condition | Explanation |
| --- | --- |
| Band > 64 | If band > 64 is selected |
| Band 24 High range | If Band 24 high frequency range is selected for the target cell |

<<Unchanged Text Skipped>>

Table 8.6.5.1.3.3-8: MobilityControlInfo (Table 8.6.5.1.3.3-7)

| Derivation Path: 36.508, Table 4.6.5-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MobilityControlInfo ::= SEQUENCE { |  |  |  |
| targetPhysCellId | PhysicalCellIdentity of Cell 1. |  |  |
| carrierFreq SEQUENCE { |  |  |  |
| dl-CarrierFreq | Same downlink EARFCN as used for Cell 1. |  |  |
| ul-CarrierFreq | Not present |  |  |
|  | Same uplink EARFCN as used for Cell 1. |  | Band 24 High range |
| } |  |  |  |
| carrierFreq | Not present |  | Band > 64 |
| carrierBandwidth SEQUENCE { |  |  |  |
| dl-Bandwidth | Same downlink system bandwidth as used for Cell 1 |  |  |
| ul-Bandwidth | Same uplink system bandwidth as used for Cell 1 |  | FDD |
| ul-Bandwidth | Not present |  | TDD |
| } |  |  |  |
| additionalSpectrumEmission | 1 |  |  |
| carrierFreq-v9e0 SEQUENCE { |  |  | Band > 64 |
| dl-CarrierFreq-v9e0 | Same downlink EARFCN as used for Cell 1 |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| FDD | FDD cell environment |
| TDD | TDD cell environment |

<<Unchanged Text Skipped>>

------------------------------------------------------------- NEXT CHANGE ----------------------------------------------------------

#### 8.6.5.2	Radio Link Failure logging / Reporting at GERAN Inter-RAT handover

<<Unchanged Text Skipped>>

8.6.5.2.3.3	Specific message contents

Table 8.6.5.2.3.3-0: Conditions for specific message contents
in Tables 8.6.5.2.3.3-2, 8.6.5.2.3.3-5 and 8.6.5.2.3.3-9

| Condition | Explanation |
| --- | --- |
| Band > 64 | If band > 64 is selected |
| Band 24 High range | If Band 24 high frequency range is selected for the target cell |

<<Unchanged Text Skipped>>

Table 8.6.5.2.3.3-5: MobilityControlInfo (Table 8.6.5.2.3.3-4)

| Derivation Path: 36.508, Table 4.6.5-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MobilityControlInfo ::= SEQUENCE { |  |  |  |
| targetPhysCellId | PhysicalCellIdentity of Cell 1. |  |  |
| carrierFreq SEQUENCE { |  |  |  |
| dl-CarrierFreq | Same downlink EARFCN as used for Cell 1. |  |  |
| ul-CarrierFreq | Not present |  |  |
|  | Same uplink EARFCN as used for Cell 1. |  | Band 24 High range |
| } |  |  |  |
| carrierFreq | Not present |  | Band > 64 |
| carrierBandwidth SEQUENCE { |  |  |  |
| dl-Bandwidth | Downlink system bandwidth under test. |  |  |
| ul-Bandwidth | Uplink Bandwidth under test. |  | FDD |
|  | Not present |  | TDD |
| } |  |  |  |
| additionalSpectrumEmission | 1 |  |  |
| carrierFreq-v9e0 SEQUENCE { |  |  | Band > 64 |
| dl-CarrierFreq-v9e0 | Same downlink EARFCN as used for Cell 1 |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| FDD | FDD cell environment |
| TDD | TDD cell environment |

<<Unchanged Text Skipped>>

------------------------------------------------------------- NEXT CHANGE ----------------------------------------------------------

#### 13.4.1.3	Intra-system mobility / E-UTRA FDD to E-UTRA TDD to E-UTRA FDD packet

<<Unchanged Text Skipped>>

13.4.1.3.3.3	Specific message contents

Table 13.4.1.3.3.3-0: Conditions for specific message contents
in Tables 13.4.1.3.3.3-3, 13.4.1.3.3.3-7 and 13.4.1.2.3.3-10

| Condition | Explanation |
| --- | --- |
| Band > 64 | If band > 64 is selected |
| Band 24 High range | If Band 24 high frequency range is selected for the target cell |

<<Unchanged Text Skipped>>

Table 13.4.1.3.3.3-10: MobilityControlInfo (step 13, Table 13.4.1.3.3.2-2)

| Derivation Path: 36.508 clause 4.6.5-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MobilityControlInfo ::= SEQUENCE { |  |  |  |
| targetPhysCellId | PhysicalCellIdentity of Cell 1 |  |  |
| carrierFreq SEQUENCE { |  |  |  |
| dl-CarrierFreq | Same downlink EARFCN as used for Cell 1 |  |  |
| ul-CarrierFreq | Not present |  |  |
|  | Same uplink EARFCN as used for Cell 1 |  | Band 24 High range |
| } |  |  |  |
| carrierFreq | Not present |  | Band > 64 |
| carrierFreq-v9e0 SEQUENCE { |  |  | Band > 64 |
| dl-CarrierFreq-v9e0 | Same downlink EARFCN as used for Cell 1 |  |  |
| } |  |  |  |
| } |  |  |  |

<<Unchanged Text Skipped>>

------------------------------------------------------------- NEXT CHANGE ----------------------------------------------------------

#### 13.4.1.4	Inter-band mobility / E-UTRA to E-UTRA packet

<<Unchanged Text Skipped>>

13.4.1.4.3.3	Specific message contents

Table 13.4.1.4.3.3-0: Conditions for specific message contents
in Tables 13.4.1.4.3.3-2 and 13.4.1.4.3.3-5

| Condition | Explanation |
| --- | --- |
| Band > 64 | If band > 64 is selected |
| Band 24 High range | If Band 24 high frequency range is selected for the target cell |

<<Unchanged Text Skipped>>

Table 13.4.1.4.3.3-5: MobilityControlInfo (Table 13.4.1.4.3.3-4)

| Derivation Path: 36.508, Table 4.6.5-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MobilityControlInfo ::= SEQUENCE { |  |  |  |
| targetPhysCellId | PhysicalCellIdentity of Cell 10 |  |  |
| carrierFreq SEQUENCE { |  |  |  |
| dl-CarrierFreq | Same downlink EARFCN as used for Cell 10 |  |  |
| ul-CarrierFreq | Not present |  |  |
|  | Same uplink EARFCN as used for Cell 10 |  | Band 24 High range |
| } |  |  |  |
| carrierFreq | Not present |  | Band > 64 |
| carrierFreq-v9e0 SEQUENCE { |  |  | Band > 64 |
| dl-CarrierFreq-v9e0 | Same downlink EARFCN as used for Cell 10 |  |  |
| } |  |  |  |
| } |  |  |  |

<<Unchanged Text Skipped>>

------------------------------------------------------------- NEXT CHANGE ----------------------------------------------------------

#### 13.4.1.5	RRC connection reconfiguration / Handover/ Full configuration / DRB establishment

<<Unchanged Text Skipped>>

13.4.1.5.3.3	Specific message contents

<<Unchanged Text Skipped>>

Table 13.4.1.5.3.3-4: MobilityControlInfo (Table 13.4.1.5.3.3-3)

| Derivation Path: 36.508, Table 4.6.5-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MobilityControlInfo ::= SEQUENCE { |  |  |  |
| targetPhysCellId | PhysicalCellIdentity of Cell 2 |  |  |
| carrierFreq SEQUENCE { |  |  |  |
| dl-CarrierFreq | Same downlink EARFCN as used for Cell 2 |  |  |
| ul-CarrierFreq | Not present |  |  |
|  | Same uplink EARFCN as used for Cell 2 |  | Band 24 High range |
| } |  |  |  |
| carrierFreq | Not present |  | Band > 64 |
| radioResourceConfigCommon | RadioResourceConfigCommon-DEFAULT |  |  |
| carrierFreq-v9e0 SEQUENCE { |  |  | Band > 64 |
| dl-CarrierFreq-v9e0 | Same downlink EARFCN as used for Cell 2 |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| Band > 64 | If band > 64 is selected |
| Band 24 High range | If Band 24 high frequency range is selected for the target cell |

<<Unchanged Text Skipped>>

------------------------------------------------------------- NEXT CHANGE ----------------------------------------------------------

#### 13.4.2.6	Inter-RAT PS Handover / from GPRS Packet_transfer to E-UTRA cell

<<Unchanged Text Skipped>>

13.4.2.6.3.3	Specific message or IE contents

<<Unchanged Text Skipped>>

Table 13.4.2.6.3.3-2: MobilityControlInfo (Table 13.4.2.6.3.3-1)

| Derivation Path: 36.508, Table 4.6.5-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MobilityControlInfo |  |  |  |
| targetPhysCellId | PhysicalCellIdentity of Cell 1. |  |  |
| carrierFreq |  |  |  |
| dl-CarrierFreq | Same downlink EARFCN as used for Cell 1. |  |  |
| ul-CarrierFreq | Not present |  |  |
|  | Same uplink EARFCN as used for Cell 1. |  | Band 24 High range |
| carrierFreq | Not present |  | Band > 64 |
| carrierBandwidth |  |  |  |
| dl-Bandwidth | Downlink system bandwidth under test. |  |  |
| ul-Bandwidth | Uplink Bandwidth under test. |  | FDD |
| ul-Bandwidth | Not present |  | TDD |
| additionalSpectrumEmission | 1 |  |  |
| carrierFreq-v9e0 |  |  | Band > 64 |
| dl-CarrierFreq-v9e0 | Same downlink EARFCN as used for Cell 1 |  |  |

| Condition | Explanation |
| --- | --- |
| FDD | FDD cell environment |
| TDD | TDD cell environment |
| Band > 64 | If band > 64 is selected |
| Band 24 High range | If Band 24 high frequency range is selected for the target cell |

<<Unchanged Text Skipped>>

------------------------------------------------------------- NEXT CHANGE ----------------------------------------------------------

#### 13.4.2.7	Inter-RAT PS Handover / Synchronised / From GPRS Packet_transfer to E-UTRA cell (CCN mode)

<<Unchanged Text Skipped>>

13.4.2.7.3.3	Specific message or IE contents

<<Unchanged Text Skipped>>

Table 13.4.2.7.3.3-4: MobilityControlInfo (Table 13.4.2.7.3.3-3)

| Derivation Path: 36.508, Table 4.6.5-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MobilityControlInfo |  |  |  |
| targetPhysCellId | PhysicalCellIdentity of Cell 1. |  |  |
| carrierFreq |  |  |  |
| dl-CarrierFreq | Same downlink EARFCN as used for Cell 1. |  |  |
| ul-CarrierFreq | Not present |  |  |
|  | Same uplink EARFCN as used for Cell 1. |  | Band 24 High range |
| carrierFreq | Not present |  | Band > 64 |
| carrierBandwidth |  |  |  |
| dl-Bandwidth | Downlink system bandwidth under test. |  |  |
| ul-Bandwidth | Uplink Bandwidth under test. |  | FDD |
| ul-Bandwidth | Not present |  | TDD |
| additionalSpectrumEmission | 1 |  |  |
| carrierFreq-v9e0 |  |  | Band > 64 |
| dl-CarrierFreq-v9e0 | Same downlink EARFCN as used for Cell 1 |  |  |

| Condition | Explanation |
| --- | --- |
| FDD | FDD cell environment |
| TDD | TDD cell environment |
| Band > 64 | If band > 64 is selected |
| Band 24 High range | If Band 24 high frequency range is selected for the target cell |

<<Unchanged Text Skipped>>

------------------------------------------------------------- NEXT CHANGE ----------------------------------------------------------

#### 13.4.2.8	Inter-RAT PS Handover / Synchronised / From GPRS Packet_transfer to  E-UTRA cell (NC2 mode)

<<Unchanged Text Skipped>>

13.4.2.8.3.3	Specific message or IE contents

<<Unchanged Text Skipped>>

Table 13.4.2.8.3.3-3: MobilityControlInfo (Table 13.4.2.8.3.3-2)

| Derivation Path: 36.508, Table 4.6.5-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MobilityControlInfo |  |  |  |
| targetPhysCellId | PhysicalCellIdentity of Cell 1. |  |  |
| carrierFreq |  |  |  |
| dl-CarrierFreq | Same downlink EARFCN as used for Cell 1. |  |  |
| ul-CarrierFreq | Not present |  |  |
|  | Same uplink EARFCN as used for Cell 1. |  | Band 24 High range |
| carrierFreq | Not present |  | Band > 64 |
| carrierBandwidth |  |  |  |
| dl-Bandwidth | Downlink system bandwidth under test. |  |  |
| ul-Bandwidth | Uplink Bandwidth under test. |  | FDD |
| ul-Bandwidth | Not present |  | TDD |
| additionalSpectrumEmission | 1 |  |  |
| carrierFreq-v9e0 |  |  | Band > 64 |
| dl-CarrierFreq-v9e0 | Same downlink EARFCN as used for Cell 1 |  |  |

| Condition | Explanation |
| --- | --- |
| FDD | FDD cell environment |
| TDD | TDD cell environment |
| Band > 64 | If band > 64 is selected |
| Band 24 High range | If Band 24 high frequency range is selected for the target cell |

<<Unchanged Text Skipped>>

------------------------------------------------------------- NEXT CHANGE ----------------------------------------------------------

### 21.1.3	SC-MCCH information acquisition/ UE handover to a cell broadcasting SIB20

<<Unchanged Text Skipped>>

21.1.3.3.3	Specific message contents

Table 21.1.2.3.3-0: Conditions for specific message contents
in Tables 21.1.2.3.3-3 and Table 21.1.3.3.3-6

| Condition | Explanation |
| --- | --- |
| Band > 64 | This condition applies if the band number is bigger than 64. |
| Band 24 High range | If Band 24 high frequency range is selected for the target cell |

<<Unchanged Text Skipped>>

Table 21.1.3.3.3-6: MobilityControlInfo (Table 21.1.3.3.3-5)

| Derivation Path: 36.508, Table 4.6.5-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MobilityControlInfo ::= SEQUENCE { |  |  |  |
| targetPhysCellId | PhysicalCellIdentity of Cell 2 |  |  |
| carrierFreq SEQUENCE { |  |  |  |
| dl-CarrierFreq | Same downlink EARFCN as used for Cell 2 |  |  |
| ul-CarrierFreq | Not present |  |  |
|  | Same uplink EARFCN as used for Cell 2 |  | Band 24 High range |
| } |  |  |  |
| carrierFreq | Not present |  | Band > 64 |
| carrierFreq-v9e0 SEQUENCE { |  |  | Band > 64 |
| dl-CarrierFreq-v9e0 | Same downlink EARFCN as used for Cell 2 |  |  |
| } |  |  |  |
| } |  |  |  |

<<Unchanged Text Skipped>>

------------------------------------------------------------- END OF CHANGES ------------------------------------------------------