3GPP TSG-RAN WG5 Meeting #93-e	R5-216664

# Online, , 8th November 2021 - 19th November 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.579-1 | CR | 0188 | rev | - | Current version: | 15.3.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of clause 5.5.2.12 - SIP REFER |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MCC TF160 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI14_Test, MCImp-UEConTest |  |  |  |  |  | Date: |  |  | 2021-10-22 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | - 24.282 only specifies SDS for MCData and the P-Preferred-Service header field shall be set to "urn:urn-7:3gpp-service.ims.icsi.mcdata.sds" - MCPTT condition missing |  |  |  |  |  |  |  |  |
| Summary of change: |  | - P-Preferred-Service corrected - MCPTT condition added |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect test specification |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.5.2.12 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

#### 5.5.2.12	SIP REFER

This message is sent by the UE outside of a dialog.

Table 5.5.2.12-1: SIP REFER

| Derivation Path: TS 24.229 [16] clause A.2.1.4.11, A.2.2.4.11 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Information Element |  | Value/remark |  | Comment |  | Reference |  | Condition |  |
| Request-Line |  |  |  |  |  | RFC 3261 [22] RFC 5031 [54] |  |  |  |
| Method |  | "REFER" |  |  |  |  |  |  |  |
| Request-URI |  | tsc_MCX_SessionID_B |  | session identity of the pre-established session |  |  |  |  |  |
| SIP-Version |  | "SIP/2.0" |  |  |  |  |  |  |  |
| Via |  |  |  |  |  | RFC 3261 [22] RFC 3581 [55] |  |  |  |
| sent-protocol |  | "SIP/2.0/UDP" |  |  |  |  |  | UDP |  |
|  |  | "SIP/2.0/TCP" |  |  |  |  |  | TCP |  |
| sent-by |  |  |  |  |  |  |  |  |  |
| host |  | IP address or FQDN |  | Either the UE’s IP address or its home domain name |  |  |  |  |  |
| port |  | protected server port of the UE |  |  |  |  |  |  |  |
| via-branch |  | Value starting with 'z9hG4bK' |  |  |  |  |  |  |  |
| Route |  |  |  |  |  | RFC 3261 [22] |  |  |  |
| addr-spec[1] |  | SIP URI |  |  |  |  |  |  |  |
| user-info and host |  | P-CSCF address of the SS |  | P-CSCF address as assigned to the UE via NAS signalling or P-CSCF discovery |  |  |  |  |  |
| port |  | protected server port of the SS |  | as assigned during registration |  |  |  |  |  |
| uri-parameters |  | "lr" |  |  |  |  |  |  |  |
| addr-spec[2] |  | SIP URI |  |  |  |  |  |  |  |
| user-info and host |  | "scscf.3gpp.org" |  |  |  |  |  |  |  |
| port |  | not present |  |  |  |  |  |  |  |
| uri-parameters |  | "lr" |  |  |  |  |  |  |  |
| From |  |  |  |  |  | RFC 3261 [22] |  |  |  |
| addr-spec |  |  |  |  |  |  |  |  |  |
| user-info and host |  | Default public user id (px_MCX_SIP_PublicUserId_A_1) |  |  |  |  |  |  |  |
| port |  | not present |  |  |  |  |  |  |  |
| tag |  | any allowed value |  |  |  |  |  |  |  |
| To |  |  |  |  |  | RFC 3261 [22] RFC 5031 [54] |  |  |  |
| addr-spec |  |  |  |  |  |  |  |  |  |
| user-info and host |  | Same URI as used in the INVITE creating the pre-established session |  |  |  |  |  |  |  |
| port |  | not present |  |  |  |  |  |  |  |
| tag |  | not present |  |  |  |  |  |  |  |
| Call-ID |  |  |  |  |  | RFC 3261 [22] |  |  |  |
| callid |  | any allowed value |  |  |  |  |  |  |  |
| CSeq |  |  |  |  |  | RFC 3261 [22] |  |  |  |
| value |  | any allowed value |  |  |  |  |  |  |  |
| method |  | "REFER" |  |  |  |  |  |  |  |
| Supported |  |  |  |  |  | RFC 3261 [22] RFC 6442 [62] RFC 4488 [36] |  |  |  |
| option-tag |  | "norefersub" |  |  |  |  |  |  |  |
| Refer-Sub |  |  |  |  |  | RFC 4488 [36] |  |  |  |
| refer-sub-value |  | "false" |  |  |  |  |  |  |  |
| Target-Dialog |  |  |  |  |  | RFC 4538 [37] |  |  |  |
| callid |  | Callid of the pre-established session |  | Callid as used by the UE in the INVITE for establishment of the pre-established session |  |  |  |  |  |
| Require |  |  |  |  |  | RFC 3261 [22] RFC 3312 [56] RFC 3329 [53] |  |  |  |
| option-tag |  | "sec-agree" |  |  |  |  |  |  |  |
| option-tag |  | "multiple-refer" |  |  |  |  |  |  |  |
| Proxy-Require |  |  |  |  |  | RFC 3261 [22] RFC 3329 [53] |  |  |  |
| option-tag |  | "sec-agree" |  |  |  |  |  |  |  |
| Security-Verify |  |  |  |  |  | RFC 3329 [53] |  |  |  |
| sec-mechanism |  | same value as Security -Server header sent by SS during registration |  |  |  |  |  |  |  |
| Contact |  |  |  |  |  | RFC 3261 [22 RFC 3840 [33] |  |  |  |
| addr-spec |  | SIP URI |  |  |  |  |  |  |  |
| user-info and host |  | IP address or FQDN |  |  |  |  |  |  |  |
| feature-param |  | "+g.3gpp.mcptt" |  | This media feature tag when used in a SIP request or a SIP response indicates that the function sending the SIP message supports Mission Critical Push To Talk (MCPTT) communication. |  |  |  | MCPTT |  |
|  |  | "+g.3gpp.mcvideo" |  | This media feature tag when used in a SIP request or a SIP response indicates that the function sending the SIP message supports Mission Critical Video (MCVideo) communication. |  |  |  | MCVIDEO |  |
|  |  | "+g.3gpp.mcdata.sds" |  | This media feature tag when used in a SIP request or a SIP response indicates that the function sending the SIP message supports Mission Critical Data (MCData) communication. |  |  |  | MCDATA |  |
| feature-param |  | "+g.3gpp.icsi-ref=urn:urn-7:3gpp-service.ims.icsi.mcptt" |  | This URN indicates that the device has the capabilities to support the mission critical push to talk (MCPTT) service. |  |  |  | MCPTT |  |
|  |  | "+g.3gpp.icsi-ref=urn:urn-7:3gpp-service.ims.icsi.mcvideo" |  | This URN indicates that the device has the capabilities to support the mission critical video (MCVideo) service. |  |  |  | MCVIDEO |  |
|  |  | "+g.3gpp.icsi-ref=urn:urn-7:3gpp-service.ims.icsi.mcdata.sds" |  | This URN indicates that the device has the capabilities to support the mission critical data (MCData) service. |  |  |  | MCDATA |  |
| feature-param |  | "audio" |  | This feature tag indicates that the device supports audio as a streaming media type. |  |  |  | MCPTT OR MCVIDEO |  |
| feature-param |  | "video" |  | This feature tag indicates that the device supports video as a streaming media type. |  |  |  | MCVIDEO |  |
| feature-param |  | "text" |  | This feature tag indicates that the device supports text as a streaming media type. |  |  |  | MCDATA |  |
| Refer-To |  |  |  |  |  | RFC 3515 [38] |  |  |  |
| addr-spec |  | a Content-ID ("cid") Uniform Resource Locator (URL) as specified in IETF RFC 2392 that points to an application/resource-lists+xml MIME body as specified in IETF RFC 5366 |  |  |  |  |  |  |  |
| Refer-To |  |  |  |  |  | RFC 3515 [38] |  | METHOD-BYE |  |
| addr-spec |  |  |  |  |  |  |  |  |  |
| user-info and host |  | tsc_MCX_SessionID_B |  | The session identity of the pre-established session to leave. |  |  |  |  |  |
| uri-parameters |  |  |  |  |  |  |  |  |  |
| id[1] |  | method |  |  |  |  |  |  |  |
| value[1] |  | "BYE" |  |  |  |  |  |  |  |
| Max-Forwards |  |  |  |  |  | RFC 3261 [22] |  |  |  |
| value |  | any allowed value |  | Non-zero value |  |  |  |  |  |
| P-Access-Network-Info |  |  |  |  |  | RFC 7315 [52] |  |  |  |
| access-net-specs |  | Access network technology and, if applicable, the cell ID |  |  |  |  |  |  |  |
| P-Preferred-Service |  |  |  |  |  | RFC 6050 [31] |  |  |  |
| Service-ID |  | "urn:urn-7:3gpp-service.ims.icsi.mcptt" |  |  |  |  |  | MCPTT |  |
|  |  | "urn:urn-7:3gpp-service.ims.icsi.mcvideo" |  |  |  |  |  | MCVIDEO |  |
|  |  | "urn:urn-7:3gpp-service.ims.icsi.mcdata.sds" |  |  |  |  |  | MCDATA |  |
| P-Preferred-Identity |  | If present |  |  |  | RFC 3325 [32] |  |  |  |
| PPreferredID-value |  | same URI as in From-header |  |  |  |  |  |  |  |
| Resource-Priority |  |  |  |  |  | RFC 4412 [40] RFC 7134 [57] RFC 8101 [45] TS 24.379 [9] clause 6.2.8.1.15 |  | EMERGENCY-CALL AND (GROUP-CALL OR PRIVATE-CALL) |  |
| r-value |  |  |  |  |  |  |  |  |  |
| namespace |  | value of the <resource-priority-namespace> element contained in the <emergency-resource-priority> element contained in the <OnNetwork> element of the MCX service configuration documents |  | As configured in Table 5.5.8.4-1 |  | TS 24.484 [14] |  |  |  |
| r-priority |  | value of the <resource-priority-priority> element contained in the <emergency-resource-priority> element contained in the <OnNetwork> element of the MCX service configuration document |  | As configured in Table 5.5.8.4-1 |  | TS 24.484 [14] |  |  |  |
| Resource-Priority |  |  |  |  |  | RFC 4412 [40] RFC 7134 [57] RFC 8101 [45] TS 24.379 [9] clause 6.2.8.1.15 |  | IMMPERIL-CALL AND (GROUP-CALL OR PRIVATE-CALL) |  |
| r-value |  |  |  |  |  |  |  |  |  |
| namespace |  | value of the <resource-priority-namespace> element contained in the <imminent-peril-resource-priority> element contained in the <OnNetwork> element of the MCX service configuration documents |  | As configured in Table 5.5.8.4-1 |  | TS 24.484 [14] |  |  |  |
| r-priority |  | value of the <resource-priority-priority> element contained in the <imminent-peril-resource-priority> element contained in the <OnNetwork> element of the MCX service configuration document |  | As configured in Table 5.5.8.4-1 |  | TS 24.484 [14] |  |  |  |
| Content-Type |  | not present |  |  |  |  |  | METHOD-BYE |  |
| Content-Type |  |  |  |  |  | RFC 5621 [58] |  |  |  |
| media-type |  | "multipart/mixed" |  |  |  |  |  |  |  |
| Content-Length |  | present in case of TCP and when there is a message body (otherwise optional) |  |  |  | RFC 3261 [22] |  |  |  |
| Value |  | any value |  | length of message-body |  |  |  |  |  |
| Message-body |  | not present |  |  |  |  |  | METHOD-BYE |  |
| Message-body |  |  |  |  |  | RFC 3261 [22] |  |  |  |
| MIME body part |  |  |  | Resource list |  | RFC 5366 [35] |  |  |  |
| MIME-part-headers |  |  |  |  |  |  |  |  |  |
| Content-Type |  | "application/resource-lists+xml" |  |  |  |  |  |  |  |
| Content-ID |  | same value as the cid URL in the Refer-To header field |  | Unique URL identifying the Resource-lists XML MIME body; used as reference in the signature MIME body too |  | TS 24.379 [9] clause 6.6.3.1 |  |  |  |
| MIME-part-body |  | Resource-lists as described in Table 5.5.3.3.1-1 with condition PRE-ESTABLISH and the uri attribute of the single <entry> element extended with the headers of Table 5.5.2.12-2 |  |  |  |  |  | MCPTT |  |
|  |  | Resource-lists as described in Table 5.5.3.3.1-2 |  |  |  |  |  | MCVIDEO |  |
|  |  | Resource-lists as described in Table 5.5.3.3.1-3 |  |  |  |  |  | MCDATA |  |
| MIME body part |  |  |  | Location info |  |  |  | LOCATION-INFO |  |
| MIME-part-headers |  |  |  |  |  |  |  |  |  |
| Content-Type |  | "application/vnd.3gpp.mcptt-location-info+xml" |  |  |  |  |  | MCPTT |  |
|  |  | "application/vnd.3gpp.mcvideo-location-info+xml" |  |  |  |  |  | MCVIDEO |  |
| Content-ID |  | any value |  | Unique URL identifying the Location-info XML MIME body; used as reference in the signature MIME body |  | TS 24.379 [9] clause 6.6.3.1 |  |  |  |
| MIME-part-body |  | Location-info as described in Table 5.5.3.4.1-1 |  |  |  | TS 24.379 [9] clause F.3 |  | MCPTT |  |
|  |  | Location-info as described in Table 5.5.3.4.1-2 |  |  |  | TS 24.281 [86] clause F.3 |  | MCVIDEO |  |
| MIME body part |  |  |  | Signature |  |  |  |  |  |
| MIME-part-headers |  |  |  |  |  |  |  |  |  |
| Content-Type |  | "application/vnd.3gpp.mcptt-signed+xml" |  |  |  | TS 24.379 [9] |  |  |  |
| MIME-part-body |  | Signatures for XML MIME bodies as described in Table 5.5.13.1-1 |  |  |  | TS 24.379 [9] |  |  |  |

Table 5.5.2.12-2: SIP header fields extending the uri attribute of the resource-lists' single entry

| Derivation Path: TS 24.379 [9] clause 10.1.1.2.2.1, 10.1.2.2.2.1, 11.1.1.2.2.1, 11.1.6.2.2.1 Editor's note: references for MCVIDEO and MCDATA to be added |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Accept-Contact |  |  | RFC 3841 [29] | GROUP-CALL OR CHAT-GROUP-CALL |
| ac-value[1] |  |  |  |  |
| feature-param | "+g.3gpp.icsi-ref=urn:urn-7:3gpp-service.ims.icsi.mcptt" |  |  | MCPTT |
|  | "+g.3gpp.icsi-ref=urn:urn-7:3gpp-service.ims.icsi.mcvideo" |  |  | MCVIDEO |
|  | "+g.3gpp.icsi-ref=urn:urn-7:3gpp-service.ims.icsi.mcdata.sds" |  |  | MCDATA |
| req-param | "require" |  |  |  |
| explicit-param | "explicit" |  |  |  |
| ac-value[2] |  |  |  |  |
| feature-param | "+g.3gpp.mcptt" |  |  | MCPTT |
|  | "+g.3gpp.mcvideo" |  |  | MCVIDEO |
|  | "+g.3gpp.mcdata.sds" |  |  | MCDATA |
| req-param | "require" |  |  |  |
| explicit-param | "explicit" |  |  |  |
| Answer-Mode | not present |  |  |  |
| Answer-Mode |  |  | RFC 5373 [34] TS 24.379 [9] cl. 11.1.1.2.2.1, 8) | PRIVATE-CALL AND (NOT FORCE) |
| answer-mode-value | "Auto" |  |  |  |
| answer-mode-value | "Manual" |  |  | MANUAL |
| Priv-Answer-Mode | not present |  |  |  |
| Priv-Answer-Mode |  |  | RFC 5373 [34] TS 24.379 [9] clause 11.1.1.2.2.1, 8) and clause 11.1.6.2.2.1, 8) | PRIVATE-CALL AND FORCE |
| answer-mode-value | "Auto" | if force of automatic commencement mode at the invited MCPTT client is requested by the MCPTT user, |  |  |
| Content-Type |  |  | RFC 5621 [58] |  |
| media-type | "multipart/mixed" |  |  |  |
| body |  | NOTE: Characters that are not formatted as ASCII characters are escaped in the following parameters in the headers portion of the SIP URI. | RFC 3261 [22] |  |
| MIME body part |  | MCPTT/MCVideo/MCData Info |  |  |
| MIME-part-headers |  |  |  |  |
| Content-Type | "application/vnd.3gpp.mcptt-info+xml" |  |  | MCPTT |
|  | "application/vnd.3gpp.mcvideo-info+xml" |  |  | MCVIDEO |
|  | "application/vnd.3gpp.mcdata-info+xml" |  |  | MCDATA |
| Content-ID | any value | Unique URL identifying the MCPTT/MCVideo/MCData Info XML MIME body; used as reference in the signature MIME body | TS 24.379 [9] clause 6.6.3.1 |  |
| MIME-part-body | MCPTT-Info as described in Table 5.5.3.2.1-1 |  | TS 24.379 [9] clause F.1 | MCPTT |
|  | MCVideo-Info as described in Table 5.5.3.2.1-2 |  | TS 24.281 [86] clause F.1 | MCVIDEO |
|  | MCData-Info as described in Table 5.5.3.2.1-3 |  | TS 24.282 [87] clause D.1 | MCDATA |
| MIME body part |  | Location info |  | (MCPTT OR MCVIDEO) AND ALLOW-LOCATION-INFO |
| MIME-part-headers |  |  |  |  |
| Content-Type | “application/vnd.3gpp.mcptt-location-info+xml” |  |  | MCPTT |
|  | “application/vnd.3gpp.mcvideo-location-info+xml” |  |  | MCVIDEO |
| Content-ID | any value | Unique URL identifying the Location-info XML MIME body; used as reference in the signature MIME body | TS 24.379 [9] clause 6.6.3.1 |  |
| MIME-part-body | Location-info as described in Table 5.5.3.4.1-1 |  | TS 24.379 [9] clause F.3 | MCPTT |
|  | Location-info as described in Table 5.5.3.4.1-2 |  | TS 24.281 [86] clause F.3 | MCVIDEO |
| MIME body part |  | Signature |  |  |
| MIME-part-headers |  |  |  |  |
| Content-Type | “application/vnd.3gpp.mcptt-signed+xml” |  | TS 24.379 [9] |  |
| MIME-part-body | Signatures for XML MIME bodies as described in Table 5.5.13.1-1 |  | TS 24.379 [9] |  |

| Condition | Explanation |
| --- | --- |
| MANUAL | Call establishment with manual commencement mode |
| FORCE | force of automatic commencement mode at the invited MCPTT client is requested by the MCPTT user |
| ALLOW-LOCATION-INFO | Implicit floor control is requested AND <allow-location-info-when-talking> element of the <ruleset> element of the MCPTT user profile document set to “true” in TS 36.579-1 [2] Table 5.5.8.3-1 |
| For further conditions see table 5.5.1-1 |  |