3GPP TSG-RAN WG5 Meeting #93-e	R5-216678

# Online, , 8th November 2021 - 19th November 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.579-1 | CR | 0202 | rev | - | Current version: | 15.3.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of clause 5.5.3.4 - Location-info |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MCC TF160 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI14_Test, MCImp-UEConTest |  |  |  |  |  | Date: |  |  | 2021-10-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | - If location information is sent for MCPTT emergency alert, 24.379 clause 6.2.9.1 does not specify any TriggerId to be included in the location information. - As for MCPTT the TriggeringCriteria's GeographicalAreaChange is mandatory in location reporting configuration for MCVideo |  |  |  |  |  |  |  |  |
| Summary of change: |  | - TriggerID changed to "not present" in Table 5.5.3.4.4-1 - TriggeringCriteria added to Table 5.5.3.4.2-2 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Invalid test specification |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.5.3.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

#### 5.5.3.4	Location-info

##### 5.5.3.4.1	Location-info (Report from the UE)

##### -	MCPTT

Table 5.5.3.4.1-1: Location-info (Report from the UE) for MCPTT

| Derivation Path: TS 24.379 [9] clause F.3 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| location-info |  |  |  |  |
| Report |  |  |  |  |
| ReportID attribute | not present | Attribute is used to return the value in the <RequestId> attribute in the <Request> element. Only present in response to a Location-Info Request. |  |  |
| ReportType attribute | "Emergency" | Required The <ReportType> attribute has two values "Emergency" and "NonEmergency" used to inform whether the client is sending the report in an emergency situation or not. |  |  |
| TriggerID | not present | An element which can occur multiple times. Contains the value of the <TriggerId> attribute associated with a trigger that has fired. Only present if a trigger is the cause of the Location-info Report. |  |  |
| CurrentLocation |  | A mandatory element that contains the location information |  |  |
| CurrentServingEcgi | Encrypted (NOTE 2) <CurrentServingEcgi> with any content if present | This is optional depending on the configuration sent by the SS |  |  |
| NeighbouringEcgi | Encrypted (NOTE 2) <NeighbouringEcgi> with any content if present | This is optional depending on the configuration sent by the SS |  |  |
| MbmsSaId | Encrypted (NOTE 2) <MbmsSaId> with any content if present | This is optional depending on the configuration sent by the SS |  |  |
| MbsfnArea | Encrypted (NOTE 2) <MbsfnArea> with any content if present | This is optional depending on the configuration sent by the SS |  |  |
| CurrentCoordinate | if present | This is optional depending on the configuration sent by the SS |  |  |
| longitude | Encrypted (NOTE 1) <longitude> with any content |  |  |  |
| latitude | Encrypted (NOTE 1) <latitude> with any content |  |  |  |
| NOTE 1: Encrypted sub-element of <CurrentCoordinate> as described in Table 5.5.3.4.1-1A NOTE 2: Encrypted sub-element of <CurrentLocation> element as described in Table 5.5.3.4.1-1B |  |  |  |  |

Table 5.5.3.4.1-1A: Encrypted sub-element of <CurrentCoordinate> sent by the UE

| Derivation Path: TS 24.379 [9] clause F.3.2 (tCoordinateType ) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| type attribute | "Encrypted" |  |  |  |
| EncryptedData | EncryptedData as described in Table 5.5.13.2-1 containing encrypted element content of the sub-element of <CurrentCoordinate> |  |  |  |

Table 5.5.3.4.1-1B: Encrypted sub-element of <CurrentLocation> sent by the UE

| Derivation Path: TS 24.379 [9] clause F.3.2 (tCurrentLocationType) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| type attribute | "Encrypted" |  |  |  |
| EncryptedData | EncryptedData as described in Table 5.5.13.2-1 containing encrypted element content of the sub-element of <CurrentLocation> |  |  |  |

##### -	MCVideo

Table 5.5.3.4.1-2: Location-info (Report from the UE) for MCVideo

| Derivation Path: TS 24.281 [86] clause F.3 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| location-info |  |  |  |  |
| Report |  |  |  |  |
| ReportID | not present | Attribute is used to return the value in the <RequestId> attribute in the <Request> element. Only present in response to a Location-Info Request. |  |  |
| ReportType | "Emergency" | Required The <ReportType> attribute has two values "Emergency" and "NonEmergency" used to inform whether the client is sending the report in an emergency situation or not. |  |  |
| TriggerID | not present | An element which can occur multiple times. Contains the value of the <TriggerId> attribute associated with a trigger that has fired. Only present if a trigger is the cause of the Location-info Report. |  |  |
| CurrentLocation |  | A mandatory element that contains the location information |  |  |
| CurrentServingEcgi | Encrypted (NOTE 2) <CurrentServingEcgi> with any content if present | This is optional depending on the configuration sent by the SS |  |  |
| NeighbouringEcgi | Encrypted (NOTE 2) <NeighbouringEcgi> with any content if present | This is optional depending on the configuration sent by the SS |  |  |
| MbmsSaId | Encrypted (NOTE 2) <MbmsSaId> with any content if present | This is optional depending on the configuration sent by the SS |  |  |
| MbsfnArea | Encrypted (NOTE 2) <MbsfnArea> with any content if present | This is optional depending on the configuration sent by the SS |  |  |
| CurrentCoordinate | if present | This is optional depending on the configuration sent by the SS |  |  |
| longitude | Encrypted (NOTE 1) <longitude> with any content |  |  |  |
| latitude | Encrypted (NOTE 1) <latitude> with any content |  |  |  |
| NOTE 1: Encrypted sub-element of <CurrentCoordinate> as described in Table 5.5.3.4.1-2A NOTE 2: Encrypted sub-element of <CurrentLocation> element as described in Table 5.5.3.4.1-2B |  |  |  |  |

Table 5.5.3.4.1-2A: Encrypted sub-element of <CurrentCoordinate> sent by the UE

| Derivation Path: TS 24.281 [86] clause F.3.2 (tCoordinateType) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| type attribute | "Encrypted" |  |  |  |
| EncryptedData | EncryptedData as described in Table 5.5.13.2-1 containing encrypted element content of the sub-element of <CurrentCoordinate> |  |  |  |

Table 5.5.3.4.1-2B: Encrypted sub-element of <CurrentLocation> sent by the UE

| Derivation Path: TS 24.281 [86] clause F.3.2 (tCurrentLocationType) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| type attribute | "Encrypted" |  |  |  |
| EncryptedData | EncryptedData as described in Table 5.5.13.2-1 containing encrypted element content of the sub-element of <CurrentLocation> |  |  |  |

##### MCData

Table 5.5.3.4.1-3: Location-info (Report from the UE) for MCData

| Derivation Path: TS 24.282 [87] clause D.4 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| location-info |  |  |  |  |
| Report |  |  |  |  |
| ReportID | not present | Attribute is used to return the value in the <RequestId> attribute in the <Request> element. Only present in response to a Location-Info Request. |  |  |
| ReportType | "Emergency" | Required The <ReportType> attribute has two values "Emergency" and "NonEmergency" used to inform whether the client is sending the report in an emergency situation or not. |  |  |
| TriggerID | not present | An element which can occur multiple times. Contains the value of the <TriggerId> attribute associated with a trigger that has fired. Only present if a trigger is the cause of the Location-info Report. |  |  |
| CurrentLocation |  | A mandatory element that contains the location information |  |  |
| CurrentServingEcgi | Encrypted (NOTE 2) <CurrentServingEcgi> with any content if present | This is optional depending on the configuration sent by the SS |  |  |
| NeighbouringEcgi | Encrypted (NOTE 2) <NeighbouringEcgi> with any content if present | This is optional depending on the configuration sent by the SS |  |  |
| MbmsSaId | Encrypted (NOTE 2) <MbmsSaId> with any content if present | This is optional depending on the configuration sent by the SS |  |  |
| MbsfnArea | Encrypted (NOTE 2) <MbsfnArea> with any content if present | This is optional depending on the configuration sent by the SS |  |  |
| CurrentCoordinate | if present | This is optional depending on the configuration sent by the SS |  |  |
| longitude | Encrypted (NOTE 1) <longitude> with any content |  |  |  |
| latitude | Encrypted (NOTE 1) <latitude> with any content |  |  |  |
| NOTE 1: Encrypted sub-element of <CurrentCoordinate> as described in Table 5.5.3.4.1-2A NOTE 2: Encrypted sub-element of <CurrentLocation> element as described in Table 5.5.3.4.1-2B |  |  |  |  |

Table 5.5.3.4.1-3A: Encrypted sub-element of <CurrentCoordinate> sent by the UE

| Derivation Path: TS 24.282 [87] clause d.4.2 (tCoordinateType) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| type attribute | "Encrypted" |  |  |  |
| EncryptedData | EncryptedData as described in Table 5.5.13.2-1 containing encrypted element content of the sub-element of <CurrentCoordinate> |  |  |  |

Table 5.5.3.4.1-3B: Encrypted sub-element of <CurrentLocation> sent by the UE

| Derivation Path: TS 24.282 [87] clause D.4.2 (tCurrentLocationType) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| type attribute | "Encrypted" |  |  |  |
| EncryptedData | EncryptedData as described in Table 5.5.13.2-1 containing encrypted element content of the sub-element of <CurrentLocation> |  |  |  |

##### 5.5.3.4.2	Location-info (Configuration sent by the SS)

##### -	MCPTT

Table 5.5.3.4.2-1: Location-info (Configuration sent by the SS) for MCPTT

| Derivation Path: TS 24.379 [9] clause F.3 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| location-info |  |  |  |  |
| Configuration |  |  |  |  |
| ConfigScope | "Full" | The MCPTT Client shall replace any previous configuration. |  |  |
| NonEmergencyLocationInformation |  |  |  |  |
| ServingEcgi | present | An optional element specifying that the serving E-UTRAN Cell Global Identity (ECGI) needs to be reported |  |  |
| NeighbouringEcgi | present | An optional element that can occur multiple times, specifying that neighbouring ECGIs need to be reported |  |  |
| MbmsSaId | present | An optional element specifying that the serving MBMS Service Area Id needs to be reported; |  |  |
| MbsfnArea | present | An optional element specifying that the MBSFN area Id needs to be reported; |  |  |
| GeographicalCoordinate | present | An optional element specifying that the geographical coordinate specified in clause 6.1 in 3GPP TS 23.032 [65] needs to be reported |  |  |
| minimumIntervalLength | "10" | A mandatory element specifying the minimum time the MCPTT client needs to wait between sending location reports. The value is given in seconds |  |  |
| EmergencyLocationInformation" |  |  |  |  |
| ServingEcgi | present | An optional element specifying that the serving E-UTRAN Cell Global Identity (ECGI) needs to be reported |  |  |
| NeighbouringEcgi | present | An optional element that can occur multiple times, specifying that neighbouring ECGIs need to be reported |  |  |
| MbmsSaId | present | An optional element specifying that the serving MBMS Service Area Id needs to be reported; |  |  |
| MbsfnArea | present | An optional element specifying that the MBSFN area Id needs to be reported; |  |  |
| GeographicalCoordinate | present | An optional element specifying that the geographical coordinate specified in clause 6.1 in 3GPP TS 23.032 [65] needs to be reported |  |  |
| minimumIntervalLength | "5" | A mandatory element specifying the minimum time the MCPTT client needs to wait between sending location reports. The value is given in seconds |  |  |
| TriggeringCriteria |  |  |  |  |
| CellChange | not present |  |  |  |
| TrackingAreaChange | not present |  |  |  |
| PlmnChange | not present |  |  |  |
| MbmsSaChange | not present |  |  |  |
| MbsfnAreaChange | not present |  |  |  |
| PeriodicReport | not present |  |  |  |
| TravelledDistance | not present |  |  |  |
| McpttSignallingEvent | not present |  |  |  |
| GeographicalAreaChange |  |  |  |  |
| AnyAreaChange | not present |  |  |  |
| EnterSpecificAreaType | not present |  |  |  |
| ExitSpecificAreaType | not present |  |  |  |
| anyExt |  | mandatory for Rel-15 and above |  |  |
| EmergencyTriggeringCriteria |  |  |  |  |
| CellChange | not present |  |  |  |
| TrackingAreaChange | not present |  |  |  |
| PlmnChange | not present |  |  |  |
| MbmsSaChange | not present |  |  |  |
| MbsfnAreaChange | not present |  |  |  |
| PeriodicReport | not present |  |  |  |
| TravelledDistance | not present |  |  |  |
| McpttSignallingEvent | not present |  |  |  |
| GeographicalAreaChange |  |  |  |  |
| AnyAreaChange | not present |  |  |  |
| EnterSpecificAreaType | not present |  |  |  |
| ExitSpecificAreaType | not present |  |  |  |

##### -	MCVideo

Table 5.5.3.4.2-2: Location-info (Configuration sent by the SS) for MCVideo

| Derivation Path: TS 24.281 [86] clause F.3 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| location-info |  |  |  |  |
| Configuration |  |  |  |  |
| ConfigScope | "Full" | The MCVideo Client shall replace any previous configuration. |  |  |
| NonEmergencyLocationInformation |  |  |  |  |
| ServingEcgi | present | An optional element specifying that the serving E-UTRAN Cell Global Identity (ECGI) needs to be reported |  |  |
| NeighbouringEcgi | present | An optional element that can occur multiple times, specifying that neighbouring ECGIs need to be reported |  |  |
| MbmsSaId | present | An optional element specifying that the serving MBMS Service Area Id needs to be reported; |  |  |
| MbsfnArea | present | An optional element specifying that the MBSFN area Id needs to be reported; |  |  |
| GeographicalCoordinate | present | An optional element specifying that the geographical coordinate specified in clause 6.1 in 3GPP TS 23.032 [65] needs to be reported |  |  |
| minimumIntervalLength | "10" | A mandatory element specifying the minimum time the MCVIdeo client needs to wait between sending location reports. The value is given in seconds |  |  |
| EmergencyLocationInformation" |  |  |  |  |
| ServingEcgi | present | An optional element specifying that the serving E-UTRAN Cell Global Identity (ECGI) needs to be reported |  |  |
| NeighbouringEcgi | present | An optional element that can occur multiple times, specifying that neighbouring ECGIs need to be reported |  |  |
| MbmsSaId | present | An optional element specifying that the serving MBMS Service Area Id needs to be reported; |  |  |
| MbsfnArea | present | An optional element specifying that the MBSFN area Id needs to be reported; |  |  |
| GeographicalCoordinate | present | An optional element specifying that the geographical coordinate specified in clause 6.1 in 3GPP TS 23.032 [65] needs to be reported |  |  |
| minimumIntervalLength | "5" | A mandatory element specifying the minimum time the MCVideo client needs to wait between sending location reports. The value is given in seconds |  |  |
| TriggeringCriteria |  |  |  |  |
| CellChange | not present |  |  |  |
| TrackingAreaChange | not present |  |  |  |
| PlmnChange | not present |  |  |  |
| MbmsSaChange | not present |  |  |  |
| MbsfnAreaChange | not present |  |  |  |
| PeriodicReport | not present |  |  |  |
| TravelledDistance | not present |  |  |  |
| McvideoSignallingEvent | not present |  |  |  |
| GeographicalAreaChange |  |  |  |  |
| AnyAreaChange | not present |  |  |  |
| EnterSpecificAreaType | not present |  |  |  |
| ExitSpecificAreaType | not present |  |  |  |

##### -	MCData

Table 5.5.3.4.2-2: Location-info (Configuration sent by the SS) for MCData

| Derivation Path: TS 24.281 [86] clause F.3 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| location-info |  |  |  |  |
| Configuration |  |  |  |  |
| ConfigScope | "Full" | The MCData Client shall replace any previous configuration. |  |  |
| NonEmergencyLocationInformation |  |  |  |  |
| ServingEcgi | present | An optional element specifying that the serving E-UTRAN Cell Global Identity (ECGI) needs to be reported |  |  |
| NeighbouringEcgi | present | An optional element that can occur multiple times, specifying that neighbouring ECGIs need to be reported |  |  |
| MbmsSaId | present | An optional element specifying that the serving MBMS Service Area Id needs to be reported; |  |  |
| MbsfnArea | present | An optional element specifying that the MBSFN area Id needs to be reported; |  |  |
| GeographicalCoordinate | present | An optional element specifying that the geographical coordinate specified in clause 6.1 in 3GPP TS 23.032 [65] needs to be reported |  |  |
| minimumIntervalLength | "10" | A mandatory element specifying the minimum time the MCData client needs to wait between sending location reports. The value is given in seconds |  |  |
| EmergencyLocationInformation" |  |  |  |  |
| ServingEcgi | present | An optional element specifying that the serving E-UTRAN Cell Global Identity (ECGI) needs to be reported |  |  |
| NeighbouringEcgi | present | An optional element that can occur multiple times, specifying that neighbouring ECGIs need to be reported |  |  |
| MbmsSaId | present | An optional element specifying that the serving MBMS Service Area Id needs to be reported; |  |  |
| MbsfnArea | present | An optional element specifying that the MBSFN area Id needs to be reported; |  |  |
| GeographicalCoordinate | present | An optional element specifying that the geographical coordinate specified in clause 6.1 in 3GPP TS 23.032 [65] needs to be reported |  |  |
| minimumIntervalLength | "5" | A mandatory element specifying the minimum time the MCData client needs to wait between sending location reports. The value is given in seconds |  |  |
| TriggeringCriteria |  |  |  |  |
| CellChange | not present |  |  |  |
| TrackingAreaChange | not present |  |  |  |
| PlmnChange | not present |  |  |  |
| MbmsSaChange | not present |  |  |  |
| MbsfnAreaChange | not present |  |  |  |
| PeriodicReport | not present |  |  |  |
| TravelledDistance | not present |  |  |  |
| McdataSignallingEvent | not present |  |  |  |
| GeographicalAreaChange |  |  |  |  |
| AnyAreaChange | not present |  |  |  |
| EnterSpecificAreaType | not present |  |  |  |
| ExitSpecificAreaType | not present |  |  |  |

##### 5.5.3.4.3	Location-info (Request sent by the SS)

##### -	MCPTT

Table 5.5.3.4.3-1: Location-info (Request sent by the SS) for MCPTT

| Derivation Path: TS 24.379 [9] clause F.3 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| location-info |  |  |  |  |
| Request |  |  |  |  |
| RequestID | "1" | The RequestID that the MCPTT Client will reference in the Report |  |  |

##### -	MCVideo

Table 5.5.3.4.3-2: Location-info (Request sent by the SS) for MCVideo

| Derivation Path: TS 24.281 [96] clause F.3 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| location-info |  |  |  |  |
| Request |  |  |  |  |
| RequestID | "1" | The RequestID that the MCVideo Client will reference in the Report |  |  |

##### 5.5.3.4.4	Location-info (Report from the SS)

##### -	MCPTT

Table 5.5.3.4.4-1: Location-info (Report from the SS) for MCPTT

| Derivation Path: TS 24.379 [9] clause F.3 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| location-info |  |  |  |  |
| Report |  |  |  |  |
| ReportID attribute | not present |  |  |  |
| ReportType attribute | "Emergency" |  |  |  |
| TriggerID | not present | . |  |  |
| CurrentLocation |  |  |  |  |
| CurrentServingEcgi | not present |  |  |  |
| NeighbouringEcgi | not present |  |  |  |
| MbmsSaId | not present |  |  |  |
| MbsfnArea | not present |  |  |  |
| CurrentCoordinate |  |  |  |  |
| longitude | Encrypted (NOTE 1) <longitude> with content as specified by the test case |  |  |  |
| latitude | Encrypted (NOTE 1) <longitude> with content as specified by the test case |  |  |  |
| NOTE 1: Encrypted tCoordinateType element as described in Table 5.5.3.4.4-1A |  |  |  |  |

Table 5.5.3.4.4-1A: Encrypted sub-element of <CurrentCoordinate> sent by the SS

| Derivation Path: TS 24.379 [9] clause F.3.2 (tCoordinateType ) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| type attribute | "Encrypted" |  |  |  |
| EncryptedData | EncryptedData as described in Table 5.5.13.2-2 containing encrypted element content of the sub-element of <CurrentCoordinate> |  |  |  |