3GPP TSG-RAN WG5 Meeting #93-e	R5-216681

# Online, , 8th November 2021 - 19th November 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.579-1 | CR | 0205 | rev | - | Current version: | 15.3.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of clause 5.5.3.9 - MCData Data Payload |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MCC TF160 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI14_Test, MCImp-UEConTest |  |  |  |  |  | Date: |  |  | 2021-10-22 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | - In case of group communication the whole DATA PAYLOAD shall be protected with the GMK whereas for 1-to-1 communication there is a Security parameters and Payload IE carrying the encrypted payload - Editorial issues |  |  |  |  |  |  |  |  |
| Summary of change: |  | - New sub-clauses to clearly distinguish group and 1-to-1 communication; new tables for 1-to-1 communication to clarify encryption of the payload - Editorial corrections |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete test specification |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.5.3.9 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

#### 5.5.3.9	MCData Data Payload

##### 5.5.3.9.1	MCData Data Payload for group communication

The MCData Data Payload messages for group communication specified in this clause are protected according to TS 33.180 clause 8.5.4, i.e. a MCData Data Payload message is contained in the protected payload of a MCData Protected Payload Message according to clause 5.5.3.10 with condition PROTECTED_MESSAGE and GMK.

Table 5.5.3.9.1-1: DATA PAYLOAD message for group communication from the UE

| Derivation Path: TS 24.282 [87] clause 15.1.4 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Data payload message identity | '00000011'B | Data payload | TS 24.282 [87] clause 15.2.2 |  |
| Number of payloads | 1 | 1 payload | TS 24.282 [87] clause 15.2.12 |  |
| Payload |  |  | TS 24.282 [87] clause 15.2.13 |  |
| Payload IEI | '78'O |  |  |  |
| Length of Payload | length of the content |  |  |  |
| Payload content type | '00000001'B | TEXT |  |  |
| Payload data | any allowed value | The data payload Example: “abcdEFGH” |  |  |

Table 5.5.3.9.1-2: DATA PAYLOAD message for group communication from the SS

| Derivation Path: TS 24.282 [87] clause 15.1.4 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Data payload message identity | '00000011'B | Data payload | TS 24.282 [87] clause 15.2.2 |  |
| Number of payloads | 1 | 1 payload | TS 24.282 [87] clause 15.2.12 |  |
| Payload |  |  | TS 24.282 [87] clause 15.2.13 | MCD_grp |
| Payload IEI | '78'O |  |  |  |
| Length of Payload | length of the content |  |  |  |
| Payload content type | '00000001'B | TEXT |  |  |
| Payload data | "Test" | The data payload |  |  |

##### 5.5.3.9.2	MCData Data Payload for one-to-one communication

Table 5.5.3.9.2-1: DATA PAYLOAD message for one-to-one communication from the UE

| Derivation Path: TS 24.282 [87] clause 15.1.4 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Data payload message identity | '00000011'B | Data payload | TS 24.282 [87] clause 15.2.2 |  |
| Number of payloads | 1 | 1 payload | TS 24.282 [87] clause 15.2.12 |  |
| Security parameters and Payload | MCData Protected Payload Message as described in Table 5.5.3.10-1 with condition PROTECTED_PAYLOAD containing the Payload as described in Table 5.5.3.9.2-1A | MCData Protected Payload Message | TS 33.180 [94] |  |

Table 5.5.3.9.2-1A: Payload contained in the Security parameters and Payload

| Derivation Path: TS 24.282 [87] clause 15.2.13 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Field | Value/remark | Comment | Reference | Condition |
| Payload IEI | '78'O |  | TS 24.282 [87] clause 15.1.4 |  |
| Length of Payload | length of the content |  |  |  |
| Payload content type | '00000001'B | TEXT |  |  |
| Payload data | any allowed value | The data payload Example: “abcdEFGH” |  |  |

Table 5.5.3.9.2-2: DATA PAYLOAD message for one-to-one communication from the SS

| Derivation Path: TS 24.282 [87] clause 15.1.4 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Data payload message identity | '00000011'B | Data payload | TS 24.282 [87] clause 15.2.2 |  |
| Number of payloads | 1 | 1 payload | TS 24.282 [87] clause 15.2.12 |  |
| Security parameters and Payload | MCData Protected Payload Message as described in Table 5.5.3.10-2 with condition PROTECTED_PAYLOAD containing the Payload as described in Table 5.5.3.9.2-2A | MCData Protected Payload Message | TS 33.180 [94] |  |

Table 5.5.3.9.2-2A: Payload contained in the Security parameters and Payload

| Derivation Path: TS 24.282 [87] clause 15.2.13 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Field | Value/remark | Comment | Reference | Condition |
| Payload IEI | '78'O |  | TS 24.282 [87] clause 15.1.4 |  |
| Length of Payload | length of the content |  |  |  |
| Payload content type | '00000001'B | TEXT |  |  |
| Payload data | "Test" | The data payload |  |  |