3GPP TSG RAN Meeting #94e									        	RP-212810

Electronic Meeting, December 6 - 17, 2021

## Status Report to TSG

Agenda item:			9.3.2.6

| WI / SI Name |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: Yes | Performance part: Yes |  | Testing part: No |
| Acronym | NR_IAB_enh |  |  |  |  |
| Unique ID | 860050 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-211548 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: N/A | Core part: 03/2022 | Performance part: 09/2022 | Testing part: N/A |  |
| Overall Completion level | Study Item: N/A | Core part: 65% | Performance Part: 0% | Testing part: N/A |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | RAN-2 |
| --- | --- | --- |
| Rapporteur | Name | Georg Hampel |
|  | Company | Qualcomm Incorporated |
|  | Email | ghampel@qti.qualcomm.com |

## 1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

## 2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

## 2.1	RAN1

#### 2.1.1	Agreements

RAN1 #106bis-e

Enhancements to resource multiplexing between child and parent links of an IAB node

Agreement

Select the following alternative to handle potential indication conflict of symbols configured as semi-static flexible by one parent node, but not the other in inter-donor DC scenarios if the IAB MT of the dual-connected IAB-node does not support simultaneous Tx and Rx on different carriers:

- Alt. 1. The IAB MT does not expect to receive conflicting DCI formats including DCI2_0 and dynamic scheduling grants from different parents. FFS: Explicitly captured in the specification or left as a network configuration error case without specification impact

Select the following alternative to handle potential indication conflict of symbols configured as semi-static flexible by both parent nodes in inter-donor DC scenarios if the IAB MT of the dual-connected IAB-node does not support simultaneous Tx and Rx on different carriers:

- Alt. 5: If a conflict occurs, the IAB MT is expected to perform as scheduled by MCG

Agreement

In DC scenarios, support per-child MT link-NA resource configuration.

- This configuration can be made available to IAB node as well

Agreement

The Rel-17 frequency domain H/S/NA configuration is provided across multiple slots and/or over a subset of slots only, with the same time-domain granularity and pattern duration as the Rel-16 H/S/NA configuration (i.e. gNB-DU Cell Resource Configuration (9.3.1.107 in TS 38.473 [8])).

- For a given slot, different H/S/NA resource types can be configured for different RB sets

- Additional signaling details (e.g. bitmap, slot pattern, etc.) can be left up to RAN3

- FFS: The number of different frequency domain configurations at a given time

Agreement

A single value for the RB set size, N, is configured for a given IAB-DU cell’s Rel-17 frequency domain H/S/NA configuration

Working Assumption

If both the Rel-16 time domain H/S/NA configuration and Rel-17 frequency domain H/S/NA configuration are provided for a given RB set within a slot, one of the following is selected:

- Alt. 1: An IAB node applies the frequency domain H/S/NA only if the IAB node is currently operating in a non-TDM multiplexing mode in the slot, otherwise the Rel-16 time domain H/S/NA configuration is applied.

Agreement

RS ID, based on the IAB node’s DU configurations, is used by a parent node to indicate beams of an IAB-DU in the direction of which simultaneous operation is restricted

- At least SSB ID and [STC index] are supported

- FFS: Whether restrictions are indicated to apply differently for H or S resources

- FFS: Informing the parent node of SRS configuration of the IAB-MT (if collocated with the IAB-DU)

Agreement

The restricted beam indication from the parent node to the IAB node may be indicated (or specified) to be associated with some combination (one or multiple) of the following IAB node’s parameters:

- [Multiplexing mode]

- [MT’s DL beam (e.g. TCI state id)] or MT’s UL beam (e.g., SRI id)

- [DU resource configuration (e.g. soft resources)]

- [Slot index]

Agreement

The MAC-CE signaling of Desired/Provided Guard Symbols is enhanced to optionally indicate the number of guard symbols required for switching between at least the following cases:

- Case#6 MT Tx and [Case #7] DU [Tx]/Rx

- Case#7 MT Tx (to support Case #7 at parent node) and DU Tx/Rx

Agreement

The recommended beam indication from the IAB-MT to the parent node are provided via MAC-CE:

- For DL Rx beam(s): using one or more of the following:

  - DL TCI state ID

    - FFS: UE/IAB-MT does not assume that DL Tx power adjustment (if provided) is applied to the SSB index (if supported) indicated as QCLed reference signal in DL TCI state ID.

  - [RS ID]

  - [R17 DL TCI, or joint DL/UL TCI ID]

- For UL Tx beam(s): using one or more of the following:

  - [Spatial relation]

  - [RS ID]

  - [R17 UL TCI, or joint DL/UL TCI ID]

  - [SRI]

Agreement

A single DCI format 2_5 can be received indicating availability for the soft resources of the respective RB sets corresponding to a given time resource of the child IAB-DU cell.

- FFS: Extension of AvailabiltyCombination to include multiple RB sets in a resourceAvailabilty indication

- FFS: Update resourceAvailability mapping table defined in TS38.213 so that the indication of availability can be applied over soft resources in frequency-domain for DL or UL or Flexible symbols.

- FFS: Need for extension of the maximum payload size of DCI format 2_5 to increase the number of IAB-DU cells that can be provided with availability information for Soft resources to accommodate the maximum number of possible RB sets for a given DU cell (if defined), or other backwards compatible signaling extensions in case the principal indication capabilities of DCI format 2_5 are increased.

Other enhancements for simultaneous operation of IAB-node’s child and parent links

Agreement

The following alternative is selected for the association between the indicated parent-node’s DL TX power adjustment, provided by an IAB-MT to its parent-node, and IAB-node’s resources and/or configurations:

- Alt 2. The desired DL TX power adjustment is indicated to be associated with some combination (one or multiple) of the following IAB-node’s configurations:

  - Multiplexing mode

  - MT’s DL beam (e.g. TCI state id)

  - (MT CC, DU cell) pair

  - DU resource configuration

  - FFS: Slot index

  - FFS: timing mode (e.g., Case-7 timing)

Agreement

Case 7 UL timing offset is indicated by the parent-node via MAC-CE.

Agreement

The granularity of Case 7 UL timing offset is the same as the UL TA granularity.

Conclusion

RAN1 does not discuss further proposals for IAB interference management enhancements in 8.10.2.

Agreement

The desired parent-node’s DL TX power adjustment, provided by an IAB-MT to its parent-node, is indicated via MAC-CE.

- The indication further includes the associated configurations and/or resources for which the indicated power adjustment is applicable.

- The indicated adjustment is in terms of a relative offset to a reference DL TX power.

  - FFS: the reference power (e.g., an RS such as CSI-RS, etc) for the indication of desired adjustment.

  - FFS: the range of values for the indicated adjustment.

Agreement

The desired IAB-MT’s UL PSD range, provided by the IAB-MT to its parent-node, is indicated to be associated with some combination (one or multiple) of the following IAB-node’s configurations:

- Multiplexing mode,

- MT’s UL beam (e.g., SRI id),

- (MT CC, DU cell) pair,

- DU resource configuration

- FFS: slot index

- FFS: timing mode (e.g., Case-6 timing)

Agreement

The desired IAB-MT’s UL PSD range, provided by an IAB-MT to its parent-node, is indicated via a new MAC-CE.

- The indication further includes the associated configurations for which the indicated PSD range is applicable.

- FFS: the range of values for the indicated PSD range and whether RAN4 input is needed.

- FFS: IAB-MT’s behaviour in case the configured/indicated UL TX power is outside the indicated desired PSD range and whether RAN4 input is needed.

Conclusion

RAN1 to further discuss, under 8.10.2, whether to support new triggering conditions to send an updated PHR (e.g., upon change of multiplexing mode, or applying a desired DL TX power adjustment indication from a child-node).

Agreement

The DL TX power adjustment, provided by the parent-node to IAB-MT, is indicated to be associated with some combination (one or multiple) of the following IAB-node’s configurations:

- Multiplexing mode

- MT’s DL beam (e.g., TCI state id, RS id)

- (MT CC, DU cell) pair

- DU resource configuration

- FFS: DL signal/channel type

- FFS: slot index

- FFS: timing mode (e.g., Case-7 timing)

Agreement

The DL TX power adjustment, provided by the parent-node to the IAB-MT, is indicated via MAC-CE.

- The indication further includes the associated configurations and/or resources for which the indicated power adjustment is applicable.

- The indicated adjustment is in terms of a relative offset to a reference DL TX power.

  - FFS: the reference power (e.g., an RS such as CSI-RS, etc) for the indication of DL Tx power adjustment.

  - FFS: the range of values for the indicated adjustment.

Agreement

The indicated DL TX power adjustment is not applied to SSBs.

- FFS: any other cell-specific/semi-static DL signal to be exempted.

- FFS: applicability of the indicated TX power adjustment to other RS/channel which share the same QCL Type-D assumption.

Agreement

RAN1 to downselect in RAN1#107-e one of the following for an OTA timing synchronization mechanism to enable/maintain Case 6 timing mode:

- Alt 1: no change or enhancement to the Rel-16 OTA synchronization specification is supported in Rel-17 for Case 6 timing.

- Alt 2: in Rel-17 the Rel-16 OTA synchronization specification is updated to support OTA synchronization for an IAB-node operating solely in Case 6 timing during IAB-MT Tx.

  - FFS range of T_delta.

NOTE: this is to provide a feasible solution to the RAN1#103-e agreement: “An IAB-node can rely on an OTA timing synchronization mechanism to enable/maintain Case 6 timing mode”

RAN1 #107-e

Enhancements to resource multiplexing between child and parent links of an IAB node

Agreement

Support indication of whether FDM is required or not for an enhanced multiplexing operation mode to donor-CU.

Agreement

An IAB-MT is provided with a Timing Case Indication via MAC-CE that explicitly indicates a list of slots and their associated UL TX timing cases (i.e., one of {Case 1, Case 6, Case 7} for each slot).

Agreement

The maximum number of non-overlapping RB sets configurable per DU cell is M

- where, M is to be selected from one of values from 4, 8, 16

- DU frequency configuration information should be provided to the parent node.

Agreement

Whether or not an IAB node can operate under a given non-TDM multiplexing mode (i.e. multiplexing info in 38.473) is left to IAB implementation in Rel-17

Agreement

In addition to SSB ID, CSI-RS ID may be additionally used as the RS ID for a restricted beam indication from the parent node to the IAB node.

- STC index may be additionally indicated along with SSB ID if more than one STC is configured at the IAB node.

- Note: This does not mean that IAB-specific CSI-RS should be developed and requires no additional specification work

Agreement

- The maximum number of recommended beams per MT CC in a given indication (including all associated parameters/conditions) is 8.

- The maximum number of restricted beams per DU cell in a given indication (including all associated parameters/conditions) is 8.

Agreement

A Timing Case Indication received from a serving cell is applicable to all other cells in the same timing advance group (TAG).

Agreement

The recommended beam indication from the IAB-MT to the parent node are provided using the following:

- For DL Rx beam(s)

  - DL TCI state ID and RS ID (SSB ID and/or CSI-RS ID)

- For UL Tx beam(s)

  - SRI

Agreement

A child IAB-MT can inform a parent node via MAC-CE whether Case 6 timing is required for simultaneous operation.

Agreement

For DCI format 2_5 indicating availability for the soft resources of the respective RB sets corresponding to a given time resource of the child IAB-DU cell:

- AvailabiltyCombination can be extended to include multiple resourceAvailabilty, where each resourceAvailabilty includes availability indication for one RB set group

  - One RB set group consists of one or multiple RB sets

Agreement

The following RAN1#106bis-e agreement is updated.

The MAC-CE signaling of Desired/Provided Guard Symbols is enhanced to optionally indicate the number of guard symbols required for switching between at least the following cases:

- Case#6 MT Tx and [Case #7] DU [Tx]/Rx

- Case#7 MT Tx (to support Case #7 at parent node) and DU Tx/Rx

- A: Case #6 MT TX to/from Case #1 DU RX

- D: Case #7 MT TX (to support Case #7 at parent node) to/from Case #1 DU RX

- G: Case #7 MT TX (to support Case #7 at parent node) to/from Case #1 DU TX

- (Working Assumption) H: Case #6 MT TX to/from Case #1 DU TX

Agreement

The value of the maximum number of contiguous and non-overlapping RB sets configurable per DU cell, M is 8.

Agreement

The IAB-DU is expected to apply the indicated beam restriction within its soft resources that are not explicitly indicated as available.

Agreement

The restricted beam indication from the parent node to the IAB node may be indicated to be associated with some combination (one or multiple) of the following IAB-node’s configurations:

- {MT CC, DU cell} pair and optionally may be indicated to be associated with only {DU cell} if independent of MT CC(s)

- Multiplexing mode info (i.e. multiplexing info in 38.473) and optionally may be indicated to be applicable to non-overlapping frequency resources

- Slot index

- Association with IAB-MT’s DL Rx beam via TCI state ID and RS ID (SSB ID and/or CSI-RS ID) or UL TX beam via SRI

Agreement

The recommended beam indication from the IAB node to the parent node may be indicated to be associated with some combination (one or multiple) of the following IAB-node’s configurations:

- {MT CC, DU cell} pair and optionally may be indicated to be associated with only {MT CC} if independent of DU cell(s)

- Multiplexing mode info (i.e. multiplexing info in 38.473) and optionally may be indicated to be applicable to non-overlapping frequency resources

- Slot index

Other enhancements for simultaneous operation of IAB-node’s child and parent links

Agreement

The dynamic range of the MAC CE case #7 timing offset indication is 12 bits.

- FFS the numerical values of the endpoints of the range

Conclusion

In Rel-17 there is no specification support for the indication of “timing mode” in the desired/provided DL TX power adjustment or the desired UL PSD range indication.

- Note: This does not prevent the indication of case 6 timing mode and UL PSD range as separate conditions to enable a given multiplexing case.

Conclusion

In Rel-17 there is no specification support for the indication of “DL signal/channel type” in the provided DL TX power adjustment indication.

Agreement

Send an LS to RAN4 to specify the range of values for the indication of the desired/provided DL TX power adjustment, as well as the range of values for the indication of the desired MT UL PSD range.

LS is endorsed in R1-211XXXX (to be reviewed and endorsed on Nov 29~30).

Agreement

The provided DL TX power adjustment is applied only to PDSCH and its associated DMRS and PTRS.

Agreement

The indicated desired/provided DL TX power adjustment is in terms of a relative offset to a CSI-RS TX power that is RRC configured.

Conclusion

For Rel-17, no further discussion in RAN1 on eIAB supporting new triggering conditions for sending an updated PHR.

Agreement

Select Alt 2 from the aforementioned RAN1#106b-e agreement without specification impact other than the following:

- Alt A: the T_delta range is updated to support Case 6 timing.

FFS: Update of one way delay estimation equation in TS38.213 subclause 14

Agreement

TCI state ID and RS ID (SSB ID and/or CSI-RS ID) is used to indicate IAB-MT’s DL beam for the desired/provided DL TX power adjustment indication by the IAB-node/the parent-node.

In case the desired/provided DL TX power adjustment indication does not include information about the associated IAB-MT’s DL beams, the adjustment is applied to all MT’s DL beams.

Agreement

SRI is used to indicate IAB-MT’s UL beam for the desired UL PSD range indication.

In case the desired UL PSD range indication does not include information about the associated IAB-MT’s UL beams, the PSD range is applied to all MT’s UL beams.

Agreement

Support optionally indicating “slot index” in the provided DL TX power adjustment indication, that comprises indicating a list of one or multiple slot indices for which the associated DL power adjustment is applied.

- FFS:  support of “slot index” indication in the desired DL TX power adjustment

- FFS:  support of “slot index” indication in the desired UL PSD range indication

Agreement

The indication of the desired/provided DL TX power adjustment and desired UL PSD range can further include:

- An indication of whether a desired/provided power configuration or adjustment is applied on FDM resources where the simultaneous MT’s and DU’s signals are non-overlapping in the frequency-domain and/or on non-FDM resources where the simultaneous MT’s and DU’s signals may overlap in the frequency-domain, for a given (MT CC, DU cell).

Agreement

Support optionally indicating “slot index” in the provided DL TX power adjustment indication, that comprises indicating a list of one or multiple slot indices for which the associated DL power adjustment is applied.

- Support of “slot index” indication in the desired DL TX power adjustment indication

#### 2.1.2	Remaining Open issues

100% of the items defined in the RAN1 WID objectives have been accomplished.

## 2.2	RAN2

#### 2.2.1	Agreements

RAN2 #116-e

MAC

- Support of Extended BSR by an IAB-MT is an optional capability.

- The same format is adopted for Extended Long and Extended Long Truncated BSR.

- Reserved values from the one-octet eLCID space are used to identify new Extended BSR formats.

- Extended LCG space (max 256 LCGs) shall also apply to pre-emptive BSR.

- Extended pre-emptive BSR format shall be identical to the Extended Long BSR format.

- When the Extended BSR is configured, the selection between Extended BSR and legacy BSR is not left to IAB-MT implementation.

- When the Extended BSR is configured, if the maximum LCGID among the configured LCGs is 7 or lower, legacy format is always sent; otherwise the Extended format is sent.

- The following format is adopted for Extended Long and Extended Long Truncated BSR: Fixed size of 256 LCGi followed by variable number of (fixed size) Buffer Size fields; related buffer size field is added only when the corresponding LCG bit is set to 1 in the bitmap.

- RAN2 will not attempt standardizing buffer size calculation for Rel-17 pre-emptive BSR, nor make any further effort to standardizing triggering of Rel-17 pre-emptive BSR.

Type-2/3 RLF indication

- Type 2 indication by dual-connected node is triggered when the node initiates RRC re-establishment resulting from BH RLF on both CGs or BH RLF on MCG with no fast MCG recovery.

- A node can transmit type-3 indication if re-establishment is successful. FFS whether to specify a detailed condition for success of re-establishment, e.g., successful transmission of RRC reestablishment complete. FFS whether to also include additional triggering condition such as successful transmission of ReconfigurationComplete, which is for the case the node initiates re-establishment and selects a CHO candidate cell and hence performs CHO successfully.

- A node can transmit type-3 indication only if it previously sent type-2 indication, i.e., type-3 indication cannot be triggered without triggering type-2 indication previously.

- Upon reception of type-2 indication, the node should perform local re-routing if possible.

- Upon reception of type-3 indication, the actions (e.g. local re-routing) triggered upon reception of a previous type-2 indication should be reversed, if possible.

- FFS if Type 2 indication by dual-connected node can be triggered when the node detects BH RLF on any BH and it cannot perform re-routing for affected traffic (if agreed see R2-2111539 for more details)

- For triggering condition of type-2 indication by a single-connected node, initiation of RRC re-establishment is a sufficient condition to trigger type-2 indication.

- If option 2) is chosen in P1 (i.e. dual-connected node triggers type 2 indication when the node detects BH RLF on any BH link) and option 2 is chosen in P7 (i.e. Received type-2 indication is further propagated),  type-2 indication sent by a single-connected node includes routing ID information indicating which routing IDs are not available. FFS whether inclusion of routing ID can be omitted in some cases. Otherwise, type-2 indication sent by a single-connected node does not carry any further information related to BH RLF.

- Conditional mobility is not triggered by reception of type-2 indication.

- For the need of further propagating received type-2 indication, FFS which option to take:

- Option 1) Received type-2 indication is not propagated further (unless a normal type-2 triggering condition is met).

- Option 2) Upon reception of type-2 indication, the node should further propagate type-2 indication to the child if it has no alternative path available.

- RAN2 does not specify UL transmission constraints (e.g. SR/BSR) to a node receiving the type-2 indication, i.e., whether the node can transmit uplink transmission is left to implementation of the node and also up to scheduling policy of a node transmitting the type-2 indication. FFS whether we need to add a Note in stage-2/3 CR.

- [032] RAN2 does not specify that IAB-support indicator is toggled by reception of type-2 indication, i.e., when how to set IAB-support indicator it is up to implementation. FFS whether we need to add a Note in stage-2/3 CR.

- To agree that the following terms are used:

  - Type-2:  “BH RLF detection indication”,

  - Type-3: “BH RLF recovery indication” , and

  - Type-4: FFS whether “BH RLF recovery failure indication” or existing name “BH RLF indication”

CP-UP separation

- The configuration of F1-C traffic on the indication of the the leg(s) used for transferring the F1-C traffic is configured to IAB-MT by a new field , e.g., f1c-TransferPath-r17  ENUMERATED {MCG, SCG, both}.

- As long as the BH RLC CH for F1-C on the indicated Cell Group is configured (the CG is indicated by the field f1c-TransferPath-r17), IAB node can be aware of whether to use F1-C transferring over BH or F1-C transferring over RRC, i.e. F1-C-over-BAP is selected as long as BH RLC CH for F1-C on the indicated CG is configured.

- It is not necessary for IAB-node to be aware whether the gNB allows “F1 over BAP” or only allows “F1-C over RRC” during cell (re)selection, in case the gNB broadcasts iab-Support.

- ONLY SRB2 is used for F1-C transport in CP/UP-separation scenario 1.

- ONLY split SRB2 is used for F1-C transport in CP/UP-separation scenario 2

- FFS if For IAB-MT’s RRC message that carries F1-C/F1-C related traffic, the IAB-MT use split SRB2 via SCG in scenario 2 if f1c-TransferPath-r17 indicates ‘SCG’ or ‘both’ regardless of the primaryPath configuration. FFS on how to capture this in specs.

- FFS if In case the split SRB2 RRC message contains both F1-C traffic and other information unrelated to IAB, the IAB-MT follows the configuration of F1-C transfer path (if configured) to transmit this RRC message.

On Topology adaptation enhancements

Inter Topology Routing

- Go with B, including the following:

  - If BAP address matches, deliver to upper layer;

- Else:

  - If routing ID matches rewriting table, perform the header rewriting;

  - perform routing and mapping to BH RLC CH.

- For downstream, the boundary node is able to identify/differentiate the traffic routed from inter-topology vs. the traffic routed from intra-topology, based on the ingress link.

- For downstream at the boundary node, for any received data from inter-topology identified by the ingress link:

- The data is delivered to upper layer, if the BAP address in the header is same as the boundary node BAP address configured in the topology of the ingress link (of this packet); otherwise, the data is determined as to be header rewritten (assumes support only of topology where decedent nodes belong to same topology).

- (This requires that traffic not terminated at the boundary node should not use the BAP address in header same as the boundary node BAP address configured in the topology of the ingress link.)

- Perform the header rewriting based on the configured rewriting table, and then perform routing and mapping to BH RLC CH.

- For upstream at the boundary node, for any received data from lower layer:

- We may keep the ingress BAP text of R16 (that is intended for donor DU but general in Stage-3), i.e. if the BAP address in header match the boundary node BAP address configured in the topology of the ingress link, deliver to upper layer.

- The data is determined as to be header rewritten and perform the header rewriting accordingly, if routing ID in header matches any “previous routing ID” in the rewriting table; and then perform routing and mapping to BH RLC CH.

Intra topology

- For Upstream, The pre-condition/criteria of “BAP header rewriting for re-routing” is that there is no available next hop found based on BAP routing ID and based on BAP address in the routing table (e.g. due to BH RLF, congestion or type2 indication, etc.), as in R16.

- Will have rewriting mapping configuration(s) Old routing ID to New routing ID that limits the possible rewriting (for all cases of re-writing), details FFS

#### 2.2.2	Remaining Open issues

60% of the items defined in the RAN2 WID objectives have been accomplished. All objectives from WID on enhancements to IAB for NR require further work.

For the RAN2-led WID objective on topology, routing and transport, RAN2 has made progress on the topics of enhancement of LCG-range, RLF indication, and local rerouting. The corresponding accomplishment level is around 65%.

For the RAN3-led WID objective on topology adaptation enhancements, RAN2’s accomplishment level is around 60%.

For the RAN1-led WID objective on duplexing enhancements, RAN2 has not yet started any work since it depends on further progress by RAN1.

## 2.3	RAN3

#### 2.3.1	Agreements

RAN3 #114-e

Agreements on inter-donor IAB-node migration - procedure details:

- For IP address addition, non-F1-terminating CU to configure IP addresses on the boundary node via Rel-16 RRC signalling, and boundary node reports the F1-U IP addresses it wants to use via Rel-16 F1AP signalling to the F1-terminating CU.

- The non-F1-terminating CU to use Rel-16 RRC procedures for replacement and release of IP addresses at the boundary node.

- The F1-terminating CU sends the information necessary for the non-F1-terminating CU to configure the DL mapping on its Donor-DU.

- WA: F1AP is used for header-rewriting configuration on the boundary node.

- A new Xn procedure is introduced to enable the inter topology migration of F1 transport. FFS if UA or NUA Xn procedure.

- For IP address reconfiguration of descendent nodes, if needed:

  - An Xn procedure between F1-terminating and non-F1-terminating CUs is used, and the F1-terminating CU adds, replaces or releases the IP addresses on the descendent node via RRC.

  - The same Xn procedure is also used for the transfer of the descendent node’s QoS info/L2 info.

  - The same Xn procedure is used for partial migration, inter-donor redundancy and RLF recovery.

  - As the baseline, the reconfiguration of the descendent node occurs after the establishment of the target path. FFS on further details.

- WA: The following information is exchanged between F1-terminating CU (CU1) and non-F1-terminating CU (CU2) for boundary node traffic:

  - CU1->CU2

    - QoS info per traffic type for non-UP traffic and per one or bundle of F1-U tunnels for UP traffic; content is FFS.

    - DL IP address info

  - CU2->CU1

    - DL: IPv6 FL/DSCP value

    - UL: UL boundary node configuration, e.g., UL BH mapping, for each QoS info; pending RAN2.

- RLF recovery uses the existing Xn procedure for fetching the context of the boundary IAB-MT, and the new Xn procedure for enabling the inter topology migration of F1 transport.

- For IP address allocation during RLF recovery, same mechanisms to be used as for partial migration.

- Revise definition of boundary node in BL CR to 38.401 in R3-214656:

- Boundary IAB-node: an IAB-node with one RRC interface terminating at a different IAB-donor-CU than the F1 interface. This definition applies to partial migration and inter donor redundancy and inter donor RLF recovery.

- Way forward: Discussions on IAB full migration are stopped for Rel17. The topic may be addressed in future releases.

Agreements on reduction of service interruption:

- WA: Solution 1 for delivery of RRCReconfiguration over the source path in intra-donor migration is agreed. This WA can be revisited if RAN2 raises objections/remarks.

- RAN3 to discuss avoidance of descendant node reconfiguration (e.g., an IP tunnel between Donor-DUs) after the baseline solution for inter-donor migration (that implies reconfiguring of descendant nodes) has been settled.

- No further enhancements for the avoidance of unnecessary UL transmissions, other than local UL rerouting, are specified in Rel-17.

- R3-216090: TP for BL CR for TS 38.401 on the Reduction of Service Interruption was Agreed

Agreements on topology redundancy:

- Agree to wait for RAN2's progress on BAP operation (e.g., header rewriting, routing, bearer mapping)

- E2E QoS requirement are divided into two parts: provided by its own topology fragment, provided by the non-F1-terminating CU’s topology fragment, which is up to implementation of CU1.

- For DL descendent node traffic:

  - CU1->CU2:

    - QoS info.

    - A list of DL IP addresses

    - FFS: L2 info (e.g. egress BAP routing ID, egress BH RLC CH)

  - CU2->CU1

    - for each traffic: a list of {DSCP/IPv6 flow label, ingress BAP routing ID, ingress BH RLC CH ID}

    - FFS: prior-hop BAP address

- For UL descendent node traffic:

  - CU1->CU2:

    - QoS info.

    - FFS: ingress BAP routing ID, ingress BH RLC CH

  - CU2->CU1

    - for each traffic,: egress BAP routing ID, egress BH RLC CH ID

    - FFS: next-hop BAP address for UL

- FFS: additional info, stage-3 details for signaling design.

- WA: If non-F1-terminating CU is not able to guarantee the per topology fragment QoS requirement, it should reject the request from F1-terminating CU.

- The granularity of the informed QoS requirement info is "per GTP-U tunnel”or "per group of GTP-U tunnels"

- About non-F1-U traffic type, the information to be exchanged between the F1-termination donor and non-F1-termination donor include:{ UE-associated F1AP, non-UE-associated F1AP, non-F1 }, FFS for other info.

- Agree to add "IAB Node Indication" to set up dual-donors DC for the IAB node, for scenario 1.

- WA: the IAB-node can be configured with the CG to be used to transmit F1-C, i.e., via f1c-TransferPath-r17 {mcg, scg, both}.  For the detailed IAB-node behavior upon reception of the CG, it is up to RAN2 to discuss.

- A node broadcasting "IAB-support" supports full IAB functionality, RAN3 will not pursue to define a third type of donor node (broadcasting "IAB-support" but not support full IAB functionality)

Agreements on congestion mitigation:

- The value of the maxnoofIABCongInd is set to 1024

- Not to specify the MPS exemption for IAB congestion indication.

- WA: per-BAP routing ID congestion indication will not be pursued in this release.

- WA: the presence of Child Node Identifier IE is Mandatory.

- R3-216184: TP for BL CR for TS 38.473 was agreed

Agreements on multi-hop performance:

- For inter-Donor-DU re-routing, the re-routed packet is only allowed to be transmitted between IAB-donor-DUs.

- The static tunnel is selected to be established between IAB-donor-DUs for inter-donor-DU re-routing.

- The re-routed packet between IAB-donor-DUs can be an UL IP packet without BAP header.

- Target donor-DU determines the UL packet to be re-routed, by comparing IP prefixes and/or a list of IP address(es) configured by donor-CU, and the source address field of the UL IP packet.

- It is up to donor-CU implementation about when to send to target donor-DU the information used for distinguishing the re-routed UL packets.

Agreements on enhancements to resource multiplexing and CLI:

- The F1-terminating donor of the boundary node forwards the boundary IAB node’s resource configuration information to the non-F1-terminating donor, via following XnAP procedures:

  - retrieve UE context procedure,

  - handover preparation procedure,

  - SN addition procedure,

  - MN initiated SN modification procedure.

  - SN initiated SN modification procedure

- Following information are exchanged over Xn interface via the procedures in Proposal 1-1

  - Activated cell list.

  - H/S/NA resource configurations.

  - DL/UL resource configurations.

  - Multiplexing info.

  - Cell specific signal/channel configurations, including at least: SSB information, CORESET 0, and RACH configurations) from/for different parent nodes.

  - other higher layer parameters listed in R1-2110573

- Parent node is aware of boundary IAB-DU cell configurations via the F1AP GNB-DU RESOURCE CONFIGURATION message

- Enhancing F1AP signaling to support per-child MT link-NA resource configuration in DC scenario.

- For Resource compatibility between the two parents, CU1 and CU2 coordinate with each other.

Endorsement of CRs as baseline:

| R3-214616 | BL CR to XnAP on Rel-17 eIAB (Samsung, Nokia, Nokia Shanghai Bell, Verizon, Qualcomm Incorporated, CATT, ZTE, Fujitsu, AT&T, KDDI, Lenovo, Motorola Mobility, LG Electronics) | CR0532r6, TS 38.423 v16.7.0, Rel-17, Cat. B |
| --- | --- | --- |
| R3-214617 | CR on CP-UP separation for Rel-17 IAB (Nokia, Nokia Shanghai Bell, Samsung, Verizon, Qualcomm Incorporated, CATT, ZTE, Fujitsu, AT&T, KDDI, Lenovo, Motorola Mobility, LG Electronics) | CR0020r4, TS 38.420 v16.0.0, Rel-17, Cat. B |
| R3-214624 | CP-based Congestion Mitigation for IAB Network (ZTE) | CR0076r2, TS 38.470 v16.5.0, Rel-17, Cat. B |
| R3-214630 | CP-based Congestion Indication for IAB Networks (Ericsson) | CR0737r8, TS 38.473 v16.7.0, Rel-17, Cat. B |
| R3-216061 | BL CR to TS 38.401 on support of eIAB (Huawei) | CR0179r4, TS 38.401 v16.7.0, Rel-17, Cat. B |

#### 2.3.2	Remaining Open issues

65% of the items defined in the RAN3 WID objectives have been accomplished. All objectives from WID on enhancements to IAB for NR require further work. This accomplishment level includes the deprioritization of IAB full migration for Rel17. Without such depriorization, the accomplishment level would be significantly lower.

## 2.4	RAN4

#### 2.4.1	Agreements

RAN4#101-e

Agreement on RF impact due to Rel-17 IAB enhancement

Simultaneous operation: RF requirement impact due to TX power imbalance between IAB-MT and IAB-DU

  - Maintain existing RF requirements, further discuss on conformance test cases for declared Tx power

    - Including some clarification on requirements applicable rules in core specification for the limitation of meeting relative ACLR requirements at least, FFS for EVM requirements.

    - Further work on detailed wording for such clarification

Case#6 timing: Timing error between own MT TX and DU TX

  - Agreement on baseline assumption: Introducing RF requirements for Timing error between own MT TX and DU TX should be defined for case#6 timing

    - Pending on further checking whether existing cell phase sync requirements already cover above timing error

    - Further check the timing error tolerance between the parent IAB-DU node and child IAB-MT

  - With above baseline assumption confirmed FFS on:

    - reference condition for this requirement

    - error requirement with below Alts

  - Alt1: 3us

  - Alt2: cyclic prefix length of largest supported SCS

  - Alt3: value as SCS dependent

Agreement on RRM impact due to Rel-17 IAB enhancement

On Case 6 Timing

- Option 1: Wait for further RAN1 agreements related to OTA timing synchronization mechanism for Case # 6 timing before drawing final conclusions on the RRM impact.

- Option 2: Reflect the difference between Case#1 and Case#6 timing in TS 38.174.

CLI

- Option 1:  RAN4 needs to define CLI measurement requirements subjecting to RAN1/2 design on mechanisms for assisting simultaneous operations.

- Option 2: For CLI measurements by IAB-MT, no RRM requirements need to be specified.

Simultaneous operation

No further RRM impact for IAB-MT or DU due to simultaneous operation between IAB-MT TX and DU RX or between IAB-MT RX and DU TX is identified.

LS out to RAN1/RAN3:

R4-2120318   Reply LS on Inter-donor migration for IAB enhancement

#### 2.4.2	Remaining Open issues

45% of the items defined in the RAN4 WID objectives have been accomplished. The following items defined in the WID objectives require further work according to current RAN4 discussion:

- Simultaneous operation: further discuss on clarification in specification for simultaneous transmission with power imbalance condition

- Case#6 timing: more discussion on both RF and RRM impact

- CLI: more discussion in RRM aspect impact

## 2.5	RAN5

#### 2.5.1	Agreements

#### 2.5.2	Remaining Open issues

#### 2.5.3	Remaining Open issues with cross-WG dependencies

## 2.6	RAN6

#### 2.6.1	Agreements

#### 2.6.2	Remaining Open issues

## 3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

NOTE: This section only needs to be filled in for WI/SIs where there is a corresponding relevant WI/SI in SA/CT.

## 3.1	SAx/CTs

#### 3.1.1	Agreements with cross-TSG impacts

#### 3.1.2	Remaining Open issues with cross-TSG impacts

NOTE: This section should also flag any critical dependencies that need TSG attention.

## 4.	References

NOTE:	This can be e.g. a list of all related Tdocs in the affected WGs since last TSG, references to LSs, produced TRs/TSs, the work/study item description or status reports of previous TSGs.

RAN1#106e-bis:

| TDoc | Title | Source |
| --- | --- | --- |
| R1-2108719 | Reply LS on inter-donor migration | RAN2, Samsung |
| R1-2108765 | Resource multiplexing between backhaul and access for IAB duplexing enhancements | Huawei, HiSilicon |
| R1-2108766 | Enhancements for simultaneous operation of MT and DU | Huawei, HiSilicon |
| R1-2108826 | Enhancements for resource multiplexing among IAB backhaul and access links | Nokia, Nokia Shanghai Bell |
| R1-2108827 | Other enhancements for simultaneous operation of IAB-node’s child and parent links | Nokia, Nokia Shanghai Bell |
| R1-2108828 | Discussion on inter-donor IAB migration and topology redundancy | Nokia, Nokia Shanghai Bell |
| R1-2108995 | Enhancement to resource multiplexing between child and parent links | vivo |
| R1-2108996 | Other enhancements for simultaneous operation of child and parent links | vivo |
| R1-2108997 | Other enhancements to Rel-17 IAB nodes | vivo |
| R1-2109152 | Rel-17 UE features for IAB enhancements | Huawei, HiSilicon |
| R1-2109261 | Enhancements to the IAB resource multiplexing | ZTE, Sanechips |
| R1-2109262 | Enhancements for simultaneous operation of child and parent links | ZTE, Sanechips |
| R1-2109263 | Consideration on reference carrier for semi-static IAB-DU resource configuration | ZTE, Sanechips |
| R1-2109264 | Discussion on NR Rel-17 IAB MT Features | ZTE, Sanechips |
| R1-2109373 | Reply LS on inter-donor migration | RAN4, ZTE Corporation |
| R1-2109510 | Enhancements to Resource Multiplexing for NR IAB | Samsung |
| R1-2109511 | Enhancements to Timing, Power Control and CLI for NR IAB | Samsung |
| R1-2109629 | Enhancements to Resource Multiplexing between Child and Parent Links of an IAB Node | Intel Corporation |
| R1-2109630 | Other Enhancements for Simultaneous Operations | Intel Corporation |
| R1-2109697 | Resource multiplexing between child and parent links of an IAB node | NTT DOCOMO, INC. |
| R1-2109698 | Other enhancements for simultaneous operation of IAB-node’s child and parent links | NTT DOCOMO, INC. |
| R1-2109755 | Other enhancements for IAB | Huawei, HiSilicon |
| R1-2109816 | Discussion on IAB resource multiplexing enhancements | ETRI |
| R1-2109817 | Discussion on simultaneous operation enhancements for eIAB | ETRI |
| R1-2109839 | Discussions on enhancements to resource multiplexing between child and parent links of an IAB node | CEWiT |
| R1-2109840 | Discussion on simultaneous operation of IAB-node’s child and parent links | CEWiT |
| R1-2109920 | Enhancements for IAB resource multiplexing | AT&T |
| R1-2109921 | Enhancements for IAB interference management | AT&T |
| R1-2109936 | Resource multiplexing in enhanced IAB systems | Lenovo, Motorola Mobility |
| R1-2109937 | Timing, interference, and power control in enhanced IAB systems | Lenovo, Motorola Mobility |
| R1-2110051 | Enhanced resource multiplexing within and across IAB nodes | Apple |
| R1-2110052 | Other enhancements for simultaneous operation of an IAB node's links | Apple |
| R1-2110101 | Discussions on IAB resource multiplexing enhancements | LG Electronics |
| R1-2110102 | Discussions on other enhancement for simultaneous operation | LG Electronics |
| R1-2110206 | Resource management for enhanced duplexing | Qualcomm Incorporated |
| R1-2110207 | Enhancements for simultaneous operation of IAB-node's child and parent links | Qualcomm Incorporated |
| R1-2110331 | Resource multiplexing and dual connectivity in enhanced IAB | Ericsson |
| R1-2110332 | Timing, interference management and power control in enhanced IAB | Ericsson |
| R1-2110333 | On RRC parameters in enhanced IAB | Ericsson |
| R1-2110334 | UE features for enhanced IAB | Ericsson |
| R1-2110442 | Feature lead summary #1 of 8.10.1 | Moderator (AT&T) |
| R1-2110443 | Feature lead summary #2 of 8.10.1 | Moderator (AT&T) |
| R1-2110497 | Summary#1 [106bis-e-NR-eIAB-02] on other enhancements for simultaneous operation of IAB-node’s child and parent links | Moderator (Qualcomm) |
| R1-2110515 | Final summary [106bis-e-NR-eIAB-02] on other enhancements for simultaneous operation of IAB-node’s child and parent links | Moderator (Qualcomm) |
| R1-2110692 | RAN1 decisions for Rel-17 eIAB | WI rapporteur (Qualcomm) |
| R1-2110701 | Summary of [106bis-e-R17-RRC-eIAB] Email discussion on Rel-17 RRC parameters for eIAB | Moderator (Qualcomm) |

RAN1#107e:

| TDoc | Title | Source |
| --- | --- | --- |
| R1-2108719 | Reply LS on inter-donor migration | RAN2, Samsung |
| R1-2108765 | Resource multiplexing between backhaul and access for IAB duplexing enhancements | Huawei, HiSilicon |
| R1-2108766 | Enhancements for simultaneous operation of MT and DU | Huawei, HiSilicon |
| R1-2108826 | Enhancements for resource multiplexing among IAB backhaul and access links | Nokia, Nokia Shanghai Bell |
| R1-2108827 | Other enhancements for simultaneous operation of IAB-node’s child and parent links | Nokia, Nokia Shanghai Bell |
| R1-2108828 | Discussion on inter-donor IAB migration and topology redundancy | Nokia, Nokia Shanghai Bell |
| R1-2108995 | Enhancement to resource multiplexing between child and parent links | vivo |
| R1-2108996 | Other enhancements for simultaneous operation of child and parent links | vivo |
| R1-2108997 | Other enhancements to Rel-17 IAB nodes | vivo |
| R1-2109152 | Rel-17 UE features for IAB enhancements | Huawei, HiSilicon |
| R1-2109261 | Enhancements to the IAB resource multiplexing | ZTE, Sanechips |
| R1-2109262 | Enhancements for simultaneous operation of child and parent links | ZTE, Sanechips |
| R1-2109263 | Consideration on reference carrier for semi-static IAB-DU resource configuration | ZTE, Sanechips |
| R1-2109264 | Discussion on NR Rel-17 IAB MT Features | ZTE, Sanechips |
| R1-2109373 | Reply LS on inter-donor migration | RAN4, ZTE Corporation |
| R1-2109510 | Enhancements to Resource Multiplexing for NR IAB | Samsung |
| R1-2109511 | Enhancements to Timing, Power Control and CLI for NR IAB | Samsung |
| R1-2109629 | Enhancements to Resource Multiplexing between Child and Parent Links of an IAB Node | Intel Corporation |
| R1-2109630 | Other Enhancements for Simultaneous Operations | Intel Corporation |
| R1-2109697 | Resource multiplexing between child and parent links of an IAB node | NTT DOCOMO, INC. |
| R1-2109698 | Other enhancements for simultaneous operation of IAB-node’s child and parent links | NTT DOCOMO, INC. |
| R1-2109755 | Other enhancements for IAB | Huawei, HiSilicon |
| R1-2109816 | Discussion on IAB resource multiplexing enhancements | ETRI |
| R1-2109817 | Discussion on simultaneous operation enhancements for eIAB | ETRI |
| R1-2109839 | Discussions on enhancements to resource multiplexing between child and parent links of an IAB node | CEWiT |
| R1-2109840 | Discussion on simultaneous operation of IAB-node’s child and parent links | CEWiT |
| R1-2109920 | Enhancements for IAB resource multiplexing | AT&T |
| R1-2109921 | Enhancements for IAB interference management | AT&T |
| R1-2109936 | Resource multiplexing in enhanced IAB systems | Lenovo, Motorola Mobility |
| R1-2109937 | Timing, interference, and power control in enhanced IAB systems | Lenovo, Motorola Mobility |
| R1-2110051 | Enhanced resource multiplexing within and across IAB nodes | Apple |
| R1-2110052 | Other enhancements for simultaneous operation of an IAB node's links | Apple |
| R1-2110101 | Discussions on IAB resource multiplexing enhancements | LG Electronics |
| R1-2110102 | Discussions on other enhancement for simultaneous operation | LG Electronics |
| R1-2110206 | Resource management for enhanced duplexing | Qualcomm Incorporated |
| R1-2110207 | Enhancements for simultaneous operation of IAB-node's child and parent links | Qualcomm Incorporated |
| R1-2110331 | Resource multiplexing and dual connectivity in enhanced IAB | Ericsson |
| R1-2110332 | Timing, interference management and power control in enhanced IAB | Ericsson |
| R1-2110333 | On RRC parameters in enhanced IAB | Ericsson |
| R1-2110334 | UE features for enhanced IAB | Ericsson |
| R1-2110442 | Feature lead summary #1 of 8.10.1 | Moderator (AT&T) |
| R1-2110443 | Feature lead summary #2 of 8.10.1 | Moderator (AT&T) |
| R1-2110497 | Summary#1 [106bis-e-NR-eIAB-02] on other enhancements for simultaneous operation of IAB-node’s child and parent links | Moderator (Qualcomm) |
| R1-2110515 | Final summary [106bis-e-NR-eIAB-02] on other enhancements for simultaneous operation of IAB-node’s child and parent links | Moderator (Qualcomm) |
| R1-2110692 | RAN1 decisions for Rel-17 eIAB | WI rapporteur (Qualcomm) |
| R1-2110701 | Summary of [106bis-e-R17-RRC-eIAB] Email discussion on Rel-17 RRC parameters for eIAB | Moderator (Qualcomm) |

RAN2#116e:

| TDoc | Title | Source |
| --- | --- | --- |
| R2-2109320 | Reply LS on Inter-donor migration (R1-2108529; contact: Huawei) | RAN1 |
| R2-2109350 | LS on BAP- and RRC-related agreements from RAN3#113-e (R3-214476; contact: Ericsson) | RAN3 |
| R2-2109363 | Reply LS on inter-donor migration (R4-2115354; contact: ZTE) | RAN4 |
| R2-2109582 | Long BSR Format with Extended LCG | CATT |
| R2-2109583 | Reduction of Service Interruption during Intra-donor IAB Migration | CATT |
| R2-2109584 | Discussion on Type-2/3 RLF indication | CATT |
| R2-2109585 | Discussion on left issues of local routing and routing | CATT |
| R2-2109611 | Discussion on remaining open issues of LCG range extension and congestion | Intel Corporation |
| R2-2109612 | IAB dual connection, RLF and local rerouting | Intel Corporation |
| R2-2109613 | Intra-donor CU service interruption reduction | Intel Corporation |
| R2-2109614 | Inter-donor CU topology migration, topology redundancy and CP-UP separation | Intel Corporation |
| R2-2109748 | BSR format and reporting in IAB | Fujitsu |
| R2-2109749 | Open issues on (re-)routing | Fujitsu |
| R2-2109750 | Open issues on Type-2 BH RLF indication | Fujitsu |
| R2-2109751 | UE handover during inter-donor-CU migration | Fujitsu |
| R2-2109775 | Discussion on the inter-CU routing | Samsung |
| R2-2109782 | LCG extension for eIAB | Huawei, HiSilicon |
| R2-2109783 | Summary of [Post115-e][088][eIAB] inter-CU routing open issues | Huawei, HiSilicon |
| R2-2109784 | Leftover proposals in Summary of [Post114-e][075][eIAB] Open Issues on Re-routing | Huawei, HiSilicon |
| R2-2109785 | RLF indication for R17-IAB | Huawei, HiSilicon |
| R2-2109786 | F1 over NR access link and CHO | Huawei, HiSilicon |
| R2-2109843 | On BSR formats for IAB | InterDigital |
| R2-2109844 | UL Congestion mitigation in multi-hop IAB | InterDigital |
| R2-2109845 | CHO in IAB | InterDigital |
| R2-2109846 | DAPS support in IAB | InterDigital |
| R2-2109854 | Discussion on LCG extension and UL hop-by-hop flow control feedback | ZTE, Sanechips |
| R2-2109855 | Discussion on inter-donor topology redundancy | ZTE, Sanechips |
| R2-2109856 | Discussion on RLF indication and local re-routing | ZTE, Sanechips |
| R2-2109861 | Discussion on inter-donor migration and service interruption reduction | ZTE, Sanechips |
| R2-2109939 | Updated Rel-17 IAB Workplan | Qualcomm Incorporated, Samsung (WI rapporteurs) |
| R2-2109940 | BAP processing at the boundary node: Modelling A and B | Qualcomm Incorporated |
| R2-2109941 | Enhancements to RLF indications in IAB | Qualcomm Incorporated |
| R2-2110203 | Routing and re-routing enhancements for eIAB | Kyocera |
| R2-2110204 | Details of BH RLF Indications for eIAB | Kyocera |
| R2-2110289 | Running CR to 37.340 for eIAB | vivo |
| R2-2110290 | Discussion on LCG extension issues | vivo |
| R2-2110291 | Discussion on issues of local re-routing based on congestion | vivo |
| R2-2110292 | Miscellaneous Issues of Topology Adaptation | vivo |
| R2-2110293 | Miscellaneous issues on CP-UP separation | vivo |
| R2-2110306 | Discussion on LCG extension and congestion migitation for IAB | Lenovo, Motorola Mobility |
| R2-2110307 | Discussion on local rerouting and local bearer remapping for IAB | Lenovo, Motorola Mobility |
| R2-2110343 | Rel-17 BAP Operations | CANON Research Centre France |
| R2-2110344 | Discussion on RLF indication enhancements | CANON Research Centre France |
| R2-2110348 | Introduce cost factor in local re-routing | Sony |
| R2-2110418 | Triggers for local rerouting | Samsung Electronics GmbH |
| R2-2110422 | Finalizing enhancements to LCG space and BSR triggering including pre-emptive BSR | Samsung Electronics GmbH |
| R2-2110453 | Running CR to 38.321 on Integrated Access and Backhaul for NR Rel-17 | Samsung Electronics GmbH |
| R2-2110723 | IAB inter-CU (re)routing | Nokia, Nokia Shanghai Bell |
| R2-2110724 | Re-routing ehnancements and RLF indications in IAB | Nokia, Nokia Shanghai Bell |
| R2-2110806 | Uplink hop-by-hop flow control | Nokia, Nokia Shanghai Bell |
| R2-2110807 | BSR formats for LCG extension | Nokia, Nokia Shanghai Bell |
| R2-2110883 | On Topology-wide Fairness, Multi-hop Latency, and Congestion in IAB Network | Ericsson |
| R2-2110885 | Boundary IAB node behaviour for partial inter-donor migration | Ericsson |
| R2-2110886 | On Local Routing and Type 2/3 RLF Handling | Ericsson |
| R2-2110888 | Remaining Issues Related to CP/UP Separation in IAB Network | Ericsson |
| R2-2110897 | On BSR formats for IAB | InterDigital |
| R2-2110898 | UL Congestion mitigation in multi-hop IAB | InterDigital |
| R2-2110899 | CHO in IAB | InterDigital |
| R2-2110900 | DAPS support in IAB | InterDigital |
| R2-2110911 | Enhancements to Rel. 17 IAB RLF indications and local routing | Futurewei Technologies |
| R2-2111057 | Open issues for type-2/3 RLF indication | ETRI |
| R2-2111088 | CP-UP separation and other topology adaptation issues | Samsung Electronics |
| R2-2111142 | Resolving open issues on BH RLF indications | LG Electronics |
| R2-2111155 | Stage-3 details of LCG extension | LG Electronics Inc. |
| R2-2111156 | Further discussion on enhancement of local re-routing | LG Electronics Inc. |
| R2-2111157 | Remaining issues on enhancements of topology adaptation and congestion mitigation | LG Electronics Inc. |
| R2-2111174 | Remaining issues -Fairness, latency, congestion | Rakuten Mobile, Inc |
| R2-2111203 | Analysis of some remaining issues for inter-donor & inter-topology routing | Futurewei Technologies |
| R2-2111227 | Running CR of TS 38.340 for eIAB updated based on approach A | Huawei, HiSilicon |
| R2-2111228 | Running CR of TS 38.340 for eIAB updated based on approach B | Huawei, HiSilicon |
| R2-2111266 | Summary of [Post115-e][088][eIAB] inter-CU routing open issues | Huawei, HiSilicon |
| R2-2111450 | Running CR to 38.300 for eIAB | Qualcomm |
| R2-2111451 | Running CR to 37.340 for eIAB | vivo |
| R2-2111500 | Summary of [AT116-e][047][eIAB] Routing and re-routing continued | Huawei, HiSilicon |
| R2-2111501 | Summary of [AT116-e][033][eIAB] CP-UP separation (vivo) | vivo |
| R2-2111520 | Summary of discussion [AT116-e][031][eIAB] MAC - LCG extension and BSR (Samsung) | Samsung |
| R2-2111532 | [AT116-e][032][eIAB] RLF indications | LGE (Rapporteur) |
| R2-2111539 | [AT116-e][032][eIAB] RLF indications | LGE (Rapporteur) |
| R2-2111595 | Summary of [AT116-e][047][eIAB] Routing and re-routing continued | Huawei, HiSilicon |
| R2-2111604 | Running CR to 38.331 on NR IAB enhancements | Ericsson |
| R2-2111637 | Running CR of TS 38.340 for eIAB | Huawei, HiSilicon |

RAN3#114e:

| TDoc | Title | Source |
| --- | --- | --- |
| R3-214616 | BL CR to XnAP on Rel-17 eIAB | Samsung, Nokia, Nokia Shanghai Bell, Verizon, Qualcomm Incorporated, CATT, ZTE, Fujitsu, AT&T, KDDI, Lenovo, Motorola Mobility, LG Electronics |
| R3-214617 | CR on CP-UP separation for Rel-17 IAB | Nokia, Nokia Shanghai Bell, Samsung, Verizon, Qualcomm Incorporated, CATT, ZTE, Fujitsu, AT&T, KDDI, Lenovo, Motorola Mobility, LG Electronics |
| R3-214624 | CP-based Congestion Mitigation for IAB Network | ZTE |
| R3-214630 | CP-based Congestion Indication for IAB Networks | Ericsson |
| R3-214656 | BL CR to TS 38.401 on support of eIAB | Huawei |
| R3-214672 | Reply LS on IAB resource multiplexing | RAN1 |
| R3-214677 | Reply LS on Inter-donor migration | RAN1 |
| R3-214681 | Reply LS on inter-donor-DU rerouting | RAN2 |
| R3-214689 | Reply LS to RAN3 on reduction of service interruption during intra-donor IAB-node migration | RAN2 |
| R3-214690 | Reply LS on inter-donor migration | RAN2 |
| R3-214701 | Reply LS on inter-donor migration | RAN4 |
| R3-214822 | IAB Inter-Donor Topology Adaptation | Ericsson |
| R3-214823 | Reduction of Service Interruption in IAB Migration | Ericsson |
| R3-214824 | Inter-Donor Routing in IAB Topology Redundancy Scenarios | Ericsson |
| R3-214825 | (TP for IAB BL CR for TS 38.473) Congestion Mitigation in IAB Networks | Ericsson |
| R3-214826 | Uplink Inter-Donor Rerouting in IAB Networks | Ericsson |
| R3-214827 | Inter-Donor Resource Coordination in IAB Networks | Ericsson |
| R3-214869 | Procedure of full migration | Fujitsu |
| R3-214870 | IP-based tunneling between IAB-donor-DUs | Fujitsu |
| R3-214873 | (TP to BL CR of TS38.401) Discussion on inter-donor IAB node migration | Samsung |
| R3-214874 | Discussion on service interruption reduction for Rel-17 IAB | Samsung |
| R3-214875 | (TP to BL CR of TS38.423) Discussion on inter-donor topology redundancy | Samsung |
| R3-214876 | (TP to BL CR of TS38.473) Discussion on the inter-donor-DU rerouting | Samsung |
| R3-214905 | Updated Workplan for Rel-17 IAB | Qualcomm Incorporated, Samsung (WI Rapporteurs) |
| R3-214906 | Inter-donor-DU local rerouting for IAB | Qualcomm Incorporated |
| R3-214924 | Discussion on IAB inter-donor topology adaptation procedures | ZTE |
| R3-214925 | Further considerations on service interruption reduction | ZTE |
| R3-214926 | Discussion on inter-donor topology redundancy | ZTE |
| R3-214927 | Discussion on inter-Donor-DU re-routing in IAB | ZTE |
| R3-214928 | Discussion on resource multiplexing in IAB | ZTE |
| R3-214929 | (TP for NR_IAB_enh BL CR for TS 38.473): Congestion indication in CP-based congestion mitigation | ZTE |
| R3-214930 | [Draft] Reply LS to RAN1 on inter-donor migration | ZTE |
| R3-214931 | [Draft] Reply LS to RAN2 on inter-donor migration | ZTE |
| R3-214953 | (TP for BL CR to 38.401) Inter-donor topology adaptation | Qualcomm Incorporated |
| R3-214954 | Reduction of service interruption during IAB migration | Qualcomm Incorporated |
| R3-214955 | Inter-donor topology transport | Qualcomm Incorporated |
| R3-215008 | (TP for IAB BLCR 38.473) IAB Congestion Mitigation MPS exemption | Peraton Labs, CISA ECD, AT&T, Verizon |
| R3-215013 | Discussion on inter-CU migration for IAB | CATT |
| R3-215014 | Discussion on reducation of service interruption | CATT |
| R3-215015 | Discussion on inter-CU topology redundancy | CATT |
| R3-215016 | Discussion on inter-donor-DU re-routing | CATT |
| R3-215302 | Discussion on IAB inter-donor migration | Lenovo, Motorola Mobility |
| R3-215303 | Service interruption reduction for IAB migration | Lenovo, Motorola Mobility |
| R3-215304 | Discussion on IAB inter-donor topology redundancy | Lenovo, Motorola Mobility |
| R3-215305 | (TP for IAB BL CR for TS 38.473) Remaining issues on congestion mitigation for IAB | Lenovo, Motorola Mobility |
| R3-215306 | Discussion on UL packet transmission for inter-donor-DU re-routing | Lenovo, Motorola Mobility |
| R3-215344 | discussion on Inter-Donor IAB Node Migration | Nokia, Nokia Shanghai Bell |
| R3-215345 | (TP for BL CR for TS 38.401) Discussion on Reduction of Service Interruption | Nokia, Nokia Shanghai Bell |
| R3-215346 | discussion on Inter-Donor IAB Topology Redundancy | Nokia, Nokia Shanghai Bell |
| R3-215347 | (TP for BL CR for TS 38.401) Inter donor-DU re-routing | Nokia, Nokia Shanghai Bell |
| R3-215348 | Resource multiplexing | Nokia, Nokia Shanghai Bell |
| R3-215495 | Harmonized proposal on alternatives for full inter-donor IAB-node migration | AT&T |
| R3-215496 | Enhancements for IAB resource multiplexing | AT&T |
| R3-215608 | (TP for NR_IAB_enh BL CR for TS 38.423) IAB resource multiplexing | Huawei |
| R3-215609 | Discussion on IAB congestion mitigation | Huawei |
| R3-215610 | Inter-donor re-routing for IAB and unnecessary transmission during topology update | Huawei |
| R3-215611 | Inter-CU topology redundancy | Huawei |
| R3-215612 | Reduction of Service Interruption for IAB topology update | Huawei |
| R3-215613 | (TP for NR_IAB_enh BL CR for TS 38.401) Inter-CU topology update | Huawei |
| R3-215614 | [Draft] LS on IAB resource multiplexing | ZTE |
| R3-215749 | On Full Inter-Donor Migration in Rel-17 IAB | Ericsson, Huawei, Nokia, Nokia Shanghai Bell |
| R3-215899 | CB: # 1301_IAB_BL_CRs - Summary of email discussion | Qualcomm - moderator |
| R3-215900 | CB: # 1302_IAB_Inter_Donor_Mig - Summary of email discussion | Qualcomm - moderator |
| R3-215901 | CB: # 1303_IAB_Red_Serv_Inter - Summary of email discussion | Ericsson - moderator |
| R3-215902 | CB: # 1304_IAB_Top_Red - Summary of email discussion | Huawei - moderator |
| R3-215903 | CB: # 1305_IAB_Con_Mit - Summary of email discussion | Lenovo - moderator |
| R3-215904 | CB: # 1306_IAB_Multi-hop - Summary of email discussion | ZTE - moderator |
| R3-215905 | CB: # 1307_IAB_Res_Multiplex - Summary of email discussion | Nokia - moderator |

RAN4#101e:

| TDoc | Title | Source |
| --- | --- | --- |
| R4-2118042 | Discussion on RRM requirements for R17 NR eIAB | Intel Corporation |
| R4-2118232 | Simultaneous operation on IAB-node’s child and parent links | Samsung |
| R4-2118233 | Timing enhancement on Rel-17 IAB | Samsung |
| R4-2118436 | Discussion on power imbalance for simultaneous operation of IAB | ZTE Corporation |
| R4-2118437 | Discussion on timing issues for simultaneous operation of IAB | ZTE Corporation |
| R4-2118736 | RF requirements for simultaneous IAB-MT and IAB-DU operation | Nokia, Nokia Shanghai Bell |
| R4-2118852 | Discussion on RRM requirements for eIAB | Huawei, Hisilicon |
| R4-2118912 | On IAB timing error | Nokia, Nokia Shaghai Bell |
| R4-2118929 | On RRM for eIAB | ZTE Corporation |
| R4-2118979 | RF impact analysis for simultaneous DU and MT operation | Ericsson |
| R4-2118980 | IAB MT /DU case 6 timing | Ericsson |
| R4-2119465 | Analysis of RRM requirements for enhanced IAB | Ericsson |
| R4-2119476 | Analysis of Rel. 17 IAB Enhanced RRM Case6 Timing Requirements | Nokia, Nokia Shanghai Bell |
| R4-2119500 | LS Response on Inter-Donor Migration | Nokia, Nokia Shanghai Bell |
| R4-2120049 | Draft CR to TS 38.174:implementation of FR2-1 and FR2-2 frequency sub-ranges for the General parts of the specification | Huawei |
| R4-2120050 | Draft CR to TS 38.175: implementation of FR2-1 frequency sub-range for the General parts of the specification | Huawei |
| R4-2120225 | Email discussion summary for [101-e][230] NR_IAB_enh_RRM | Moderator (ZTE) |
| R4-2120318 | Reply LS on Inter-donor migration for IAB enhancement | ZTE Corporation |
| R4-2120319 | WF on RRM aspects of IAB enhancement | ZTE Corporation |
| R4-2120372 | Email discussion summary for [101-e][230] NR_IAB_enh_RRM | Moderator (ZTE) |
| R4-2120620 | Email discussion summary for [101-e][313] NR_eIAB | Moderator (Samsung) |
| R4-2120679 | WF on RF specification impact for eIAB | Samsung |
| R4-2120744 | Email discussion summary for [101-e][313] NR_eIAB | Moderator (Samsung) |

20.04.2020		minor adaptations for RAN #88e

18.02.2020		minor adaptations for RAN #87e

14.11.2019		minor adaptations for RAN #86

18.08.2019		minor adaptations for RAN #85

12.05.2019		minor adaptations for RAN #84

27.02.2019		minor adaptations for RAN #83

21.11.2018		completion levels with colours added (for RAN #82)

v04.81	31.07.2018		simplification of template and addition of cross-TSG aspects (for RAN #81)

v04.80	21.05.2018		minor adaptations for RAN #80

v04.79	26.02.2018		minor adaptations for RAN #79

v04.78	18.11.2017		minor adaptations for RAN #78

v04.77	06.08.2017		minor adaptations for RAN #77

v04.76	15.05.2017		minor adaptations for RAN #76

v04.75	31.01.2017		minor adaptations for RAN #75

v04.74	28.10.2016		minor adaptations for RAN #74

v04.73	01.09.2016		adaptations for RAN #73 (time units in extra Excel table, RAN6 reporting included)

v04.72	26.05.2016		adaptations for RAN #72 (introduction of NR & GERAN TUs)

v04.71	10.02.2016		minor adaptations for RAN #71

v04.70	30.10.2015		minor adaptations for RAN #70

v04.69	12.08.2015		minor adaptations for RAN #69

v04.68	21.05.2015		minor adaptations for RAN #68

v04.67	01.02.2015		minor adaptations for RAN #67

v04.66	16.11.2014		minor adaptations for RAN #66

v04.65	16.08.2014		minor adaptations for RAN #65

v04.64	22.05.2014		minor adaptations for RAN #64

v04.63	24.01.2014		restructuring for RAN #63 to cover Core & Perf. in one doc file

v03.62	11.11.2013		section 1.2.3 adapted for RAN #62

v03	11.08.2013		section 1.2.3 added on time budget

v02	07.05.2010		history added, some spelling corrections

v01	13.11.2009		First version of the template