3GPP TSG RAN meeting #94-e								               RP 212822

Electronic Meeting, December 06-17, 2021

## Status Report to TSG

Agenda item:	9.3.2.7 - NR small data transmissions in INACTIVE state [RAN2 WI: NR_SmallData_INACTIVE]

| WI / SI Name | NR small data transmissions in INACTIVE state |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: Yes | Performance part: Yes |  | Testing part: - |
| Acronym | NR_SmallData_INACTIVE |  |  |  |  |
| Unique ID | 860051 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-212594 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: n/a | Core part: 03/2022 | Performance part: 09/2022 | Testing part: n/a |  |
| Overall Completion level | Study Item: n/a | Core part: Overall: 60% RAN2: 70% RAN3: 30% RAN4: 30% | Performance Part: 0% | Testing part: n/a |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | RAN2 |
| --- | --- | --- |
| Rapporteur | Name | Eswar Vutukuri |
|  | Company | ZTE Corporation |
|  | Email | eswar.vutukuri@zte.com.cn |

## 1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

## 2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

## 2.1	RAN1

#### 2.1.1	Agreements

- RAN1#107-e

Agreements

Agreement

- UE specific power control parameters P0 and alpha should be configured for initial UL transmission for CG-SDT

  - Existing closed loop power control mechanism can be reused for re-transmission and subsequent data transmission.

- For RA-SDT power control parameters preambleReceivedTargetPower and powerRampingStep:

  - For separate ROs, the power control parameters can be RA-SDT specific

Agreement

Separate common search space that can be configured for RA-SDT within the initial DL BWP can also be configured for CG-SDT.

Conclusion

No need to restrict the same value of mapping ratio for all CG configurations.

Conclusion

RAN1 cannot reach consensus on whether to support multiple CG occasions per CG period

- Note that the CG PUSCH with multiple DMRS is considered as one CG occasion.

Conclusion

During subsequent data transmission, no need to explicitly report beam to gNB.

Conclusion

RA-SDT and CG-SDT can be supported for RedCap UEs without considering specific optimization for Redcap, at least when RedCap UE share both the initial DL BWP and initial UL BWP with non-RedCap UEs.

Agreement

Reply LS on the physical layer aspects of small data transmission is endorsed in R1-2112782

LS in:

- R1-2112630           Reply LS on the physical layer aspects of small data transmission        from RAN2

LS out:

- R1-2112782	Reply LS on the physical layer aspects of small data transmission 	to RAN2

- RAN1#106bis-e

Agreements

Agreement

Multi-layer PUSCH transmission is not supported for CG-SDT.

Agreement

When SSB set indication is absent, UE assumes the SSB set includes all actually transmitted SSBs configured by SIB1.

Agreement

- RAN1 confirms that common PUCCH resources (i.e. those that are shared with non-SDT UEs) can also be used for HARQ-ACK feedback for Msg4 /MsgB and subsequent SDT transmissions.

- RAN1 thinks there is no need for any other PUCCH resources than common PUCCH resources shared with non-SDT UEs.

Agreement

BFD/BFR procedure is not required for SDT in Rel-17.

- FFS: whether or not to support reporting the beam change to gNB.

Agreement

For CG-SDT, the UE can assume the PDCCH carrying the DCI has the same DM-RS antenna port quasi co-location properties as for a SSB associated to the CG PUSCH transmission e.g. for detection of retransmission DCI in response to a CG PUSCH transmission.

Conclusion

No need to define UL/DL pattern type of validation rule specific for paired spectrum at least for non-RedCap UEs.

- FFS the case for RedCap UEs

Conclusion

It is RAN1’s common understanding that dynamic grant based retransmission has already been supported.

Conclusion

RA-SDT resource cannot be configured on non-initial BWP.

Conclusion

From RAN1’s perspective, there is no other L1 configuration for RA-SDT and CG-SDT to support subsequent data transmission.

Agreement

The pathloss for CG-SDT PUSCH power control can be determined by the measurement of selected SSB associated with the CG PUSCH.

Conclusion

- RAN1 cannot reach a consensus on whether to confirm RAN2 agreement that CG-SDT resource can be configured on separate SDT BWP.

- Capture the following in the LS: the concern is on the necessity.

Conclusion

- RAN1 cannot reach consensus on reusing CG-DFI mechanism for CG-SDT for operation in licensed band.

Agreement

- Mapping ratio of SSB to CG PUSCH is configured per CG configuration.

  - FFS whether to restrict the same value for all CG configuration and/or allow different value for different CG configurations.

- For the candidate value set of SSB to CG PUSCH mapping ratio, support at least {1, 2, 4, 8, 16}

  - FFS {1/8,1/4,1/2}

Agreement

- RAN1 confirms the working assumption in RAN2 that UE-specific search space is configured for UEs performing CG-SDT. This does not exclude the configuration of CSS for UEs performing CG-SDT.

- CORESET for UE performing RA-SDT should be a common CORESET.

Agreement

A CG PUSCH occasion is not valid if it overlaps with any valid PRACH occasion.

- FFS overlapping between CG PUSCH occasions and MsgA PUSCH occasion

LS in:

- R1-2108715   LS on agreements related to SDT   from RAN2

LS out:

- R1-2110661   Reply LS on the physical layer aspects of small data transmission   to RAN2

#### 2.1.2	Remaining Open issues

- Remaining design details of association period for CG-SDT

## 2.2	RAN2

#### 2.2.1	Agreements

RAN2#116-e

- RAN2 changes the agreements and as a baseline we will focus on initial BWP for RA and CG SDT.  FFS if further work on CG SDT for non-initial BWP will be needed, based on RAN1 consensus.

- Assumption that we won’t have L1 feedback as a functionality.  Discuss subsequent and autonomous CG transmissions with email discussion.

- The statusReportRequired is automatically enabled at termination of SDT procedure, i.e. PDCP status report is temporarily disabled during SDT procedure. (22/22)

- BSR format enhancements are not considered for SDT. (21/23)

- BSR calculation take suspended RBs into consideration during SDT. (21/23)

- If NAS data arrives at PDCP layer of suspended RBs, the NAS data should be just stored in PDCP SDU buffer without further processing. (23/23). How to ensure this is up to UE implementation, and no spec change is needed.

- PDCP header is not considered for the SDT data volume calculation. (23/23). No spec change is needed.

- Buffered packets in PDCP/RLC entities should be counted in SDT data volume calculation. (21/23). Whether and how to avoid any buffered packets in PDCP/RLC entities at the time of SDT data volume calculation is FFS.

- The legacy TAT (i.e. timeAlignmentTimerCommon in SIB) is used for UL timing maintenance during RA-SDT procedure. (21/23)

- The legacy TAT (i.e. timeAlignmentTimerCommon in SIB) starts/restarts when RAR TAC or TAC MAC CE is received, regardless of SDT procedure. No spec change is needed. (23/23)

- CG-SDT resource is not released even if the legacy TAT expires. (23/23)

- The token bucket mechanism is applied for SDT. (21/23)

- Confirm that PHR is triggered at initiation of SDT procedure based on the existing PHR trigger.  All the triggered PHRs are cancelled if all SDT data are included in the UL grant, if there is NO room in the MAC PDU to fit the PHR.

- The R15/R16 PUSCH skipping mechanism is supported for CG-SDT

- PHR is configured only by default MAC Cell Group configuration

- BSR is configured only by default MAC Cell Group configuration

- For SDT, ROHC continuity functionality can be configurable between the cell and RNA.  Send LS to RAN3

- LCH restrictions can be applied, re-using existing signalling, and it is up to gNB how restrictions are configured and MAC applies current specification rules)

- If LCH restriction is applied for SDT, it is applied both for CG-SDT and RA-SDT.

- FFS whether the logicalChannelSR-DelayTimer is not applied for logical channels configured with SDT

- The NAS data can arrive at PDCP layer even if the RB is suspended. When does the NAS deliver UL data to AS is up to UE implementation.  No spec changes are needed

- If NAS data does not arrive at PDCP layer of suspended RBs, the SDT data volume is calculated by UE implementation. No spec changes are needed.  A NOTE can be added to clarify calculation of data volume and can be discussed in the running CR.

- FFS if the size of CCCH message is considered in SDT data volume calculation

- Highest N SSBs of all SSBs actually transmitted as indicated in SIB1 is used for RSRP based TA validation

- The Rel-16 CG configuration mechanism in licensed band is reused the baseline for CG-SDT.

- At least for initial transmission we will have a mechanism to allow the UE to transmit the message again.  FFS for retransmission for subsequent.

- The UE uses/selects the same HARQ process for retransmission

- The “CG-SDT timer” starts at the first “valid” PDCCH occasion from the end of the CG-SDT PUSCH transmission. The first “valid” PDCCH occasion is defined in RAN1

- The “CG-SDT timer” can be started/restarted during for initial and subsequent transmissions

- The UE restarts the “CG-SDT timer” at least:

  - upon the PUSCH retransmission indicated by the CS-RNTI PDCCH

  - after each CG-SDT transmission

- The “CG-SDT timer” stops at least:

  - When the UE receives RRC feedback messages (e.g. RRCResume, RRCSetup, RRCRelease and RRCReject)

- The Rel-16 calculation on the HARQ process ID of the CG type-1 for licensed band is reused as the baseline for CG-SDT

- The UE is allowed to initiate subsequent UL data transmission only after the reception of confirmation of initial transmission from the gNB

- The UE can use multiple CG resources for the HARQ initial transmission as Rel-16 in the subsequent CG transmission phase

- The following CG-SDT configurations are per UE:

  - The new TA timer in RRC_INACTIVE

  - The RSRP change threshold for TA validation mechanism in SDT

  - The SSB RSRP threshold for beam selection

In addition to the above, RAN2 has handled the following LSs for SDT:

LS in:

- R2-2109308	Reply LS on Small data transmission						CT1

- R2-2109321	Reply LS on on physical layer aspects of small data transmission	RAN1

- R2-2109330	LS on the TA validation and mapping details for CG-SDT			RAN1

- R2-2111219	Reply LS on the physical layer aspects of small data transmission 	RAN1

LS out:

- R2-2111611	LS to RAN1  on RAN2 agreements ZTE

- R2-2111446	LS to RAN3 on RAN2 agreements Xiaomi

Latest versions of the running CRs:

- R2-2110808	Stage-2 running CR Introduction of SDT	Nokia, Nokia Shanghai Bell

- R2-2110185	Running MAC CR for small data	Huawei, HiSilicon

- R2-2110573	RRC Running CR for SDT	ZTE Corporation (rapporteur)

#### 2.2.2	Remaining Open issues

- Detailed procedure for CG-SDT

- Solution for indication of non-SDT data arrival

- Stage-3 procedure details and signalling

- Finalization of RRC, MAC and stage-2 CRs

## 2.3	RAN3

#### 2.3.1	Agreements

###### RAN3#114-e

- For CG based SDT, RAN3 will further discuss impacts and mainly consider split-gNB case.

- Subsequent UL/DL transmission following UL SDT without transitioning to RRC_CONNECTED is supported for SDT

- The existing Retrieve UE Context procedure can be reused for both with and without anchor relocation scenarios with possible enhancements. Details will be discussed later.

- UL data for SDT is buffered at the receiving node in the successful context retrieval procedure. For other cases, the common understanding is that UL data may need to be buffered as well, details are pending.

- The last serving gNB, i.e., anchor gNB, will be the decision maker on whether to relocate anchor or not. Assistance information provided by the receiving gNB may help on the decision. Details of assistance information are pending future discussion

- For RA-SDT, “SDT Indicator” is introduced in RETRIEVE UE CONTEXT REQUEST message, and the message may include other SDT Assistance Information.

- For split CU/DU case, gNB-DU sends “SDT indicator”to gNB-CU in the INITIAL UL RRC MESSAGE TRANSFER message.

- During SDT procedure, SRB PDCP PDU (FFS on the first SDT payload) shall be transferred between new gNB and anchor gNB, either via extending the XnAP RRC TRANSFER message or via defining a new XnAP class-2 procedure.

- For RA-SDT without anchor relocation case, SDT related UE context may be retrieved from anchor gNB to the receiving gNB.

- For RA-SDT without anchor relocation case, for SDT DRB transfer, its stage2 overall procedure shall enhance either legacy Periodic RNA update without UE context relocation procedure or legacy RNA update with UE context relocation procedure. For SDT SRB transfer is FFS.

- The receiving gNB forwards PDCP PDUs (RLC SDUs) via GTP-U tunnel per SDT DRB to the anchor gNB.

- In case of no anchor relocation, the anchor gNB is not allowed to transfer its security related IEs included in “UE Context Information – Retrieve UE Context Response” IE to the receiving gNB. (refer to SA3 LS: R2-2109065)

- SDT related RLC bearer configuration for SDT DRB/SRB should be transferred from anchor gNB to the receiving gNB. How to include those information is FFS.

- Enhance RNA update without UE context relocation procedure for RA-SDT without anchor relocation case

- Endorse R3-216060 (Samsung) as CG-SDT BL CR to 38.473

- CG-SDT query indication IE is provided to the gNB-DU in UE CONTEXT MODIFICATION REQUEST message to request the gNB-DU to provide the CG-SDT configuration.

- CG-SDT resource configuration is provided to the gNB-CU in UE CONTEXT MODIFICATION RESPONSE message.

- When receiving RRCResumeRequest message for CG-SDT, the gNB-DU triggers the UL RRC Message Transfer procedure.

- WA: Lower layer configuration for SDT DRBs, F1AP association, and F1 tunnel information are kept in gNB-DU when gNB-CU sends the UE to RRC_INACTIVE.

- WA: Once the UE initiates RRC Resume procedure from another cell, the gNB-CU shall indicate to the gNB-DU to release the assigned CG-SDT resource.

- WA: New IE is included into the E1AP BEARER CONTEXT MODIFICATION REQUEST message to indicate resume or suspend operation for SDT bearers.

- WA: for the switch from SDT to non SDT,

  - Way 1 (FFS for Way 2) is applied for SDT with anchor relocation

  - Way 2 (FFS for Way 1) is applied for SDT without anchor relocation

  - where

    - Way 1: sending RRCResume message during SDT session

    - Way 2: sending RRCRelease message to end SDT session first, and then another RRC Resume procedure is triggered for anchor relocation

  - WA: when applying Way 1 for SDT with anchor relocation, no RAN3 stage 3 impact is identified

  - WA: when applying Way 2 for SDT without anchor relocation, RAN3 assumes the anchor could move the UE back to RRC Inactive by using RRCRelease message.

- BLCR allocation

RA-SDT BLCR to TS 38.300, ZTE

RA-SDT BLCR to TS 38.401, Intel

RA-SDT BLCR to TS 38.423, Ericsson

RA-SDT BLCR to TS 38.473, Nokia

RA-SDT BLCR to TS 38.463, China Telecommunication Corp

CG-SDT BLCR to TS 38.401, HW

CG-SDT BLCR to TS 38.473, Samsung

LS in:

- None

LS out:

- None

#### 2.3.2	Remaining Open issues

- See the WI objectives for general overview of remaining open issues

- FFS: How to transfer SDT SRB PDCP PDU

- Extending the XnAP: RRC TRANSFER message or defining a new XnAP class-2 procedure.

- Carrying the first SRB PDCP PDU in the RETRIEVE UE CONTEXT REQUEST message.

- FFS: The detail information included in SDT related RLC bearer configuration to be transferred from anchor gNB to receiving gNB in case of SDT without anchor relocation.

- FFS: If agreed to enhance Periodic RNA update without UE context relocation procedure, whether it is acceptable to use the Retrieve UE context failure message to send SDT related UE context information

- FFS: whether to allow optional transport of first DRB payload in the RETRIEVE UE CONTEXT REQUEST message.

- FFS on whether to keep other information (e.g., C-RNTI) or maintain full UE context in gNB-DU.

- FFS on details of this new IE. FFS on whether to reuse the existing IE.

- FFS on whether to use UE Context Setup procedure to exchange CG-SDT query indication and CG-SDT resource configuration.

- FFS on whether to use the Initial UL RRC Message Transfer to send the CG-SDT resource configuration.

- FFS on whether to exchange the updated tunnel information in UE context release procedure.

- FFS on whether there is F1 impact to support the release of CG-SDT resource configuration due to TAT-SDT expiry.

- FFS on whether the gNB-DU reconfigures or releases the old CG-SDT resource configuration via the new UE associated logical F1 connection in case of CG-SDT fallback to RA-SDT or non-SDT, and if yes, how to resolve it.

- For CG-SDT, whether to include SDT indication and SDT assistant information in F1AP message and whether to include that access happened using CG SDT.

- FFS on whether gNB-DU should be aware of which bearers are SDT bearers.

- FFS on how to indicate that CG-SDT configuration should be kept in gNB-DU.

## 2.4	RAN4

#### 2.4.1	Agreements

- RAN4#101-e

Agreements

- Workplan approved (R4-2120339)

  - CR work-split:

| Draft CR | Responsible Company |
| --- | --- |
| UE synchronization | [Ericsson] |
| TA validation | [Nokia] |
| UE transmit timing | [Huawei] |
| Big CR | [ZTE] |

- R4-2120338

- RAN4 does not introduce BFD/BFR requirements for SDT, unless there is RAN1/2 decision to support BFD/BFR for SDT

- RAN4 does not introduce any new UE RRM requirements for RA-SDT

- RAN4 to clarify that UE shall meet the existing requirements when performing 2-step/4-step RA-SDT.

- RAN4 introduce requirements for CG-SDT in a similar approach to LTE PUR

- RAN4 reuse the existing UE initial transmission requirements for CG-SDT

- RAN4 define two window sizes and leave to RAN2 to defines two thresholds for TA validation.

#### 2.4.2	Remaining Open issues

- See the WI objectives for general overview of remaining open issues

- Sync requirements for CG-SDT transmission

## 3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

NOTE: This section only needs to be filled in for WI/SIs where there is a corresponding relevant WI/SI in SA/CT.

## 3.1	SAx/CTs

#### N/A

#### 3.1.1	Agreements with cross-TSG impacts

N/A

#### 3.1.2	Remaining Open issues with cross-TSG impacts

NOTE: This section should also flag any critical dependencies that need TSG attention.

N/A

## 4.	References

NOTE:	This can be e.g. a list of all related Tdocs in the affected WGs since last TSG, references to LSs, produced TRs/TSs, the work/study item description or status reports of previous TSGs.

## 4.1	RAN1

RAN1#106bis-e

| R1-2110774 | RAN1 aspects for NR small data transmissions in INACTIVE state | Ericsson |
| --- | --- | --- |
| R1-2110812 | Physical layer aspects of CG-SDT | Huawei, HiSilicon |
| R1-2110973 | Remaining RAN1 impacts for small data transmission | vivo |
| R1-2111083 | Discussion on physical layer aspects of small data transmission | Spreadtrum Communications |
| R1-2111356 | Discussion on the remaining physical layer issues of small data transmission | ZTE, Sanechips |
| R1-2111379 | Remaining issues of physical layer aspects for SDT | Sony |
| R1-2111473 | Discussion on physical layer aspects of small data transmission | Intel Corporation |
| R1-2111539 | Physical layer aspects for NR small data transmissions in INACTIVE state | Xiaomi |
| R1-2111711 | Discussion on physical layer aspects for NR small data transmissions in INACTIVE state | Samsung |
| R1-2111844 | Discussion on physical layer aspects of small data transmission | Apple |
| R1-2112044 | Discussion on physical layer aspects of small data transmission | LG Electronics |
| R1-2112189 | Draft reply LS on the physical layer aspects of small data transmission | Qualcomm Incorporated |
| R1-2110495 | Summary on the physical layer aspects of small data transmission | Moderator (ZTE) |
| R1-2110661 | Reply LS on the physical layer aspects of small data transmission | RAN1 |

RAN1#107-e

| R1-2108752 | Physical layer aspects of CG-SDT | Huawei, HiSilicon |
| --- | --- | --- |
| R1-2108894 | Discussion on physical layer aspects of small data transmission | Spreadtrum Communications |
| R1-2108950 | Discussion on RAN1 impacts for small data transmission | vivo |
| R1-2109026 | Discussion on remaining physical layer issues of small data transmission | ZTE, Sanechips |
| R1-2109377 | Physical layer aspects for NR small data transmissions in INACTIVE state | Xiaomi |
| R1-2109465 | Discussion on physical layer aspects for NR small data transmissions in INACTIVE state | Samsung |
| R1-2109590 | Discussion on physical layer aspects of small data transmission | Intel Corporation |
| R1-2109727 | Discussion on L1 feedback for CG-SDT | Sierra Wireless. S.A. |
| R1-2109762 | Physical layer aspects for NR small data transmissions in INACTIVE state | L.M. Ericsson Limited |
| R1-2109771 | Remaining issues of physical layer aspects for SDT | Sony |
| R1-2109911 | Physical layer aspects of small data transmission | InterDigital, Inc. |
| R1-2109960 | Discussion on physical layer aspects of small data transmission | LG Electronics |
| R1-2110012 | Discussion on physical layer aspects of small data transmission | Apple |
| R1-2110164 | Discussion on PHY Impacts of SDT | Qualcomm Incorporated |
| R1-2110297 | On physical layer aspects of small data transmission | Nokia, Nokia Shanghai Bell |
| R1-2112779 | Summary on the physical layer aspects of small data transmission | Moderator (ZTE) |
| R1-2112781 | [Draft] Reply LS on the physical layer aspects of small data transmission | Moderator (ZTE) |
| R1-2112782 | Reply LS on the physical layer aspects of small data transmission | RAN1 |

## 4.2	RAN2

###### RAN2#116-e

| R2-2109308 | Reply LS on Small data transmission (C1-215152; contact: Apple) | CT1 |
| --- | --- | --- |
| R2-2109321 | Reply LS on on physical layer aspects of small data transmission (R1-2108533; contact: vivo) | RAN1 |
| R2-2109330 | LS on the TA validation and mapping details for CG-SDT (R1-2108649; contact: ZTE) | RAN1 |
| R2-2109437 | Further Discussion on User Plane Aspect of Small Data Transmission | vivo |
| R2-2109438 | Handling of non-SDT Data Arrival via BSR | vivo |
| R2-2109439 | Discussion on RRC-controlled Small Data Transmission | vivo |
| R2-2109440 | Supporting subsequent UL transmission during RA-SDT | vivo |
| R2-2109441 | Supporting Small Data Transmission via CG PUSCH | vivo |
| R2-2109524 | User Plane Common Aspects of RACH and CG based SDT | Samsung Electronics Co., Ltd |
| R2-2109525 | Control Plane Aspects of SDT Procedure | Samsung Electronics Co., Ltd |
| R2-2109526 | Handling legacy control plane operations during SDT procedure | Samsung Electronics Co., Ltd |
| R2-2109527 | RACH configuration for Small Data Transmission. | Samsung Electronics Co., Ltd |
| R2-2109528 | TAT-SDT expiry handing during the CG-SDT procedure | Samsung Electronics Co., Ltd |
| R2-2109591 | RACH based small data transmission | Ericsson |
| R2-2109592 | Details of CG based SDT | Ericsson |
| R2-2109593 | Common aspects for SDT | Ericsson |
| R2-2109594 | SDT Faliure Handling | Ericsson |
| R2-2109595 | CP aspects for SDT | Ericsson |
| R2-2109617 | DCCH-based indication of non-SDT data arrival | Intel Corporation, ZTE corporation, Sanechips, Samsung, CMCC, Qualcomm, OPPO, Sharp, Xiaomi, Sony, CATT, FGI, Asia Pacific Telecom, Radisys |
| R2-2109618 | Draft CR for introduction of DCCH solution for non-SDT data arrival | ZTE corporation, Sanechips, Intel Corporation, Samsung, CMCC, Qualcomm, OPPO, Sharp, Xiaomi, Sony, CATT, FGI, Asia Pacific Telecom, Radisys |
| R2-2109619 | DCCH vs CCCH based approach for indication of non-SDT data arrival | Intel Corporation, ZTE corporation, Sanechips, Samsung, Qualcomm, OPPO, Sharp, Xiaomi, Sony, CATT, Apple |
| R2-2109620 | Control plane leftover issues for SDT procedure | Intel Corporation |
| R2-2109621 | User plane leftover issues for SDT procedure | Intel Corporation |
| R2-2109622 | RA-SDT leftover issues | Intel Corporation |
| R2-2109623 | CG-SDT leftover issues | Intel Corporation |
| R2-2109645 | Discussion on left issue for CG-SDT resource release | SHARP Corporation |
| R2-2109711 | Remaining UP open issues | Fujitsu |
| R2-2109712 | Handling of SDTF detection timer | Fujitsu |
| R2-2109713 | RAN paging reception and response during SDT | Fujitsu |
| R2-2109768 | Discussion on user plane issues of SDT | OPPO |
| R2-2109769 | Discussion on control plane issues of SDT | OPPO |
| R2-2109770 | Discussion on swiching from RA-SDT to legacy RACH | OPPO |
| R2-2109771 | Discussion on the procedure of CG-SDT | OPPO |
| R2-2109772 | Discussion on handling of CG-SDT resources | OPPO |
| R2-2110030 | User plane aspects of SDT | Apple |
| R2-2110031 | Control plane aspects of SDT | Apple |
| R2-2110032 | SDT specific NAS and AS interaction | Apple |
| R2-2110033 | Power Saving for SDT | Apple |
| R2-2110034 | CG specific SDT procedure | Apple |
| R2-2110123 | Discussion on RACH-based SDT | Spreadtrum Communications |
| R2-2110182 | User plane common aspects for SDT | Huawei, HiSilicon |
| R2-2110183 | CG-based schemes for SDT | Huawei, HiSilicon |
| R2-2110184 | Discussion on the NAS aspects of Small Data | Huawei, HiSilicon |
| R2-2110185 | Running MAC CR for small data | Huawei, HiSilicon |
| R2-2110186 | Remaining issue for MAC spec | Huawei, HiSilicon |
| R2-2110187 | Summary of [Post115-e][507][SDT] MAC running CR update (Huawei) | Huawei, HiSilicon |
| R2-2110208 | C-RNTI handling for SDT | FGI, Asia Pacific Telecom |
| R2-2110209 | Remaining Issues on the Arrival of Non-SDT Traffic | FGI, Asia Pacific Telecom |
| R2-2110210 | Issues of the Subsequent Data Transmission | FGI, Asia Pacific Telecom |
| R2-2110245 | Further details on CG based small data transmission | Lenovo, Motorola Mobility |
| R2-2110248 | Additional aspects of CG based SDT | Ericsson |
| R2-2110254 | Remaining control plane aspects of SDT | NEC |
| R2-2110255 | Remaining user plane aspects of SDT | NEC |
| R2-2110328 | The UP common issues for small data transmissions | Lenovo, Motorola Mobility |
| R2-2110329 | Discussion on CP data transmission over SDT | Lenovo, Motorola Mobility |
| R2-2110330 | Analysis on open issues of RA based SDT | Lenovo, Motorola Mobility |
| R2-2110349 | Remaining issues of RACH-based SDT in NR | Sony |
| R2-2110397 | Consideration on UP remaining issues of SDT? | CATT |
| R2-2110398 | Consideration on NAS and AS Interaction | CATT |
| R2-2110399 | Consideration on CP issues | CATT |
| R2-2110400 | Anchor relocation during SDT | CATT |
| R2-2110401 | Remaining issues for CG-SDT | CATT |
| R2-2110572 | Control plane common aspects of SDT | ZTE Corporation, Sanechips |
| R2-2110573 | RRC Running CR for SDT | ZTE Corporation (rapporteur) |
| R2-2110574 | Open issues for CG based SDT | ZTE Corporation, Sanechips |
| R2-2110575 | User plane common aspects for SDT | ZTE Corporation, Sanechips |
| R2-2110576 | [DRAFT] Reply LS on the physical layer aspects of small data transmission | ZTE Corporation, Sanechips |
| R2-2110594 | Small data transmission with RA-based schemes | Huawei, HiSilicon |
| R2-2110595 | Control plane common aspects for SDT | Huawei, HiSilicon |
| R2-2110596 | Non-SDT data arrival | Huawei, HiSilicon, InterDigital, LGE, Ericsson, ASUSTeK, Nokia, Nokia Shanghai Bell, Google, Rakuten Mobile, Fujitsu, NEC |
| R2-2110624 | Discussion on RA-based small data transmission | Google Inc. |
| R2-2110625 | Discussion on CG-based small data transmission | Google Inc. |
| R2-2110667 | Clarification on the data volume computation | Xiaomi Communications |
| R2-2110668 | Paging reception during SDT | Xiaomi Communications |
| R2-2110669 | RACH failure in subsequent data transmission phase | Xiaomi Communications |
| R2-2110670 | Summary of [Post115-e][509][SDT] CG open issues (Xiaomi) | Xiaomi Communications |
| R2-2110671 | Remaining issues of CG SDT in RAN2 | Xiaomi Communications |
| R2-2110752 | Remaining issues on UP aspects of SDT | Qualcomm Incorporated |
| R2-2110753 | Remaining issues on CP aspects of SDT | Qualcomm Incorporated |
| R2-2110760 | Remaining issues on RACH based SDT | Qualcomm Incorporated |
| R2-2110761 | Remaining issues on CG based SDT | Qualcomm Incorporated |
| R2-2110764 | CG-SDT Switch to RA during subsequent transmissions | NEC Telecom MODUS Ltd. |
| R2-2110797 | Draft LS to CT1 on small data transmission | Apple |
| R2-2110808 | Stage-2 running CR Introduction of SDT | Nokia, Nokia Shanghai Bell |
| R2-2110809 | UP aspects for SDT | Nokia, Nokia Shanghai Bell |
| R2-2110810 | RA specific aspects for SDT | Nokia, Nokia Shanghai Bell |
| R2-2110818 | SDT control plane aspects | Nokia, Nokia Shanghai Bell |
| R2-2110819 | RRC procedure for SDT | Nokia, Nokia Shanghai Bell |
| R2-2110865 | Untreated proposal from [Post113-e][503] | InterDigital |
| R2-2110914 | CG-based SDT selection and configuration | InterDigital |
| R2-2110915 | User plane aspects of small data transmission | InterDigital |
| R2-2110961 | Discussion on open issues for CG based SDT | China Telecommunications |
| R2-2110983 | Handling of legacy TAT and CG-SDT-TAT | LG Electronics Inc. |
| R2-2110984 | Switching cases of SDT and non-SDT | LG Electronics Inc. |
| R2-2110986 | Remaining CG-SDT issues in SDT | LG Electronics Inc. |
| R2-2111002 | Discussion on fallback to legacy RA for RA-SDT | ASUSTeK |
| R2-2111031 | Aspects specific to CG-SDT | Nokia, Nokia Shanghai Bell |
| R2-2111038 | Discussion on RACH based SDT | CMCC |
| R2-2111039 | Leftover UP common issues of SDT | CMCC |
| R2-2111124 | Remaining UP issues in SDT | LG Electronics Inc. |
| R2-2111125 | Autonomous retransmission in CG-SDT | LG Electronics Inc. |
| R2-2111185 | Discussion on CG based Small Data Transmission | TCL Communication Ltd. |
| R2-2111199 | Open issues for CG based SDT | ZTE Corporation, Sanechips |
| R2-2111219 | Reply LS on the physical layer aspects of small data transmission (R1-2110661; contact: ZTE) | RAN1 |
| R2-2111275 | Comments on the proposed CCCH solution for non-SDT data arrival | Intel Corporation, ZTE corporation, Apple |
| R2-2111446 | LS on the ROHC continuity for SDT | RAN2 |
| R2-2111509 | Further clarifications on non-SDT arrival solutions | Huawei, HiSilicon, InterDigital, LGE, Ericsson, ASUSTeK, Nokia, Nokia Shanghai Bell, Google, NEC, Fujitsu, Rakuten Mobile |
| R2-2111519 | [AT116-e][503][SData] Summary of UP SDT open issues | LG Electronics Inc. (Rapporteur) |
| R2-2111523 | Comments to R2-2111509 for non-SDT data arrival | Intel Corporation, ZTE corporation, Sanechips, Samsung, Xiaomi, Qualcomm, Apple, Radisys |
| R2-2111611 | Reply LS on the physical layer aspects of small data transmission | RAN2 |

## 4.3	RAN3

RAN3#114-e

| R3-214844 | Work plan for the INACTIVE small data WI | ZTE |
| --- | --- | --- |
| R3-214845 | Discussion on RACH based SDT | ZTE |
| R3-214846 | Discussion on CG based SDT | ZTE |
| R3-214847 | BLCR 38.300 for support of SDT | ZTE |
| R3-214848 | BLCR 38.401 for support of SDT | ZTE |
| R3-214882 | CR to F1AP on Rel-17 SDT | Samsung |
| R3-214883 | Discussion on RA-based small data transmission | Samsung |
| R3-214884 | Discussion on other issues related to SDT | Samsung |
| R3-214904 | Further discussion on RAN3 impacts of RACH based SDT | Qualcomm Incorporated |
| R3-215000 | (TPs to TS 38.300 38.473 38.401BL CRs) Support of RACH based SDT | Huawei |
| R3-215001 | Supporting of RACH based SDT | Huawei |
| R3-215002 | (TP to TS 38.473 BL CR) Support of CG based SDT | Huawei |
| R3-215003 | Support of NR Small Data Transmissions in INACTIVE state | Huawei |
| R3-215279 | Context fetch and data forwarding for NR SDT | Ericsson |
| R3-215280 | Support of NR SDT over Xn | Ericsson |
| R3-215281 | F1 impacts by supporting CG-based SDT | Ericsson |
| R3-215282 | Support of NR SDT over F1 | Ericsson |
| R3-215319 | SDT Related Procedures | Lenovo, Motorola Mobility |
| R3-215320 | Support of CG-SDT | Lenovo, Motorola Mobility |
| R3-215321 | DL non-SDT data and signalling arrival during SDT procedure | Lenovo, Motorola Mobility |
| R3-215356 | Support of RACH-based SDT | Nokia, Nokia Shanghai Bell |
| R3-215368 | Support of RACH-based SDT | Nokia, Nokia Shanghai Bell |
| R3-215369 | Support of CG-based SDT | Nokia, Nokia Shanghai Bell |
| R3-215370 | Support of CG-based SDT | Nokia, Nokia Shanghai Bell |
| R3-215394 | Discussion on CG-based small data transmission | Samsung |
| R3-215599 | Discussion on RA-based SDT with anchor relocation | CATT |
| R3-215600 | Discussion on RA-based SDT without anchor relocation | CATT |
| R3-215601 | Support of RA-based SDT | CATT |
| R3-215602 | Discussion on CG based SDT in CU-DU split architecture | CATT |
| R3-215672 | Discussion of E1 impact on CG-based SDT | China Telecom |
| R3-215709 | Consideration on small data transmission without anchor relocation | LG Electronics |
| R3-215710 | Support of RACH-based SDT over Xn | LG Electronics |
| R3-215711 | Discussion on CG-based SDT in CU-DU split | LG Electronics |
| R3-215712 | Support of CG-SDT in CU-DU split | LG Electronics |
| R3-215745 | Context retrieval and data forwarding for RACH based SDT | Intel Corporation |
| R3-215746 | Assistance Information from New gNB for RACH based SDT | Intel Corporation |
| R3-215747 | [Draft] SDT BL CR to 38.423 | Intel Corporation |
| R3-215748 | CG based SDT impact on CU-DU split | Intel Corporation |
| R3-215750 | CR to 38.463 on support of SDT in E1 interface | China Telecomunication Corp. |
| R3-215893 | CB: # SDT1_RACHbased - Summary of email discussion | ZTE - moderator |
| R3-215894 | CB: # SDT2_CGbased - Summary of email discussion | LG Electronics - moderator |
| R3-215895 | CB: # SDT3_others - Summary of email discussion | Samsung - moderator |
| R3-215996 | RA-SDT BLCR to TS 38.300 | ZTE |
| R3-215997 | RA-SDT BLCR to TS 38.401 | Intel Corporation |
| R3-215998 | RA-SDT BLCR to TS 38.423 | Ericsson |
| R3-215999 | RA-SDT BLCR to TS 38.473 | Nokia |
| R3-216000 | RA-SDT BLCR to TS 38.463 | China Telecommunication |
| R3-216012 | CB: # SDT1_RACHbased - Summary of email discussion | ZTE - moderator |
| R3-216022 | CB: # SDT1_RACHbased - Summary of email discussion | ZTE - moderator |
| R3-216023 | CB: # SDT2_CGbased - Summary of email discussion | LG Electronics - moderator |
| R3-216024 | CB: # SDT3_others - Summary of email discussion | Samsung - moderator |
| R3-216059 | CG-SDT BLCR to TS 38.401 | Huawei |
| R3-216060 | CG-SDT BLCR to TS 38.473 | Samsung |
| R3-216069 | (TP for RA-SDT BLCR to TS 38.473) Support of RA-based SDT | Qualcomm |
| R3-216070 | (TP for RA-SDT BLCR to TS 38.300) Support of RA-based SDT | CATT |
| R3-216071 | (TP for RA-SDT BLCR to TS 38.401) Support of RA-based SDT | Huawei |
| R3-216081 | (TP for RA-SDT BLCR to TS 38.423) Support of RA-based SDT | Nokia, Nokia Shanghai Bell |
| R3-216229 | CB: # SDT1_RACHbased - Summary of email discussion | ZTE - moderator |
| R3-216274 | RA-SDT BLCR to TS 38.300 | ZTE |
| R3-216275 | RA-SDT BLCR to TS 38.401 | Intel Corporation |
| R3-216276 | RA-SDT BLCR to TS 38.423 | Ericsson |
| R3-216277 | RA-SDT BLCR to TS 38.473 | Nokia, Nokia Shanghai Bell |

## 4.1	RAN4

RAN4#101-e

| R4-2118590 | RRM requirements for SDT | ZTE |
| --- | --- | --- |
| R4-2118591 | Workplan for SDT RRM requirements | ZTE |
| R4-2118961 | On RRM core requirements for SDT | Nokia, Nokia Shanghai Bell |
| R4-2119066 | Overview of RRM impact of Small Data Transmissions | Ericsson |
| R4-2119067 | Overview of TA validation requirements for Small Data Transmissions | Ericsson |
| R4-2119194 | Discussion on reply LS on Beam correspondence with Small Data Transmission in Inactive State | ZTE |
| R4-2119364 | Initial discussion on RRM impacts of SDT | Huawei, Hisilicon |
| R4-2120339 | Revised workplan for SDT RRM requirements (Approved) | ZTE |
| R4-2120338 |  | ZTE |
| R4-2120235 | Summary on [101-e][240] NR_SmallData_INACTIVE_NWM (1st round) | Moderator(ZTE) |
| R4-2120382 | Summary on [101-e][240] NR_SmallData_INACTIVE_NWM (2nd round) | Moderator(ZTE) |

END