3GPP TSG-RAN WG4 Meeting #101-e		   R4-2118480

Electronic Meeting, 1th– 12th November, 2021

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-1 | CR | 0958 | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Big CR on Introduction of completed 5 bands inter-band CA into TS 38.101-1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_CADC_R17_5BDL_xBUL-Core |  |  |  |  |  | Date: |  |  | 2021-11-15 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | This CR is to introduce the five bands inter-band CA combinations as below: CA_n1A-n3A-n5A-n7A-n78A CA_n1A-n3A-n5A-n7B-n78A The UL configurations for CA_n1A-n3A-n7B-n28A-n78A |  |  |  |  |  |  |  |  |
| Summary of change: |  | The specific RF requirements for the following band combinations have been specified. CA_n1A-n3A-n5A-n7A-n78A CA_n1A-n3A-n5A-n7B-n78A The UL configurations for CA_n1A-n3A-n7B-n28A-n78A |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The new five bands inter-band CA combinations can’t be supported in RF spec. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2A.2.4, 5.5A.3.4, 6.2A.4.2.6, 7.3A.3.2.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521-1 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<Start of Change>>

5.2A.2.4	Inter-band CA (five bands)

Table 5.2A.2.4-1: Inter-band CA operating bands involving FR1 (five bands)

| NR CA Band | NR Band (Table 5.2-1) |
| --- | --- |
| CA_n1-n3-n5-n7-n78 | n1, n3, n5, n7, n78 |
| CA_n1-n3-n7-n28-n78 | n1, n3, n7, n28, n78 |

<<Next of Change>>

5.5A.3.4	Configurations for inter-band CA (five bands)

Table 5.5A.3.4-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (five bands)

| NR CA configuration | Uplink CA configuration | NR Band | Channel bandwidth (MHz) (NOTE 1) |  |  |  |  |  |  |  |  |  |  |  |  | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
|  | CA_n1A-n3A CA_n1A-n5A | n1 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |
|  | CA_n1A-n7A CA_n1A-n78A | n3 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  |  |
| CA_n1A-n3A-n5A-n7A-n78A | CA_n3A-n5A CA_n3A-n7A | n5 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 0 |
|  | CA_n3A-n78A CA_n5A-n7A | n7 | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 |  |  |  |  |  |  |
|  | CA_n5A-n78A CA_n7A-n78A | n78 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
|  | CA_n1A-n3A CA_n1A-n5A | n1 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |
|  | CA_n1A-n7A CA_n1A-n78A | n3 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  |  |
| CA_n1A-n3A-n5A-n7B-n78A | CA_n3A-n5A CA_n3A-n7A | n5 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 0 |
|  | CA_n3A-n78A CA_n5A-n7A | n7 | See CA_n7B bandwidth combination set 0 in Table 5.5A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CA_n5A-n78A CA_n7A-n78A CA_n7B | n78 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
| CA_n1A-n3A-n7A-n28A-n78A | - | n1 | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 |  |  |  |  |  | 0 |
|  |  | n3 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  |  |
|  |  | n7 | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 |  |  |  |  |  |  |
|  |  | n28 | 5 | 10 | 15 | 20 |  | 30 |  |  |  |  |  |  |  |  |
|  |  | n78 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
|  | CA_n1A-n3A CA_n1A-n7A CA_n1A-n28A CA_n1A-n78A | n1 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 1 |
|  | CA_n3A-n7A CA_n3A-n28A CA_n3A-n78A | n3 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  |  |
|  | CA_n7A-n28A CA_n7A-n78A | n7 | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 |  |  |  |  |  |  |
|  | CA_n28A-n78A | n28 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
| CA_n1A-n3A-n7B-n28A-n78A | CA_n1A-n3A CA_n1A-n7A CA_n1A-n28A CA_n1A-n78A. CA_n3A-n7A CA_n3A-n28A CA_n3A-n78A. CA_n7A-n28A CA_n7A-n78A. CA_n28A-n78A CA_n7B. | n1 | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 |  |  |  |  |  | 0 |
|  |  | n3 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  |  |
|  |  | n7 | See CA_n7B Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n28 | 5 | 10 | 15 | 20 |  | 30 |  |  |  |  |  |  |  |  |
|  |  | n78 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
| NOTE 1: The SCS of each channel bandwidth for NR band refers to Table 5.3.5-1. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

<<Next of Change>>

6.2A.4.2.6	ΔTIB,c for Inter-band CA (five bands)

Table 6.2A.4.2.6-1: ΔTIB,c due to NR CA (five bands)

| Inter-band CA combination | NR Band | ΔTIB,c (dB) |
| --- | --- | --- |
|  | n1 | 0.6 |
|  | n3 | 0.6 |
| CA_n1-n3-n5-n7-n78 | n5 | 0.6 |
|  | n7 | 0.6 |
|  | n78 | 0.8 |
| CA_n1-n3-n7-n28-n78 | n1 | 0.7 |
|  | n3 | 0.7 |
|  | n7 | 0.7 |
|  | n28 | 0.6 |
|  | n78 | 0.8 |

<<Next of Change>>

7.3A.3.2.5	ΔRIB,c for five bands

Table 7.3A.3.2.5-1: ΔRIB,c due to CA (five bands)

| Inter-band CA combination | NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
|  | n1 | 0.2 |
|  | n3 | 0.2 |
| CA_n1-n3-n5-n7-n78 | n5 | 0.2 |
|  | n7 | 0.2 |
|  | n78 | 0.5 |
| CA_n1-n3-n7-n28-n78 | n1 | 0.2 |
|  | n3 | 0.2 |
|  | n7 | 0.2 |
|  | n28 | 0.2 |
|  | n78 | 0.5 |

<<End of Change>>