3GPP TSG RAN Meeting #94-e	RP-212939

e-meeting,  6-17th December 2021

Source: 	Thales, Hughes Network Systems, Intelsat, Inmarsat, ESA, Sateliot, Leonardo, Omnispace

Title:	NTN specific requirements in RAN4 NR related TS

TDOC Type:	discussion

Agenda Item:	9.3.2.2 - Solutions for NR to support NTN [RAN2 WI: NR_NTN_solutions]

Document for:	Decision

Release:	Rel-17

# Introduction

During RAN#93-e, an email discussion took place which outcomes is captured in RP-212616 “Moderator's summary for discussion [93e-34-NR-NTN-WID]”. It resulted in the following proposal

- Proposal 2: In view of the diversity of expressed views, further discuss on line under RAN chair arbitration

  - RAN#93-e to take the decision about incorporating NTN specific requirements in new RAN4 TS versus in existing RAN4 TS (38.101-1, 38.133)

  - Postpone decision to RAN#94-e to allocate more time for companies to digest the PROs and CONs of each option

During the on line RAN#93-e session, it was agreed that proposal 2 will be postponed to RAN #94e.

This TDOC discuss further on how to implement NTN specific requirements in 3GPP RAN4 NR technical specifications.

# Discussion

There is common understanding as well as in the working assumptions that NTN capable handheld UE operating in FR1 are capable to operate in “Terrestrial Network”. The NTN characteristics and features will be adapted in 3GPP UE specifications.

In line with this understanding and working assumptions, RAN2 & RAN3 are discussing definition of enablers to support NTN - Terrestrial Network mobility.

See objective “Service continuity for mobility from TN to NTN and from NTN to TN systems (to be addressed when connected mode mobility has sufficiently progressed)” in the WID [1].

The recent RAN4 coexistence analysis on FR1 has demonstrated that ACS and ACLR requirements applying to UE operating with NTN are lower than the ones of UE operating with “Terrestrial Network”.

Moreover, it is assumed that operation of UE with FR1 NTN can be supported with the same sensitivity and transmit power compared to operation with FR1 “Terrestrial Network”.

Therefore the inclusion of NTN specific requirements in TS 38.101-1 [2] will likely be limited to the addition of NTN bands while other requirements will remain the same.

As approved in the NTN-NR Rel-17 WID [1], the NTN UE requirements will be incorporated into the existing NR TS, including TS 38.101-1 [2] and TS 38.133 [3] unless stated otherwise.

Besides when comparing the approach in terms of complexity, baseline is simpler to implement:

- Baseline: Implementation of NTN specifics in 3GPP TS 38.101-1 would add reference to the NTN bands (e.g. n255, n256) in some different tables (see in annex an example of modification to Table 5.2-1: NR operating bands in FR1 in clause 5.2 Operating bands) => Typical RAN4 process.

- Alternative: Creation of a new TS for NTN capable UE requirements would lead to copy selected parts of the TS 38.101-1 and add the NTN specific requirements => Specific process that may lead to numerous discrepancies and errors.

Proposal 1: As approved in the NTN-NR Rel-17 WID, NTN RF specific requirements for UE operating in FR1 will be included in TS 38.101-1.

For non-FR1 band, there has not been any meaningful discussion on the handling of NTN specific requirements for UE operating in non-FR1.

Proposal 2: Handling of NTN specific requirements for UE operating in non-FR1 may be  assessed in future release.

As per RRM NTN specific requirements, RAN4 has already decided that they will be added in the TS 38.133 [3].

Proposal 3: As approved in the NTN-NR Rel-17 WID, NTN RRM specific requirements will be included in TS 38.133.

# Conclusion

For handling of NTN specific requirements in 3GPP RAN4 NR technical specifications, the following is proposed:

Proposal 1: As approved in the NTN-NR Rel-17 WID, NTN RF specific requirements for UE operating in FR1 will be included in TS 38.101-1.

Proposal 2: Handling of NTN specific requirements for UE operating in non-FR1 may be  assessed in future release.

Proposal 3: As approved in the NTN-NR Rel-17 WID, NTN RRM specific requirements will be included in TS 38.133.

# References

[1]	RP-211784 Rel-17 WI Solutions for NR to support non-terrestrial networks (NTN)

[2]	TS 38.101-1 “NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone”

[3]	TS 38.133 “NR; Requirements for support of radio resource management”

# Annex: example of modification to TS 38.101-1

Table 5.2-1: NR operating bands in FR1

| NR operating band | Uplink (UL) operating band BS receive / UE transmit FUL_low – FUL_high | Downlink (DL) operating band BS transmit / UE receive FDL_low – FDL_high | Duplex Mode |
| --- | --- | --- | --- |
| n1 | 1920 MHz – 1980 MHz | 2110 MHz – 2170 MHz | FDD |
| n2 | 1850 MHz – 1910 MHz | 1930 MHz – 1990 MHz | FDD |
| n3 | 1710 MHz – 1785 MHz | 1805 MHz – 1880 MHz | FDD |
| n5 | 824 MHz – 849 MHz | 869 MHz – 894 MHz | FDD |
| n7 | 2500 MHz – 2570 MHz | 2620 MHz – 2690 MHz | FDD |
| n8 | 880 MHz – 915 MHz | 925 MHz – 960 MHz | FDD |
| n12 | 699 MHz – 716 MHz | 729 MHz – 746 MHz | FDD |
| n13 | 777 MHz – 787 MHz | 746 MHz – 756 MHz | FDD |
| n14 | 788 MHz – 798 MHz | 758 MHz – 768 MHz | FDD |
| n18 | 815 MHz – 830 MHz | 860 MHz – 875 MHz | FDD |
| n20 | 832 MHz – 862 MHz | 791 MHz – 821 MHz | FDD |
| n2416 | 1626.5 MHz – 1660.5 MHz | 1525 MHz – 1559 MHz | FDD |
| n25 | 1850 MHz – 1915 MHz | 1930 MHz – 1995 MHz | FDD |
| n26 | 814 MHz – 849 MHz | 859 MHz – 894 MHz | FDD |
| n28 | 703 MHz – 748 MHz | 758 MHz – 803 MHz | FDD |
| n29 | N/A | 717 MHz – 728 MHz | SDL |
| n303 | 2305 MHz – 2315 MHz | 2350 MHz – 2360 MHz | FDD |
| n34 | 2010 MHz – 2025 MHz | 2010 MHz – 2025 MHz | TDD |
| n3810 | 2570 MHz – 2620 MHz | 2570 MHz – 2620 MHz | TDD |
| n39 | 1880 MHz – 1920 MHz | 1880 MHz – 1920 MHz | TDD |
| n40 | 2300 MHz – 2400 MHz | 2300 MHz – 2400 MHz | TDD |
| n41 | 2496 MHz – 2690 MHz | 2496 MHz – 2690 MHz | TDD |
| n46 | 5150 MHz – 5925 MHz | 5150 MHz – 5925 MHz | TDD13 |
| n4711 | 5855 MHz – 5925 MHz | 5855 MHz – 5925 MHz | TDD |
| n48 | 3550 MHz – 3700 MHz | 3550 MHz – 3700 MHz | TDD |
| n50 | 1432 MHz – 1517 MHz | 1432 MHz – 1517 MHz | TDD1 |
| n51 | 1427 MHz – 1432 MHz | 1427 MHz – 1432 MHz | TDD |
| n53 | 2483.5 MHz – 2495 MHz | 2483.5 MHz – 2495 MHz | TDD |
| n65 | 1920 MHz – 2010 MHz | 2110 MHz – 2200 MHz | FDD4 |
| n66 | 1710 MHz – 1780 MHz | 2110 MHz – 2200 MHz | FDD |
| n67 | N/A | 738 MHz – 758 MHz | SDL |
| n70 | 1695 MHz – 1710 MHz | 1995 MHz – 2020 MHz | FDD |
| n71 | 663 MHz – 698 MHz | 617 MHz – 652 MHz | FDD |
| n74 | 1427 MHz – 1470 MHz | 1475 MHz – 1518 MHz | FDD |
| n75 | N/A | 1432 MHz – 1517 MHz | SDL |
| n76 | N/A | 1427 MHz – 1432 MHz | SDL |
| n7712 | 3300 MHz – 4200 MHz | 3300 MHz – 4200 MHz | TDD |
| n78 | 3300 MHz – 3800 MHz | 3300 MHz – 3800 MHz | TDD |
| n79 | 4400 MHz – 5000 MHz | 4400 MHz – 5000 MHz | TDD |
| n80 | 1710 MHz – 1785 MHz | N/A | SUL |
| n81 | 880 MHz – 915 MHz | N/A | SUL |
| n82 | 832 MHz – 862 MHz | N/A | SUL |
| n83 | 703 MHz – 748 MHz | N/A | SUL |
| n84 | 1920 MHz – 1980 MHz | N/A | SUL |
| n85 | 698 MHz – 716 MHz | 728 MHz – 746 MHz | FDD |
| n86 | 1710 MHz – 1780 MHz | N/A | SUL |
| n89 | 824 MHz – 849 MHz | N/A | SUL |
| n90 | 2496 MHz – 2690 MHz | 2496 MHz – 2690 MHz | TDD5 |
| n91 | 832 MHz – 862 MHz | 1427 MHz – 1432 MHz | FDD9 |
| n92 | 832 MHz – 862 MHz | 1432 MHz – 1517 MHz | FDD9 |
| n93 | 880 MHz – 915 MHz | 1427 MHz – 1432 MHz | FDD9 |
| n94 | 880 MHz – 915 MHz | 1432 MHz – 1517 MHz | FDD9 |
| n958 | 2010 MHz – 2025 MHz | N/A | SUL |
| n9614 | 5925 MHz – 7125 MHz | 5925 MHz – 7125 MHz | TDD13 |
| n9715 | 2300 MHz – 2400 MHz | N/A | SUL |
| n9815 | 1880 MHz – 1920 MHz | N/A | SUL |
| n9916 | 1626.5 MHz – 1660.5 MHz | N/A | SUL |
| n25517 | 1626.5 MHz – 1660.5 MHz | 1525 MHz – 1559 MHz | FDD |
| n25617 | 1980 MHz – 2010 MHz | 2170 MHz – 2200 MHz | FDD |
| NOTE 1: UE that complies with the NR Band n50 minimum requirements in this specification shall also comply with the NR Band n51 minimum requirements. NOTE 2: UE that complies with the NR Band n75 minimum requirements in this specification shall also comply with the NR Band n76 minimum requirements. NOTE 3: Uplink transmission is not allowed at this band for UE with external vehicle-mounted antennas. NOTE 4: A UE that complies with the NR Band n65 minimum requirements in this specification shall also comply with the NR Band n1 minimum requirements. NOTE 5: Unless otherwise stated, the applicability of requirements for Band n90 is in accordance with that for Band n41; a UE supporting Band n90 shall meet the requirements for Band n41. A UE supporting Band n90 shall also support band n41. NOTE 6: A UE that supports NR Band n66 shall receive in the entire DL operating band. NOTE 7: A UE that supports NR Band n66 and CA operation in any CA band shall also comply with the minimum requirements specified for the DL CA configurations CA_n66B and CA_n66(2A) in the current version of the specification. NOTE 8: This band is applicable in China only. NOTE 9: Variable duplex operation does not enable dynamic variable duplex configuration by the network, and is used such that DL and UL frequency ranges are supported independently in any valid frequency range for the band. NOTE 10: When this band is used for V2X SL service, the band is exclusively used for NR V2X in particular regions. NOTE 11: This band is unlicensed band used for V2X service. There is no expected network deployment in this band. NOTE 12: In the USA this band is restricted to 3700 – 3980 MHz. NOTE 13: This band is restricted to operation with shared spectrum channel access as defined in 37.213. NOTE 14: This band is applicable in the USA only subject to FCC Report and Order FCC 20-51 NOTE 15: The requirements for this band are applicable only where no other NR or E-UTRA TDD operating band(s) are used within the frequency range of this band in the same geographical area. For scenarios where other NR or E-UTRA TDD operating band(s) are used within the frequency range of this band in the same geographical area, special co-existence requirements may apply that are not covered by the 3GPP specifications. NOTE 16: DL operation in this band is restricted to 1526 – 1536 MHz and UL operation is restricted to 1627.5 – 1637.5 MHz and 1646.5 – 1656.5 MHz. NOTE 17: This band is used for satellite service (NR NTN) |  |  |  |

END