3GPP TSG RAN meeting #94e										RP-212953

Electronic Meeting, December 6-17, 2021

## Status Report to TSG

Agenda item:			9.3.2.1

| WI / SI Name | Enhanced Industrial Internet of Things (IoT) and ultra-reliable and low latency communication (URLLC) support for NR |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: Yes | Performance part: Yes |  | Testing part: No |
| Acronym | NR_IIOT_URLLC_enh |  |  |  |  |
| Unique ID | 860045 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-210854 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: | Core part: 03/2022 | Performance part: 09/2022 | Testing part: |  |
| RAN1 Completion level |  | 90% |  |  |  |
| Overall Completion level | Study Item: | Core part: 65% | Performance Part: 0% | Testing part: |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | RAN2 |
| --- | --- | --- |
| Rapporteur | Name | Ping-Heng Wallace Kuo |
|  | Company | Nokia |
|  | Email | Ping-Heng.Kuo@nokia.com |

## 1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

## 2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

## 2.1	RAN1

#### 2.1.1	Agreements

RAN1#106bis-e

| UE feedback enhancements for HARQ-ACK (RAN1 AI 8.3.1.1) | Agreement For PUCCH carrier switching, support PUCCH carrier switching only among different TDD cells with PUCCH configured on the NUL carrier in Rel-17 Agreement For semi-static PUCCH cell switching, PCell / PSCell / PUCCH-SCell is reference cell: - The time domain pattern configurations are based on the numerology of the reference cell. - The PDSCH to HARQ-ACK offset k1 is interpreted based on the numerology and PUCCH configuration of a reference cell to be able to apply the time-domain PUCCH cell switching pattern. - Note: There may not be a need to define a ‘reference cell’ in the specification. This terminology is used for further clarifications of the procedure. Agreement For PUCCH cell switching, support independent TPC per PUCCH cell including - Separate P0 / TPC configuration per PUCCH cell - Note: This flexibility is already provided as PUCCH-config is per UL BWP of a PUCCH cell - Accumulating closed loop power control commands only within the same PUCCH target cell by reusing Rel-15 procedure, i.e. - For dynamic PUCCH cell indication, the TPC command in the DCI scheduling the PUCCH only applies for the dynamically indicated PUCCH target cell - For semi-static / time-domain pattern, the TPC command in the DCI scheduling the PUCCH only applies for the determined PUCCH target (using the time-domain pattern) - Separate TPC command indication using DCI format 2_2 for the individual PUCCH cells - Note: this requires configuration of individual TPC command starting points for each PUCCH cell within DCI format 2_2 Agreement UE does not expect overlapping PUCCH slots with dynamic PUCCH cell indication on more than one cell, i.e., gNB should only dynamically indicate a single PUCCH cell for a final PUCCH slot. Agreement For semi-static PUCCH cell switching, the time-domain pattern configuration is based on the following properties: - A single time-domain pattern is configured per PUCCH cell group - The granularity of the time-domain pattern is one slot of the PCell / PSCell / PUCCH-SCell - The time-domain pattern is applied periodically - FFS on period / pattern length (e.g., 10ms, RRC configured, …). - The pattern defines for each slot of the PCell / PSCell / PUCCH-SCell at least the applicable target PUCCH cell Agreement For semi-static PUCCH cell switching, the PUCCH resource indicator (PRI) is interpreted based on the PUCCH configuration of determined target PUCCH cell. Conclusion For SPS HARQ-ACK deferral, only SPS HARQ-ACK bits subject to deferral from one or more initial slots which have not reached the maximum deferral value are jointly deferred to the next available PUCCH (other SPS HARQ-ACK is dropped). Agreement For SPS HARQ-ACK deferral, the bit ordering of deferred SPS HARQ-ACK information from one or more initial slots in the target PUCCH slot is based on the Rel.16 SPS HARQ-ACK bit order principle as in clause 9.1.2 of TS38.213 is applied, i.e., based on serving cell index, SPS configuration index, SPS PDSCH slot index. Conclusion No additional enhanced Type 3 CB ‘types’ (such as activated CCs, of specific SPS configurations, etc.) in terms of RRC configuration are supported. Agreement For one enhanced Type 3 HARQ-ACK CB, the same CBG and NDI configuration applies to both PHY priorities following the RAN1#106-e agreement. Agreement The same set of enhanced Type 3 CBs (incl. CBG and NDI configuration) is applied for triggering using DCI format 1_1 and 1_2. Agreement Reuse the legacy 1-bit ‘one-shot HARQ-ACK request’ for triggering indication of the enhanced Type 3 HARQ-ACK CB of smaller size. - At least if only a single enhanced Type 3 HARQ-ACK CB is configured, the triggering DCI with the triggering bit set to ‘1’ is also able to schedule PDSCH. Agreement Support triggering of one-shot HARQ re-transmission on PUCCH using DCI format 1_2. Agreement To align with Rel-16 slot-based PUCCH repetition operation, support sub-slot based PUCCH repetition configured with / using nrofSlots (i.e., not using dynamic indication) of all UCI types (incl. HARQ, SR & CSI). Agreement For sub-slot based PUCCH repetition, the following agreement from Cov. Enh. WI for slot-based PUCCH repetition is adopted also for sub-slot based PUCCH repetition: Agreement - for a PUCCH resource, if both a new repetition parameter corresponding to Rel-17 dynamic PUCCH repetition factor indication and the Rel-15/16 nrofSlots are configured, the new repetition parameter overrides nrofSlots.. Agreement For sub-slot based PUCCH repetition, the following agreement from Cov. Enh. WI for slot-based PUCCH repetition is adopted also for sub-slot based PUCCH repetition: Agreement: Dynamic PUCCH repetition factor indication for SR or P/SP-CSI on PUCCH is not supported in Rel-17.. Agreement For PUCCH cell switching based on dynamic indication in the DCI, introduce a new, dedicated DCI field for the DCI scheduling PDSCH to indicate the target PUCCH cell. Agreement In addition, the dynamic target PUCCH cell indication also applies to HARQ-ACK corresponding to SCell dormancy indication without scheduling PDSCH. Agreement The periodicity / length of the time-domain pattern for semi-static PUCCH cell switching is directly determined by the RRC configuraton of the time domain pattern pucchCellPattern - Note: pucchCellPattern has a variable length of (1… maxNrofSlots) Agreement For semi-static and dynamic indication of PUCCH cell switching, the PUCCH repetition factor is determined based on the PUCCH format or PUCCH resource on the target PUCCH cell for the first repetition. Agreement The CBG and NDI usage can be independently configured for different enhanced Type 3 HARQ-ACK CBs. Agreement For Type-1 HARQ-ACK codebook for sub-slot based PUCCH configuration in Rel-17, the TDRA pruning/grouping is performed per DL slot after TDRA determination per sub-slot. - Strive to minimize the impact on relevant pseudo-code Conclusion For SPS HARQ-ACK deferral, the operation in the ‘initial’ slot is further clarified as: - The UE performs first the (Rel-16) UCI multiplexing operation. If after the UCI multiplexing operation into a PUCCH or PUSCH if any, and if the UE would be transmitting SPS HARQ-ACK using the PUCCH SPS-PUCCH-AN-List-r16 or n1PUCCH-AN which is not valid, the SPS HARQ-ACK configured for deferral is deferred. Agreement The maximum number of simultaneously configurable enhanced Type 3 CB is indicated by the UE through UE capability signaling from the set of {1, 2, 4, 8}. Agreement PUCCH cell switching between 2 cells is supported in Rel-17. Agreement For PUCCH repetition enhancements: - Support inter-slotFrequencyHopping for PUCCH repetition operation of PUCCH Format 0 and Format 2 for slot-based PUCCH configurations. - Support inter-subslot Frequency Hopping for PUCCH repetition operation of PUCCH Formats 0, 1, 2, 3 and 4 for 7OS slot-based PUCCH configurations. - The UE applies the inter-subslot FH operation from sub-slot to sub-slot, if configured with inter-slotFrequencyHopping in the respective PUCCH_config. - (Working Assumption) Support inter-subslot Frequency Hopping for PUCCH repetition operation of PUCCH Format 0 and Format 2 for 2OS slot-based PUCCH configurations. - The UE applies the inter-subslot FH operation from sub-slot to sub-slot, if configured with inter-slotFrequencyHopping in the respective PUCCH_config. - Note: As for Rel-15, the configuration / enabling of inter-slotFrequencyHopping and intraSlotFrequencyHopping is not supported. Agreement For sub-slot based PUCCH repetition, the following agreement from Cov. Enh. WI for slot-based PUCCH repetition is adopted also for sub-slot based PUCCH repetition: Agreement - In Rel-17, reuse the Rel-16 PUCCH repetition factors 2, 4, 8. - Do not support PUCCH repetition factor larger than 8 In Rel-17.. Agreement The RAN1#106-e agreement on the target slot definition is updated as follows (in RED): Agreement (from RAN1#106-e) For SPS HARQ-ACK deferral, the target PUCCH slot is defined as the next PUCCH slot, where after performing the (Rel-16) UCI multiplexing operation into a PUCCH or PUSCH if any, the UE would be either (i) transmitting HARQ-ACK using a PUCCH/PUSCH other than the PUCCH determined from PUCCH SPS-PUCCH-AN-List-r16 or n1PUCCH-AN or (ii) would be transmitting HARQ-ACK using a PUCCH resource configured in PUCCH SPS-PUCCH-AN-List-r16 or n1PUCCH-AN being regarded as valid. sps-PUCCH-AN-List-r16 or n1PUCCH-AN PUCCH resource is regarded as valid, or a PUCCH resource (from PUCCH-ResourceSet, i.e. DG PDSCH HARQ multiplexed) is dynamically indicated - The target PUCCH slot determination is based on the total HARQ-ACK payload size including deferred SPS HARQ-ACK information and non-deferred HARQ-ACK information (if any) of a candidate target PUCCH slot - The final PUCCH resource selection in the target PUCCH slot in terms of PUCCH resource set and PUCCH resource ID follows the Rel-16 procedures.. Agreement Support PUCCH cell switching based on dynamic indication in the DCI using DCI format 1_2 for a UE supporting DCI format 1_2. - The presence of the ‘PUCCH carrier switching’ bitfield in DCI format 1_2 is RRC configured. Conclusion If the UE is not configured with Rel-17 Intra-UE multiplexing, SPS HARQ for deferral of different PHY priorities can be separately deferred with the target PUCCHs separately determinated according to their respective PHY priorities. - FFS on the PHY priority handling for SPS HARQ deferral if the UE configured with Rel-17 Intra-UE multiplexing Agreement For one-shot HARQ re-transmission on PUCCH, the triggering DCI dynamically indicates a ‘HARQ re-tx offset’ which is used to define the offset in number of PUCCH slots/sub-slots between the triggering DCI and the PUCCH slot/sub-slot of the HARQ-ACK codebook to be re-transmitted. For the triggering DCI received in slot/sub-slot m, indicating the HARQ-ACK re-tx in slot/sub-slot m+k and indicating HARQ_retx_offset, the PUCCH slot/sub-slot n of the HARQ-ACK codebook to be re-transmitted is determined as either: - Alt. 1: n = m - HARQ_retx_offset - Alt. 2: n = m + k - HARQ_retx_offset - FFS: value range of the HARQ-retx_offset Agreement Down-select in RAN1#107-e from Alt. 1 & Alt. 3 below: For PUCCH carrier switching based on semi-static operation, for the case the PCell slot to be longer than the target PUCCH cell slot or sub-slot (i.e. multiple target PUCCH cell slots overlapping with a single PCell slot), the following PUCCH cell slot is used for UCI transmission: - Alt. 1: the first target PUCCH slot overlapping with the PCell slot - Alt. 3: using a relative slot-offset within the reference cell slot, the relative slot offset is configured in the time domain pattern (i.e. time domain pattern contains ‘cell index’ & ‘slot_offset’ for each reference cell slot) - Note: different relative slot offset can be configured for each reference cell slot in the time domain pattern, details see R1-2108829 Agreement Down-select in RAN1#107-e from Alt. 2 & Alt. 4 below: For PUCCH carrier switching based on semi-static operation, for the case the PCell slot to be shorter than the target PUCCH cell slot, - Alt. 2: the UE does not expect the same UCI type (i.e. HARQ-ACK, SR or CSI) from more than one PCell PUCCH slot to be overlapping with a single dynamically indicated PUCCH cell slot - Note: there can be e.g. HARQ-ACK only be present in either of the overlapping slots, but not in more than one overlapping slot. - Alt. 4: the UE does not expect a semi-static PUCC cell configuration, where a single target PUCCH slot / sub-slot would be overlapping with more than one PCell slot/sub-slot. Conclusion There is no consensus to support multiplexing of HARQ-ACK (without dynamic PUCCH cell indication), SR and P/SP-CSI on the dynamically indicated PUCCH cell (other than PCell / PSCell / PUCCH-SCell) in Rel-17. - FFS: further handling, incl. e.g., UE does not expect overlapping HARQ-ACK (without dynamic PUCCH cell indication), SR and P/SP-CSI or overlapping HARQ-ACK (without dynamic PUCCH cell indication), SR and P/SP-CSI is to be dropped - FFS: overlapping definition for SR and P/SP-CSI in terms of PUCCH slot or PUCCH resource Agreement For one-shot triggering of HARQ-ACK re-transmission on PUCCH, - in case the dynamic Type 2 HARQ-ACK codebook is configured, the HARQ-ACK codebook per PHY priority on the indicated PUCCH is constructed by appending the Type 2 HARQ-ACK codebook to be re-transmitted to the Type 2 HARQ-ACK codebook of the indicated PUCCH (carrying new, initial HARQ-ACK information) per PHY priority. - in case the semi-static Type 1 HARQ-ACK codebook is configured, the HARQ-ACK codebook per PHY priority on the indicated PUCCH is constructed by appending the Type 1 HARQ-ACK codebook to be re-transmitted to the Type 1 HARQ-ACK codebook of the indicated PUCCH (carrying new, initial HARQ-ACK information) per PHY priority. |
| --- | --- |
| CSI feedback enhancements (RAN1 AI 8.3.1.2) | Agreement When subband CQI reporting is configured with 4-bits per subband, UE includes wideband CQI in report. |
| Enhancements for unlicensed band URLLC/IIoT (RAN1 AI 8.3.2) | Agreement In semi-static channel access mode, for PUSCH repetition Type B: If a nominal repetition overlaps with a set of symbols in an idle period associated to gNB’s FFP in case UE shares gNB-initiated COT for the nominal repetition or associated to UE’s FFP in case UE assumes UE-initiated COT for the nominal repetition, all the symbols in the idle period should be considered as invalid symbols which are not considered for an actual repetition as in Rel-16. - Segmentation before and/or after the idle period is applied when applicable. - FFS on impact of processing timeline for PUSCH on the UE behaviour Agreement In semi-static channel access mode, for PUSCH repetition Type B, orphan symbol(s) are dropped as in Rel-16 Agreement In semi-static channel access mode, the configuration of energy detection threshold to perform sensing at UE is based on maxEnergyDetectionThreshold. - That means that in semi-static channel access mode, configuration of ul-toDL-COT-SharingED-Threshold is not applicable. - As the consequence, energy detection threshold to perform sensing at UE is based on maxEnergyDetectionThreshold if maxEnergyDetectionThreshold is configured. Otherwise (i.e., if maxEnergyDetectionThreshold is not configured), energy detection threshold to perform sensing at UE is based on the UE maximum transmit power. Agreement Support configuration of harq-ProcID-Offset2 for operation in unlicensed spectrum when the cg-RetransmissionTimer-r16 is not configured. Agreement The following RRC parameters are NOT needed when cg-RetransmissionTimer is configured for CG operation with shared spectrum channel access. - pusch-RepTypeIndicator - startingFromRV0 Agreement The RRC parameter of phy-PriorityIndex is applicable for CG operation in unlicensed band. Agreement Introduce new RRC parameters ul-AccessConfigListDCI-0-2 and ul-AccessConfigListDCI-1-2 to support indication of CP extension, LBT type, and CAPC with DCI 0_2 and 1_2 with dynamic channel access. Agreement In semi-static channel access mode, a DL transmission burst based on sharing of a UE initiated COT corresponding to a UE FFP, shall include scheduled DL transmission or a DCI intended for the UE that initiated that FFP. - A DL transmission to any other UE in the cell than the COT initiating UE and/or a broadcast transmission can be additionally included in the DL transmission burst if the gNB fulfils the following condition: - It is gNB‘s responsibility to ensure that other UEs do not assume gNB-initiated COT based transmission for a UL transmission based on the detection of any transmission in the DL transmission burst. Agreement In semi-static channel access mode for a UE which is allowed to operate as an initiating device, CG-StartingOffsets is not applicable. - Note: That is, CG-StaringOffsets is not applicable at all for a UE configured with UE FFP parameters (e.g. period, offset) regardless whether the UE would initiate its own COT or would share gNB’s COT. Agreement - When performing Intra-UE multiplexing procedure, if a PUCCH with HARQ-ACK overlaps with a CG-PUSCH and the cg-RetransmissionTimer is configured: - If the HARQ-ACK and the CG-PUSCH have the same priority and the CG-PUSCH is selected for HARQ-ACK multiplexing: - If cg-UCI-Multiplexing is enabled for that CG-PUSCH, HARQ-ACK would be multiplexed in CG-PUSCH. - Otherwise, CG-PUSCH would be dropped. - If the HARQ-ACK and the CG-PUSCH have different priority and the CG-PUSCH is selected for HARQ-ACK multiplexing: - If multiplexing HARQ-ACK on the CG-PUSCH with different priroity is not indicated, - The LP channel between PUCCH or CG-PUSCH would be dropped as in Rel-16. - If multiplexing HARQ-ACK on the CG-PUSCH with different priroity is indicated, - If cg-UCI-Multiplexing is enabled for that CG-PUSCH, HARQ-ACK would be multiplexed in CG-PUSCH. - Otherwise, the LP channel would be dropped. |
| Intra-UE multiplexing enhancements (RAN1 AI 8.3.3) | Agreement The following working assumption is confirmed. For handling overlapping PUCCHs/PUSCHs with different priorities in R17 - Step 1: Resolve overlapping PUCCHs and/or PUSCHs with the same priority - Step 2: Resolve overlapping PUCCHs and/or PUSCHs with different priorities Note: Avoid recursive pseudo-code to implement this procedure Note: It is expected that Rel-15 intra-UE UCI multiplexing timeline will be applicable Agreement For multiplexing a high-priority (HP) HARQ-ACK and a low-priority (LP) HARQ-ACK into a PUCCH in R17, in case the total number of LP and HP HARQ-ACK bits is 2: - Use a PUCCH resource in the second PUCCH-Config (the PUCCH-config containing the PUCCH resource of the HP HARQ-ACK). Agreement For both the subslot-based PUCCH and slot-based PUCCH, if simultaneous PUCCH/PUSCH transmission is not enabled, reuse Rel-16 procedure for Step 1 Agreement For multiplexing a high-priority (HP) HARQ-ACK and a low-priority (LP) HARQ-ACK into a PUSCH in R17, if HP HARQ-ACK and LP HARQ-ACK would be transmitted on HP/LP PUSCH without CSI, - HP HARQ-ACK and LP HARQ-ACK are separately encoded according to R15 TS 38.212 Clause 5.3.1 and Clause 5.3.3. - Reuse R15 HARQ-ACK rate matching/puncturing and RE mapping for HP HARQ-ACK in principle. FFS details. - For LP HARQ-ACK, reuse R15 Part 1 CSI rate matching and RE mapping. Agreement For determining the PUCCH resource to carry the multiplexed high-priority and low-priority HARQ-ACKs, - The number of RBs for multiplexing HP HARQ-ACK and LP HARQ-ACK on a PUCCH format 3 is determined as following: - If OHP_UCI+OCRC, HP_UCIrHP_UCI∙Qm+OLP_UCI+OCRC, LP_UCIrLP_UCI∙Qm ≤MRBPUCCH∙Nsc, ctrlRB∙Nsymb-UCIPUCCH, the minimum number of RBs is determined as the number of MRB, minPUCCH, satisfying OHP_UCI+OCRC, HP_UCIrHP_UCI∙Qm+OLP_UCI+OCRC, LP_UCIrLP_UCI∙Qm≤MRB,minPUCCH∙Nsc, ctrlRB∙Nsymb-UCIPUCCH and OHP_UCI+OCRC, HP_UCIrHP_UCI∙Qm+OLP_UCI+OCRC, LP_UCIrLP_UCI∙Qm>MRB,minPUCCH-1∙Nsc, ctrlRB∙Nsymb-UCIPUCCH. - Note: rHP_UCI∙rLP_UCI∙Qm is multiplied at both sides to avoid mismatch between gNB and UE due to floating point operation. Editor to capture as suggested. - Otherwise, - Alt1: the number of RBs is MRBPUCCH. FFS: Whether/How LP HARQ-ACK is dropped. - Alt2: the number of RBs is determined by HP ACK payload size. LP HARQ-ACK is fully dropped. - Other alternatives are not precluded. - r_HP_UCI is maxCodeRate configured for HP bits and r_LP_UCI is maxCodeRate configured for LP bits in the second PUCCH-Config (the PUCCH-config containing the PUCCH resource of the HP HARQ-ACK). - FFS whether more than one maxCodeRate can be configured for one priority. - If MRB, minPUCCH is not equal to according to [4, TS 38.211], MRB, minPUCCH is increased to the nearest allowed value of nrofPRBs for PUCCH-format3 provided by the second PUCCH-Config [12, TS 38.331]. - HP coded bits and LP coded bits are not transmitted using the same RE(s) - FFS for PUCCH format 2. Agreement For collision between HP CG PUSCH and LP DG PUSCH, if MAC delivers two MAC PDUs to PHY, PHY layer can make the prioritization so that the UE is expected to transmit the CG PUSCH and cancel the DG PUSCH at latest from the first symbol that is overlapping with the CG PUSCH. - Note: For the DG PUSCH, it is up to UE implementation to handle OFDM symbols of the DG PUSCH before the start of HP CG PUSCH which are nonoverlapping with the HP CG PUSCH. - FFS: How to handle the collision when there is repetition for CG and/or DG PUSCH |
| Propagation delay compensation / Time Synchronization (RAN1 AI 8.3.4) | Agreement For evaluation of the overall time synchronization error for RTT-based propagation delay compensation, - Alt.1 for RTT-based PDC Agreement For evaluation of the overall time synchronization error for TA-based propagation delay compensation, - Alt.1 for TA-based PDC Agreement For evaluation of the overall time synchronization error for RTT-based propagation delay compensation with Alt.1, it is assumed that - The UE Rx-Tx time difference measurement accuracy based on PRS defined in Table 10.1.25.2-2 in TS 38.133 v17.3.0 is taken as the reference for the UE Rx-Tx time difference measurement accuracy - The gNB Rx-Tx time difference accuracy based on SRS for positioning defined in Table 13.2.2.2-1 in TS 38.133 v17.3.0 is taken as the reference for the gNB Rx-Tx time difference accuracy based on SRS for PDC Agreement For RTT-based PDC, only a single pair of CSI-RS for tracking (TRS)/PRS and SRS configuration, i.e. one CSI-RS for tracking (TRS)/PRS configuration for Rx – Tx time difference estimation at UE side and one SRS configuration for Rx – Tx time difference estimation at gNB side, is configured for PDC in Rel-17, if RTT-based PDC is supported. Agreement If RTT-based propagation delay compensation is supported and performed at the gNB side, the Rx-Tx measurement report provided from the UE to the gNB should include at least: - UE Rx-Tx time difference at a given granularity Conclusion When evaluating enhanced TA-based PDC, there is no need to replace Te by TA adjustment error. Agreement Send an LS to RAN2 and CC RAN4 with the content including: - The latest available status on PDC methods in RAN1, e.g. key agreements achieved for TA-based PDC and RTT-based PDC. LS is endorsed in R1-2110647 Agreement For evaluation and comparison of enhanced TA-based PDC and RTT-based PDC, the timing detection error = 0.5/(RS BW) = 0.5/(N_PRB*12*SCS) can be used to achieve errorUE,DL,RX and errorBS,UL,RX, if needed in the evaluation equation separately, where N_PRB is the number of PRBs of the RS bandwidth used in the detection by UE and gNB, respectively. - Note: Detection error achieved by evaluations is not precluded if available. Agreement If enhanced TA-based PDC with reduced Te based on TRS is supported in Rel-17, one CSI-RS for tracking (TRS) configuration is configured for enhanced TA-based PDC. - FFS whether/how to configure UL signal for enhanced TA-based PDC Agreement If enhanced TA-based PDC with enhanced TA command indication granularity is supported in Rel-17, - The enhanced TA command indication granularity introduced for enhanced PDC is applied for PDC purpose, which doesn’t have impact on normal TA procedure, i.e. normal TA procedure will still follow the existing TA command indication granularity. Agreement If RTT-based propagation delay compensation is supported, the Rx-Tx time difference is reported with granularity 2k*Tc, where k is an integer satisfying 0<=k<=5. - FFS the value of k - FFS the reporting range of Rx-Tx time difference measurement for PDC |

RAN1#107-e

| UE feedback enhancements for HARQ-ACK (RAN1 AI 8.3.1) | Agreement The maximum SPS HARQ-ACK deferral value in terms of k1+k1def per SPS configuration is RRC configured from a value range of {1…32}. Agreement The list enhanced Type 3 HARQ-ACK codebooks is configured per PUCCH cell group (i.e., separately configurable for primary and secondary PUCCH cell group). Agreement The one-shot HARQ re-transmission on PUCCH is configured per PUCCH cell group (i.e., separately configurable for primary and secondary PUCCH cell group). Agreement For one-shot HARQ re-transmission on PUCCH, the ‘HARQ re-tx offset’ is determined as Alt. 1: n = m - HARQ_retx_offset Conclusion There is no consensus to support the simultaneous configuration of one-shot HARQ-ACK re-transmission and dynamic PUCCH cell switching in Rel-17. Conclusion For SPS HARQ-ACK deferral, if a UE is not configured with Rel-17 intra-UE multiplexing but configured with Rel-16 PHY prioritization, the UE first performs Rel-16 UCI multiplexing and PHY prioritization in both initial slot and target slot and if a LP SPS HARQ-ACK PUCCH is deprioritized, the LP SPS HARQ-ACK is not deferred. - Note: If the SPS HARQ-ACK is deprioritized in any slot, no further deferral. Agreement Support simultaneous configuration of SPS HARQ-ACK deferral and PUCCH cell switching based on the semi-static time domain pattern: For the target slot determination of SPS HARQ-ACK deferral, - Step 1: the UE first determines a next PUCCH slot on the cell for PUCCH transmission using the semi-static time-domain PUCCH cell pattern and the related rules for semi-static PUCCH cell switching, followed by - Step 2: the UE determines based on the SPS HARQ-ACK deferral rules if this PUCCH slot on the PUCCH cell for transmission is the target PUCCH slot or not. - Note: In step 1, k is increased on PCell/PScell/PUCCH-Scell. “The next PUCCH slot” represents the slot on the PUCCH cell based on PUCCH cell pattern, which is mapped from the PCell/PScell/PUCCH-Scell slot with increased K1. - Note: The maximum deferral limitation checking is based on the effective k + kdef value based on the granularity of PCell / PScell/PUSCCH-Scell Agreement Support simultaneous configuration of one-shot HARQ-ACK re-transmission and semi-static PUCCH cell switching: - the ‘backward HARQ-ACK slot-offset’ is interpreted with the granularity of a PUCCH slot of the respective PHY priority of PCell /PSCell / PUCCH SCell Agreement Confirm the following RAN1 working assumption from RAN1#106bis-e with the additional agreement on UE capability (in RED): - (Working Assumption) Support inter-subslot Frequency Hopping for PUCCH repetition operation of PUCCH Format 0 and Format 2 for 2OS slot-based PUCCH configurations. - The UE applies the inter-subslot FH operation from sub-slot to sub-slot, if configured with inter-slotFrequencyHopping in the respective PUCCH_config.. - Support single UE capability indication of inter-subslot FH for PUCCH repetition operation. Agreement Apply a 1-bit triggering DCI field for triggering indication of one-shot HARQ re-transmission on PUCCH. - The triggering DCI with the triggering bit set to ‘1’ is not able to schedule PDSCH. - Some unused bit field in the DCI is used to indicate the HARQ slot offset. - FFS: if the ‘one-shot HARQ-ACK request’ field can be reused - FFS: which unsed DCI field in the DCI is used for HARQ slot offset indication - FFS: The indication of whether the PDSCH is not scheduled will reuse Rel-16 type-3 HARQ ACK CB UE behavior Agreement The earlier RAN1 agreements on the valid symbol definition in the initial and target PUCCH slot for SPS HARQ-ACK deferral are further clarified as: - For SPS HARQ-ACK deferral, for the determination of valid symbols in the initial and target PUCCH slot/sub-slot a collision with semi-static DL symbols, SSB and symbols indicated by pdcch-ConfigSIB1 in MIB for a CORESET for Type0-PDCCH CSS set is regarded as ‘invalid’ or ‘no symbols for UL transmission’. Conclusion There is no consensus to support the simultaneous configuration of the Rel-16 Type 3 HARQ-ACK CB and Rel-17 one-shot re-tx HARQ triggering for a UE in Rel-17. Conclusion There is no consensus to support the simultaneous configuration of the Rel-17 Enhanced Type 3 HARQ-ACK CB and Rel-17 one-shot HARQ re-tx triggering for a UE in Rel-17. Agreement Support simultaneous configuration of enhanced Type 3 CB triggering and PUCCH cell switching. Conclusion For PUCCH cell switching DCI field size alignment is done by: - For dynamic PUCCH cell switching, the bit width of the PDSCH-to-HARQ_feedback timing indicator in DCI format 1_1 and 1_2 is determined by the largest K1 set among the K1 sets of all candidate PUCCH cells for PUCCH cell switching based on dynamic indication - i.e., a number of most significant bits with value set to '0' are inserted to smaller field until the bit width of the field for all the PUCCH cells are the same - Note: for semi-static PUCCH cell switching only the K1 set of PCell is needed - For semi-static and dynamic PUCCH cell switching, the bit width of the PRI field in DCI format 1_2 is determined by the largest value of numberOfBitsForPUCCH-ResourceIndicatorDCI-1-2 of all PUCCH cells - i.e., a number of most significant bits with value set to '0' are inserted to smaller field until the bit width of the field for all the PUCCH cells are the same - FFS: If similar handling is applied for ChannelAccess-CPext DCI field (0 or 2 bit) Agreement For PUCCH cell switching and a PUCCH transmission on the alternative PUCCH cell, the alternative PUCCH cell is used to derive the downlink pathloss estimate PLb,f,c(qd), i.e., replace in the main bullet of the PLb,f,c(qd) determination in Sec. 7.2.1 of 38.213 the ‘primary cell’ with ‘cell for PUCCH transmission’ Agreement For PUCCH cell switching based on semi-static operation, for the case the PCell slot to be longer than the target PUCCH cell slot or sub-slot (i.e., multiple target PUCCH cell slots overlapping with a single PCell slot), adopt Alt 1, i.e., the first target PUCCH slot overlapping with the PCell slot is used for UCI transmission. Agreement Support simultaneous configuration of Rel-16 Type 3 HARQ-ACK codebook or enhanced Type 3 HARQ-ACK codebook triggering and SPS HARQ-ACK deferral. - In case a R16 Type 3 HARQ-ACK CB or an enhanced Type 3 HARQ-ACK codebook is triggered for transmission in a PUCCH slot, the UE stops the deferral procedure of pending SPS HARQ-ACK in that PUCCH slot and that PUCCH slot is not considered as a potential target slot for SPS HARQ-ACK deferral anymore. Agreement One enhanced Type 3 HARQ-ACK codebook is RRC configured either as: - a subset of CC, i.e., all HARQ processes of the subset of CCs are part of the codebook, OR pdsch-HARQ-ACK-enhType3perCC; Configure the one enhanced Type 3 HARQ-ACK codebook using per CC configuration; (1..maxNrofServingCells) of Integer (0,1). - a subset of configured HARQ processes per CC, i.e., different subsets of HARQ processes can be configured for each CC. pdsch-HARQ-ACK-enhType3perHARQ; Configure the one enhanced Type 3 HARQ-ACK codebook using a per HARQ process and CC configuration; (1..maxNrofServingCells) of Bit String (Size (16)). Agreement For one-shot triggering of HARQ re-transmission, introduce a new 1-bit DCI field in DCI format 1_1 and in DCI format 1_2 (if DCI format 1_2 is configured with one-shot triggering of HARQ-ACK re-transmission). Agreement The time-domain pattern for semi-static PUCCH cell switching is separately configurable for the primary and secondary PUCCH cell group. Agreement The time-domain pattern for semi-static PUCCH cell switching is based on the reference SCS configuration provided by tdd-UL-DL-ConfigurationCommon and is common to every configured UL BWP (of PCell / SPCell / PUCCH SCell). Agreement For PUCCH cell switching based on semi-static operation, adopt Alt. 4, i.e., the UE does not expect a semi-static PUCCH cell configuration, where a single target PUCCH slot / sub-slot would be overlapping with more than one PCell slot/sub-slot. Agreement For semi-static PUCCH cell switching, if the alternative PUCCH cell (i.e. PUCCH sCell) is deactivated or the alternative PUCCH Cell is dormant, the UE does not apply time-domain pattern and the UCI is to be transmitted on PCell / SPCell / PUCCH SCell. Conclusion There is no consensus to support simultaneous configuration of semi-static PUCCH cell switching and dynamic PUCCH cell switching in Rel-17. Working Assumption For one-shot triggering of HARQ re-transmission, in addition to one-shot triggering of HARQ re-transmission after the initial PUCCH transmission slot, the triggering is supported before the initial PUCCH transmission slot - Re-transmission triggering does not change processing for the initial PUCCH transmission (i.e., HARQ multiplexing / dropping / transmission) - The UE expects the PUCCH carrying the HARQ-ACK re-transmission to be scheduled in a slot/sub-slot after the initial PUCCH transmission slot/sub-slot. - The support for the triggering before the initial PUCCH transmission slot is subject to separate UE capability indication Agreement If more than one (M>1) enhanced Type 3 HARQ-ACK codebook is configured and the triggering DCI with the ‘one-shot HARQ-ACK request’ set to ‘1’, - If the FDRA field is not valid, i.e. all “1s” or all “0s” as per Rel-16, then PDSCH is not scheduled: - If a new field with N=ceiling(log2 (M)) bits is configured in the triggering DCI, the UE uses this new field to indicate one of M configured e-Type 3 HARQ-ACK CBs - If the new field is not configured in the triggering DCI, the UE uses the MCS field to indicate one of M configured e-Type 3 HARQ-ACK CBs - If the FDRA field is valid, then a PDSCH is scheduled - If a new field with N=ceiling(log2 (M)) bits is configured in the triggering DCI, the UE uses this new field to indicate one of M configured e-Type 3 HARQ-ACK CBs - If the new field is not configured in the triggering DCI, the UE selects the 1st indexed e-Type 3 HARQ-ACK CB in the M configured e-Type 3 HARQ-ACK CBs Agreement For one-shot HARQ-ACK re-transmission, the value range for HARQ re-tx offset is fixed in the specification Conclusion For dynamic PUCCH cell switching, the UE does not expect a PUCCH slot with UCI on PCell /SPCell / PUCCH SCell to overlap with a PUCCH slot with HARQ-ACK on the dynamically indicated alternative PUCCH PUCCH cell. - The UCI on PCell /SPCell / PUCCH SCell dropped due to collision with semi-static DL symbols, SSB, and symbols indicated by pdcch-ConfigSIB1 in MIB for a CORESET for Type0-PDCCH CSS set is exempted and is not multiplexed on the PUCCH on the alternative PUCCH cell. |
| --- | --- |
| CSI feedback enhancements (part of RAN1 AI 8.3.4) |  |
| Enhancements for unlicensed band URLLC/IIoT (RAN1 AI 8.3.2) | Agreement In semi-static channel access mode, for a transmission burst that includes multiple transmissions, the associated COT-ownership for all transmissions in the transmission burst should be the same. Agreement In semi-static channel access mode, when a UE is enabled to initiate a channel occupancy: - If single DCI schedules multiple UL transmissions, the COT initiator assumption indicated by the single DCI is applied for all the UL transmissions scheduled by the single DCI. Agreement In semi-static channel access mode, when a DCI schedules a UL transmission in the same g-FFP and the UL transmission is not aligned with a u-FFP boundary and the DCI indicates UE initiated COT, the following are applied: - If the UE has initiated the COT in that u-FFP and satisfies the applicable sensing conditions, the UL transmission occurs. Otherwise, the UL transmission is dropped. Agreement The following channel access procedures for consecutive scheduled UL transmissions are applicable to the semi-static channel access mode. - If a UE is scheduled by a gNB to transmit a set of UL transmissions including PUSCH or SRS symbol(s) using a UL grant, the UE shall not apply a CP extension for the remaining UL transmissions in the set after the first UL transmission after accessing the channel. - If a UE is scheduled to transmit a set of consecutive UL transmissions without gaps including PUSCH using one or more UL grant(s), PUCCH using one or more DL grant(s), or SRS with one or more DL grant(s) or UL grant(s) and the UE transmits one of the scheduled UL transmissions in the set after accessing the channel, the UE may continue transmission of the remaining UL transmissions in the set, if any. - Note: The procedures above are based on description in Clause 4.2.1.0.1 of TS 37.213. Agreement In semi-static channel access mode, when the gNB schedules by a DCI a UL transmission and the scheduling DCI and the scheduled UL transmission are in a same g-FFP but on a different RB sets of the g-FFP bandwidth: - If DCI indicates gNB initiated COT, validation of the gNB-initiated COT (based on the detection of DL transmission from the gNB) for the RB sets with scheduled UL can be skipped. Agreement The symbol offset for the UE FFP configuration is determined based on the smallest SCS among configured SCSs in a serving cell. Conclusion PUSCH repetition Type B for DG on unlicensed spectrum in Rel-17 is supported. Conclusion In semi-static channel access mode when a UE can operate as an initiating device, for a configured UL transmission, the required time to determine whether the configured UL transmission could correspond to gNB’s COT or UE’s COT is up to UE implementation. Conclusion In semi-static channel access mode when a UE can operate as an initiating device, for a scheduled UL transmission, when the scheduling DCI and the first symbol of the scheduled UL transmission are in the same g-FFP, the processing time for the scheduled UL transmission satisfies the time required to the UE determine whether the scheduled UL transmission could correspond to the COT initiator assumption indicated in the DCI. Conclusion In semi-static channel access mode when a UE can operate as an initiating device, for a scheduled UL transmission, when the scheduling DCI and the first symbol of the scheduled UL transmission are in different g-FFPs, and if the DCI indicates gNB as the COT initiator: - the required time to determine whether the gNB had initiated a COT before the start of the scheduled UL transmission is up to UE implementation. Agreement For operation in a cell with shared spectrum access, a UE configured with multiple CG configurations does not expect to operate in the cell with more than one active CG configurations for which the cg-RetransmissionTimer is provided in one active CG configuration and not provided in another. - Note: That means that the UE operates with a same CG type (i.e., Rel-16 NR-U CG type or Rel-16 URLLC CG type per previous agreements) per cell in a shared spectrum. Agreement EnableConfiguredUL is not applicable if cg-RetransmissionTimer is not configured in Rel-17. Agreement In semi-static channel access mode, when the cg-RetransmissionTimer-r16 is enabled and a UE operates as an initiating device, the RRC parameter cg-COT-SharingList-16 is reused, and the UE is not expected to provide any relevant information related to CAPC to the gNB. - Channel Occupancy Time (COT) sharing information bit-field in CG-UCI is as the following: - log2C bits if higher layer parameter cg-COT-SharingList is configured, where C is the number of combinations configured in cg-COT-SharingList; - 0 bit otherwise; |
| Intra-UE multiplexing enhancements (RAN1 AI 8.3.3) | Agreement For handling overlapping PUCCHs/PUSCHs with different priorities, Step 2 consists of the following sub-steps: - Step 2.1: Resolve collision of LP PUCCHs and HP PUCCHs. - Step 2.2: Resolve collision of PUCCHs and PUSCHs of different priorities. Conclusion There is no consensus in RAN1 to support simultaneous PUCCH/PUSCH transmission of same priority over different cells in Rel-17. Conclusion There is no consensus in RAN1 to support simultaneous PUCCH/PUSCH transmissions on different cells for intra-band CA in Rel-17. Agreement If multiplexing of PUCCHs and/or PUSCHs with different priorities is enabled by RRC, support both of the following UE capabilities to resolve collision of PUCCHs and/or PUSCHs with different priorities in step 2: - Capability #1: It is not expected that Rel-15 multiplexing timeline is not met for all overlapping channels [FFS the overlapping channels are resultant channels after step 1]. UE performs multiplexing or dropping of PUCCHs and/or PUSCHs with different priorities according to Rel-17 rules. - Dynamic enabling/disabling multiplexing for different priorities is not supported for Capability #1 - (Working assumption) Capability #3: Rel-17 multiplexing for different priorities is dynamically enabled/disabled in step 2. - Dynamic indication of enabling/disabling multiplexing for different priorities can be enabled only if multiplexing of PUCCHs/PUSCHs with different priorities is enabled by RRC configuration. - If dynamic multiplexing for different priorities is indicated as enabled for a PUCCH / PUSCH, the UE performs Rel-17 multiplexing operation using the Rel-15 timeline - The gNB is responsible to ensure that all the DCIs associated with all overlapping channels involved in multiplexing in step 2 meet the Rel-15 timeline for multiplexing. - If dynamic multiplexing for different priorities is indicated as disabled for a PUCCH / PUSCH, the UE does not apply the Rel-17 intra-UE multiplexing - If the UL channel associated with the DCI disabling multiplexing collides with another UL channel of a different priority, UE performs R16 PHY prioritization, using Rel-16 timeline. The gNB is responsible to ensure that the UE meets R16 PHY prioritization timeline. - If the UL channel associated with the DCI disabling multiplexing does not collide with another UL channel of a different priority, UE transmits the UL channel as is. - FFS: whether the UL channel associated with the DCI disabling multiplexing can collide with another UL channel of a same priority. - UE does not expect to receive a dynamic indication resulting in demultiplexing of previously multiplexed PUCCHs/PUSCHs channels after the Rel-15 multiplexing deadline has passed - FFS: UE does not expect to receive a dynamic indication resulting in demultiplexing of previously multiplexed PUCCHs/PUSCHs channels without any associated DCIs - Note: demultiplexing of two previously multiplexed channels means decoupling two channels already multiplexed, dropping one channel, and multiplexing the other channel with another channel(s). The above behaviors of Capability#3 at least apply to resolving collision of two UL channels resulting from Step 1 with different priorities. FFS: more than two UL channels. - FFS whether dynamic indication in multiple DCIs associated with a group of overlapping channels have to be consistent - FFS: Configuration of prioritization / multiplexing of channels without dynamic indication - Note: Capability 3 procedure is a super-set of Capability 1 procedure - FFS: Time unit to apply Rel-15 timeline (e.g. slot based, sub-slot based) - FFS: The set of PUSCH and PUCCH that eligible for Rel-15 multiplexing consideration Note: “collision” refers to overlapping PUCCHs, overlapping PUCCH and PUSCH (excluding PUSCH supporting simultaneous transmission with PUCCH), overlapping PUSCHs on a same cell. Note: “Rel-15 multiplexing timeline” means Rel15 timeline calculation in Rel-16 spec, including all the formula and all the values for the variables Note: “Rel-16 prioritization timeline” means Rel-16 cancellation timeline calculation in Rel-16 spec, including all the formula and all the values for the variables Agreement For collision of LP DG-PUSCH and HP CG-PUSCH of different priorities, the cancellation is applied per actual repetition, if LP DG-PUSCH and/or HP CG-PUSCH is repeated. Agreement For multiplexing a high-priority (HP) HARQ-ACK and a low-priority (LP) HARQ-ACK into a PUSCH in R17, if HP HARQ-ACK, LP HARQ-ACK, and LP CSI consisting of two parts would be transmitted on LP PUSCH conveying UL-SCH, - The CSI part 2 is dropped. - Reuse R15 HARQ-ACK rate matching/puncturing and RE mapping for HP HARQ-ACK in principle. FFS details. - Reuse R15 CSI part 1 rate matching and RE mapping for LP HARQ-ACK. - Reuse R15 CSI part 2 rate matching and RE mapping for LP CSI part 1. - FFS for LP CSI consisting of single part. Note: Apple raised concern on CSI being dropped unnecessarily which could cause performance and degrade usefulness of URLLC enhancement. Agreement For the overlapping between LP CG and HP DG, if MAC delivers two MAC PDUs to PHY, PHY layer can make the prioritization so that the UE is expected to cancel the overlapping low priority CG PUSCH by the first overlapping symbol at the latest. - On top of Rel-16 cancellation time (N2+d1) for PUCCH/PUCCH or PUCCH/PUSCH collision, additional time d3 is needed (which results N2+d1+d3 in total cancellation time) for LP CG-PUSCH and HP DG-PUSCH collision resolution. - (Working assumption) d3 = {0, 1,…,2μ+1}symbol(s) upon UE capability report, where μ=0,1,2,3 for SCS=15/30/60/120kHz, respectively. Agreement For determining the PUCCH resource to carry the multiplexed high-priority and low-priority HARQ-ACKs, if OHP_UCI+OCRC, HP_UCIrHP_UCI∙Qm+OLP_UCI+OCRC, LP_UCIrLP_UCI∙Qm >MRBPUCCH∙Nsc, ctrlRB∙Nsymb-UCIPUCCH - The number of RBs is MRBPUCCH. Then follow Rel-15 procedure, i.e., LP HARQ-ACK is mapped to the rest REs after HP HARQ-ACK. Agreement For multiplexing a high-priority (HP) HARQ-ACK and a low-priority (LP) HARQ-ACK into a PUCCH in R17, - At least for PUCCH format 3/4, use the HP UCI bit number and HP RE number for ∆TF,b,f,c(i)∆TF, b, f,ci formula selection and calculation - For PUCCH format 1, use the total UCI bit number for ∆TF,b,f,c(i) calculation. - FFS for PUCCH format 2. Agreement For collision of HP DG-PUSCH and LP CG-PUSCH, the cancellation is applied per actual repetition, if HP DG-PUSCH and/or LP CG-PUSCH is repeated. |
| Propagation delay compensation / Time Synchronization (RAN1 AI 8.3.4) | Agreement If RTT-based PDC is supported, a single granularity 32Tc (i.e. k=5) is supported for Rx-Tx measurement report. Agreement For Rel-17 - Support RTT-based PDC method - Support PDC method based on legacy TA-based mechanism - No RAN1/RAN4 specification impact expected Agreement For RTT-based PDC, existing definitions of UE Rx – Tx time difference (i.e. section 5.1.30 in TS 38.215) and gNB Rx – Tx time difference (i.e. section 5.2.3 in TS 38.215) are reused, with updates at least to reflect the single pair of TRS/PRS and SRS configured for RTT-based PDC. Agreement Send an LS to RAN2 and RAN4 with the content including: - The agreements made in RAN1#107-e for propagation delay compensation. - Ask RAN4 to define the following for RTT-based propagation delay compensation: - UE Rx-Tx time difference measurement accuracy based on CSI-RS for tracking - UE Rx-Tx time difference measurement accuracy based on PRS (including reuse existing spec if appropriate) - gNB Rx-Tx time difference absolute accuracy based on SRS (including reuse existing spec if appropriate) - Inform RAN4 that enhanced TA-based PDC with reduced Te and enhanced TA command granularity is precluded in RAN1. LS is endorsed in R1-2112834 Conclusion For RTT-based PDC, it is assumed that the transmission of DL TRS/PRS, UL SRS and reference time information are associated with a same TRP. Note: No RAN1 specification impact is expected for this conclusion Agreement For RTT-based propagation delay compensation, the Rx-Tx time difference is reported via RRC signaling. Conclusion The reporting range of Rx-Tx time difference measurement for RTT-based PDC is up to RAN4. |

#### 2.1.2	Remaining Open issues

- UE feedback enhancements for HARQ-ACK (RAN1 AI 8.3.1.1) – estimated completion by RAN#95-e (end of Q1/2022)

  - Value range and used DCI field(s) for the indication of the HARQ offset for one-shot HARQ-ACK re-transmission

    - Comment: isolated RAN1 impact, Sec. 9.1.5 of TS 38.213

  - Type 1 HARQ-ACK codebook generation for PUCCH cell switching

    - Comment: islotated RAN1 impact, Sec. 9.1.2 of TS 38.213

- Intra-UE multiplexing enhancements (RAN1 AI 8.3.3) – earliest estimated completion by RAN#96 (end of Q2/2022)

  - Rel-17 Intra-UE multiplexing framework and UE categories (impact to several sections of 38.213, potentially RRC & UE capabilities)

    - Confirmation of the RAN1 working assumption on support of Capability #3 type of UE

    - Details for Capability #1 type of UE operation (i.e. step 2 details incl. timeline requirement and handling of more than two overlapping channels in step 2)

    - Details for Capability #3 type of UE operation incl. at least

      - Dynamic enabling/disabling restrictions

      - Handling of PUCCH / PUSCH without dynamic enabling indication

      - Handling of more than two overlapping channels in step 2

    - Interaction with simultaneous PUSCH / PUCCH operation for step 2 for both Capabilities #1 and #3

  - Multiplexing details of UCI of different priorities on PUCCH (impact to several sections of 38.212 & 38.213, potentially RRC & UE capabilities)

    - Multiplexing on PUCCH format 2

    - 1-bit encoding & RE mapping for in total more than 2 bits on PUCCH

    - 2-bit encoding & RE mapping for in total more than 2 bits on PUCCH

    - Reliability of LP HARQ CB size determination

    - SR and HARQ-ACK multiplexing

    - How to dynamically indicate the multiplexing enabling for PUCCH (for UE category #3)

  - Multiplexing details of UCI of different priorities on PUSCH (impact to several sections of 38.212 & 38.213, potentially RRC & UE capabilities)

    - Multiplexing operation for HP HARQ-ACK, LP HARQ-ACK and HP A-CSI on PUSCH

    - Enhanced UCI multiplexing parameters (beta value range, alpha)

    - Handling of limited resources for UCI on PUSCH (i.e. full or partial dropping, compression)

    - How to dynamically indicate the multiplexing enabling for PUSCH (for UE category #3)

- Propagation delay compensation / Time Synchronization (RAN1 AI 8.3.4) – estimated latest completion by RAN#95-e (end of Q1/2022)

  - Unclear support of positioning SRS for RTT-based propagation delay compensation

    - Comment: Unclear situation at the time of the RAN#94-e submission as the related discussions on RRC parameters and UE features in RAN1 from RAN1#107-e are still ongoing. If not resolved there, this may need further discussions in RAN1 (impact to Sec. 6.2.1 of TS 38.214, potentially RRC and UE capabilities)

## 2.2	RAN2

#### 2.2.1	Agreements

| Time Synchronization (RAN2 AI 8.5.2) | Agreements 1. The gNB can enable/disable UE-side PDC via unicast and broadcast RRC signalling. 2. A new RRC parameter can be introduced to explicitly enable/disable UE-side PDC 3. When reference time information is received in both the DLInformationTransfer message and the SIB9, the UE applies the reference time info in the DLInformationTransfer message. The UE will follow dedicated signaling if timing reference is received in both unicast and broadcast 4. The timing synchronization in I-IoT should focus on the signaling between the UE and gNB, i.e. different from Multi-RTT based signalling flow which involving LMF and AMF |
| --- | --- |
| URLLC in Unlicensed (RAN2 AI 8.5.3) | Agreements: 1. If HARQ process ID selection is among the retransmissions whose HARQ processes are with equal priority, it is up to UE implementation to select the prioritized HARQ process ID. 2. If HARQ process ID selection is among the initial transmissions whose HARQ processes are with equal priority, it is up to UE implementation to select the prioritized HARQ process ID. 3. The priority of the HARQ process associated with a MAC PDU in which no data for logical channels is multiplexed or can be multiplexed is lower than the priority of the HARQ process that associated with a MAC PDU in which any logical channels are multiplexed or can be multiplexed. 4. RAN2 confirms the naming/usage of configuration “intraCG-Prioritization”. 5. Autonomous retransmission is triggered in a subsequent and available CG if the UL grant for autonomous retransmission is deprioritized and the corresponding HARQ process status is pending. No spec changes are needed. |
| RAN enhancement for New QoS (RAN2 AI 8.5.4) | Agreements: 1. A RRC parameter is configured for a DRB with Survival Time support 2. MAC entity shall handle the determination of triggering survival state based on HARQ-NACK 3. For the DRB configured with Survival Time support, the network can control the duplication state for the DRB via legacy activation/deactivation MAC CE. No specification change is foreseen. 4. For the issue that there may be packets already sent to RLC before the pre-configured PDCP duplication configuration is activated, following entry into the Survival Time state, it is up to gNB/UE implementation to handle and no need to specify extra behaviour 5. RAN2 not to consider the interaction between Survival Time solution and handover procedure in Rel-17 6. No specification enhancement will be pursued for CG activation command as Survival Time state trigger 7. The baseline mechanism for Survival Time support is “CG resources will be used for service with Survival Time requirements, such that the mapping relation between the service and the retransmission grant is commonly known to both gNB and UE, and CG retransmission scheduling (addressed by CS-RNTI) can be used for Survival Time state triggering”. a) FFS how UE identifies the corresponding DRB that should enter Survival Time state and other details (i.e. resource allocation) b) FFS on unlicensed band 8. Deprioritize autonomous activation of PDCP duplication based on inputs other than retransmission grant |

#### 2.2.2	Remaining Open issues

- Enhancements for support of time synchronization (RAN2 AI 8.5.2)

  - Details of signaling/messages to support propagation delay compensation mechanisms. According to the latest RAN2 agreement, RAN1 input is needed for RAN2 to proceed the related work.

- Uplink enhancements for URLLC in unlicensed controlled environments (RAN2 AI 8.5.3)

  - Some clarification about behaviour of CG retransmission timer upon deprioritization. For instance, handling of deprioritized MAC PDU in case where CG retransmission timer is configured but Autonomous transmission is not configured, and whether configured grant retransmission timer should be stopped upon deprioritization of a MAC PDU when Autonomous transmission is not configured.

- RAN enhancements based on new QoS (RAN2 AI 8.5.4)

  - Details (such as MAC/PDCP procedures and UE behaviour) about survival time solution based on HARQ NACK trigger.

  - Details about pre-configuration of PDCP duplication for survival time state

  - Clarifications of issues relating to resource availability upon survival time state triggering.

## 2.3	RAN3

#### 2.3.1	Agreements

| General (RAN3 AI 21.1) | - Baseline CRs were re-endorsed: TS 38.413 BL CR in R3-214625 TS 38.423 BL CR in R3-214657 TS 38.463 BL CR in R3-214626 TS 38.473 BL CR in R3-214627 |
| --- | --- |
| Time Synchronization (RAN3 AI 21.2) | - Agreements - - Introduce the Time Synchronisation Assistance Information IE as an optional UE-level parameter in: - - NGAP (INITIAL CONTEXT SETUP REQUEST, UE CONTEXT MODIFICATION REQUEST, HANDOVER REQUEST, and PATH SWITCH REQUEST ACKNOWLEDGEMENT [FFS]), - - XnAP (HANDOVER REQUEST and RETRIEVE UE CONTEXT RESPONSE) - - The Time Synchronisation Assistance Information IE includes two sub-IEs: - - Time Distribution Indication IE, encoded as ENUMERATED type with two codepoints (enabled, disabled). - - Uu Time Synchronisation Error Budget IE encoded as INTEGER type having range 1ns to 1ms, and 1ns granularity. TPs agreed in R3-215953 (NGAP) and R3-215954 (XnAP) - Working Assumptions - - The Uu Time Synchronisation Error Budget IE is needed over F1AP, details FFS. |
| Enhancements based on new QoS related parameters (RAN3 AI 21.3) | - Agreements - - No need to increase the maximum value of the periodicity. - - The maximum value of the Survival Time is 1.92s. - - The uplink Survival Time assistance information is out of the scope of RAN3. - - RAN3 continues to evaluate and discuss the solutions for the downlink Survival Time assistance information. TPs agreed in R3-215978 (NGAP), R3-215979 (XnAP), R3-215980 (F1AP), and R3-215981 (E1AP). |

#### 2.3.2	Remaining Open issues

- RAN3 impacts of propagation delay compensation enhancements, if any.

  - F1AP impacts of Uu Time Synchronisation Error Budget.

  - It is FFS on whether assistance information (e.g., UE TSN timing reference, referenceTimeInfo delivery periodicity, timestamp) should be delivered during HO.

- RAN3 impacts of new QoS related parameters, e.g. survival time, burst spread, decided in SA2.

  - Evaluate and discuss the solutions for the downlink Survival Time assistance information.

## 2.4	RAN4

#### 2.4.1	Agreements

| Propagation delay compensation enhancements (RAN4 AI 8.23.2.1) | Agreements - There is no consensus in RAN4 whether it is feasible to support a smaller value than the current Te assuming the existing conditions in TS 38.133 for Te requirement. - It is feasible to support a smaller Te value than the current Te for the use of propagation delay compensation under assumption of using TRS (or other RS used for Te estimation) instead of SSB - Smaller Te can be achieved when TRS (or other RS) bandwidth is larger than SSB bandwidth . - Smaller Te can be achieved for UE is operating in RRC_CONNECTED mode. - FFS whether a smaller Te can be achieved for the first transmission in the DRX cycle. - Support of Smaller Te can be defined as an optional Rel-17 capability and should not apply to all UEs. - It is feasible to introduce enhanced TA command indication granularity. It is RAN4 understanding that - Enhanced granularity will apply for UE supporting PDC . - This agreement does not impact UL timing accuracy requirement. TA command indication granularity can be reduced to 64Tc for both 15kHz SCS and 30kHz SCS. - The conditions for enhanced Te are - Use the same channel model as used in the test case: LOS/AWGN channel. - At least use same SINR as used in the test case. Other SINR levels are not prohibited. |
| --- | --- |
| Reference point for Te requirements (RAN4 AI 8.23.2.2) | Agreements - Do not define UE timing error requirements side consitions. |
| Others (RAN4 AI 8.23.2.3) | Agreements - RAN4 do not define RRM core requirements for PUCCH carrier switching. - RAN4 do not define test case for PUCCH carrier switching. |

#### 2.4.2	Remaining Open issues

RRM core requirements:

- RAN4 impacts of propagation delay compensation enhancements, if any.

- whether it should be clarified that the reference point/downlink timing refer to the signal of the first path being received/arrives at the UE antenna.

RRM performance requirements are not started yet.

## 3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

NOTE: This section only needs to be filled in for WI/SIs where there is a corresponding relevant WI/SI in SA/CT.

## 3.1	SA2

#### 3.1.1	Agreements with cross-TSG impacts

The work item is completed in SA2 and is in maintenance phase. No RAN dependency is observed.

#### 3.1.2	Remaining Open issues with cross-TSG impacts

All open issues are resolved, the WI of IIoT is considered as completed in SA2.

NOTE: This section should also flag any critical dependencies that need TSG attention.

## 4.	RAN Plenary Intervention

Based on the observations of the current status in RAN1, as also indicated by the RAN1 chair, the WI from RAN1 perspective cannot be regarded as completed:

- The minor pending final details on HARQ-ACK feedback enhancements and RTT-based propagation delay compensation should be possible to finalize in Q1/2022, but the Dec. 2021 specifications will not be complete.

- On Rel-17 Intra-UE multiplexing enhancements the situation is more critical, as there are many more pending issues, needed design agreements and clarifications required also considering the last minute RAN1#107-e agreement on the Rel-17 Intra-UE multiplexing framework (incl. Capability #1 and the RAN1 working assumption on Capabilty #3). Therefore, for the support of the Rel-17 Intra-UE multiplexing more time than just one additional quarter seems to be needed for completion.

Therefore, some RAN guidance on how to proceed with the completion of the Rel-17 URLLC & IIoT WI in RAN1 is needed.

## 5.	References

NOTE:	This can be e.g. a list of all related Tdocs in the affected WGs since last TSG, references to LSs, produced TRs/TSs, the work/study item description or status reports of previous TSGs.

RAN1#106-e Tdocs List

R1-2108726	UE feedback enhancements for HARQ-ACK	Huawei, HiSilicon

R1-2108727	CSI feedback enhancements	Huawei, HiSilicon

R1-2108728	Intra-UE multiplexing enhancements	Huawei, HiSilicon

R1-2108729	Uplink enhancements for URLLC in unlicensed controlled environments	Huawei, HiSilicon

R1-2108788	UE initiated COT for semi-static channel access	FUTUREWEI

R1-2108829	HARQ-ACK Enhancements for IIoT/URLLC	Ericsson

R1-2108830	CSI Feedback Enhancements for IIoT/URLLC	Ericsson

R1-2108831	Enhancements for IIoT/URLLC on Unlicensed Band	Ericsson

R1-2108832	Intra-UE Multiplexing/Prioritization Enhancements for IIoT/URLLC	Ericsson

R1-2108833	Propagation Delay Compensation Enhancements for Time Synchronization	Ericsson

R1-2108840	Discussion on HARQ-ACK enhancements for eURLLC	ZTE

R1-2108841	Discussion on CSI feedback enhancements for eURLLC	ZTE

R1-2108842	Discussion on unlicensed band URLLC/IIoT	ZTE

R1-2108843	Discussion on enhanced intra-UE multiplexing	ZTE

R1-2108844	Discussion on propagation delay compensation enhancements	ZTE

R1-2108906	Discussion on HARQ-ACK feedback enhancements for Rel-17 URLLC	Spreadtrum Communications

R1-2108907	Discussion on enhancements for unlicensed band URLLCIIoT	Spreadtrum Communications

R1-2108908	Discussion on intra-UE multiplexing/prioritization	Spreadtrum Communications

R1-2108966	HARQ-ACK enhancements for Rel-17 URLLC	vivo

R1-2108967	CSI feedback enhancements for Rel-17 URLLC	vivo

R1-2108968	Enhancements for unlicensed band URLLC/IIoT	vivo

R1-2108969	Intra-UE Multiplexing/Prioritization for Rel-17 URLLC	vivo

R1-2108970	Discussion on propagation delay compensation enhancements	vivo

R1-2109093	HARQ-ACK enhancements for Rel-17 URLLC/IIoT	OPPO

R1-2109094	CSI feedback enhancements for URLLC	OPPO

R1-2109095	Enhancements for unlicensed band URLLC/IIoT	OPPO

R1-2109096	Enhancements on intra-UE multiplexing/prioritization	OPPO

R1-2109097	Enhancement for support of time synchronization	OPPO

R1-2109131	UE feedback enhancements for HARQ-ACK	NEC

R1-2109132	Discussion on Intra-UE prioritization and multiplexing	NEC

R1-2109138	UL enhancements for IIoT/URLLC in unlicensed controlled environment	Nokia, Nokia Shanghai Bell

R1-2109159	HARQ-ACK Feedback Enhancements for URLLC/IIoT	Nokia, Nokia Shanghai Bell

R1-2109160	On UL intra-UE prioritization and multiplexing enhancements	Nokia, Nokia Shanghai Bell

R1-2109161	Discussion on enhancements for propagation delay compensation	Nokia, Nokia Shanghai Bell

R1-2109162	Moderator summary #1 on HARQ-ACK feedback enhancements for NR Rel-17 URLLC/IIoT	Moderator (Nokia)

R1-2109215	UE feedback enhancements for HARQ-ACK	CATT

R1-2109216	CSI feedback enhancements	CATT

R1-2109217	Discussion on remaining issues on enhancements for unlicensed band URLLC/IIoT	CATT

R1-2109218	Intra-UE multiplexing and prioritization	CATT

R1-2109219	Discussion on propagation delay compensation enhancements	CATT

R1-2109256	Discussion on some remaining issues for UE HARQ-ACK feedback enhancements	China Telecom

R1-2109259	Discussion on CSI Feedback Enhancements	Quectel, Langbo

R1-2109260	Discussion on Intra-UE Multiplexing/Prioritization	Quectel, Langbo

R1-2109277	Discussion on UE feeback enhancements for HARQ-ACK	CMCC

R1-2109278	Discussion on CSI feeback enhancements for URLLC	CMCC

R1-2109342	UE feedback enhancements for HARQ-ACK	CAICT

R1-2109354	UE feedback enhancements for HARQ-ACK	TCL Communication Ltd.

R1-2109355	Intra-UE multiplexing and prioritization	TCL Communication Ltd.

R1-2109356	Enhancements for unlicensed band URLLC/IIoT	NEC

R1-2109406	UE feedback enhancements for HARQ-ACK	Xiaomi

R1-2109407	Enhancement for unlicensed band URLLC IIoT	Xiaomi

R1-2109408	Intra-UE multiplexing prioritization for URLLC IIoT	Xiaomi

R1-2109453	Enhancements for unlicensed band URLLC/IIoT	Panasonic Corporation

R1-2109454	Discussion on Intra-UE multiplexing of different priority	Panasonic Corporation

R1-2109482	On HARQ-ACK reporting enhancements	Samsung

R1-2109483	Enhancements for unlicensed band URLLC/IIoT	Samsung

R1-2109484	Uplink intra-UE multiplexing and prioritization	Samsung

R1-2109485	Discussion for propagation delay compensation enhancements	Samsung

R1-2109575	On UE feedback enhancements for HARQ-ACK	MediaTek Inc.

R1-2109576	On the enhancements for unlicensed band URLLC/IIoT	MediaTek Inc.

R1-2109577	Methods for intra-UE multiplexing and prioritization	MediaTek Inc.

R1-2109604	Remaining issues of enhanced HARQ-ACK feedback procedures	Intel Corporation

R1-2109605	Remaining issues of enhanced sub-band CQI indication granularity	Intel Corporation

R1-2109606	Further Details for Enabling URLLC/IIOT in Unlicensed Band	Intel Corporation

R1-2109607	Further details of intra-UE uplink channel multiplexing and prioritization	Intel Corporation

R1-2109608	Remaining issues on propagation delay compensation	Intel Corporation

R1-2109671	Discussion on HARQ-ACK feedback enhancements for Rel.17 URLLC	NTT DOCOMO, INC.

R1-2109672	Discussion on CSI feedback enhancements for Rel.17 URLLC	NTT DOCOMO, INC.

R1-2109673	Discussion on enhancements for unlicensed band URLLC	NTT DOCOMO, INC.

R1-2109674	Discussion on intra-UE multiplexing/prioritization for Rel.17 URLLC	NTT DOCOMO, INC.

R1-2109729	CSI feedback enhancements	InterDigital, Inc.

R1-2109730	Intra-UE multiplexing and prioritization	InterDigital, Inc.

R1-2109743	Enhancements for support of time synchronization	Huawei, HiSilicon

R1-2109782	Considerations on HARQ-ACK enhancements for URLLC	Sony

R1-2109783	Remaining issues in 4 bits sub-band CQI reporting	Sony

R1-2109784	Considerations on unlicensed URLLC	Sony

R1-2109785	Considerations on intra-UE UL multiplexing	Sony

R1-2109809	UE feedback enhancements for HARQ-ACK	ETRI

R1-2109810	Enhancements for unlicensed band URLLC/IIoT	ETRI

R1-2109811	Intra-UE Multiplexing/Prioritization	ETRI

R1-2109821	Discussion on UE feedback enhancements for HARQ-ACK	Panasonic

R1-2109822	Discussion on UE feedback enhancements for HARQ-ACK	FGI, Asia Pacific Telecom

R1-2109893	HARQ enhancements for IIoT and URLLC	InterDigital, Inc.

R1-2109894	Enhancements for unlicensed band URLLC/IIoT	InterDigital, Inc.

R1-2109910	Feature lead summary#1 on CSI feedback enhancements for enhanced URLLC/IIoT	Moderator (InterDigital, Inc.)

R1-2109940	HARQ-ACK feedback enhancement for IIoT/URLLC	Lenovo, Motorola Mobility

R1-2109941	CSI feedback enhancements for URLLC/IIoT	Lenovo, Motorola Mobility

R1-2109942	Enhancements for unlicensed band URLLC/IIoT	Lenovo, Motorola Mobility

R1-2109943	Intra-UE multiplexing enhancement for IIoT/URLLC	Lenovo, Motorola Mobility

R1-2109970	Discussion on UE feedback enhancement for HARQ-ACK	LG Electronics

R1-2109971	Discussion on CSI feedback enhancements for URLLC	LG Electronics

R1-2109972	Discussion on unlicensed band URLLC/IIOT	LG Electronics

R1-2109973	Discussion on Intra-UE multiplexing/prioritization	LG Electronics

R1-2109974	Discussion on propagation delay compensation enhancements	LG Electronics

R1-2109994	Enhancements for unlicensed band URLLC/IIoT	Sharp

R1-2109995	Enhancements of channel collision resolution and intra-UE UCI multiplexing on PUCCH and PUSCH	Sharp

R1-2110027	Rel-17 URLLC UE feedback enhancements for HARQ-ACK	Apple

R1-2110028	Rel-17 URLLC UE feedback enhancement for CSI	Apple

R1-2110029	URLLC uplink enhancements for unlicensed spectrum	Apple

R1-2110030	Rel-17 URLLC intra-UE multiplexing/prioritization	Apple

R1-2110178	HARQ-ACK enhancement for IOT and URLLC	Qualcomm Incorporated

R1-2110179	CSI enhancement for IOT and URLLC	Qualcomm Incorporated, Samsung

R1-2110180	Uplink enhancements for URLLC in unlicensed controlled environments	Qualcomm Incorporated

R1-2110181	Intra-UE multiplexing and prioritization for IOT and URLLC	Qualcomm Incorporated

R1-2110182	Enhancements for support of time synchronization for enhanced IIoT and URLLC	Qualcomm Incorporated

R1-2110244	On the UE feedback enhancements for HARQ-ACK	ITRI

R1-2110245	Discussion on intra-UE multiplexing and prioritization	ITRI

R1-2110287	Discussion on PUCCH carrier switch for HARQ-ACK enhancement	ASUSTeK

R1-2110303	CSI feedback enhancements for URLLC/IIoT use cases	Nokia, Nokia Shanghai Bell

R1-2110323	Discussion on enhancement for unlicensed URLLC/IIoT	WILUS Inc.

R1-2110324	Discussion on intra-UE multiplexing/prioritization for URLLC/IIoT	WILUS Inc.

R1-2110395	Discussion on CSI feedback enhancements for eURLLC	ZTE

R1-2110396	Discussion on unlicensed band URLLC/IIoT	ZTE

R1-2110397	Discussion on propagation delay compensation enhancements	ZTE

R1-2110416	Rel-17 URLLC intra-UE multiplexing/prioritization	Apple

R1-2110423	Summary#1 - Enhancements for IIOT/URLLC on Unlicensed Band	Moderator (Ericsson)

R1-2110424	Summary#2 - Enhancements for IIOT/URLLC on Unlicensed Band	Moderator (Ericsson)

R1-2110425	Summary#3 - Enhancements for IIOT/URLLC on Unlicensed Band	Moderator (Ericsson)

R1-2110426	Summary#4 - Enhancements for IIOT/URLLC on Unlicensed Band	Moderator (Ericsson)

R1-2110427	Summary#5 - Enhancements for IIOT/URLLC on Unlicensed Band	Moderator (Ericsson)

R1-2110463	Summary#1 of email thread [106bis-e-NR-R17-IIoT-URLLC-04]	Moderator (OPPO)

R1-2110472	Summary#2 of email thread [106bis-e-NR-R17-IIoT-URLLC-04]	Moderator (OPPO)

R1-2110473	Feature lead summary#1 on propagation delay compensation enhancements	Moderator (Huawei)

R1-2110474	Feature lead summary#2 on propagation delay compensation enhancements	Moderator (Huawei)

R1-2110509	Feature lead summary#2 on CSI feedback enhancements for enhanced URLLC/IIoT	Moderator (InterDigital, Inc.)

R1-2110510	Further discussion on UCI multiplexing and power control in Rel-17 URLLC	Apple

R1-2110527	Moderator summary #2 on HARQ-ACK feedback enhancements for NR Rel-17 URLLC/IIoT	Moderator (Nokia)

R1-2110547	Summary#3 of email thread [106bis-e-NR-R17-IIoT-URLLC-04]	Moderator (OPPO)

R1-2110551	Feature lead summary #3 on CSI feedback enhancements for enhanced URLLC/IIoT	Moderator (InterDigital, Inc.)

R1-2110558	Feature lead summary#3 on propagation delay compensation enhancements	Moderator (Huawei)

R1-2110559	Feature lead summary#4 on propagation delay compensation enhancements	Moderator (Huawei)

R1-2110561	Moderator summary #3 on HARQ-ACK feedback enhancements for NR Rel-17 URLLC/IIoT	Moderator (Nokia)

R1-2110562	Final moderator summary on HARQ-ACK feedback enhancements for NR Rel-17 URLLC/IIoT	Moderator (Nokia)

R1-2110563	Summary of [106bis-e-R17-RRC-IIoT-URLLC] Email discussion on Rel-17 RRC parameters for IIoT and URLLC	Moderator (Nokia)

R1-2110594	Draft LS on propagation delay compensation	Huawei

R1-2110636	Summary#4 of email thread [106bis-e-NR-R17-IIoT-URLLC-04]	Moderator (OPPO)

R1-2110647	LS on propagation delay compensation	RAN1, Huawei

R1-2110651	Final feature lead summary on propagation delay compensation enhancements	Moderator (Huawei)

R1-2110653	Summary#6 - Enhancements for IIOT/URLLC on Unlicensed Band	Moderator (Ericsson)

R1-2110670	List of agreements of Rel-17 URLLC/IIoT WI (post RAN1#106bis-e)	WI rapporteur (Nokia)

R1-2110700	Happy Birthday Patrick	RAN1

RAN1#107-e Tdocs List

R1-2110818	UE feedback enhancements for HARQ-ACK	Huawei, HiSilicon

R1-2110819	Intra-UE multiplexing enhancements	Huawei, HiSilicon

R1-2110820	Uplink enhancements for URLLC in unlicensed controlled environments	Huawei, HiSilicon

R1-2110878	UE initiated COT for semi-static channel access	FUTUREWEI

R1-2110914	Discussion on HARQ-ACK enhancements for eURLLC	ZTE

R1-2110915	Discussion on unlicensed band URLLC/IIoT	ZTE

R1-2110916	Discussion on enhanced intra-UE multiplexing	ZTE

R1-2110917	Discussion on propagation delay compensation enhancements	ZTE

R1-2111005	Remaining issues on HARQ-ACK enhancements for Rel-17 URLLC	vivo

R1-2111006	Remaining issues on enhancements for unlicensed band URLLC/IIoT	vivo

R1-2111007	Remaining issues on intra-UE Multiplexing/Prioritization for Rel-17 URLLC	vivo

R1-2111008	Remaining issues on propagation delay compensation enhancements	vivo

R1-2111094	Discussion on HARQ-ACK feedback enhancements for Rel-17 URLLC	Spreadtrum Communications

R1-2111095	Discussion on enhancements for unlicensed band URLLC/IIoT	Spreadtrum Communications

R1-2111096	Discussion on intra-UE multiplexing/prioritization	Spreadtrum Communications

R1-2111139	HARQ-ACK Feedback Enhancements for URLLC/IIoT	Nokia, Nokia Shanghai Bell

R1-2111140	On UL intra-UE prioritization and multiplexing enhancements	Nokia, Nokia Shanghai Bell

R1-2111141	Discussion on enhancements for PDC and CSI	Nokia, Nokia Shanghai Bell

R1-2111142	Moderator summary #1 on HARQ-ACK feedback enhancements for NR Rel-17 URLLC/IIoT	Moderator (Nokia)

R1-2111176	Enhancements for unlicensed band URLLC/IIoT	NEC

R1-2111188	HARQ-ACK Enhancements for IIoT/URLLC	Ericsson

R1-2111189	Enhancements for IIoT/URLLC on Unlicensed Band	Ericsson

R1-2111190	Intra-UE Multiplexing/Prioritization Enhancements for IIoT/URLLC	Ericsson

R1-2111191	Propagation Delay Compensation Enhancements for Time Synchronization	Ericsson

R1-2111248	UE feedback enhancements for HARQ-ACK	CATT

R1-2111249	Discussion on remaining issues on enhancements for unlicensed band URLLC/IIoT	CATT

R1-2111250	Intra-UE multiplexing and prioritization	CATT

R1-2111251	Discussion on propagation delay compensation enhancements	CATT

R1-2111341	HARQ-ACK enhancements for Rel-17 URLLC/IIoT	OPPO

R1-2111342	Enhancements for unlicensed band URLLC/IIoT	OPPO

R1-2111343	Enhancements on intra-UE multiplexing/prioritization	OPPO

R1-2111344	Enhancement for support of time synchronization	OPPO

R1-2111359	Discussion on Intra-UE Multiplexing/Prioritization	Quectel, Langbo

R1-2111363	UL enhancements for IIoT/URLLC in unlicensed controlled environment	Nokia, Nokia Shanghai Bell

R1-2111390	Remaining issues in HARQ-ACK enhancements for URLLC	Sony

R1-2111391	Remaining issues in Unlicensed URLLC	Sony

R1-2111392	Remaining issues in Intra-UE UL Multiplexing	Sony

R1-2111434	Discussion on PUCCH carrier switching for HARQ feedback enhancement	China Telecom

R1-2111436	Discussion on Intra-UE multiplexing of different priority	Panasonic Corporation

R1-2111489	Remaining open issues of UE HARQ feedback enhancements	Intel Corporation

R1-2111490	Remaining Details for Enabling URLLC/IIoT in Unlicensed Band	Intel Corporation

R1-2111491	Remaining open details of intra-UE uplink channel multiplexing and prioritization	Intel Corporation

R1-2111492	Remaining open issues for propagation delay compensation	Intel Corporation

R1-2111567	UE feedback enhancements for HARQ-ACK	Xiaomi

R1-2111568	Enhancement for unlicensed band URLLC IIoT	Xiaomi

R1-2111569	Intra-UE multiplexing prioritization for URLLC IIoT	Xiaomi

R1-2111604	Discussion on UE feeback enhancements for HARQ-ACK	CMCC

R1-2111678	Discussion on UE feedback enhancements for HARQ-ACK	Panasonic

R1-2111704	UE feedback enhancements for HARQ-ACK	NEC

R1-2111705	Discussion on Intra-UE prioritization and multiplexing	NEC

R1-2111730	On HARQ-ACK reporting enhancements	Samsung

R1-2111731	Enhancements for unlicensed band URLLC/IIoT	Samsung

R1-2111732	Uplink intra-UE multiplexing and prioritization	Samsung

R1-2111733	Discussion for propagation delay compensation enhancements	Samsung

R1-2111791	Intra-UE multiplexing and prioritization	InterDigital, Inc.

R1-2111839	HARQ enhancements for IIoT and URLLC	InterDigital, Inc.

R1-2111840	Enhancements for unlicensed band URLLC/IIoT	InterDigital, Inc.

R1-2111867	HARQ-ACK feedback enhancements for URLLC	Apple

R1-2111868	URLLC uplink enhancements for unlicensed spectrum	Apple

R1-2111869	Intra-UE Multiplexing/Prioritization in Rel-17	Apple

R1-2111926	Enhancements for support of time synchronization	Huawei, HiSilicon

R1-2111942	HARQ-ACK feedback enhancement for IIoT/URLLC	Lenovo, Motorola Mobility

R1-2111943	Enhancements for unlicensed band URLLC/IIoT	Lenovo, Motorola Mobility

R1-2111944	Intra-UE multiplexing enhancement for IIoT/URLLC	Lenovo, Motorola Mobility

R1-2111988	UE feedback enhancements for HARQ-ACK	ETRI

R1-2111989	Remaining issues on enhancements for unlicensed band URLLC/IIoT	ETRI

R1-2111990	Intra-UE Multiplexing/Prioritization	ETRI

R1-2112013	Enhancements for unlicensed band URLLC/IIoT	Sharp

R1-2112014	Enhancements on intra-UE UCI multiplexing and channel collision resolution framework	Sharp

R1-2112052	Discussion on UE feedback enhancement for HARQ-ACK	LG Electronics

R1-2112053	Discussion on unlicensed band URLLC/IIOT	LG Electronics

R1-2112054	Discussion on Intra-UE multiplexing/prioritization	LG Electronics

R1-2112055	Discussion on propagation delay compensation enhancements	LG Electronics

R1-2112076	Discussion on UE feedback enhancements for HARQ-ACK	FGI, Asia Pacific Telecom

R1-2112081	UE feedback enhancements for HARQ-ACK	TCL Communication Ltd.

R1-2112083	Intra-UE multiplexing and prioritization	TCL Communication Ltd.

R1-2112102	Discussion on HARQ-ACK feedback enhancements for Rel.17 URLLC	NTT DOCOMO, INC.

R1-2112103	Discussion on intra-UE multiplexing/prioritization for Rel.17 URLLC	NTT DOCOMO, INC.

R1-2112173	On the UE feedback enhancements for HARQ-ACK	ITRI

R1-2112174	Discussion on intra-UE multiplexing	ITRI

R1-2112209	HARQ-ACK enhancement for IOT and URLLC	Qualcomm Incorporated

R1-2112210	Uplink enhancements for URLLC in unlicensed controlled environments	Qualcomm Incorporated

R1-2112211	Intra-UE multiplexing and prioritization for IOT and URLLC	Qualcomm Incorporated

R1-2112212	Enhancements for support of time synchronization for enhanced IIoT and URLLC	Qualcomm Incorporated

R1-2112285	On UE feedback enhancements for HARQ-ACK	MediaTek Inc.

R1-2112286	On the enhancements for unlicensed band URLLC/IIoT	MediaTek Inc.

R1-2112287	Methods for intra-UE multiplexing and prioritization	MediaTek Inc.

R1-2112386	Remaining issues on enhancement for unlicensed URLLC/IIoT	WILUS Inc.

R1-2112387	Discussion on intra-UE multiplexing/prioritization for URLLC/IIoT	WILUS Inc.

R1-2112395	UE feedback enhancements for HARQ-ACK for NR Rel-17 URLLC/IIoT	CAICT

R1-2112429	Introduction of UE initiating a channel occupancy in semi-static channel access mode for enhanced IIoT and URLLC operation on shared spectrum for NR	Ericsson

R1-2112444	Introduction of IIoT/URLLC enhancements in NR	Samsung

R1-2112475	Introduction of Rel-17 enhanced IIoT and URLLC	Huawei

R1-2112476	Feature lead summary#1 on propagation delay compensation enhancements	Moderator (Huawei)

R1-2112477	Feature lead summary#2 on propagation delay compensation enhancements	Moderator (Huawei)

R1-2112484	Introduction of enhanced Industrial Internet of Things (IoT) and ultra-reliable and low latency communication (URLLC) support for NR	Nokia

R1-2112514	Introduction of IIoT/URLLC enhancements in NR	Qualcomm

R1-2112545	Summary#1 - Enhancements for IIOT/URLLC on Unlicensed Band	Moderator (Ericsson)

R1-2112546	Summary#2 - Enhancements for IIOT/URLLC on Unlicensed Band	Moderator (Ericsson)

R1-2112547	Summary#3 - Enhancements for IIOT/URLLC on Unlicensed Band	Moderator (Ericsson)

R1-2112548	Summary#4 - Enhancements for IIOT/URLLC on Unlicensed Band	Moderator (Ericsson)

R1-2112549	Summary#5 - Enhancements for IIOT/URLLC on Unlicensed Band	Moderator (Ericsson)

R1-2112550	Summary#6 - Enhancements for IIOT/URLLC on Unlicensed Band	Moderator (Ericsson)

R1-2112564	Summary #1 of email thread [107-e-NR-R17-IIoT-URLLC-03]	Moderator (CATT)

R1-2112604	Summary#1 of email thread [107-e-NR-R17-IIoT-URLLC-04]	Moderator (OPPO)

R1-2112616	Moderator summary #2 on HARQ-ACK feedback enhancements for NR Rel-17 URLLC/IIoT	Moderator (Nokia)

R1-2112667	Summary#2 of email thread [107-e-NR-R17-IIoT-URLLC-04]	Moderator (OPPO)

R1-2112694	Summary #1 of [107-e-NR-R17-IIoT-URLLC-06] on remaining issues on CSI feedback enhancement	Moderator (InterDigital, Inc.)

R1-2112712	Summary #2 of email thread [107-e-NR-R17-IIoT-URLLC-03]	Moderator (CATT)

R1-2112728	Feature lead summary#3 on propagation delay compensation enhancements	Moderator (Huawei)

R1-2112729	Draft LS on propagation delay compensation	Huawei

R1-2112757	Moderator summary #3 on HARQ-ACK feedback enhancements for NR Rel-17 URLLC/IIoT	Moderator (Nokia)

R1-2112758	Final moderator summary on HARQ-ACK feedback enhancements for NR Rel-17 URLLC/IIoT	Moderator (Nokia)

R1-2112760	Summary of [107-e-R17-RRC-IIoT-URLLC] Email discussion on Rel-17 RRC parameters for IIoT and URLLC	Moderator (Nokia)

R1-2112761	List of agreements of Rel-17 URLLC/IIoT WI (post RAN1#107-e)	WI rapporteur (Nokia)

R1-2112785	Summary#3 of email thread [107-e-NR-R17-IIoT-URLLC-04]	Moderator (OPPO)

R1-2112834	LS on propagation delay compensation	RAN1, Huawei

R1-2112835	Final feature lead summary on propagation delay compensation enhancements	Moderator (Huawei)

RAN2#116-e Tdocs List

R2-2109327	LS on TA-based propagation delay compensation (R1-2108635; contact: Huawei)

R2-2111217	LS on propagation delay compensation (R1-2110647; contact: Huawei)

R2-2110441	Stage-2 Running CR for Rel-17 IIoT/URLLC	Nokia, Nokia Shanghai Bell

R2-2110495	MAC Running CR for Rel-17 IIoT/URLLC	Samsung

R2-2109302	RE: LS on Time Synchronization	IEEE 1588 WG	LS in	To:RAN, SA	Cc:RAN2

R2-2109599	Discussion about propagation delay compensation for accurate time synchronization	Huawei, HiSilicon

R2-2109776	Summary of PDC Issues	Ericsson

R2-2109925	Propagation Delay Compensation for TSN	Qualcomm Incorporated

R2-2109990	Discussion on propagation delay compensation	vivo

R2-2110107	Remaining FFSs on time synchronization and PDC	ZTE Corporation, Sanechips, China Southern Power Grid Co., Ltd

R2-2110199	Discussion on propagation delay compensation for TSN 	NTT DOCOMO INC.

R2-2110318	Left issues for propagation delay compensation		Lenovo, Motorola Mobility

R2-2110442	Views on Support of Propagation Delay Compensation		Nokia, Nokia Shanghai Bell

R2-2110496	Issues on Propagation Delay Compensation	Samsung

R2-2110587	Consideration on the support of time synchronization enhancement	OPPO

R2-2110801	Remaining issues of timing synchronization		Intel Corporation

R2-2110963	Discussion about propagation delay compensation enhancements	China Telecommunications

R2-2111046	Time synchronization for TSN based on RAN1 progress		CMCC

R2-2111257	Summary of AI 8.5.2 on Time Synchronization	CMCC

R2-2109600	Remaining issues about uplink enhancements for URLLC in UCE	Huawei, HiSilicon

R2-2109653	cg-RetransmissionTimer configured without autonomousTx	CATT

R2-2109777	Harmonizing UL CG enhancements in NR-U and URLLC	Ericsson

R2-2109926	CG Harmonization for Unlicensed Controlled Environment	Qualcomm Incorporated

R2-2109991   Autonomous Re-transmission		vivo

R2-2110243	Remaining details on enhancements for URLLC in UCE		Lenovo, Motorola Mobility

R2-2110443	Remaining Issues on HARQ Process Selection for Configured Grant	Nokia, Nokia Shanghai Bell

R2-2110497	Remaining Issues on Intra-CG Prioritization and LCH-based Prioritization in UCE	Samsung

R2-2110588	Consideration on URLLC over NR-U		OPPO

R2-2110623	Further Consideration on the Intra-UE multiplexing in UCE	ZTE Corporation,Sanechips

R2-2110672	Remaining issues of CG harmonization	Xiaomi Communications

R2-2110754	Remaining issues for UCE	MediaTek Inc.

R2-2110916	IIoT operation in unlicensed controlled environments	InterDigital

R2-2111104	Remaining issues of IIoT in UCE	III

R2-2111169	Remaining issues in intraCG-Prioritization	LG Electronics Inc.

R2-2111262	Summary of Agenda Item 8.5.3: Uplink enhancements for URLLC in unlicensed controlled environments	OPPO

R2-2109602	Summary of [Post115-e][513][IIoT] QoS survival time	Huawei, HiSilicon

R2-2109601	Discussion on two-level PERs for survival time handling	Huawei, HiSilicon

R2-2109603	TP of baseline CR for Survival Time state operation	Huawei, HiSilicon

R2-2109654	HARQ NACK solution: addressing concerns and design details	CATT, CMCC

R2-2109655	TPs capturing HARQ-NACK solution		CATT

R2-2109709	L1/L2 configuration adaptation		Fujitsu

R2-2109710	Additional thought on supporting N>1	Fujitsu

R2-2109778	RAN enhancements based on new QoS related parameters	Ericsson

R2-2109927	RAN Enhancement to support Survival Time	Qualcomm Incorporated

R2-2109992	Discussion on HARQ NACK solution	vivo

R2-2110067	Remaining QoS solution aspects	Apple

R2-2110068	Adaptive configuration for CG/SPS	Apple

R2-2110069	Further considerations on survival time for new QoS	Apple

R2-2110108	N and combined Tx-side timer for IIoT QoS	ZTE, Sanechips, China Southern Power Grid Co., Ltd, TCL Communication Ltd., vivo

R2-2110201	Discussion on survival time state	NTT DOCOMO INC.

R2-2110227	Remaining issues on the support of survival time	Lenovo, Motorola Mobility

R2-2110263	Discussion on the RAN solution for introduction of new QoS parameters	CMCC

R2-2110345	Finalising Survival Time related enhancements	Samsung Electronics GmbH

R2-2110444	An Overview of Survival Time Enhancements	Nokia, Nokia Shanghai Bell

R2-2110589	Consideration on the support of survival time	OPPO

R2-2110673	Clarification on the survival time requirement	Xiaomi Communications

R2-2110791	On counting HARQ-NACKs for triggering survival time state	Futurewei Technologies

R2-2110802	Survival time handling	Intel Corporation

R2-2110913	Enhancements based on new QoS requirements	InterDigital

R2-2110918	Issues with UE Survival Time support	Sequans Communications

R2-2110965	Discussion on RAN enhancement to support survival time 	China Telecommunications

R2-2111167	Remaining aspects in ST mechanism	LG Electronics Inc.

R2-2111183	Discussion of RAN Enhancements to Support Survival Time	TCL Communication Ltd.

RAN3#114-e Tdoc List

R3-214625 Introduction of Enhanced IIoT support over NG (CATT, Nokia, Nokia Shanghai Bell, Samsung, Huawei, Ericsson, ZTE)

R3-214626 Introduction of Enhanced IIoT support over E1 (ZTE, Nokia, Nokia Shanghai Bell, Samsung, CATT, Huawei, Ericsson)

R3-214627 Introduction of Enhanced IIoT support over F1 (Huawei, Nokia, Nokia Shanghai Bell, CATT, Ericsson, ZTE, Samsung)

R3-214657 Introduction of Enhanced IIoT support over Xn (Ericsson, Samsung, Huawei, ZTE,CATT, Nokia, Nokia Shanghai Bell)

R3-214735 (TP for Introduction of Enhanced IIoT support over NG and Xn) Further discussion on Time Synchronization enhancements (ZTE)

R3-214736 (TP for Introduction of Enhanced IIoT support over F1)Time Synchronization enhancements (ZTE)

R3-214828 (TP for NR_IIOT_URLLC_enh BL CR for TS 38.413) Time synchronisation assistance information (Nokia, Nokia Shanghai Bell)

R3-214829 (TP for NR_IIOT_URLLC_enh BL CR for TS 38.423) Impact of handover on time synchronization (Nokia, Nokia Shanghai Bell)

R3-214903 (TP for BL CR for 38.413 IIOT) AS Time Distribution (Qualcomm Incorporated)

R3-215081 Discussion on supporting the gNB-based Packet Delay Compensation (Samsung)

R3-215082 (TP for NR_IIOT_URLLC_enh BL CR for TS 38.413) Discussion on supporting the time synchronization error budget (Samsung)

R3-215122 TP for BLCR for 38.413 on Propagation Delay Compensation Enhancements (CATT)

R3-215131 (TP for eIIOT BLCR for TS 38.413) Supporting propagation delay compensation enhancements (Huawei)

R3-215132 (TP for eIIOT BLCR for TS 38.423) Supporting propagation delay compensation enhancements (Huawei)

R3-215154 Discussion on Further enhanced NR-IIoT: Enhancements for support of time synchronization (Ericsson)

R3-215155 Enhancements for support of time synchronization (Ericsson)

R3-215156 Enhancements for support of time synchronization (Ericsson)

R3-214737 (TP for Introduction of Enhanced IIoT support over NG/Xn/F1/E1) Analysis of New QoS Related parameters (ZTE)

R3-214830 (TP for NR_IIOT_URLLC_enh BL CR for TS 38.423) Survival Time and handover (Nokia, Nokia Shanghai Bell)

R3-215083 Discussion on the remaining issues for the Survival Time (Samsung)

R3-215123 Discussion on new QoS related parameters (CATT)

R3-215124 (TP for BLCR for TS 38413) new QoS related parameters (CATT)

R3-215133 (TP for eIIOT BLCR for TS 38.473) Survival time remaining issues (Huawei)

RAN4#101-e Tdoc List

R4-2117497	On propagation delay compensation enhancements for TSN	Qualcomm Incorporated

R4-2117744	Discussion and draft LS on propagation delay compensation enhancement	CMCC

R4-2117764	Further discussion on propagation delay compensation enhancements	vivo

R4-2117765	Further discussion on reference point for Te requirements	vivo

R4-2118040	Discussion on reference point for Te requirements	Intel Corporation

R4-2118266	LS reply on TA-based propagation delay compensation	Ericsson

R4-2118930	Discussions on the reference point for Te requirements	ZTE Corporation

R4-2119270	Propagation delay compensation enhancements	MediaTek inc.

R4-2119271	Reference point for Te requirements	MediaTek inc.

R4-2119317	RRM impact on PUCCH carrier switching	Nokia

R4-2119319	Discussion on TA-based propagation delay compensation enhancement	Nokia

R4-2119361	On PDC enhancements	Huawei, Hisilicon

R4-2119362	On reference point for Te requirements	Huawei, Hisilicon

R4-2119363	Discussion on RRM impacts of PUCCH carrier switching	Huawei, Hisilicon

R4-2119454	LS response on UE transmit timing error	Ericsson, Nokia, Nokia Shanghai Bell

R4-2120233	Email discussion summary for [101-e][238] NR_IIOT_URLLC_enh	Moderator (Nokia)

R4-2120335	WF on NR enhanced IIoT and URLLC support	Nokia, Nokia Shanghai Bell

R4-2120336	Reply LS on TA-based propagation delay compensation	Nokia, Nokia Shanghai Bell

R4-2120380   Email discussion summary for [101-e][238] NR_IIOT_URLLC_enh	Moderator (Nokia)

09.11.2020		minor adaptations for RAN #90e

31.08.2020		minor adaptations for RAN #89e

20.04.2020		minor adaptations for RAN #88e

18.02.2020		minor adaptations for RAN #87e

14.11.2019		minor adaptations for RAN #86

18.08.2019		minor adaptations for RAN #85

12.05.2019		minor adaptations for RAN #84

27.02.2019		minor adaptations for RAN #83

21.11.2018		completion levels with colours added (for RAN #82)

v04.81	31.07.2018		simplification of template and addition of cross-TSG aspects (for RAN #81)

v04.80	21.05.2018		minor adaptations for RAN #80

v04.79	26.02.2018		minor adaptations for RAN #79

v04.78	18.11.2017		minor adaptations for RAN #78

v04.77	06.08.2017		minor adaptations for RAN #77

v04.76	15.05.2017		minor adaptations for RAN #76

v04.75	31.01.2017		minor adaptations for RAN #75

v04.74	28.10.2016		minor adaptations for RAN #74

v04.73	01.09.2016		adaptations for RAN #73 (time units in extra Excel table, RAN6 reporting included)

v04.72	26.05.2016		adaptations for RAN #72 (introduction of NR & GERAN TUs)

v04.71	10.02.2016		minor adaptations for RAN #71

v04.70	30.10.2015		minor adaptations for RAN #70

v04.69	12.08.2015		minor adaptations for RAN #69

v04.68	21.05.2015		minor adaptations for RAN #68

v04.67	01.02.2015		minor adaptations for RAN #67

v04.66	16.11.2014		minor adaptations for RAN #66

v04.65	16.08.2014		minor adaptations for RAN #65

v04.64	22.05.2014		minor adaptations for RAN #64

v04.63	24.01.2014		restructuring for RAN #63 to cover Core & Perf. in one doc file

v03.62	11.11.2013		section 1.2.3 adapted for RAN #62

v03	11.08.2013		section 1.2.3 added on time budget

v02	07.05.2010		history added, some spelling corrections

v01	13.11.2009		First version of the template