3GPP TSG-RAN WG1 Meeting #107-e	R1-2112852

# e-Meeting, November 11th – 19th, 2021

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.214 | CR | 0221 | rev | - | Current version: | 15.14.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-15 editorial corrections for TS 38.214 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2021-11-19 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In clause 3.3, DM-RS is not differentiated by “common” or “dedicated” in NR. Its full name should be “demodulation reference signal”, which is correctly captured in TS 38.211. While in TS 38.214, “dedicated demodulation reference signals” is used for the abbreviation DM-RS. In clause 5.2.3, related to Clause 6.3.2.1.2 [TS 38.212], for Type I codebook and Type II codebook, mapping order of CSI fields of one CSI report for CSI Part 2 wideband is given as shown in the following table. Table 6.3.2.1.2-4: Mapping order of CSI fields of one CSI report, CSI part 2 wideband CSI report number; CSI fields. CSI report #n CSI part 2 wideband; Wideband CQI for the second TB as in Tables 6.3.1.1.2-3/4/5, if present and reported. Layer Indicator as in Tables 6.3.1.1.2-3/4/5, if reported. PMI wideband information fields , from left to right as in Tables 6.3.1.1.2-1/2 or 6.3.2.1.2-1/2, if reported. PMI wideband information fields , from left to right as in Tables 6.3.1.1.2-1/2 or 6.3.2.1.2-1/2, or codebook index for 2 antenna ports according to Clause 5.2.2.2.1 in [6, TS38.214], if pmi-FormatIndicator= widebandPMI and if reported. It can be observed that layer indicator (LI) and PMI are included in CSI Part 2 if they are reported. However, in Clause 5.2.3 [TS 38.214], Part 2 does not contain LI for Type I and Type II CSI feedback. For Type I CSI feedback, it is specefied that Part 2 contains PMI (if reported) and contains the CQI for the second codeword (if reported) when RI (if reported) is larger than 4 in Clause 5.2.3 [TS 38.214]. In fact, no matter RI is reported or not, there are always two codewords when RI is larger than 4. Therefore, Part 2 should contain the CQI for the second codeword (if reported) when RI is larger than 4 even if RI is not reported. Hence, the condition in bracket for RI is unnecessary. For Type II CSI feedback, it is specified that the elements of i1,4,l, i2,1,l (if reported) and i2,2,l (if reported) are reported in the increasing order of their indices, i=0,1,…, L-1, where the element of the lowest index is mapped to the most significant bits and the element of the highest index is mapped to the least significant bits, in Clause 5.2.3 [TS 38.214]. There are 2L elements for i1,4,l, i2,1,l and i2,2,l according to the illustration in Caluse 5.2.2.2.3 [TS 38.214]. Hence, their indices i should be 0,1,…, 2L-1. |
| Summary of change: |  | In clause 3.3, Change “Dedicated demodulation reference signals” to “Demodulation reference signal” as the full name of DM-RS. In clause 5.2.3, clarified that the contents of Part 2 for Type I, Type II CSI feedback and corrected the index value of the elements of i1,4,l, i2,1,l and i2,2,l. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | For the changes in clause 3.3, Misalignment of the full name will cause confusion. For the changes in clause 5.2.3, UE’s behaviour of CSI reporting for Part 2 is not clear for Type I and Type II. The elements of i1,4,l, i2,1,l(if reported) and i2,2,l (if reported) with their indices i = L,…,2L-1 are not reported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.3, 5.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<omitted text>

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

BWP	Bandwidth part

CBG	Code block group

CP	Cyclic prefix

CQI	Channel quality indicator

CPU	CSI processing unit

CRB	Common resource block

CRC	Cyclic redundancy check

CRI	CSI-RS Resource Indicator

CSI	Channel state information

CSI-RS	Channel state information reference signal

CSI-RSRP	CSI reference signal received power

CSI-RSRQ	CSI reference signal received quality

CSI-SINR	CSI signal-to-noise and interference ratio

CW	Codeword

DCI	Downlink control information

DL	Downlink

DM-RS	Demodulation reference signals

EPRE	Energy per resource element

L1-RSRP	Layer 1 reference signal received power

LI	Layer Indicator

MCS	Modulation and coding scheme

PDCCH	Physical downlink control channel

PDSCH	Physical downlink shared channel

PSS	Primary Synchronisation signal

PUCCH	Physical uplink control channel

QCL	Quasi co-location

PMI	Precoding Matrix Indicator

PRB	Physical resource block

PRG	Precoding resource block group

PT-RS	Phase-tracking reference signal

RB	Resource block

RBG	Resource block group

RI	Rank Indicator

RIV	Resource indicator value

RS	Reference signal

SLIV	Start and length indicator value

SR	Scheduling Request

SRS	Sounding reference signal

SS	Synchronisation signal

SSS	Secondary Synchronisation signal

SS-RSRP	SS reference signal received power

SS-RSRQ	SS reference signal received quality

SS-SINR	SS signal-to-noise and interference ratio

TB	Transport Block

TCI	Transmission Configuration Indicator

TDM	Time division multiplexing

UE	User equipment

UL	Uplink

<omitted text>

### 5.2.3	CSI reporting using PUSCH

A UE shall perform aperiodic CSI reporting using PUSCH on serving cell c upon successful decoding of a DCI format 0_1 which triggers an aperiodic CSI trigger state.

An aperiodic CSI report carried on the PUSCH supports wideband, and sub-band frequency granularities. An aperiodic CSI report carried on the PUSCH supports Type I and Type II CSI.

A UE shall perform semi-persistent CSI reporting on the PUSCH upon successful decoding of a DCI format 0_1 which activates a semi-persistent CSI trigger state. DCI format 0_1 contains a CSI request field which indicates the semi-persistent CSI trigger state to activate or deactivate. Semi-persistent CSI reporting on the PUSCH supports Type I and Type II CSI with wideband, and sub-band frequency granularities. The PUSCH resources and MCS shall be allocated semi-persistently by an uplink DCI.

CSI reporting on PUSCH can be multiplexed with uplink data on PUSCH except that semi-persistent CSI reporting on PUSCH activated by a DCI format is not expected to be multiplexed with uplink data on the PUSCH. CSI reporting on PUSCH can also be performed without any multiplexing with uplink data from the UE.

Type I CSI feedback is supported for CSI Reporting on PUSCH. Type I wideband and sub-band CSI is supported for CSI Reporting on the PUSCH. Type II CSI is supported for CSI Reporting on the PUSCH.

For Type I and Type II CSI feedback on PUSCH, a CSI report comprises of two parts. Part 1 has a fixed payload size and is used to identify the number of information bits in Part 2. Part 1 shall be transmitted in its entirety before Part 2.

-	For Type I CSI feedback, Part 1 contains RI (if reported), CRI (if reported), CQI for the first codeword (if reported). Part 2 contains PMI (if reported), LI (if reported) and contains the CQI for the second codeword (if reported) when RI is larger than 4.

-	For Type II CSI feedback, Part 1 contains RI (if reported), CQI, and an indication of the number of non-zero wideband amplitude coefficients per layer for the Type II CSI (see clause 5.2.2). The fields of Part 1 – RI (if reported), CQI, and the indication of the number of non-zero wideband amplitude coefficients for each layer – are separately encoded. Part 2 contains the PMI and LI (if reported) of the Type II CSI. The elements of i1,4,l, i2,1,l (if reported) and i2,2,l (if reported) are reported in the increasing order of their indices, i=0,1,…, L-1, where the element of the lowest index is mapped to the most significant bits and the element of the highest index is mapped to the least significant bits. Part 1 and 2 are separately encoded.

A Type II CSI report that is carried on the PUSCH shall be computed independently from any Type II CSI report that is carried on the PUCCH formats 3 or 4 (see clause 5.2.4 and 5.2.2).

<omitted text>