3GPP TSG-WG1 Meeting # 107-e	R1-2112912

# e-Meeting, November 11th - 19th, 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.213 | CR | 1413 | rev | - | Current version: | 16.7.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of NB-IoT/eMTC support for Non-Terrestrial Networks in 36.213 s10-s13 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Motorola Mobility |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NBIOT_eMTC_NTN-Core |  |  |  |  |  | Date: |  |  | 2021-11-28 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Introduction of NB-IoT/eMTC support for Non-Terrestrial Networks. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Support for NB-IoT/eMTC support for Non-Terrestrial Networks features according to RAN1 agreements: - timing relationship enhancements. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No support of NB-IoT/eMTC support for Non-Terrestrial Networks. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10, 10.1.1, 10.1.2.1, 10.1.3, 10.1.3.1, 10.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | x |  | Other core specifications |  |  |  | TS 36.211 |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# 10	Physical uplink control channel procedures

If the UE is configured with shortTTI, PUCCH in this clause refers to SPUCCH defined in [3] if the HARQ-ACK is sent in response to PDSCH scheduled by DCI format 7-1A/7-1B/7-1C/7-1D/7-1E/7-1F/7-1G or scheduling request is sent on resources configured by higher layer parameter sr-slotSPUCCH-IndexFH or sr-slotSPUCCH-IndexNoFH or sr-subslotSPUCCH-Resource for slot/subslot-based transmissions, unless otherwise noted.

If the UE is not configured with shortTTI or the UE is configured with shortTTI, and UCI is to be transmitted in a subframe, the term 'subframe/slot/subslot' or 'subframe/slot' refers to a subframe in this clause.

If the UE is configured with shortTTI, and UCI is to be transmitted in a slot, the term 'subframe/slot/subslot' or 'slot/subslot' or 'subframe/slot' refers to a slot in this clause.

If the UE is configured with shortTTI, and UCI is to be transmitted in a subslot, the term 'subframe/slot/subslot' or 'slot/subslot' refers to a subslot in this clause.

If the UE is configured with a SCG, the UE shall apply the procedures described in this clause for both MCG and SCG

-	When the procedures are applied for MCG, the terms 'secondary cell', 'secondary cells', 'serving cell', 'serving cells' in this clause refer to secondary cell, secondary cells, serving cell, serving cells belonging to the MCG respectively.

-	When the procedures are applied for SCG, the terms 'secondary cell', 'secondary cells', 'serving cell', 'serving cells' in this clause refer to secondary cell, secondary cells (not including PSCell), serving cell, serving cells belonging to the SCG respectively. The term 'primary cell' in this clause refers to the PSCell of the SCG.

If the UE is configured with a PUCCH-SCell, the UE shall apply the procedures described in this clause for both primary PUCCH group and secondary PUCCH group

-	When the procedures are applied for the primary PUCCH group, the terms 'secondary cell', 'secondary cells', 'serving cell', 'serving cells' in this clause refer to secondary cell, secondary cells, serving cell, serving cells belonging to the primary PUCCH group respectively.

-	When the procedures are applied for secondary PUCCH group, the terms 'secondary cell', 'secondary cells', 'serving cell', 'serving cells' in this clause refer to secondary cell, secondary cells (not including the PUCCH-SCell), serving cell, serving cells belonging to the secondary PUCCH group respectively. The term 'primary cell' in this clause refers to the PUCCH-SCell of the secondary PUCCH group.

If a UE is configured with a LAA Scell, the UE shall apply the procedures described in this clause assuming frame structure type 1 for the LAA Scell unless stated otherwise.

A UE is not expected to be configured with PUCCH on a LAA SCell.

Throughout this clause,

-	if the UE is configured with higher layer parameter shortProcessingTime and the corresponding PDCCH with CRC scrambled by C-RNTI is in the UE-specific search space,  and otherwise,

-	if the UE is configured with higher layer parameter shortTTI and the corresponding PDCCH/SPDCCH with DCI format 7-1A/7-1B/7-1C/7-1D/7-1E/7-1F/7-1G is detected in a subslot,  is determined based on higher layer parameter min-proc-TimelineSubslot from .

-	for a BL/CE UE, the value of Koffset is given by,

-	if the UE is configured with the higher layer parameter CellSpecificKoffset,

-	Koffset= Kcell_offset-KUE_offset where

Kcell_offset is the parameter CellSpecificKoffset provided by higher layers, and

KUE_offset is the parameter UESpecificKoffset provided by higher layers, otherwise KUE_offset=0

-	otherwise,

-	Koffset=0.

For a UE configured with EN-DC/NE-DC,

-	if serving cell frame structure type 1 and if the UE is configured with tdm-PatternConfig/tdm-PatternConfigNE-DC for the serving cell, or

-	if the UE configured with EN-DC, and if serving cell frame structure type 1, and if the UE is configured with tdm-PatternConfig2 for the serving cell, or

-	if the UE configured with EN-DC, and if primary cell frame structure type 2 and if the UE is configured with tdm-PatternConfig2 for the primary cell, and if the UE configured with more than one serving cells, and if secondary serving cell frame structure type 2 with different UL/DL configuration than the primary cell, or if secondary serving cell is frame structure type 1,

-	the UE shall apply the procedures described in this clause assuming FDD-TDD and primary cell frame structure type 2 with "UL/DL configuration" given by tdm-PatternConfig/tdm-PatternConfigNE-DC/tdm-PatternConfig2 and serving cell frame structure type 1. The UE shall apply an offset value given by harq-Offset-r15/harq-Offset-r16 to the subframe index in the UL/DL configuration when applying the procedures in this clause.

-	if the UE configured with EN-DC, and if primary cell frame structure type 2 and if the UE is configured with tdm-PatternConfig2 for the primary cell, and if the UE configured with more than one serving cells, and if secondary serving cell has the same UL/DL configuration as the primary cell,

-	the UE shall apply the procedures described in this clause assuming the UE is configured with more than one serving cell with "UL/DL configuration" given by tdm-PatternConfig2 and serving cell frame structure type 2. The UE shall apply an offset value given by harq-Offset-r16 to the subframe index in the UL/DL configuration when applying the procedures in this clause.

-	if serving cell frame structure type 1 and if the UE is configured with tdm-PatternConfig/tdm-PatternConfigNE-DC for the serving cell, or if the UE configured with EN-DC, and if the UE does not indicate a capability for dynamic power sharing (as specified in [17]), and if the UE is configured with tdm-PatternConfig2 for the serving cell, the UE is not expected to transmit any uplink physical channel or signal in the serving cell on subframes other than the offset-UL subframes, where the offset-UL subframes are determined by applying the offset value to the subframes denoted as uplink in the UL/DL configuration.

-	The UE is configured by higher layers to use PUCCH format 3/4/5 for transmission of HARQ-ACK.

-	if the UE configured with EN-DC, and if primary cell frame structure type 2 and if the UE is configured with tdm-PatternConfig2 for the primary cell, and if the UE only receives the DCI format with value of the DAI field equal to '1' and/or only receives a PDSCH transmission on the primary cell where there is not a corresponding PDCCH/EPDCCH detected, the UE shall use PUCCH format 3 for transmission of HARQ-ACK with the PUCCH resource corresponding to the 1st PUCCH resource value configured by the higher layers.

-	The UE shall use  and  (defined in Clause 10.1.3.2.2/10.1.3.2.3/10.2.3.2.4) when applying the procedures described in Clause 10.1.3.2.2/10.1.3.2.3/10.2.3.2.4.

<Unchanged parts are omitted>

### 10.1.1	PUCCH format information

Using the PUCCH formats defined in Clause 5.4.1, 5.4.2, 5.4.2A, 5.4.2B, 5.4.2C, 5.4A.2, 5.4A.3, and 5.4A.4 in [3], the following combinations of UCI on PUCCH are supported:

-	Format 1a for 1-bit HARQ-ACK or in case of FDD or FDD-TDD primary cell frame structure type 1 for 1-bit HARQ-ACK with positive SR.

-	Format 1b for 2-bit HARQ-ACK or for 2-bit HARQ-ACK with positive SR.

-	Format 1b for up to 4-bit HARQ-ACK with channel selection when the UE is configured with more than one serving cell or, in the case of TDD, when the UE is configured with a single serving cell.

-	Format 1 for positive SR.

-	Format 2 for a CSI report when not multiplexed with HARQ-ACK.

-	Format 2a for a CSI report multiplexed with 1-bit HARQ-ACK for normal cyclic prefix.

-	Format 2b for a CSI report multiplexed with 2-bit HARQ-ACK for normal cyclic prefix.

-	Format 2 for a CSI report multiplexed with HARQ-ACK for extended cyclic prefix.

-	For subframe-PUCCH, format 3 for up to 10-bit HARQ-ACK for FDD or FDD-TDD primary cell frame structure type 1 and for up to 20-bit HARQ-ACK for TDD and for up to 21 bit HARQ-ACK for FDD-TDD primary cell frame structure type 2.

-	For subframe-PUCCH, format 3 for up to 11-bit corresponding to 10-bit HARQ-ACK and 1-bit positive/negative SR for FDD or FDD-TDD and for up to 21-bit corresponding to 20-bit HARQ-ACK and 1-bit positive/negative SR for TDD and for up to 22-bit corresponding to 21-bit HARQ-ACK and 1-bit positive/negative SR for FDD-TDD primary cell frame structure type 2.

-	For subframe-PUCCH, format 3 for HARQ-ACK, 1-bit positive/negative SR (if any) and CSI report(s).

-	For subframe-PUCCH, format 3 for up to 22 bits of UCI including HARQ-ACK, SR (if any) and periodic CSI report(s) (if any) for UE configured with Format 4 or Format 5 or for UE configured with more than 5 serving cells..

-	For slot-PUCCH, Format 3 for up to 11-bits of UCI including HARQ-ACK, SR.

-	For subframe-PUCCH, format 4 for more than 22 bits of UCI including HARQ-ACK, SR (if any) and periodic CSI report(s) (if any).

-	For slot-PUCCH, Format 4 for more than 11 bits of UCI including HARQ-ACK, SR (if any) for UE configured with slot-PUCCH format 3

-	For slot-PUCCH, Format 4 for more than 2 bits of UCI including HARQ-ACK, SR (if any) for UE not configured with slot-PUCCH format 3

-	For subslot-PUCCH, Format 4 for more than 3 bits of UCI including HARQ-ACK, SR (if any)

-	For subframe-PUCCH, format 5 for more than 22 bits of UCI including HARQ-ACK, SR (if any) and periodic CSI report(s) (if any).

-	For subframe-PUCCH, format 4 for more than one CSI report, SR (if any) and HARQ-ACK corresponding to PDSCH transmission only on the primary cell (if any).

-	For subframe-PUCCH, format 5 for more than one CSI report, SR (if any) and HARQ-ACK corresponding to PDSCH transmission only on the primary cell (if any).

For slot-PUCCH only PUCCH formats 1/1a/1b, 3, 4 are supported. For subslot-PUCCH only PUCCH formats 1/1a/1b, and 4 are supported.

For a UE configured with PUCCH format 3, not configured with PUCCH format 4/5, and for HARQ-ACK transmission on PUSCH or using PUCCH format 3, or for a UE configured with two serving cells and PUCCH format 1b with channel selection and HARQ-ACK transmission on PUSCH, or for a non BL/CE UE configured with one serving cell and PUCCH format 1b with channel selection according to Tables 10.1.3-5, 10.1.3-6, 10.1.3-7 and HARQ-ACK transmission on PUSCH or for a UE configured with PUCCH format 4/5 and HARQ-ACK transmission on PUSCH or using PUCCH format 3/4/5:

-	for subframe-PDSCH, if the configured downlink transmission mode for a serving cell supports up to 2 transport blocks and only one transport block is received in a subframe, the UE shall generate a NACK for the other transport block if spatial HARQ-ACK bundling is not applied.

-	if neither PDSCH nor PDCCH/EPDCCH/SPDCCH indicating downlink SPS release is detected in a subframe/slot/subslot for a serving cell, the UE shall generate two NACKs when the configured downlink transmission mode supports up to 2 transport blocks and the UE shall generate a single NACK when the configured downlink transmission mode supports a single transport block.

For a UE configured with PUCCH format 4/5 and with a transmission mode supporting two transport blocks in at least one serving cell, the HARQ-ACK response for subframe-PDSCH, and for a serving cell configured with a transmission mode supporting one transport block is associated with the first codeword. The UE shall generate a NACK for the second codeword if spatial bundling is not applied, and shall generate the same HARQ-ACK response for the second codeword as that for the first codeword if spatial bundling is applied.

For a BL/CE UE configured with PUCCH format 1b with channel selection according to Tables 10.1.3-5, 10.1.3-6, 10.1.3-7, if neither PDSCH nor MPDCCH indicating downlink SPS release is detected in a subframe for a serving cell, the UE shall generate a single NACK.

The scrambling initialization of PUCCH format 2, 2a, 2b, 3, 4 and 5 is by C-RNTI.

For a non-BL/CE UE that is configured with a single serving cell and is not configured with PUCCH format 3, in case of collision between a periodic CSI report and an HARQ-ACK in a same subframe without PUSCH, the periodic CSI report is multiplexed with HARQ-ACK on PUCCH if the parameter simultaneousAckNackAndCQI provided by higher layers is set TRUE, otherwise the CSI is dropped.

A UE that is configured with PUCCH format 4/5 is not expected to be configured with different values for simultaneousAckNackAndCQI-Format3 and simultaneousAckNackAndCQI-Format4-Format5.

For a BL/CE UE,

-	if both pucch-NumRepetitionCE-format1 and pucch-NumRepetitionCE-format2 equal 1, in case of collision among two or more of a periodic CSI report, an HARQ-ACK and a SR in a same subframe without PUSCH, the UE behavior follows that of a non-BL/CE UE.

-	if at least one of pucch-NumRepetitionCE-format1 and pucch-NumRepetitionCE-format2 is larger than 1, in case of collision among two or more of a periodic CSI report, an HARQ-ACK, and a SR in a same subframe without PUSCH, the highest priority UCI is transmitted, where the priority of HARQ-ACK is higher than SR and the priority of SR is higher than periodic CSI report.

For TDD and for a UE that is configured with a single serving cell and with PUCCH format 3, in case of collision between a periodic CSI report and an HARQ-ACK in a same subframe without PUSCH, if the parameter simultaneousAckNackAndCQI provided by higher layers is set TRUE or if the parameter simultaneousAckNackAndCQI-Format3-r11 provided by higher layers is set TRUE, the periodic CSI report is multiplexed with HARQ-ACK or dropped as described in Clause 7.3, otherwise the CSI is dropped.

For FDD or for FDD-TDD and primary cell frame structure type 1 and for a UE that is configured with more than one serving cell and is not configured with PUCCH format 4/5, in case of collision between a periodic CSI report and an HARQ-ACK in a same subframe without PUSCH,

-	if the parameter simultaneousAckNackAndCQI provided by higher layers is set TRUE and if the HARQ-ACK corresponds to a PDSCH transmission or PDCCH/EPDCCH indicating downlink SPS release only on the primary cell,

then the periodic CSI report is multiplexed with HARQ-ACK on PUCCH using PUCCH format 2/2a/2b

-	else if the UE is configured with PUCCH format 3 and if the parameter simultaneousAckNackAndCQI-Format3-r11 provided by higher layers is set TRUE, and if PUCCH resource is determined according to Clause 10.1.2.2.2, and

-	if the total number of bits in the subframe corresponding to HARQ-ACKs, SR (if any), and the CSI is not larger than 22 or

-	if the total number of bits in the subframe corresponding to spatially bundled HARQ-ACKs, SR (if any), and the CSI is not larger than 22

then the periodic CSI report is multiplexed with HARQ-ACK on PUCCH using the determined PUCCH format 3 resource according to [4]

-	otherwise,

CSI is dropped.

For FDD or for FDD-TDD and primary cell frame structure type 1, for a UE configured with PUCCH format 4 or PUCCH format 5, and if the UE has HARQ-ACK/SR and periodic CSI reports to transmit in a subframe,

-	if a PUCCH format 3 is determined to transmit the HARQ-ACK/SR according to Clause 10.1.2.2.3 or 10.1.2.2.4, the UE shall use the determined PUCCH format 3 for transmission of the HARQ-ACK/SR and periodic CSI report(s) if the parameter simultaneousAckNackAndCQI-Format3-r11 provided by higher layers is set TRUE; otherwise, the UE shall drop the periodic CSI report(s) and transmit only HARQ-ACK/SR;

-	if a PUCCH format 4 is determined to transmit the HARQ-ACK/SR according to Clause 10.1.2.2.3 or a PUCCH format 5 is determined to transmit the HARQ-ACK/SR according to 10.1.2.2.4, the UE shall used the determined PUCCH format 4 or PUCCH format 5 for transmission of the HARQ-ACK/SR and periodic CSI report(s) if the parameter simultaneousAckNackAndCQI-Format4-Format5-r13 provided by higher layers is set TRUE; otherwise, the UE shall drop the periodic CSI report(s) and transmit only HARQ-ACK/SR;

-	if there is no PUCCH format 3 or 4 determined to transmit the HARQ-ACK/SR according to Clause 10.1.2.2.3 and there is no PUCCH format 3 or 5 determined to transmit the HARQ-ACK/SR according to Clause 10.1.2.2.4 and there are more than one periodic CSI report(s) in the subframe,

-	if the parameter simultaneousAckNackAndCQI-Format4-Format5-r13 provided by higher layers is set TRUE and if the UE is configured with a single PUCCH format 4 resource  according to higher layer parameter format4-MultiCSI-resourceConfiguration, the PUCCH format 4 resource  is used for transmission of the HARQ-ACK/SR and periodic CSI report(s);

-	if the parameter simultaneousAckNackAndCQI-Format4-Format5-r13 provided by higher layers is set TRUE and if the UE is configured with a PUCCH format 5 resource according to higher layer parameter format5-MultiCSI-resourceConfiguration, the PUCCH format 5 resource is used for transmission of the HARQ-ACK/SR and periodic CSI report(s);

-	if the parameter simultaneousAckNackAndCQI-Format4-Format5-r13 provided by higher layers is set TRUE and if the UE is configured with two PUCCH format 4 resources  and according to higher layer parameter format4-MultiCSI-resourceConfiguration, if , the PUCCH format 4 resource with the smaller  between  and  is used for transmission of the HARQ-ACK/SR and periodic CSI report(s); otherwise, the PUCCH format 4 resource with the larger  between  and  is used for transmission of the HARQ-ACK/SR and periodic CSI report(s), where

-	 is the total number of HARQ-ACK bits in the subframe;

-	if there no scheduling request bit in the subframe and  otherwise

-	 is the total number of CSI report bits in the subframe;

-	 is the number of CRC bits;

-	, , is the number of PRBs for  and  respectively, according to higher layer parameter numberOfPRB-format4-r13 according to Table 10.1.1-2;

-	 if shortened PUCCH format 4 is used in the subframe and  otherwise; and

-	 is the code rate given by higher layer parameter maximumPayloadCoderate-r13 according to Table 10.1.1-1;

-	otherwise, the UE shall drop the periodic CSI report(s) and transmit only HARQ-ACK/SR;

-	if there is no PUCCH format 3 or 4 determined to transmit the HARQ-ACK/SR according to Clause 10.1.2.2.3 and there is no PUCCH format 3 or 5 determined to transmit the HARQ-ACK/SR according to Clause 10.1.2.2.4 and there are only one periodic CSI report in the subframe,

-	if there is no positive SR and the parameter simultaneousAckNackAndCQI provided by higher layers is set TRUE and if the HARQ-ACK corresponds to a PDSCH transmission or PDCCH/EPDCCH indicating downlink SPS release only on the primary cell, then the periodic CSI report is multiplexed with HARQ-ACK on PUCCH using PUCCH format 2/2a/2b

-	else, the UE shall drop the CSI and transmit the HARQ-ACK according to Clause 10.1.2.2.3 or 10.1.2.2.4 when UE shall transmit HARQ-ACK only or UE shall drop the CSI and transmit the HARQ-ACK and SR according to the procedure for FDD with PUCCH format 1a/1b when there is positive SR.

-	If a UE transmits HARQ-ACK/SR and periodic CSI report(s) using either a PUCCH format 4  or PUCCH format 5 in a subframe

-	if , the UE shall transmit the HARQ-ACK/SR and periodic CSI bits using the PUCCH format 4  or the PUCCH format 5  ;

-	if , the UE shall select  CSI report(s) for transmission together with HARQ-ACK/SR in ascending order of , where ,  and are determined according to Clause 7.2.2; the value of  satisfies  and , and  is the number of CSI report bits for the nth CSI report in ascending order of .

For a UE configured with higher layer parameter shortTTI, if the UE has subslot-SR and subframe-PUSCH/PUCCH including more than 2 HARQ-ACK bits either on the same serving cell or on different serving cells in a subframe when the UE is not capable of simultaneous transmission of different uplink signal durations on different serving cells as indicated by the UE capability simultaneousTx-differentTx-duration, and if the UE does not have HARQ-ACK bits corresponding to any subslot-PDSCH in the subslot in which the subslot-SR to be transmitted:

-	if the UE is configured with a single subslot-PUCCH format 4 resource according to the higher layer parameter format4-MultiResource-subslotConfiguration, the UE shall multiplex HARQ-ACK and SR bits on .

-	if the UE is configured with two subslot-PUCCH format 4 resources and according to the higher layer parameter format4-MultiResource-subslotConfiguration, if  ,  the subslot-PUCCH format 4 resource with the smaller  between  and  is used for transmission of the HARQ-ACK/SR; otherwise, the subslot-PUCCH format 4 resource with the larger  between  and  is used for transmission of the HARQ-ACK/SR, where

-	 is the total number of HARQ-ACK bits on the subframe-PUSCH/PUCCH;

-

-	 is the number of CRC bits;

-	, , is the number of PRBs for  and  respectively, according to higher layer parameter n4numberOfPRBSubslot according to Table 10.1.1-2;

-	 is the code rate given by higher layer parameter n4maxCoderateSubsloPUCCHt-r15 according to Table 10.1.1-1;

For a UE configured with higher layer parameter shortTTI, if the UE has slot-SR and subframe-PUSCH/PUCCH including more than 2 HARQ-ACK bits either on the same serving cell or on different serving cells when the UE is not capable of simultaneous transmission of different uplink signal durations on different serving cells as indicated by the UE capability simultaneousTx-differentTx-duration, and if the UE does not have HARQ-ACK bits corresponding to any subslot/slot-PDSCH in the slot in which the slot-SR to be transmitted:

-	if the UE is either not configured with a slot-PUCCH format 4 according to the higher layer parameter format4-MultiResource-slotConfiguration, or the number of HARQ-ACK and SR bits is smaller than or equal to 11 bits, the UE shall multiplex HARQ-ACK and SR bits on slot-PUCCH format 3 resource .

-	if the UE is configured with a single PUCCH format 4 resource according to the higher layer parameter format4-MultiResource-slotConfiguration, and if either the UE is not configured with a slot-PUCCH resource  or the number of HARQ-ACK and SR bits is larger than 11 bits, the UE shall multiplex HARQ-ACK and SR bits on .

-	if the UE is not configured with a slot-PUCCH resource  or if the number of HARQ-ACK and SR bits is larger than 11 bits, and if the UE is configured with two slot-PUCCH format 4 resources  and  according to the higher-layer parameter format4-MultiResource-slotConfiguration, if  ,  the slot-PUCCH format 4 resource with the smaller  between  and  is used for transmission of the HARQ-ACK/SR; otherwise, the slot-PUCCH format 4 resource with the larger  between  and  is used for transmission of the HARQ-ACK/SR, where

-	 is the total number of HARQ-ACK bits on the subframe-PUSCH/PUCCH;

-

-	 is the number of CRC bits;

-	, , is the number of PRBs for  and  respectively, according to higher layer parameter n4numberOfPRB according to Table 10.1.1-2;

-	 if shortened slot-PUCCH format 4 is used and  otherwise; and

-	 is the code rate given by higher layer parameter n4maxCoderateSlotPUCCH-r15 according to Table 10.1.1-1;

For TDD or for FDD-TDD and primary cell frame structure type 2 and for a UE that is configured with more than one serving cell, in case of collision between a periodic CSI report and an HARQ-ACK in a same subframe without PUSCH, if the parameter simultaneousAckNackAndCQI provided by higher layers is set TRUE or if the parameter simultaneousAckNackAndCQI-Format3-r11 provided by higher layers is set TRUE or if the parameter simultaneousAckNackAndCQI-Format4-Format5-r13 provided by higher layers is set TRUE, the periodic CSI report is multiplexed with HARQ-ACK or dropped as described in Clause 7.3, otherwise the CSI is dropped.

In case of collision between a periodic CSI report and a HARQ-ACK in a same subframe with PUSCH, the periodic CSI is multiplexed with the HARQ-ACK in the PUSCH transmission in that subframe if the UE is not configured by higher layers for simultaneous PUCCH and PUSCH transmissions or if the UE is provided by higher layers a parameter simultaneousAckNackAndCQI-Format4-Format5-r13 that is set FALSE. If the UE is configured by higher layers for simultaneous PUCCH and PUSCH transmissions, and if the UE does not determine PUCCH format 4/5 for periodic CSI and HARQ-ACK transmission or if the UE is provided by higher layers a parameter simultaneousAckNackAndCQI-Format4-Format5-r13 that is set FALSE, the HARQ-ACK is transmitted in the PUCCH and the periodic CSI is transmitted in the PUSCH (other than a LAA SCell). If the UE is configured by higher layers for simultaneous PUCCH and PUSCH transmissions and if the UE determines PUCCH format 4/5 for periodic CSI and HARQ-ACK transmission and if the UE is provided by higher layers a parameter simultaneousAckNackAndCQI-Format4-Format5-r13 that is set TRUE, the periodic CSI and HARQ-ACK is transmitted in PUCCH format 4/5.

In case of collision between a periodic CSI report and a HARQ-ACK in a same subframe with PUSCH and if an aperiodic CSI report is not triggered for the same subframe, and if a UE is transmitting PUSCH only on LAA SCell(s), the HARQ-ACK and periodic CSI transmission follows the procedure for the case of collision between a periodic CSI report and a HARQ-ACK in the same subframe without PUSCH.

For a BL/CE UE, in case of collision between a UCI and a PUSCH transmission in a same subframe, if the number of PUCCH repetitions defined for the UCI in [3] is larger than 1 or if the indicated PUSCH repetition number in DCI format 6-0A/6-0B is larger than 1 or if the PUSCH resource assignment is using uplink resource allocation type 5, the PUSCH transmission is dropped in that subframe.

For a BL/CE UE in half-duplex FDD operation, in case of collision between a PUCCH format 2 transmission including half-duplex guard subframe and a PDSCH reception with repetitions, the PUCCH format 2 transmission is dropped if:

-	the PUCCH is transmitted starting in subframe n, and the MPDCCH scheduling the PDSCH ends in subframe k, with n-k-Koffset ≥4, or

-	the PDSCH is semi-statically scheduled.

For a BL/CE UE, in case of collision between at least one physical resource block to be used for transmission of UCI on PUCCH (defined in [3]) and physical resource blocks corresponding to configured PRACH resources for BL/CE UEs or non-BL/CE UEs (defined in [3]) in a same subframe, the PUCCH is dropped in that subframe.

If a UE is configured with higher layer parameter shortTTI, the UE is not expected to transmit slot-PUCCH in UpPTS of the special subframe in frame structure type 2.

If each of the serving cells configured for the UE has frame structure type 1, UE procedures for HARQ-ACK feedback are given in Clause 10.1.2.

If each of the serving cells configured for the UE has frame structure type 2, UE procedures for HARQ-ACK feedback are given in Clause 10.1.3.

If the UE is configured for more than one serving cell, and if the frame structure type of any two configured serving cells is different, and if the primary cell is frame structure type 1, UE procedure for HARQ-ACK feedback is given in Clause 10.1.2A.

If the UE is configured for more than one serving cell, and if the frame structure type of any two configured serving cells is different, and if the primary cell is frame structure type 2, UE procedure for HARQ-ACK feedback is given in Clause 10.1.3A.

Table 10.1.1-1: code rate  corresponding to higher layer parameter maximumPayloadCoderate-r13 or n4maxCoderateSlotPUCCH-r15 or n4maxCoderateSubslotPUCCH-r15

| Value of maximumPayloadCoderate-r13 or n4maxCoderateSlotPUCCH-r15 or n4maxCoderateSubslotPUCCH-r15 | Code rate |
| --- | --- |
| 0 | 0.08 |
| 1 | 0.15 |
| 2 | 0.25 |
| 3 | 0.35 |
| 4 | 0.45 |
| 5 | 0.60 |
| 6 | 0.80 |
| 7 | Reserved |

Table 10.1.1-2: Number of PRBs for PUCCCH format 4  corresponding to higher layer parameter numberOfPRB-format4-r13 or n4numberOfPRBSubslot or n4numberOfPRB

| Value of numberOfPRB-format4-r13 or n4numberOfPRBSubslot or n4numberOfPRB |  |
| --- | --- |
| 0 | 1 |
| 1 | 2 |
| 2 | 3 |
| 3 | 4 |
| 4 | 5 |
| 5 | 6 |
| 6 | 8 |
| 7 | Reserved |

If a UE is configured with more than 5 serving cells and is configured with PUCCH format 3 and not configured with PUCCH format 4/5:

-	The UE can assume that the total number of bits in a given

-	subframe corresponding to HARQ-ACK (if any), SR (if any), and periodic CSI (if any) is not larger than 22.

-	slot corresponding to HARQ-ACK (if any), SR (if any) is not larger than 11.

-	For calculating the HARQ-ACK bits to be transmitted, the UE shall follow the procedure in clauses 10.1.2.2.3, 10.1.2A, 10.1.3.2.3, 10.1.3A by assuming that PUCCH format 4 is configured.

### 10.1.2	FDD HARQ-ACK feedback procedures

For FDD and for a UE not configured with PUCCH format 4/5 and transmitting HARQ-ACK using PUCCH format 1b with channel selection or PUCCH format 3, the UE shall determine the number of HARQ-ACK bits, , based on the number of configured serving cells and the downlink transmission modes configured for each serving cell. The UE shall use two HARQ-ACK bits for a serving cell configured with a downlink transmission mode that support up to two transport blocks; and one HARQ-ACK bit otherwise.

If a UE is configured with higher layer parameters shortTTI, codebooksizeDetermination= cc, and codebooksizeDeterminationsSTTI= cc for a PUCCH group and transmitting slot/subslot-PUCCH, the UE shall determine the number of HARQ-ACK bits, , based on the number of configured serving cells for subframe-PDSCH, number of configured serving cells for slot/subslot-PDSCH and the downlink transmission modes configured for each serving cell.

A UE that supports aggregating at most 2 serving cells with frame structure type 1 shall use PUCCH format 1b with channel selection for transmission of HARQ-ACK corresponding to subframe-PDSCH when configured with more than one serving cell with frame structure type 1.

A UE that supports aggregating more than 2 serving cells with frame structure type 1 is configured by higher layers to use either PUCCH format 1b with channel selection or PUCCH format 3/4/5 for transmission of HARQ-ACK corresponding to subframe-PDSCH when configured with more than one serving cell with frame structure type 1.

The FDD HARQ-ACK feedback procedure for one configured serving cell is given in Clause 10.1.2.1 and procedures for more than one configured serving cell are given in Clause 10.1.2.2.

#### 10.1.2.1	FDD HARQ-ACK procedure for one configured serving cell

HARQ-ACK transmission on two antenna ports  is supported for PUCCH format 1a/1b.

For FDD and one configured serving cell, the UE shall use PUCCH resource  for transmission of HARQ-ACK in subframe  for  mapped to antenna port p for PUCCH format 1a/1b [3], where

- for a PDSCH transmission indicated by the detection of a corresponding PDCCH in subframe , or for a PDCCH indicating downlink SPS release (defined in Clause 9.2) in subframe  , the UE shall use  for antenna port , where  is the number of the first CCE (i.e. lowest CCE index used to construct the PDCCH) used for transmission of the corresponding DCI assignment and  is configured by higher layers. For two antenna port transmission the PUCCH resource for antenna port  is given by .

- for a non-BL/CE UE, and for a PDSCH transmission on the primary cell where there is not a corresponding PDCCH/EPDCCH detected in subframe , the value of  is determined according to higher layer configuration and Table 9.2-2. For a UE configured for two antenna port transmission, a PUCCH resource value in Table 9.2-2 maps to two PUCCH resources with the first PUCCH resource  for antenna port  and the second PUCCH resource  for antenna port , otherwise, the PUCCH resource value maps to a single PUCCH resource  for antenna port .

- for a PDSCH transmission indicated by the detection of a corresponding EPDCCH in subframe  , or for an EPDCCH indicating downlink SPS release (defined in Clause 9.2) in subframe  , the UE shall use

-	if EPDCCH-PRB-set is configured for distributed transmission

-	if EPDCCH-PRB-set is configured for localized transmission

for antenna port , where  is the number of the first ECCE (i.e. lowest ECCE index used to construct the EPDCCH) used for transmission of the corresponding DCI assignment in EPDCCH-PRB-set ,  is determined from the HARQ-ACK resource offset field in the DCI format of the corresponding EPDCCH as given in Table 10.1.2.1-1,  for EPDCCH-PRB-set is configured by the higher layer parameter pucch-ResourceStartOffset-r11 , for EPDCCH-PRB-set is given in Clause 6.8A.1 in [3], is determined from the antenna port used for localized EPDCCH transmission which is described in Clause 6.8A.5 in [3]. For two antenna port transmission the PUCCH resource for antenna port  is given by

-	if EPDCCH-PRB-set is configured for distributed transmission

-	if EPDCCH-PRB-set is configured for localized transmission

- for a BL/CE UE, and for a PDSCH on the primary cell where there is not a corresponding MPDCCH detected and subframe  n- k-Koffset is the last subframe in which the PDSCH is transmitted, the value of  is determined according to higher layer configuration and Table 9.2-2.

- for a PDSCH transmission indicated by the detection of a corresponding MPDCCH, or for an MPDCCH indicating downlink SPS release (defined in Clause 9.2) where subframe n- k-Koffset is the last subframe in which the PDSCH is transmitted, or for HD-FDD HARQ-ACK bundling, subframe n- k- Koffset  is the last subframe in which the PDSCH is detected, the UE shall use

-	if MPDCCH-PRB-set is configured for distributed transmission

-	if MPDCCH-PRB-set is configured for localized transmission

for antenna port , where  is the number of the first ECCE (i.e. lowest ECCE index used to construct the MPDCCH) used for transmission of the corresponding DCI assignment in MPDCCH-PRB-set , or for HD-FDD HARQ-ACK bundling  is the number of the first ECCE (i.e. lowest ECCE index used to construct the MPDCCH) in the last detected MPDCCH used for transmission of the corresponding DCI assignment in MPDCCH-PRB-set ,  is determined from the HARQ-ACK resource offset field in the DCI format of the corresponding MPDCCH as given in Table 10.1.2.1-1,  for MPDCCH-PRB-set  is configured

-	by the higher layer parameter n1PUCCH-AN-r13, if configured; otherwise:

-	by the higher layer parameter n1PUCCH-AN-InfoList-r13 for the corresponding CE level,

for MPDCCH-PRB-set  is given in Clause 6.8A.1 in [3] where the same  value is used for each subframe containing a repeat of a MPDCCH transmission,  is determined from the antenna port used for localized MPDCCH transmission which is described in Clause 6.8A.5 in [3]. When an MPDCCH-PRB-set is 2+4, following procedures is applied.

-	if the last detected MPDCCH is located within 2 PRB set, is obtained by above procedure.

-	if the last detected MPDCCH is located within 4 PRB set, is the sum between and the value obtained by above procedure.

-	if the last detected MPDCCH is MPDCCH format 5, is obtained by the above procedure with  = 0.

Table 10.1.2.1-1: Mapping of ACK/NACK Resource offset Field 
in DCI format 1A/1B/1D/1/2A/2/2B/2C/2D/6-1A/6-1B to values

| ACK/NACK Resource offset field in DCI format 1A/1B/1D/1/2A/2/2B/2C/2D |  |
| --- | --- |
| 0 | 0 |
| 1 | -1 |
| 2 | -2 |
| 3 | 2 |

For slot-PUCCH, and for transmission of up to 2 HARQ-ACK bits in slot , when at least one HARQ-ACK bit is sent in response to a PDSCH transmission indicated by the detection of a corresponding PDCCH/SPDCCH with DCI format 7-1A/7-1B/7-1C/7-1D/7-1E/7-1F/7-1G or a semi-persistently scheduled slot-PDSCH transmission or for a PDCCH/SPDCCH indicating downlink SPS release in

-	slot  or

-	when scheduling request is sent in slot on a resource configured by higher layer parameter sr-slotSPUCCH-IndexFH or sr-slotSPUCCH-IndexNoFH,

the UE shall use PUCCH resource  for transmission of HARQ-ACK in slot  for  mapped to antenna port p for slot-PUCCH transmission using format 1a/1b [3], where the value of  is determined according to higher layer configuration and Table 10.1.2.1-2. For a UE configured for two antenna port transmission for PUCCH format 1a/1b, a PUCCH resource value in Table 10.1.2.1-2 maps to two PUCCH resources with the first PUCCH resource  for antenna port  and the second PUCCH resource  for antenna port , otherwise, the PUCCH resource value maps to a single PUCCH resource  for antenna port .

Table 10.1.2.1-2: PUCCH Resource Value for HARQ-ACK Resource for slot-PUCCH

| Value of 'SPUCCH resource indication' field as defined in clause 5.3.3.1.17 of [4] | or |
| --- | --- |
| '00' | The 1st PUCCH resource value configured by the higher layers |
| '01' | The 2nd PUCCH resource value configured by the higher layers |
| '10' | The 3rd PUCCH resource value configured by the higher layers |
| '11' | The 4th PUCCH resource value configured by the higher layers |
| NOTE: are respectively determined from the first and second PUCCH resource lists configured by n1SlotSPUCCH-hoppingAN-List in [11] for slot-PUCCH when hopping is enabled, and by n1SlotSPUCCH-nohoppingAN-List in [11] for slot-PUCCH when hopping is disabled. |  |

For subslot-PUCCH, and for transmission of up to 2 HARQ-ACK bits, in subslot , when at least one HARQ-ACK bit is sent in response to a PDSCH transmission indicated by the detection of a corresponding PDCCH/SPDCCH with DCI format 7-1A/7-1B/7-1C/7-1D/7-1E/7-1F/7-1G in subslot  or a semi-persistently scheduled subslot-PDSCH transmission in subslot  or for a PDCCH/SPDCCH indicating downlink SPS release in subslot  or when scheduling request is sent in subslot on a resource configured by higher layer parameter sr-subslotSPUCCH-Resource, the UE shall use PUCCH resource  for transmission of HARQ-ACK in subslot  for  mapped to antenna port p for subslot-PUCCH transmission using format 1a/1b [3], where

-	for a PDSCH transmission indicated by the detection of a corresponding PDCCH/SPDCCH with DCI format 7-1A/7-1B/7-1C/7-1D/7-1E/7-1F/7-1G in subslot or a semi-persistently scheduled subslot-PDSCH transmission in subslot or for a PDCCH/SPDCCH indicating downlink SPS release in subslot , PUCCH resource group is determined by higher layer configuration and Table 10.1.2.1-3, and  is determined according to higher layer configuration and Table 10.1.2.1-4 for PUCCH format 1a, and according to higher layer configuration and Table 10.1.2.1-5 for PUCCH format 1b.

Table 10.1.2.1-3: PUCCH Resource Group Value for HARQ-ACK Resource for PUCCH

| Value of 'SPUCCH' resource indication field' | PUCCH resource group |
| --- | --- |
| '00' | The 1st PUCCH resource group value configured by the higher layers |
| '01' | The 2nd PUCCH resource group value configured by the higher layers |
| '10' | The 3rd PUCCH resource group value configured by the higher layers |
| '11' | The 4th PUCCH resource group value configured by the higher layers |

Table 10.1.2.1-4: Transmission of Format 1a HARQ-ACK for subslot-PUCCH

| HARQ-ACK |  |
| --- | --- |
| NACK | The 1st PUCCH resource value configured by the higher layers for the indicated PUCCH resource group |
| ACK | The 2nd PUCCH resource value configured by the higher layers for the indicated PUCCH resource group |

Table 10.1.2.1-5: Transmission of Format 1b HARQ-ACK for subslot-PUCCH

| HARQ-ACK(0) | HARQ-ACK(1) |  |
| --- | --- | --- |
| NACK | NACK | The 1st PUCCH resource value configured by the higher layers for the indicated PUCCH resource group |
| ACK | NACK | The 2nd PUCCH resource value configured by the higher layers for the indicated PUCCH resource group |
| NACK | ACK | The 3rd PUCCH resource value configured by the higher layers for the indicated PUCCH resource group |
| ACK | ACK | The 4th PUCCH resource value configured by the higher layers for the indicated PUCCH resource group |

<Unchanged parts are omitted>

### 10.1.3	TDD HARQ-ACK feedback procedures

For TDD and a UE that does not support aggregating more than one serving cell with frame structure type 2, two HARQ-ACK feedback modes are supported by higher layer configuration.

- HARQ-ACK bundling and

- HARQ-ACK multiplexing

For TDD and a BL/CE UE, if multiple TBs are not scheduled by a single DCI,

-	if the UE is configured with csi-NumRepetitionCE equal to 1 and mPDCCH-NumRepetition equal to 1,

-	the UE may be configured with HARQ-ACK bundling or HARQ-ACK multiplexing;

-	HARQ-ACK multiplexing can be configured only if pucch-NumRepetitionCE-format1 equal 1 and HARQ-ACK multiplexing is performed according to the set of Tables 10.1.3-5/6/7

-	else

-	the UE is not expected to receive more than one PDSCH transmission, or more than one of PDSCH and MPDCCH indicating downlink SPS releases, with transmission ending within subframe(s) 
 n- k-Koffset, where  and  is defined in Table 10.1.3.1-1 intended for the UE;

For TDD UL/DL configuration 5 and a UE that does not support aggregating more than one serving cell with frame structure type 2 and the UE is not configured with EIMTA-MainConfigServCell-r12 for the serving cell, only HARQ-ACK bundling is supported.

A UE that supports aggregating more than one serving cell with frame structure type 2 is configured by higher layers to use either PUCCH format 1b with channel selection or PUCCH format 3/4/5 for transmission of HARQ-ACK corresponding to subframe-PDSCH when configured with more than one serving cell with frame structure type 2.

For subframe-PUCCH and a UE that supports aggregating more than one serving cell with frame structure type 2 and is not configured with the parameter EIMTA-MainConfigServCell-r12 for any serving cell is configured by higher layers to use HARQ-ACK bundling, PUCCH format 1b with channel selection according to the set of Tables 10.1.3-2/3/4 or according to the set of Tables 10.1.3-5/6/7, or PUCCH format 3 for transmission of HARQ-ACK when configured with one serving cell with frame structure type 2.

For subframe-PUCCH and a UE that is configured with the parameter EIMTA-MainConfigServCell-r12 and configured with one serving cell is configured by higher layers to use PUCCH format 1b with channel selection according to the set of Tables 10.1.3-5/6/7, or PUCCH format 3 for transmission of HARQ-ACK. A UE that is configured with the parameter EIMTA-MainConfigServCell-r12 for at least one serving cell and configured with more than one serving cell is configured by higher layers to use PUCCH format 1b with channel selection according to the set of Tables 10.1.3-5/6/7, or PUCCH format 3/4/5 for transmission of HARQ-ACK.

PUCCH format 1b with channel selection according to the set of Tables 10.1.3-2/3/4 or according to the set of Tables 10.1.3-5/6/7 is not supported for TDD UL/DL configuration 5.

TDD HARQ-ACK bundling is performed per codeword across  multiple subframes/slots in downlink or special subframes associated with a single UL subframe/slot n, where  is the number of elements in the set  defined in Table 10.1.3.1-1C if the UE is configured with higher layer parameter shortTTI for slot-PDSCH, in Table 10.1.3.1-1B if the UE is configured with higher layer parameter shortProcessingTime and the corresponding PDCCH with CRC scrambled by C-RNTI is in the UE-specific search space for subframe-PDSCH and in Table 10.1.3.1-1 otherwise, by a logical AND operation of all the individual PDSCH transmission (with and without corresponding PDCCH/EPDCCH/MPDCCH/SPDCCH) HARQ-ACKs and ACK in response to PDCCH/EPDCCH/MPDCCH/SPDCCH indicating downlink SPS release. For one configured serving cell the bundled 1 or 2 HARQ-ACK bits are transmitted using PUCCH format 1a or PUCCH format 1b, respectively.

For TDD HARQ-ACK multiplexing and a subframe n with , where  is the number of elements in the set  defined in Table 10.1.3.1-1B if the UE is configured with higher layer parameter shortProcessingTime and the corresponding PDCCH with CRC scrambled by C-RNTI is in the UE-specific search space for subframe-PDSCH and in Table 10.1.3.1-1 otherwise, spatial HARQ-ACK bundling across multiple codewords within a downlink or special subframe is performed by a logical AND operation of all the corresponding individual HARQ-ACKs. PUCCH format 1b with channel selection is used in case of one configured serving cell. For TDD HARQ-ACK multiplexing and a subframe n with , spatial HARQ-ACK bundling across multiple codewords within a downlink or special subframe is not performed, 1 or 2 HARQ-ACK bits are transmitted using PUCCH format 1a or PUCCH format 1b, respectively for one configured serving cell.

In the case of TDD and more than one configured serving cell with PUCCH format 1b with channel selection and more than 4 HARQ-ACK bits for  multiple downlink or special subframes associated with a single UL subframe n, where  is defined in Clause 10.1.3.2.1, and for the configured serving cells, spatial HARQ-ACK bundling across multiple codewords within a downlink or special subframe for all configured cells is performed and the bundled HARQ-ACK bits for each configured serving cell is transmitted using PUCCH format 1b with channel selection. For TDD and more than one configured serving cell with PUCCH format 1b with channel selection and up to 4 HARQ-ACK bits for multiple downlink or special subframes associated with a single UL subframe n, where  is defined in Clause 10.1.3.2.1, and for the configured serving cells, spatial HARQ-ACK bundling is not performed and the HARQ-ACK bits are transmitted using PUCCH format 1b with channel selection.

In the case of TDD and more than one configured serving cell with PUCCH format 3 and without PUCCH format 4/5 configured and more than 20 HARQ-ACK bits for  multiple downlink or special subframes associated with a single UL subframe n, where  is the number of elements in the set  defined in Clause 10.1.3.2.2 and for the configured serving cells, spatial HARQ-ACK bundling across multiple codewords within a downlink or special subframe is performed for each serving cell by a logical AND operation of all of the corresponding individual HARQ-ACKs and PUCCH format 3 is used. For TDD and more than one configured serving cell with PUCCH format 3 and up to 20 HARQ-ACK bits for  multiple downlink or special subframes associated with a single UL subframe n, where  is the number of elements in the set  defined in Clause 10.1.3.2.2 and for the configured serving cells, spatial HARQ-ACK bundling is not performed and the HARQ-ACK bits are transmitted using PUCCH format 3.

For TDD with PUCCH format 3 without PUCCH format 4/5 configured, a UE shall determine the number of HARQ-ACK bits, , associated with an UL subframe/slot n

according to  where  is the number of configured cells, and is the number of HARQ-bits for the c-th serving cell defined in Clause 7.3.

TDD HARQ-ACK feedback procedures for one configured serving cell are given in Clause 10.1.3.1 and procedures for more than one configured serving cell are given in Clause 10.1.3.2.

#### 10.1.3.1	TDD HARQ-ACK procedure for one configured serving cell

HARQ-ACK transmission on two antenna ports  is supported for PUCCH format 1a/1b with TDD HARQ-ACK bundling feedback mode and for PUCCH format 3.

A UE that supports aggregating more than one serving cell with frame structure type 2 can be configured by higher layers for HARQ-ACK transmission on two antenna ports  for PUCCH format 1b with channel selection.

The TDD HARQ-ACK procedure for a UE configured with PUCCH format 3 is as described in Clause 10.1.3.2.2 when the UE receives PDSCH and/or SPS release PDCCH/EPDCCH only on the primary cell.

If the UE is not configured with the higher layer parameter EIMTA-MainConfigServCell-r12, and the UE is not configured with the higher layer parameter shortProcessingTime, for TDD HARQ-ACK bundling or TDD HARQ-ACK multiplexing for one configured serving cell and a subframe n with  where  is the number of elements in the set  defined in Table 10.1.3.1-1, the UE shall use PUCCH resource  for transmission of HARQ-ACK in subframe  for  mapped to antenna port p for PUCCH format 1a/1b, where

-	If there is PDSCH transmission indicated by the detection of corresponding PDCCH/EPDCCH or there is PDCCH/EPDCCH indicating downlink SPS release within subframe(s) , where  and  (defined in Table 10.1.3.1-1) is a set of M elements  depending on the subframe n and the UL/DL configuration (defined in Table 4.2-2 in [3]), and if PDCCH indicating PDSCH transmission or downlink SPS release is detected in subframe , where  is the smallest value in set such that UE detects a PDCCH/EPDCCH indicating PDSCH transmission or downlink SPS release within subframe(s)  and , the UE first selects a  value out of {0, 1, 2, 3} which makes  and shall use  for antenna port , where  is configured by higher layers, , and  is the number of the first CCE used for transmission of the corresponding PDCCH in subframe and the corresponding m. When two antenna port transmission is configured for PUCCH format 1a/1b, the PUCCH resource for HARQ-ACK bundling for antenna port  is given by  .

-	For a non-BL/CE UE and if there is only a PDSCH transmission where there is not a corresponding PDCCH/EPDCCH detected within subframe(s) , where and  is defined in Table 10.1.3.1-1, the UE shall use PUCCH format 1a/1b and PUCCH resource  with the value of  is determined according to higher layer configuration and Table 9.2-2. For a UE configured for two antenna port transmission for PUCCH format 1a/1b and HARQ-ACK bundling, a PUCCH resource value in Table 9.2-2 maps to two PUCCH resources with the first PUCCH resource for antenna port  and the second PUCCH resource for antenna port , otherwise, the PUCCH resource value maps to a single PUCCH resource for antenna port .

-	If there is PDSCH transmission indicated by the detection of corresponding PDCCH/EPDCCH or there is PDCCH/EPDCCH indicating downlink SPS release within subframe(s) , where  and  (defined in Table 10.1.3.1-1) is a set of M elements  depending on the subframe n and the UL/DL configuration (defined in Table 4.2-2 in [3]), and if EPDCCH indicating PDSCH transmission or downlink SPS release is detected in subframe , where  is the smallest value in set such that UE detects a PDCCH/EPDCCH indicating PDSCH transmission or downlink SPS release within subframe(s)  and , the UE shall use

-	if EPDCCH-PRB-set is configured for distributed transmission

-	if EPDCCH-PRB-set is configured for localized transmission

for antenna port , where  is the number of the first ECCE (i.e. lowest ECCE index used to construct the EPDCCH) used for transmission of the corresponding DCI assignment in EPDCCH-PRB-set in subframe  and the corresponding m,  for EPDCCH-PRB-set is configured by the higher layer parameter pucch-ResourceStartOffset-r11 , for EPDCCH-PRB-set in subframe is given in Clause 6.8A.1 in [3], is determined from the antenna port used for EPDCCH transmission in subframe which is described in Clause 6.8A.5 in [3]. If ,  is determined from the HARQ-ACK resource offset field in the DCI format of the corresponding EPDCCH as given in Table 10.1.2.1-1. If ,  is determined from the HARQ-ACK resource offset field in the DCI format of the corresponding EPDCCH as given in Table 10.1.3.1-2. If the UE is configured to monitor EPDCCH in subframe ,  is equal to the number of ECCEs in EPDCCH-PRB-set configured for that UE in subframe. If the UE is not configured to monitor EPDCCH in subframe ,  is equal to the number of ECCEs computed assuming EPDCCH-PRB-set is configured for that UE in subframe . For normal downlink CP, if subframe  is a special subframe with special subframe configuration 0, 5 or 10 if configured by ssp10-CRS-LessDwPTS, is equal to 0. For extended downlink CP, if subframe  is a special subframe with special subframe configuration 0 or 4 or 7, is equal to 0. When two antenna port transmission is configured for PUCCH format 1a/1b, the PUCCH resource for HARQ-ACK bundling for antenna port  is given by

-	if EPDCCH-PRB-set is configured for distributed transmission

-	if EPDCCH-PRB-set is configured for localized transmission

-	For a BL/CE UE, if there is only a PDSCH transmission within one or more consecutive BL/CE downlink subframe(s) where subframe n- k-Koffset, is the last subframe in which the PDSCH is transmitted where and  is defined in Table 10.1.3.1-1 and there is not a corresponding MPDCCH, the UE shall use PUCCH format 1a and PUCCH resource  where the value of is determined according to higher layer configuration and Table 9.2-2.

-	If there is PDSCH transmission indicated by the detection of corresponding MPDCCH or there is MPDCCH indicating downlink SPS release within subframe(s) n- k-Koffset, where  and  (defined in Table 10.1.3.1-1) is a set of M elements  depending on the subframe n and the UL/DL configuration (defined in Table 4.2-2 in [3]) and subframe n- km-Koffset is the last subframe in which the PDSCH or MPDCCH indicating downlink SPS release is transmitted and there is no  where  and subframe n- km'-Koffset is the last subframe in which a PDSCH indicated by the detection of corresponding MPDCCH or MPDCCH indicating downlink SPS release is transmitted, the UE shall use

-	if MPDCCH-PRB-set is configured for distributed transmission

-	if MPDCCH-PRB-set is configured for localized transmission

for antenna port , where  is the number of the first ECCE (i.e. lowest ECCE index used to construct the MPDCCH) used for transmission of the corresponding DCI assignment in MPDCCH-PRB-set ,  for MPDCCH-PRB-set  is configured

-	by the higher layer parameter n1PUCCH-AN-r13, if configured; otherwise:

-	by the higher layer parameter n1PUCCH-AN-InfoList-r13 for the corresponding CE level,

for MPDCCH-PRB-set  is given in Clause 6.8A.1 in [3], is determined from the antenna port used for the MPDCCH transmission which is described in Clause 6.8A.5 in [3]. If ,  is determined from the HARQ-ACK resource offset field in the DCI format of the corresponding MPDCCH as given in Table 10.1.2.1-1. If ,  is determined from the HARQ-ACK resource offset field in the DCI format of the corresponding MPDCCH as given in Table 10.1.3.1-2. If subframe n- ki1-Koffset is a BL/CE downlink subframe,  is equal to the number of ECCEs in MPDCCH-PRB-set configured for that UE in subframe n- ki1-Koffset. If subframe n- ki1-Koffset is not a BL/CE downlink subframe,  is equal to 0. If subframe n- ki1-Koffset is a BL/CE downlink special subframe in which MPDCCH is not supported, the UE shall calculate  by assuming for normal cyclic prefix and for extended cyclic prefix. If an MPDCCH-PRB-set is 2+4, then . When an MPDCCH-PRB-set is 2+4, following procedures is applied.

-	if the detected MPDCCH is located within 2 PRB set, is obtained by above procedure.

-	if the detected MPDCCH is located within 4 PRB set, is the sum between and the value obtained by above procedure.

-	if the detected MPDCCH is MPDCCH format 5, is obtained by the above procedure with  = 0.

Table 10.1.3.1-1: Downlink association set :  for TDD

| UL/DL Configuration | Subframe n |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 |
| 0 | - | - | 6 | - | 4 | - | - | 6 | - | 4 |
| 1 | - | - | 7, 6 | 4 | - | - | - | 7, 6 | 4 | - |
| 2 | - | - | 8, 7, 4, 6 | - | - | - | - | 8, 7, 4, 6 | - | - |
| 3 | - | - | 7, 6, 11 | 6, 5 | 5, 4 | - | - | - | - | - |
| 4 | - | - | 12, 8, 7, 11 | 6, 5, 4, 7 | - | - | - | - | - | - |
| 5 | - | - | 13, 12, 9, 8, 7, 5, 4, 11, 6 | - | - | - | - | - | - | - |
| 6 | - | - | 7 | 7 | 5 | - | - | 7 | 7 | - |

Table 10.1.3.1-1A: eIMTA downlink association set : for TDD

| Higher layer parameter 'eimta-HARQ-ReferenceConfig-r12' | Higher layer parameter 'subframeAssignment' | Subframe n |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 |
| 2 | 0 | - | - | 7,8,4 | - | - | - | - | 7,8,4 | - | - |
|  | 1 | - | - | 8,4 | - | - | - | - | 8,4 | - | - |
|  | 6 | - | - | 6,8,4 | - | - | - | - | 8,6,4 | - | - |
| 4 | 0 | - | - | 12,7,11,8 | 7,4,5,6 | - | - | - | - | - | - |
|  | 1 | - | - | 12,8,11 | 7,5,6 | - | - | - | - | - | - |
|  | 3 | - | - | 12,8 | 4,7 | - | - | - | - | - | - |
|  | 6 | - | - | 12,11,8 | 4,5,6 | - | - | - | - | - | - |
| 5 | 0 | - | - | 12,7,11,13,8,4,9,5 | - | - | - | - | - | - | - |
|  | 1 | - | - | 13,12,8,11,4,9,5 | - | - | - | - | - | - | - |
|  | 2 | - | - | 13,12,9,11,5 | - | - | - | - | - | - | - |
|  | 3 | - | - | 13,12,5,4,8,9 | - | - | - | - | - | - | - |
|  | 4 | - | - | 13,5,4,6,9 | - | - | - | - | - | - | - |
|  | 6 | - | - | 13,12,11,6,8,4,9,5 | - | - | - | - | - | - | - |

Table 10.1.3.1-1B: Downlink association set : for TDD and UE configured with higher layer parameter shortProcessingTime

| UL/DL Configuration | Subframe n |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 |
| 0 | - | - | - | 3 | 3 | - | - | - | 3 | 3 |
| 1 | - | - | 3 | 3 | - | - | - | 3 | 3 | - |
| 2 | - | - | 3 | - | - | - | - | 3 | - | - |
| 3 | - | - | 5 | 4 | 3 | - | - | - | - | - |
| 4 | - | - | 6 | 3 | - | - | - | - | - | - |
| 5 | - | - | 3 | - | - | - | - | - | - | - |
| 6 | - | - | 6 | 4 | 4 | - | - | 6 | 3 | - |

Table 10.1.3.1-1C: Downlink association set :  for TDD with special subframe configuration 1, 2, 6, 7 and UE configured with higher layer parameter shortTTI

| UL/DL Configuration | slot n |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 |
| 0 |  |  |  |  | 4 | 4 | 4 |  |  |  |  |  |  |  | 4 | 4 | 4 |  |  |  |
| 1 |  |  |  |  | 6,5 | 5,4 | 4 |  |  |  |  |  |  |  | 6,5 | 5,4 | 4 |  |  |  |
| 2 |  |  |  |  | 8,7, 12 | 7,6, 5,4 |  |  |  |  |  |  |  |  | 8,7,12 | 7,6,5,4 |  |  |  |  |
| 3 |  |  |  |  | 14, 13, 12 | 12,11, 10 | 10,9 | 9,8 | 8,7 | 7 |  |  |  |  |  |  |  |  |  |  |
| 4 |  |  |  |  | 16, 15, 14, 13 | 13,12, 11,10 | 10,9, 8,7 | 7,6,5 |  |  |  |  |  |  |  |  |  |  |  |  |
| 5 |  |  |  |  | 18, 17, 16, 15, 14, 13, 12,22 | 12,11, 10,9, 8,7,6,5,4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 6 |  |  |  |  | 6 | 6 | 6 | 6 | 6 |  |  |  |  |  | 4 | 4 | 4 |  |  |  |

Table 10.1.3.1-1D: Downlink association set :  for TDD with special subframe configuration 3, 4 or 8 and UE configured with higher layer parameter shortTTI

| UL/DL Configuration | slot n |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 |
| 0 |  |  |  |  | 4 | 4 | 4 | 4 |  |  |  |  |  |  | 4 | 4 | 4 | 4 |  |  |
| 1 |  |  |  |  | 6,5 | 5,4 | 4 | 4 |  |  |  |  |  |  | 6,5 | 5,4 | 4 | 4 |  |  |
| 2 |  |  |  |  | 8,7, 12,11 | 7,6, 5,4 |  |  |  |  |  |  |  |  | 8,7,12,11 | 7,6,5,4 |  |  |  |  |
| 3 |  |  |  |  | 14, 13, 12 | 12,11, 10 | 10,9 | 9,8 | 8,7 | 7,6 |  |  |  |  |  |  |  |  |  |  |
| 4 |  |  |  |  | 16, 15, 14, 13 | 13,12, 11,10 | 10,9, 8,7 | 7,6,5,4 |  |  |  |  |  |  |  |  |  |  |  |  |
| 5 |  |  |  |  | 18, 17, 16, 15, 14, 13, 12,22,21 | 12,11, 10,9, 8,7,6,5,4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 6 |  |  |  |  | 6 | 6 | 6 | 6 | 6 | 6 |  |  |  |  | 4 | 4 | 4 | 4 |  |  |

Table 10.1.3.1-1E: Downlink association set :  for TDD with special subframe configuration 0, 5, 9, 10 and UE configured with higher layer parameter shortTTI

| UL/DL Configuration | slot n |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 |
| 0 |  |  |  |  | 4 | 4 | 4 |  |  |  |  |  |  |  | 4 | 4 | 4 |  |  |  |
| 1 |  |  |  |  | 6,5 | 5 | 5 | 5 |  |  |  |  |  |  | 6,5 | 5 | 5 | 5 |  |  |
| 2 |  |  |  |  | 8,7, 12 | 7,6, 5,4 |  |  |  |  |  |  |  |  | 8,7,12 | 7,6,5,4 |  |  |  |  |
| 3 |  |  |  |  | 14, 13, 12 | 12,11, 10 | 10,9 | 9,8 | 8,7 | 7 |  |  |  |  |  |  |  |  |  |  |
| 4 |  |  |  |  | 16, 15, 14, 13 | 13,12, 11,10 | 10,9, 8,7 | 7,6,5 |  |  |  |  |  |  |  |  |  |  |  |  |
| 5 |  |  |  |  | 18, 17, 16, 15, 14, 13, 12,22 | 12,11, 10,9, 8,7,6,5,4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 6 |  |  |  |  | 6,5 |  | 6,5 |  | 6 |  |  |  |  |  | 4 | 4 | 4 |  |  |  |

Table 10.1.3.1-2: Mapping of ACK/NACK Resource offset Field in DCI format 1A/1B/1D/1/2A/2/2B/2C/2D/6-1A/6-1B to values for TDD when

| ACK/NACK Resource offset field in DCI format 1A/1B/1D/1/2A/2/2B/2C/2D |  |
| --- | --- |
| 0 | 0 |
| 1 |  |
| 2 |  |
| 3 | 2 |

Table 10.1.3.1-3: Mapping of ACK/NACK Resource offset Field in DCI format 1A/1B/1D/1/2A/2/2B/2C/2D to values for TDD when and

| ACK/NACK Resource offset field in DCI format 1A/1B/1D/1/2A/2/2B/2C/2D |  |
| --- | --- |
| 0 | 0 |
| 1 |  |
| 2 |  |
| 3 | 2 |

If the UE is not configured with the higher layer parameter EIMTA-MainConfigServCell-r12, and the UE is not configured with the higher layer parameter shortProcessingTime, for TDD HARQ-ACK multiplexing and sub-frame  with  and one configured serving cell, where  is the number of elements in the set  defined in Table 10.1.3.1-1, denote  as the PUCCH resource derived from sub-frame  and HARQ-ACK(i) as the ACK/NACK/DTX response from sub-frame , where (defined in Table 10.1.3.1-1) and .

-	For a PDSCH transmission indicated by the detection of corresponding PDCCH or a PDCCH indicating downlink SPS release in sub-frame  where , the PUCCH resource , where  is selected from {0, 1, 2, 3} such that , ,  is the number of the first CCE used for transmission of the corresponding PDCCH in subframe , and  is configured by higher layers.

-	For a PDSCH transmission where there is not a corresponding PDCCH/EPDCCH detected in subframe , the value of  is determined according to higher layer configuration and Table 9.2-2.

-	For a non-BL/CE UE and for a PDSCH transmission indicated by the detection of corresponding EPDCCH or a EPDCCH indicating downlink SPS release in sub-frame  where , the UE shall use

-	if EPDCCH-PRB-set is configured for distributed transmission

-	if EPDCCH-PRB-set is configured for localized transmission

where  is the number of the first ECCE (i.e. lowest ECCE index used to construct the EPDCCH) used for transmission of the corresponding DCI assignment in EPDCCH-PRB-set in subframe ,  for EPDCCH-PRB-set is configured by the higher layer parameter pucch-ResourceStartOffset-r11 , for EPDCCH-PRB-set in subframe is given in Clause 6.8A.1 in [3], is determined from the antenna port used for EPDCCH transmission in subframe which is described in Clause 6.8A.5 in [3]. If ,  is determined from the HARQ-ACK resource offset field in the DCI format of the corresponding EPDCCH as given in Table 10.1.2.1-1. If ,  is determined from the HARQ-ACK resource offset field in the DCI format of the corresponding EPDCCH as given in Table 10.1.3.1-2, where the variable in the table is substituted with . If the UE is configured to monitor EPDCCH in subframe ,  is equal to the number of ECCEs in EPDCCH-PRB-set configured for that UE in subframe . If the UE is not configured to monitor EPDCCH in subframe ,  is equal to the number of ECCEs computed assuming EPDCCH-PRB-set is configured for that UE in subframe . For normal downlink CP, if subframe  is a special subframe with special subframe configuration 0, 5 or 10 if configured by ssp10-CRS-LessDwPTS, is equal to 0. For extended downlink CP, if subframe  is a special subframe with special subframe configuration 0 or 4 or 7, is equal to 0.

-	For a BL/CE UE, for a PDSCH transmission detected in subframe n- ki-Koffsetwithout a corresponding MPDCCH, the value of  is determined according to higher layer configuration and Table 9.2-2.

-	For a BL/CE UE, for a PDSCH transmission in sub-frame n- ki-Koffset where  indicated by the detection of corresponding MPDCCH or a MPDCCH indicating downlink SPS release in sub-frame n- ki-Koffset where , the UE shall use

-	if MPDCCH-PRB-set is configured for distributed transmission

-	if MPDCCH-PRB-set is configured for localized transmission

where  is the number of the first ECCE (i.e. lowest ECCE index used to construct the MPDCCH) used for transmission of the corresponding DCI assignment in MPDCCH-PRB-set ,  for MPDCCH-PRB-set  is configured

-	by the higher layer parameter n1PUCCH-AN-r13, if configured; otherwise:

-	by the higher layer parameter n1PUCCH-AN-InfoList-r13 for the corresponding CE level,

for MPDCCH-PRB-set  is given in Clause 6.8A.1 in [3], is determined from the antenna port used for the MPDCCH transmission which is described in Clause 6.8A.5 in [3]. If ,  is determined from the HARQ-ACK resource offset field in the DCI format of the corresponding MPDCCH as given in Table 10.1.2.1-1. If ,  is determined from the HARQ-ACK resource offset field in the DCI format of the corresponding MPDCCH as given in Table 10.1.3.1-2, where the variable in the table is substituted with . If subframe n- ki1-Koffset is a BL/CE downlink subframe,  is equal to the number of ECCEs in MPDCCH-PRB-set configured for that UE in subframe n- ki1-Koffset. If subframe n- ki1-Koffset is not a BL/CE downlink subframe,  is equal to 0. If subframe n- ki1-Koffset is a BL/CE downlink special subframe in which MPDCCH is not supported, the UE shall calculate  by assuming for normal cyclic prefix and for extended cyclic prefix. If an MPDCCH-PRB-set is 2+4, then . When an MPDCCH-PRB-set is 2+4, following procedures is applied.

-	if the detected MPDCCH is located within 2 PRB set, is obtained by above procedure.

-	if the detected MPDCCH is located within 4 PRB set, is the sum between and the value obtained by above procedure.

-	if the detected MPDCCH is MPDCCH format 5, is obtained by the above procedure with  = 0.

If a UE is configured with the higher layer parameter EIMTA-MainConfigServCell-r12, or the UE is configured with the higher layer parameter shortProcessingTime, then where the set is defined in Table 10.1.3.1-1B if the UE is configured with higher layer parameter shortProcessingTime and the corresponding PDCCH with CRC scrambled by C-RNTI is in the UE-specific search space for subframe-PDSCH and in Table 10.1.3.1-1 otherwise (where "UL/DL configuration" in the table refers to the higher layer parameter subframeAsssignment if the UE is configured with the higher layer parameter EIMTA-MainConfigServCell-r12), and is the number of elements in set .

If a UE is configured with the higher layer parameter EIMTA-MainConfigServCell-r12, or the UE is configured with the higher layer parameter shortProcessingTime, then the set for the rest of this Clause is as defined in Clause 10.2, and is the number of elements for subframe n in the set

If the UE is configured with the higher layer parameter EIMTA-MainConfigServCell-r12, or the UE is configured with the higher layer parameter shortProcessingTime, for TDD HARQ-ACK multiplexing and sub-frame , denote  as the PUCCH resource derived from sub-frame and HARQ-ACK(i0) as the ACK/NACK/DTX response from sub-frame, where , and .

-	 corresponding to each subframe , is determined as follows

Set ;

for

if the value of  is the same as the value of an element  in set , where ,

corresponding to subframe =;

end if

end for

for

if the value of is same as the value of an element  in set , where  (defined in Table 10.1.3.1-1A if the UE is configured with the higher layer parameter EIMTA-MainConfigServCell-r12, or Table 10.1.3.1-1B if the UE is configured with the higher layer parameter shortProcessingTime)

corresponding to subframe =;

end if

end for

-	For a PDSCH transmission indicated by the detection of corresponding PDCCH or a PDCCH indicating downlink SPS release in sub-frame ,

-	if the value of is same as the value of an element in set , the PUCCH resource  is given by ;

-	if the value of is same as the value of an element  in set , where (defined in Table 10.1.3.1-1A if the UE is configured with the higher layer parameter EIMTA-MainConfigServCell-r12), the PUCCH resource  is given by ;

-	if the value of  is same as the value of an element  in set , where (defined in Table 10.1.3.1-1B if the UE is configured with the higher layer parameter shortProcessingTime), the PUCCH resource  is given by ;

where  is the number of elements in the set  defined in Table 10.1.3.1-1A if the UE is configured with the higher layer parameter EIMTA-MainConfigServCell-r12, or Table 10.1.3.1-1B if the UE is configured with the higher layer parameter shortProcessingTime,  is selected from {0, 1, 2, 3} such that , ,  is the number of the first CCE used for transmission of the corresponding PDCCH in subframe , and , , are configured by higher layers.

-	For a PDSCH transmission where there is not a corresponding PDCCH/EPDCCH detected in subframe , the value of  is determined according to higher layer configuration and Table 9.2-2.

-	For a PDSCH transmission indicated by the detection of corresponding EPDCCH or a EPDCCH indicating downlink SPS release in sub-frame  where , the UE shall use

-	if EPDCCH-PRB-set is configured for distributed transmission

-	if EPDCCH-PRB-set is configured for localized transmission

where

-	if the value of is same as the value of an index , where , then and ;

-	otherwise, if the value of is same as the value of an index , where , then and ;

, and where  is the number of the first ECCE (i.e. lowest ECCE index used to construct the EPDCCH) used for transmission of the corresponding DCI assignment in EPDCCH-PRB-set in subframe ,  for EPDCCH-PRB-set is configured by the higher layer parameter pucch-ResourceStartOffset-r11 , for EPDCCH-PRB-set in subframe  is given in Clause 6.8A.1 in [3], is determined from the antenna port used for EPDCCH transmission in subframe  which is described in Clause 6.8A.5 in [3].

is determined as follows

-	If  and ,  is determined from the HARQ-ACK resource offset field in the DCI format of the corresponding EPDCCH as given in Table 10.1.2.1-1.

-	If  and ,  is determined from the HARQ-ACK resource offset field in the DCI format of the corresponding EPDCCH as given in Table 10.1.3.1-2, where the variable in the table is substituted with, the variable in the table is substituted with , the variable  in the table is substituted with and the variable in the table is substituted with .

-	If  and ,  is determined from the HARQ-ACK resource offset field in the DCI format of the corresponding EPDCCH as given in Table 10.1.3.1-3,

For a given subframe ,  is determined as follows

-	If the UE is configured to monitor EPDCCH in subframe ,  is equal to the number of ECCEs in EPDCCH-PRB-set configured for that UE in subframe .

-	If the UE is not configured to monitor EPDCCH in subframe ,  is equal to the number of ECCEs computed assuming EPDCCH-PRB-set is configured for that UE in subframe .

-	For normal downlink CP, if subframe  is a special subframe with special subframe configuration 0, 5 or 10 if configured by ssp10-CRS-LessDwPTS,  is equal to 0.

-	For extended downlink CP, if subframe  is a special subframe with special subframe configuration 0 or 4 or 7,  is equal to 0.

For a non-BL/CE UE, if the UE is not configured with two antenna port transmission for PUCCH format 1b with channel selection, and if the UE is not configured with the higher layer parameter EIMTA-MainConfigServCell-r12, based on higher layer signalling the UE configured with a single serving cell will perform channel selection either according to the set of Tables 10.1.3-2, 10.1.3-3, and 10.1.3-4 or according to the set of Tables 10.1.3-5, 10.1.3-6, and 10.1.3-7.

If a UE is configured with two antenna port transmission for PUCCH format 1b with channel selection, and if the UE is not configured with the higher layer parameter EIMTA-MainConfigServCell-r12, then the UE will perform channel selection according to the set of Tables 10.1.3-5, 10.1.3-6, and 10.1.3-7.

If the UE is configured with the higher layer parameter EIMTA-MainConfigServCell-r12, the UE configured with a single serving cell will perform channel selection according to the set of Tables 10.1.3-5, 10.1.3-6, and 10.1.3-7.

For the selected table set, the UE shall transmit  on PUCCH resource in sub-frame  for  mapped to antenna port  using PUCCH format 1b according to Clause 5.4.1 in [3] where

-	=for antenna port and the value of  and the PUCCH resource  are generated by channel selection according to the selected set of Tables for M = 2, 3, and 4 respectively

-	 for antenna port, where  is selected from PUCCH resources configured by higher layers where , according to selected set of Tables for M = 2, 3, and 4 respectively by replacing with and replacing  with , when the UE is configured with two antenna port transmission for PUCCH format 1b with channel selection.

Table 10.1.3-2: Transmission of HARQ-ACK multiplexing for M = 2

| HARQ-ACK(0), HARQ-ACK(1) |  |  |
| --- | --- | --- |
| ACK, ACK |  | 1, 1 |
| ACK, NACK/DTX |  | 0, 1 |
| NACK/DTX, ACK |  | 0, 0 |
| NACK/DTX, NACK |  | 1, 0 |
| NACK, DTX |  | 1, 0 |
| DTX, DTX | No transmission |  |

Table 10.1.3-3: Transmission of HARQ-ACK multiplexing for M = 3

| HARQ-ACK(0), HARQ-ACK(1), HARQ-ACK(2) |  |  |
| --- | --- | --- |
| ACK, ACK, ACK |  | 1, 1 |
| ACK, ACK, NACK/DTX |  | 1, 1 |
| ACK, NACK/DTX, ACK |  | 1, 1 |
| ACK, NACK/DTX, NACK/DTX |  | 0, 1 |
| NACK/DTX, ACK, ACK |  | 1, 0 |
| NACK/DTX, ACK, NACK/DTX |  | 0, 0 |
| NACK/DTX, NACK/DTX, ACK |  | 0, 0 |
| DTX, DTX, NACK |  | 0, 1 |
| DTX, NACK, NACK/DTX |  | 1, 0 |
| NACK, NACK/DTX, NACK/DTX |  | 1, 0 |
| DTX, DTX, DTX | No transmission |  |

Table 10.1.3-4: Transmission of HARQ-ACK multiplexing for M = 4

| HARQ-ACK(0), HARQ-ACK(1), HARQ-ACK(2), HARQ-ACK(3) |  |  |
| --- | --- | --- |
| ACK, ACK, ACK, ACK |  | 1, 1 |
| ACK, ACK, ACK, NACK/DTX |  | 1, 0 |
| NACK/DTX,NACK/DTX,NACK,DTX |  | 1, 1 |
| ACK, ACK, NACK/DTX, ACK |  | 1, 0 |
| NACK, DTX, DTX, DTX |  | 1, 0 |
| ACK, ACK, NACK/DTX, NACK/DTX |  | 1, 0 |
| ACK, NACK/DTX, ACK, ACK |  | 0, 1 |
| NACK/DTX, NACK/DTX, NACK/DTX, NACK |  | 1, 1 |
| ACK, NACK/DTX, ACK, NACK/DTX |  | 0, 1 |
| ACK, NACK/DTX, NACK/DTX, ACK |  | 0, 1 |
| ACK, NACK/DTX, NACK/DTX, NACK/DTX |  | 1, 1 |
| NACK/DTX, ACK, ACK, ACK |  | 0, 1 |
| NACK/DTX, NACK, DTX, DTX |  | 0, 0 |
| NACK/DTX, ACK, ACK, NACK/DTX |  | 1, 0 |
| NACK/DTX, ACK, NACK/DTX, ACK |  | 1, 0 |
| NACK/DTX, ACK, NACK/DTX, NACK/DTX |  | 0, 1 |
| NACK/DTX, NACK/DTX, ACK, ACK |  | 0, 1 |
| NACK/DTX, NACK/DTX, ACK, NACK/DTX |  | 0, 0 |
| NACK/DTX, NACK/DTX, NACK/DTX, ACK |  | 0, 0 |
| DTX, DTX, DTX, DTX | No transmission |  |

Table 10.1.3-5: Transmission of HARQ-ACK multiplexing for M = 2

| HARQ-ACK(0), HARQ-ACK(1) |  |  |
| --- | --- | --- |
| ACK, ACK |  | 1, 0 |
| ACK, NACK/DTX |  | 1, 1 |
| NACK/DTX, ACK |  | 0, 1 |
| NACK, NACK/DTX |  | 0, 0 |
| DTX, NACK/DTX | No Transmission |  |

Table 10.1.3-6: Transmission of HARQ-ACK multiplexing for M = 3

| HARQ-ACK(0), HARQ-ACK(1), HARQ-ACK(2) |  |  |
| --- | --- | --- |
| ACK, ACK, ACK |  | 1, 1 |
| ACK, ACK, NACK/DTX |  | 1, 0 |
| ACK, NACK/DTX, ACK |  | 1, 0 |
| ACK, NACK/DTX, NACK/DTX |  | 1, 1 |
| NACK/DTX, ACK, ACK |  | 0, 1 |
| NACK/DTX, ACK, NACK/DTX |  | 0, 1 |
| NACK/DTX, NACK/DTX, ACK |  | 0, 0 |
| NACK, NACK/DTX, NACK/DTX |  | 0, 0 |
| DTX, NACK/DTX, NACK/DTX | No Transmission |  |

Table 10.1.3-7: Transmission of HARQ-ACK multiplexing for M = 4

| HARQ-ACK(0), HARQ-ACK(1), HARQ-ACK(2), HARQ-ACK(3) |  |  |
| --- | --- | --- |
| ACK, ACK, ACK, ACK |  | 1, 1 |
| ACK, ACK, ACK, NACK/DTX |  | 1, 1 |
| ACK, ACK, NACK/DTX, ACK |  | 1, 0 |
| ACK, ACK, NACK/DTX, NACK/DTX |  | 1, 0 |
| ACK, NACK/DTX, ACK, ACK |  | 1, 1 |
| ACK, NACK/DTX, ACK, NACK/DTX |  | 1, 0 |
| ACK, NACK/DTX, NACK/DTX, ACK |  | 0, 1 |
| ACK, NACK/DTX, NACK/DTX, NACK/DTX |  | 1, 1 |
| NACK/DTX, ACK, ACK, ACK |  | 0, 0 |
| NACK/DTX, ACK, ACK, NACK/DTX |  | 0, 1 |
| NACK/DTX, ACK, NACK/DTX, ACK |  | 1, 0 |
| NACK/DTX, ACK, NACK/DTX, NACK/DTX |  | 0, 1 |
| NACK/DTX, NACK/DTX, ACK, ACK |  | 0, 1 |
| NACK/DTX, NACK/DTX, ACK, NACK/DTX |  | 0, 0 |
| NACK/DTX, NACK/DTX, NACK/DTX, ACK |  | 0, 0 |
| NACK, NACK/DTX, NACK/DTX, NACK/DTX |  | 0, 0 |
| DTX, NACK/DTX, NACK/DTX, NACK/DTX | No Transmission |  |

<Unchanged parts are omitted>

## 10.2	Uplink HARQ-ACK timing

For TDD or for FDD-TDD and primary cell frame structure type 2 or for FDD-TDD and primary cell frame structure type 1, if a UE configured with EIMTA-MainConfigServCell-r12 for a serving cell, "UL/DL configuration" of the serving cell in Clause 10.2 refers to the UL/DL configuration given by the parameter eimta-HARQ-ReferenceConfig-r12 for the serving cell unless specified otherwise.

For TDD serving cell not configured for PUSCH/PUCCH transmission, "UL/DL configuration" of the serving cell in Clause 10.2 refers to the UL/DL configuration given by the parameter harq-ReferenceConfig-r14 for the serving cell unless specified otherwise

For a non-BL/CE UE, for FDD or for FDD-TDD and primary cell frame structure type 1, the UE shall upon detection of a PDSCH transmission in subframe n-4 intended for the UE and for which an HARQ-ACK shall be provided, transmit the HARQ-ACK response in subframe n. If HARQ-ACK repetition is enabled, upon detection of a PDSCH transmission in subframe n-4 intended for the UE and for which HARQ-ACK response shall be provided, and if the UE is not repeating the transmission of any HARQ-ACK in subframe  corresponding to a PDSCH transmission in subframes , … , , the UE:

-	shall transmit only the HARQ-ACK response (corresponding to the detected PDSCH transmission in subframe ) on PUCCH in subframes , , …, ;

-	shall not transmit any other signal/channel in subframes , , …, ; and

-	shall not transmit any HARQ-ACK response repetitions corresponding to any detected PDSCH transmission in subframes , …, .

For TDD and a UE configured with EIMTA-MainConfigServCell-r12 for at least one serving cell, if the UE is configured with one serving cell or if the UE is configured with more than one serving cell and the TDD UL/DL configuration of all the configured serving cells is the same, the DL-reference UL/DL configuration for a serving cell is the UL/DL configuration of the serving cell.

For FDD-TDD and primary cell frame structure type 1, if a serving cell is a secondary serving cell with frame structure type 2, the DL-reference UL/DL configuration for the serving cell is the UL/DL configuration of the serving cell.

For TDD, if the UE is configured with more than one serving cell and if at least two serving cells have different UL/DL configurations and if a serving cell is a primary cell, then the primary cell UL/DL configuration is the DL-reference UL/DL configuration for the serving cell.

For FDD-TDD and primary cell frame structure type 2, if a serving cell is a primary cell or if a serving cell is a secondary cell with frame structure type 1, then the primary cell UL/DL configuration is the DL-reference UL/DL configuration for the serving cell.

For TDD and if the UE is configured with more than one serving cell and if at least two serving cells have different UL/DL configurations and if the UE is not configured with harqTimingTDD = TRUE and if a serving cell is a secondary cell, or for FDD-TDD and primary cell frame structure type 2 and if the UE is not configured with harqTimingTDD = TRUE and if a serving cell is a secondary cell with frame structure type 2

-	if the pair formed by (primary cell UL/DL configuration, serving cell UL/DL configuration ) belongs to Set 1 in Table 10.2-1 or

-	if the UE is not configured to monitor PDCCH/EPDCCH in another serving cell for scheduling the serving cell, and if the pair formed by (primary cell UL/DL configuration, serving cell UL/DL configuration ) belongs to Set 2 or Set 3 in Table 10.2-1 or

-	if the UE is configured to monitor PDCCH/EPDCCH in another serving cell for scheduling the serving cell, and if the pair formed by (primary cell UL/DL configuration, serving cell UL/DL configuration) belongs to Set 4 or Set 5 in Table 10.2-1

then the DL-reference UL/DL configuration for the serving cell is defined in the corresponding Set in Table 10.2-1.

For TDD and if the UE is configured with more than one serving cell and if at least two serving cells have different UL/DL configurations and if the UE is configured with harqTimingTDD = TRUE and if a serving cell is a secondary cell, or for FDD-TDD and primary cell frame structure type 2 and if the UE is configured with harqTimingTDD = TRUE and if a serving cell is a secondary cell with frame structure type 2

-	if the UE is configured to monitor PDCCH/EPDCCH in another serving cell for scheduling the serving cell, and if the pair formed by (primary cell UL/DL configuration, serving cell UL/DL configuration) belongs to Set 1 or Set 4 or Set 5 in Table 10.2-1, then the DL-reference UL/DL configuration for the serving cell is defined in the corresponding Set in Table 10.2-1;

-	if the UE is not configured to monitor PDCCH/EPDCCH in another serving cell for scheduling the serving cell, and then the primary cell UL/DL configuration is the DL-reference UL/DL configuration for the serving cell.

For a UE not configured with PUCCH format 4 or PUCCH format 5, for TDD and if a UE is configured with more than one serving cell and if at least two serving cells have different UL/DL configurations or for FDD-TDD and primary cell frame structure type 2, if the DL-reference UL/DL configuration for at least one serving cell is TDD UL/DL Configuration 5, then the UE is not expected to be configured with more than two serving cells.

For TDD and a non-BL/CE UE not configured with EIMTA-MainConfigServCell-r12 for any serving cell, if the UE is configured with one serving cell, or the UE is configured with more than one serving cell and the UL/DL configurations of all serving cells is same, then the UE shall upon detection of a PDSCH transmission within subframe(s) , where  and  is defined in Table 10.1.3.1-1 intended for the UE and for which HARQ-ACK response shall be provided, transmit the HARQ-ACK response in UL subframe n.

For a UE not configured with harqTimingTDD = TRUE, for TDD and if a UE is configured with more than one serving cell and if at least two serving cells have different UL/DL configurations, or if a UE is configured with EIMTA-MainConfigServCell-r12 for at least one serving cell, or for FDD-TDD and primary cell frame structure type 2 and if a serving cell c is frame structure type 2, then the UE shall upon detection of a PDSCH transmission within subframe(s)  for serving cell , where  intended for the UE and for which HARQ-ACK response shall be provided, transmit the HARQ-ACK response in UL subframe n, wherein set contains values of such that subframe n-k corresponds to a DL subframe or a special subframe for serving cell , where DL subframe or special subframe of serving cell  is according to the higher layer parameter eimta-HARQ-ReferenceConfig-r12 if the UE is configured with the higher layer parameter EIMTA-MainConfigServCell-r12 for serving cell  and according to harq-ReferenceConfig-r14 if configured;  defined in Table 10.1.3.1-1 (where "UL/DL configuration" in Table 10.1.3.1-1 refers to the "DL-reference UL/DL configuration") is associated with subframe n.

For a UE configured with harqTimingTDD = TRUE, for TDD and if a UE is configured with more than one serving cell and if at least two serving cells have different UL/DL configurations, or for FDD-TDD and primary cell frame structure type 2 and if a serving cell c is frame structure type 2,

-	if the UE is configured to monitor PDCCH/EPDCCH in another serving cell for scheduling the serving cell , then the UE shall upon detection of a PDSCH transmission within subframe(s)  for serving cell , where  intended for the UE and for which HARQ-ACK response shall be provided, transmit the HARQ-ACK response in UL subframe n, wherein set contains values of such that subframe n-k corresponds to a DL subframe or a special subframe for serving cell , where  is defined in Table 10.1.3.1-1 (where "UL/DL configuration" in Table 10.1.3.1-1 refers to the "DL-reference UL/DL configuration") is associated with subframe n.

-	if the UE is not configured to monitor PDCCH/EPDCCH in another serving cell for scheduling the serving cell , then the UE shall upon detection of a PDSCH transmission within subframe(s)  for serving cell , where  intended for the UE and for which HARQ-ACK response shall be provided, transmit the HARQ-ACK response in UL subframe n, wherein set contains values of such that subframe n-k corresponds to a DL subframe or a special subframe for serving cell , where  is defined in Table 10.1.3A-1 (where "UL/DL configuration" in Table 10.1.3A-1 refers to the "DL-reference UL/DL configuration") is associated with subframe n.

For a non-BL/CE UE, and for FDD-TDD and primary cell frame structure type 2, if a serving cell  is frame structure type 1 and a UE is not configured to monitor PDCCH/EPDCCH in another serving cell for scheduling the serving cell , then the UE shall upon detection of a PDSCH transmission within subframe(s)  for serving cell , where , and is defined in Table 10.1.3A-1 intended for the UE and for which HARQ-ACK response shall be provided, transmit the HARQ-ACK response in subframe n.

For FDD-TDD and primary cell frame structure type 2, if a serving cell  is frame structure type 1 and a UE is configured to monitor PDCCH/EPDCCH in another serving cell for scheduling serving cell , then the UE shall upon detection of a PDSCH transmission within subframe(s)  for serving cell , where , and is defined in Table 10.1.3.1-1, intended for the UE and for which HARQ-ACK response shall be provided, transmit the HARQ-ACK response in subframe n, where "UL/DL configuration" in Table 10.1.3.1-1 refers to the "DL-reference UL/DL configuration" of serving cell .

For TDD, if HARQ-ACK repetition is enabled, upon detection of a PDSCH transmission within subframe(s) , where  and  is defined in Table 10.1.3.1-1 intended for the UE and for which HARQ-ACK response shall be provided, and if the UE is not repeating the transmission of any HARQ-ACK in subframe  corresponding to a PDSCH transmission in a downlink or special subframe earlier than subframe , the UE:

-	shall transmit only the HARQ-ACK response (corresponding to the detected PDSCH transmission in subframe) on PUCCH in UL subframe and the next UL subframes denoted as , …,;

-	shall not transmit any other signal/channel in UL subframe , , …,; and

-	shall not transmit any HARQ-ACK response repetitions corresponding to any detected PDSCH transmission in subframes , where , is the set defined in Table 10.1.3.1-1 corresponding to UL subframe , and .

For TDD, HARQ-ACK bundling, if the UE detects that at least one downlink assignment has been missed as described in Clause 7.3, the UE shall not transmit HARQ-ACK on PUCCH if HARQ-ACK is the only UCI present in a given subframe.

For FDD, a BL/CE UE shall upon detection of a PDSCH intended for the UE and for which an HARQ-ACK shall be provided, transmit the HARQ-ACK response using the same  derived according to Clause 10.1.2.1 in subframe(s) n+ki with i =0,1, …, N-1, where

-	subframe n-k-Koffset is the last subframe in which the PDSCH is transmitted, where

-	if the UE is in half-duplex FDD operation and is configured with CEModeA and higher layer parameter ce-HARQ-AckBundling and the 'HARQ-ACK bundling flag' in the corresponding DCI is set to 1, or if the UE is configured with higher layer parameter ce-SchedulingEnhancement

-	k is given by the 'HARQ-ACK delay' field in the corresponding DCI, and the HARQ-ACK delay value k is determined based on the higher layer parameters according to Table 7.3.1-2;

-	otherwise

-	k=4

-	0≤k0<k1<…,kN-1 and the value of and  is provided by higher layer parameter pucch-NumRepetitionCE-format1, if configured, otherwise it is provided by higher layer parameter pucch-NumRepetitionCE-Msg4-Level0-r13, pucch-NumRepetitionCE-Msg4-Level1-r13, pucch-NumRepetitionCE-Msg4-Level2-r13 or pucch-NumRepetitionCE-Msg4-Level3-r13 depending on whether the most recent PRACH coverage enhancement level for the UE is 0, 1, 2 or 3, respectively; and

if N>1

-	subframe(s) n+ki with i=0,1,…,N-1 are N consecutive BL/CE UL subframe(s) immediately after subframe n-1, and the set of BL/CE UL subframes are configured by higher layers;

otherwise

-	k0 =0

except if the UE is configured with higher layer parameter ce-PDSCH-MultiTB-Config and multiple TB are scheduled in the corresponding DCI.

For FDD, if a BL/CE UE is configured with CEModeA, and if the UE is not configured with higher layer parameter harq-AckBundling in ce-PDSCH-MultiTB-Config and multiple TB are scheduled in the corresponding DCI, the BL/CE UE shall upon detection of a PDSCH intended for the UE and for which an HARQ-ACK shall be provided, transmit the HARQ-ACK response using the same  derived according to Clause 10.1.2.1 in subframe(s) sb+ki+Koffset sb+ki with b=0,1,⋯NTB-1,  i =0,1, …, N-1, where

-	is the number of scheduled TB determined in the corresponding DCI;

-	if the UE is not configured with higher layer parameter interleaving in ce-PDSCH-MultiTB-Config and the UE is not in half-duplex FDD operation

-	s0=n0+4, sb=max {nb+4,  sb-1+Nb-1}, b≠0

-	otherwise

-	s0=max n0+4,  nL+2, sb=max {nb+4,  sb-1+Nb-1}, b≠0

-	nb is the last subframe in which the PDSCH containing TB b is transmitted;

-	subframe nL is the last subframe in which the PDSCH is transmitted;

-	Nb denotes the number of consecutive subframes including non-BL/CE subframes where the PUCCH with HARQ ACK for TB b with repetition number of N is transmitted;

and

-	0≤k0<k1<…,kN-1 and the value of and  is provided by higher layer parameter pucch-NumRepetitionCE-format1, if configured, otherwise it is provided by higher layer parameter pucch-NumRepetitionCE-Msg4-Level0-r13, pucch-NumRepetitionCE-Msg4-Level1-r13, pucch-NumRepetitionCE-Msg4-Level2-r13 or pucch-NumRepetitionCE-Msg4-Level3-r13 depending on whether the most recent PRACH coverage enhancement level for the UE is 0, 1, 2 or 3, respectively; and

if N>1

-	subframe(s) sb+ki+Koffsetsb+ki with i=0,1,…,N-1 for TB b are N consecutive BL/CE UL subframe(s) immediately after subframe sb-1+Koffsetsb-1, and the set of BL/CE UL subframes are configured by higher layers;

otherwise

-	k0 =0

For FDD, if a BL/CE UE is configured with CEModeA, and if the UE is configured with higher layer parameter harq-AckBundling in ce-PDSCH-MultiTB-Config and multiple TB are scheduled in the corresponding DCI, the BL/CE UE shall upon detection of a PDSCH intended for the UE and for which an HARQ-ACK shall be provided, transmit the HARQ-ACK response using the same  derived according to Clause 10.1.2.1 in subframe(s) sb+ki+Koffsetsb+ki with b=0,1,⋯NTBM-1,  i =0,1, …, N-1, where

-	is the number of scheduled TB determined in the corresponding DCI;

-	 is the multi-TB HARQ-ACK bundling size;

-	if the UE is not configured with higher layer parameter interleaving in ce-PDSCH-MultiTB-Config and the UE is not in half-duplex FDD operation

-	s0=n0+4, sb=max {nb+4,  sb-1+Nb-1}, b≠0

-	otherwise

-	subframe s0=max n0+4,  nL+2, sb=max {nb+4,  sb-1+Nb-1}, b≠0

-	subframe nb is the last subframe in which the PDSCH containing TB bundle b is transmitted;

-	subframe nL is the last subframe in which the PDSCH is transmitted;

-	Nb denotes the number of consecutive subframes including non-BL/CE subframes where the PUCCH with HARQ ACK for TB bundle b with repetition number of N is transmitted;

and

-	0≤k0<k1<…,kN-1 and the value of and  is provided by higher layer parameter pucch-NumRepetitionCE-format1, if configured, otherwise it is provided by higher layer parameter pucch-NumRepetitionCE-Msg4-Level0-r13, pucch-NumRepetitionCE-Msg4-Level1-r13, pucch-NumRepetitionCE-Msg4-Level2-r13 or pucch-NumRepetitionCE-Msg4-Level3-r13 depending on whether the most recent PRACH coverage enhancement level for the UE is 0, 1, 2 or 3, respectively; and

if N>1

-	subframe(s) sb+ki+Koffsetsb+ki with i=0,1,…,N-1 for TB bundle b are N consecutive BL/CE UL subframe(s) immediately after subframesb-1+Koffset sb-1, and the set of BL/CE UL subframes are configured by higher layers;

otherwise

-	k0 =0

For TDD, a BL/CE UE shall upon detection of a PDSCH within subframe(s) n- k-Koffset, where  and  is defined in Table 10.1.3.1-1 intended for the UE and for which HARQ-ACK response shall be provided, transmit the HARQ-ACK response using the same  derived according to Clause 10.1.3.1 in subframe(s) n+ki with i =0,1, …, N-1, where

-	subframe n-k -Koffset is the last subframe in which the PDSCH is transmitted; and

-	0≤k0<k1<…,kN-1 and the value of and  is provided by higher layers parameter pucch-NumRepetitionCE-format1, if configured, otherwise it is provided by higher layer parameter pucch-NumRepetitionCE-Msg4-Level0-r13, pucch-NumRepetitionCE-Msg4-Level1-r13, pucch-NumRepetitionCE-Msg4-Level2-r13 or pucch-NumRepetitionCE-Msg4-Level3-r13 depending on whether the most recent PRACH coverage enhancement level for the UE is 0, 1, 2 or 3, respectively; and

if N>1

-	subframe(s) n+ki with i=0,1,…,N-1 are N consecutive BL/CE UL subframe(s) immediately after subframe n-1, and the set of BL/CE UL subframes are configured by higher layers;

otherwise

-	k0 =0

except if the UE is configured with higher layer parameter ce-PDSCH-MultiTB-Config and multiple TB are scheduled in the corresponding DCI.

For TDD, if a BL/CE UE is configured with higher layer parameter ce-PDSCH-MultiTB-Config and multiple TBs are scheduled in the corresponding DCI, the BL/CE UE shall upon detection of a PDSCH intended for the UE and for which HARQ-ACK response shall be provided, transmit the HARQ-ACK response using the same  derived according to Clause 10.1.3.1 in subframe(s) sb+ki+Koffsetsb+ki with b=0,1,…,B-1,  i =0,1, …, N-1, where

-	B is the number of TB bundles

-	if the UE is not configured with higher layer parameter harq-AckBundling in ce-PDSCH-MultiTB-Config, B=NTB with bundle b consisting of only TB b.

-	Else, the value of B and the corresponding TBs in each bundle is determined according to clause 7.3

-	is the number of scheduled TB determined in the corresponding DCI;

-	s0=n0+4, sb=max {nb+4,  sb-1+Nb-1}, b≠0

-	nb is the last subframe in which the PDSCH containing TB bundle b is transmitted;

-	Nb denotes the number of consecutive subframes including subframes that are not BL/CE UL subframes where the PUCCH with HARQ ACK for TB bundle b with repetition number of N is transmitted;

and

-	0≤k0<k1<…,kN-1 and the value of and  is provided by higher layers parameter pucch-NumRepetitionCE-format1, if configured, otherwise it is provided by higher layer parameter pucch-NumRepetitionCE-Msg4-Level0-r13, pucch-NumRepetitionCE-Msg4-Level1-r13, pucch-NumRepetitionCE-Msg4-Level2-r13 or pucch-NumRepetitionCE-Msg4-Level3-r13 depending on whether the most recent PRACH coverage enhancement level for the UE is 0, 1, 2 or 3, respectively; and

-	subframe(s) sb+ki+Koffsetsb+ki with i=0,1,…,N-1 for TB bundle b are N consecutive BL/CE UL subframe(s) immediately after subframe sb-1+Koffsetsb-1, and the set of BL/CE UL subframes are configured by higher layers.

The uplink timing for the ACK corresponding to a detected PDCCH/EPDCCH indicating downlink SPS release shall be the same as the uplink timing for the HARQ-ACK corresponding to a detected PDSCH, as defined above.

For a BL/CE UE, the uplink timing for the ACK corresponding to a detected MPDCCH indicating downlink SPS release shall be the same as the uplink timing for the HARQ-ACK corresponding to a detected PDSCH, as defined above.

For a BL/CE UE, if a first HARQ-ACK transmission associated to a first set of PDSCH partially collides with a second HARQ-ACK transmission associated to a second set of PDSCH transmissions, the last PDSCH of the first set of PDSCH transmissions being detected before the last PDSCH of the second set of PDSCH transmissions, the UE shall drop the second HARQ-ACK transmission.

Table 10.2-1: DL-reference UL/DL configuration for serving cell based on pair formed by (primary cell UL/DL configuration, secondary cell UL/DL configuration)

| Set # | (Primary cell UL/DL configuration, Secondary cell UL/DL configuration) | DL-reference UL/DL configuration |
| --- | --- | --- |
| Set 1 | (0,0) | 0 |
|  | (1,0),(1,1),(1,6) | 1 |
|  | (2,0),(2,2),(2,1),(2,6) | 2 |
|  | (3,0),(3,3),(3,6) | 3 |
|  | (4,0),(4,1),(4,3),(4,4),(4,6) | 4 |
|  | (5,0),(5,1),(5,2),(5,3),(5,4),(5,5),(5,6) | 5 |
|  | (6,0),(6,6) | 6 |
| Set 2 | (0,1),(6,1) | 1 |
|  | (0,2),(1,2),(6,2) | 2 |
|  | (0,3),(6,3) | 3 |
|  | (0,4),(1,4),(3,4),(6,4) | 4 |
|  | (0,5),(1,5),(2,5),(3,5),(4,5),(6,5) | 5 |
|  | (0,6) | 6 |
| Set 3 | (3,1),(1,3) | 4 |
|  | (3,2),(4,2),(2,3),(2,4) | 5 |
| Set 4 | (0,1),(0,2),(0,3),(0,4),(0,5),(0,6) | 0 |
|  | (1,2),(1,4),(1,5) | 1 |
|  | (2,5) | 2 |
|  | (3,4),(3,5) | 3 |
|  | (4,5) | 4 |
|  | (6,1),(6,2),(6,3),(6,4),(6,5) | 6 |
| Set 5 | (1,3) | 1 |
|  | (2,3),(2,4) | 2 |
|  | (3,1),(3,2) | 3 |
|  | (4,2) | 4 |

<Unchanged parts are omitted>