3GPP TSG RAN meeting #94e										RP-213066

Electronic Meeting, Dec 06 - 17, 2021

## Status Report to TSG

Agenda item:			9.3.4.9

| WI / SI Name | Further RRM enhancement for NR and MR-DC |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: Yes | Performance part: No |  | Testing part: No |
| Acronym | NR_RRM_enh2 |  |  |  |  |
| Unique ID | 890157 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-202874 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: NA | Core part: 03/2022 | Performance part: 09/2022 | Testing part: 09/2022 |  |
| Overall Completion level | Study Item: NA | Core part: 75% | Performance Part: 0% | Testing part: |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | RAN WG4 |
| --- | --- | --- |
| Rapporteur | Name | Jie Cui |
|  | Company | Apple Corporation |
|  | Email | jie_cui@apple.com |
| Rapporteur | Name | Qiuge Guo |
|  | Company | CATT |
|  | Email | guoqiuge@catt.cn |

## 1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

## 2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

## 2.4	RAN4

#### 2.4.1	Agreements

RAN4 #101-e meeting

(1) SRS antenna port switching

The WF R4-2120295 on SRS antenna port switching was agreed.

Issue 1-1-1: whether scheduling restriction requirement would be defined in RRM for SRS antenna port switching

Issue1-1-1a: Clarification on performance degradation due to SRS antenna port switching:

Agreement:

The performance degradation can be expected on 1 OFDM symbol before and after each SRS resource configured for antenna switching which is not overlapped with the guard period defined in TS 38.214 on the carrier where SRS antenna switching occurs.

Issue 1-2-2: Impact of SRS antenna port switching to RRM requirements in EN-DC or NE-DC

Agreements: Follow the conclusion from issue 1-2-1

Issue 1-2-3: Impact of SRS antenna port switching to RRM requirements in NR-DC

Agreements: Follow the conclusion from issue 1-2-1

Issue 1-3-1: Interruption requirement applicability

Agreements:

The victim cells due to SRS switching shall be defined for UL interruption and DL interruption respectively, e.g. an UL interruption is allowed on any of the serving cells indicated in txSwitchWithAnotherBand, and a DL interruption is allowed on any of the serving cells indicated by txSwitchImpactToRx.

Issue 1-3-3: txSwitchImpactToRx for intra-band case

Agreements:

No need to have clarification for txSwitchImpactToRx with intra-band contiguous CA and intra-band non-contiguous CA case.

Issue 1-4-2: The components within interruption time of SRS antenna port switching in FR1

Agreement:

- The components of interruption time of SRS antenna port switching in FR1 are

  - Antenna switching time before and after SRS transmission occasion (2*15us)

  - SRS transmission time of X symbols

    - Requirements would be defined for two scenarios:

      - Scenario 1: when X=1 SRS symbol is configured in a slot for SRS antenna port switching, the configured number of SRS symbols is used as SRS transmission time

      - Scenario 2: otherwise, using X=6 SRS symbols in a slot as assumption of SRS transmission time

Issue 1-5-1: Interruption requirement related with prioritization rule in RAN1

Agreement:

- RAN4 doesn’t define the rules to avoid collisions except what has been defined in RAN1 and NR measurement.

  - FFS: Interruption of SRS antenna switching is applicable only when the interruption time is not colliding with any other transmission with higher priority defined in clause 6.2.1 of TS 38.214.

(2) Handover with PSCell

The WF R4-2120297 and LS R4-2120298 on handover with PSCell was agreed.

Issue 2-2-1a-1: Condition of parallel processing without considering RACH for NR SA to EN-DC

RAN4 agrees to send an LS to RAN2 asking for clarification on configuration of HO with PSCell for NR SA to EN-DC scenario.

Issue 2-2-1b: Whether requirements for sequential processing are needed if parallel processing is only possible under certain condition

- RRM requirements for HO with PSCell are defined for both parallel processing cases and sequential processing cases when applicable, irrelevant of deployment scenarios.

Issue 2-2-2b-1: Timeline for parallel processing delay requirements without considering Tprocessing and RA procedures

- For parallel processing cases, PCell HO and PSCell addition are performed in parallel independently

Issue 2-2-2b-2: Timeline for sequential processing delay requirements without considering Tprocessing and RA procedures

- Sequential processing is used for

  - Cell search

  - SSB processing margin (i.e. 2ms)

  - Fine time/frequency tracking and acquiring timing information for PCell

Issue 2-2-5: Ending point of the delay requirement for HO with PSCell

- Defining delay requirements for HO and PSCell addition/change separately with the ending points defined as PCell PRACH and PSCell PRACH respectively.

Issue 2-2-8: Delay requirements design

- New HO with PSCell RRM requirements are specified in dedicated sections.

Issue 2-2-2a: How the requirements for parallel processing and sequential processing are defined without considering Tprocessing and RA procedures

- Different requirements for parallel processing cases and sequential processing cases

Issue 2-2-2c: SMTC for target NR PSCell

- In HO with PSCell for NR-DC to NR-DC, if SMTC of target unknown PSCell is not configured in either targetcellSMTC-SCG-r16 or reconfigurationWithSync,

  - If either source PCell or source PSCell has configured the UE with an MO which have the same SSB frequency and SCS as target PSCell, UE uses the SMTC in the configured MO, or

  - If both source PCell and source PSCell have configured the UE with MOs which have the same SSB frequency and SCS as target PSCell, it is FFS which SMTC in which configured MO the UE uses, or

  - If neither source PCell nor source PSCell has configured the UE with MO which have the same SSB frequency and SCS as target PSCell, UE assumes 5ms as SSB periodicity for target PSCell.

- In HO with PSCell for NR SA to EN-DC, if SMTC of target unknown PSCell is not configured,

  - FFS.

- In HO with PSCell for EN-DC to EN-DC, if SMTC of target unknown PSCell is not configured,

  - If either source LTE PCell or source NR PSCell has configured the UE with an MO which have the same SSB frequency and SCS as target PSCell, UE uses the SMTC in the configured MO, or

  - If both source LTE PCell and source NR PSCell have configured the UE with MOs which have the same SSB frequency and SCS as target PSCell, it is FFS which SMTC in which configured MO the UE uses , or

  - If neither source LTE PCell nor source NR PSCell has configured the UE with MO which have the same SSB frequency and SCS as target PSCell, UE assumes 5ms as SSB periodicity for target PSCell.

Issue 2-2-3e: Tprocessing for PSCell addition/change

It is for common understanding of existing PSCell addition/change requirements.

- For PSCell change for NR-DC and EN-DC

  - 20ms, when source and target cells are in the same FR

  - 40ms, when source and target cells are in different FRs

- For PSCell addition for NR-DC and EN-DC

  - 20ms, when NR PSCell is in the same FR as PCell

  - 40ms, when NR PSCell is in the different FR from PCell

Issue 2-3-2a: Interruption requirements, similar as Tinterrupt for in legacy handover requirements, for HO with PSCell

- Interruption in legacy handover delay requirement can be applied for PCell. No interruption is defined for PSCell addition/change.

Agreements in the 2st round email discussion

Issue 2-3-2b: Interruption requirements on PCell due to PSCell RF retuning

- For parallel processing cases, no additional interruption requirements should be defined during HO with PSCell

- FFS cases when sequential processing is used for HO with PSCell

(3) PUCCH SCell activation/deactivation requirements

The WF R4-2120299 and LS R4-2120420 on PUCCH SCell activation/deactivation requirements was agreed.

Issue 1-1-1: Which CSI reporting types (periodic, aperiodic, semi-persistent) can be used for PUCCH SCell activation?

Agreements:

- The CSI reporting types about PUCCH SCell activation need not to be specified in RAN4 specification.

- Periodic and semi-persistent CSI-RS are assumed for CSI/CQI reporting when defining PUCCH SCell activation delay requirements.

Issue 1-2-1: Whether the beam information (L1-RSRP measurement result) of PUCCH Scell for TCI determination is needed to be indicated for unknown cell?

Agreements:

- Same as the beam information indication for determining the associated SSB in PDCCH order for RA.

- If the target PUCCH Scell is unknown cell in FR2:

  - If there is at least one active serving cell on that FR2 band (following the same conditions in TS38.133 section 8.3.2 for intra-band FR2 Scell activation), no need to indicate the beam information of PUCCH Scell to network for TCI determination.

  - Otherwise, need to indicate the beam information of PUCCH Scell to network for TCI determination.

- If the target PUCCH Scell is unknown cell in FR1:

  - If it is contiguous to an active serving cell in the same band (following the same conditions in TS38.133 section 8.3.2 for intra-band contiguous FR1 Scell activation), no need to indicate the beam information of PUCCH Scell to network for TCI determination.

  - Otherwise, need to indicate the beam information of PUCCH Scell to network for TCI determination.

- Note: The beam information indication if sent can be used for both determining the associated SSB in PDCCH order for RA and TCI determination.

Issue 1-3-1: For Tactivation_time, whether the PL-RS, TCI sate and spatial relation should follow the L3 and L1-RSRP measurement for known and unknown PUCCH SCell, respectively?

Agreements:

- For Tactivation_time,

  - For known PUCCH SCell,

    - TCI sate, PL-RS and spatial relation indication are assumed to be based on the L3 measurement.

  - For unknown PUCCH SCell,

    - TCI sate, PL-RS and spatial relation indication are assumed to be based on L1-RSRP measurement.

Issue 1-3-2: For Tactivation_time, whether the TCI state indication will introduce extra delay time?

Agreements:

- For Tactivation_time, TCI state indication would not introduce additional delay time.

Issue 1-3-3: How to reflect the time uncertainty of MAC CE for UL spatial relation and PL-RS activation and TCI state indication in PUCCH Scell activation delay requirements?

Agreements in GTW (11.3):

  - No additional delay time is needed if UL spatial relation and PL-RS activation command and TCI activation command are received in the same MAC PDU.

  - For both valid TA and invalid TA cases in FR2 PUCCH SCell activation, the uncertainty for receiving UL spatial relation and PL-RS activation command and TCI activation command could be defined as below,

    - Tuncertainty_MAC is the time period between reception of the last activation command for PDCCH TCI, PDSCH TCI (when applicable), UL spatial relation and PL-RS relative to

      - SCell activation command for known case;

      - First valid L1-RSRP reporting for unknown case.

Issue 1-3-4: Whether the conclusions in issue 1-3-1, issue 1-3-2, issue 1-3-2a, issue 1-3-2b and issue 1-3-3 also applied to invalid TA case?

Agreements:

- The conclusions in issue 1-3-1, issue 1-3-2 issue 1-3-2a, issue 1-3-2b and issue 1-3-3 also applied to invalid TA case.

Issue 1-5-2 Whether to send LS to RAN1/2 to clarify the UE capability diffNumerologyAcrossPUCCH-Group

- Agreements in GTW(11.3)

  - Reuse the interruption requirement of normal Scell activation when target PUCCH SCell RACH has same SCS as spCell data/control channel

  - FFS how to handle case when target PUCCH SCell RACH has different SCS from spCell data/control channel

  - Ask RAN1/2 to clarify the following

    - Applicability of UE capability diffNumerologyAcrossPUCCH-Group to PRACH

    - UE behavior to handle the case when target PUCCH SCell RACH has different SCS from spCell data/control channel

    - Further discuss the LS to RAN1/2

- The LS can be found in R4-2120300 and its further revision.

Issue 1-5-3 The interruption requirements applicability

Agreements:

- Interruption of PUCCH SCell activation is allowed on:

  - PCell and activated SCell(s) in the same FR as the target PUCCH SCell being activated during the PUCCH SCell activation procedure if UE supports per-FR MG.

  - PCell and activated SCell(s) in both FR1 and FR2 during the PUCCH SCell activation procedure if UE does not support per-FR MG.

Issue 1-6-4 Applicability on multiple SCells:

Agreements:

- Postpone the requirement of activation for multiple SCells with one PUCCH SCell until the single PUCCH SCell activation requirement has been finalized.

#### 2.4.2	Remaining Open issues

(1) SRS antenna port switching (open issues in R4-2120295)

- Issue1-1-1b: Whether to capture conclusion of issue 1-1-1a in RRM spec or in WF only

- Issue 1-1-2: RAN4 requirement scope with different SRS resource configuration

- Issue 1-2-1: Impact of SRS antenna port switching to RRM requirements in NR

- Issue 1-2-4: Impact of SRS antenna port switching to other specific RRM requirements

- Issue 1-2-5: Impact of SRS antenna port switching to CSF and other RS

- Issue 1-3-2: Whether and how to specify interruption requirement for sync case

- Issue 1-4-1: The interruption requirement is defined based on slot level or symbol level

- Issue 1-4-3: Interruption requirement proposals for async case

- Issue 1-4-4: Interruption requirement proposals for sync case

- Issue 1-5-2: Two SRS colliding on same symbol

(2) Handover with PSCell (open issues in R4-2120297)

- Issue 2-2-2c-1: If both source PCell and source PSCell configured MOs which have the same SSB frequency and SCS as target PSCell, UE use the SMTC in the MO

- Issue 2-2-3a: Timeline of Tprocessing (UE SW processing and RF warm-up(if needed) time) for HO with PSCell

- Issue 2-2-3b: If UE SW processing and RF warm-up for PCell HO and PSCell addition/change are performed in parallel

- Issue 2-2-8a: How the HO with PSCell delay requirements are specified

- Issue 2-3-2b: Interruption requirements on PCell due to PSCell RF retuning

- Issue 2-4-1: 2 step and 4 step RACH for HO with PSCell

- Issue 2-4-2: RACH occasion collision between Pcell and PSCell

- Issue 2-4-4: CSI-RS based CFRA

- Issue 2-5-1: Requirements for HO with PSCell for NR-U

(3) PUCCH SCell activation/deactivation requirements (open issues in R4-2120299)

- Issue 1-3-2a: For Tactivation_time, whether spatial relation will introduce extra delay time?

- Issue 1-3-2b: For Tactivation_time, whether the PL-RS will introduce extra delay time?

### Issue 1-4-1: The PUCCH SCell activation requirements for invalid TA case

### Issue 1-4-1a: Whether TPDCCH is needed in the PUCCH SCell activation requirements for invalid TA case

### Issue 1-4-1b: Whether TCSI_reporting is needed in the PUCCH SCell activation requirements for invalid TA case.

### Issue 1-4-2: The delay for obtaining a valid TA command for the sTAG to which the Scell configured with PUCCH belongs (i.e. T2)

### Issue 1-4-3: The components of Tactivation_time

### Issue 1-6-1 Applicability on interruption

### Issue 1-6-2: Applicability on PDCCH order receiving

### Issue 1-6-3: Applicability on use cases

## 3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

NOTE: This section only needs to be filled in for WI/SIs where there is a corresponding relevant WI/SI in SA/CT.

## 3.1	SAx/CTs

#### 3.1.1	Agreements with cross-TSG impacts

#### 3.1.2	Remaining Open issues with cross-TSG impacts

NOTE: This section should also flag any critical dependencies that need TSG attention.

## 4.	References

NOTE:	This can be e.g. a list of all related Tdocs in the affected WGs since last TSG, references to LSs, produced TRs/TSs, the work/study item description or status reports of previous TSGs.

RAN4 #101-e meeting

| TDoc | Title | Source |
| --- | --- | --- |
| R4-2117325 | Further discussion on SRS antenna port switching | CATT |
| R4-2117326 | Requirements for SRS antenna port switching | CATT |
| R4-2117327 | Further discussion on HO with PSCell | CATT |
| R4-2117328 | Requirements for HO with PSCell | CATT |
| R4-2117329 | Requirements for HO with PSCell for EN-DC | CATT |
| R4-2117330 | Further discussion on PUCCH SCell activation_deactivation | CATT |
| R4-2117331 | Requirements for PUCCH SCell activation_deactivation | CATT |
| R4-2117446 | Discussion on SRS antenna port switching | Apple |
| R4-2117447 | Discussion on RRM requirement for handover with PSCell | Apple |
| R4-2117448 | Discussion on PUCCH SCell activation and deactivation | Apple |
| R4-2117525 | Discussions on PUCCH SCell Activation/Deactivation delay requirements | NTT DOCOMO, INC. |
| R4-2117621 | On SRS antenna switching | Qualcomm, Inc. |
| R4-2117695 | Discussion on HO with PSCell | CMCC |
| R4-2117705 | Discussion on PUCCH SCell activation/deactivation | CMCC |
| R4-2117706 | Discussion on SRS antenna port switching | CMCC |
| R4-2117798 | Further considerations on PUCCH SCell activation and deactivation | vivo |
| R4-2117811 | Further discussion on RRM requirements for SRS antenna switching | Xiaomi |
| R4-2117821 | Further discussion on RRM requirements for handover with PSCell | Xiaomi |
| R4-2117822 | Further discussion on SCell activation and deactication requirements for PUCCH SCell | Xiaomi |
| R4-2117837 | Discussion on interruption due to SRS antenna port switching | LG Electronics Inc. |
| R4-2118023 | Discussion on SRS antenna port switching | Intel Corporation |
| R4-2118024 | Discussion on HO with PSCell | Intel Corporation |
| R4-2118025 | Discussion on PUCCH SCell activation | Intel Corporation |
| R4-2118096 | Discussion on SRS antenna port switching | MediaTek Inc. |
| R4-2118097 | Discussion on PUCCH SCell activation and deactivation | MediaTek Inc. |
| R4-2118254 | Discussion on RRM requirements for SRS antenna port switching | vivo |
| R4-2118255 | Discussion on RRM requirements for HO with PSCell | vivo |
| R4-2118361 | RRM requirements for SRS ant port switch | OPPO |
| R4-2118362 | RRM requirements for HO with PSCell | OPPO |
| R4-2118363 | RRM requirements for PUCCH SCell ActivationDeactivation | OPPO |
| R4-2118414 | Discussion on the activation delay for deactivated PUCCH SCell | Nokia, Nokia Shanghai Bell |
| R4-2118415 | 38.133 draft CR on PUCCH SCell activation delay requirements | Nokia, Nokia Shanghai Bell |
| R4-2118416 | Interruption requirements at SRS antenna port switching | Nokia, Nokia Shanghai Bell |
| R4-2118417 | 38.133 draft CR on interruption requirement at SRS antenna switching | Nokia, Nokia Shanghai Bell |
| R4-2118426 | Discussion on RRM requirements for HO with PSCell | ZTE Corporation |
| R4-2118427 | Discussion on RRM requirements for PUCCH SCell activation and deactivation | ZTE Corporation |
| R4-2118428 | Discussion on RRM requirements for SRS antenna port switching | ZTE Corporation |
| R4-2118441 | discussion on HO with PSCell | Nokia, Nokia Shanghai Bell |
| R4-2118442 | dratCR on HO with PSCell | Nokia, Nokia Shanghai Bell |
| R4-2118752 | Discussion on RRM requirements for SRS antenna port switching | Ericsson |
| R4-2118753 | Discussion on RRM requirements for handover with PSCell | Ericsson |
| R4-2118754 | Discussion on RRM requirements for SCell (de)activation with PUCCH | Ericsson |
| R4-2118755 | Draft CR on HO requirements with PSCell | Ericsson |
| R4-2118842 | Discussion on RRM requirements for SRS antenna switching | Huawei, Hisilicon |
| R4-2118843 | Draft CR on introducing RRM requirements for SRS antenna switching | Huawei, Hisilicon |
| R4-2118844 | Discussion on RRM requirements for HO with PSCell | Huawei, Hisilicon |
| R4-2118845 | Draft CR on introducing RRM requirements for HO with PSCell | Huawei, Hisilicon |
| R4-2118846 | Discussion on PUCCH SCell activation | Huawei, Hisilicon |
| R4-2118995 | RF impact On FR2 RedCap | Ericsson |
| R4-2119218 | Discussion on HO with PSCell | MediaTek inc. |
| R4-2119581 | HO with PSCell | Qualcomm Incorporated |
| R4-2119582 | PUCCH SCell activation and deactivation | Qualcomm Incorporated |
| R4-2120213 | Email discussion summary for [101-e][218] NR_RRM_enh2_1 | Moderator (Apple) |
| R4-2120214 | Email discussion summary for [101-e][219] NR_RRM_enh2_2 | Moderator (vivo) |
| R4-2120215 | Email discussion summary for [101-e][220] NR_RRM_enh2_3 | Moderator (CATT) |
| R4-2120294 | Email discussion summary for [101-e][218] NR_RRM_enh2_1 | Moderator (Apple) |
| R4-2120295 | WF on further RRM enhancement for NR and MR-DC - SRS antenna port switching | Apple |
| R4-2120296 | LS on SRS antenna port switching | Vivo |
| R4-2120297 | WF on further RRM enhancement for NR and MR-DC – HO with PSCell | Vivo |
| R4-2120298 | LS on HO with PSCell from NR SA to EN-DC | MediaTek |
| R4-2120299 | WF on further RRM enhancement for NR and MR-DC - PUCCH SCell activation/deactivation requirements | CATT |
| R4-2120300 | LS on interruption for PUCCH SCell activation in invalid TA case | MediaTek, CATT |
| R4-2120360 | Email discussion summary for [101-e][218] NR_RRM_enh2_1 | Moderator (Apple) |
| R4-2120361 | Email discussion summary for [101-e][219] NR_RRM_enh2_2 | Moderator (vivo) |
| R4-2120362 | Email discussion summary for [101-e][220] NR_RRM_enh2_3 | Moderator (CATT) |
| R4-2120420 | LS on interruption for PUCCH SCell activation in invalid TA case | MediaTek, CATT |

28.01.2021		minor adaptations for RAN #91e

09.11.2020		minor adaptations for RAN #90e

31.08.2020		minor adaptations for RAN #89e

20.04.2020		minor adaptations for RAN #88e

18.02.2020		minor adaptations for RAN #87e

14.11.2019		minor adaptations for RAN #86

18.08.2019		minor adaptations for RAN #85

12.05.2019		minor adaptations for RAN #84

27.02.2019		minor adaptations for RAN #83

21.11.2018		completion levels with colours added (for RAN #82)

v04.81	31.07.2018		simplification of template and addition of cross-TSG aspects (for RAN #81)

v04.80	21.05.2018		minor adaptations for RAN #80

v04.79	26.02.2018		minor adaptations for RAN #79

v04.78	18.11.2017		minor adaptations for RAN #78

v04.77	06.08.2017		minor adaptations for RAN #77

v04.76	15.05.2017		minor adaptations for RAN #76

v04.75	31.01.2017		minor adaptations for RAN #75

v04.74	28.10.2016		minor adaptations for RAN #74

v04.73	01.09.2016		adaptations for RAN #73 (time units in extra Excel table, RAN6 reporting included)

v04.72	26.05.2016		adaptations for RAN #72 (introduction of NR & GERAN TUs)

v04.71	10.02.2016		minor adaptations for RAN #71

v04.70	30.10.2015		minor adaptations for RAN #70

v04.69	12.08.2015		minor adaptations for RAN #69

v04.68	21.05.2015		minor adaptations for RAN #68

v04.67	01.02.2015		minor adaptations for RAN #67

v04.66	16.11.2014		minor adaptations for RAN #66

v04.65	16.08.2014		minor adaptations for RAN #65

v04.64	22.05.2014		minor adaptations for RAN #64

v04.63	24.01.2014		restructuring for RAN #63 to cover Core & Perf. in one doc file

v03.62	11.11.2013		section 1.2.3 adapted for RAN #62

v03	11.08.2013		section 1.2.3 added on time budget

v02	07.05.2010		history added, some spelling corrections

v01	13.11.2009		First version of the template