3GPP TSG RAN Meeting #94-e	RP-213366

Electronic Meeting, December 6 - 17, 2021

Source: 	Huawei, HiSilicon

Title: 	Comments on Rel-18 smart repeaters draft SID

Agenda item:	8A.1

Document for:	Decision

# Introduction

Based on the RAN Rel-18 email discussion on smart repeater post RAN#93-e [1], the moderator provided the following draft SID [2].

Moderator’s draft justification: Coverage is a fundamental aspect of cellular network deployments. Mobile operators rely on different types of network nodes to offer blanket coverage in their deployments. Deployment of regular full-stack cells is one option but it may not be always possible (e.g., no availability of backhaul) or economically viable. As a result, new types of network nodes have been considered to increase mobile operators’ flexibility for their network deployments. For example, Integrated Access and Backhaul (IAB) was introduced in Rel-16 and enhanced in Rel-17 as a new type of network node not requiring a wired backhaul. Another type of network node is the RF repeater which simply amplify-and-forward any signal that they receive. RF repeaters have seen a wide range of deployments in 2G, 3G and 4G to supplement the coverage provided by regular full-stack cells. In Rel-17, RAN4 specified RF and EMC requirements for such RF repeaters for NR targeting both FR1 and FR2. While an RF repeater presents a cost effective means of extending network coverage, it has its limitations. An RF repeater simply does an amplify-and-forward operation without being able to take into account various factors that could improve performance. Such factors may include information on semi-static and/or dynamic downlink/uplink configuration, adaptive transmitter/receiver spatial beamforming, ON-OFF status, etc. A smart repeater is an enhancement over conventional RF repeaters with the capability to receive and process side control information from the network. Side control information could allow a smart repeater to perform its amplify-and-forward operation in a more efficient manner. Potential benefits could include mitigation of unnecessary noise amplification, transmissions and receptions with better spatial directivity, and simplified network integration. Moderator’s draft objectives: The study on NR smart repeaters are to focus on the following scenarios and assumptions: - Consider smart repeaters used for extension of network coverage on FR1 and FR2 bands. Prioritize FR2 TDD deployments for both outdoor and O2I scenarios. - For only single hop stationary smart repeaters - Assuming smart repeaters are transparent to UEs - Smart repeater can maintain the gNB-repeater link and repeater-UE link simultaneously - Cost efficiency is a key consideration point for smart repeaters Study and identify which side control information is necessary for smart repeaters including at least [RAN1, RAN2, RAN4] - Beamforming information - Timing information to align transmission / reception boundaries of smart repeater - Information on UL-DL TDD configuration - Power control information for efficient interference management - ON-OFF information for efficient interference management and improved energy efficiency Study the following aspect of smart repeater management - Identification and authorization of smart repeaters [RAN2, RAN3] - RF requirements for smart repeater, including max transmission power and other applicable RF requirements [RAN4]

In this contribution, we present our analysis and comments for the draft SID.

# Discussion

## Justification

As we have commented during the email discussion [1], the difference between smart repeater and already supported network nodes such as IAB and RF repeater are still not thoroughly compared. Our brief comparison of IAB, smart repeater, and RF repeater are summarized in Table 1 [3].

Table 1 Comparisons of IAB, Smart repeater, and RF repeater

| Aspects | Node capability |  |  |
| --- | --- | --- | --- |
|  | IAB | Smart repeater | RF repeater |
| Architecture | DU + MT | RU + MT | RU |
| Maximum output power | Maximum power is the same as normal Cell | Maximum power is limited by backhaul(gNB-repeater)-access(repeater-UE) antenna isolation and amplifying gain, and is lower than a normal Cell | EIRP may be smaller than smart repeater |
| Beamforming | Same as normal Cell | Number of repeater-UE beams N and number of smart repeaters M are limited by the overall common channel/signal overhead Beamforming gain is limited | One fixed pattern beam Beamforming gain is limited |
| Signal quality | Depends only on the SINR of access link | Receiver signal includes amplified smart repeater noise The end-to-end SINR is less than the minimum SINR of backhaul and access link | Receiver signal includes amplified repeater noise The end-to-end SINR is less than the minimum SINR of backhaul and access link |
| Coverage | Easy to achieve 360° coverage | Much less than 360°: backhaul and access beams cannot be in the same direction due to the backhaul-access antenna isolation | Smaller than smart repeater |

Observation 1: Compared to an IAB node, a smart repeater is expected to have worse performance in terms of output power, beamforming gain, signal quality, and coverage area.

For smart repeater, it is not clear whether the claimed cost effective means of extending network coverage can be achieved or not compared to IAB. Firstly, both smart repeater and IAB are composed of two parts and the MT is common, as analysed in Table 1, which means that the cost of a smart repeater may be only slightly lower than that of an IAB. Secondly, the overall cost of deploying smart repeaters in the network may not be lower than that of deploying IAB nodes, because it is expected that the number of sites would be increased to achieve similar performance as deploying IAB nodes, since a smart repeater would have lower output power and thus smaller coverage compared to IAB. In addition, it is expected that the increased number of sites would increase the deployment difficulty.

Observation 2: Although the cost of a smart Repeater is expected to be lower than that of an IAB node, the overall cost of smart repeaters in the network is not expected to be lower than that of IAB nodes, because more smart repeater nodes are needed to reach similar performance.

Based on the above discussions, before sufficient evaluation and analysis are done in the study phase, it is not appropriate to conclude on whether smart repeater is a more efficient manner or not. Benefits of smart repeater need to be further evaluated in the potential study item phase also. Therefore, we propose to revise the justification as:

Proposal 1: The justification of the draft SID is revised as:

Coverage is a fundamental aspect of cellular network deployments. Mobile operators rely on different types of network nodes to offer blanket coverage in their deployments. Deployment of regular full-stack cells is one option but it may not be always possible (e.g., no availability of backhaul) or economically viable. As a result, new types of network nodes have been considered to increase mobile operators’ flexibility for their network deployments. For example, Integrated Access and Backhaul (IAB) was introduced in Rel-16 and enhanced in Rel-17 as a new type of network node not requiring a wired backhaul. Another type of network node is the RF repeater which simply amplify-and-forward any signal that they receive. RF repeaters have seen a wide range of deployments in 2G, 3G and 4G to supplement the coverage provided by regular full-stack cells. In Rel-17, RAN4 specified RF and EMC requirements for such RF repeaters for NR targeting both FR1 and FR2. While an RF repeater presents a cost effective means of extending network coverage, it has its limitations. An RF repeater simply does an amplify-and-forward operation without being able to take into account various factors that could improve performance. Such factors may include information on semi-static and/or dynamic downlink/uplink configuration, adaptive transmitter/receiver spatial beamforming, ON-OFF status, etc. A smart repeater is an enhancement over conventional RF repeaters with the capability to receive and process side control information from the network. Side control information could might allow a smart repeater to perform its amplify-and-forward operation in a more efficient manner. Potential benefits could might include mitigation of unnecessary noise amplification, transmissions and receptions with better spatial directivity, and simplified network integration.

## Objectives

Based on the previous discussions, the potential side control information of a smart repeater proposed by companies include beamforming, timing, power control, and ON-OFF [2].

### Comments on scenarios and assumptions

We still think that the study should focus on FR2, then can be further discussed whether it is beneficial to extend to other application scenarios. If even for FR2 the gain is very marginal, then there is no point to study other application scenarios like FR1.

In addition, beamforming is claimed as one of the key enabling technologies for smart repeater. However, although beamforming improves the smart repeater performance, the common channel/signal overhead is increased and thus the overall system performance might be degraded. Firstly, for every smart repeater attached to a gNB, the repeater-UE beam sweeping should be driven by base station, and the beam sweeping will occupy the time domain resources of the gNB, which will result in that the beam sweeping overhead (e.g., SSB/PRACH and beam training for repeater-UE link) linearly increases with the number of repeater-UE beams N, although the coverage performance increases with N. Moreover, the overall overhead for a gNB linearly scales with the number of attached smart repeaters M and number of repeater-UE beams N, thus a larger M or N may lead to overall system performance degradation. Therefore, the overall performance benefits of smart repeater should be thoroughly studied considering different numbers of smart repeaters M and the repeater-UE beams N.

Based on the above discussion, the following proposal is given:

Proposal 2: Take the following changes on scenarios and assumptions in the objectives:

The study on NR smart repeaters are to focus on the following scenarios and assumptions: - Consider smart repeaters used for extension of network coverage on FR1 and FR2 bands. Prioritize FR2 TDD deployments for both outdoor and O2I scenarios. - For only single hop stationary smart repeaters - Assuming smart repeaters are transparent to UEs - Smart repeater can maintain the gNB-repeater link and repeater-UE link simultaneously - Cost efficiency is a key consideration point for smart repeaters. Spectral efficiency should be taken into account as well.

### Comments on objective for side control information

Sufficient evaluations should be performed in the study phase for all the potential side control information listed in the objective, in order to justify the necessity of a potential side control information and also simultaneously to show whether smart repeater is beneficial or not. In general, at this stage the corresponding objective looks fine, however we don’t see the necessity to involve RAN2 in the study phase.

Proposal 3: Take the following change on the objective for side control information:

Study and identify which side control information is necessary for smart repeaters including at least [RAN1, RAN2, RAN4] - Beamforming information - Timing information to align transmission / reception boundaries of smart repeater - Information on UL-DL TDD configuration - Power control information for efficient interference management - ON-OFF information for efficient interference management and improved energy efficiency

### Comments on objective for smart repeater management

In order to check the feasibility/benefits for smart repeater, RAN4 needs to be involved to study RF requirements, e.g. the maximum transmission power. Therefore, we think the bullet “RF requirements for smart repeater, including max transmission power and other applicable RF requirements” is necessary to be kept here. The evaluations on the side control information need to consider the maximum transmission power.

Regarding the bullet “Identification and authorization of smart repeaters”, as discussed during the previous email discussion, there may be need of coordination with SA3. Therefore, we propose to add the following note:

Proposal 4: Take the following change on the objective for smart repeater management:

Study the following aspect of smart repeater management - Identification and authorization of smart repeaters [RAN2, RAN3] Note: coordination with SA3 may be needed. - RF requirements for smart repeater, including max transmission power and other applicable RF requirements [RAN4]

# Conclusions

In this contribution, we discuss the Rel-18 smart repeater draft SID, and have the following observations and proposals:

Observation 1: Compared to an IAB node, a smart repeater is expected to have worse performance in terms of output power, beamforming gain, signal quality, and coverage area.

Observation 2: Although the cost of a smart Repeater is expected to be lower than that of an IAB node, the overall cost of smart repeaters in the network is not expected to be lower than that of IAB nodes, because more smart repeater nodes are needed to reach similar performance.

Proposal 1: The justification of the draft SID is revised as:

Coverage is a fundamental aspect of cellular network deployments. Mobile operators rely on different types of network nodes to offer blanket coverage in their deployments. Deployment of regular full-stack cells is one option but it may not be always possible (e.g., no availability of backhaul) or economically viable. As a result, new types of network nodes have been considered to increase mobile operators’ flexibility for their network deployments. For example, Integrated Access and Backhaul (IAB) was introduced in Rel-16 and enhanced in Rel-17 as a new type of network node not requiring a wired backhaul. Another type of network node is the RF repeater which simply amplify-and-forward any signal that they receive. RF repeaters have seen a wide range of deployments in 2G, 3G and 4G to supplement the coverage provided by regular full-stack cells. In Rel-17, RAN4 specified RF and EMC requirements for such RF repeaters for NR targeting both FR1 and FR2. While an RF repeater presents a cost effective means of extending network coverage, it has its limitations. An RF repeater simply does an amplify-and-forward operation without being able to take into account various factors that could improve performance. Such factors may include information on semi-static and/or dynamic downlink/uplink configuration, adaptive transmitter/receiver spatial beamforming, ON-OFF status, etc. A smart repeater is an enhancement over conventional RF repeaters with the capability to receive and process side control information from the network. Side control information could might allow a smart repeater to perform its amplify-and-forward operation in a more efficient manner. Potential benefits could might include mitigation of unnecessary noise amplification, transmissions and receptions with better spatial directivity, and simplified network integration.

Proposal 2: Take the following changes on scenarios and assumptions in the objectives:

The study on NR smart repeaters are to focus on the following scenarios and assumptions: - Consider smart repeaters used for extension of network coverage on FR1 and FR2 bands. Prioritize FR2 TDD deployments for both outdoor and O2I scenarios. - For only single hop stationary smart repeaters - Assuming smart repeaters are transparent to UEs - Smart repeater can maintain the gNB-repeater link and repeater-UE link simultaneously - Cost efficiency is a key consideration point for smart repeaters. Spectral efficiency should be taken into account as well.

Proposal 3: Take the following change on the objective for side control information:

Study and identify which side control information is necessary for smart repeaters including at least [RAN1, RAN2, RAN4] - Beamforming information - Timing information to align transmission / reception boundaries of smart repeater - Information on UL-DL TDD configuration - Power control information for efficient interference management - ON-OFF information for efficient interference management and improved energy efficiency

Proposal 4: Take the following change on the objective for smart repeater management:

Study the following aspect of smart repeater management - Identification and authorization of smart repeaters [RAN2, RAN3] Note: coordination with SA3 may be needed. - RF requirements for smart repeater, including max transmission power and other applicable RF requirements [RAN4]

# References

- RP-212663, RAN94e-R18Prep-03, Moderator's summary for discussion on [RAN94e-R18Prep-03] Additional topological improvements - smart repeater, RAN1 Chair (Samsung).

- RP-212703, New SID on NR Smart Repeaters, 8A.1, RAN1 Chair (Samsung).

- RP-212279, “Views on smart repeaters”, Huawei, HiSilicon, RAN#93 e-meeting, Sept. 2021.