3GPP TSG RAN WG #94-e		     RP-213195

e-Meeting, Dec. 06th – Dec. 17th, 2021

Agenda item:	8A.5

Source: 	Reliance JIO, Radisys, WiSig, Tejas Networks, Saankhya Labs, IIT-M, CEWiT

Title: 	Handling Machine Emergency

Document for:	Discussion / Decision

# Introduction

With Industrial IOT and an ambition to achieve closer integration of factory machines in a cellular network setup will need support for emergencies that may arise in a factory / industrial setup. The existing emergency definition and handling mechanism in 3GPP caters to human emergencies and does not address machine emergencies. The introduction of IoT and especially IIoT (Industrial IoT) opens numerous types of emergencies that will have to be handled differently from human emergencies. If 3GPP plans to address the issues around IIOT, the first thing to handle is the issue of “Machine Emergency”, we must identify different types of machine emergencies and have mechanisms to handle such emergencies.

Machine emergency will have to be handled at all levels of the network including RAN and the core network. Machine emergency could be related to a sensor / device, a piece of machinery critical to the plant, a production line or the plant itself (combination of multiple factors). Machine emergencies may differ based on the vertical in which a cellular (public / Private) network is deployed such as Industry 4.0, Railways, Power Grids etc.

It must be understood that if a 5G cellular network must be used in an IIOT scenario, there are local government and specific enterprise “emergency handling requirements” that may have to be adhered to.

It is important that 3GPP initiates a study of such possible emergencies and understand the support necessary in the RAN system and in the Core network.

# Discussion

## Why do we need Machine Emergency?

Handling emergencies in a 3GPP network has so far meant handling human emergencies. The nature of handling human emergencies is very different from that of handling machine emergencies.

IIoT refers to the extension and use of the internet of things (IoT) in industrial sectors and applications. With a strong focus on machine-to-machine (M2M) communication, big data, and machine learning. IIoT enables industries and enterprises to have better efficiency and “reliability” in their operations. The reliability aspect can be addressed because of the possibility to access inputs from multiple devices / sensors and formulate if there is an emergency in an IIOT scenario.

IIoT encompasses industrial applications such as robotics, factories plants and machinery, medical deployments / plants etc. IIoT goes beyond consumer devices and is associated with the internetworking of physical devices usually associated with the IoT. IIOT is distinct as it is an intersection of information technology (IT) and operational technology (OT). OT refers to the networking of operational processes and industrial control systems (ICSs), including human machine interfaces (HMIs), supervisory control and data acquisition (SCADA) systems, distributed control systems (DCSs), and programmable logic controllers (PLCs). Such a system can help industries to grow tremendously due to the provision of data availability helping in decisions holistically. IIOT is also expected to help in providing data automatically for the health of the overall system. For ex: in a manufacturing plant, sensors can help tell us if there are any harmful gas leaks, water leaks or machine overload / failure points and needs a shutdown. Such situations are can be termed as “Machine emergency” in general. Such emergency sessions are the most fundamental and critical services offered by telecommunications networks in an IIOT deployment.

Observation 1: Machine Emergencies and the regulatory aspects that govern such emergency handling are different from Human emergencies.

## What is the difference between Human emergency and Machine Emergency?

Human emergency call is initiated by a human user via a mobile terminal. This call may be routed by the IMS to the appropriate Emergency servers based on the Mobile Network Operator’s configurations on handling Emergency call. There may also be country specific regulatory requirement for emergency call handling. Once the emergency call is received by the concerned person at the emergency handling site, the call is typically a two way conversations link with appropriate QoS handling.

Below is an architecture and example call flow to handle emergency in the existing system –

Incase of machine emergency call, there may be no human intervention. Hence the regular routing of human emergency call at the IMS may not work here. The machine emergency call needs to be routed to the appropriate enterprise / private servers or a multitude of emergency handling end points including machine / human joint handling.

It is important that these aspects are studied under a 3GPP RAN item as recent trends seem to suggest that regulators are keen to understand if an IIOT supporting network supports multiple granularities of emergency handling rather than just human emergency.

Observation 2: “Machine emergency” itself may have multiple levels of granularity which needs a study.

It must be understood that IIOT deployments are also governed by regulatory / enterprise concerns (some still under formulation) and some of such regulations needs an IIOT network to support capability to handle different granularities of Industrial emergencies.

Observation 3: “Machine emergency” support may be necessary by regulation in an IIOT deployment. So far such handling (where supported) is proprietary.

## What RAN and Core aspects needs to be considered for Machine Emergency?

The Machine emergency and human emergency call needs to be differentiated at the RAN first. This is required so that RAN can make appropriate admission control and send appropriate Ng message to the core for handling of the emergency call.

When the machine emergency call is requested to be served by the Core Network, Core network needs to setup the appropriate PDU sessions to treat the emergency call. There may be different classes/types of machine emergency. Core Network needs to choose the right QoS flow to treat different classes of machine emergency.

Observation 4: Supporting “Machine emergency” types will have both RAN and SA impacts. RAN impact is to identify and differentiate possible different “machine emergency” types. SA impact is to provide appropriate routing support with appropriate QoS flows.

Assertion: It must be understood that Non-Support of such native machine emergency handling in a 3GPP network may prohibit cellular operators to bid for such IIOT contracts (Public or Private).

Based on the above discussions and observations our proposal is the below –

Proposal 1: A Study Item is needed to understand “Machine Emergency” and its handling in both RAN and SA in Rel-18.

# Conclusions

In this paper, we discussed the necessary for machine emergency and RAN and SA impacts for machine emergency. The following are our Observations and Proposals -

Observation 1: Machine Emergencies and the regulatory aspects that govern such emergency handling are different from Human emergencies.

Observation 2: “Machine emergency” itself may have multiple levels of granularity which needs a study.

Observation 3: “Machine emergency” support may be necessary by regulation in an IIOT deployment. So far such handling (where supported) is proprietary.

Observation 4: Supporting “Machine emergency” types will have both RAN and SA impacts. RAN impact is to identify and differentiate possible different “machine emergency” types. SA impact is to provide appropriate routing support with appropriate QoS flows.

Proposal 1: A Study Item is needed to understand “Machine Emergency” and its handling in both RAN and SA in Rel-18.

# Reference

- https://www.3gpp.org/news-events/partners-news/1648-emergency

- https://www.controleng.com/articles/scada-can-accelerate-automation-iiot-implementations/