3GPP TSG RAN Meeting #94-e							        RP-213208

Electronic Meeting, December 6 - 17, 2021

Source: 	    CMCC

Title: 	Discussion on SID on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface

Agenda item:	8.6.1

Document for:	Discussion

- Introduction

The project of AI/ML for Air Interface has been discussed in Rel-18 workshop and subsequent email discussions, and the draft SID was provided in RP-212708[1]. During the email discussion, the use cases for the candidate project on AI/ML for Air Interface were heavily debated [2]. Five use cases were listed as the initial set of use cases to identify a common AI/ML framework, and use cases for characterization and baseline performance evaluations will be selected from the initial set of use cases. The related descriptions from the SID are listed below [1].

Use cases to focus on: - Initial set of use cases includes: - CSI feedback enhancement, e.g., overhead reduction, improved accuracy, prediction [RAN1] - Beam management, e.g., beam prediction in time, and/or spatial domain for overhead and latency reduction, beam selection accuracy improvement [RAN1] - Positioning accuracy enhancements for different scenarios including, e.g., those with heavy NLOS conditions [RAN1] - RS overhead reduction [RAN1] - RRM Mobility, e.g., prediction in time or frequency for robustness, interruption and overhead reduction [RAN2] - Finalize representative set of use cases (reduced from the initial set and minimizing sub use cases) for characterization and baseline performance evaluations

In this contribution, we provide our views of use cases for the SID of AI/ML for Air Interface, and the updated objectives are attached.

- Discussion

For the draft SID of AI/ML for Air Interface, five use cases were listed as the initial set of use cases. For each use case, there may be different sub use cases and categories. For example, for CSI feedback enhancement, it includes payload reduction, RS overhead reduction, accuracy improvement and CSI prediction. Even for each sub use cases, there can be different levels of collaboration between UE and gNB for AI/ML deployment. Considering the workload, it is unrealistic to have a thorough and comprehensive discussion for all the five use cases. Instead of discussing the use cases in a general way, we prefer to focus on two use cases for detailed discussion and evaluation. Given this is the first study of AI/ML for Air Interface in 3GPP, we have to say that the scope to be discussed for each use cases is not totally clear. We think it is not efficient to investigate too many use cases in Rel-18. To use the TU more efficiently, the discussion of framework and lifecycle management of AI model can be based on two use cases, and we can focus on the dataset, evaluation methodology and potential spec impact for each use cases. The framework and evaluation methodology of this project can provide a foundation and reference for other use cases in future Release of AI/ML based Air Interface study.

For the five listed use cases, we think CSI feedback and positioning are two important use cases. For beam management, it is not essential, especially for FR1. For RS overhead reduction, we think other use cases, for example CSI feedback enhancement, may include RS overhead reduction, hence it is unnecessary to study RS overhead reduction as a standalone use case. For RRM Mobility, it is a RAN-2 lead study, and RAN2 already has a WI on mobility enhancement in Rel-18. We prefer to study AI based RRM Mobility enhancement in future-release. Therefore, we think CSI feedback enhancement and Positioning accuracy enhancement can be selected for detailed study and evaluations for the project of AI/ML for Air Interface.

Proposal 1: It is proposed to select CSI feedback enhancement and Positioning accuracy enhancement as the representative use cases for characterization and baseline performance evaluations.

Modification in the SID text description

Study the 3GPP framework for AI/ML for air interface corresponding to each target use case regarding aspects such as performance, complexity, and potential specification impact. Use cases to focus on: - Initial set of use cases for characterization and evaluation includes: - CSI feedback enhancement, e.g., overhead reduction, improved accuracy, prediction [RAN1] - Positioning accuracy enhancements for different scenarios including, e.g., those with heavy NLOS conditions [RAN1] AI/ML model and description to identify common and specific characteristics for framework investigations: - Characterize the defining stages of AI/ML related algorithms and associated complexity: - Model generation, e.g., model training (including input/output, pre-/post-process, as applicable), model validation, model testing, as applicable - Inference operation, e.g., input/output, pre-/post-process, as applicable - Identify various levels of collaboration between UE and gNB, e.g., - No collaboration: implementation-based only AI/ML algorithms without information exchange [for comparison purposes] - Various levels of UE/gNB collaboration targeting at separate or joint ML operation. - Identify lifecycle management of AI/ML model: e.g., model deployment (initiation/configuration), model monitoring, model updating, and model transfer - Data set for training, inference, validation, and testing - Identify common notation and terminology for AI/ML related functions, procedures and interfaces - Consider the work done for FS_NR_ENDC_data_collect as and when appropriate For the use cases under consideration: - Evaluate performance benefits of AI/ML based algorithms: - Methodology based on statistical models (from TR 38.901 and TR 38.857 [positioning]), for link and system level simulations. - Extensions of 3GPP evaluation methodology for better suitability to AI/ML based techniques should be considered as needed. - Whether field data are needed to further assess the performance and robustness in real-world environments should be discussed as part of the study. - User data privacy needs to be preserved. - Need for common dataset construction for training, validation and test for the selected use cases - Consider adequate model training strategy and associated implications, e.g., offline training vs. online training of models. - KPIs: Determine the common KPIs and corresponding requirements for the AI/ML operations. Determine the use-case specific KPIs and benchmarks of the selected use-cases. - Performance and computational complexity of AI/ML based algorithms should be compared to that of a state-of-the-art (non-AI/ML and/or implementation-based AI/ML) baseline - Overhead, power consumption (including computational), memory storage, and hardware requirements (including for given processing delays) associated with enabling respective AI/ML scheme, as well as generalization capability should be considered and documented. The need to define Typical AI model(s) for calibration shall be discussed as part of this study. - Assess potential specification impact, specifically for the agreed use cases in the final representative set and for a common framework: - PHY layer aspects including (RAN1) - Consider aspects related to, e.g., the specification of the AI Model lifecycle management, and dataset construction for training, validation and test for the selected use cases - Use case and collaboration level specific specification impact, such as new signalling, assistance information, measurement, and feedback - Protocol aspects including (Except use case study, RAN2 only start following general assessment after there is sufficient progress on use study in RAN1) - Consider aspects related to, e.g., capability indication, configuration procedures (training/inference), validation and testing procedures, and management of data and AI/ML model - Collaboration level specific specification impact per use case including signalling design to support the collaboration identified in RAN1 - Interoperability and testability aspects (RAN4 only start the work after there is sufficient progress on use case study in RAN1 and RAN2) - UE and gNB requirements and testing frameworks to validate AI/ML based performance enhancements and ensuring that UE and gNB with AI/ML meet or exceed the existing minimum requirements - Consider the need and implications for AI/ML processing capabilities definition Note: specific AI/ML models are not expected to be specified and are left to implementation.

- Conclusion

Proposal 1: It is proposed to select CSI feedback enhancement and Positioning accuracy enhancement as the representative use cases for characterization and baseline performance evaluations.

- Reference

- RP-212708, New SID on AI/ML for NR Air Interface, Moderator (Qualcomm), 3GPP TSG RAN Meeting #94e, Electronic Meeting, Dec. 6 - 17, 2021.

- RP-212668, Moderator's summary for discussion [RAN94e-R18Prep-08] AI (Artificial Intelligence)/ML (Machine Learning) for Air interface, 3GPP TSG RAN Meeting #94e, Electronic Meeting, Dec. 6 - 17, 2021.