3GPP TSG RAN Meeting #94-e	RP-213221

Electronic Meeting, Dec 06-17, 2021

Source:	Huawei, HiSilicon

Title:	Comments on Rel-18 mobility draft WID

Agenda item:	8A.2

Document for:	Discussion and Decision

Introduction

In Rel-18 email discussion for RAN#94e preparation, the Rel-18 mobility was discussed [1] and a draft WID [2] was provided by moderator. According to the draft WID, the following objectives are provided by moderator.

- To specify mechanism and procedures of L1/L2 based inter-cell mobility for mobility latency reduction: - Configuration and maintenance for multiple candidate cells to allow fast application of configurations for candidate cells [RAN2, RAN3] - Dynamic switch mechanism among candidate serving cells (including SpCell and SCell) for the potential applicable scenarios based on L1/L2 signalling [RAN2, RAN1] - L1 enhancements, including inter-cell beam management, L1 measurement and reporting, beam indication, and for non-synchronized scenario to handle TA management [RAN1, RAN2] - CU-DU interface signaling to support L1/L2 mobility, if needed [RAN3] Note 1: FR2 specific enhancements are not precluded, if any. Note 2: The procedure of L1/L2 based inter-cell mobility are applicable to the following scenarios: - Standalone, CA and NR-DC case with serving cell change within one CG - Intra-CU case and intra-CU inter-DU case (applicable for Standalone and CA) - Both intra-frequency and inter-frequency - Both FR1 and FR2 - To specify mechanism and procedures of MR-DC with selective activation of the cell groups via L3 enhancements: - To allow subsequent CPC/CPAC after changing SCG without reconfiguration and re-initiation of CPC/CPAC [RAN2, RAN3, RAN4] - TBD: whether Rel-17 CPC/CPAC mechanism is used as the baseline - TBD: whether to limit MR-DC to EN-DC and NR-DC - To specify CHO including target MCG and target SCG if it cannot be completed in Rel-17 [RAN3, RAN2] - To specify CHO including target MCG and candidate SCG for CPC/CPAC [RAN3, RAN2] - CHO including target MCG and target SCG is used as the baseline

According to the summary [1] provided by the moderator, most of companies are fine with the motivations and scopes of objective#1 and objective#4. In this contribution, we will mainly focus on the objective#2 and #3, and some wording improvement in objective#1.

Discussion

Objective#1

For the objective#1, we are in general fine to the scope of the objective. There is one typo on the Note 2. The “intra-CU case and intra-CU inter-DU case” should be “intra-DU case and intra-CU inter-DU case”. For the case of intra-CU inter-DU case, we understand the solution is based on existing interface and the enhancements should not introduce new RAN interfaces.

Objective#2

For the MR-DC with selective activation of the cell groups via L3 enhancements, according to the email discussion, the motivations are to reduce the interruption time and signalling overhead for the SCG change.

In R17 inter-SN CPC, RAN can configure multi candidate SNs for the UE. The UE can select the suitable SNs based on the configured execution condition. RAN3 also introduced the early data forwarding for the inter-SN CPC. Therefore we think the R17 inter-SN CPC can be used as the baseline. The main enhancement for the MR-DC with selective activation of the cell groups for RAN2 is that the UE can store the previous received candidate SCG configuration after successfully completion of accessing to the candidate SCG. The impact on RAN3 is to enhance storage of the SCG configurations based on the current CPC/CPAC mechanism, .e.g. the MN does not need send the SN release message to the source and other candidate SNs after the UE accesses to the target SN. The overall changes are small.

In the email discussion [1], some companies propose to include the option that the network triggers the SCG change. In our understanding, the only difference is who will trigger the SCG change. In the solution of SCG change triggered by the network, RAN2 needs to discuss how to send the prepared SCG configurations of candidate SNs to the UE, and also needs to discuss how to send the SCG change command to the UE after the MN or source SN decides to change the SCG. In RAN3, in order to reduce the interruption time, the network needs to prepare the candidate SN resources before the SCG change and also needs to perform the early/late data forwarding procedure. In Rel-17 CPAC, RAN3 has already spent a long time to discuss these procedures in Xn/X2/F1/E1 interfaces. If this is not reused, RAN3 needs to spend a long time again to discuss how to design the procedures in these interfaces, including the preparation and modification of the candidate SN resources, early/late data forwarding. In addition, RAN3 needs to introduce new procedures for the MN and the source SN to trigger the SCG change after the preparation of the candidate SNs. Therefore we think this solution will increase the work of RAN2 and RAN3 unnecessarily.

In R17, RAN3 and RAN2 have agreed to support the CPAC only for the EN-DC and NR-DC. However considering the scenarios are mainly targeting NR FR1+FR2 combinations, NR-DC seems the most relevant case to be supported. If workload needs to be considered, we can limit the MR-DC to NR-DC case.

In summary, for the objective#2, we suggest to reuse the Rel-17 CPC/CPAC mechanism and limit the MR-DC to NR-DC.

Objective#3

In the last meeting of RAN3#114, RAN3 has agreed to assign a dedicated TEI17 agenda for the CHO including target MCG and target SCG, and also RAN3 has agreed to add a CHO indicator in the SN addition request and SN modification request message. In our estimation, RAN3 can finish the specification of CHO including target MCG and SCG in R17. In RAN2’s specifications, there is one limitation that the RRCReconfiguration message contained in condRRCReconfig cannot contain the configuration for target SCG for CHO. In our understanding, RAN2 only need to remove this limitation for the CHO with SCG configuration. Therefore RAN2 also can finish the specification in R17. In this case we think objective#3 can be removed.

In summary, we propose to revise the objectives of the draft WID as below:

- To specify mechanism and procedures of L1/L2 based inter-cell mobility for mobility latency reduction: - Configuration and maintenance for multiple candidate cells to allow fast application of configurations for candidate cells [RAN2, RAN3] - Dynamic switch mechanism among candidate serving cells (including SpCell and SCell) for the potential applicable scenarios based on L1/L2 signalling [RAN2, RAN1] - L1 enhancements, including inter-cell beam management, L1 measurement and reporting, beam indication, and for non-synchronized scenario to handle TA management [RAN1, RAN2] - CU-DU interface signaling to support L1/L2 mobility, if needed [RAN3] Note 1: FR2 specific enhancements are not precluded, if any. Note 2: The procedure of L1/L2 based inter-cell mobility are applicable to the following scenarios: - Standalone, CA and NR-DC case with serving cell change within one CG - Intra-DU case and intra-CU inter-DU case (applicable for Standalone and CA) - Both intra-frequency and inter-frequency - Both FR1 and FR2 Note 3: The procedure of L1/L2 based inter-cell mobility reuses the existing RAN interfaces. - To specify mechanism and procedures of MR-DC with selective activation of the cell groups via L3 enhancements: - To allow subsequent CPC/CPAC after changing SCG without reconfiguration and re-initiation of CPC/CPAC [RAN2, RAN3, RAN4] - Rel-17 CPC/CPAC mechanism is used as the baseline - Limit MR-DC to NR-DC - To specify CHO including target MCG and candidate SCG for CPC/CPAC [RAN3, RAN2] - CHO including target MCG and target SCG is used as the baseline

Conclusions

In this contribution, potential objectives for Rel-18 mobility are discussed, where we have provided the revised draft WID as following:

- To specify mechanism and procedures of L1/L2 based inter-cell mobility for mobility latency reduction: - Configuration and maintenance for multiple candidate cells to allow fast application of configurations for candidate cells [RAN2, RAN3] - Dynamic switch mechanism among candidate serving cells (including SpCell and SCell) for the potential applicable scenarios based on L1/L2 signalling [RAN2, RAN1] - L1 enhancements, including inter-cell beam management, L1 measurement and reporting, beam indication, and for non-synchronized scenario to handle TA management [RAN1, RAN2] - CU-DU interface signaling to support L1/L2 mobility, if needed [RAN3] Note 1: FR2 specific enhancements are not precluded, if any. Note 2: The procedure of L1/L2 based inter-cell mobility are applicable to the following scenarios: - Standalone, CA and NR-DC case with serving cell change within one CG - Intra-DU case and intra-CU inter-DU case (applicable for Standalone and CA) - Both intra-frequency and inter-frequency - Both FR1 and FR2 Note 3: The procedure of L1/L2 based inter-cell mobility reuses the existing RAN interfaces. - To specify mechanism and procedures of MR-DC with selective activation of the cell groups via L3 enhancements: - To allow subsequent CPC/CPAC after changing SCG without reconfiguration and re-initiation of CPC/CPAC [RAN2, RAN3, RAN4] - Rel-17 CPC/CPAC mechanism is used as the baseline - Limit MR-DC to NR-DC - To specify CHO including target MCG and candidate SCG for CPC/CPAC [RAN3, RAN2] - CHO including target MCG and target SCG is used as the baseline

Reference

- RP-212670, Moderator’s summary for discussion [RAN94e-R18Prep-10] Mobility enhancements

- RP-212710, New WID: NR further mobility enhancements