3GPP TSG RAN meeting #94-e								RP-213247

Electronic Meeting, December 6-17, 2021

Agenda Item:	9.2.1

Source:	Ericsson

Title:	Regulatory updates on 5.925-7.125 GHz frequency range

Document for:	Discussion

# Introduction

The RAN-led study item [1] on 6 GHz band for LTE and NR was updated in the last RAN.

This contribution is relating what happened since last RAN#93-e meeting, with major updates related to the IMT identification of the band the 6.425 – 7.125 GHz band.

# Discussion

## Europe and the 6.425 – 7.125 GHz band

Radio Spectrum Policy Groups has provided an Interim Opinion on WRC-23 ([2]), and more particularly on Agenda Item 1.2, which considers IMT identification of 6.425-7.025 GHz (Region 1) and 7.025-7.125 GHz (globally).

RSPG recognizes that this band may respond to additional spectrum demand in mid-band since it has similar propagation conditions to the 5G pioneer band 3 400 - 3800 MHz. But it also mentions that there is a need to study compatibility with incumbent services and those in adjacent bands

## Africa and the 6.425 – 7.125 GHz band

In its report of the 2nd preparatory meeting for WRC-23 ([3]), the UAT (Union africaine des télécommunications) or ATU supported, as a preliminary decision, to identify the 6.425-7.125 GHz band for IMT, taking into account ITU coexistence studies outcomes.

The UAT will also organize a survey with its members to identify existing services in this band.

## RCC and the 6.425 – 7.125 GHz band

In its coming meeting (6-10 December), RCC will consider for approval the draft Recommendation ([4]) related to the harmonized conditions for 5G-NR / IMT-2020 systems in the RCC countries in the frequency band 6425-7125 MHz.

The draft recommendation includes the following elements:

- This TDD band would consist of 7 reference blocks of 100 MHz, each block might be further reduced to ensure compatibility with other systems, still optimizing spectrum utilization.

- The unwanted emissions of 5G-NR / IMT-2020 base stations and user equipment must be in compliance with the Category B limits for stations in the mobile service, as provided in Recommendation ITU-R SM.329-12.

# Conclusions

This contribution provides an update of current status and next steps related to Regulatory work on 6 GHz band in the world.

# References

- RP-182312, SID on 6 GHz for LTE and NR in Licensed and Unlicensed Operations, Ericsson

- RSPG21-031 FINAL, RADIO SPECTRUM POLICY GROUP, Interim Opinion on WRC-23

- RPA23-2, Rapport de la 2ème réunion préparatoire africaine en vue de la conférence mondiale des radiocommunications 2023, 6-10 septembre 2021.

- [4]	Draft RCC Recommendation - 1.3_П1_17_заседание_РГ_РЧС.zip (yandex.ru)