3GPP TSG RAN meeting #94-e									      RP-213348

Electronic Meeting, December 6 - 17, 2021

## Status Report to TSG

Agenda item: 9.3.2.3 UE power saving enhancements for NR [RAN2 WI: NR_UE_pow_sav_enh]

| WI / SI Name | UE power saving enhancements for NR |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: - | Core part: Yes | Performance part: No |  | Testing part: - |
| Acronym | NR_UE_pow_sav_enh |  |  |  |  |
| Unique ID | 860047 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-212630 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: - | Core part: 03/2022 | Performance part: 09/2022 | Testing part: - |  |
| Overall Completion level | Study Item: - | Core part: Overall: 85% | Performance Part: Overall: 0% | Testing part: - |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | RAN2 |
| --- | --- | --- |
| Rapporteur | Name | Weide Wu |
|  | Company | MediaTek Inc. |
|  | Email | weide.wu@mediatek.com |

## 1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table: N/A

## 2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

## 2.1	RAN1

#### 2.1.1	Agreements

Relating to scope item 1) – a): - Specify enhancements for idle/inactive-mode UE power saving, considering system performance aspects [RAN2, RAN1] - Study and specify paging enhancement(s) to reduce unnecessary UE paging receptions, subject to no impact to legacy UEs [RAN2, RAN1] - NOTE: RAN1 to check and update, if needed, evaluation methodology in RAN1 #102-e meeting The following agreements are achieved:

RAN #93-e meeting Conclusion - Support PDCCH-based PEI as the only option • Only essential function for PEI is support • New DCI format • Higher layer configuration, including SS • Details of the procedures of PEI monitoring, and identification of MOs before PO • Only Behv-A (per RAN1#104e agreement) is supported • If TRS availability indication is agreed to be supported in both paging DCI and the DCI format for PEI, same mechanism/principle for TRS availability indication is adopted for the two DCI formats • Supporting TRS availability indication in DCI format for PEI shall not delay the completion of essential functionality of PEI

RAN1 #106-bis-e meeting Agreement For NR Rel-17, paging indications to UE subgroups are carried only in PEI. Agreement For PEI, a new DCI format is supported to include at least paging indications to UE group(s)/subgroups of the associated PO(s) - One bit in the DCI payload indicating one UE subgroup of a PO or one UE group/PO - The maximum number of total bits for paging indication field in PEI DCI format is x - One PEI can be configured to indicate up to 4 PO(s) in a PF - FFS whether to supporting map PEI to 3 POs in a PF - FFS: 1 PEI for POs across multiple PFs - FFS: value of x Agreement A PEI occasion (PEI-O) is a set of S consecutive PDCCH monitoring occasions when nrofPDCCH-MonitoringOccasionPerSSB-InPO is not configured - S is the number of actual transmitted SSBs determined according to ssb-PositionsInBurst in SIB1 - The K-th PDCCH monitoring occasion for PEI in the PEI-O has the same QCL assumption as that of the K-th PDCCH monitoring occasion for paging in the PO. - Note: QCL reference is SSB - FFS: Determination of the PEI-O location - FFS: Support of unlicensed spectrum operation with nrofPDCCH-MonitoringOccasionPerSSB-InPO configured Agreement CORESET # 0 or commonControlResourceSet in SIB1 can be used for PEI - Note: The number of CORESETs configured for a UE follows the requirement of UE feature 3-1 Agreement Support configuration of a dedicated search space (‘peiSearchSpace’) for PEI - FFS: Configuration details and whether and how to reuse legacy search space sets, including pagingSearchSpace and searchSpaceSetZero Agreement Determination of PEI-O location for a target PO is based on one of the following alternatives: - Alt 1: The first PDCCH monitoring occasion of the PEI-O is provided w.r.t. the start of a reference frame determined by a frame-level offset to the PF of the target PO - FFS: The unit and the range of the frame-level offset - FFS: The unit and the range of the configuration for the first PDCCH monitoring occasion (e.g., to be the same as those of firstPDCCH-MonitoringOccasionOfPO) - Alt 2: The first PDCCH monitoring occasion of the PEI-O is provided w.r.t. the L-th SS burst before the first PDCCH monitoring occasion of the target PO. - FFS: the case that a SSB burst overlaps in time with the target PO - FFS: L = 1, 2 or 3 - FFS: Reference the “start” or “end” of the L-th SS burst - FFS: The unit and the range of the configuration for the first PDCCH monitoring occasion - Alt 3: The first PDCCH monitoring occasion of the PEI-O is provided by a time offset w.r.t. a reference time for the target PO. - FFS: The exact definition of the reference time, e.g. the first MO of the target PO, the first MO of the first PO indicated by the PEI, the start of the PF for the target PO - FFS: The unit and the range of the time offset - FFS: Whether any SS burst or TRS burst is needed between PEI-O and PO - Configuration for one PEI indicating multiple POs within a PF should be taken into consideration in the determination of PEI occasion Decide one of the above alternatives or a single merged solution based on the alternatives in RAN1#107-e meeting. FFS: Extension for the case one PEI indicates multiple POs across multiple PFs, if supported [106bis-e-NR-R17-PowSav-04] – Weide (MediaTek) Discuss incoming LS in x8716 for a possible reply LS R1-2108716 LS on UE Power Saving RAN2, MediaTek R1-2110607 [Draft] Reply LS on UE Power Saving Moderator (MediaTek) Decision: As per email decision posted on Oct 21st, the draft LSis endorsed in principle. Final version is approved in R1-2110608.

RAN1 #107-e meeting Agreement For unlicensed operation, A PEI-O is a set of 'S*X ' consecutive PDCCH monitoring occasions where 'S' is the number of actual transmitted SSBs determined according to ssb-PositionsInBurst in SIB1 and X is the nrofPDCCH-MonitoringOccasionPerSSB-InPO if configured or is equal to 1 otherwise. The [x*S+K]th PDCCH monitoring occasion for PEI in the PEI-O corresponds to the Kth transmitted SSB, where x=0,1,…,X-1, K=1,2,…,S. - If X > 1, when the UE detects a PEI within its PEI-O, the UE is not required to monitor the subsequent monitoring occasion(s) associated with the same PEI-O Note: The QCL reference is SSB Agreement Support mapping one PEI to POnumPerPEI PO(s) in one or multiple PF(s)  POnumPerPEI is a factor of (total PO number in a paging cycle) and configurable via SIB for the cell with the value range of {1, 2, 4, 8} - The Maximum number of PF associated with one PEI is up to 2  Note: Maximum number of paging indication bits in DCI format 2_7 can be kept the same for any configuration of POnumPerPEI, e.g., by applying a smaller subgroupsNumPerPO and a larger POnumPerPEI.  Note: Larger value of POnumPerPEI can reduce the average PEI overhead per PO, but there can also cause potentially larger paging latency and larger UE power consumption due to longer UE wake-up time before PO monitoring, which can be significant with large value of (T/N). Agreement Confirm the following working assumption: Working Assumption - The paging indication field of PEI DCI format comprises of POnumPerPEI segment(s) of K bit - K = 1, if is absent or set to 0 or 1, - K = , if is configured. - UE identifies its paging indication bit as follows: - Let denote the relative PO index, with starting value of 0, among the POs associated with the PEI - , where are as defined in clause 7 of TS 38.304 - when K = 1 and UE is not provided a subgroup index - when UE is provided a subgroup index - UE checks the corresponding paging indication from -th bit of the paging indication field where the starting bit index is 0 - If the corresponding paging indication value is set to ‘1’, it indicates the UE to monitor the PO - If the corresponding paging indication value is set to ‘0’, it indicates the UE is not required to monitor the PO Agreement For PEI DCI format, defined as DCI format 2_7, - Total number of bits for paging indication filed is POnumPerPEI, if is absent or set to 0 or 1, and the number is , if is configured. - For Rel-17, UE does not expect paging indication filed size is larger than the DCI payload size - Whether and how TRS availability indication field is included is up to Agenda Item 8.7.1.2 - Support configurable DCI payload size which should be no larger than payload size of paging DCI - Unused bits, when applicable, are regarded as reserved bits - Note: A smaller payload size is beneficial for PEI detection performance Agreement - Determination of PEI-O location for UE’s PO is based on deciding a reference point and an offset from the reference point to the start of the first PDCCH MO of the PEI-O. o The reference point is the start of a reference frame determined by a frame-level offset from the start of the first PF of the PF(s) associated with the PEI-O and configured via SIB for the cell. - FFS: The range of the frame-level offset o There is a symbol-level offset from the reference point to the start of the first PDCCH MO of PEI-O, provided by firstPDCCH-MonitoringOccasionOfPEI-O and configured via SIB for the cell. - FFS: The range of the symbol-level offset - Note: When PEI-O is placed close to or overlapped with an earlier SS burst before its associated POs, the total UE wake-up time can be reduced for better power saving gain. Network can configure the PEI-O location accounting the power saving benefit and potential impact on gNB flexibility. Agreement The CCE aggregation levels and maximum number of PDCCH candidates per CCE aggregation level for PEI PDCCH monitoring occasion are given as the following table. Actual aggregation levels and PDCCH candidates are provided by ‘peiSearchSpace’ configuration in SIB. CE Aggregation Level; Number of Candidates. 4; 4. 8; 2. 16; 1. Agreement ‘peiSearchSpace’ can be configured to one of up to 4 common SS sets configured by commonSearchSpaceList with SearchSpaceId > 0

Relating to scope item 1) – b): - Specify enhancements for idle/inactive-mode UE power saving, considering system performance aspects [RAN2, RAN1] - Specify means to provide potential TRS/CSI-RS occasion(s) available in connected mode to idle/inactive-mode UEs, minimizing system overhead impact [RAN1] - NOTE: Always-on TRS/CSI-RS transmission by gNodeB is not required The following agreements are achieved:

RAN1 #106-bis-e Meeting Conclusion No consensus to support SIB based signaling for availability information of TRS/CSI-RS occasions for idle/inactive UEs Working Assumption If TRS resource is configured in SIB, L1 based availability indication is always enabled based on the configuration. Agreement For L1 based availability indication of TRS/CSI-RS at the configured occasion(s) to the idle/inactive UEs, support availability information for configured RS resources using a bitmap. where each bit indicates whether associated TRS resource(s) are available. - Support L1 availability indication at an occasion can provide availability information RS resources with QCL references not confined to be the same as for the L1 availability indication occasion - FFS associated TRS resource(s) per bit, e.g. a bit is associated with a TRS resource set - Bitmap size is up to X bits - X = [6] for paging PDCCH based L1 availability indication. - FFS X for PEI DCI based L1 availability indication - FFS details about how to configure the DCI field: e.g. start and length of bitmap (e.g. explicitly/implicitly configured) - for paging PDCCH based L1 availability indication, support L1 availability indication at an occasion can provide availability information for all configured RS resources - FFS whether this needs to be supported regardless of the number of beams or for some configured RS resources - FFS: PEI DCI provides L1 availability indication information only for RS resources with QCL references to be the same as for the L1 availability indication occasion - FFS: indication of unavailability Agreement At least for paging PDCCH based L1 availability indication of TRS/CSI-RS at the configured occasion(s) to the idle/inactive UEs, the L1 availability indication is valid for a time duration starting from a reference point, where - the time duration is a validity duration configured by higher layer, - FFS applicable values, e.g. # of DRX cycles, or multiple of default paging cycle duration (i.e. modification period) - FFS UE doesn’t expect inconsistent L1 based indication during the time duration - the reference point for start of the validity duration is one of the following alternatives: - Alt1: SFN of the first PF from the next DRX cycle - Alt2: SFN of the first PF from the current DRX cycle where UE receives the indication - Alt3: based on SFN configured by higher layer, i.e. modification period configured as multiple of default paging cycle duration - Alt4: start of the PF for the PO where UE receives the indication - Note: the DRX cycle in Alt1 and Alt2 is the default paging cycle broadcast in SIB - Note: The SFN for the first PF is for (UE mod N) = 0, and can be calculated by (SFN + PF_offset) mod T = 0 - the time duration can be optionally configured by gNB - when the time duration is not configured, one of the following alternatives can be considered: - Alt1: the availability indication is valid until when the UE receives another availability indication. - Alt2: the availability indication is valid until L1 availability indication is changed by network - Alt3: default time duration e.g. default paging cycle - FFS whether and how to handle the miss detection issue of L1 signaling Agreement Configuration of TRS/CSI-RS occasion(s) for idle/inactive UEs include a list of one or more TRS resource sets, where: - a TRS resource set can be configured to include - a set of TRS resources up to two consecutive slots, - Note: a TRS resource is same as Rel-15/16, i.e. a CSI-RS in a symbol. - at least common configuration parameters: - a QCL reference - firstOFDMSymbolInTimeDomain, - ‘frequencyDomainAllocation for row1’, ‘startingRB’ ,‘nrofRBs’,’powerControlOffsetSS’, periodicityAndOffset’ - FFS - scramblingID, - a TRS resource set ID, number of slots {1, 2} or number of symbols {2, 4} if supported - Note: the ‘TRS resource set’ configuration is not (necessarily) identical to ‘NZP-CSI-RS-ResourceSet’ configuration for TRS in R15/16.

RAN1 #107-e Meeting Agreement For the maximum number of TRS resource sets configured by higher layer, X, - X = 64 - FFS: the number of configured TRS resource sets is not larger than the number of actual transmitted SSBs determined according to ssb-PositionsInBurst in SIB1 Agreement For L1 availability indication using a bitmap, the following is supported: - Number of bits in the bitmap, N, is up to 6 bits - a bit is associated with a group of TRS resource sets. The associated TRS resource sets for each bit can be based on - explicit configuration of TRS resource set group, where - each TRS resource set is configured with a ID i, with value from {0, …, N-1}, for the association with an indication bit in TRS availability indication field. - the ith bit maps to all the TRS resource set(s) associated with ID i. - start of the bitmap is the first bit of the reserved bits in paging PDCCH - Note: It is left to RAN2 decision on whether explicit parameter is used for N or it can be implicitly determined by the TRS resource set configurations. Agreement The reference point for start of the validity duration is SFN of the first PF from the current default DRX cycle where UE receives the availability indication - FFS: Whether the availability indication is transmitted [only once] during the validity duration Note: Qualcomm and Huawei have concern on Alt a Agreement For the validity duration configured by higher layer at least for paging PDCCH based L1 availability indication, support - time unit is one default paging cycle, - applicable values: {1, 2, 4, 8, 16, 32, [64], [128], [256],[512]} When the validity duration is not configured, UE assumes a default time duration to be 2 default paging cycle(s): Agreement Confirm the following working assumption Working Assumption - Support paging PDCCH based availability indication of TRS/CSI-RS occasions for idle/inactive UEs. - Support PEI based availability indication of TRS/CSI-RS occasions for idle/inactive UEs at least if PDCCH-based PEI is down-selected.

Relating to scope item 2) – a): - Study and specify, if agreed, enhancements on power saving techniques for connected-mode UE, subject to minimized system performance impact [RAN1, RAN4] - Study and specify, if agreed, extension(s) to Rel-16 DCI-based power saving adaptation during DRX Active Time for an active BWP, including PDCCH monitoring reduction when C-DRX is configured [RAN1] - NOTE: Rel-15 and Rel-16 available power saving solutions should be supported by the UE and included in the evaluation. RAN1 will ask the confirmation from RAN2 that Rel-15 and Rel-16 available power saving solutions are properly utilized. The following agreements are achieved:

RAN1 #106-bis-e Meeting Agreement Confirm the four working assumptions(extracted from package 1) Working assumption: - Beh 1: PDCCH skipping is not activated Working assumption: - Indication of Beh 1A for current SSSG when two SSSG(s) are configured is supported Working assumption: - At most 3 SSSGs is supported to be configured for PDCCH monitoring adaptation. Working assumption: - Beh 2B: stop monitoring SS sets associated with SSSG#0 and SSSG#1 and monitoring of SS sets associated to SSSG#2. Agreement Scheduling DCIs indicating timer value for a SSSG is not supported. Agreement For Beh 1A, - The UE can be configured to be indicated by DCI a value of X slots (i.e., skipping duration) among multiple RRC configured values by scheduling DCIs indicating PDCCH schedules data - The bits for indicating PDCCH monitoring adaptation also indicating skipping duration. Details FFS Agreement The bit mapping of DCI indication PDCCH monitoring adaptation is as follows, - For Case 1 (i.e., PDCCH skipping), the following is supported - 1-bit in scheduling DCI is supported to indicate PDCCH monitoring adaptation UE behaviors if M=1 - ‘0’ is Beh 1 and ‘1’ is Beh 1A - 2-bit in scheduling DCI is supported to indicate PDCCH monitoring adaptation UE behaviors if M=2 or 3 - ‘00’ is Beh 1 - ‘01’ is Beh 1A with skipping duration 1 - ‘10’ is Beh 1A with skipping duration 2 - ‘11’ is Beh 1A with skipping duration 3 if M=3, reserved if M=2 - For Case 2 (i.e., 2 SSSG switching) , the following is supported - 1-bit in scheduling DCI is supported to indicate PDCCH monitoring adaptation UE behaviors - ‘0’ is Beh 2 and ‘1’ is Beh 2A - For Case 3 (i.e., 3 SSSG switching) , the following is supported - 2-bit in scheduling DCI is supported to indicate PDCCH monitoring adaptation UE behaviors - ‘00’ is Beh 2 - ‘01’ is Beh 2A - ‘10’ is Beh 2B - [‘11’ is reserved] - For Case 4 (i.e., 2 SSSG switching with PDCCH skipping) , the following is supported - 2-bit in scheduling DCI is supported to indicate PDCCH monitoring adaptation UE behaviors, - FFS details bit mapping - FFS: For Case 5 (i.e., 3 SSSG switching and skipping) - 2-bit in scheduling DCI is supported to indicate PDCCH monitoring adaptation UE behaviors - ‘00’ is Beh 2 - ‘01’ is Beh 2A - ‘10’ is Beh 2B - ‘11’ is Beh 1A - FFS Timer behavior when Beh 1A is indicated - Note: The UE can be configured to be indicated by DCI a value of X slots (i.e., skipping duration) among M RRC configured values by scheduling DCIs indicating PDCCH schedules data - FFS whether to restrict Skipping duration to be shorter than SSSG initial timer value - FFS whether the configuration is same or different for DCI format x_1 and DCI format x_2 Agreement - The value of the SSSG switching timer in slots for SSSG#1 and/or SSSG#2 can be configured as - {[1...20,40,60,80,100]} for 15 kHz SCS, - {[1...40, 80,100,160,200]} for 30 kHz SCS, - {[1...80, 160,200,320,400]} for 60kHz SCS, - {[1...160,320,400,640,800]} for 120kHz SCS Agreement - If the UE monitors PDCCH according to SSSG#1 and the timer expires, the UE starts monitoring PDCCH according to Beh 2. - If the UE monitors PDCCH according to SSSG#2 and the timer expires, - Alt 1: the UE monitoring PDCCH according to Beh 2 - Other alternatives are not precluded - Timer can be optionally configured. Agreement Select one of the alternatives from the following: - Alt 1: Separate RRC configuration for timer value(s) is supported for switching from SSSG#2 to SSSG#0 and from SSSG#1 to SSSG#0 respectively. - Alt 2: the timer value(s) for switching from SSSG#2 to SSSG#0 and from SSSG#1 to SSSG#0 is common and configured per cell. - Alt 3: the timer value(s) for switching from SSSG#2 to SSSG#0 and from SSSG#1 to SSSG#0 is common and configured per BWP. Agreement The following application delay for a scheduling DCI based PDCCH monitoring adaptation indication can be considered, - For PDCCH skipping, - Option b - Option f - Option d for downlink grant and Option c for uplink grant - Option i - Option j - Note: down-select based on the options in RAN1#107-E - For SSSG switching, - Option a - Option d for downlink grant and Option c for uplink grant - Option h - Option b - Option d for downlink grant and Option g for uplink grant - Note: down-select based on the options in RAN1#107-E - The Options a – j are defined as follows, - Option a: the application timelines provided in Table 10.4-1 in TS38.213 for search-space group switching for unlicensed band form is reused. - for SCS configuration , FFS X = 25 or 39 - FFS: - Option b: the application delay needed for PDCCH processing for Rel-16 minimum application delay for K0min/K2min indication is reused/extended. - Option c: PDCCH monitoring adaptation command applies after PUSCH transmission if triggered by UL DCI - Option d: PDCCH monitoring adaptation command applies after HARQ-ACK transmission (or plus some margin for HARQ-ACK decoding). - Option e: after successfully decoding TB. - Option f: Application delay should be “ZERO” for PDCCH monitoring adaptation. PDCCH monitoring adaptation would be applied after UE receive the additional PDCCH monitoring adaptation control signaling bit(s) in DCI - Option g: Application delay(s) are configured via RRC signaling - Option h: Application delay applies after drx-RetransmissionTimerUL expires - Option i: Leave up to implementation - Option j: UE applies the skipping immediately (e.g. next symbol/slot) after the UE receives the indication in DL assignment. If the UE fails to decode the associated PDSCH and transmits a NACK, skipping is canceled in the slots after the NACK transmission. Option g ( application delay configured via RRC signaling) is used for uplink grant. If RRC signaling is not provided, UE applies the skipping immediately (e.g. next symbol/slot) after the UE receives the indication in UL grant. - Other options not precluded. - FFS reference points for the application delay, - FFS whether the same or different and how application delay for PDCCH monitoring adaptation indicated by DCI and timer expiration - FFS non-scheduling DCI if supported

RAN1 #107-e Meeting Agreement The following agreement in RAN1#106-E is updated as follows, Agreement(extracted from RAN1#106-E) - At most 2 bit indication in self-scheduling DCIs (i.e., DCI format 1-1/0-1/1-2/0-2) can be specified for triggering the PDCCH monitoring adaptation in a single cell - FFS: the bit size of the indication is configurable - FFS: bit mapping to the PDCCH monitoring behaviour - FFS: details of indication of multiple cells case - A new indication field in scheduling DCI is used for indicating PDCCH monitoring adaptation Agreement Confirm the working assumptions with the following updates (extract from RAN1#106-bis agreements) - The value of the timer in slots for monitoring PDCCH in the active DL BWP of the serving cell before moving to the default search space group is - {1,2,3,…,20,30, 40, 50, 60, 80, 100} for 15 kHz SCS, - {1,2,3,…,40, 60, 80, 100, 100,160,200} for 30 kHz SCS, - {1,2,3,…,80, 120, 160, 200, 240, 320,400} for 60kHz SCS, - {1,2,3,…,160, 240, 320,400, 480, 640,800} for 120kHz SCS Agreement - For Case 4 (i.e., 2 SSSG switching with PDCCH skipping) , one of the following is supported - Alt a: 2-bit in scheduling DCI is supported to indicate PDCCH monitoring adaptation UE behaviors - If M = 1, then - ‘00’ is Beh 2 - ‘01’ is Beh 2A - ‘10’ is Beh 1A (stay in current SSSG) - [‘11’ is reserved] - If M = 2, then - ‘00’ is Beh 2 - ‘01’ is Beh 2A - ‘10’ is Beh 1A for duration T1 (stay in current SSSG) - ‘11’ is Beh 1A for duration T2 (stay in current SSSG) - Note: for ‘00’ and ‘01’, UE performs Beh 1(i.e., does not perform PDCCH skipping ) - Note: By a single DCI indication, a UE is not expected to simultaneously perform two behaviors of SSSG switching and PDCCH skipping. - FFS SSSG switching timer behavior, and timer value related to the value of T1/T2 Agreement If a UE is provided with a timer value by searchSpaceSwitchTimer-r17 for PDCCH monitoring on a serving cell and the timer is running, the UE - resets the timer after a slot of the active DL BWP of the serving cell when the UE detects a DCI format in a PDCCH reception in the slot o Alt 2a: for the Type3-PDCCH CSS set or the USS set with group index of either 1 or 2 o Alt 2b: for the Type3-PDCCH CSS set or the USS set o Alt 2c: with CRC scrambled by C-RNTI/CS-RNTI/MCS-C-RNTI - otherwise, decrease the timer value by one after each slot. - FFS: When the timer expires in a slot Agreement - For value X in Beh 1A, candidate skipping values are o Up to [100ms] length is supported, o The X is configured and indicated in the unit of slot. o Working assumption for candidate values for X o {1,2,3,…,20,30, 40, 50, 60, 80, 100} for 15 kHz SCS, o {1,2,3,…,40, 60, 80, 100, 120,160,200} for 30 kHz SCS, o {1,2,3,…,80, 120, 160, 200, 240, 320,400} for 60kHz SCS, o {1,2,3,…,160, 240, 320,400, 480, 640,800} for 120kHz SCS o FFS: Equal to or longer than the applicable minimum scheduling offset o FFS: additional symbol level / PDCCH monitoring period level skipping duration

#### 2.1.2	Remaining open issues

RAN1 has specified designs that are sufficient for generating RRC parameters and UE feature definitions for the above scope items. RAN1 will enter maintenance phase and resolve the outstanding FFS (see above) and critical issue(s), if identified. The remaining RAN1 work is not expected to cause any blockage to the progress of any other WG.

## 2.2	RAN2

#### 2.2.1	Agreements

Relating to scope item 1) - a) - Specify enhancements for idle/inactive-mode UE power saving, considering system performance aspects [RAN2, RAN1] - Study and specify paging enhancement(s) to reduce unnecessary UE paging receptions, subject to no impact to legacy UEs [RAN2, RAN1] - NOTE: RAN1 to check and update, if needed, evaluation methodology in RAN1 #102-e meeting The following agreements are achieved:

- RAN2 #116-e Meeting Assume that one subgroup indication refer to either CN assigned subgroups or UE-ID based subgroup (no overlapping) Both UE ID based and CN based subgrouping can be supported simultaneously in a cell, it is allowed to just support one of them. FFS if the total number of CN-assigned subgroups is OAM configured. Max would be 8 as this is what RAN support. The total number of CN-assigned subgroups that is used is not fixed can be configured up to 8 (e.g. by OAM). No impact on signalling is assumed. RAN introduces a new parameter Nsg-UEID to indicate its support of UE-ID based subgrouping. RAN does not support any type of subgrouping if its configuration for subgrouping is either absent or nullified (e.g. subgroupsNumPerPO is either absent or set to zero). FFS for the signalling details. We assume separate indications for UE capability of CN based subgrouping and UEID based subgrouping. UE’s capability of supporting the UE ID based subgrouping is reported to RAN by AS UE capability signalling while R2 assumes that UE’s capability of supporting the CN-assigned subgrouping is reported to CN by NAS signalling. We send an LS (short post email discussion) [Post116-e][060][ePowSav] LS out on paging subgouping and PEI (XIaomi) Scope: LS out on RAN2 agreements on paging subgrouping and PEI, to concerned groups Intended outcome: Approved LS out (R2-2111415) Deadline: Super Short (not for RP) RAN2 assumes that if PEI is detected, and the PEI indicates that the UE has to monitor the associated PO, then the UE monitors paging DCI in the associated PO, including scheduling information for paging PDSCH (if included) as in legacy. This assumption may be updated based on RAN1 agreements. As a baseline RAN2 has a preference to support PEI with both DRX and eDRX, but potential issues (e.g. PEI and PTW) are FFS. For UE-ID based subgroups the UE identity is UE_ID = 5G-S-TMSI mod X, where X is 8192 (1024*8). Introduce a UERadioPagingInfo IE in the UECapabilityInformation message in NR in Rel-17. If the UE was not able to monitor the PEI occasion corresponding to its PO the UE shall monitor the PO.

Relating to scope item 1) – b): - Specify enhancements for idle/inactive-mode UE power saving, considering system performance aspects [RAN2, RAN1] - Specify means to provide potential TRS/CSI-RS occasion(s) available in connected mode to idle/inactive-mode UEs, minimizing system overhead impact [RAN1] - NOTE: Always-on TRS/CSI-RS transmission by gNodeB is not required The following agreements are achieved:

- RAN2 #116-e Meeting The scope of the new SIB-X is configurable (either cell or area scope) based on NW implementation. RAN2 to wait for additional RAN1 feedback, before finalizing aspects on SIB-X sizing, segmentation etc. RAN2 to wait for further RAN1 input on whether TRS/CSI-RS configuration can be split as common and TRS specific part. The new SIB-X can be made on demand, and it is up to NW configuration. There are no UE side impacts due to any additional NW side restriction on on-demand SIB-X. IDLE/INACTIVE UEs do NOT have to report any feedback on its TRS/CSI-RS resource usage. RAN2 assumes to support current RAN1 working agreement of L1 based signalling for TRS/CSI-RS availability indication. FFS whether it should be possible to enable / disable the TRS/CSI-RS L1 based availability mechanism by broadcast signalling. R2 assumes that additional TRS/CSI-RS configuration by dedicated signalling is not supported. Can revisit e.g. based on R1 provided info if needed. Postpone further discussion on TRS/CSI-RS applicability for eDRX UEs. Can consider later

Relating to scope item 2) - b) - Study and specify, if agreed, enhancements on power saving techniques for connected-mode UE, subject to minimized system performance impact [RAN1, RAN4] - Study the feasibility and performance impact of relaxing UE measurements for RLM and/or BFD, particularly for low mobility UE with short DRX periodicity/cycle, and specify, if agreed, relaxation in the corresponding requirements [RAN4] - NOTE: Supplementary RAN2 work, if needed, can be triggered by RAN4 LS The following agreements are achieved:

- RAN2 #116-e Meeting RLM/BFD relaxation criteria are configured by dedicated signalling (e.g. RadioLinkMonitoringConfig) as a baseline, if RAN4 decides to provide parameters instead of predefined or by implementation. R2 assumes to use AS capability procedure to report UE capability of supporting RLM/BFD relaxation. Details FFS. RAN2 wait for RAN4 progress on the designing of low mobility criterion. RAN2 assumes the presence/absence of configuration for RLM/BFD relaxation criteria in signalling indicates to the UE whether the UE can/should evaluate the criteria.

#### 2.2.2	Remaining open issues

With the above, the following are remaining issues for idle-mode power saving enhancements in RAN2:

- For scope item 1) – a):

  - Signalling details for PEI configuration, based on RAN1 LS on Rel-17  RRC parameters post RAN1#107-e

  - Descriptions of PEI occasion and UE monitoring behavior in TS 38.304, taking into account RAN1 agreements in RAN1#106-bis-e and RAN1#107-e

  - Details for PEI support with eDRX (e.g. PTW)

  - Whether PEI is sent only in last serving cell

- For scope item 1) – b):

  - Signalling details for TRS/CSI-RS configuration for Idle and Inactive, based on RAN1 LS on Rel-17  RRC parameters post RAN1#107-e

  - Configurations, e.g., SIB-X size, segmentation, whether to split into common and TRS

- For scope item 2) – b):

  - Signalling details for low mobility criterion, based on RAN4 LS on RLM/BFD relaxation post RAN4#101-e

## 2.3	RAN3

#### 2.3.1	Agreements: N/A (Normative work is not yet started)

#### 2.3.2	Remaining Open issues: N/A

From RAN2 LS to RAN3 (R2-2111415), the following is the identified open issue related to UE sub-grouping enhancement:

- Signalling between AMF and gNB(s) to inform gNB(s) about the related subgroup information for paging a UE in RRC_IDLE/RRC_INACTIVE, e.g., to convey Subgroup ID, UE capability, etc.

## 2.4	RAN4

#### 2.4.1	Agreements

Relating to scope item 2) - b) - Study and specify, if agreed, enhancements on power saving techniques for connected-mode UE, subject to minimized system performance impact [RAN1, RAN4] - Study the feasibility and performance impact of relaxing UE measurements for RLM and/or BFD, particularly for low mobility UE with short DRX periodicity/cycle, and specify, if agreed, relaxation in the corresponding requirements [RAN4] - NOTE: Supplementary RAN2 work, if needed, can be triggered by RAN4 LS The following agreements are achieved:

RAN4 #101-e Meeting Approved: WF on RLM/BFD relaxation for UE Power Saving enhancements (R4-2120313): ## Sub-topic 0 Specification structure and work split #### Issue 1-1: Specification structure Agreement The relaxed RLM/BFD requirements is introduced in new subsections within the existing RLM/BFD sections TS 38.133 #### Issue 1-2: Work split and specification changes Agreement RRM requirement; Expected impact; Section in 38.133; Volunteer. General of RLM; applicability of the relaxed requirements; 8.1.1; MediaTek. Minimum requirement for SSB based radio link monitoring for UE configured with relaxed measurement criterion; fulfilled conditions and the corresponding relaxed minimum requirement; 8.1.2.X; Huawei. Minimum requirement for CSI-RS based radio link monitoring for UE configured with relaxed measurement criterion; fulfilled conditions and the corresponding relaxed minimum requirement; 8.1.3.X; Vivo. [Relaxed measurement criteria for RLM]; Criterion; 8.1.Y1; Nokia. General of BFD; applicability of the relaxed requirements; 8.1.5; Ericsson. Minimum requirement for SSB based beam failure detection for UE configured with relaxed measurement criterion; fulfilled conditions and the corresponding relaxed minimum requirement; 8.5.2.X; CMCC. Minimum requirement for CSI-RS based beam failure detection for UE configured with relaxed measurement criterion; fulfilled conditions and the corresponding relaxed minimum requirement; 8.5.3.X; Xiaomi. [Relaxed measurement criteria for BFD]; Criterion; 8.5.Y2; CATT. Note: If sections in square brackets are to be merged with other sections based on RAN4 decision, then CR is not needed. ## Sub-topic 1 Relaxation applicability ##### Issue 1-1-2: UE capability for low mobility criteria? Proposals - Option 1: RAN4 to introduce an UE capability on supporting low mobility criterion in Rel-17 power saving - Option 2: No need to introduce an UE capability on supporting low mobility criterion in Rel-17 power saving - Other options are not precluded. ##### Issue 1-1-B: whether the low mobility criterion is mandatory to be configured, when network would like to enable RLM/BFD relaxation? - Conclusion: No. The criterion is NOT mandatory to be configured to enable RLM/BFD relaxation - Note: UE shall evaluate the low mobility criterion if it is configured. ##### Issue 1-2-B: whether the good serving cell quality criterion is mandatory to be configured, when network would like to enable RLM/BFD relaxation, assuming the good serving cell quality criterion is configurable? - Note: UE shall evaluate the good serving cell criterion if it is configured. - Options: - Option 1: No. The criterion is NOT mandatory to be configured to enable RLM/BFD relaxation (OPPO, [Nokia], ZTE, Ericsson, Nokia, Ericsson, Apple) - Note: if the criteria is not configured, the good serving cell quality state can be determined by network implementation - Option 2: Yes. The criterion is mandatory to be configured to enable RLM/BFD relaxation. (CATT, MTK, Huawei, Intel, CMCC, Qualcomm, vivo, Xiaomi) ##### Issue 1-2-C: whether to have an explicit indication to enable RLM/BFD relaxation, assuming the good serving cell quality criterion is predefined? - Option 1: Yes. An explicit indication to indicate the good serving cell quality criterion shall be evaluated - Option 2: No. UE shall evaluate the predefined criterion. - Note: Whether UE can enter the relaxation mode depends on the outcome of Issue 1-1-B, regarding whether the low mobility criterion is mandatory to be configured ##### Issue 1-3: dedicated signaling to indicate the UE when it is allowed to relax the RLM/BFD measurements Proposals - Option 1: Allow explicit relaxation indication to the UE when it is allowed to relax the RLM/BFD measurements irrespective of the relaxation criteria configuration - Option 2: No ##### Issue 1-4: Relaxation when both serving cell quality criteria and low mobility criteria are configured Agreement - Option 1: both criteria are fulfilled. #### Sub-topic 2 Low motility criteria ##### Issue 2-1-1: RS for L3 RSRP in Low mobility criteria Agreement Intra-frequency L3 RSRP measurement of serving cell based on SSB is used for low mobility criteria evaluation. - FFS: L3 CSI-RS - FFS support beam-level low mobility criterion at least for UE configured with BFD ##### Issue 2-1-2: Accuracy requirements for low mobility criteria - Proposals - Option 1: The RRM measurements used for low mobility evaluation shall fulfill the accuracy requirements defined in TS 38.133 section 10. (Nokia, Intel, Ericsson) - Other options are not precluded. ##### Issue 2-1-3: thresholds for R17 low mobility criteria Agreement For low mobility criterion, the threshold on RSRP variation and the time period over which the RSRP variation is evaluated for relaxed RLM/BFD measurement are configured by network. - Thresholds for R16 low mobility criterion and R17 low mobility criterion can be configured separately. ##### Issue 2-1-4: Additional Low mobility criteria - Proposals - Option 1: RAN4 additionally to define a low mobility criterion based on the number of serving beam changes over time (e.g. TCI state change) (Nokia) - It is up to network to configure if the low mobility criteria is based on RSRP variation or TCI changes, or the two in combination. (Nokia) - Option 1a: Relaxed mode operation for RLM/BFD is allowed if UE has not done any beam failure detection over last X (e.g. X=1) evaluation period. (Ericsson) - Option 2: Use the following low mobility evaluation for BFD: (Qualcomm) - For a serving cell, the change in the difference between SINR of its BFD RSs and the largest SINR of other non-QCLed beams is lower than a threshold configured by network. Network can configure BFD RS with two non-QCLed RSs to enable the SINR comparison between serving and other non-QCLed beams. - Option 2a: Define L1-SINR measurement accuracy requirement for BFD low mobility evaluation purpose. (Qualcomm) - Option 3: not to define any additional low mobility criteria. (Huawei, Apple, Intel, vivo, CATT, MTK, Xiaomi, OPPO) ##### Issue 2-2: Low mobility criteria configuration type - Proposals - Option 1: Low mobility criterion is configured on per-UE basis, and UE needs only to identify low mobility state according to RRM measurements in the NR PCell for the case of NR single carrier, NR CA, NE-DC and NR-DC, and according to that in the NR PSCell for the case of EN-DC. (Vivo, MTK, CMCC, Oppo) - Option 2: on per-cell basis (CMCC) - Option 3: leave for RAN2 to decide. ##### Issue 2-3: RAN4 or RAN2 to define the low mobility criteria - Proposals - Option 1: Low mobility criterion is preferred to be further discussed in RAN2. (Vivo, Qualcomm) - Option 2: Be discussed in RAN4. (MTK, Nokia, Ericsson, Oppo) #### Sub-topic 3 Good serving cell quality criteria ##### Issue 3-1: SINR definition for good serving cell quality criteria - Agreements - Reuse the existing method to evaluate “downlink radio link quality” for RLM/BFD - hypothetical BLER with corresponding PDCCH parameters is used to evaluate good serving cell quality criterion Issue 3-2-1: good serving cell quality criteria for RLM Agreement The good serving cell quality criteria for RLM is based on an offset X dB and Qx, while Qx is derived from PDCCH transmission parameters. - Option 1: Qx = Qout. - Note: Larger value of X can be considered. - Note: Companies are encouraged to provide the range/value of offset X. - Option 2: Qx = Qin - Option 2a: Qx = Qin, while set offset as X = 0 dB. ##### Issue 3-2-2: good serving cell quality criteria for BFD The good serving cell quality criteria for BFD is - Option 1: radio link quality > Qout_LR + Y (dB). - Note: Companies are encouraged to provide the range/value of offset Y. - Note: Larger value of Y can be considered. - Option 2: radio link quality > Qin_LR + Y (dB). - Option 2a: The value of Y can consider the PDCCH BLER performance for BFD based on SSB or CSI-RS. - Option 2b: Y = 0. ##### Issue 3-3-1: predefined or configured offset (X in Issue 3-2-1, Y in Issue 3-2-2) Proposals - Option 1: The offset values are configured to the UE by the network. - Option 2: Use predetermined offset value. - Option 2a: The threshold for determining the good serving cell quality is pre-defined as in existing RLM evaluation principle and offset values on top of the existing requirement can be considered. - Option 2b: Use predetermined offset value of 5dB. - Option 2c: The threshold for determining the good serving cell quality is pre-defined as Qin, and no need to define any offset values. ##### Issue 3-3-2: if offset is predefined for RLM, the offset value X Proposals - Option 1: Where X depends on max(TDRX, TSSB). - X = X1 when max(TDRX, TSSB) < 40 ms - X = X2 when max(TDRX, TSSB) ≥ 40 ms. - X1 and X2 are predefined and decided based on summary of simulation results that was conducted earlier in WI.. - X is smaller in FR2 compared to FR1. - Option 1a: X = 0. Threshold is same as existing Qin. - Option 2: a unified offset value for RLM. ##### Issue 3-3-3: if offset is predefined for BFD, the offset value Y Proposals - Option 1: Where Y depends on max(TDRX, TSSB) and - Y = Y1 when max(TDRX, TSSB) < 40 ms - Y = Y2 when max(TDRX, TSSB) ≥ 40 ms. - Y1 and Y2 are predefined and decided based on summary of simulation results that was conducted earlier in WI. - Y is smaller in FR2 compared to FR1. - Option 1a: Y = 0. Threshold is same as existing Qin_LR - Option 2: a unified offset value for BFD. ##### Issue 3-3-4: different offsets for RLM and BFD - Proposals - Option 1: The offset values for deriving the threshold used for good serving cell quality criterion can be different for RLM relaxation and BFD relaxation. - Option 2: Same threshold if the same set of RSs are used. - Option 3: using Qin and Qin_LR as the entering criteria, there is no need to define offset value, or the offset values are assumed to be 0. ##### Issue 3-4: Good serving cell quality criteria configuration type - Proposals - Option 1: Cell quality criterion is effective on per-cell basis, either activated by explicit thresholds configuration on per-cell basis or by other broadcast/dedicated ignaling on per-cell basis. (Vivo, CMCC) - Option 2: per-UE basis (MTK) - Option 3: leave for RAN2 to decide. #### Sub-topic 4 Exiting Relaxation criteria ##### Issue 4-1: Exiting relaxation criteria Proposals - Option 1: No additional criteria are needed, previous agreement from 98-e-bis and 99-e-bis are sufficient. - Option 2: Set exit threshold as entering threshold with a hysteresis value. - FFS the exit threshold is configurable. - Option 3: Use Qout as exit threshold i.e. the UE will exit from relaxation mode when OOS is detected. #### Sub-topic 5 During Relaxation mode ##### Issue 5-1: lower bound of relaxed evaluation period Proposals - Option 1: the lower bound of relaxed evaluation period is also relaxed. (CATT, Xiaomi, Ericsson, MTK, vivo) - Option 2: the lower bound of relaxed evaluation period is NOT relaxed. (CMCC, Nokia, Qaulcomm, Apple, Oppo) ##### Issue 5-2: relaxation factors Agreement - The maximum allowed relaxation factor should be less than 8 - The relaxation factor for FR1: - TRS is the periodicity of SSB for the case of SSB based, and the periodicity of CSI-RS for the case of CSI-RS based. - K0, FR1 =1 for 80 ms < MAX(TDRX, TRS) ≤ 160 ms. - K1, FR1=[2, 3 or 4] for 40 ms < MAX(TDRX, TRS) ≤ 80 ms - K2, FR1=[2, 3, or 4] for MAX(TDRX, TRS) ≤ 40 ms - FFS select between [2,3,4] - Option 1: based on N310/T310 and DRx, - e.g., RLM relaxation - MAX(TDRX, TRS) <= 40ms N310\T310; <=320ms; 320~1280ms; >=1280ms. <8; 2; 3; 4. >=8; 3; 3; 4. - MAX(TDRX, TRS) between 80ms and 40ms N310\T310; <=320ms; 320~1280ms; >=1280ms. <8; 2; 2; 3. >=8; 2; 3; 3. - BFD relaxation: when N310>=8, use K = 3, otherwise, K = 2 - Other options are not precluded. - The relaxation factor for FR2 SSB: - K0, FR2, SSB = 1 for [80] ms < MAX(TDRX, TSSB) ≤ 160 ms - K1, FR2, SSB= [1.5 or 2] for MAX(TDRX, TSSB) ≤ [80] ms for SSB based relaxation. - The relaxation factor for FR2 CSI-RS: - K0, FR2, CSI-RS =1 for 80 ms < MAX(TDRX, TCSI-RS) ≤ 160 ms - K1, FR2, CSI-RS = 2 for MAX(TDRX, TCSI-RS) ≤ 80 ms for CSI-RS based relaxation. ##### Issue 5-3: OOS indication during relaxation mode - Proposals - Option 1: Same as in legacy RLM procedure, UE indicates OOS when the measured SINR becomes worse than Qout during the relaxed mode. (CATT, CMCC, Ericsson, Apple) - Option 2: Do not send OOS indication in relaxation mode. UE shall exit from the relaxed RLM/BFD measurements at the 1st Qout occurrence. (Qualcomm, Nokia, Huawei, Intel) - Option 3: no need to further discuss (MTK, Xiaomi) - Option 4: depends on other issue (vivo, OPPO) ##### Issue 5-4: Additional N310/N311 values for relaxation mode - Proposals - Option 1: It is allowed for the network to configure different values of the RLF parameters, e.g. T310/N310/N311, for the relaxed operation to reduce the negative impact to the system performance. (Nokia) - Option 2: no need (Huawei, Ericsson) #### Sub-topic 6 Other Aspects ##### Issue 6-1: Relaxation criteria for multiple RLM-RS/BFD-RS Proposals - Option 1: Entering power saving mode when at least one of the configured resources are better than the entering threshold. (Qualcomm, MTK, Xiaomi, Oppo) - Option 2 (CMCC, Ericsson) - The UE is allowed to operate RLM/BFD in relaxed mode for a certain cell (SpCell or SCell) when the radio link quality is better than the threshold (Qout + X1) for all RLM-RS resource. - The shall exit the relaxed mode when the radio link quality is worse than the threshold (Qout + X2) for any the RLM-RS resources. - The values of X1, X2 are FFS. - Option 3: The UE behaviour on checking the entering/exiting condition of cell quality criterion regarding multiple RLM-RSs/BFD-RSs is not specified. (vivo, MTK) - Option 4: relaxation is based on per-RS basis (Nokia) ##### Issue 6-2: Relaxation criteria in NR-DC and inter-band CA Agreement For the case of NR-DC and inter-band CA, UE can make the relaxation decisions separately for each serving cell configured for either RLM and/or BFD evaluation. ##### Issue 6-3: Interaction with PDCCH monitoring relaxation Agreement RAN4 can FFS the impact if both RLM/BFD relaxation and PDCCH monitoring relaxation are enabled. LS to RAN2 on further agreements on RLM and BFD relaxation for UE power saving enhancements (R4-2120314)

#### 2.4.2	Remaining open issues

With the above, the following are remaining open issues for RLM/BFD relaxation in RAN4:

- Good serving cell quality criteria, including

  - Predefined or configured threshold

  - Good serving cell quality criteria for RLM

  - Good serving cell quality criteria for BFD

- Relaxed evaluation period of RLM/BFD, including relaxation factors and the lower bound

## 2.5	RAN5

#### 2.5.1	Agreements: N/A (RAN5 is not involved in the WI)

#### 2.5.2	Remaining Open issues: N/A

#### 2.5.3	Remaining Open issues with cross-WG dependencies: N/A

## 2.6	RAN6

#### 2.6.1	Agreements: N/A (RAN6 is not involved in the WI)

#### 2.6.2	Remaining Open issues: N/A

## 3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

NOTE: This section only needs to be filled in for WI/SIs where there is a corresponding relevant WI/SI in SA/CT.

## 3.1	SAx/CTs

#### 3.1.1	Agreements with cross-TSG impacts: N/A

#### 3.1.2	Remaining Open issues with cross-TSG impacts: N/A

## 4.	References

- Previous status report(s) RP-212612 Status report for WI_UE Power Saving Enhancements for NR Rapporteur (MediaTek) RP-211452 Status report for WI_UE Power Saving Enhancements for NR Rapporteur (MediaTek) RP-210733 Status report for WI_UE Power Saving Enhancements for NR Rapporteur (MediaTek) RP-202685 Status report for WI_UE Power Saving Enhancements for NR Rapporteur (MediaTek) RP-201701 Status report for WI_UE Power Saving Enhancements for NR Rapporteur (MediaTek)

RAN1 Contributions

RAN1 #106-bis-e Meeting - General: R1-2110616 Session notes for 8.7 (UE Power Saving Enhancements) Ad-hoc Chair (CMCC) R1-2110607 [Draft] Reply LS on UE Power Saving Moderator (MediaTek) R1-2110608 Reply LS on UE Power Saving RAN1, MediaTek R1-2110679 Summary of email discussion on Rel-17 RRC parameters for UE power saving Moderator (MediaTek) R1-2110695 Collection of RAN1-related agreements for UE power saving enhancements for NR Rapporteur (MediaTek) - Potential paging enhancements: R1-2109582 Summary #1 of paging enhancements MediaTek Inc. R1-2110504 Summary#2 of paging enhancements Moderator (MediaTek) R1-2110539 Summary#3 of paging enhancements Moderator (MediaTek) R1-2110606 Summary#4 of paging enhancements Moderator (MediaTek) R1-2110671 Summary#5 of paging enhancements Moderator (MediaTek) R1-2108744 Paging enhancements for UE power saving in IDLE/inactive mode Huawei, HiSilicon R1-2108836 Paging enhancements for idle/inactive UE power saving TCL Communication Ltd. R1-2108864 Discussion on power saving enhancements for paging ZTE, Sanechips R1-2108888 Discussion on PEI Design Transsion Holdings R1-2108916 Discussion on potential paging enhancements for UE power saving Spreadtrum Communications R1-2108985 Paging enhancements for idle/inactive mode UE power saving vivo R1-2108987 Discussion on paging grouping vivo R1-2109085 Further discussion on Paging enhancements for power saving OPPO R1-2109235 Paging enhancement for UE power saving CATT R1-2109237 Multiple POs associated with one PEI CATT R1-2109292 Discussion on paging early indication design CMCC R1-2109422 Paging enhancement for power saving Xiaomi R1-2109500 Discussion on paging enhancements Samsung R1-2109581 On PDCCH-based PEI for paging enhancements MediaTek Inc. R1-2109621 On remaining details of paging early indication design Intel Corporation R1-2109689 Discussion on paging enhancement NTT DOCOMO, INC. R1-2109797 Paging enhancements for idle/inactive UE power saving Sony R1-2109855 On paging enhancement Panasonic R1-2109944 Paging enhancement for UE power saving Lenovo, Motorola Mobility R1-2109952 Discussion on paging enhancements for UE power saving InterDigital, Inc. R1-2109980 Discussion on potential paging enhancements LG Electronics R1-2110043 Paging enhancements for idle/inactive-mode UE Apple R1-2110136 Design of Paging Enhancements Ericsson R1-2110197 Paging enhancements for idle/inactive UE power saving Qualcomm Incorporated R1-2110199 Aligning Paging Occasions to SSB for UE idle and inactive mode power saving Qualcomm Incorporated R1-2110283 On paging early indication Nordic Semiconductor ASA R1-2110311 On paging enhancements for UE power saving Nokia, Nokia Shanghai Bell R1-2110315 Design of PDCCH-based Paging Early Indication DENSO CORPORATION R1-2110400 Discussion on paging enhancement NTT DOCOMO, INC. - TRS/CSI-RS occasion(s) for idle/inactive UEs R1-2109502 Moderator Summary#1 for TRS/CSI-RS occasion(s) for idle/inactive UEs Moderator (Samsung) R1-2110483 Moderator Summary #2 on TRS/CSI-RS occasion(s) for idle/inactive UEs Moderator (Samsung) R1-2110484 Moderator Summary #3 on TRS/CSI-RS occasion(s) for idle/inactive UEs Moderator (Samsung) R1-2110580 Moderator Summary #4 on TRS/CSI-RS occasion(s) for idle/inactive UEs Moderator (Samsung) R1-2110581 Final Moderator Summary on TRS/CSI-RS occasion(s) for idle/inactive UEs Moderator (Samsung) R1-2108745 Assistance RS occasions for IDLE/inactive mode Huawei, HiSilicon R1-2108837 TRS/CSI-RS occasions for IDLE/inactive mode TCL Communication Ltd. R1-2108865 TRS for RRC idle and inactive UEs ZTE, Sanechips R1-2108917 Discussion on TRS/CSI-RS occasion(s) for idle/inactive UEs Spreadtrum Communications R1-2108986 TRS/CSI-RS occasion(s) for idle/inactive UEs vivo R1-2109086 Further discussion on RS occasion for idle/inactive UEs OPPO R1-2109236 Configuration of TRS/CSI-RS for paging enhancement CATT R1-2109293 Discussion on TRS/CSI-RS occasion(s) for IDLE/INACTIVE-mode UEs CMCC R1-2109423 On TRS/CSI-RS configuration and indication for idle/inactive UEs Xiaomi R1-2109501 Discussion on TRS/CSI-RS occasion(s) for idle/inactive UEs Samsung R1-2109583 On TRS/CSI-RS occasion(s) for idle/inactive mode UE power saving MediaTek Inc. R1-2109622 Discussion on periodic TRS occasions in idle/inactive mode Intel Corporation R1-2109690 Discussion on TRS/CSI-RS occasion for idle/inactive Ues NTT DOCOMO, INC. R1-2109798 On TRS/CSI-RS occasion(s) for idle/inactive UEs Sony R1-2109856 Potential enhancements for TRS/CSI-RS occasion(s) for idle/inactive UEs Panasonic R1-2109945 Provision of TRS/CSI-RS for idle/inactive UEs Lenovo, Motorola Mobility R1-2109953 Remaining issues on TRS/CSI-RS occasion(s) for idle/inactive UEs InterDigital, Inc. R1-2109981 Discussion on TRS/CSI-RS occasion(s) for idle/inactive UEs LG Electronics R1-2109999 Discussion on TRS/CSI-RS occasions for idle/inactive UEs Sharp R1-2110044 Indication of TRS configurations for idle/inactive-mode UE power saving Apple R1-2110137 Provisioning TRS occasions to Idle/Inactive UEs Ericsson R1-2110198 TRS/CSI-RS for idle/inactive UE power saving Qualcomm Incorporated R1-2110284 On TRS design for idle/inactive UEs Nordic Semiconductor ASA R1-2110312 On RS information to IDLE/Inactive mode Ues Nokia, Nokia Shanghai Bell - Potential extension(s) to Rel-16 DCI-based power saving adaptation during DRX Active Time R1-2110406 FL summary#1 of DCI-based power saving adaptation Moderator (vivo) R1-2110407 FL summary#2 of DCI-based power saving adaptation Moderator (vivo) R1-2110517 FL summary#3 of DCI-based power saving adaptation Moderator (vivo) R1-2110518 FL summary#4 of DCI-based power saving adaptation Moderator (vivo) R1-2108746 Extensions to Rel-16 DCI-based power saving adaptation for an active BWP Huawei, HiSilicon R1-2108867 Extension to Rel-16 DCI-based power saving adaptation during DRX Active Time ZTE, Sanechips R1-2108918 Discussion on power saving techniques for connected-mode UEs Spreadtrum Communications R1-2108988 Discussion on DCI-based power saving adaptation in connected mode vivo R1-2109087 DCI-based power saving adaptation solutions OPPO R1-2109238 PDCCH monitoring adaptation CATT R1-2109239 PDCCH skipping and SS group switching CATT R1-2109294 Discussion on PDCCH monitoring reduction during DRX active time CMCC R1-2109361 Discussion on DCI-based power saving adaptation NEC R1-2109503 Discussion on DCI-based power saving techniques Samsung R1-2109584 On enhancements to DCI-based UE power saving during DRX active time MediaTek Inc. R1-2109623 On remaining issues of PDCCH monitoring adaptation in active time Intel Corporation R1-2109691 Discussion on extension to DCI-based power saving adaptation NTT DOCOMO, INC. R1-2109813 DCI-based power saving adaptation during DRX active time ETRI R1-2109831 Discussion on extension(s) to Rel-16 DCI-based power saving adaptation FGI, Asia Pacific Telecom R1-2109857 Potential extension(s) to Rel-16 DCI-based power saving adaptation during DRX ActiveTime Panasonic R1-2109946 Enhanced DCI based power saving adaptation Lenovo, Motorola Mobility R1-2109955 DCI-based power saving adaptation during DRX ActiveTime InterDigital, Inc. R1-2109982 Discussion on DCI-based power saving adaptation during DRX ActiveTime LG Electronics R1-2110045 Enhanced DCI-based power saving adaptation Apple R1-2110130 A common framework for SSSG switching and PDCCH skipping ASUSTeK R1-2110139 Design of active time power savings mechanisms Ericsson R1-2110200 DCI-based power saving adaptation during DRX ActiveTime Qualcomm Incorporated R1-2110285 On PDCCH monitoring adaptation Nordic Semiconductor ASA R1-2110310 DCI-based Power Saving Enhancements Fraunhofer HHI, Fraunhofer IIS R1-2110313 UE power saving enhancements for Active Time Nokia, Nokia Shanghai Bell - Others R1-2108866 Additional simulation results of UE power consumption in RRC idle and inactive state ZTE, Sanechips R1-2108868 Further discussion on potential power saving schemes for RRC connected UEs ZTE, Sanechips R1-2109747 Analysis on power consumption for IDLE mode UE Huawei, HiSilicon R1-2109748 Other considerations on power saving in Rel-17 Huawei, HiSilicon R1-2109954 Wake-up signal for low power wake-up radio InterDigital, Inc. R1-2110138 Modeling of NR gNB power consumption Ericsson R1-2110140 Evaluation results for UE power saving schemes Ericsson

RAN1 #106-bis-e Meeting - Rel-17 CR: R1-2112449 Introduction of UE power savings enhancements in NR Samsung R1-2112474 Introduction of Rel-17 UE power saving enhancements Huawei R1-2112491 Introduction of NR UE Power Saving Enhancements Nokia - General: R1-2112794 Session notes for 8.7 (UE Power Saving Enhancements) Ad-hoc Chair (CMCC) R1-2112876 Session notes for 8.7 (UE Power Saving Enhancements) Ad-hoc Chair (CMCC) R1-2112887 Summary of email discussion on Rel-17 RRC parameters for UE power saving Moderator (MediaTek Inc.) - Potential paging enhancements: R1-2112307 Summary #1 of paging enhancements MediaTek Inc. R1-2112662 Summary#2 of paging enhancements Moderator (MediaTek Inc.) R1-2112886 Summary#3 of paging enhancements Moderator (MediaTek Inc.) R1-2110837 Paging enhancements for UE power saving in IDLE/inactive mode Huawei, HiSilicon R1-2110937 Discussion on power saving enhancements for paging ZTE, Sanechips R1-2111023 Remaining issues on paging enhancements for idle/inactive mode UE power saving vivo R1-2111062 Paging enhancements for idle/inactive UE power saving TCL Communication Ltd. R1-2111103 Discussion on potential paging enhancements for UE power saving Spreadtrum Communications R1-2111266 Paging enhancement for UE power saving CATT R1-2111268 Multiple POs associated with one PEI CATT R1-2111325 Further discussion on Paging enhancements for power saving OPPO R1-2111404 Remaining issues on potential paging enhancement Sony R1-2111504 On remaining aspects of paging early indication design Intel Corporation R1-2111582 Paging enhancement for power saving Xiaomi R1-2111617 Discussion on paging early indication design CMCC R1-2111674 Discussion on PEI Design Transsion Holdings R1-2111675 On paging enhancement Panasonic R1-2111747 Discussion on paging enhancements Samsung R1-2111884 Paging enhancements for idle/inactive-mode UE Apple R1-2111945 Paging enhancement for UE power saving Lenovo, Motorola Mobility R1-2111960 Discussion on paging enhancements for UE power saving InterDigital, Inc. R1-2112060 Discussion on potential paging enhancements LG Electronics R1-2112116 Discussion on paging enhancement NTT DOCOMO, INC. R1-2112149 Design of Paging Enhancements Ericsson R1-2112226 Paging enhancements for idle/inactive UE power saving Qualcomm Incorporated R1-2112228 Aligning Paging Occasions to SSB for UE idle and inactive mode power saving Qualcomm Incorporated R1-2112308 Remaining design details of paging early indication MediaTek Inc. R1-2112370 Open items on paging enhancements for UE power saving Nokia, Nokia Shanghai Bell R1-2112379 On paging early indication Nordic Semiconductor ASA - TRS/CSI-RS occasion(s) for idle/inactive UEs R1-2111749 Moderator Summary#1 on TRS/CSI-RS occasion(s) for idle/inactive UEs Moderator (Samsung) R1-2112682 Moderator Summary #2 on TRS/CSI-RS occasion(s) for idle/inactive UEs Moderator (Samsung) R1-2112683 Moderator Summary #3 on TRS/CSI-RS occasion(s) for idle/inactive UEs Moderator (Samsung) R1-2110838 Assistance RS occasions for IDLE/inactive mode Huawei, HiSilicon R1-2110938 TRS for RRC idle and inactive UEs ZTE, Sanechips R1-2111024 Remaining issues on TRS/CSI-RS occasion(s) for idle/inactive UEs vivo R1-2111063 TRS/CSI-RS occasions for IDLE/inactive mode TCL Communication Ltd. R1-2111104 Discussion on TRS/CSI-RS occasion(s) for idle/inactive UEs Spreadtrum Communications R1-2111267 Configuration of TRS/CSI-RS for paging enhancement CATT R1-2111326 Further discussion on RS occasion for idle/inactive UEs OPPO R1-2111405 Remaining issues on TRS occasion(s) for idle/inactive UEs Sony R1-2111505 Discussion on TRS occasions in idle/inactive mode Intel Corporation R1-2111583 On TRS/CSI-RS configuration and indication for idle/inactive UEs Xiaomi R1-2111618 Discussion on TRS/CSI-RS occasion(s) for IDLE/INACTIVE-mode UEs CMCC R1-2111676 Potential enhancements for TRS/CSI-RS occasion(s) for idle/inactive UEs Panasonic R1-2111748 Discussion on TRS/CSI-RS occasion(s) for idle/inactive UEs Samsung R1-2111885 Indication of TRS configurations for idle/inactive-mode UE power saving Apple R1-2111946 Provision of TRS/CSI-RS for idle/inactive UEs Lenovo, Motorola Mobility R1-2111961 Remaining issues on TRS/CSI-RS occasion(s) for idle/inactive UEs InterDigital, Inc. R1-2112018 Discussion on TRS/CSI-RS occasions for idle/inactive UEs Sharp R1-2112061 Discussion on TRS/CSI-RS occasion(s) for idle/inactive UEs LG Electronics R1-2112117 Discussion on TRS/CSI-RS occasion for idle/inactive UEs NTT DOCOMO, INC. R1-2112150 Provisioning TRS occasions to Idle/Inactive UEs Ericsson R1-2112227 TRS/CSI-RS for idle/inactive UE power saving Qualcomm Incorporated R1-2112309 On TRS/CSI-RS occasion(s) for idle/inactive mode UE power saving MediaTek Inc. R1-2112371 Open issues on TRS information for IDLE/INACTIVE mode UEs Nokia, Nokia Shanghai Bell R1-2112380 On TRS design for idle/inactive UEs Nordic Semiconductor ASA - Potential extension(s) to Rel-16 DCI-based power saving adaptation during DRX Active Time R1-2112578 FL summary#1 of DCI-based power saving adaptation Moderator (vivo) R1-2112579 FL summary#2 of DCI-based power saving adaptation Moderator (vivo) R1-2112769 FL summary#3 of DCI-based power saving adaptation Moderator (vivo) R1-2112878 Final FL summary of DCI-based power saving adaptation Moderator (vivo) R1-2110840 Extensions to Rel-16 DCI-based power saving adaptation for an active BWP Huawei, HiSilicon R1-2110940 Extension to Rel-16 DCI-based power saving adaptation during DRX Active Time ZTE, Sanechips R1-2111026 Remaining issues on DCI-based power saving adaptation in connected mode vivo R1-2111105 Discussion on power saving techniques for connected-mode UEs Spreadtrum Communications R1-2111181 Discussion on DCI-based power saving adaptation NEC R1-2111269 PDCCH monitoring adaptation CATT R1-2111270 PDCCH skipping and SS group switching CATT R1-2111327 DCI-based power saving adaptation solutions OPPO R1-2111506 Discussion on DCI-based power saving adaptation in active time Intel Corporation R1-2111619 Discussion on PDCCH monitoring reduction during DRX active time CMCC R1-2111677 Potential extension(s) to Rel-16 DCI-based power saving adaptation during DRX ActiveTime Panasonic R1-2111750 Discussion on DCI-based power saving techniques Samsung R1-2111886 Enhanced DCI-based power saving adaptation Apple R1-2111947 Enhanced DCI based power saving adaptation Lenovo, Motorola Mobility R1-2111962 DCI-based power saving adaptation during DRX Active Time InterDigital, Inc. R1-2111992 Remaining issues on DCI-based power saving adaptation during DRX active time ETRI R1-2112062 Discussion on DCI-based power saving adaptation during DRX ActiveTime LG Electronics R1-2112075 Discussion on extension(s) to Rel-16 DCI-based power saving adaptation FGI, Asia Pacific Telecom R1-2112118 Discussion on extension to DCI-based power saving adaptation NTT DOCOMO, INC. R1-2112152 Design of active time power savings mechanisms Ericsson R1-2112229 DCI-based power saving adaptation during DRX ActiveTime Qualcomm Incorporated R1-2112310 On enhancements to DCI-based UE power saving during DRX active time MediaTek Inc. R1-2112372 Open issues on PDCCH monitoring adaptation for UE power saving Nokia, Nokia Shanghai Bell R1-2112381 On PDCCH monitoring adaptation Nordic Semiconductor ASA R1-2112397 DCI-based Power Saving Enhancements Fraunhofer HHI, Fraunhofer IIS - Others R1-2110939 Additional simulation results of UE power consumption in RRC idle and inactive state ZTE, Sanechips R1-2110941 Further discussion on potential power saving schemes for RRC connected UEs ZTE, Sanechips R1-2110839 Analysis on power consumption for IDLE mode UE Huawei, HiSilicon R1-2111930 Other considerations on power saving in Rel-17 Huawei, HiSilicon R1-2112151 Modeling of NR gNB power consumption Ericsson R1-2112153 Evaluation results for UE power saving schemes Ericsson

RAN2 Contributions

RAN2 #116-e Meeting CRs: R2-2110975 38.300 running CR for introduction of UE power saving enhancements Huawei, HiSilicon R2-2111491 38.300 running CR for introduction of UE power saving enhancements Huawei, HiSilicon LS in: R2-2109337 LS on RAN3 work associated with UE Power Saving (R3-214281; contact: Nokia) RAN3 R2-2109362 LS on criteria for RLM/BFD relaxation (R4-2115349; contact: vivo & MediaTek) RAN4 R2-2111234 LS Reply on UE Power Saving (S2-2107856; contact: Huawei) SA2 R2-2111247 Reply LS on UE Power Saving (R1-2110608; contact: MediaTek) RAN1 LS out: R2-2111415 LS out on paging subgrouping and PEI RAN2 Architecture UE subgrouping R2-2109647 Summary of [Post115-e][089][ePowSav] Paging Subgrouping Beijing Xiaomi Mobile R2-2111524 Summary of [AT116-e][045][ePowSav] Paging Subgrouping (Xiaomi) Xiaomi Communications (email discussion rapporteur) R2-2109490 Discussion on grouping-based paging OPPO R2-2109520 Further details of UE Subgrouping Samsung Electronics Co., Ltd R2-2109736 Architecture for paging enhancement by UE subgrouping vivo R2-2109880 Further considerations for subgrouping Intel Corporation R2-2110402 Further Consideration on Paging Subgrouping CATT R2-2110413 CN assigned paging subgroups Ericsson R2-2110481 Consideration on Idle/inactive-mode UE power saving Lenovo, Motorola Mobility R2-2110538 General scenario consideration on paging subgrouping Huawei, HiSilicon R2-2110618 Further Consideration on supporting CN Assigned Subgrouping ZTE Corporation,Sanechips R2-2110967 UE Paging Subgroup Assignment MediaTek Inc. R2-2111032 Remaining details on subgrouping Nokia, Nokia Shanghai Bell UE ID and CN assigned subgroup R2-2110545 On the co-existence of UE-ID and CN assigned subgroups Interdigital, Inc. R2-2110792 On supporting both CN-assigned subgrouping and UEID-based subgrouping in a same cell Futurewei Technologies R2-2111074 Considerations on UE-ID based subgrouping CMCC UE assistance information R2-2111535 Summary of [AT116-e][034][ePowSav] UE assistance for CN subgroups CMCC R2-2110546 UE assistance for CN assigned subgroups Interdigital, Inc. R2-2111073 Considerations on assistance information and signaling for paging subgrouping CMCC Control and Procedure details Subgrouping R2-2109455 Subgrouping for paging occasions Qualcomm Incorporated R2-2109737 UE subgrouping procedure for paging enhancement vivo R2-2109779 Further discussion on CN-assigned paging grouping Transsion Holdings R2-2110051 NR UE Power Save Paging Subgrouping aspects Apple R2-2110352 Discussion on paging subgroupingenhancements for idle/inactive-mode UE power saving Sony R2-2110380 CN assigned subgroup LG Electronics Inc. R2-2110381 UE ID based subgroup LG Electronics Inc. R2-2110482 Considerations on the configuration for UE paging grouping Lenovo, Motorola Mobility R2-2110539 Detailed design on paging subgrouping Huawei, HiSilicon R2-2110547 Subgroup determination Interdigital, Inc. R2-2110620 Consideration on the paging enhancement for idle or inactive UE ZTE Corporation,Sanechips R2-2110968 Paging Monitoring with UE Subgrouping MediaTek Inc. PEI R2-2111562 Summary of [AT116-e][046][ePowSav] Paging Early Indication Ericsson R2-2109453 PEI configuration and monitoring procedures Qualcomm Incorporated R2-2109491 Discussion on PEI monitoring OPPO R2-2109521 UE Idenity for paging subgrouping Samsung Electronics Co., Ltd R2-2110415 PEI monitoring in last used cell Ericsson, Vodafone R2-2111135 Remaining issues on PEI monitoring Beijing Xiaomi Mobile Capability R2-2111033 UE and NW capabilities on subgrouping Nokia, Nokia Shanghai Bell Other R2-2109522 DRX cycle for monitoring paging Samsung Electronics Co., Ltd Other aspects RAN2 impacts UE capability R2-2109878 Consideration of UE capability for Rel-17 UE power saving Intel Corporation General R2-2109493 Power saving enhancement for connected mode UE OPPO R2-2109523 Other RAN2 aspects of UE Power Saving Samsung Electronics Co., Ltd R2-2110619 Initial Consideration on DCI based Power Saving ZTE Corporation,Sanechips R2-2110414 Other aspects on UE power saving Ericsson R2-2111034 RAN2 impact on connected mode power saving Nokia, Nokia Shanghai Bell TRS CSI-RS configuration for Idle and Inactive R2-2111285 Summary of agenda 8.9.3: Other aspects RAN2 impacts - TRS CSI-RS for RRC-IDLE and RRC-INACTIVE Apple R2-2109492 Discussion on signaling aspects of TRS/CSI-RS occasion(s) for idle/inactive Ues OPPO R2-2109648 Discussion on TRS CSI-RS for RRC-IDLE and RRC-INACTIVE State UE Beijing Xiaomi Mobile R2-2109738 Discussion on TRS CSI-RS in idle inactive mode vivo R2-2110052 TRS/CSI-RS Signaling Aspects for IDLE/INACTIVE UEs Apple R2-2110335 TRS/CSI-RS configuration for Idle/inactive mode UE Lenovo, Motorola Mobility R2-2110353 Discussion on dedicated signaling of TRS/CSI-RS configuration Sony R2-2110403 TRS/CSI-RS SI update mechanism for DRX and eDRX Ues CATT R2-2110416 Provisioning of TRS occasions to Idle and Inactive UEs Ericsson R2-2110540 Discussion on potential TRS/CSI-RS Huawei, HiSilicon R2-2110820 Potential TRS/CSI-RS occasion(s) Nokia, Nokia Shanghai Bell RLM BFD relaxation R2-2111528 Summary of [AT116-e][036][ePowSav] RLM/BFD relaxation (Xiaomi) Xiaomi R2-2109454 Criteria and configuration for BFD relaxations Qualcomm Incorporated R2-2109879 Signalling aspect on criteria of RLM/BFD relaxation Intel Corporation R2-2109739 RAN2 impact on RLM/BFD relaxation for power saving vivo R2-2110194 Discussion on RLM_BFD measurement relaxation Xiaomi Communications R2-2110541 Discussion on criteria for the RLM/BFD relaxation Huawei, HiSilicon R2-2110404 Configurations for RLM/BFD Relaxation CATT

RAN4 Contributions

RAN4 #101-e Meeting General: R4-2120313 WF on RLM/BFD relaxation for UE Power Saving enhancements MediaTek Inc. R4-2120221 Email discussion summary for [101-e][226] NR_UE_pow_sav_enh Moderator (MediaTek) R4-2120368 Email discussion summary for [101-e][226] NR_UE_pow_sav_enh Moderator (MediaTek) R4-2118419 Simulation results for RLM+BFD measurement relaxation Nokia, Nokia Shanghai Bell R4-2119216 General issues for Rel-17 power saving WI MediaTek inc. LS out: R4-2120314 LS on further agreements on RLM and BFD relaxation for UE Power Saving enhancements vivo RRM core requirements: R4-2117624 On Power Saving RRM Requirement Qualcomm, Inc. R4-2118022 Discussions on UE power saving for RLM and BM Intel Corporation UE measurements relaxation for RLM and/or BFD: R4-2117298 UE measurements relaxation for RLM and/or BFD Apple R4-2117370 Further discussion on RLM/BFD relaxation for UE power saving enhancement CATT R4-2117743 Discussion on RLM/BFD relaxation for NR power saving enhancement CMCC R4-2117812 Further discussion on UE measurements relaxation for RLM and/or BFD Xiaomi R4-2118257 Discussion on R17 RLM and BFD relaxation for NR vivo R4-2118366 Discussion on RRM requirements for R17 RLM/BFD relaxation OPPO R4-2118418 Discussion about RLM/BFD measurement relaxation Nokia, Nokia Shanghai Bell R4-2118835 Discussion on UE measurement relaxation for RLM/BFD Huawei, Hisilicon R4-2118934 RLM and RLF relaxation for UE power saving ZTE Corporation R4-2119065 Discussions on UE power saving for RLM and BM Ericsson R4-2119217 Discussion on Rel-17 RLM/BFD measurement relaxation MediaTek Inc.