3GPP TSG-RAN Meeting # 94-e 	RP-213384

Electronic Meeting, Nov. 1-12, 2021

Source:	Ericsson

Title:	Considerations on Extended CP for Rel-18 MBS

Agenda item:	8A.2

Document for:	Discussion

# 1	Introduction

In this contribution we discuss and analyze the need for Extended Cyclic Prefix (ECP) in NR MBSFN.

# 2	Discussion

For the general case of MBSFN we have to have:

CP ≥ ΔTsync + ΔTpropagation + Tchannel		(1)

where:

CP : 			Cyclic prefix

ΔTsync : 		Cell Phase Synchronization

ΔTpropagation : 	The difference between frame start of carriers received from different sites

Tchannel : 		The radio channel dispersion

In LTE TS 36.133 the existing rel-16 specification looks like this:

## 7.25	Cell phase synchronization accuracy for MBMS services (FDD)

### 7.25.1	Definition

Cell phase synchronization accuracy is defined as the maximum absolute deviation in frame start timing between any pair of cells that have overlapping coverage areas in the same MBSFN area.

### 7.25.2	Minimum requirements

For eNodeB capable of supporting MBMS services, the cell phase synchronization accuracy measured at BS antenna connectors shall be better than the requirement specified in table 7.25.2-1.

Table 7.25.2-1: Cell phase synchronization requirement for MBMS services (FDD)

| CP length for MBSFN subframe | Requirement |
| --- | --- |
| 16.67 µs | £ 5 ms |
| 33.33 µs | £ 5 ms |
| 200 µs | £ 5 ms |
| 100 µs | £ 5 ms |
| 300 µs | £ [5] ms |

Note 1: 	When MBSFN subframe using is configured for a MBSFN area, the CP length for MBSFN subframes is 16.67µs. When MBSFN subframe using  is configured for a MBSFN area, the CP length for MBSFN subframes is 33.33µs.When MBSFN subframe using  is configured for a MBSFN area, the CP length for MBSFN subframes is 200µs. When MBSFN subframe using Δf=2.5 kHz is configured for a MBSFN area, the CP length for MBSFN subframes is 100 µs. When MBSFN subframe using Δf=370.73 Hz is configured for a MBSFN area, the CP length for MBSFN subframes is 300 µs.

The LTE case was analyzed in [1] where CP = ECP = 16.67 µs was used together with ΔTsync = 5 µs, 
ΔTpropagation = 6.67 µs (2 km) and Tchannel = 5 µs (Extended Typical Urban-ETU). Using Equation (1) we then get:

CP ≥ ΔTsync + ΔTpropagation + Tchannel

16.67 µs ≥ 5 µs + 6.67 µs + 5 µs

This analysis was based on the key observations that LTE MBMS shall:

- Not be limited to specific small areas with very symmetric RF propagation (small ΔTpropagation)

- Not be limited to areas with small channel delay spread (Tchannel)

- Avoid very strict sync which would not be compatible with e.g.  LTE TDD (3 µs) and become a bottleneck for introducing the service.

Observation 1: LTE MBMS based on ECP is able to fit reasonable requirements for the difference between frame start of carriers received from different sites, Cell Phase Synchronization and radio channel dispersion.

For the case of NR we have for SCS = 15 kHz a CP = 144*64 Tc = 4.69 µs. This is very limiting to fit sum of difference between frame start of carriers + Cell Phase Synchronization + radio channel dispersion.

Observation 2: NR MBMS with CP = 144*64 Tc = 4.69 µs does not fit sum of difference between frame start of carriers + Cell Phase Synchronization + radio channel dispersion.

Proposal 1: Introduce Extended Cyclic Prefix (ECP), as an integral part of any rel-18 NR MBS WI.

# 3	Conclusion

Observation 1: LTE MBMS based on ECP is able to fit reasonable requirements for the difference between frame start of carriers received from different sites, Cell Phase Synchronization and radio channel dispersion.

Observation 2: NR MBMS with CP = 144*64 Tc = 4.69 µs does not fit sum of difference between frame start of carriers + Cell Phase Synchronization + radio channel dispersion.

Proposal 1: Introduce Extended Cyclic Prefix (ECP), as an integral part of any rel-18 NR MBS WI.

# References

- R4-1609530