3GPP TSG RAN Meeting #94-e                                             RP-213432

Electronic Meeting, Dec. 6 - 17, 2021

Source: 	Huawei, HiSilicon

Title: 	Motivation for Rel-18 new WI on enhancement of UE RF FR1

Agenda Item:	8A.4

Document for:	Information

# Background

In recent RAN email discussions on scope of the Rel-18 RAN4 topics, lots of enhancement for UE RF FR1 have been discussed. The latest information can be found in [1], and the part related to UE RF FR1 is copied as below:

Proposal #1: for UE FR1 requirements evolution, the following objectives can be considered - Enable 4Tx on a single band for FWA/CPE - Investigate framework and architecture - Example bands: n41, n77 and n78 (other bands to be introduced in the release indepent way later) - Specify the UE RF requirements to support 4Tx - Specify the BS demodulation performance requirements to support 4Tx UE operation - Enable 8Rx for FWA/CPE on higher frequency bands (RAN4) - Consider NR TDD bands higher than 1.8GHz and example bands are n41, n77 and n78 (other bands to be introduced in the release indpendent way later) - Specify the UE RF requirements to support 8Rx - Specify RLM test cases with 8Rx - Specify UE demodulation performance and CSI requirements to support 8Rx - Investigate the feasiblity and if neccesary define requirements w/ MIMO layer larger than 4 - Investigate and enable 6Rx on higher frequency bands targeting at support of smartphone (RAN4) - Investigate the feasibility whether 6Rx can be extended to the smartphone, and decide which UE type (smartphone and/or FWA/CPE) will be considered - Feasibility study includes performance gain and form factor - Consider NR TDD bands higher than 1.8GHz and example bands are n41, n77 and n78 (other bands to be introduced in the release indpendent way later) - Specify the UE RF requirements to support 6Rx Specify the requirements to support 6Rx subject to the conclusion of feasiblity study - Specify RLM test cases with 6Rx - Specify UE demodulation performance and CSI requirements to support 6Rx Investigate the feasiblity and if necessary define requirements with MIMO layer larger than 4 - Investigate and introduce lower MSD for inter-band CA/EN-DC combinations (RAN4) - [FFS example band combinations - Study how the MSD performance can be improved for example band combiantions - Study whether the generic approach can be applied based on UE indication of capability - Study and if possible specify the requirements and/or capability signaling - Power class 2 (PC2) is considered] NOTE: for the lower MSD, the above objecives are subject to modifications based on RAN4 progress NOTE: for the above working areas (four big bullets), how to organize the projects in terms of WI and/or SI needs further discussion after stablizing the scope. Proposal #2: for simultaneous 3Tx working area under FR1 requirement topic, it is proposed - Drop the potention objective in uUse case #1: 3Tx on single uplink band for MIMO enhancement - FFS the potential objective of 3Tx for transmission diversity on single uplink band for power enhancement with the understanding that there is no RAN1 impact Proposal #3: for UE FR1 requirements evolution, the following objectives can be considered Investigate and if necesary, specify requirements to support simulatenous 3Tx considering two use cases (RAN4) 229 - 3Tx simultaenous transmissions with two bands with 1Tx on one uplink operating band and 2Tx on the other uplink operating band - Tx switching transmission enhancement (RAN2, RAN4) - Cover inter-band UL CA and SUL case ◇ FFS example band combinations - RAN2 signaling design - Identify and specify the necessary RF requirements - Power enhancement - FFS example band combination(s) - Study the definition of power class Both power class defined per band combination as total power and power class defined per band are considered - Identify and specify the other necessary RF requirements - [3Tx simulation transsmision for three bands inter-band UL CA with 1Tx on each uplink bands - Study the power class framework, impact on MPR/A-MPR and impact on MSD - Example band combinations - Consider TDD+TDD+TDD without simultaneous Tx/Rx - FFS other band combinations, e.g., FDD+FDD+TDD, FDD+TDD+TDD] Proposal #4: for UE FR1 requirements evolution, the following objectives can be considered Investigate and define UE requirements with smaller A-MPR (RAN4) - The example band is [n1] - Study how to define smaller A-MPR considering the following solutions - Generic approach meaning that a certain improvement is always applicable to A-MPR for any conditions, and/or - e.g., If 1dB improvement is possible, the 1dB is applied to all the specified A-MPR values regardless of position, length of RBs, modulation, waveform and/or CBW etc - A specific approach meaning that a certain improvement is applicable to A-MPR only for specifif condition, and/or - e.g., the conditions are tied with the position of the LO location - Other ways are not precluded The following working areas (main bullets) may need further discussions. - New low power class for Uu and/or sidelink (14dBm) - Simultaneous Rx/Tx for intra-band NC CA (REF: RP-211776) - Example scenario: two non-contiguous carriers in n79 - Study and if needed specify requirements to enable simultaneous Rx/Tx for Intra-band non-contiguous CA/DC in TDD band - Study the feasiblity on self-interference and deployment scenario Self-interference study to see the required MSD levels with MPR/A-MPR, UL RB restriction and frequency separation Deployment scenario including BS co-channel/non-collocation with single/multiple NR operator NOTE: in multiple NR operator deployment, consider restricted scenarios such as inter-operator deployment with Macro-to-Indoor and Indoor-to-indoor - Define the related RF requirements depending on the result of the feasiblity study. - UL Tx switching for UL CA with multiple TAG - Enhance UL CA RF requirements - Enable multiple TAG for TX switching (if not addressed already in Rel-17)

Based on the discussion, a new WI on further enhancement of UE RF FR1 should be approved.

# Proposed WI objectives

The proposed WI objectives are:

Core part:

The objectives of core part for Rel-18 RF FR1 requirement focus evolution include:

- Specify UE RF requirements for >2Tx on a single band including 4Tx for FWA or CPE [and/or 3Tx]

  - Investigate UE architecture to support >2Tx

  - Decide the framework including which transmission scheme is used as baseline to specify the requirements

- Specify UE RF requirements for >4Rx including 8Rx [and 6Rx] for FWA or CPE

  - Rx RF requirements: applicable bands, and corresponding reference sensitivity

- Enhance RF requirements for UL CA and EN-DC

  - Support PC1.5 and PC1 (if needed) for inter-band UL CA and intra-band UL CA

      - Investigate the scheduling restriction

  - Support inter-band UL CA with UL-MIMO on one band

  - Power enhancement for inter-band combinations

- Investigate and improve MSD for inter-band CA/EN-DC combinations

- Investigate and improve UE RF performance of A-MPR

- [Investigate and specify the UE power class <23dBm]

- [Investigate Simultaneous Rx/Tx for intra-band NC CA]

- [Investigate UL Tx switching for UL CA with multiple TAG]

Perf part:

The objectives of performance part for Rel-18 RF FR1 requirement focus evolution include:

- Specify release independence requirements in TS 38.307.

- Specify RRM performance requirements if needed.

- Specify BS and UE demodulation performance requirements if needed.

# References

[1] [RAN94e-R18Prep-22] Moderator summary for discussion of potential RAN4 enhancements