3GPP TSG RAN Meeting #94-e									RP-213473

Electronic Meeting, Dec 6 - 17, 2021

Status Report to TSG

Agenda item:			9.3.3.4

| WI / SI Name | NR QoE management and optimizations for diverse services |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: Yes | Performance part: No |  | Testing part: No |
| Acronym | NR_QoE-Core |  |  |  |  |
| Unique ID | 911008 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-211406 |  |  |  |  |
| Target Completion Date (indicate if changed) | Work Item: N/A | Core part: 03/2022 | Performance part: N/A | Testing part: N/A |  |
| Overall Completion level | Study Item: N/A | Core part: 60% | Performance Part: N/A | Testing part: N/A |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | RAN3 |
| --- | --- | --- |
| Rapporteur | Name | Gen Cao |
|  | Company | China Unicom |
|  | Email | caogen@chinauincom.cn |

1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

2.	Detailed progress in RAN WGs for all TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

2.1	RAN1

2.1.1	Agreements

2.1.2	Remaining Open issues

2.2	RAN2

2.2.1	Agreements

RAN2#113b-e (April 2021)

- Configuration architecture general aspect

	- Configure QoE measurements for NR in RRCReconfiguration.

	- Add configuration of QoE measurements in OtherConfig in RRCReconfiguration.

	- Add the configuration of QoE measurements by means of list to enable configuration of multiple simultaneous measurements.

	- R2 assumes that for RRC an ID is required to identify a measurement.

	- Define SRB4 for transmission of QoE reports in NR.

	- Define an RRC message MeasReportAppLayer for the transmission of QoE reports in NR.

	- RAN2 assumes that QoE support for NR includes (as the LTE framework): activation by Trace Function, both signalling and management-based configuration and RRC procedures supporting AppLayer config and report.

	- From RAN2 point of view, the UE shall follow gNB commands and, NG-RAN can in principle release by RRC the application layer measurement configuration towards the UE at any time, e.g. if required due to load or other reasons (Note that other WGs are responsible to define the normal system procedures for release and which nodes are responsible etc).

	- The UE Inactive AS context includes the UE AS configuration for the QoE (it is not released when UE goes to Inactive).

- Start and Stop

	- “QoE pause” indication from the network is used to temporarily stop QoE reports from being sent from the UE to the network. Application layer behaviour upon UE receiving “pause/resume” indications is out of RAN2 scope.

	- The following are options considered by RAN2 for QoE report handling during RAN overload via “QoE report pause indication”:

Option 1: Application layer is responsible for storing QoE reports when the UE receives QoE pause indication.

Option 2: AS layer is responsible for storing QoE reports when the UE receives QoE pause indication.

Option 3: The QoE container received from application layer is discarded during pause.

- Support for Mobility

None

- Radio-related measurements and information for QoE

None

- RAN visible QoE

None

RAN2#114-e (May 2021)

- Configuration architecture general aspect

	- gNB can release a list of QoE measurement configurations in one RRCReconfiguration message.

	- If a QoE measurement configuration is released, RRC layer informs the upper layer to release the QoE measurement configuration. This could be revisited based on other issues’ progress.

	- If the UE enters IDLE state, UE should release all of the QoE measurement configurations.

	- QoE configuration and report are encapsulated in a transparent container in the RRC messages.

	- At lease service type and RRC level ID (Reference ID or shorten ID) together with corresponding QMC configuration container should be included for each QoE configuration in RRCReconfiguration message when the network setups QoE measurement to the UE.

	- At least RRC level ID (Reference ID or shorten ID) together with corresponding QMC report container should be included in MeasReportAppLayer message for each QoE report.

	- RAN2 confirms logged MDT framework for QoE data retrieval and reporting is not supported in Rel-17.

- Start and Stop

	- At reception of QoE release, the UE shall discard any unsent QoE reports corresponding to the released QoE configuration.

- Support for Mobility

None

- Radio-related measurements and information for QoE

None

- RAN visible QoE

None

RAN2#115-e (Aug. 2021)

- Configuration architecture general aspect

- It is the RAN2 understanding the RRC ID, MeasConfigAppLayerId, is sufficient to identify the QoE configuration between UE and gNB.

	- RAN2 assumes that gNB keeps the mapping between MeasConfigAppLayerId and QoE Reference. The mapping is sent to the target gNB as part of QoE configuration and information at handover.

	- Confirm that RAN2 deprioritizes QoE measurement in RRC_IDLE/RRC_INACTIVE in Rel-17.

- Start and Stop

No contributions were treated in this RAN2 meeting.

- Support for Mobility

	- RAN2 assumes that all QoE mobility related agreements made by RAN2 are applicable at least to signalling based QoE.

	- Area scope parameter is not introduced in RRC procedures supporting QoE.

	- When the UE resumes the connection in a gNB supporting QoE, the target gNB should explicitly indicate which QoE measurement configurations should be kept by the UE during RRC resume procedure, e.g. in RRCResume message. The UE shall release all QoE measurement configurations not indicated by the gNB for restoration.

	- During the handover to target gNB which supports QoE, the target gNB decides which QoE configurations to keep and which to release during a handover, e.g. based on QoE configuration information received from the source gNB in Xn/Ng signalling (exact information is up to RAN3) including the RRC container.

	- The UE discards the reports received from application layer in case it has no associated QoE configuration configured.

	- In case the UE resumes the connection in a gNB not supporting QoE, the UE should release all QoE measurement configurations.

- Radio-related measurements and information for QoE

None

- RAN visible QoE

None

RAN2#116-e (Nov. 2021)

- Configuration architecture general aspect

- Forward the measConfigAppLayerId from the AS layer to the application layer together with the QoE configuration.

- Forward the measConfigAppLayerId from the application layer to the AS layer together with the QoE report.

- Support RRC segmentation for the Reporting

- Reply to SA4 that the size limitation of the QoE report has chanegd. RAN2 has agreed to optionally support RRC segmentation for transmission of QoE reports, and we indicate the new limits

- Size limit of QoE configuration = size of one PDCP SDU.

- Inform CT1 and SA4 of these agreements and ask them to specify the measConfigAppLayerId (e.g. in AT command). Can also discuss whether we need to have an action related to size limitation (whether to inform application of the size that is supported).

- Start and Stop

- We go with selective pause resume (with the understanding that we will not work further on the information the gNB may use for election).

- Support for Mobility

- An LS will be sent to SA4 on the requirements related to mobility.

- Radio-related measurements and information for QoE

None

- RAN visible QoE

RAN2 assumes that RAN2 is responsible to define the procedure to support RVQOE configuration and reporting, and leave the definition of RAN QoE metrics and what should be included in RVQOE configuration and report to other WGs, e.g. RAN3, SA4.

RAN2 confirms the following is feasible from RAN2 point of view.

- It is feasible to configure RVQOE using explicit RRC IEs

- Multiple simultaneous QoE measurements can be supported for RVQOE.  Each RVQOE measurement configuration is identified by the MeasConfigAppLayerId (or change to another generic term) corresponding to the regular QoE configuration.

- UE RRC layer forwards the received RVQOE configuration to the upper (application) layer, indicating the service type.

- RAN configures the required RVQOE metrics in the RVQOE configuration for UE to report.

2.2.2	Remaining Open issues

- FFS if to allow multiple QoE reports in the same RRC message, but leave it to UE.

- FFS how the indication that gNB indicate which QoE measurement configurations should be kept by the UE during RRC resume procedure looks like, e.g. granularity per QoE configuration or common for all QoE configurations.

- FFS on the maximum number of simultaneous QoE configurations in the UE (depend on RAN2).

- FFS on how to include pause status information in an appropriate inter-node RRC message.

- FFS on how to define UE capability to support RAN visible QoE measurement.

- FFS on whether to support RAN overload handling based on pause/resume.

- FFS whether the gNB needs to know the QoE configurations for which there are ongoing QoE sessions, e.g. to enable QoE configuration handling upon mobility (pending SA4 reply on the ongoing QoE measurement session continuity requirement).

2.3	RAN3

2.3.1	Agreements

RAN3#112-e (May 2021)

- QoE measurement Configuration and Reporting

	- Liaise SA5 on the support of (de)activation of NR QoE, including concerns on whether current Trace Function could support QoE mechanism, decoupling of deactivation, failure handling and QoE Reference

	- Introduce a new IE "QoE Reference" explicitly over interfaces at least for s-based, whether it can be applied to m-based and whether it is per service type or per slice depends on feedback from SA5

	- Introduce the following additional new IEs:

- a list of UE Application layer measurement configuration IE for each service type.

- inside each UE Application layer measurement configuration IE:

- Container.

- a numerated IE indicating service type (e.g., Streaming services, MTSI services, VR, MBMS, XR).

- Area scope (a list of cells/TA/TAI/PLMN).

- Support for Mobility

	- Include signaling based QoE measurement configuration in handover preparation messages i.e. in XnAP: HANDOVER REQUEST, NGAP: HANDOVER REQUEST.

	- Signaling based QoE measurement configuration is stored in NG-RAN when UE enters RRC_INACTIVE and is propagated to new serving NG-RAN using Retrieve UE context procedure when UE resumes RRC connection in another NG-RAN i.e. include signaling based QoE configuration in RETRIEVE UE CONTEXT RESPONSE in XnAP.

	- Include multiple sets of signaling-based QoE measurements configuration in Xn/NG: HANDOVER REQUEST and Xn: RETRIEVE UE CONTEXT RESPONSE.

	- Management based QoE should not override an existing signaling based QoE configuration.

	- Option 1 is agreed by RAN3 on area handling for QoE i.e. the network is responsible for keeping track of whether the UE is inside or outside the area and the network configures/releases configuration accordingly. Send LS to RAN2 and SA4 informing RAN3 agreements.

	- Upon the reception of QoE configuration on a non-supporting node, the target node should not set up any QoE session with MCE and should not initiate any QoE measurement collection.

	- Liaise SA4 to check if QoE requirement for ongoing session continuity is also applicable for NR QMC and in case QoE configuration release is received during an ongoing session.

- Per slice QoE measurement

None

- RAN visible QoE

	- The service types supported in the Rel17 RAN-visible QoE framework are DASH streaming and VR.

	- WA: The following metrics, pertaining to DASH streaming and VR services, should be supported in the Rel17 RVQOE framework:

- Buffer Level

- Average Throughput

- Playout Delay

	- LS to other WGs, based on the resolution of the WA above, is expected at the next RAN3 meeting.

	- The following is supported within the RVQOE framework:

- RAN-visible QoE metrics: a subset of legacy QoE metrics data collected from UE, which are useful for RAN.

- RAN-visible QoE values: a set of values derived from QoE metrics data through a model/function defined in collaboration with SA4 (pending SA4).

	- WA: The RAN generates the RVQOE measurement configuration

	- The UE is assumed to indicate to the RAN its capability with respect to providing RVQOE metrics (LS to RAN2 seems needed).

	- WA: RVQOE collection can be configured only if QoE measurements are configured for the same service type.

	- Together with the QoE measurements, the RVQOE is supported in the following aspects:

- Activation, and deactivation procedures

- WA: Multiple simultaneous QoE measurements

	- WA: the ID used to identify QoE measurements is reused for identifying the RVQOE measurements.

	- WA: the RVQOE report is provided inside a dedicated IE, outside the QoE report container.

	- Whether transfer of RVQOE configuration to the target be supported will be discussed after the basic solution for mobility has been defined.

	- Whether the RVQOE report can be signaled from the target to the source at handover will be discussed after the basic solution for mobility has been defined.

	- WA: gNB-CU may signal RVQoE report to gNB-DU over F1

- Alignment of Radio-Related Measurement and QoE Measurements

	- Immediate MDT is taken as baseline for the collection of Radio-related Measurements to assist QoE analysis.

	- Existing measurements specified for immediate MDT can be used for Radio-related measurements for QoE analysis.

	- New radio-related measurements, if any, should be defined in the SON/MDT WI.

	- Radio-related measurement and QoE measurement can be configured simultaneously by OAM for the alignment.

	- OAM (e.g. TCE or MCE) is responsible for correlation.

- LSs out

	- LS on how to support the (de)activation and failure handling of NR QMC (to: SA5; cc: SA2)

	- LS on the mapping between service types and slice at application (to: SA4,CT1,SA5; cc: RAN2, SA2)

	- LS on the area handling for QoE during mobility (to: RAN2,SA4; cc: SA5)

	- LS on the QoE requirement for ongoing session continuity (to: SA4; cc: SA5,RAN2)

RAN3#113-e (Aug 2021)

- QoE measurement Configuration and Reporting

	- Wait the reply LS from SA5, before we make decision on whether to reuse Trace or not.

	- In NGAP, at least INITIAL CONTEXT SETUP REQUEST, along with HANDOVER REQUEST should be enhanced for NR QoE.

	- Agree on supported service types for NR QoE management in Rel-17: Streaming services, MTSI service, VR.

	- Wait for SA5’s feedback: 1) introduction of QoE Reference for each service type of QoE measurement (i.e. support multi service QoE measurements in one message); 2) a separate and single MCE address is used for the QoE measurements of all service type in one message.

	- Slice scope is a list of S-NSSAI.

	- To include slice scope outside the configuration container over NG.

	- Slice related identifier should be included in the QoE measurement report from UE.

	- No additional requirements on QoE measurement to support roaming UEs.

- Support for Mobility

	- Include signaling based QoE measurement configuration in handover preparation messages i.e. in XnAP: HANDOVER REQUEST, NGAP: at least HANDOVER REQUEST.

	- Signalling based QoE can override an existing management based QoE configuration.

- Per slice QoE measurement

- RAN visible QoE

	- RAN visible QoE measurement activation, UE AS indicates to UE APP that RAN visible QoE measurement has been triggered, potentially with RAN visible QoE metrics needed to be collected at UE APP as requested by RAN.

	- RAN visible QoE measurement deactivation, UE AS indicates to UE APP that RAN visible QoE measurement has been terminated, and then UE APP stops to provide RVQoE measurement results to UE AS.

	- Turn into an agreement the WA that the RAN generates the RVQoE measurement configuration.

	- Turn into an agreement the WA that the ID used to identify QoE measurements is reused for identifying the RVQoE measurements.

	- Turn into an agreement the WA stating that RVQoE collection can be configured only if QoE measurements are configured for the same service type.

	- Turn into an agreement the WA stating that multiple simultaneous RVQoE measurements are supported.

	- The RVQoE configuration can be configured flexibly (i.e., it is not fixed).

	- The RVQoE configuration sent to UE should contain:

•	Metrics to be reported, as a mandatory IE.

	- The decision about the final list is expected at the next meeting.

	- Turn into an agreement the WA stating that the RVQoE report is provided inside a dedicated IE, outside the QoE report container.

	- The RAN decides whether RVQOE measurement collection and reporting is activated.

	- The gNB-CU may signal RVQoE report to gNB-DU over F1.

- Alignment of Radio-Related Measurement and QoE Measurements

	- Postpone the discussion on alignment for the case that MDT is configured before QoE configuration till clarification is received from SA5 on QoE activation/deactivation procedure (i.e., whether to reuse trace function for QoE and if multiple trace sessions can be supported).

	- An indicator is required in the QoE configuration to NG-RAN to inform whether it should perform MDT and QoE measurements in a time-aligned manner. FFS whether an explicit or implicit indicator.

	- WA: NG-RAN should include Trace Reference and Trace Recording Session Reference in the QoE report sent to MCE

	- WA: NG-RAN should NOT include the Trace Reference and Trace Recording Session Reference in the QoE configuration sent to UE

	- NG-RAN can include session start and session end time stamp information related to MDT and QoE reports autonomously (e.g., using the same clock for MDT and QoE )to assist the correlation entity.

	- QoE and related MDT report can be sent to the same collection entity.

- LSs out

	- LS on RAN3 agreements for NR QoE (to: RAN2, SA4, SA5)

RAN3#114-e (Nov 2021)

- QoE measurement Configuration and Reporting

	- Function separation between QMC and Trace.

	- Decouple QMC framework from Trace framework. Define new IEs named ‘QMC Activation IE’ and ‘QMC Deactivation IE’ to support the activation and deactivation of NR QoE. The specific solution can be described as:

Include QMC Activation IE inside the following messages over NGAP:

- INITIAL CONTEXT SETUP REQUEST

- UE CONTEXT MODIFICATION REQUEST

- HANDOVER REQUEST

- HANDOVER REQUIRED

Include QMC Deactivation IE inside the following message over NGAP:

- UE CONTEXT MODIFICATION REQUEST

	- MBS and XR would not be supported in R17.

	- Agree to include a list of QoE Reference in deactivation message

	- There is no need to introduce QoE measurement configuration modification procedure over NG

	- MCE IP address is configured per QoE measurement/per QoE reference

	- There is no need to introduce measurement configuration application layer ID info over NG during QoE activation

	- For legacy QoE measurement, RAN3 agree not to introduce criteria, e.g. time-based, threshold-based or, event-based, for RAN to trigger/stop the QoE measurement in R17.

	- RAN3 assumes that slice ID is included inside the transparent QoE reporting container, which is up to SA4’s decision. Send an LS to SA4 with the RAN3 assumption and asking SA4 to revert back once there is specification support for the same.

	- There is no need to include slice ID as an explicit IE over Uu outside the QoE configuration and reporting container for legacy QoE.

	- WA: RAN3 will not pursue prioritization mechanism of different service types or slices for the UE to send pending QoE reports after RAN overload is solved.

	- There is no need for prioritization mechanism configured by OAM over NG to guide RAN behavior to release or pause in case of RAN overload situation.

- Support for Mobility

	- For a service type, in Rel17, a UE can be simultaneously configured with multiple s- and/or m-based configurations, as long as the maximum number of simultaneous configurations at a UE is not exceeded.

	- The network can replace a configuration with another one of m- or s-based configuration by deactivating an existing measurement and configuring another measurement of the same configuration type.

	- RAN3 assumes that the OAM will never provide the same QoE Reference to different QoE configurations irrespective of QoE type.

	- A UE should continue an ongoing measurement once it leaves the Area, unless the network indicates to the UE to release the QoE configuration.

	- The following information about an m-based measurement configuration should be explicitly passed to the target during handover:

- The Measurement Configuration Application Layer ID corresponding to the QoE Reference.

- MDT Alignment info.

- MCE IP address.

- WA: Measurement status.

	- For m-based QoE, the QoE configuration container (XML file) is not included in NGAP and XnAP handover messages.

	- The following information is explicitly passed to the target at handover:

- QoE reference.

- MCE IP address.

- The Measurement Configuration Application Layer ID corresponding to the QoE Reference.

- WA: Measurement status.

- MDT Alignment info.

- Area Scope.

- Slice list.

	- For both s- and m-based QoE, the QMC Information IE (which does not include the QoE configuration container) is explicitly included in the XnAP RETRIEVE UE CONTEXT RESPONSE.

	- In case of mobility to a target node not supporting QoE, the target node can release the QoE configuration.

- RAN visible QoE

	- Interaction latency or comparable quality viewport switching latency metric is NOT considered as a RAN visible QoE metric in Rel-17

	- Buffer level is confirmed as a RAN visible QoE metric for DASH and VR service types

	- Playout delay for media startup is confirmed as a RAN visible QoE metric for DASH and VR service types.

	- In split gNB architecture, gNB-CU should generate the RAN visible QoE configuration.

	- RAN Visible QoE and legacy QoE can be configured together or separately. In case RAN visible QoE is configured separately, it can be configured only after configuring legacy QoE.

	- NG-RAN can release a list of RAN visible QoE configurations while not releasing the corresponding legacy QoE configurations.

	- If the legacy QoE configuration is released, the corresponding RAN visible QoE configuration is released as well.

	- RAN visible QoE configuration can include at least the RAN visible QoE metrics to be reported, service type and a measurement ID for the RAN visible QoE. Whether existing IEs can be reused for service type and measurement ID and the signaling design is up to RAN2.

	- There is no need to consider Start Time, Duration and Sample Percentage in the RAN Visible QoE configuration in Rel-17.

	- RAN3 should discuss whether the existing identified RAN visible QoE metrics (or values if agreed) justifies the need of a separate reporting periodicity for RAN visible QoE.

	- RAN3’s decision on whether to have a different reporting periodicity for RAN visible QoE is independent of RAN2’s decision on which SRB to use for RAN visible QoE

	- Send an LS to SA4 checking the feasibility of supporting a different reporting periodicity for RAN visible QoE metrics, from the application perspective.

	- NG-RAN can configure RAN visible QoE for only a subset of those metrics which are already configured as part of legacy QoE configuration.

	- The OAM sends a list of the available RAN visible QoE metrics to the RAN node, outside the legacy QoE configuration container.

	- The details of alignment between radio-related measurements and RVQoE measurements can be discussed in RAN3#114-bis-e.

	- Introduce a new class-1 message for QoE information transfer over F1.

	- WA: If the legacy QoE configuration is paused/resumed, the corresponding RVQOE configuration is paused/resumed as well

	- WA: Include PDU or QoS related information in RVQoE report

	Alignment of Radio-Related Measurement and QoE Measurements

	- For alignment of MDT and QoE measurement reporting, OAM may activate/deactivate appropriately.

	- No RAN3 specification impact is needed in Rel-17 to ensure that the duration of QoE associated MDT covers all the QoE sessions if multiple QoE session configured.

	- In case of aligned MDT/QMC (s-based activation), OAM includes Trace Reference and Trace Recording Session Reference of the MDT configuration in the QMC configuration sent to NG-RAN.

	- In case of aligned MDT/QMC (m-based activation), OAM includes Trace Reference of the MDT configuration in the QMC configuration sent to NG-RAN.

	- (WA turned into agreement) In case of aligned MDT/QMC, NG-RAN includes Trace Reference and Trace Recording Session Reference in the QoE report sent to MCE.

	- The gNB does NOT include QoE reference in MDT report sent to the TCE.

	- If RAN nodes are responsible for passing the mapping relation between QoE and MDT during mobility, NG-RAN node does not include the Trace Reference and Trace Recording Session Reference of the MDT session in QoE configuration sent to UE.

	- An indicator (whether Trace Reference and Trace Recording Session Reference is FFS) is required in the QoE configuration to NG-RAN to inform whether it should forward the QoE report to MCE along with the MDT related trace details.

	- To enable time alignment between an already ongoing Immediate MDT and a QoE measurement started later, the start time and end time of the QoE measurement, in addition to the Trace Reference and Trace Recording Session ID, needs to be added to the QoE measurement report at the NG-RAN node.

	- Rel-17 NR QMC to support the following activation scenario: S-based QoE and s-based MDT, M-based QoE and m-based MDT.

	- The alignment of RVQoE and MDT measurements reuses the solution for the alignment of legacy QoE and MDT measurements. RAN node can reuse RRM measurements as well.

	- MDT/QMC alignment in split architecture scenarios should be considered.

- LSs out

	- LS to SA4 on the support of including slice ID in the QoE reporting container.

	- Send an LS to RAN2 informing about our agreements on RAN visible QoE requesting them to provide the necessary specification support.

2.3.2	Remaining Open issues

	- FFS: RVQoE and legacy QOE can be reported separately.

	- FFS on the RVQoE report can be signalled from the target to the source node after a successful handover.

	- FFS on the RVQoE configuration is propagated from the source to target node upon mobility in RRC_CONNECTED and during context retrieval upon resumption from RRC_INACTIVE. The target/new RAN node may assemble a different RVQoE configuration.

	- FFS whether UE should reports the time elapsed between generating the QoE report and the time of reporting the QoE report i.e., when reporting is resumed, in case of alignment between MDT and a paused QoE.

	- FFS whether any enhancements for MDT/QMC alignment in split architecture scenarios should be considered.

	- Whether slice ID should be configured as an explicit IE to UE over Uu, at least for RAN visible QoE metric configuration.

	- Whether to extend some NG messages, e.g. NGAP INITIAL UE MESSAGE, with a new QoE Measurement Capabilities IE including a UE Application Layer Measurement Capability and a Max Number of UE Application Layer Measurements.

- RAN3 assumes that slice ID is included inside the transparent QoE reporting container, which is up to SA4’s decision.

Cross-TSG aspects:

- SA5: Extend stage 2 to cover NR QMC, including signalling based activation

- SA4/CT1: Specification support for RAN visible QoE metrics.

2.4	RAN4

2.4.1	Agreements

2.4.2	Remaining Open issues

2.5	RAN5

2.5.1	Agreements

2.5.2	Remaining Open issues

2.6	RAN6

2.6.1	Agreements

2.6.2	Remaining Open issues

3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

NOTE: This section only needs to be filled in for WI/SIs where there is a corresponding relevant WI/SI in SA/CT.

3.1	SA4

3.1.1	Agreements with cross-TSG impacts

3.1.2	Remaining Open issues with cross-TSG impacts

NOTE: This section should also flag any critical dependencies that need TSG attention.

4.	References

NOTE:	This can be e.g. a list of all related Tdocs in the affected WGs since last TSG, references to LSs, produced TRs/TSs, the work/study item description or status reports of previous TSGs.

RAN2#113b-e

- R2-2102633	Conclusion of NR QoE Management and Optimizations for Diverse Services SI in RAN3 (R3-211234; contact: China Unicom)

- R2-2102643	Reply LS on QoE Measurement Collection for LTE (RP-210922; contact: Ericsson)

- R2-2102760	Workplan for Rel-17 NR QoE in RAN2

- R2-2102958	QoE measurement configuration and reporting

- R2-2102963	QoE configuraiton and reporting general aspects

- R2-2102967	Stop and start of QoE measurement reporting

- R2-2103049	Configuration and reporting of QoE measurements

- R2-2103050	Pause and resume of QoE measurements

- R2-2103146	Discussion on QoE measurement pausing and resuming

- R2-2103147	Discussion on QoE measurement collection in NR

- R2-2103290	LS reply on QoE Measurement Collection

- R2-2103377	QoE measurement configuration and reporting

- R2-2103378	QoE measurement handling

- R2-2103425	QoE measurements in NR

- R2-2103555	Considerations on QoE scope

- R2-2103556	QoE reporting control by RAN awareness on QoE parameter

- R2-2103692	Configuration and reporting for NR QoE measurement

- R2-2103693	Start and stop for NR QoE measurement

- R2-2103835	Discussions on the QoE SI Metrics and Collection Procedures

- R2-2103910	Discussion on QoE measurement configuration and reporting

- R2-2103911	QoE measurement handling at RAN overload

- R2-2103934	General framework for QoE measurements

- R2-2104034	Discussion on NR QoE configuration

- R2-2104035	Discussion on QoE collection start and stop

- R2-2104082	Issues for NR QoE measurement

- R2-2104270	Discussion on NR QoE Configuration

- R2-2104271	Discussion on pause/resume NR QoE reporting

- R2-2104627	Report of offline discussion: [AT113bis-e][037][eQoE] Pause Resume

- R2-2104628	[DRAFT] LS on QoE handling during RAN

RAN2#114-e

- R2-2104992	QoE pause and resume handling

- R2-2104994	QoE confiugration and reporting

- R2-2105214	Further discussion on QoE measurement collection in NR standalone

- R2-2105215	QoE report handling during RAN overload

- R2-2105336	Discussion on QoE measurement configuration

- R2-2105337	Discussion on start and stop of QoE measurement

- R2-2105479	QoE configuration and general ascpects

- R2-2105525	Discussion on QoE measurement pausing and resuming

- R2-2105526	Discussion on QoE measurement collection in NR

- R2-2105580	Discussion on QoE measurement configuration and reporting

- R2-2105581	QoE measurement handling at RAN overload

- R2-2105646	Discussion on NR QoE

- R2-2105893	Configuration and reporting of QoE measurements

- R2-2105894	Pause and resume of QoE measurements

- R2-2105895	Running RRC CR for QoE measurements

- R2-2105920	QoE reporting control

- R2-2106061	Harmonised general framework for QoE measurements

- R2-2106159	Discussion on QoE collection start and stop

- R2-2106167	Discussion on NR QoE configuration

- R2-2106220	Further discussion on configuration and reporting

- R2-2106222	Further discussion on start and stop

- R2-2106346	Stop and start for QoE measurement reporting

- R2-2106348	QoE measurement configuration

- R2-2106402	Issues for NR QoE measurement

- R2-2106431	Discussion on pause/resume NR QoE reporting

- R2-2106432	Discussion on NR QoE configuration

- R2-2106653	Configuration Reporting General

- R2-2106654	QoE configuration and reporting related issues

- R2-2106661	Report from email discussion [AT114-e][027][QoE] Start and Stop (Lenovo)

- R2-2106683	Running RRC CR for QoE measurements

- R2-2106684	Running stage-2 CR for QoE measurements

- R2-2106761	Report from email discussion [AT114-e][027][QoE] Start and Stop (Lenovo)

- R2-2106762	Draft LS on QoE report handling at QoE pause

- R2-2106775	LS on QoE report handling at QoE pause

- R2-2106776	QoE configuration and reporting related issues

- R2-2106776   LS on QoE configuration and reporting related issues

- R2-2106775   LS on QoE report handling at QoE pause

RAN2#115-e

- R2-2106938	LS on the mapping between service types and slice at application (R3-212904; contact: Qualcomm)	RAN3	LS in	Rel-17	NR_QoE	To:SA4, CT1, SA5	Cc:RAN2, SA2

- R2-2106945	LS on requirement for configuration changes of ongoing QMC sessions (R3-212953; contact: Qualcomm)	RAN3	LS in	Rel-17	NR_QoE	To:SA4	Cc:SA5, RAN2

- R2-2106949	LS on the area handling for QoE during mobility (R3-212976; contact: Qualcomm)	RAN3	LS in	Rel-17	NR_QoE	To:RAN2, SA4	Cc:SA5

- R2-2108108	Running RRC CR for QoE measurements	Ericsson	draftCR	Rel-17	38.331	16.5.0	B	NR_QoE-Core

- R2-2108209	38.300 running CR for introduction of QoE measurements in NR	Huawei, China Unicom, HiSilicon	draftCR	Rel-17	38.300	16.6.0	NR_QoE-Core

- R2-2107099	General aspects in QoE	Samsung	discussion	Rel-17

- R2-2107380	Discussion on NR QoE configuration	CATT	discussion	NR_QoE-Core

- R2-2107396	Further discussion on QoE measurement collection in NR	OPPO	discussion	Rel-17	NR_QoE-Core

- R2-2107513	QoE handling in RAN	Nokia, Nokia Shanghai Bell	discussion	Rel-17	NR_QoE-Core	R2-2105479

- R2-2107816	Left issues for QoE configuration and reporting	Qualcomm Incorporated	discussion	NR_QoE-Core

- R2-2108109	Configuration and reporting of QoE measurements	Ericsson	discussion	Rel-17	NR_QoE-Core

- R2-2108197	Discussion on QoE measurement and configuration	China Unicom, China Southern Power Grid	discussion	Rel-17	NR_QoE-Core

- R2-2108206	Discussion on QoE measurement configuration and reporting	Huawei, HiSilicon	discussion	Rel-17	NR_QoE-Core

- R2-2108207	QoE handling during UE mobility	Huawei, HiSilicon	discussion	Rel-17	NR_QoE-Core

- R2-2108227	Discussion on NR QoE configuration	ZTE Corporation, Sanechips	discussion	Rel-17

- R2-2108514	More considerations on configuration and reporting	CMCC	discussion	Rel-17

- R2-2108594	Discussion on QoE measurement configuration	vivo	discussion	Rel-17	NR_QoE-Core

- R2-2109036	[Pre115-e][008][QoE] Summary Support for Mobility	Huawei, HiSilicon	discussion	Rel-17	NR_QoE-Core

- R2-2107100	Pause and resume in QoE	Samsung	discussion	Rel-17

- R2-2107101	Storing QoE reports in AS at pause	Samsung	discussion	Rel-17

- R2-2107381	Activation and deactivation for QoE collection	CATT	discussion	NR_QoE-Core

- R2-2107382	Discussion on QoE collection start and stop	CATT	discussion	NR_QoE-Core

- R2-2107397	Discussion on QoE measurement pausing and resuming	OPPO	discussion	Rel-17	NR_QoE-Core

- R2-2107514	RAN control on QoE reporting	Nokia, Nokia Shanghai Bell	discussion	Rel-17	NR_QoE-Core

- R2-2107515	QoE pausing	Nokia, Nokia Shanghai Bell	discussion	Rel-17	NR_QoE-Core	R2-2105920

- R2-2107615	Pause/Resume functionality	Apple	discussion	Rel-17	DUMMY

- R2-2107817	Left issues for QoE pause and resume procedure	Qualcomm Incorporated	discussion	NR_QoE-Core

- R2-2107852	Discussion on the partial QoE reporting and buffering at RAN overload	ITRI	discussion	NR_QoE-Core

- R2-2107882	Stop and start for QoE measurement reporting	LG Electronics Inc.	discussion	Rel-17

- R2-2108213	Discussion on pause and resume mechanism	China Unicom, China Southern Power Grid	discussion	Rel-17	NR_QoE-Core

- R2-2108226	Discussion on NR QoE start and stop	ZTE Corporation, Sanechips	discussion	Rel-17

- R2-2108515	More considerations on start and stop	CMCC	discussion	Rel-17

- R2-2107818	Support of RAN visible QoE	Qualcomm Incorporated	discussion	NR_QoE-Core

- R2-2108110	Mobility Support for NR QoE Management	Ericsson	discussion	Rel-17	NR_QoE-Core

- R2-2108111	[Draft] Support for Session Start and Session End Indication	Ericsson	LS out	Rel-17	NR_QoE-Core	To:CT1

- R2-2108208	Initial thoughts on non-RAN2 led objectives	Huawei, HiSilicon	discussion	Rel-17	NR_QoE-Core

- R2-2108228	Discussion on NR QoEcontinuity in handover	ZTE Corporation, Sanechips	discussion	Rel-17

- R2-2108595	Discussion on QoE continuity during mobility	vivo	discussion	Rel-17	NR_QoE-Core

- R2-2109038	[Pre115-e][007][QoE] Summary 8.14.2.1 Excluding Mobility	Ericsson 	discussion	Rel-17	NR_QoE-Core

- R2-2109200	QoE Reference and maximum number of QoE configurations in RRC	RAN2 	LSout

- R2-2109105	Report of offline: [AT115-e][046][QoE] Mobility (Huawei)		Huawei

RAN2#116-e

- R2-2109348	Reply LS on QoE configuration and reporting related issues (R3-214471; contact: CMCC)	RAN3	LS in	Rel-17	NR_QoE-Core	To:RAN2

- R2-2109351	LS on RAN3 agreements for NR QoE (R3-214477; contact: China Unicom)	RAN3	LS in	Rel-17	NR_QoE-Core	To:RAN2, SA4, SA5

- R2-2109372	Reply LS on the mapping between service types and slice at application (S2-2106537; contact: Qualcomm)	SA2	LS in	Rel-17	NR_slice-Core	To:RAN3	Cc:SA4, CT1, SA5, RAN2

- R2-2109382	Reply LS on the mapping between service types and slice at application (S4-211225; contact: Qualcomm)	SA4	LS in	Rel-17	NR_QoE	To:RAN3	Cc:CT1, SA4, RAN2, SA2

- R2-2109383	LS on TS 28.404/TS 28.405 Clarification (S4-211234; contact: Qualcomm)	SA4	LS in	Rel-17	NR_QoE-Core	To:SA5	Cc:RAN2

- R2-2109384	LS Reply on requirement for configuration changes of ongoing QMC sessions (S4-211248; contact: Huawei)	SA4	LS in	Rel-17	NR_QoE	To:RAN3	Cc:SA5, RAN2

- R2-2109385	LS Reply on QoE report handling at QoE pause (S4-211290; contact: Huawei)	SA4	LS in	Rel-17	NR_QoE-Core	To:RAN2, SA5	Cc:SA3

- R2-2109386	Reply LS on QoE configuration and reporting related issues (S4-211291; contact: Huawei)	SA4	LS in	Rel-17	NR_QoE-Core	To:RAN2	Cc:RAN3, SA5

- R2-2109389	Reply LS on QoE report handling at QoE pause (S5-214519; contact: Huawei)	SA5	LS in	Rel-17	NR_QoE-Core	To:RAN2, SA4	Cc:SA3

- R2-2109390	Reply LS on QoE configuration and reporting related issues (S5-214520; contact: Huawei)	SA5	LS in	Rel-17	NR_QoE-Core	To:RAN2	Cc:SA4, RAN3

- R2-2111225	Reply LS on QoE Reference and maximum number of QoE configurations in RRC (S5-215213; contact: Huawei)	SA5	LS in	Rel-17	eQoE	To:RAN2, RAN3	Cc:SA4

- R2-2109865	Running RRC CR for QoE measurements	Ericsson	draftCR	Rel-17	38.331	16.6.0	B	NR_QoE-Core

- R2-2111064	Running CR of UE capability for NR QoE	CMCC	draftCR	Rel-17	38.306	16.6.0	NR_QoE

- R2-2111162	38.300 running CR for Introduction of QoE measurements in NR	China Unicom, Huawei, HiSilicon	draftCR	Rel-17	38.300	16.7.0	B	NR_QoE-Core

- R2-2109565	QoE configuration, reporting and mobility	Qualcomm Incorporated	discussion	NR_QoE-Core

- R2-2109662	QoE measurement configuration and general aspects	Intel Corporation	discussion	Rel-17	NR_QoE-Core

- R2-2109832	Further discussion on transmission of QoE reports	Lenovo, Motorola Mobility	discussion	Rel-17	NR_QoE-Core

- R2-2109866	Configuration and reporting of QoE measurements	Ericsson	discussion	Rel-17	NR_QoE-Core

- R2-2109867	QoE measurements at handover, resume and re-establishment	Ericsson, China Unicom	discussion	Rel-17	NR_QoE-Core

- R2-2109984	Discussion on QoE configuration	vivo	discussion	Rel-17	NR_QoE-Core

- R2-2110073	Supporting mobility for NR QoE	Apple	discussion	Rel-17	NR_QoE-Core

- R2-2110099	Discussion on QoE measurement collection in NR	OPPO	discussion	Rel-17	NR_QoE-Core

- R2-2110605	Discussion on QoE measurement configuration and reporting	Huawei, HiSilicon	discussion	Rel-17	NR_QoE-Core

- R2-2110720	QoE configuration handling	Nokia, Nokia Shanghai Bell	discussion	Rel-17	NR_QoE-Core	R2-2107513

- R2-2110991	Discussion on NR QoE configuration	ZTE Corporation, Sanechips	discussion	Rel-17

- R2-2110993	Discussion on NR QoE configuration	CATT	discussion	NR_QoE-Core

- R2-2111062	Remaining issues on configuration and reporting	CMCC	discussion	Rel-17

- R2-2111132	QoE configuration in general aspects	Samsung	discussion	Rel-17

- R2-2111133	RRC segmentation for QoE configuration and report	Samsung	discussion	Rel-17

- R2-2111188	Discussion on NR QoE measurement and configurations	China Unicom	discussion	NR_QoE-Core

- R2-2109567	QoE pause and resume handling	Qualcomm Incorporated	discussion	NR_QoE-Core

- R2-2109574	Draft reply LS on QoE report handling at QoE pause	Qualcomm Incorporated	LS out	NR_QoE-Core	To:SA4	Cc:SA5, RAN3

- R2-2109833	Further discussion on QoE report handling at QoE pause	Lenovo, Motorola Mobility	discussion	Rel-17	NR_QoE-Core

- R2-2109868	Pause and resume of QoE measurements	Ericsson	discussion	Rel-17	NR_QoE-Core

- R2-2109985	Discussion on start and stop of QoE measurement	vivo	discussion	Rel-17	NR_QoE-Core

- R2-2110075	Pause/Resume functionality	Apple	discussion	Rel-17	NR_QoE-Core

- R2-2110100	[Draft] LS reply on further questions regarding QoE reporting handling at QoE pause	OPPO	LS out	Rel-17	NR_QoE-Core	To:SA4

- R2-2110101	Discussion on QoE measurement pausing and resuming	OPPO	discussion	Rel-17	NR_QoE-Core

- R2-2110281	Discussion on the partial QoE reporting and buffering at RAN overload	ITRI	discussion	NR_QoE-Core	R2-2107852

- R2-2110382	QoE pause and resume procedure	LG Electronics Inc.	discussion	Rel-17

- R2-2110608	Discussion on SA4/SA5 reply for QoE pause	Huawei, HiSilicon	discussion	Rel-17	NR_QoE-Core

- R2-2110721	QoE stop and pause	Nokia, Nokia Shanghai Bell	discussion	Rel-17	NR_QoE-Core	R2-2107515

- R2-2110722	RAN control on QoE reporting	Nokia, Nokia Shanghai Bell	discussion	Rel-17	NR_QoE-Core	R2-2107514

- R2-2110989	Discussion on NR QoE start and stop	ZTE Corporation, Sanechips	discussion	Rel-17

- R2-2110990	Discussion on buffer for NR QoE start and stop	ZTE Corporation, Sanechips	discussion	Rel-17

- R2-2110996	Discussion on QoE collection start and stop	CATT	discussion	NR_QoE-Core

- R2-2111131	Pause and resume in QoE	Samsung	discussion	Rel-17

- R2-2109568	Support of RAN visible QoE and per-slice QoE	Qualcomm Incorporated	discussion	NR_QoE-Core

- R2-2109986	Discussion on other WI objectives	vivo	discussion	Rel-17	NR_QoE-Core

- R2-2110074	RRC segmentation for NR QoE	Apple	discussion	Rel-17	NR_QoE-Core

- R2-2110606	QoE handling during UE mobility	Huawei, HiSilicon	discussion	Rel-17	NR_QoE-Core

- R2-2110607	RAN visible QoE	Huawei, HiSilicon	discussion	Rel-17	NR_QoE-Core

- R2-2110609	Draft reply LS on QoE configuration and reporting related issues	Huawei, HiSilicon	LS out	Rel-17	NR_QoE-Core	To:SA4, SA5	Cc:RAN3

- R2-2111063	Discussion on UE capability for NR QoE	CMCC	discussion	Rel-17

- R2-2111191	Discussion on RAN visible of QoE	China Unicom	discussion	NR_QoE-Core

- R2-2111633	Running RRC CR for QoE measurements	Ericsson	draftCR	Rel-17	38.331	16.6.0	B	NR_QoE-Core

- R2-2111634	38.300 running CR for introduction of QoE measurements in NR 	China Unicom, Huawei, HiSilicon	draftCR	Rel-17	38.300	16.7.0	NR_QoE-Core

- R2-2111603	LS on QoE visible QoE	RAN2 	LSout

RAN3#112-e

- R3-211732	NR QoE configuration and reporting

- R3-211733	(TP for 38.413 and 38.423) NR QoE configuration and reporting

- R3-211734	Per slice QoE measurements

- R3-211735	QoE measurement collection and reporting continuity in mobility scenarios

- R3-211736	RAN visible QoE

- R3-211737	Alignment of Radio-Related Measurement and QoE Measurements

- R3-211835	Discussion on NR QoE configuration procedures

- R3-211836	Discussion on NR QoE configuration details

- R3-211837	Discussion on per-slice QoE measurement

- R3-211838	Discussion on Measurement Collection and Continuity in Mobility

- R3-211839	Discussion on relevant set of RAN-visible QoE parameters

- R3-211840	Discussion on RAN visible QoE configuration and reporting

- R3-211841	Discussion on Alignment of MDT and QoE Measurements

- R3-211978	RAN3 impacts for supporting QoE Measurement Collection in NR

- R3-211979	Support of NR QoE Measurement Collection

- R3-211980	Support of NR QoE Measurement Collection

- R3-211981	Discussion on RAN visible QoE

- R3-211982	Discussion on the alignment of Radio-Related Measurement and QoE Measurement

- R3-211985	Procedures for Configuration, Activation and Deactivation of QMC

- R3-211986	QoE Configuration and Reporting

- R3-211987	paper 3

- R3-211988	QoE Mobility Support

- R3-211989	RAN-visible QoE Services and Metrics

- R3-211990	RAN-visible QoE - Configuration and Reporting

- R3-211991	The Alignment of Radio-Related Measurements and QoE Measurements

- R3-212322	Baseline and stage 2 aspects for NR QoE

- R3-212323	Introduction of QoE Measurement Collection for NR

- R3-212324	Principles for configuration of NR QoE measurements

- R3-212325	Analysis of QoE metrics for use by the NG-RAN

- R3-212326	On the alignment of QoE measurements and MDT measurements

- R3-212380	paper 33

- R3-212381	CR TS 38.413 QoE Configuration and Reporting - Signalling Design

- R3-212394	Workplan for Rel-17 NR QoE in RAN3

- R3-212434	Consideration on NR QoE activation procedure

- R3-212435	Discussion on NR QoE Configuration

- R3-212440	 (TP for TS 38.300) Introduce NR QoE

- R3-212441	Discussion on configuration details in NR standalone mode

- R3-212442	 (TP for TS 38.413) NR QOE configuration

- R3-212443	 (TP for TS 38.423) NR QOE configuration

- R3-212445	Measurement Collection and Continuity in Intra-System Intra-RAT Mobility

- R3-212448	Further consideration on RAN visible QoE

- R3-212449	Alignment of MDT and QoE Measurements

- R3-212452	(TP for 38.401) Alignment of MDT and QoE Measurements

- R3-212453	(TP for 38.473) Alignment of MDT and QoE Measurements

- R3-212455	(TP for 38.463) Alignment of MDT and QoE Measurements

- R3-212496	Alignment of radio related measurement and QoE measurement

- R3-212497	RAN visible QoE metrics

- R3-212498	Per-slice QoE measurement

- R3-212515	Further analysis on spec impacts of the potential solutions to QoE visibility

- R3-212516	[Draft] LS on QoE visibility at RAN

- R3-212517	Further analysis on spec impacts of the potential solutions to RAN assitsted measurement

- R3-212518	Further analysis on spec impacts of the potential solutions to QoE measurement per slice

- R3-212519	Further analysis on spec impacts on configuration and reporting

- R3-212520	CR to 38.413 on Introduction of QoE measurement

- R3-212521	Further analysis on spec impacts on RAN intervention of QoE measurement

- R3-212591	On per-slice QoE measurement

- R3-212970     LS on how to support the (de)activation and failure handling of NR QMC

- R3-212904     LS on the mapping between service types and slice at application

- R3-212976     LS on the area handling for QoE during mobility

- R3-212953     LS on the QoE requirement for ongoing session continuity

RAN3#113-e

- R3-212953     LS on the QoE requirement for ongoing session continuity

- R3-213316	CR TS 38.423 Mobility Support for NR QoE Measurement Collection

- R3-213317	QoE Configuration and Reporting

- R3-213318	CR TS 38.413 QoE Configuration and Reporting - Signalling Design

- R3-213319	QoE Mobility Support

- R3-213320	RAN-visible QoE Metrics

- R3-213321	RAN-visible QoE Configuration and Reporting

- R3-213322	The Alignment of Radio-Related Measurements and QoE Measurements

- R3-213489	QoE Configuration and Reporting

- R3-213490	QoE measurement in mobility scenarios

- R3-213491	RAN visible QoE configuration and reporting

- R3-213492	Updated Workplan for Rel-17 NR QoE

- R3-213493	38300 running CR for NR QoE_

- R3-213654	QoE configuration and reporting

- R3-213655	QoE measurement collection and reporting continuity in mobility scenarios

- R3-213656	Support for RAN Visible QoE

- R3-213657	Alignment of Radio-Related Measurement and QoE Measurements

- R3-213681	Discussion on stage 2 description for NR QMC

- R3-213682	Introduction of QoE Measurement Collection for NR

- R3-213683	Choices for configuration of NR QoE measurements

- R3-213684	Open issues on mobility

- R3-213685	Analysis of metrics for RAN visible QoE

- R3-213686	Remaining open issues for alignment of radio-related measurements and QoE measurements

- R3-213791	Discussion on Measurement Collection and Continuity in Intra-System Intra-RAT Mobility

- R3-213944	Discussion on NR QoE configuration procedures

- R3-213945	Introduction of NR QoE measurements on Xn interface

- R3-213946	Discussion on NR QoE configuration details

- R3-213947	Discussion on Measurement Collection and Continuity in Intra-System Intra-RAT Mobility

- R3-213948	Discussion on RAN visible QoE configuration and reporting

- R3-213949	[Draft]LS on the configuration and report of the RAN-visible QoE

- R3-213950	Discussion on Alignment of MDT and QoE Measurements

- R3-213964	Further discussion on QoE Measurement Collection in NR

- R3-213965	(CR for TS 38.413) Support of NR QoE

- R3-213966	(CR for TS38.423) Support of NR QoE

- R3-213967	Discussion on RAN visible QoE

- R3-213968	(CR for TS38.473) Support of QoE information transfer

- R3-213969	Discussion on the alignment of Radio-Related Measurement and QoE Measurement

- R3-214040	Further consideration on NR QoE (de)activation procedure

- R3-214041	(TP for TS 38.300) Introduce NR QoE

- R3-214042	(TP for TS 38.413) NR QOE configuration

- R3-214043	(TP for TS 38.423) NR QOE configuration

- R3-214044	Discussion on NR QOE configuration

- R3-214045	Discussion on Measurement Collection and Continuity in Intra-System Intra-RAT Mobility

- R3-214046	Further consideration on RAN visible QoE

- R3-214047	[draft] LS on RAN visible QoE

- R3-214048	Discussion on alignment of MDT and QoE Measurements

- R3-214049	(TP for 38.401) Alignment of MDT and QoE Measurements

- R3-214050	(TP for E1/F1) Alignment of MDT and QoE Measurements

- R3-214070	Further discussions on configuration and reporting of QoE measurements

- R3-214071	CR to 38.413 on Introduction of QoE measurement

- R3-214072	Further discussions on overland handling

- R3-214073	Further discussions on per-slice QoE measurement

- R3-214074	Further discussions on measurement Collection and Continuity in Intra-System Intra-RAT Mobility

- R3-214075	Further discussions on RAN-visible QoE

- R3-214076	Draft LS on RAN-visible QoE conclusions

- R3-214077	Further discussions on Radio-Related Measurement and QoE Measurements

- R3-214108	Further discussions on alignment of radio related measurement and QoE measurement

- R3-214109	Further discussions on RAN visible QoE

- R3-214110	Further discussions on per-slice QoE measurement

- R3-214131	On the reception of QoE configuration in a non-supporting node

- R3-214194	CB: # QoE1_Workplan - Summary of email discussion

- R3-214195	CB: # QoE2_ Stage2 - Summary of email discussion

- R3-214196	CB: # QoE3_Configuration_Report - Summary of email discussion

- R3-214197	CB: # QoE4_Mobility - Summary of email discussion

- R3-214198	CB: # QoE5_RANVisible - Summary of email discussion

- R3-214199	CB: # QoE6_MDTAlignment - Summary of email discussion

RAN3#114-e

- R3-214633	CR to 38.413 on Introduction of QoE measurement

- R3-214634	BLCR to 38.300

- R3-214694	QoE Reference and maximum number of QoE configurations in RRC

- R3-214704	Reply LS on the mapping between service types and slice at application

- R3-214716	Reply LS on the mapping between service types and slice at application

- R3-214717	LS Reply on requirement for configuration changes of ongoing QMC sessions

- R3-214720	Reply LS on (de)activation and failure handling of NR QMC

- R3-214726	Procedures for Configuration, Activation and Deactivation of QMC

- R3-214727	(TP for QoE BL CR for TS 38.413) QoE Configuration and Reporting

- R3-214728	Mobility Support for NR QoE Management

- R3-214729	CR TS 38.423 Mobility Support for NR QoE Measurement Collection

- R3-214730	RAN Visible QoE Metrics

- R3-214731	Configuration and Reporting of RAN Visible QoE

- R3-214732	The Alignment of Radio-Related Measurements and QoE Measurements

- R3-214769	BLCR to 38.410: Support of QoE Measurement Collection for NR

- R3-214907	QoE Configuration, Activation and Deactivation Procedures

- R3-214908	QoE configuration details

- R3-214909	Per slice QoE

- R3-214910	QoE measurement collection and reporting continuity in mobility scenarios

- R3-214911	RAN Visible QoE

- R3-214912	Alignment of Radio-Related Measurement and QoE Measurements

- R3-214979	[DRAFT] LS Reply on QoE Reference and Maximum Number of QoE Configurations in RRC

- R3-214980	[DRAFT] Reply LS on Mapping Between Service Types and Slice at Application

- R3-214981	[DRAFT] LS Reply on requirement for configuration changes of ongoing QMC sessions

- R3-215021	Distribution of QMC Job Attributes for Management Based QoE

- R3-215115	Discussion on NR QoE configuration procedures

- R3-215116	Introduction of NR QoE measurements on Xn interface

- R3-215117	Discussion on NR QoE configuration details

- R3-215118	Discussion on Measurement Collection and Continuity in Intra-System Intra-RAT Mobility

- R3-215119	Discussion on RAN visible QoE configuration and reporting

- R3-215120	TP for 38.423 on RAN visible QoE configuration and reporting

- R3-215121	Discussion on Alignment of MDT and QoE Measurements

- R3-215308	(TP for BL CR to TS 38.300) Stage 2 updates following replies from other WGs

- R3-215309	(TP for BL CR to TS 38.410) QMC function definition

- R3-215310	Stage 3 updates following replies from other WGs

- R3-215311	Inter-node propagation of management-based QMC configuration

- R3-215312	Handling of open points for RAN visible QoE

- R3-215313	Remaining open issues for alignment of radio-related measurements and QoE measurements

- R3-215543	NR QoE Activation and Deactivation Procedures

- R3-215544	NR QoE Configuration Details

- R3-215545	Mobility issues of NR QoE

- R3-215546	RAN visible QoE

- R3-215547	(CR for TS38.473) Support of QoE information transfer

- R3-215548	Alignment of MDT and QoE

- R3-215631	Discussion on QoE activation and deactivation procedures

- R3-215633	Discussion on NR QoE Configuration

- R3-215635	(TP for BL CR of TS 38.413) NR QoE Configuration

- R3-215637	(TP for BL CR of TS 38.423) NR QoE Configuration

- R3-215639	Further discussion on Measurement Collection and Continuity in Intra-System Intra-RAT Mobility

- R3-215641	Discussion on configuration and reporting of RVQoE

- R3-215644	Consideration on RAN visible QoE

- R3-215647	[draft] LS on the support for RAN visible QoE

- R3-215657	Further discussions on configuration details

- R3-215658	TP to 38.413 on configuration details

- R3-215659	Further discussions on RAN visible QoE metrics

- R3-215660	Draft LS on RAN visible QoE conclusions

- R3-215661	Further discussions on RAN related measurements and informaiton

- R3-215662	Further discussions on measurement collection and mobility continuity

- R3-215663	Stage 2 TPs to 38.300 on RAN related measurements and information

- R3-215667	Further discussion on alignment of MDT and QoE Measurements

- R3-215668	(TP for 38.401) Alignment of MDT and QoE Measurements

- R3-215669	(TP for E1/F1) Alignment of Radio-Related Measurement and QoE Measurements

- R3-215688	Remaining open issues on alignment of radio related measurement and QoE measurement

- R3-215689	Remaining open issues on per-slice QoE measurement

- R3-215703	Updated Workplan for Rel-17 NR QoE

- R3-215704	Clarification for QoE modification and overriding

- R3-215706	Per-slice QoE measurement configuration and reporting

- R3-215708	QoE Configuration and Reporting

- R3-215865	CB: # QoE1_Workplan - Summary of email discussion

- R3-215866	CB: # QoE2_Activation_Deactivation - Summary of email discussion

- R3-215867	CB: # QoE3_Configuration_Report - Summary of email discussion

- R3-215868	CB: # QoE4_Mobility - Summary of email discussion

- R3-215869	CB: # QoE5_RANVisible - Summary of email discussion

- R3-215870	CB: # QoE6_MDTAlignment - Summary of email discussion