3GPP TSG RAN meeting #94e	RP-213521

Electronic Meeting, December 6 - 17, 2021	(revision of RP-212732)

# Source:	Ericsson

# Title:	New SID on further NR RedCap UE complexity/cost reduction

# Document for:	Approval

Agenda Item:	8A.1

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

# Title: 	Study on further NR RedCap UE complexity/cost reduction

## Acronym: FS_redCap-redCo

## Unique identifier: 	 xxxxxx

| This WID includes a Core part |  |
| --- | --- |
| This WID includes a Performance part |  |

## 1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  | X | X |  |  |
| No | X |  |  | X | X |
| Don't know |  |  |  |  |  |

## 2	Classification of the Work Item and linked work items

### 2.1	Primary classification

This work item is a

|  | Feature |
| --- | --- |
|  | Building Block |
|  | Work Task |
| X | Study Item |

### 2.2	Parent Work Item

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |

### 2.3	Other related Work Items and dependencies

| Other related Work Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |
| 860035 | Study on support of reduced capability NR devices |  |
| 900062 | Support of reduced capability NR devices |  |

## 3	Justification

5G aims to accelerate industrial transformation and digitalization, which improve flexibility, enhance productivity and efficiency, reduce maintenance cost, and improve operational safety. Industrial sensors play an important role for realizing such a vision. Not only widely used in industrial automation and digitalization use cases, industrial sensors are also widely used in the general environmental monitoring use cases such as monitoring of critical infrastructure (e.g., buildings, bridges, water dams, etc.) or monitoring for natural disasters (e.g., wild fire, flood, tsunami, earthquake, etc.).

Another emerging new class of new 5G use cases is the smart city vertical, which covers data collection and processing to more efficiently monitor and control city resources, and to provide services to city residents. Especially, the deployment of surveillance cameras is an essential part of the smart city but also of factories and industries.

Furthermore, there have been increasing interests in wearables use cases such as smart watches, eHealth related devices, and medical monitoring devices. These use cases call for different design considerations and have different requirements in terms of form factor, UE complexity and energy efficiency, compared to eMBB devices.

The support of industrial sensors, video surveillance, and wearables were the motivations behind Rel-17 RedCap. Through the Rel-17 NR RedCap work item, 3GPP has established a framework for enabling reduced capability NR devices suitable for a range of use cases, including the industrial sensors, video surveillance, and wearables use cases, with requirements on low UE complexity and sometimes also on low UE power consumption.

Now when the foundation has been laid in Rel-17, enhancements can be considered to improve the support for the mentioned use cases and also to expand RedCap into a new range of use cases such as smart grid.

To further expand the market for RedCap use cases with relatively low cost, low energy consumption, and low data rate requirements, e.g., industrial wireless sensor network use cases, some further cost and complexity reduction enhancements should be considered.

Rel-18 RedCap should provide NR support for low-tier devices between existing LPWA UEs and the capabilities of Rel-17 RedCap UEs. The supported peak data rate for Rel-18 RedCap targets to 10Mbps. Rel-18 RedCap should not overlap with existing LPWA solutions.

The enhancements should be introduced while maintaining the integrity of the RedCap ecosystem and maximizing the benefit of economies of scale. The SI targets enhancements applicable to the RedCap framework defined in Rel-17, including principles of network awareness of device capabilities.

## 4	Objective

### 4.1	Objective of Core part WI

To further reduce the complexity/cost of RedCap devices, the following should be studied, and the results should be captured in TR 38.875:

- Study further UE complexity/cost reduction techniques based on Rel-17 evaluation methodology [RAN1]

  - Consider network impact, coexistence of Rel-17 and Rel-18 RedCap and non-RedCap UEs in a cell, UE impact, specification impact

  - Potential solutions, which may complement each other, for reducing device complexity/cost are focusing on:

    - UE bandwidth reduction to 5MHz in FR1,

      - Note: The basic design of Rel-15 SSB/CORESET#0 should be reused.

      - Note: Operation in BWP with/without SSB and without/with RF retuning should be considered.

    - reduced UE peak data rate in FR1,

      - including the possibility of restricted bandwidth for PDSCH and/or PUSCH

    - relaxed UE processing timeline for PDSCH and/or PUSCH and/or CSI

    - reduced maximum number of HARQ processes for DL and/or UL

    - Note: It is not precluded that solutions for FR1 can be applied to FR2 in WI stage.

### 4.2	Objective of Performance part WI

### 4.3	RAN time budget request

additional comments to the time budget request in the attached Excel table:

## 5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Proposed Spec no. or series | Type (see note 1) | Title | For info at TSG# | For approval at TSG# | Remarks |

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| 38.875 | Study on support of reduced capability NR devices | RAN#97 (Sep. ‘22) |  |

## 6	Work item Rapporteur(s)

Johan Bergman, Ericsson, johan.bergman@ericsson.com

## 7	Work item leadership

Primary: RAN WG1

Secondary: none

## 8	Aspects that involve other WGs

## 9	Supporting Individual Members

Supporting IM name

Ericsson

NTT DOCOMO, INC.

Panasonic

DENSO CORPORATION

Vivo

CATT

Xiaomi

CAICT

Continental Automotive

Transsion

Vodafone

Sharp

Honor

NEC

ZTE Corporation

Sanechips

Fujitsu

Oppo

Nokia

Nokia Shanghai Bell

Lenovo

Motorola Mobility

Fraunhofer IIS

Fraunhofer HHI

FUTUREWEI

Philips

SONY

Sequans