3GPP TSG RAN Meeting #94-e 																RP-213686

Electronic Meeting, December 6- 17, 2021

Agenda item:			9.4.3.2

Source:	RAN4 Vice-Chair (Samsung)

Title:	Moderator's summary of discussion [94e-59-CR-RP-212944]

Document for:	Discussion & Decision

Introduction

This document provides as summary of the following email discussion during RAN#94-e:

| [94e-59-CR-RP-212944] | RP-212944 | Haijie Qiu, RAN4 VC | 9.4.3.2 |
| --- | --- | --- | --- |

Initial round

This thread started from intermediate round, no initial round.

Intermediate round

Open issues

Issue 1: CR RP-212944

As explained in the cover sheet of company CR RP-212944:

| Reason for change: | n46E is for aggregated BW > 300MHz and the widest available spectrum is UNII-2C with 260MHz. so BW class E is not usable in n46 and this is also why 802.11be has no 320MHz channel in 5GHz band. |
| --- | --- |
| Summary of change: | Delete DC configurations DC_n46E-n48A, DC_n46E-n48B, DC_n46E-n48C |

Collection of company views

| Company | Comments |
| --- | --- |
| ZTE | We understand the intention of this CR, and basically we agree with this CR except one minor comment: the uplink NR DC configuration of 'DC_n46A-n48B' in table 5.5B.1-1 is not existed in the current specfication, which means it should be wrote in change mark. |
| Skyworks | I also have signaled this issue a few time in RAN4. Also the same combinations using the proper n46N BW class exist so the operators needs are covered. |
| CHTTL | Though in the summary of change, it mentions the CR is to delete something, but actually this CR is adding more new combinations to the spec. Also nomarlly this should be discussed in RAN4 rather than in RAN, though it seems that there was some offline discussion with the rapporteurs, but next time it will be good to follow the normal basket workflow. Also in the summary of change, it mentions "Delete DC configurations DC_n46E-n48A, DC_n46E-n48B, DC_n46E-n48C", but actually there is no DC_n46E-n48B, DC_n46E-n48C in the current spec. Also the error pointed by ZTE needs to be corrected. |
| Charter | Let me address everyone’s comment and can also provide revision to address the concerns. To ZTE: I will add DC n46A-N48B UL configuration in change mark. To Skyworks and CHTTL: During revision of big basket WID, Charter noticed that the CA combinations with n46E had been corrected through a tp for tr in RAN4 to n46N but that the DC equivalent combination was not addressed. After offline discussion with rapporteur and MCC, the suggestion was to create a company CR to correct the mistake in RAN-P; hence this Cr was created. For the summary Charter followed the plenary MCC suggestion but we can also revised to highlight the addition of the n46N combinations. |

Summary and recommendation

Given the comments received, a revision needed to address the comments can be further checked in final round.

Final round

Collection of company views

Summary and recommendation

No specific comments received in final round, the latest revision agreeable.

Conclusion

Moderator Recommendations:

| T-doc | Title | Source | Recommended decision |
| --- | --- | --- | --- |
| RP-213687 (revision of RP-212944) | Bandwidth class correction for DC_n46N-n48A DC_n46N-n48B DC_n46N-n48C combos | Charter Communications,Inc. | Agreeable |