3GPP TSG-RAN5 Meeting #94-e	R5-220235

# Online, , 21st Feb - 4th Mar 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-5 | CR | 0309 | rev | - | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of IMS5GS TC 8.39a |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-02-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | To fulfil the WP. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added test case 8.39a. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | WP not fulfilled |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.39a (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# *** 1st Change ***

## 8.39a	HTTP Digest Authentication / 5GS

8.39a.1	Test Purpose (TP)

(1)

with { UE being registered to IMS and configured to use Digest authentication }

ensure that {

when { UE is made to activate OIP }

then { UE authenticates itself using Digest }

}

8.39a.2	Conformance Requirements

The conformance requirements covered in the present test case are, unless otherwise stated, Rel-15 requirements.

[TS 24.623, clause 5.2.3.2.1]:

On receiving an HTTP request that does not contain an Authorization header the AS shall:

a)	challenge the user by generating a 401 Unauthorized response that contains the proper Digest authentication parameters (e.g. realm), according to IETF RFC 2617 [3]. Provisioning of credentials to authenticate the user is outside the scope of the present document; and

b)	forward the 401 Unauthorized response to the sender of the HTTP request.

On receiving an HTTP request that contains an Authorization header, the AS shall:

a)	apply the authentication procedures defined in IETF RFC 2617 [3]; and

b)	authorize or deny authorization depending on the authenticated identity.

8.39a.3	Test description

8.39a.3.1	Pre-test conditions

System Simulator:

-	1 NR Cell connected to 5GC, default parameters.

UE:

-	UE contains either ISIM and USIM applications or only USIM application on UICC.

-	UE is configured to register for IMS after switch on.

Preamble:

-	UE is in state 1N-A (TS 38.508-1 [21]) and registered to IMS

8.39a.3.2	Test procedure sequence

Table 8.39a.3.2-1: Main Behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | UE is made to attempt to attempt activation of supplementary service Originating Identification Presentation | - | - | - | - |
| 2 | UE sends an initial HTTP Request (Step 2 of A.21) | --> | GET/PUT/DELETE | - | - |
| 3 | Conditional (according to A.21): SS sends 401 Unauthorized (Step 3a of A.21) | <-- | 401 Unauthorized |  |  |
| 4 | Conditional (according to A.21): UE sends HTTP request with valid authorization credentials (Step 3b of A.21) | --> | GET/PUT/DELETE | 1 | P |
| 5 | SS sends 200 OK (Step 4 of A.21) | <-- | 200 OK | - | - |
| 6-10 | UE and SS complete the acivation of the supplementary service and then de-activate it again (Steps 5-9 of A.21) | - | - | - | - |

8.39a.3.3	Specific message content

None as fully specified in Annex A.21.

# *** End of changes ***