3GPP TR 21.917 V0.3.0 (2022-01)

Technical Report

3rd Generation Partnership Project;

Technical Specification Group Services and System Aspects;

Release 17 Description;

Summary of Rel-17 Work Items

(Release 17)

The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP.
The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented.
This Report is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification.
Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices.

Keywords

Description, Summary, Release 17

3GPP

Postal address

3GPP support office address

650 Route des Lucioles - Sophia Antipolis

Valbonne - FRANCE

Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16

Internet

http://www.3gpp.org

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

© 2021, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

UMTS™ is a Trade Mark of ETSI registered for the benefit of its members

3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners
LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners

GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	5

Introduction	5

1	Scope	7

2	References	7

3	Definitions of terms, symbols and abbreviations	7

3.1	Terms	7

3.2	Symbols	7

3.3	 Abbreviations	7

4	Rel-17 Executive Summary	8

5	5G Services to "verticals"	8

5.1	Network and application enablement for verticals	8

5.2	Location and positionning	9

5.3	Railways	9

5.4	Support of Non-Public and Private Networks	9

5.5	Industrial IoT	10

5.6	Media production	10

5.7	Other services to verticals	10

6	5G access via Satellite/Non-Terrestrial (NTN) link	11

7	V2V/D2D/Sidelink related	11

8	Enhancements to network architecture and application enablement	13

8.1	Edge computing	13

8.2	Slicing	14

8.3	Access Traffic Steering, Switch and Splitting support in the 5G system architecture; Phase 2	14

8.4	Self-Organizing (SON)/Autonomous Network	15

8.5	Minimization of service Interruption	15

9	Multicast-Broadcast Services (MBS) and Mission Critical (MC)	15

10	Multi-(U)SIM	18

11	Messaging improvements	18

12	Drone/UAS/UAV/EAV identification	20

13	Authentication and key management for applications based on 3GPP credential in 5G	21

14	Multimedia Priority Service (MPS) Phase 2	21

15	NR enhancements	22

16	Other NR-only improvements	23

17	NR Carrier Aggregation	24

17.1	NR intra band Carrier Aggregation (itrabCA)	24

17.2	NR inter band Carrier Aggregation (iterbCA)	25

18	NR and LTE	25

19	NR new/modified bands	27

20	LTE new/modified bands	28

21	LTE inter-band Carrier Aggregation (LTE iterbCA)	28

22	User plane improvements	29

23	Signalling optimisations	30

24	Other Release 17 Features	34

25	Management-only Features	37

26	Features without summary	38

Annex A: List of expected contributions	39

Annex A: Template for contributing	51

Annex B: Process to get further information	52

B.1	General	52

B.2	Direct method	53

B.3	Step by step method	53

Annex C: Change history	56

# Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

# Introduction

The present document provides a summary of each and every 3GPP Release 17 Feature, and more generally of all Work Items for which a summary has been agreed to be provided.

These summaries are based on the inputs issued by the Work Item Rapporteurs, sometimes modified by the Work Plan manager to ensure overall consistency of this document. The original inputs can be retrieved as temporary document (tdoc). The original tdoc number is provided just below the table of each clause.

The present document presents the "initial state" of the Features introduced in Release 17, i.e. as they are by the time of publication of this document. Each Feature is subject to be later modified or enhanced, over several years, by the means of Change Requests (CRs). To further outline a feature at a given time, it is recommended to retrieve all the CRs which relate to the given Feature, as explained in Annex C. As a first approach, the list of all the Change Requests related to a Work Item can be retrieved by selecting "TSG Status = Approved" in the Work Item’s page of the 3GPP Portal at:

https://portal.3gpp.org/ChangeRequests.aspx?q=1&workitem= [6-digit identifier of the Work Item, separated by a coma if several Work Items]

E.g. for “NR support for high speed train scenario”: the table at the beginning of its section indicates that the corresponding Work Items have the numbers 840192 and 840292. So, all the CRs related to this Feature are retrieved by selecting "TSG Status = Approved" in https://portal.3gpp.org/ChangeRequests.aspx?q=1&workitem=840192,840292

Editor’s note: INPUTS ARE EXPECTED from rapporteurs. The list of expected contributors is provided at the beginning of each section. 
It also appears in “Annex B: List of contributors”, in the cell highlighted in yellow. Annex A provides the expected rapporteur in term of Working Group. E.g. if “SA2” is indicated, it means: “The SA2 WID rapporteur is expected to provide the summary for the full feature”.
The name of the rapporteur is provided in “Annex C: Full list of Rel-16 Work Items”, in the column “WI_rapporteur_e_mail”.

Editor’s note: to the contributors: 
The template to be used for the contribution is provided in “Annex A: Template for contributing”. The input type in 3GU is “WI Summary”.
The target audience of this document are readers inside and outside the 3GPP community. Contributions should be written as to be understandable by non-specialist. 
The summary should reflect the actual outcome of the Feature - and not the initial intentions given in the WID.
An exhaustive list of all the CRs and TS/TRs is not to be provided in the summary (instead, the procedure on how to retrieve this information is given in section 4). Only the essential CRs and TS/TRs can be mentioned in the Reference. 
When the usual Stage 1/Stage 2/Stage 3 process is not followed -and in particular if a full stage is missing- explain why.

# 1	Scope

The present document provides a summary of each Release 15 Feature or, whenever needed, of each significant Work Item.

The information provided in the present document is limited to an overview of each Feature, explaining briefly its purpose and the main lines of the system's behaviour to execute the Feature.

More information is available by consulting the 3GPP Ultimate web site, as explained in "Annex C: Process to get further information".

The present document presents the "initial state" of the Features introduced in Release 15, i.e. as they are by the time of publication of the present document. Each Feature is subject to be later modified or enhanced, over several years, by the means of Change Requests (CRs). It is therefore recommended to retrieve all the CRs which relate to the given Feature, as explained in Annex C, to further outline a feature at a given time.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	TR 21.905: "Vocabulary for 3GPP Specifications".

NOTE:	Due to the specificity of the present document, consisting in a collection of independent summaries, the references are given at the end of each clause rather than in this clause.

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

## 3.2	Symbols

Void.

## 3.3		Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. 
An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1]. Abbreviations specific to a given clause are provided in the clause they appear.

KPI	Key Performance Indicator

Rel	3GPP Release

# 4	Rel-17 Executive Summary

Editor's note: To be provided later.

# 5	5G Services to "verticals"

## 5.1	Network and application enablement for verticals

| UID | Name | Acronym | WG | WID | WI rapporteur name/company |
| --- | --- | --- | --- | --- | --- |
| 920049 | Enhanced Service Enabler Architecture Layer for Verticals | eSEAL |  | SP-200987 | Basavaraj (Basu) Pattan, Samsung |
| 900024 | Stage 2 of eSEAL | eSEAL | S6 | SP-200987 | Basavaraj (Basu) Pattan, Samsung |
| 920006 | CT1 aspects of eSEAL | eSEAL | C1 | CP-212098 | Sapan Shah (Samsung Electronics) |
| 920050 | CT3 aspects of eSEAL | eSEAL | C3 | CP-212098 | Sapan Shah (Samsung Electronics) |

| 920058 | Rel-17 Enhancements of 3GPP Northbound Interfaces and Application Layer APIs | NBI17 |  | CP-211197 | Yali Yan, Huawei |
| --- | --- | --- | --- | --- | --- |
| 920013 | CT1 aspects of NBI17 | NBI17 | C1 | CP-211197 | Yali Yan, Huawei |
| 920053 | CT3 aspects of NBI17 | NBI17 | C3 | CP-211197 | Yali Yan, Huawei |
| 920054 | CT4 aspects of NBI17 | NBI17 | C4 | CP-211197 | Yali Yan, Huawei |

Summary based on the input provided by Huawei in CP-220347.

The 3GPP Northbound Interfaces and APIs are specified in 3GPP in order to enable external entities and third party Application Servers/Functions to access a set of exposed 3GPP network services and capabilities in a secure and controlled manner. The 3GPP application layer APIs are defined by 3GPP so as to allow the support of various applications/services (e.g. V2X, UAS, EDGE, etc.) over 3GPP networks and ensure the efficient use and deployment of these applications/services over 3GPP systems via an optimized application layer framework.

3GPP northbound interfaces/APIs as well as application layer APIs, which are accessed by various external entities including third party Application Servers/Functions, should be continuously maintained and optimized. Therefore, the main work carried out was to introduce pure stage 3 (i.e. there are no related stage 1 nor stage 2 requirements) technical improvements and enhancements, i.e. improvement of the overall efficiency, reliability and flexibility, enhancement of the signalling efficiency, consolidation of the common protocol aspects, alignment with the 5GC service based principles and guidelines when relevant, corrections/changes missed in the previous 3GPP Releases, etc., especially that such enhancements may not be covered by the other more dedicated and functionality-driven work items.

References

List of related CRs: select "TSG Status = Approved" in: https://portal.3gpp.org/ChangeRequests.aspx?q=1&workitem=920058,920013,920053,920054

[1]	TS 29.122: "T8 reference point for Northbound APIs"

[2]	TS 29.522: "5G System; Network Exposure Function Northbound APIs; Stage 3"

[3]	TS 29.549: "Service Enabler Architecture Layer for Verticals (SEAL); Application Programming Interface (API) specification; Stage 3"

[4]	TS 29.558: "Enabling Edge Applications; Application Programming Interface (API) specification; Stage 3"

| 920012 | Enhancement of 5G PCC related services in Rel-17 | en5GPccSer17 | C3 | CP-211193 | Xiaoyun Zhou, Huawei |
| --- | --- | --- | --- | --- | --- |

Summary based on the input provided by Huawei in CP-211193.

The stage 3 procedures related to EPS functionality which are not completely covered in 5GS_Ph1-CT and en5GPccSer are defined.

The protocols and APIs for policy and charging control have been specified in the previous 3GPP Releases. When the 5GS was deployed, it was found that some functionalities defined in Gx/Rx for EPS are still applicable in the 5GS and EPS interworking scenario. There are also some aspects of 5GS that are not supported by the Rx interface when the Rx interworks with the 5GS.

In order to optimize the operator’s management, alignments have been made between the EPS and 5GS interfaces.

References

List of related CRs: select "TSG Status = Approved" in: https://portal.3gpp.org/ChangeRequests.aspx?q=1&workitem=920012

[1]		3GPP TS 29.512: "5G System; Session Management Policy Control Service ".

[2]		3GPP TS 29.513: "5G System; Policy and Charging Control signalling flows and QoS parameter mapping; Stage 3".

[3]		3GPP TS 29.514: "5G System; Policy Authorization Service; Stage 3".

[4]		3GPP TS 29.214: "Policy and Charging Control over Rx reference point 5".

## 5.2	Location and positionning

| UID | Name | Acronym | WG | WID | WI rapporteur name/company |
| --- | --- | --- | --- | --- | --- |
| 910052 | Enhancement to the 5GC LoCation Services-Phase 2 | 5G_eLCS_ph2 |  | SP-200082 | Ming Ai, CATT |
| 870001 | Stage 2 of 5G_eLCS_ph2 | 5G_eLCS_ph2 | S2 | SP-200082 | Ming Ai, CATT |
| 910006 | CT aspects of 5G_eLCS_ph2 | 5G_eLCS_ph2 | ct | CP-211090 | Chenxi Bao, CATT |
| 910053 | CT1 aspects of 5G_eLCS_ph2 | 5G_eLCS_ph2 | C1 | CP-211090 | Chenxi Bao, CATT |
| 910054 | CT3 aspects of 5G_eLCS_ph2 | 5G_eLCS_ph2 | C3 | CP-211090 | Chenxi Bao, CATT |
| 910055 | CT4 aspects of 5G_eLCS_ph2 | 5G_eLCS_ph2 | C4 | CP-211090 | Chenxi Bao, CATT |
| 900060 | RAN aspects of NR positioning enhancements | NR_pos_enh |  | RP-210903 | Intel |
| 860034 | Study on NR positioning enhancements | FS_NR_pos_enh | R1 | RP-202094 | CATT |
| 900160 | Core part: NR positioning enhancements | NR_pos_enh-Core | R1 | RP-210903 | Intel |
| 900260 | Perf. part: NR positioning enhancements | NR_pos_enh-Perf | R4 | RP-210903 | Intel |

Summary based on the input provided by XXX in xP-22xxxx.

## 5.3	Railways

| UID | Name | Acronym | WG | WID | WI rapporteur name/company |
| --- | --- | --- | --- | --- | --- |
| 840042 | Complete Gap Analysis for Railways Mobile Communication System | MONASTERYEND | S1 | SP-190312 | Gach, Guillaume, UIC |
| 890035 | Enhancements to Application Architecture for the Mobile Communication System for Railways Phase 2 | eMONASTERY2 |  | SP-191104 | Oettl, Martin, Nokia |
| 840037 | Stage 2 of eMONASTERY2 | eMONASTERY2 | S6 | SP-191104 | Oettl, Martin, Nokia |
| 890063 | Stage 3 of eMONASTERY2 | eMONASTERY2 | C1 | CP-202256 | Gkatzikis, Lazaros, Nokia(Nokia) |
| 840043 | Study on Future Railway Mobile Communication System3 | FS_FRMCS3 | S1 | SP-200578 | Gach, Guillaume, UIC, |

Summary based on the input provided by XXX in xP-22xxxx.

See also "Introduction of 900MHz NR band for Europe for Rail Mobile Radio (RMR)" and "Introduction of 1900MHz NR TDD band for Europe for Rail Mobile Radio (RMR)" under "NR enhancements" section

## 5.4	Support of Non-Public and Private Networks

| UID | Name | Acronym | WG | WID | WI rapporteur name/company |
| --- | --- | --- | --- | --- | --- |
| 840024 | Study on enhanced support of Non-Public Networks | FS_eNPN | S2 | SP-200094 | Peter Hedman |
| 910065 | Enhanced support of Non-Public Networks | eNPN |  | SP-200980 | Hedman, Peter, Ericsson |
| 900015 | Stage 2 for eNPN | eNPN | S2 | SP-200980 | Hedman, Peter, Ericsson |
| 910016 | CT aspects of eNPN | eNPN | ct | CP-212103 | Sedlacek, Ivo, Ericsson |
| 910066 | CT1 aspects of eNPN | eNPN | C1 | CP-212103 | Sedlacek, Ivo, Ericsson |
| 910067 | CT3 aspects of eNPN | eNPN | C3 | CP-212103 | Sedlacek, Ivo, Ericsson |
| 910068 | CT4 aspects of eNPN | eNPN | C4 | CP-212103 | Sedlacek, Ivo, Ericsson |
| 920025 | Security Aspects of eNPN | eNPN | S3 | SP-210422 | Jost, Christine, Ericsson |
| 880008 | Study on enhanced security support for Non-Public Networks | FS_eNPN_SEC | S3 | SP-200353 | Normann, Henrik, Ericsson |
| 890049 | Enhancement of Private Network support for NG-RAN | NG_RAN_PRN_enh | R3 | RP-212585 | China Telecom |
| 890149 | Core part: Enhancement of Private Network support for NG-RAN | NG_RAN_PRN_enh-Core | R3 | RP-212585 | China Telecom |

Summary based on the input provided by XXX in xP-22xxxx.

## 5.5	Industrial IoT

| UID | Name | Acronym | WG | WID | WI rapporteur name/company |
| --- | --- | --- | --- | --- | --- |
| 850012 | Study on enhanced support of Industrial IoT | FS_IIoT | S2 | SP-200298 | Nokia, Devaki Chandramouli, |
| 880010 | Study on security for enhanced support of Industrial IoT | FS_IIoT_SEC | S3 | SP-200355 | Nokia, Anja Jerichow |
| 920024 | Security for enhanced support of Industrial IoT | IIoT_SEC | S3 | SP-210421 | Nokia, Anja Jerichow |
| 910059 | Support of Enhanced Industrial IIoT | IIoT |  | SP-200973 | Devaki Chandramouli, Nokia |
| 900008 | Stage 2 for IIoT | IIoT | S2 | SP-200973 | Devaki Chandramouli, Nokia |
| 910014 | CT aspects of support of enhanced IIoT | IIoT | ct | CP-212100 | Won, Sung Hwan, Nokia |
| 910060 | CT1 aspects of support of enhanced IIoT | IIoT | C1 | CP-212100 | Won, Sung Hwan, Nokia |
| 910061 | CT3 aspects of support of enhanced IIoT | IIoT | C3 | CP-212100 | Won, Sung Hwan, Nokia |
| 910062 | CT4 aspects of support of enhanced IIoT | IIoT | C4 | CP-212100 | Won, Sung Hwan, Nokia |
| 860044 | Additional enhancements for NB-IoT and LTE-MTC | NB_IOTenh4_LTE_eMTC6 |  | RP-211340 | Huawei |
| 860144 | Core part: Additional enhancements for NB-IoT and LTE-MTC | NB_IOTenh4_LTE_eMTC6-Core | R1 | RP-211340 | Huawei |
| 860244 | Perf. part: Additional enhancements for NB-IoT and LTE-MTC | NB_IOTenh4_LTE_eMTC6-Perf | R4 | RP-211340 | Huawei |
| 860051 | NR small data transmissions in INACTIVE state | NR_SmallData_INACTIVE |  | RP-212594 | ZTE |
| 860151 | Core part: NR small data transmissions in INACTIVE state | NR_SmallData_INACTIVE-Core | R2 | RP-212594 | ZTE |
| 860251 | Perf. part: NR small data transmissions in INACTIVE state | NR_SmallData_INACTIVE-Perf | R4 | RP-212594 | ZTE |
| 860045 | Enhanced Industrial Internet of Things (IoT) and ultra-reliable and low latency communication (URLLC) support for NR | NR_IIOT_URLLC_enh |  | RP-210854 | Nokia |
| 860145 | Core part: Enhanced IoT and URLLC support for NR | NR_IIOT_URLLC_enh-Core | R2 | RP-210854 | Nokia |
| 860245 | Perf. Part: Enhanced IoT and URLLC support for NR | NR_IIOT_URLLC_enh-Perf | R4 | RP-210854 | Nokia |

Summary based on the input provided by XXX in xP-22xxxx.

## 5.6	Media production

| UID | Name | Acronym | WG | WID | WI rapporteur name/company |
| --- | --- | --- | --- | --- | --- |
| 840045 | Audio-Visual Service Production | AVPROD |  | SP-190304 | Wagdin, Ian, BBC |
| 800014 | Study on AVPROD | FS_AVPROD | S1 | SP-181015 | Roland Beutler, EBU |
| 840031 | Stage 1 of AVPROD | AVPROD | S1 | SP-191041 | Wagdin, Ian, BBC |
| 910001 | Study on Media Production over 5G NPN | FS_NPN4AVProd | S4 | SP-210241 | Lohmar, Thorsten, Ericsson |

Summary based on the input provided by XXX in xP-22xxxx.

See also "New bands and bandwidth allocation for 5G terrestrial broadcast - part 1"

## 5.7	Other services to verticals

| UID | Name | Acronym | WG | WID | WI rapporteur name/company |
| --- | --- | --- | --- | --- | --- |
| 840050 | Enhancements for cyber-physical control applications in vertical domains | eCAV |  | SP-190310 | Bahr, Michael, Siemens AG |
| 830020 | Study on eCAV | FS_eCAV | S1 | SP-190092 | Michael Bahr, Siemens AG |
| 840041 | Stage 1 of eCAV | eCAV | S1 | SP-191043 | Bahr, Michael, Siemens AG |

Summary based on the input provided by XXX in xP-22xxxx.

| 850046 | Asset Tracking for 5G | ATRAC |  | SP-190816 | Thierry Berisot, NOVAMINT |
| --- | --- | --- | --- | --- | --- |
| 810017 | Study on ATRAC | FS_5G_ATRAC | S1 | SP-180922 | Thierry Berisot, NOVAMINT |
| 850037 | Stage 1 of ATRAC | ATRAC | S1 | SP-190931 | Thierry Berisot, NOVAMINT |

| 840047 | Communication Service Requirements for Critical Medical Applications | CMED |  | SP-190306 | Lagrange, Mathieu, b<>com |
| --- | --- | --- | --- | --- | --- |
| 810016 | Study on CMED | FS_CMED | S1 | SP-180783 | Mathieu Lagrange, b<>com |
| 840033 | Stage 1 of CMED | CMED | S1 | SP-190306 | Lagrange, Mathieu, b<>com |

Summary based on the input provided by XXX in xP-22xxxx.

| 820025 | Study on application layer support for Factories of the Future in 5G network | FS_FFAPP | S6 | SP-200836 | Shao Weixiang, ZTE Corporation |
| --- | --- | --- | --- | --- | --- |
| 840025 | Study on enhancement of support for 5G LAN-type service | FS_5GLAN_enh | S2 | SP-190626 | Runze Zhou, Huawei |
| 900022 | Study on Charging Aspect of 5G LAN-type Services | FS_5GLAN_CH | S5 | SP-201081 | CHEN SHAN, Huawei |

# 6	5G access via Satellite/Non-Terrestrial (NTN) link

| UID | Name | Acronym | WG | WID | WI rapporteur name/company |
| --- | --- | --- | --- | --- | --- |
| 890034 | Integration of satellite components in the 5G architecture | 5GSAT_ARCH |  | SP-191335 | Cyril MICHEL, thales |
| 800048 | Stage 1 of 5GSAT | 5GSAT | S1 | SP-180326 | MICHEL, Cyril, Thales |
| 800026 | Study on architecture aspects for using satellite access in 5G | FS_5GSAT_ARCH | S2 | SP-181253 | MICHEL, Cyril |
| 860005 | (Stage 2 of) Integration of satellite components in the 5G architecture | 5GSAT_ARCH | S2 | SP-191335 | Cyril MICHEL, thales |
| 911030 | CT aspects of 5GC architecture for satellite networks | 5GSAT_ARCH-CT | ct | CP-210149 | Catovic, Amer, Qualcomm |
| 890005 | CT1 aspects of 5GC architecture for satellite networks | 5GSAT_ARCH-CT | C1 | CP-210149 | Catovic, Amer, Qualcomm |
| 920057 | CT3 aspects of 5GC architecture for satellite networks | 5GSAT_ARCH-CT | C3 | CP-210149 | Catovic, Amer, Qualcomm |
| 911031 | CT4 aspects of 5GC architecture for satellite networks | 5GSAT_ARCH-CT | C4 | CP-210149 | Catovic, Amer, Qualcomm |
| 930044 | CT6 aspects of 5GC architecture for satellite networks | 5GSAT_ARCH-CT | C6 | CP-210149 | Catovic, Amer, Qualcomm |
| 860046 | Solutions for NR to support non-terrestrial networks (NTN) | NR_NTN_solutions |  | RP-210908 | Thales |
| 860146 | Core part: Solutions for NR for NTN | NR_NTN_solutions-Core | R2 | RP-210908 | Thales |
| 860246 | Perf. Part: Solutions for NR for NTN | NR_NTN_solutions-Perf | R4 | RP-210908 | Thales |
| 920069 | NB-IoT/eMTC support for Non-Terrestrial Networks | LTE_NBIOT_eMTC_NTN | R1 | RP-211601 | MediaTek |
| 920169 | Core part: NB-IoT/eMTC support for Non-Terrestrial Networks | LTE_NBIOT_eMTC_NTN-Core | R1 | RP-211601 | MediaTek |
| 930019 | Architecture support for NB-IoT/eMTC Non-Terrestrial Networks in EPS | IoT_SAT_ARCH_EPS | S2 | SP-211124 | SEBIRE, Guillaume, MediaTek Inc. |
| 830025 | Study on management and orchestration aspects with integrated satellite components in a 5G network | FS_5GSAT_MO | S5 | SP-190138 | Floris Drijver, TNO |
| 860033 | Study on NB-IoT/eMTC support for NTN | FS_LTE_NBIOT_eMTC_NTN | R1 | RP-202689 | MediaTek |

Summary based on the input provided by XXX in xP-22xxxx.

# 7	V2V/D2D/Sidelink related

| 910037 | Support of advanced V2X services - Phase 2 | eV2XARC_Ph2 |  | SP-210090 | LaeYoung Kim, LG Electronics |
| --- | --- | --- | --- | --- | --- |
| 850013 | Study on V2X services – Phase 2 | FS_eV2XARC_Ph2 | S2 | SP-190631 | LaeYoung Kim, LG Electronics |
| 910021 | Stage 2 of eV2XARC_Ph2 | eV2XARC_Ph2 | S2 | SP-210090 | LaeYoung Kim, LG Electronics |
| 920005 | CT1 aspects of eV2XARC_Ph2 | eV2XARC_Ph2 | C1 | CP-211116 | Herrero Veron, Christian (Huawei) |
| 920047 | CT3 aspects of eV2XARC_Ph2 | eV2XARC_Ph2 | C3 | CP-211116 | Herrero Veron, Christian (Huawei) |
| 920048 | CT6 aspects of eV2XARC_Ph2 | eV2XARC_Ph2 | C6 | CP-211116 | Herrero Veron, Christian (Huawei) |

Summary based on the input provided by LG Electronics in SP-220277.

The study on the Phase 2 of the architecture enhancements for 3GPP support of advanced Vehicle-to-Everything (V2X) services (TR 23.776 [1]) concluded on the need for some normative work. This is what is specified by this work item, and deals with V2X communication over NR PC5 reference point (device-to-device) with power efficiency for pedestrian UEs, i.e. UEs for Vulnerable Road Users.

The support of QoS-aware NR PC5 power efficiency for pedestrian UEs is specified in TS 23.287 [2] as below:

-	Overall description about support of QoS-aware NR PC5 power efficiency for pedestrian UEs regarding NR PC5 Discontinuous Reception (DRX) operations.

-	PC5 DRX configuration, e.g. the mapping of PC5 QoS profile(s) to PC5 DRX cycle(s), default PC5 DRX configuration, for broadcast and groupcast when the UE is "not served by E-UTRA" and "not served by NR" as provisioned parameters for V2X communications over PC5 reference point.

-	NR Tx Profile for broadcast and groupcast as provisioned parameters for V2X communications over PC5 reference point.

For NR based unicast, groupcast and broadcast mode communication over PC5 reference point, PC5 DRX operations are supported to enable pedestrian UE power saving.

The V2X layer determines the respective V2X service types, and derives the corresponding PC5 QoS parameters based on either the mapping of V2X service types to PC5 QoS parameters, or the V2X Application Requirements for the V2X service type provided by the application layer. The V2X layer passes the PC5 QoS parameters and destination Layer-2 ID to the AS layer. For broadcast and groupcast, the V2X layer also determines the NR Tx Profile for the respective V2X service type based on the mapping of V2X service types to NR Tx Profiles and provides the NR Tx Profile to the AS layer.

When the PC5 DRX operation is needed, e.g. based on the NR Tx Profile in case of broadcast or groupcast, the AS layer determines the PC5 DRX parameter values for V2X communication over PC5 reference point, taking into account, e.g., PC5 QoS parameters and/or destination Layer-2 ID provided by the V2X layer.

For unicast, two UEs may negotiate the PC5 DRX configuration in the AS layer, and the PC5 DRX parameter values can be configured per pair of source and destination Layer-2 IDs and per direction in the AS layer.

For broadcast and groupcast when the UE is "not served by E-UTRA" and "not served by NR", the UE uses the provisioned PC5 DRX configuration for PC5 DRX operation.

Based on the Stage 2 requirements to support NR PC5 DRX operations, Stage 3 normative works are specified in TS 24.587 [3], TS 24.588 [4] and TS 31.102 [5].

References

List of related CRs: select "TSG Status = Approved" in: https://portal.3gpp.org/ChangeRequests.aspx?q=1&workitem=910037,850013,910021,920005,920047,920048

[1]	TR 23.776: "Study on architecture enhancements for 3GPP support of advanced Vehicle-to-Everything (V2X) services; Phase 2".

[2]	TS 23.287: "Architecture enhancements for 5G System (5GS) to support Vehicle-to-Everything (V2X) services".

[3]	TS 24.587: "Vehicle-to-Everything (V2X) services in 5G System (5GS); Stage 3".

[4]	TS 24.588: "Vehicle-to-Everything (V2X) services in 5G System (5GS); User Equipment (UE) policies; Stage 3".

[5]	TS 31.102: "Characteristics of the Universal Subscriber Identity Module (USIM) application".

| UID | Name | Acronym | WG | WID | WI rapporteur name/company |
| --- | --- | --- | --- | --- | --- |
| 860042 | NR Sidelink enhancement | NR_SL_enh |  | RP-202846 | LG Electronics |
| 860142 | Core part: NR Sidelink enhancement | NR_SL_enh-Core | R1 | RP-202846 | LG Electronics |
| 860242 | Perf. part: NR Sidelink enhancement | NR_SL_enh-Perf | R4 | RP-202846 | LG Electronics |
| 860038 | Study on NR Sidelink relay | FS_NR_SL_relay | R2 | RP-202208 | OPPO |
| 840035 | Study on enhancements to application layer support for V2X services | FS_eV2XAPP | S6 | SP-200110 | Niranth Amogh, Huawei Telecommunications India |
| 910075 | Enhanced application layer support for V2X services | eV2XAPP |  | SP-200831 | Niranth Amogh, Huawei Telecommunications India |
| 890036 | Stage 2 of eV2XAPP | eV2XAPP | S6 | SP-200831 | Niranth Amogh, Huawei Telecommunications India |
| 910019 | CT aspects of eV2XAPP | eV2XAPP | ct | CP-211109 | Herrero Veron, Christian (Huawei) |
| 910076 | CT1 aspects of eV2XAPP | eV2XAPP | C1 | CP-211109 | Herrero Veron, Christian (Huawei) |
| 910077 | CT3 aspects of eV2XAPP | eV2XAPP | C3 | CP-211109 | Herrero Veron, Christian (Huawei) |
| 900030 | Proximity based Services in 5GS | 5G_ProSe |  | SP-200972 | TBD |
| 830033 | Study on System enhancement for Proximity based Services in 5GS | FS_5G_ProSe | S2 | SP-190443 | Qiang Deng, CATT |
| 900007 | Stage 2 for Proximity based Services in 5GS | 5G_ProSe | S2 | SP-201096 | Qiang Deng, CATT |
| 910018 | CT aspects of proximity based services in 5GS | 5G_ProSe | ct | CP-212105 | Yong Jiang, CATT |
| 910072 | CT1 aspects of 5G_ProSe | 5G_ProSe | C1 | CP-212105 | Yong Jiang, CATT |
| 910073 | CT3 aspects of 5G_ProSe | 5G_ProSe | C3 | CP-212105 | Yong Jiang, CATT |
| 910074 | CT4 aspects of 5G_ProSe | 5G_ProSe | C4 | CP-212105 | Yong Jiang, CATT |
| 920081 | CT6 aspects of 5G_ProSe | 5G_ProSe | C6 | CP-212105 | Yong Jiang, CATT |
| 890018 | Study on charging aspects of Proximity-based Services in 5GS | FS_5G_Prose_CH | S5 | SP-200767 | Shu, Min, CATT |
| 880005 | Study on Security Aspects of Enhancement for Proximity Based Services in 5GS | FS_5G_ProSe_Sec | S3 | SP-200350 | Wei Zhou, CATT |
| 930008 | Security Aspects of Proximity based Services in 5GS | 5G_ProSe | S3 | SP-211120 | Wei Zhou, CATT |
| 911005 | NR Sidelink Relay | NR_SL_relay |  | RP-212601 | OPPO |
| 911105 | Core part: NR Sidelink Relay | NR_SL_relay-Core | R2 | RP-211050 | OPPO |
| 911205 | Perf. part: NR Sidelink Relay | NR_SL_relay-Perf | R4 | RP-211050 | OPPO |

Summary based on the input provided by XXX in xP-22xxxx.

# 8	Enhancements to network architecture and application enablement

## 8.1	Edge computing

| 920019 | Edge Computing Management | ECM | S5 | SP-210388 | Deepanshu Gautam, Samsung |
| --- | --- | --- | --- | --- | --- |

Summary based on the input provided by Samsung in SP-220020.

The ECM Work Item and the resulting specification in TS 28.538[4] provides management provisions and solutions for edge computing considering related requirements from SA6 EDGEAPP WI including lifecycle management, provisioning, performance assurance and fault supervision for EDGEAPP defined entities.

Edge computing is a well-known industry concept, with 3GPP 5G System Architecture supporting Edge computing deployments by enabling certain features as listed in sub clause 5.13 of TS 23.501[1]. SA6, as part of TS 23.558[2], have defined the functionality and concepts required for enabling Edge Applications in 3GPP networks. The functionalities defined to be deployed in 3GPP networks include Edge Application, Edge Enabler Server and Edge Data Network Configuration Server. The management provisions for these functionalities are addressed as part of this WI. The management solutions in preview of this WI includes.

| - Provisioning and LCM: Provisioning includes configuration and lifecycle management. It deals with defining NRMs (Network Resource Model) for the edge entities to be managed and also defining usage of those NRMs by the management services to achieve provisioning and lifecycle management. - Performance Assurance: It deals with defining edge specific performance measurements and KPIs. The collection mechanism for the same is also defined. - Fault Supervision: It deals with defining edge specific alarm. The collection mechanism for the same is also defined. - Connection to 5GC: Connecting edge nodes to 5GC is crucial part of edge computing management. The defined NRM fragments to enable the same. |  |
| --- | --- |

References

List of related CRs: select "TSG Status = Approved" in: https://portal.3gpp.org/ChangeRequests.aspx?q=1&workitem=920019

[1]	TS 23.501: “System architecture for the 5G System (5GS)”

[2]	TS 23.558: “Architecture for enabling Edge Applications”

[3]	TR 28.814: “Study on enhancements of edge computing management”

[4]	TS 28.538: “Edge Computing Management”

| UID | Name | Acronym | WG | WID | WI rapporteur name/company |
| --- | --- | --- | --- | --- | --- |
| 880042 | Architecture for enabling Edge Applications | EDGEAPP |  | SP-200109 | Gupta, Nishant, Samsung |
| 830008 | Study on Application Architecture for enabling Edge Applications | FS_EDGEAPP | S6 | SP-190065 | Nishant Gupta, Samsung |
| 860006 | Architecture for enabling Edge Applications | EDGEAPP | S6 | SP-200886 | Gupta, Nishant, Samsung |
| 900006 | CT aspects for Enabling Edge Applications | EDGEAPP |  | CP-211196 | Narendranath Durga Tangudu |
| 900034 | CT1 aspects for Enabling Edge Applications | EDGEAPP | C1 | CP-211196 | Narendranath Durga Tangudu |
| 900035 | CT3 aspects for Enabling Edge Applications | EDGEAPP | C3 | CP-211196 | Narendranath Durga Tangudu |
| 830032 | Study on enhancement of support for Edge Computing in 5GC | FS_enh_EC | S2 | SP-200093 | Hui Ni, Huawei |
| 910048 | Enhancement of support for Edge Computing in 5G Core network | eEDGE_5GC |  | SP-201107 | Hui Ni, Huawei Technologies |
| 900016 | Stage 2 of eEDGE_5GC | eEDGE_5GC | S2 | SP-201107 | Hui Ni, Huawei Technologies |
| 910005 | CT aspects of 5G eEDGE | eEDGE_5GC | ct | CP-212021 | Qi Caixia, Huawei |
| 910049 | CT1 aspects of 5G eEDGE | eEDGE_5GC | C1 | CP-212021 | Qi Caixia, Huawei |
| 910050 | CT3 aspects of 5G eEDGE | eEDGE_5GC | C3 | CP-212021 | Qi Caixia, Huawei |
| 910051 | CT4 aspects of 5G eEDGE | eEDGE_5GC | C4 | CP-212021 | Qi Caixia, Huawei |
| 920026 | Security Aspects of Enhancements of Support for Edge Computing in 5GC | eEDGE_5GC | S3 | SP-210423 | Bo Zhang, Huawei Technologies |
| 880002 | Study on Security Aspects of Enhancement of Support for Edge Computing in 5GC | FS_eEDGE_Sec | S3 | SP-200347 | Bo Zhang, Huawei Technologies |
| 930034 | Charging aspects of Edge Computing | EDGE_CH | S5 | SP-210861 | Yao, Yizhi, Intel |
| 880030 | Study on charging aspects of Edge Computing | FS_EDGE_CH | S5 | SP-200467 | Yizhi Yao, Intel |
| 930011 | Charging aspects of Edge Computing | EDGE_CH | S5 | SP-210861 | Yao, Yizhi, Intel |
| 870029 | Study on enhancements of edge computing management | FS_eEDGE_Mgt | S5 | SP-200195 | Joey Chou, Intel |

Summary based on the input provided by XXX in xP-22xxxx.

## 8.2	Slicing

| 9500xx | Network Slice charging based on 5G Data Connectivity | NETSLICE_DC_CH | S5 | SP-220158 | MATRIXX Software, Gerald Görmer |
| --- | --- | --- | --- | --- | --- |

Summary based on the input provided by MATRIXX Software in SP-220075.

In 5G system when Network Slices allocated to third party providers are deployed by Mobile Network Operators, how these third-party providers can be charged for usage of assigned Network Slice(s), is not described. This work item introduces a description focusing on one particular type of Network Slice usage, leveraging from capabilities specified from Rel-15.

5G Data Connectivity charging specified in 5GS between SMF and CHF for individual UEs PDU sessions from Rel-15, is used with Converged Charging System (CCS) hosting the CHF extended to also cover the third-party provider (addressed under the tenant concept).

By receiving the S-NSSAI for each UE PDU session, the enhanced CCS is able to perform per tenant charging based on tenant's Network Slice total UEs data connectivity usage, the Network Slice being identified by the S-NSSAI.

The internal behaviour of the enhanced CCS is not specified in this release.

References

List of related CRs: SP-220157: "Introduction of Annex on Network slice charging"

[1]	TS 32.255: "Telecommunication management; Charging management; 5G Data connectivity domain charging; stage 2".

| UID | Name | Acronym | WG | WID | WI rapporteur name/company |
| --- | --- | --- | --- | --- | --- |
| 900032 | Enhancement of Network Slicing Phase 2 | eNS_Ph2 |  | SP-200976 | ZTE, Jinguo Zhu |
| 850010 | Study on Enhancement of Network Slicing Phase 2 | FS_eNS_Ph2 | S2 | SP-190931 | So, Tricci, ZTE |
| 900011 | Stage 2 for Enhancement of Network Slicing Phase 2 | eNS_Ph2 | S2 | SP-200976 | ZTE, Jinguo Zhu |
| 920059 | Stage 3 for Enhancement of Network Slicing Phase 2 | eNS_Ph2 | ct | CP-211091 | Hannah Wang, ZTE |
| 910041 | CT1 aspects of eNS_Ph2 | eNS_Ph2 | C1 | CP-211091 | Hannah Wang, ZTE |
| 910042 | CT3 aspects of eNS_Ph2 | eNS_Ph2 | C3 | CP-211091 | Hannah Wang, ZTE |
| 910043 | CT4 aspects of eNS_Ph2 | eNS_Ph2 | C4 | CP-211091 | Hannah Wang, ZTE |
| 911007 | Enhancement of RAN slicing for NR | NR_slice | R2 | RP-212534 | CMCC |
| 911107 | Core part: Enhancement of RAN slicing for NR | NR_slice-Core | R2 | RP-212534 | CMCC |
| 860022 | Study on network slice management enhancement | FS_NSMEN | S5 | SP-200766 | Brendan Hassett, Huawei Technologies Sweden AB, |
| 900023 | Study on Charging Aspects for Network Slicing Phase 2 | FS_NETSLICE_CH_Ph2 | S5 | SP-201082 | Matrixx, Gerald Görmer |
| 910023 | Study on enhanced security for network slicing Phase 2 | FS_eNS2_SEC | S3 | SP-210106 | Zander Lei, Huawei |
| 910026 | Study on network slice management capability exposure | FS_NSCE | S5 | SP-210131 | Xiaobo Yu, Alibaba Group |
| 910095 | Study on 5G NR UE full stack testing for Network Slicing | FS_NR_Slice_Test | R5 | RP-211977 | CMCC |

Summary based on the input provided by XXX in xP-22xxxx.

## 8.3	Access Traffic Steering, Switch and Splitting support in the 5G system architecture; Phase 2

| UID | Name | Acronym | WG | WID | WI rapporteur name/company |
| --- | --- | --- | --- | --- | --- |
| 900033 | Access Traffic Steering, Switch and Splitting support in the 5G system architecture; Phase 2 | ATSSS_Ph2 |  | SP-200977 | Apostolis Salkintzis, Lenovo |
| 840084 | Study on ATSSS_Ph2 | FS_ATSSS_Ph2 | S2 | SP-200095 | So, Tricci, ZTE |
| 900012 | Stage 2 of ATSSS_Ph2 | ATSSS_Ph2 | S2 | SP-200977 | Apostolis Salkintzis, Lenovo |
| 910013 | CT aspects of ATSSS_Ph2 | ATSSS_Ph2 | ct | CP-210136 | ZHOU Xingyue (Joy), ZTE |
| 910056 | CT1 aspects of ATSSS_Ph2 | ATSSS_Ph2 | C1 | CP-210136 | ZHOU Xingyue (Joy), ZTE |
| 910057 | CT3 aspects of ATSSS_Ph2 | ATSSS_Ph2 | C3 | CP-210136 | ZHOU Xingyue (Joy), ZTE |
| 910058 | CT4 aspects of ATSSS_Ph2 | ATSSS_Ph2 | C4 | CP-210136 | ZHOU Xingyue (Joy), ZTE |

Summary based on the input provided by XXX in xP-22xxxx.

## 8.4	Self-Organizing (SON)/Autonomous Network

| UID | Name | Acronym | WG | WID | WI rapporteur name/company |
| --- | --- | --- | --- | --- | --- |
| 860053 | Enhancement of data collection for SON (Self-Organising Networks)/MDT (Minimization of Drive Tests) in NR and EN-DC | NR_ENDC_SON_MDT_enh |  | RP-193255 | CMCC |
| 860153 | Core part: Enhancement of data collection for SON (Self-Organising Networks)/MDT (Minimization of Drive Tests) in NR and EN-DC | NR_ENDC_SON_MDT_enh-Core | R3 | RP-193255 | CMCC |
| 910029 | Plug and connect support for management of Network Functions | PACMAN | S5 | SP-210260 | Per Elmdahl, Ericsson |
| 870028 | Enhancements of Self-Organizing Networks (SON) for 5G networks | eSON_5G | S5 | SP-200194 | Joey Chou, Intel |
| 850032 | Study on autonomous network levels | FS_ANL | S5 | SP-190928 | China Mobile |
| 880027 | Autonomous network levels | ANL | S5 | SP-200464 | Cao, Xi, China Mobile |
| 900031 | Enablers for Network Automation for 5G - phase 2 | eNA_Ph2 |  | SP-200975 | TBD |
| 840022 | Study on Enablers for Network Automation for 5G - phase 2 | FS_eNA_Ph2 | S2 | SP-200098 | Xiaobo Wu, Huawei Technologies |
| 890015 | Study on security aspects of enablers for Network Automation (eNA) for 5GS Phase 2 | FS_eNA_SEC | S3 | SP-200722 | Xiaoting Huang, China Mobile |
| 900010 | Stage 2 of eNA_Ph2 | eNA_Ph2 | S2 | SP-200975 | TBD |
| 910012 | CT aspects of eNA_Ph2 | eNA_Ph2 | ct | CP-211191 | Huang Zhenning (China Mobile) |
| 910088 | CT3 aspects of eNA_Ph2 | eNA_Ph2 | C3 | CP-211191 | Huang Zhenning (China Mobile) |
| 910089 | CT4 aspects of eNA_Ph2 | eNA_Ph2 | C4 | CP-211191 | Huang Zhenning (China Mobile) |
| 930007 | Security aspects of eNA_Ph2 | eNA_Ph2 | S3 | SP-210837 | Chang Liu, China Mobile |

Summary based on the input provided by XXX in xP-22xxxx.

## 8.5	Minimization of service Interruption

| UID | Name | Acronym | WG | WID | WI rapporteur name/company |
| --- | --- | --- | --- | --- | --- |
| 850045 | Support for Minimization of service Interruption | MINT |  | SP-190814 | SungDuck Chun; LG Electronics |
| 830018 | Study on MINT | FS_MINT | S1 | SP-190090 | SungDuck Chun; LG Electronics |
| 850036 | Stage 1 of MINT | MINT | S1 | SP-190938 | SungDuck Chun; LG Electronics |
| 920062 | Stage 2 of MINT | MINT | S2 | SP-210582 | Hyunsook Kim, LG Electronics |
| 900004 | Study on the CT aspects of Support for Minimization of service Interruption | FS_MINT-CT | C1 | CP-203232 | Sang Min Park, LG Electronics |
| 930003 | Stage 3 of MINT | MINT | ct | CP-212166 | Sang Min Park |
| 930049 | CT1 aspects of MINT | MINT | C1 | CP-212166 | Sang Min Park |
| 930045 | CT4 aspects of MINT | MINT | C4 | CP-212166 | Sang Min Park |
| 930048 | CT6 aspects of MINT | MINT | C6 | CP-212166 | Sang Min Park |

Summary based on the input provided by XXX in xP-22xxxx.

# 9	Multicast-Broadcast Services (MBS) and Mission Critical (MC)

| 890039 | Mission Critical Data | eMCData3 |  | SP-191106 | Shih, Jerry, AT&T |
| --- | --- | --- | --- | --- | --- |
| 860007 | Stage 2 of MCData3 | eMCData3 | S6 | SP-191106 | Shih, Jerry, AT&T |
| 890038 | CT aspects of eMCData3 | eMCData3 | C1 | CP-201177 |  |

Summary based on the input provided by at&t in SP-220089.

For Release 17, the enhancements for the MCData service were defined by the two work items above. The following functionalities have been introduced:

- new network-based MCData notification server. The MCData notification server provides the centralized notification function in the network that allows an application (e.g. resident in the UE) to create a communication channel to receive real-time notifications from the network in either Pull or Push mode.

- MCData communication (SDS or file distribution) supports using functional alias as target end points, except FD using HTTP.

- new "search folder" and "retrieve folder content" operations to MCData message store operations.

- Support application specific metadata container in MCData communication for application specific handling.

References

List of related CRs: select "TSG Status = Approved" in: https://portal.3gpp.org/ChangeRequests.aspx?q=1&workitem=890039,860007,890038

[1]	TS 22.282 Mission Critical Data services; Stage 1;

[2]	TS 22.280 Mission Critical Services Common Requirements (MCCoRe); Stage 1;

[3]	TS 23.282 Functional architecture and information flows to support Mission Critical Data (MCData) Stage 2 (together with 23.280, it specifies the architecture, including information flows, procedures, and configuration) ;

[4]	TS 23.280 Common functional architecture to support mission critical services; Stage 2;

[5]	TS 23.303 Proximity-based services (ProSe); Stage 2 (ProSe is an enabler for MC services);

[6]	TS 23.468 Group Communication System Enablers for LTE (GCSE_LTE); Stage 2;

[7]	TS 24.282 Mission Critical Data (MCData) signalling control (specifies the protocol aspects for call control);

[8]	TS 24.582 Mission Critical Data (MCData) media plane control(specifies the protocol aspects for media plane);

[9]	TS 24.481 Mission Critical Services (MCS) group management; Protocol specification;

[10]	TS 24.482 Mission Critical Services (MCS) identity management; Protocol specification;

[11]	TS 24.483 Mission Critical Services (MCS) Management Object (MO);

[12]	TS 24.484 Mission Critical Services (MCS) configuration management; Protocol specification;

[13]	TS 29.213 Policy and Charging Control signalling flows and Quality of Service (QoS) parameter mapping;

[14]	TS 29.214: Policy and Charging Control over Rx reference point;

[15]	TS 29.283: Diameter data management application (specifies the protocol aspects for data management related to MC service user profile )

[16]	TS 33.180: Security of the mission critical service.

| 890003 | CT aspects of Enhanced Mission Critical Communication Interworking with Land Mobile Radio Systems | eMCCI_CT | C1 | eMCCI_CT | Mike Dolan, FirstNet |
| --- | --- | --- | --- | --- | --- |

Summary based on the input provided by FirstNet in CP-220110.

eMCCI_CT refers to stage 3 aspects of the stage 2 work defined by eMCCI in Rel-16.

Note: 	This does not comply with the 3GPP methodology: all stages shall be defined within a same Release and use the same Feature/acronym.

It covers enhancements to interworking 3GPP mission critical systems with Land Mobile Radio (LMR, i.e. public safety communication networks) systems in Rel-17 to provide support for a conference event package, affiliation on behalf of a set of users, and private call floor control.

The Rel-17 IWF (interworking function) now supports the ability for the IWF to handle subscriptions to events that may occur in the LMR system, and for the IWF to be able to subscribe on behalf of LMR users to events in the 3GPP mission critical system (MCPTT and MCData).

Affiliation on behalf of a set of users is now handled from the IWF toward the 3GPP mission critical system to potentially reduce the signalling load due to repetitive messaging.

Private call floor control is now supported between an MCPTT system and an IWF.

References

List of related CRs: select "TSG Status = Approved" in: https://portal.3gpp.org/ChangeRequests.aspx?q=1&workitem=890003

[1]	TS 29.379: “Mission Critical Push To Talk (MCPTT) call control interworking with Land Mobile Radio (LMR) systems”

[2]	TS 29.380: “Mission Critical Push To Talk (MCPTT) media plane control interworking with Land Mobile Radio (LMR) systems”

[3]	TS 29.582: “Mission Critical Data (MCData) interworking with Land Mobile Radio (LMR) systems”

| 870034 | Enhanced Mission Critical Push-to-talk architecture phase 3 | enh3MCPTT |  | SP-200108 | Dom Lazara; Motorola Solutions |
| --- | --- | --- | --- | --- | --- |
| 870016 | Stage 2 aspects of enh3MCPTT | enh3MCPTT | S6 | SP-200108 | Dom Lazara; Motorola Solutions |
| 900039 | Stage 3 aspects of enh3MCPTT | enh3MCPTT-CT | C1 | CP-202188 |  |

Summary based on the input provided by FirstNet in CP-220111.

This WI covers work on enhancements to the MCPTT feature as to provide support for an MCPTT user logged into multiple devices, support for z-axis location measurements, preconfigured regroup support, and unicast media stop/resume.

The Rel-17 system now supports multiple registrations for the same mission critical user, such as might happen if a public safety person uses a tablet, an in-vehicle computer, and a smartphone.

Location services reports can now include z-axis information in support of situational awareness, such as might be needed to determine what floor of a building a firefighter is on.

User and group regroups can now be based on a group that is to be used only for preconfigured regroups. Clients in the device will reject attempts to make group calls directly to groups marked for preconfigured group use only. In addition, the preconfigured group capabilities have been added to MCVideo and MCData.

For MCPTT, it is now possible for a client to indicate to the server that it wants to stop/resume receiving media for a given group call. This may be needed to preserve radio resources when the MCPTT user is participating in another higher priority group call.

Finally, a number of issues that were found during ETSI Plugtest events have been fixed or clarified.

References

List of related CRs: select "TSG Status = Approved" in: https://portal.3gpp.org/ChangeRequests.aspx?q=1&workitem=870034,870016,900039

[1]	TS 24.281: “Mission Critical Video (MCVideo) signalling control”

[2]	TS 24.282: “Mission Critical Data (MCData) signalling control”

[3]	TS 24.379: “Mission Critical Push To Talk (MCPTT) call control”

[4]	TS 24.380: “Mission Critical Push To Talk (MCPTT) media plane control”

[5]	TS 24.483: “Mission Critical Services (MCS) Management Object (MO)”

[6]	TS 24.484: “Mission Critical Services (MCS) configuration management”

| 890011 | Mission critical security enhancements phase 2 | MCXSec2 | S3 | SP-200879 | Woodward, Tim, Motorola Solutions, Inc. |
| --- | --- | --- | --- | --- | --- |

Summary based on the input provided by Motorola Solutions in SP-220019.

Mission critical (MC) services security enhancements phase 2 defines the confidentiality, integrity, user authentication, service authorization, and overall security architecture for Release 17 mission critical services (MCPTT, MCVideo, MCData, MC Location, MC Interworking, MC Interconnection, and MC Railway).

Release 17 expands on the mission critical security architecture already defined in previous releases and includes some mission critical security clarifications and corrections.

In this release, mission critical user service authorization and security for the mission critical MCData message store service.  Similar to user service authorization for the other MC services, an appropriately scoped access token obtained from the Identity Management server permits only authorized users the authorization to access and use the MCData message store service.

Security for Preconfigured Group Regroup and Preconfigured User Regroup calls defines the use of the preconfigured group to establish the security context.

Enhancements to the security architecture to support mission critical security services over a 5G system.  This includes the mission critical security architecture, which describes the use and integration of 5G nodes and servers.

References

List of related CRs: select "TSG Status = Approved" in: https://portal.3gpp.org/ChangeRequests.aspx?q=1&workitem=890011

[1]	TS 33.180: “Security of the Mission Critical (MC) service; (Release 17)”

| UID | Name | Acronym | WG | WID | WI rapporteur name/company |
| --- | --- | --- | --- | --- | --- |
| 900038 | Multicast-broadcast services in 5G | 5MBS |  | SP-201106 | Meng Li, Huawei |
| 830030 | Study on Architectural enhancements for 5G multicast-broadcast services | FS_5MBS | S2 | SP-200690 | Meng Li, Huawei |
| 900009 | Stage 2 for 5MBS | 5MBS | S2 | SP-201106 | Meng Li, Huawei |
| 920043 | CT1 aspects of 5MBS | 5MBS | C1 | CP-212022 | Gulbani, Giorgi, Huawei |
| 920044 | CT3 aspects of 5MBS | 5MBS | C3 | CP-212022 | Gulbani, Giorgi, Huawei |
| 910002 | CT4 aspects of 5MBS | 5MBS | C4 | CP-212022 | Gulbani, Giorgi, Huawei |
| 920023 | Security Aspects of Enhancements for 5G Multicast-Broadcast Services | 5MBS | S3 | SP-210420 | Longhua Guo, Huawei Technologies |
| 870014 | Study on Multicast Architecture Enhancements for 5G Media Streaming | FS_5GMS_Multicast | S4 | SP-200238 | Peng Tan, Telus |
| 810055 | Study on location enhancements for mission critical services | FS_enhMCLoc | S6 | SP-190725 | Dom Lazara, Motorola Solutions |
| 850040 | Broadcast / Multicast requirements supporting Mission Critical Services in 5G | 5MBS_eMC | S1 | SP-190942 | Toobe, Jens, BDBOS |
| 880006 | Study on Security Aspects of Enhancements for 5G Multicast-Broadcast Services | FS_5MBS_SEC | S3 | SP-200351 | Longhua Guo, Huawei Technologies |
| 850035 | Study on Mission Critical services over 5G multicast-broadcast system | FS_MC5MBS | S6 | SP-190929 | Val Oprescu, AT&T |
| 860048 | NR multicast and broadcast services | NR_MBS | R2 | RP-193248 | Huawei |
| 860148 | Core part: NR multicast and broadcast services | NR_MBS-Core | R2 | RP-201038 | Huawei |
| 800023 | Study on Mission Critical services support over 5G System | FS_MCOver5GS | S6 | SP-200837 | Verweij, Kees, The Police of the Netherlands |
| 920051 | Mission Critical Services over 5GS | MCOver5GS |  | SP-200833 | Wendler, Ingo, UIC |
| 890027 | Stage 2 of MCOver5GS | MCOver5GS | S6 | SP-200833 | Wendler, Ingo, UIC |
| 920007 | CT1 aspects of MCOver5GS | MCOver5GS | C1 | CP-211118 | Gkatzikis, Lazaros, Nokia |
| 820040 | Mission Critical system migration and interconnection | MCSMI_CT | C1 | CP-190143 | Dom Lazara, Motorola Solutions |

Summary based on the input provided by XXX in xP-22xxxx.

# 10	Multi-(U)SIM

| UID | Name | Acronym | WG | WID | WI rapporteur name/company |
| --- | --- | --- | --- | --- | --- |
| 840049 | Support for Multi-USIM Devices | MUSIM |  | SP-190309 | Liao, Ellen C, Intel |
| 830019 | Study on MUSIM | FS_MUSIM | S1 | SP-190091 | Liao, Ellen C, Intel |
| 840040 | Stage 1 of MUSIM | MUSIM | S1 | SP-190309 | Liao, Ellen C, Intel |
| 820012 | Study on Stage 2 (System Enablers) for MUSIM | FS_MUSIM | S2 | SP-200297 | Sašo Stojanovski, Intel |
| 900013 | System enablers for Multi-USIM devices | MUSIM | S2 | SP-210091 | Sašo Stojanovski, Intel |
| 910015 | CT aspects of MUSIM | MUSIM | ct | CP-212102 | Vivek Gupta, Intel |
| 910063 | CT1 aspects of Enabling Multi-USIM Devices | MUSIM | C1 | CP-212102 | Vivek Gupta, Intel |
| 910064 | CT4 aspects of Enabling Multi-USIM Devices | MUSIM | C4 | CP-212102 | Vivek Gupta, Intel |
| 900017 | Study on the security of the system enablers for devices having multiple Universal Subscriber Identity Modules (USIM) | FS_MUSIM_SEC | S3 | SP-201018 | Abhijeet Kolekar, Intel Corporation |
| 860063 | Support for Multi-SIM devices for LTE/NR | LTE_NR_MUSIM |  | RP-212610 | Vivo |
| 860163 | Core part: Support for Multi-SIM devices for LTE/NR | LTE_NR_MUSIM-Core | R2 | RP-212610 | Vivo |

Summary based on the input provided by XXX in xP-22xxxx.

# 11	Messaging improvements

| UID | Name | Acronym | WG | WID | WI rapporteur name/company |
| --- | --- | --- | --- | --- | --- |
| 930030 | Support of the 5GMSG Service | FS_5GMARCH | S6 | SP-200835 | Liu, Yue, China Mobile |
| 840036 | Study on support of the 5GMSG Service | FS_5GMARCH | S6 | SP-200835 | Liu, Yue, China Mobile |
| 890010 | Study on security aspects of the 5GMSG Service (St1 in Rel16) | FS_SEC_5GMSG | S3 | SP-200878 | Xiaoting Huang, China Mobile |
| 930005 | Security aspects of the 5GMSG Service | 5GMSG | S3 | SP-210835 | Xiaoting Huang, China Mobile |
| 930040 | Application Architecture for MSGin5G Service | 5GMARCH |  | SP-200832 | Liu, Yue, China Mobile |
| 890026 | Application Architecture for MSGin5G Service | 5GMARCH | S6 | SP-200832 | Liu, Yue, China Mobile |
| 930004 | CT aspects for enabling MSGin5G Service | 5GMARCH | ct | CP-212106 | Liu, Yue, China Mobile |
| 930041 | CT1 aspects for enabling MSGin5G Service | 5GMARCH | C1 | CP-212106 | Liu, Yue, China Mobile |
| 930042 | CT3 aspects for enabling MSGin5G Service | 5GMARCH | C3 | CP-212106 | Liu, Yue, China Mobile |

Summary based on the input provided by China Mobile in SP-220285.

This Feature improves the messaging communication capability of the 5G System, especially for Massive Internet of Things (MIoT). It is based on the study in TR 23.700-24 [4].

Note: 	The different stages have been spread on different Releases, and on different Features, using different acronyms: Stage 1 is defined in Rel-16 by "MSGin5G" (TS 22.262 [2]), Stages 2 (TS 23.554 [1] ) and Stage 3 are defined in Rel-17 by "5GMARCH" and Security aspects have been defined in Rel-17 by "5GMSG". This does not comply with the 3GPP methodology (all stages shall be defined within a same Release and using the same Feature/acronym). The name used to refer to this service in the specifications and in this summary is "MSGin5G".

The following figure, from TS 23.554 [1], provides the high level illustration of the MSGin5G service:

Figure 1: Application Architecture of the MSGin5G Service

The following functions are specified:

a)	Configuration of MSGin5G UE and Non-MSGin5G UE to get the MSGin5G Service configuration information (e.g. UE Service ID);

b)	Registration of MSGin5G UE and registration of Message Gateway on behalf of the Non-3GPP UEs;

c)	Message delivery procedures for the message communication models listed below (the message can be delivered between different PLMNs):

•	Point-to-point message which happens between a person and a thing or two things;

•	Application-to-point message/ Point-to-application message which happens between an application server and an IoT device;

•	Group message which originates at a UE and terminated at a group of UEs, all members in the group can send and receive the message;

•	Broadcast message which originates at an application sever in the network or an UE and terminated at all the UEs in a specific service area within a cell or multiple cells;

•	Message delivery based on messaging topic which the message is delivered to all subscribers (UE or Application Server) of this messaging topic.

d)	Message Aggregation can be used to optimize communications towards the same target by aggregating one or more messages into a single message;

e)	MSGin5G message can be segmented transmission if the content is larger than the maximum payload length of a message and reassembled by the suitable recipient;

f)	MSGin5G message can be stored by the MSGin5G Server if a UE is unavailable (disconnected or power off) for future delivery once the UE becomes available;

g)	3GPP network functionalities of UE reachability status monitoring and device triggering can be leveraged by the MSGin5G Service via Core Network exposure.

h)	The Service Enabler Architecture Layer for Verticals (SEAL) capabilities "Group management service" and "Configuration management service" can be used by the MSGin5G service. These services are specified in TS 23.434 [6] and TS 23.554 [1].

References

List of related CRs: select "TSG Status = Approved" in: https://portal.3gpp.org/ChangeRequests.aspx?q=1&workitem=930030,840036,890010,930005,930040,890026,930004,930041,930042

[1]	TS 23.554: "Application architecture for MSGin5G Service; Stage 2" (specifies functional architecture, procedures, information flows and APIs )

[2]	TS 22.262: "Message Service within the 5G System".

[3]	TS 33.501: "Security architecture and procedures for 5G System".

[4]	TS 24.538: "Enabling MSGin5G Service;Protocol specification" (specifies detailed procedures over CoAP protocol between MSGin5G Client and MSGin5G Server)

[5]	TS 29.538: "Enabling MSGin5G Service;Application Programming Interfaces (API) specification;" (specifies the RESTful APIs provided by MSGin5G Server towards the Application Server and Message Gateway)

[6]	TS 23.434: "Service Enabler Architecture Layer for Verticals".

| 890001 | Service-based support for SMS in 5GC | SMS_SBI |  | CP-212023 | HAOUARI, Wafa, Orange |
| --- | --- | --- | --- | --- | --- |
| 890028 | CT1 aspects of SMS_SBI | SMS_SBI | C1 | CP-212023 | HAOUARI, Wafa, Orange |
| 890029 | CT4 aspects of SMS_SBI | SMS_SBI | C4 | CP-212023 | HAOUARI, Wafa, Orange |

# 12	Drone/UAS/UAV/EAV identification

| UID | Name | Acronym | WG | WID | WI rapporteur name/company |
| --- | --- | --- | --- | --- | --- |
| 840083 | 5G Enhancement for UAVs | EAV |  | SP-190308 | Qun Wei, China Unicom |
| 810019 | Study on EAV | FS_EAV | S1 | SP-180909 | Qun Wei, China Unicom |
| 840039 | Stage 1 of EAV | EAV | S1 | SP-190308 | Qun Wei, China Unicom |
| 820011 | Study on supporting Uncrewed Aerial Systems Connectivity, Identification, and Tracking | FS_ID_UAS_SA2 | S2 | SP-200097 | Qualcomm, Stefano Faccin |
| 820026 | Study on application layer support for Uncrewed Aerial System (UAS) | FS_UASAPP | S6 | SP-200111 | Niranth Amogh, Huawei |
| 920045 | Application layer support for Uncrewed Aerial System (UAS) | UASAPP |  | SP-200988 | Niranth Amogh, Huawei Telecommunications India |
| 900025 | Stage 2 of Application layer support for Uncrewed Aerial System (UAS) | UASAPP | S6 | SP-200988 | Niranth Amogh, Huawei Telecommunications India |
| 920004 | CT1 Aspects of Application Layer Support for Uncrewed Aerial Systems (UAS) | UASAPP | C1 | CP-211111 | Lin Shu, Huawei, |
| 920046 | CT3 Aspects of Application Layer Support for Uncrewed Aerial Systems (UAS) | UASAPP | C3 | CP-211111 | Lin Shu, Huawei |
| 880007 | Study on security aspects of Uncrewed Aerial Systems (UAS) | FS_UAS_SEC | S3 | SP-200352 | Adrian Escott, Qualcomm |
| 810049 | Remote Identification of Uncrewed Aerial Systems | ID_UAS |  | SP-180771 | Eddy HALL, Qualcomm |
| 810013 | Stage 1 of Remote Identification of Uncrewed Aerial Systems | ID_UAS | S1 | SP-180771 | Eddy HALL, Qualcomm |
| 900014 | (Stage 2 of) Support of Uncrewed Aerial Systems Connectivity, Identification, and Tracking | ID_UAS | S2 | SP-200979 | Faccin, Stefano, Qualcomm |
| 910017 | CT aspects for ID_UAS | ID_UAS | ct | CP-211112 | Sunghoon Kim, Qualcomm |
| 910069 | CT1 aspects for ID_UAS | ID_UAS | C1 | CP-211112 | Sunghoon Kim, Qualcomm |
| 910070 | CT3 aspects for ID_UAS | ID_UAS | C3 | CP-211112 | Sunghoon Kim, Qualcomm |
| 910071 | CT4 aspects for ID_UAS | ID_UAS | C4 | CP-211112 | Sunghoon Kim, Qualcomm |
| 920028 | Security aspects of Uncrewed Aerial Systems | ID_UAS | S3 | SP-210425 | Escott, Adrian, Qualcomm |

Summary based on the input provided by XXX in xP-22xxxx.

# 13	Authentication and key management for applications based on 3GPP credential in 5G

| UID | Name | Acronym | WG | WID | WI rapporteur name/company |
| --- | --- | --- | --- | --- | --- |
| 890030 | Authentication and key management for applications based on 3GPP credential in 5G | AKMA |  | SP-190711 | Xiaoting Huang, China Mobile |
| 850021 | SA3 aspects of AKMA | AKMA | S3 | SP-190711 | Xiaoting Huang, China Mobile |
| 890008 | CT aspects of AKMA | AKMA-CT | ct | CP-203107 | Huang Zhenning (China Mobile) |
| 890031 | CT1 aspects of AKMA | AKMA-CT | C1 | CP-203107 | Huang Zhenning (China Mobile) |
| 890032 | CT3 aspects of AKMA | AKMA-CT | C3 | CP-203107 | Huang Zhenning (China Mobile) |
| 890033 | CT4 aspects of AKMA | AKMA-CT | C4 | CP-203107 | Huang Zhenning (China Mobile) |

Summary based on the input provided by China Mobile in SP-220289.

Authentication and key management for applications based on 3GPP credential in 5G (AKMA) is a cellular-network-based delegated authentication system specified for the 5G system, helping establish a secure tunnel between the end user and the application server. Using AKMA, a user can log in to an application service only based on the 3GPP credential which is the permanent key stored in the user’s tamper-resistant smart card UICC. The application service provider can also delegate the task of user authentication to the mobile network operator by using AKMA.

The AKMA architecture and procedures are specified by SA3 in TS 33.535 [1], with the related study showing how its general principles are derived documented in TR 33.835 [2]. The AKMA feature introduces a new Network Function into the 5G system, which is the AKMA Anchor Function (AAnF). Its detailed services and API definitions are specified by CT3 in TS 29.535[3]. Earlier generations of cellular networks include two similar standards specified by SA3, which are generic bootstrapping architecture (GBA) and battery-efficient security for very low throughput machine type communication devices (BEST). Since the AKMA feature is deemed as a successor of these systems, the work is launched by SA3 without the involvement of stage 1.

References

List of related CRs: select "TSG Status = Approved" in: https://portal.3gpp.org/ChangeRequests.aspx?q=1&workitem=890030,850021,890008,890031,890032,890033

[1]	TS 33.535: “Authentication and Key Management for Applications (AKMA) based on 3GPP credentials in the 5G System (5GS)”

[2]	TR 33.835: “Study on authentication and key management for applications based on 3GPP credential in 5G”

[3]	TS 29.535: “5G System; AKMA Anchor Services; Stage 3”

# 14	Multimedia Priority Service (MPS) Phase 2

| UID | Name | Acronym | WG | WID | WI rapporteur name/company |
| --- | --- | --- | --- | --- | --- |
| 840046 | Multimedia Priority Service (MPS) Phase 2 | MPS2 |  | SP-190305 | Singh, Ray P; Perspecta Labs |
| 780005 | Study on MPS2 | FS_MPS2 | S1 | SP-190315 | Singh, Ray P; Vencore Labs |
| 850011 | Study on MPS2, Stage 2 | FS_MPS2_St2 | S2 | SP-190629 | Don Lukacs; Perspecta Labs |
| 840032 | Stage 1 of MPS2 | MPS2 | S1 | SP-190561 | Singh, Ray P; Perspecta Labs |
| 870002 | Stage 2 of MPS2 | MPS2 | S2 | SP-200519 | Streijl, Robert, Perspecta Labs |
| 880031 | Stage 3 of MPS2 | MPS2 | ct | CP-201207 | Peter Monnes, Perspecta Labs |
| 880050 | CT1 aspects of MPS2 | MPS2 | C1 | CP-201207 | Peter Monnes, Perspecta Labs |
| 880058 | CT3 aspects of MPS2 | MPS2 | C3 | CP-201207 | Peter Monnes, Perspecta Labs |
| 880051 | CT4 aspects of MPS2 | MPS2 | C4 | CP-201207 | Peter Monnes, Perspecta Labs |

Summary based on the input provided by XXX in xP-22xxxx.

# 15	NR enhancements

| UID | Name | Acronym | WG | WID | WI rapporteur name/company |
| --- | --- | --- | --- | --- | --- |
| 920070 | UE RF requirements for Transparent Tx Diversity (TxD) for NR | NR_RF_TxD | R4 | RP-211940 | Qualcomm |
| 920170 | Core part: UE RF requirements for Transparent Tx Diversity (TxD) for NR | NR_RF_TxD-Core | R4 | RP-211940 | Qualcomm |
| 920270 | Perf. part: UE RF requirements for Transparent Tx Diversity (TxD) for NR | NR_RF_TxD-Perf | R4 | RP-211940 | Qualcomm |
| 860040 | Further enhancements on MIMO for NR | NR_feMIMO |  | RP-212535 | Samsung |
| 860140 | Core part: Further enhancements on MIMO for NR | NR_feMIMO-Core | R1 | RP-212535 | Samsung |
| 860240 | Perf. part: Further enhancements on MIMO for NR | NR_feMIMO-Perf | R4 | RP-212535 | Samsung |
| 860043 | NR Dynamic spectrum sharing (DSS) | NR_DSS |  | RP-211345 | Ericsson |
| 860143 | Core part: NR DSS | NR_DSS-Core | R1 | RP-211345 | Ericsson |
| 860047 | UE power saving enhancements for NR | NR_UE_pow_sav_enh |  | RP-212630 | MediaTek |
| 860147 | Core part: UE power saving enhancements for NR | NR_UE_pow_sav_enh-Core | R2 | RP-212630 | MediaTek |
| 860247 | Perf. part: UE power saving enhancements for NR | NR_UE_pow_sav_enh-Perf | R4 | RP-212630 | MediaTek |
| 860049 | Further Multi-RAT Dual-Connectivity enhancements | LTE_NR_DC_enh2 |  | RP-201040 | Huawei |
| 860149 | Core part: Further Multi-RAT Dual-Connectivity enhancements | LTE_NR_DC_enh2-Core | R2 | RP-201040 | Huawei |
| 860249 | Perf. part: Further Multi-RAT Dual-Connectivity enhancements | LTE_NR_DC_enh2-Perf | R4 | RP-201040 | Huawei |
| 860050 | Enhancements to Integrated Access and Backhaul (IAB) for NR | NR_IAB_enh |  | RP-211548 | Qualcomm |
| 860150 | Core part: Enhancements to IAB for NR | NR_IAB_enh-Core | R2 | RP-211548 | Qualcomm |
| 860250 | Perf. part: Enhancements to IAB for NR | NR_IAB_enh-Perf | R4 | RP-211548 | Qualcomm |
| 830021 | Study on Security for NR_IAB | FS_NR_IAB_Sec | S3 | SP-201016 | Rajavelsamy Rajadurai, Samsung, |
| 860052 | Enhanced eNB(s) architecture evolution for E-UTRAN and NG-RAN | LTE_NR_arch_evo_enh |  | RP-211409 | China Unicom |
| 860152 | Core part: LTE_NR_arch_evo_enh | LTE_NR_arch_evo_enh-Core | R3 | RP-211409 | China Unicom |
| 890055 | Further enhancement on NR demodulation performance | NR_demod_enh2 |  | RP-212636 | China Telecom |
| 890255 | Perf. part: NR_demod_enh2 | NR_demod_enh2-Perf | R4 | RP-212636 | China Telecom |
| 890056 | Introduction of DL 1024QAM for NR frequency range 1 (FR1) | NR_DL1024QAM_FR1 |  | RP-202886 | Ericsson |
| 890156 | Core part: Introduction of DL 1024QAM for NR FR1 | NR_DL1024QAM_FR1-Core | R4 | RP-202886 | Ericsson |
| 890256 | Perf. part: Introduction of DL 1024QAM for NR FR1 | NR_DL1024QAM_FR1-Perf | R4 | RP-202886 | Ericsson |
| 890057 | NR RRM further enhancement | NR_RRM_enh2 |  | RP-202874 | Apple |
| 890157 | Core part: Further RRM enhancement for NR and MR-DC | NR_RRM_enh2-Core | R4 | RP-202874 | Apple |
| 890257 | Perf. part: Further RRM enhancement for NR and MR-DC | NR_RRM_enh2-Perf | R4 | RP-202874 | Apple |
| 890058 | Enhanced NR support for high speed train scenario for frequency range 1 (FR1) | NR_HST_FR1_enh |  | RP-210833 | CMCC |
| 890158 | Core part: Enhanced NR support for high speed train scenario for FR1 | NR_HST_FR1_enh-Core | R4 | RP-210833 | CMCC |
| 890258 | Perf. part: Enhanced NR support for high speed train scenario for FR1 | NR_HST_FR1_enh-Perf | R4 | RP-210833 | CMCC |
| 890059 | Further enhancements of NR RF requirements for frequency range 2 (FR2) | NR_RF_FR2_req_enh2 |  | RP-212092 | Nokia |
| 890159 | Core part: NR_RF_FR2_req_enh2 | NR_RF_FR2_req_enh2-Core | R4 | RP-212092 | Nokia |
| 890259 | Perf. part: NR_RF_FR2_req_enh2 | NR_RF_FR2_req_enh2-Perf | R4 | RP-212092 | Nokia |
| 890060 | NR support for high speed train scenario in frequency range 2 (FR2) | NR_HST_FR2 |  | RP-210800 | Samsung |
| 890160 | Core part: NR support for high speed train scenario in FR2 | NR_HST_FR2-Core | R4 | RP-210800 | Samsung |
| 890260 | Perf. Part: NR support for high speed train scenario in FR2 | NR_HST_FR2-Perf | R4 | RP-210800 | Samsung |
| 890061 | NR measurement gap enhancements | NR_MG_enh |  | RP-211591 | MediaTek |
| 890161 | Core part: NR and MR-DC measurement gap enhancements | NR_MG_enh-Core | R4 | RP-211591 | MediaTek |
| 890261 | Perf. part: NR and MR-DC measurement gap enhancements | NR_MG_enh-Perf | R4 | RP-211591 | MediaTek |
| 890062 | RF requirements enhancement for NR frequency range 1 (FR1) | NR_RF_FR1_enh |  | RP-212527 | Huawei |
| 890162 | Core part: RF requirements enhancement for NR FR1 | NR_RF_FR1_enh-Core | R4 | RP-212527 | Huawei |
| 890262 | Perf. part: RF requirements enhancement for NR FR1 | NR_RF_FR1_enh-Perf | R4 | RP-212527 | Huawei |

Summary based on the input provided by XXX in xP-22xxxx.

# 16	Other NR-only improvements

| UID | Name | Acronym | WG | WID | WI rapporteur name/company |
| --- | --- | --- | --- | --- | --- |
| 911008 | NR QoE magement and optimizations for diverse services | NR_QoE |  | RP-211406 | China Unicom |
| 911108 | Core part: NR QoE management and optimizations for diverse services | NR_QoE-Core | R3 | RP-211406 | China Unicom |
| 911009 | Rel-17 Power Class 2 UE for NR inter-band CA with or without SUL configurations with x (16>=x>2) bands DL and y (y=1, 2) bands UL | NR_UE_PC2_R17_CADC_SUL_xBDL_yBUL |  | RP-212182 | Huawei |
| 911109 | Core part: NR_UE_PC2_R17_CADC_SUL_xBDL_yBUL | NR_UE_PC2_R17_CADC_SUL_xBDL_yBUL-Core | R4 | RP-212182 | Huawei |
| 911209 | Perf. part: NR_UE_PC2_R17_CADC_SUL_xBDL_yBUL | NR_UE_PC2_R17_CADC_SUL_xBDL_yBUL-Perf | R4 | RP-212182 | Huawei |
| 911010 | Introduction of UE TRP (Total Radiated Power) and TRS (Total Radiated Sensitivity) requirements and test methodologies for FR1 (NR SA and EN-DC) | NR_FR1_TRP_TRS |  | RP-212030 | Vivo |
| 911110 | Core part: NR_FR1_TRP_TRS | NR_FR1_TRP_TRS-Core | R4 | RP-212030 | Vivo |
| 911210 | Perf. part: NR_FR1_TRP_TRS | NR_FR1_TRP_TRS-Perf | R4 | RP-212030 | Vivo |
| 911012 | High power UE (power class 1.5) for NR band n79 | NR_UE_PC1_5_n79 |  | RP-210843 | CMCC |
| 911112 | Core part: NR_UE_PC1_5_n79 | NR_UE_PC1_5_n79-Core | R4 | RP-210843 | CMCC |
| 911212 | Perf. part: NR_UE_PC1_5_n79 | NR_UE_PC1_5_n79-Perf | R4 | RP-210843 | CMCC |
| 930052 | UE Conformance - High power UE (power class 1.5) for NR band n79 | NR_UE_PC1_5_n79-UEConTest | R5 | RP-211979 | CMCC |
| 911013 | High power UE (power class 2) for NR band n34 | NR_UE_PC2_n34 |  | RP-210844 | CMCC |
| 911113 | Core part: NR_UE_PC2_n34 | NR_UE_PC2_n34-Core | R4 | RP-210844 | CMCC |
| 911213 | Perf. part: NR_UE_PC2_n34 | NR_UE_PC2_n34-Perf | R4 | RP-210844 | CMCC |
| 930053 | UE Conformance - High power UE (power class 2) for NR band n34 | NR_UE_PC2_n34-UEConTest | R5 | RP-211980 | CMCC |
| 911014 | High power UE (power class 2) for NR band n39 | NR_UE_PC2_n39 |  | RP-210845 | CMCC |
| 911114 | Core part: NR_UE_PC2_n39 | NR_UE_PC2_n39-Core | R4 | RP-210845 | CMCC |
| 911214 | Perf. part: NR_UE_PC2_n39 | NR_UE_PC2_n39-Perf | R4 | RP-210845 | CMCC |
| 930054 | UE Conformance - High power UE (power class 2) for NR band n39 | NR_UE_PC2_n39-UEConTest | R5 | RP-211981 | CMCC |
| 911015 | Introduction of FR2 FWA (Fixed Wireless Access) UE with maximum TRP of 23dBm for band n259 | NR_FR2_FWA_Bn259 |  | RP-210875 | SoftBank Corp., KDDI, NTT DOCOMO, Rakuten Mobile, Qualcomm |
| 911115 | Core part: NR_FR2_FWA_Bn259 | NR_FR2_FWA_Bn259-Core | R4 | RP-210875 | SoftBank Corp., KDDI, NTT DOCOMO, Rakuten Mobile, Qualcomm |
| 911215 | Perf. part: NR_FR2_FWA_Bn259 | NR_FR2_FWA_Bn259-Perf | R4 | RP-210875 | SoftBank Corp., KDDI, NTT DOCOMO, Rakuten Mobile, Qualcomm |
| 911006 | NR Uplink Data Compression (UDC) | NR_UDC |  | RP-211203 | CATT |
| 911106 | Core part: NR_UDC | NR_UDC-Core | R2 | RP-211203 | CATT |
| 910097 | Study on optimizations of pi/2 BPSK uplink power in NR | FS_NR_Opt_pi2BPSK | R4 | RP-210910 | Huawei |
| 910096 | Study on band combination handling in RAN4 | FS_NR_ENDC_combo_rules | R4 | RP-211146 | ZTE |
| 880078 | Multiple Input Multiple Output (MIMO) Over-the-Air (OTA) requirements for NR UEs | NR_MIMO_OTA |  | RP-201800 | CAICT |
| 880178 | Core part: NR_MIMO_OTA | NR_MIMO_OTA-Core | R4 | RP-201800 | CAICT |
| 880278 | Perf. Part: NR_MIMO_OTA | NR_MIMO_OTA-Perf | R4 | RP-201800 | CAICT |
| 870064 | Introduction of FR2 FWA (Fixed Wireless Access) UE with maximum TRP (Total Radiated Power) of 23dBm for band n257 and n258 | NR_FR2_FWA_Bn257_Bn258 |  | RP-202565 | Softbank |
| 870164 | Core part: NR_FR2_FWA_Bn257_Bn258 | NR_FR2_FWA_Bn257_Bn258-Core | R4 | RP-202565 | Softbank |
| 870264 | Perf. Part: NR_FR2_FWA_Bn257_Bn258 | NR_FR2_FWA_Bn257_Bn258-Perf | R4 | RP-202565 | Softbank |
| 880092 | Adding channel bandwidth support to existing NR bands | NR_bands_R17_BWs |  | RP-212531 | Ericsson |
| 880192 | Core part: NR_bands_R17_BWs | NR_bands_R17_BWs-Core | R4 | RP-212531 | Ericsson |
| 880093 | Introduction of channel bandwidths 35MHz and 45MHz for NR FR1 | NR_FR1_35MHz_45MHz_BW |  | RP-211386 | Huawei |
| 880193 | Core part: NR_FR1_35MHz_45MHz_BW | NR_FR1_35MHz_45MHz_BW-Core | R4 | RP-211386 | Huawei |
| 880293 | Perf. Part: NR_FR1_35MHz_45MHz_BW | NR_FR1_35MHz_45MHz_BW-Perf | R4 | RP-211386 | Huawei |
| 880097 | SAR schemes for UE power class 2 (PC2) for NR inter-band Carrier Aggregation and supplemental uplink (SUL) configurations with 2 bands UL | NR_SAR_PC2_interB_SUL_2BUL |  | RP-212530 | China Telecom |
| 880197 | Core part: NR_SAR_PC2_interB_SUL_2BUL | NR_SAR_PC2_interB_SUL_2BUL-Core | R4 | RP-212530 | China Telecom |
| 880297 | Perf. Part: NR_SAR_PC2_interB_SUL_2BUL | NR_SAR_PC2_interB_SUL_2BUL-Perf | R4 | RP-212530 | China Telecom |
| 920065 | UE Conformance - SAR schemes for UE power class 2 (PC2) for NR inter-band Carrier Aggregation and supplemental uplink (SUL) configurations with 2 bands UL | NR_SAR_PC2_interB_SUL_2BUL-UEConTest | R5 | RP-211139 | China Telecom |
| 900061 | NR coverage enhancements | NR_cov_enh |  | RP-211566 | China Telecom |
| 860036 | Study on on NR coverage enhancements | FS_NR_cov_enh | R1 | RP-200861 | China Telecom |
| 900161 | Core part: NR_cov_enh | NR_cov_enh-Core | R1 | RP-211566 | China Telecom |
| 900261 | Perf. part: NR_cov_enh | NR_cov_enh-Perf | R4 | RP-211566 | China Telecom |
| 900062 | Support of reduced capability NR devices | NR_redcap |  | RP-211574 | Ericsson |
| 860035 | Study on support of reduced capability NR devices | FS_NR_redcap | R1 | RP-202704 | Ericsson |
| 900162 | Core part: NR_redcap | NR_redcap-Core | R1 | RP-211574 | Ericsson |
| 900262 | Perf. part: NR_redcap | NR_redcap-Perf | R4 | RP-211574 | Ericsson |
| 930018 | Architecture Enhancement for NR Reduced Capability Devices | ARCH_NR_REDCAP | S2 | SP-211100 | Aihua Li, China Mobile |
| 900064 | Additional NR bands for UL-MIMO power class 3 (PC3) in Rel-17 | NR_bands_UL_MIMO_PC3_R17 |  | RP-212184 | Huawei |
| 900164 | Core part: NR_bands_UL_MIMO_PC3_R17 | NR_bands_UL_MIMO_PC3_R17-Core | R4 | RP-212184 | Huawei |
| 900264 | Perf. part: NR_bands_UL_MIMO_PC3_R17 | NR_bands_UL_MIMO_PC3_R17-Perf | R4 | RP-212184 | Huawei |
| 900167 | Introduction of bandwidth combination set 4 (BCS4) for NR | NR_BCS4-Core | R4 | RP-202832 | Ericsson |
| 900068 | Downlink interruption for NR and EN-DC band combinations to conduct dynamic Tx Switching in Uplink | DL_intrpt_combos_TxSW_R17 |  | RP-210478 | China Telecom |
| 900168 | Core part: DL_intrpt_combos_TxSW_R17 | DL_intrpt_combos_TxSW_R17-Core | R4 | RP-202885RP-210478 | China Telecom |
| 900268 | Perf. part: DL_intrpt_combos_TxSW_R17 | DL_intrpt_combos_TxSW_R17-Perf | R4 | RP-202885RP-210478 | China Telecom |
| 900069 | High-power UE (power class 1.5) operation in NR bands n77 and n78 | HPUE_PC1_5_n77_n78 |  | RP-202912 | Qualcomm |
| 900169 | Core part: HPUE_PC1_5_n77_n78 | HPUE_PC1_5_n77_n78-Core | R4 | RP-202912 | Qualcomm |
| 930055 | UE Conformance - High-power UE (power class 1.5) operation in NR bands n77 and n78 | HPUE_PC1_5_n77_n78-UEConTest | R5 | RP-212581 | Verizon |
| 900070 | NR repeaters | NR_repeaters |  | RP-212129 | Qualcomm |
| 900170 | Core part: HPUE_PC1_5_n77_n78 | NR_repeaters-Core | R4 | RP-212129 | Qualcomm |
| 930250 | Perf. part: NR repeaters | NR_repeaters-Perf | R4 | RP-212129 | Qualcomm |
| 930056 | Increasing UE power high limit for CA and DC | Power_Limit_CA_DC | R4 | RP-212622 | Qualcomm |
| 930156 | Core part: Increasing UE power high limit for CA and DC | Power_Limit_CA_DC-Core | R4 | RP-212622 | Qualcomm |
| 930057 | Introduction of upper 700MHz A block E-UTRA band for the US | LTE_upper_700MHz_A | R4 | RP-212618 | Puloli |
| 930157 | Core part: Introduction of upper 700MHz A block E-UTRA band for the US | LTE_upper_700MHz_A-Core | R4 | RP-212618 | Puloli |
| 930257 | Perf. part: Introduction of upper 700MHz A block E-UTRA band for the US | LTE_upper_700MHz_A-Perf | R4 | RP-212618 | Puloli |
| 930058 | High power UE (power class 2) for one NR FDD band | NR_PC2_UE_FDD | R4 | RP-212633 | China Unicom |
| 930158 | Core part: High power UE (power class 2) for one NR FDD band | NR_PC2_UE_FDD-Core | R4 | RP-212633 | China Unicom |
| 930258 | Perf. part: High power UE (power class 2) for one NR FDD band | NR_PC2_UE_FDD-Perf | R4 | RP-212633 | China Unicom |

Summary based on the input provided by XXX in xP-22xxxx.

# 17	NR Carrier Aggregation

## 17.1	NR intra band Carrier Aggregation (itrabCA)

| UID | Name | Acronym | WG | WID | WI rapporteur name/company |
| --- | --- | --- | --- | --- | --- |
| 881005 | NR itrabCA for xCC DL/yCC UL including contiguous and non-contiguous spectrum (x>=y) | NR_CA_R17_Intra |  | RP-211757 | Ericsson |
| 881105 | Core part: NR_CA_R17_Intra | NR_CA_R17_Intra-Core | R4 | RP-211757 | Ericsson |
| 881205 | Perf. Part: NR_CA_R17_Intra | NR_CA_R17_Intra-Perf | R4 | RP-211757 | Ericsson |

Summary based on the input provided by XXX in xP-22xxxx.

## 17.2	NR inter band Carrier Aggregation (iterbCA)

| UID | Name | Acronym | WG | WID | WI rapporteur name/company |
| --- | --- | --- | --- | --- | --- |
| 900063 | High power UE for NR TDD intra-band Carrier Aggregation in frequency range FR1 | NR_intra_HPUE_R17 |  | RP-212180 | Huawei |
| 900163 | Core part: NR_intra_HPUE_R17 | NR_intra_HPUE_R17-Core | R4 | RP-212180 | Huawei |
| 900263 | Perf. Part: NR_intra_HPUE_R17 | NR_intra_HPUE_R17-Perf | R4 | RP-212180 | Huawei |
| 881006 | NR iterbCA/Dual Connectivity for 2 bands DL with x bands UL (x=1,2) | NR_CADC_R17_2BDL_xBUL |  | RP-212511 | ZTE |
| 881106 | Core part: NR_CADC_R17_2BDL_xBUL | NR_CADC_R17_2BDL_xBUL-Core | R4 | RP-212511 | ZTE |
| 881206 | Perf. part: NR_CADC_R17_2BDL_xBUL | NR_CADC_R17_2BDL_xBUL-Perf | R4 | RP-212511 | ZTE |
| 881007 | NR iterbCA for 3 bands DL with 1 band UL | NR_CA_R17_3BDL_1BUL |  | RP-212239 | CATT |
| 881107 | Core part: NR_CA_R17_3BDL_1BUL | NR_CA_R17_3BDL_1BUL-Core | R4 | RP-212239 | CATT |
| 881207 | Perf. part: NR_CA_R17_3BDL_1BUL | NR_CA_R17_3BDL_1BUL-Perf | R4 | RP-212239 | CATT |
| 881008 | NR iterbCA/Dual Connectivity for 3 bands DL with 2 bands UL | NR_CADC_R17_3BDL_2BUL |  | RP-212512 | ZTE |
| 881108 | Core part: NR_CADC_R17_3BDL_2BUL | NR_CADC_R17_3BDL_2BUL-Core | R4 | RP-212512 | ZTE |
| 881208 | Perf. part: NR_CADC_R17_3BDL_2BUL | NR_CADC_R17_3BDL_2BUL-Perf | R4 | RP-212512 | ZTE |
| 881009 | NR iterbCA for 4 bands DL with 1 band UL | NR_CA_R17_4BDL_1BUL |  | RP-211759 | Ericsson |
| 881109 | Core part: NR_CA_R17_4BDL_1BUL | NR_CA_R17_4BDL_1BUL-Core | R4 | RP-211759 | Ericsson |
| 881209 | Perf. part: NR_CA_R17_4BDL_1BUL | NR_CA_R17_4BDL_1BUL-Perf | R4 | RP-211759 | Ericsson |
| 881010 | NR iterbCA/Dual connectivity for DL 4 bands and 2UL bands | NR_CADC_R17_4BDL_2BUL |  | RP-212110 | Samsung |
| 881110 | Core part: NR_CADC_R17_4BDL_2BUL | NR_CADC_R17_4BDL_2BUL-Core | R4 | RP-212110 | Samsung |
| 881210 | Perf. part: NR_CADC_R17_4BDL_2BUL | NR_CADC_R17_4BDL_2BUL-Perf | R4 | RP-212110 | Samsung |
| 881011 | NR iterbCA for 5 bands DL with x bands UL (x=1, 2) | NR_CADC_R17_5BDL_xBUL |  | RP-212176 | Huawei |
| 881111 | Core part: NR_CADC_R17_5BDL_xBUL | NR_CADC_R17_5BDL_xBUL-Core | R4 | RP-212176 | Huawei |
| 881211 | Perf. part: NR_CADC_R17_5BDL_xBUL | NR_CADC_R17_5BDL_xBUL-Perf | R4 | RP-212176 | Huawei |
| 890054 | Rel-17 High power UE for NR inter-band Carrier Aggregation with 2 bands downlink and x bands uplink (x=1,2) | NR_PC2_CA_R17_2BDL_2BUL |  | RP-211831 | China Telecom |
| 890154 | Core part: NR_PC2_CA_R17_2BDL_2BUL | NR_PC2_CA_R17_2BDL_2BUL-Core | R4 | RP-211831 | China Telecom |
| 890254 | Perf. part: NR_PC2_CA_R17_2BDL_2BUL | NR_PC2_CA_R17_2BDL_2BUL-Perf | R4 | RP-211831 | China Telecom |
| 920066 | UE Conformance - Rel-17 High power UE for NR inter-band Carrier Aggregation with 2 bands downlink and x bands uplink (x=1,2) | NR_PC2_CA_R17_2BDL_2BUL-UEConTest | R5 | RP-211140 | China Telecom |

Summary based on the input provided by XXX in xP-22xxxx.

# 18	NR and LTE

| UID | Name | Acronym | WG | WID | WI rapporteur name/company |
| --- | --- | --- | --- | --- | --- |
| 920072 | LTE/NR spectrum sharing in Band 34/n34 and Band 39/n39 | DSS_LTE_B34_NR_Bn34_LTE_B39_NR_Bn39 | R4 |  | CMCC |
| 920172 | Core part: LTE/NR spectrum sharing in Band 34/n34 and Band 39/n39 | DSS_LTE_B34_NR_Bn34_LTE_B39_NR_Bn39-Core | R4 | RP-211996 | CMCC |
| 920073 | High-power UE operation for fixed-wireless/vehicle-mounted use cases in LTE bands and NR bands | LTE_NR_HPUE_FWVM | R4 | RP-212533 | Nokia |
| 920173 | Core part: High-power UE operation for fixed-wireless/vehicle-mounted use cases in LTE bands and NR bands | LTE_NR_HPUE_FWVM-Core | R4 | RP-212533 | Nokia |
| 920273 | Perf. part: High-power UE operation for fixed-wireless/vehicle-mounted use cases in LTE bands and NR bands | LTE_NR_HPUE_FWVM-Perf | R4 | RP-212533 | Nokia |
| 920074 | Rel-17 Dual Connectivity (DC) of x bands (x=1,2) LTE inter-band CA (xDL1UL) and 4 bands NR inter-band CA (4DL1UL) | DC_R17_xBLTE_4BNR_yDL2UL | R4 | RP-212606 | Huawei |
| 920174 | Core part: Rel-17 Dual Connectivity (DC) of x bands (x=1,2) LTE inter-band CA (xDL1UL) and 4 bands NR inter-band CA (4DL1UL) | DC_R17_xBLTE_4BNR_yDL2UL-Core | R4 | RP-212606 | Huawei |
| 920274 | Perf. part: Rel-17 Dual Connectivity (DC) of x bands (x=1,2) LTE inter-band CA (xDL1UL) and 4 bands NR inter-band CA (4DL1UL) | DC_R17_xBLTE_4BNR_yDL2UL-Perf | R4 | RP-212606 | Huawei |
| 911019 | Addition of MSD (Maximum Sensitivity Degradation) for inter-band EN-DC combinations (1 band LTE+1 band NR FR1) due to added channel bandwidths | ENDC_R17_1BLTE_1BNR_MS |  | RP-212187 | Huawei, HiSilicon |
| 911119 | Core part: ENDC_R17_1BLTE_1BNR_MS | ENDC_R17_1BLTE_1BNR_MSD-Core |  | RP-212187 | Huawei, HiSilicon |
| 911011 | Rel-17 Power Class 2 for EN-DC with x LTE bands + y NR band(s) in DL and with 1 LTE band +1 TDD NR band in UL (either x= 2, 3, y=1 or x=1, 2, y=2) | ENDC_PC2_R17_xLTE_yNR |  | RP-211760 | Ericsson |
| 911111 | Core part: ENDC_PC2_R17_xLTE_yNR | ENDC_PC2_R17_xLTE_yNR-Core | R4 | RP-211760 | Ericsson |
| 911211 | Perf. Part: ENDC_PC2_R17_xLTE_yNR | ENDC_PC2_R17_xLTE_yNR-Perf | R4 | RP-211760 | Ericsson |
| 930051 | UE Conformance - Power Class 2 for EN-DC with x LTE bands + y NR band(s) in DL and with 1 LTE band +1 TDD NR band in UL (either x= 2, 3, y=1 or x=1, 2, y=2) | ENDC_PC2_R17_xLTE_yNR-UEConTest | R5 | RP-211870 | Ericsson |
| 911018 | Simultaneous Rx/Tx band combinations for NR CA/DC, NR SUL and LTE/NR DC | LTE_NR_Simult_RxTx |  | RP-211382 | Huawei |
| 911118 | Core part: LTE_NR_Simult_RxTx | LTE_NR_Simult_RxTx-Core | R4 | RP-211382 | Huawei |
| 911218 | Perf. part: LTE_NR_Simult_RxTx | LTE_NR_Simult_RxTx-Perf | R4 | RP-211382 | Huawei |
| 890052 | Rel-17 Dual Connectivity (DC) of 5 bands LTE inter-band CA (5DL/1UL) and 1 NR band (1DL/1UL) | DC_R17_5BLTE_1BNR_6DL2UL |  | RP-211195 | Samsung |
| 890152 | Core part: DC_R17_5BLTE_1BNR_6DL2UL | DC_R17_5BLTE_1BNR_6DL2UL-Core | R4 | RP-211195 | Samsung |
| 890252 | Perf. part: DC_R17_5BLTE_1BNR_6DL2UL | DC_R17_5BLTE_1BNR_6DL2UL-Perf | R4 | RP-211195 | Samsung |
| 890053 | Rel-17 Dual Connectivity (DC) of x bands (x=2, 3, 4) LTE inter-band CA (xDL/1UL) and 1 NR FR1 band (1DL/1UL) and 1 NR FR2 band (1DL/1UL) | DC_R17_xBLTE_2BNR_yDL3UL |  | RP-202542 | Samsung |
| 890153 | Core part: DC_R17_xBLTE_2BNR_yDL3UL | DC_R17_xBLTE_2BNR_yDL3UL-Core | R4 | RP-202542 | Samsung |
| 890253 | Perf. part: DC_R17_xBLTE_2BNR_yDL3UL | DC_R17_xBLTE_2BNR_yDL3UL-Perf | R4 | RP-202542 | Samsung |
| 880080 | Band combinations for concurrent operation of NR/LTE Uu bands/band combinations and one NR/LTE V2X PC5 band | NR_LTE_V2X_PC5_combos |  | RP-210588 | CATT |
| 880180 | Core part: NR_LTE_V2X_PC5_combos | NR_LTE_V2X_PC5_combos-Core | R4 | RP-210588 | CATT |
| 880280 | Perf. part: NR_LTE_V2X_PC5_combos | NR_LTE_V2X_PC5_combos-Perf | R4 | RP-210588 | CATT |
| 880094 | High power UE (power class 2) for EN-DC with 1 LTE band + 1 NR TDD band | ENDC_UE_PC2_R17_NR_TDD |  | RP-211853 | China Unicom |
| 880194 | Core part: ENDC_UE_PC2_R17_NR_TDD | ENDC_UE_PC2_R17_NR_TDD-Core | R4 | RP-211853 | China Unicom |
| 880294 | Perf. part: ENDC_UE_PC2_R17_NR_TDD | ENDC_UE_PC2_R17_NR_TDD-Perf | R4 | RP-211853 | China Unicom |
| 911000 | UE Conformance - ENDC_UE_PC2_R17_NR_TDD | ENDC_UE_PC2_R17_NR_TDD-UEConTest | R5 | RP-211420 | China Unicom |
| 880095 | LTE/NR spectrum sharing in Band 38/n38 | DSS_LTE_B38_NR_Bn38 |  | RP-201314 | Vodafone |
| 880195 | Core part: DSS_LTE_B38_NR_Bn38 | DSS_LTE_B38_NR_Bn38-Core | R4 | RP-201314 | Vodafone |
| 880096 | LTE/NR spectrum sharing in Band 40/n40 | DSS_LTE_B40_NR_Bn40 |  | RP-202084 | Reliance Jio |
| 880196 | Core part: DSS_LTE_B40_NR_Bn40 | DSS_LTE_B40_NR_Bn40-Core | R4 | RP-202084 | Reliance Jio |
| 880296 | Perf. part: DSS_LTE_B40_NR_Bn40 | DSS_LTE_B40_NR_Bn40-Perf | R4 | RP-202084 | Reliance Jio |
| 880098 | Dual Connectivity (DC) of 1 LTE band (1DL/1UL) and 1 NR band (1DL/1UL) | DC_R17_1BLTE_1BNR_2DL2UL |  | RP-212096 | CHTTL |
| 880198 | Core part: DC_R17_1BLTE_1BNR_2DL2UL | DC_R17_1BLTE_1BNR_2DL2UL-Core | R4 | RP-212096 | CHTTL |
| 880298 | Perf. part: DC_R17_1BLTE_1BNR_2DL2UL | DC_R17_1BLTE_1BNR_2DL2UL-Perf | R4 | RP-212096 | CHTTL |
| 880099 | Dual Connectivity (DC) of 2 bands LTE inter-band CA (2DL/1UL) and 1 NR band (1DL/1UL) | DC_R17_2BLTE_1BNR_3DL2UL |  | RP-212174 | Huawei |
| 880199 | Core part: DC_R17_2BLTE_1BNR_3DL2UL | DC_R17_2BLTE_1BNR_3DL2UL-Core | R4 | RP-212174 | Huawei |
| 890263 | Perf. part: DC_R17_2BLTE_1BNR_3DL2UL | DC_R17_2BLTE_1BNR_3DL2UL-Perf | R4 | RP-212174 | Huawei |
| 881000 | Dual Connectivity (DC) of 3 bands LTE inter-band CA (3DL/1UL) and 1 NR band (1DL/1UL) | DC_R17_3BLTE_1BNR_4DL2UL |  | RP-211758 | Ericsson |
| 881100 | Core part: DC_R17_3BLTE_1BNR_4DL2UL | DC_R17_3BLTE_1BNR_4DL2UL-Core | R4 | RP-211758 | Ericsson |
| 881200 | Perf. part: DC_R17_3BLTE_1BNR_4DL2UL | DC_R17_3BLTE_1BNR_4DL2UL-Perf | R4 | RP-211758 | Ericsson |
| 881001 | Dual Connectivity (DC) of 4 bands LTE inter-band CA (4DL/1UL) and 1 NR band (1DL/1UL) | DC_R17_4BLTE_1BNR_5DL2UL |  | RP-212532 | Nokia |
| 881101 | Core part: DC_R17_4BLTE_1BNR_5DL2UL | DC_R17_4BLTE_1BNR_5DL2UL-Core | R4 | RP-212532 | Nokia |
| 881201 | Perf. part: DC_R17_4BLTE_1BNR_5DL2UL | DC_R17_4BLTE_1BNR_5DL2UL-Perf | R4 | RP-212532 | Nokia |
| 881002 | Dual Connectivity (DC) of x bands (x=1,2,3,4) LTE inter-band CA (xDL/1UL) and 2 bands NR inter-band CA (2DL/1UL) | DC_R17_xBLTE_2BNR_yDL2UL |  | RP-211750 | LG Electronics |
| 881102 | Core part: DC_R17_xBLTE_2BNR_yDL2UL | DC_R17_xBLTE_2BNR_yDL2UL-Core | R4 | RP-211750 | LG Electronics |
| 881202 | Perf. part: DC_R17_xBLTE_2BNR_yDL2UL | DC_R17_xBLTE_2BNR_yDL2UL-Perf | R4 | RP-211750 | LG Electronics |
| 881003 | Dual Connectivity (DC) of x bands (x=1,2) LTE inter-band CA (xDL/xUL) and y bands (y=3-x) NR inter-band CA (yDL/yUL) | DC_R17_xBLTE_yBNR_3DL3UL |  | RP-211803 | ZTE |
| 881103 | Core part: DC_R17_xBLTE_yBNR_3DL3UL | DC_R17_xBLTE_yBNR_3DL3UL-Core | R4 | RP-211803 | ZTE |
| 881203 | Perf. part: DC_R17_xBLTE_yBNR_3DL3UL | DC_R17_xBLTE_yBNR_3DL3UL-Perf | R4 | RP-211803 | ZTE |
| 881004 | Dual Connectivity (DC) of x bands (x=1,2,3) LTE inter-band CA (xDL/1UL) and 3 bands NR inter-band CA (3DL/1UL) | DC_R17_xBLTE_3BNR_yDL2UL |  | RP-211805 | ZTE |
| 881104 | Core part: DC_R17_xBLTE_3BNR_yDL2UL | DC_R17_xBLTE_3BNR_yDL2UL-Core | R4 | RP-211805 | ZTE |
| 881204 | Perf. part: DC_R17_xBLTE_3BNR_yDL2UL | DC_R17_xBLTE_3BNR_yDL2UL-Perf | R4 | RP-211805 | ZTE |
| 881012 | band combinations for SA NR supplementary uplink (SUL), NSA NR SUL, NSA NR SUL with UL sharing from the UE perspective (ULSUP) | NR_SUL_combos_R17 |  | RP-212178 | Huawei |
| 881112 | Core part: NR_SUL_combos_R17 | NR_SUL_combos_R17-Core | R4 | RP-212178 | Huawei |
| 881212 | Perf. part: NR_SUL_combos_R17 | NR_SUL_combos_R17-Perf | R4 | RP-212178 | Huawei |
| 900056 | UE Conformance - Rel-17 NR CA and DC; and NR and LTE DC Configurations | NR_CADC_NR_LTE_DC_R17-UEConTest | R5 | RP-211357 | Huawei |

Summary based on the input provided by XXX in xP-22xxxx.

# 19	NR new/modified bands

| UID | Name | Acronym | WG | WID | WI rapporteur name/company |
| --- | --- | --- | --- | --- | --- |
| 911016 | Introduction of 900MHz NR band for Europe for Rail Mobile Radio (RMR) | NR_RAIL_EU_900MHz |  | RP-211495 | UIC |
| 911116 | Core part: NR_RAIL_EU_900MHz | NR_RAIL_EU_900MHz-Core | R4 | RP-211495 | UIC |
| 911216 | Perf. part: NR_RAIL_EU_900MHz | NR_RAIL_EU_900MHz-Perf | R4 | RP-211495 | UIC |
| 911017 | Introduction of 1900MHz NR TDD band for Europe for Rail Mobile Radio (RMR) | NR_RAIL_EU_1900MHz_TDD |  | RP-211542 | UIC |
| 911117 | Core part: NR_RAIL_EU_1900MHz_TDD | NR_RAIL_EU_1900MHz_TDD-Core | R4 | RP-211542 | UIC |
| 911217 | Perf. part: NR_RAIL_EU_1900MHz_TDD | NR_RAIL_EU_1900MHz_TDD-Perf | R4 | RP-211542 | UIC |
| 860041 | Extending current NR operation to 71GHz | NR_ext_to_71GHz | R1 | RP-212637 | Qualcomm |
| 860141 | Core part: Extending current NR operation to 71GHz | NR_ext_to_71GHz-Core | R1 | RP-212637 | Qualcomm |
| 860241 | Perf. part: Extending current NR operation to 71GHz | NR_ext_to_71GHz-Perf | R4 | RP-212637 | Qualcomm |
| 860037 | Study on supporting NR from 52.6 GHz to 71 GHz | FS_NR_52_to_71GHz | R1 | RP-201838 | Intel |
| 870060 | Study on IMT parameters for 6.425-7.025GHz, 7.025-7.125GHz and 10.0-10.5GHz | FS_NR_IMT_param | R4 | RP-202276 | Ericsson |
| 870063 | Introduction of NR band n13 | NR_n13 | R4 | RP-200480 | Huawei |
| 870163 | Core part: Introduction of NR band n13 | NR_n13-Core | R4 | RP-200480 | Huawei |
| 870263 | Perf. part: Introduction of NR band n13 | NR_n13-Perf | R4 | RP-200480 | Huawei |
| 880082 | Introduction of NR 47GHz band | NR_47GHz_band | R4 | RP-212528 | Ericsson |
| 880182 | Core part: Introduction of NR 47GHz band | NR_47GHz_band-Core | R4 | RP-212528 | Ericsson |
| 880282 | Perf. part: Introduction of NR 47GHz band | NR_47GHz_band-Perf | R4 | RP-212528 | Ericsson |
| 880084 | Introduction of NR band 24 | NR_band_n24 | R4 | RP-211505 | Ligado Networks |
| 880184 | Core part: Introduction of NR band 24 | NR_band_n24-Core | R4 | RP-211505 | Ligado Networks |
| 880284 | Perf. part: Introduction of NR band 24 | NR_band_n24-Perf | R4 | RP-211505 | Ligado Networks |
| 890050 | Introduction of 6GHz NR licensed bands | NR_6GHz | R4 | RP-202844 | Huawei |
| 890150 | Core part: Introduction of 6GHz NR licensed bands | NR_6GHz-Core | R4 | RP-202844 | Huawei |
| 890250 | Perf. part: Introduction of 6GHz NR licensed bands | NR_6GHz-Perf | R4 | RP-202844 | Huawei |
| 920075 | Introduction of operation in full unlicensed band 5925-7125MHz for NR | NR_6GHz_unlic_full | R4 | RP-212302 | Apple |
| 920175 | Core part: Introduction of operation in full unlicensed band 5925-7125MHz for NR | NR_6GHz_unlic_full-Core | R4 | RP-212302 | Apple |
| 920275 | Perf. part: Introduction of operation in full unlicensed band 5925-7125MHz for NR | NR_6GHz_unlic_full-Perf | R4 | RP-212302 | Apple |
| 890051 | Introduction of lower 6GHz NR unlicensed operation for Europe | NR_6GHz_unlic_EU | R4 | RP-212625 | Nokia |
| 890151 | Core part: Introduction of lower 6GHz NR unlicensed operation for Europe | NR_6GHz_unlic_EU-Core | R4 | RP-212625 | Nokia |
| 890251 | Perf. part: Introduction of lower 6GHz NR unlicensed operation for Europe | NR_6GHz_unlic_EU-Perf | R4 | RP-212625 | Nokia |
| 900065 | Introduction of NR band n67 | NR_n67 | R4 | RP-202829 | Ericsson |
| 900165 | Core part: Introduction of NR band n67 | NR_n67-Core | R4 | RP-202829 | Ericsson |
| 900265 | Perf. Part: Introduction of NR band n67 | NR_n67-Perf | R4 | RP-202829 | Ericsson |
| 900066 | Introduction of NR band n85 | NR_n85 | R4 | RP-210707 | Ericsson |
| 900166 | Core part: Introduction of NR band n85 | NR_n85-Core | R4 | RP-210707 | Ericsson |
| 900266 | Perf. part: Introduction of NR band n85 | NR_n85-Perf | R4 | RP-210707 | Ericsson |
| 880083 | Introduction of 1.6 GHz NR supplemental uplink (SUL) band with same uplink frequency range of Band 24 | NR_SUL_UL_n24 |  | RP-210341 | Ligado Networks |
| 880183 | Core part: NR_SUL_UL_n24 | NR_SUL_UL_n24-Core | R4 | RP-210341 | Ligado Networks |
| 880283 | Perf. part: NR_SUL_UL_n24 | NR_SUL_UL_n24-Perf | R4 | RP-210341 | Ligado Networks |
| 880085 | Introduction of NR supplemental uplink (SUL) band 1880-1920MHz | NR_SUL_band_1880_1920MHz |  | RP-201363 | CMCC |
| 880185 | Core part: NR_SUL_band_1880_1920MHz | NR_SUL_band_1880_1920MHz-Core | R4 | RP-201363 | CMCC |
| 880285 | Perf. part: NR_SUL_band_1880_1920MHz | NR_SUL_band_1880_1920MHz-Perf | R4 | RP-201363 | CMCC |
| 880086 | Introduction of NR supplemental uplink (SUL) band 2300-2400MHz | NR_SUL_band_2300_2400MHz |  | RP-201364 | CMCC |
| 880186 | Core part: NR_SUL_band_2300_2400MHz | NR_SUL_band_2300_2400MHz-Core | R4 | RP-201364 | CMCC |
| 880286 | Perf. part: NR_SUL_band_2300_2400MHz | NR_SUL_band_2300_2400MHz-Perf | R4 | RP-201364 | CMCC |
| 900055 | UE Conformance - New Rel-17 NR licensed bands and extension of existing NR bands | NR_lic_bands_BW_R17-UEConTest | R5 | RP-202567 | Huawei |

Summary based on the input provided by XXX in xP-22xxxx.

# 20	LTE new/modified bands

| UID | Name | Acronym | WG | WID | WI rapporteur name/company |
| --- | --- | --- | --- | --- | --- |
| 911020 | New bands and bandwidth allocation for 5G terrestrial broadcast - part 1 | LTE_terr_bcast_bands_part1 | R1 | RP-211144 | EBU |
| 911120 | New bands and bandwidth allocation for 5G terrestrial broadcast - part 1 | LTE_terr_bcast_bands_part1 | R1 | RP-211144 | EBU |
| 880079 | Additional LTE bands for UE categories M1/M2/NB1/NB2 | LTE_bands_R17_M1_M2_NB1_NB2 |  | RP-211857 | Ericsson |
| 880179 | Core part: LTE_bands_R17_M1_M2_NB1_NB2 | LTE_bands_R17_M1_M2_NB1_NB2-Core | R4 | RP-211857 | Ericsson |
| 880279 | Perf. part: LTE_bands_R17_M1_M2_NB1_NB2 | LTE_bands_R17_M1_M2_NB1_NB2-Perf | R4 | RP-211857 | Ericsson |
| 880081 | Modification of LTE Band 24 Specifications to comply with updated regulatory emission limits | LTE_B24_mod |  | RP-211507 | Ligado Networks |
| 880181 | Core part: LTE_B24_mod | LTE_B24_mod-Core | R4 | RP-211507 | Ligado Networks |
| 880281 | Perf. part: LTE_B24_mod | LTE_B24_mod-Perf | R4 | RP-211507 | Ligado Networks |
| 920067 | UE Conformance - Modification of LTE Band 24 Specifications to comply with updated regulatory emission limits | LTE_B24_mod-UEConTest | R5 | RP-212497 | Ligado Networks |

Summary based on the input provided by XXX in xP-22xxxx.

# 21	LTE inter-band Carrier Aggregation (LTE iterbCA)

| UID | Name | Acronym | WG | WID | WI rapporteur name/company |
| --- | --- | --- | --- | --- | --- |
| 880087 | LTE iterbCA for 2 bands DL with 1 band UL | LTE_CA_R17_2BDL_1BUL |  | RP-211859 | Qualcomm |
| 880187 | Core part: LTE_CA_R17_2BDL_1BUL | LTE_CA_R17_2BDL_1BUL-Core | R4 | RP-211859 | Qualcomm |
| 880287 | Perf. part: LTE_CA_R17_2BDL_1BUL | LTE_CA_R17_2BDL_1BUL-Perf | R4 | RP-211859 | Qualcomm |
| 880088 | LTE iterbCA for 3 bands DL with 1 band UL | LTE_CA_R17_3BDL_1BUL |  | RP-212192 | Huawei |
| 880188 | Core part: LTE_CA_R17_3BDL_1BUL | LTE_CA_R17_3BDL_1BUL-Core | R4 | RP-212192 | Huawei |
| 880288 | Stopped - Perf. part: LTE iterbCA for 3 bands DL with 1 band UL | LTE_CA_R17_3BDL_1BUL-Perf | R4 | RP-202615 | Huawei |
| 880089 | LTE iterbCA Core for x bands DL (x=4, 5, 6) with 1 band UL | LTE_CA_R17_xBDL_1BUL |  | RP-211885 | Nokia |
| 880189 | Core part: LTE_CA_R17_xBDL_1BUL | LTE_CA_R17_xBDL_1BUL-Core | R4 | RP-211885 | Nokia |
| 880289 | Perf. part: LTE_CA_R17_xBDL_1BUL | LTE_CA_R17_xBDL_1BUL-Perf | R4 | RP-211885 | Nokia |
| 880090 | LTE iterbCA for 2 bands DL with 2 bands UL | LTE_CA_R17_2BDL_2BUL |  | RP-212194 | Huawei |
| 880190 | Core part: LTE_CA_R17_2BDL_2BUL | LTE_CA_R17_2BDL_2BUL-Core | R4 | RP-212194 | Huawei |
| 880290 | Perf. part: LTE_CA_R17_2BDL_2BUL | LTE_CA_R17_2BDL_2BUL-Perf | R4 | RP-212194 | Huawei |
| 880091 | LTE iterbCA for x bands DL (x= 3, 4, 5) with 2 bands UL | LTE_CA_R17_xBDL_2BUL |  | RP-211748 | LG |
| 880191 | Core part: LTE_CA_R17_xBDL_2BUL | LTE_CA_R17_xBDL_2BUL-Core | R4 | RP-211748 | LG |
| 880291 | Perf. part: LTE_CA_R17_xBDL_2BUL | LTE_CA_R17_xBDL_2BUL-Perf | R4 | RP-211748 | LG |

Summary based on the input provided by XXX in xP-22xxxx.

# 22	User plane improvements

| 820003 | Support of Immersive Teleconferencing and Telepresence for Remote Terminals | ITT4RT | S4 | SP-180985 | Ozgur Oyman, Intel |
| --- | --- | --- | --- | --- | --- |

Summary based on the input provided by Nokia Corporation in SP-220275.

This Work Item extends the functionality of Multimedia Telephony Service for IMS (MTSI) in TS 26.114 by adding the Virtual Reality (VR) unidirectional video transmission capability.

Earlier TS 26.114 was capable of handling real-time multimedia communications of traditional media (e.g., audio and video). The completed WI on ITT4RT enables, on top of the mentioned specification, new VR use cases and allows transmitting and receiving, in addition to traditional media, also uni-directional 360-degree video that can be viewed using Head Mounted Displays and 5G devices. This makes the end-user experience more compelling and immersive.

In addition, two more documents have been produced as part of this WI: TR 26.962 (Immersive Teleconferencing and Telepresence for Remote Terminals (ITT4RT) Operation and Usage Guidelines) and TR 26.862 (Immersive Teleconferencing and Telepresence for Remote Terminals (ITT4RT) Use Cases, Requirements and Potential Solutions).

The ITT4RT WI adds to TS 26.114 the following:

-	Support of still images, image sequences and still 360-degree background

-	Support of 360-degree video for H.265

-	Support of overlays on top of 360-degree video

-	Support of multiple video projection formats

-	Support for fisheye video

-	Support of camera calibration for Network-based Stitching

-	Support of picture packing for 360-degree video

-	Support of viewport dependent processing

-	Support of improved feedback for 360-degree video

-	Support of captured content replacement for screen sharing

-	Recommended audio mixing gains

-	Examples SDP offers and answers for 360-degree video.

References

List of related CRs: select "TSG Status = Approved" in: https://portal.3gpp.org/ChangeRequests.aspx?q=1&workitem=820003

[1]	TR 26.962. S4-220269

[2]	TR 26.862. S4-220323

| 900002 | Restoration of PDN Connections in PGW-C/SMF Set | RPCPSET | C4 | CP-203020 | LANDAIS, Bruno, Nokia |
| --- | --- | --- | --- | --- | --- |

Summary based on the input provided by Nokia in CP-220149.

This Work Item extends the functionality of Multimedia Telephony Service for IMS (MTSI) in TS 26.114 by adding the Virtual Reality (VR) unidirectional video transmission capability.

The work item defines PDN connection restoration procedures that enable to restore PDN connections in EPC after a PGW-C/SMF failure, restart or scale-in operation, by allowing to move PDN connections to a different PGW-C/SMF of the PGW-C/SMF set.

SMF scalability and resiliency can be supported for PDU sessions in 5GS by deploying an SMF set, i.e. a set of SMF instances that are functionally equivalent and inter-changeable and that share the same contexts (as defined in Release 16). This enables e.g. an AMF, PCF or UPF to reselect a different SMF in the same SMF set when the SMF serving a PDU session fails, restarts or is removed from the SMF set (scale-in operation), without interrupting the services and the PDN connectivity of the PDU session.

Inter-system mobility between 5GS and EPS relies on combo PGW-C/SMF. Combo PGW-C/SMF can be deployed in a PGW-C/SMF set. The work item defines PDN connection restoration procedures to restore the PDN connections served by a PGW-C/SMF, when a PGW-C/SMF in a PGW-C/SMF set fails, restarts or is removed from the PGW-C/SMF set. This allows to fully leverage the benefits of deploying PGW-C/SMF set by enabling:

-	to scale-in a PGW-C/SMF set without tearing down and re-establishing all the PDN connections of the PGW-C/SMF that is removed from the set;

-	an MME or ePDG to reselect a different PGW-C/SMF in the PGW-C/SMF set for an on-going PDN connection or a group of PDN connections, when the PGW-C/SMF that was serving the PDN connection or the group of PDN connections fails or restarts, and to maintain the services and PDN connectivity of the PDN connection or group of PDN connections; and

-	a PGW-C/SMF to instruct an MME, ePDG and/or PGW-U/UPF to use a new PGW-C/SMF for an on-going PDN connection or group of PDN connections, when e.g. a PCF or PGW-U/UPF reselects a different PGW-C/SMF from the PGW-C/SMF set.

References

List of related CRs: select "TSG Status = Approved" in: https://portal.3gpp.org/ChangeRequests.aspx?q=1&workitem=900002

| 830005 | Terminal Audio quality performance and Test methods for Immersive Audio Services | ATIAS | S4 | SP-190040 | Stéphane Ragot, Orange |
| --- | --- | --- | --- | --- | --- |
| 860012 | Handsets Featuring Non-Traditional Earpieces | HaNTE | S4 | SP-190989 | Schevciw, Andre, Qualcomm |
| 920008 | 5GMS AF Event Exposure | EVEX | S4 | SP-210374 | Lo, Charles, Qualcomm Inc., |
| 920009 | Edge Extensions to the 5G Media Streaming Architecture | 5GMS_EDGE | S4 | SP-210375 | Bouazizi, Imed, Qualcomm Inc. |
| 920010 | 5G Multicast-Broadcast User Service Architecture and related 5GMS Extensions | 5MBUSA | S4 | SP-210376 | TAN, PENG, TELUS |
| 920011 | 8K Television over 5G | 8K_TV_5G | S4 | SP-210381 | Thomas Stockhammer |

# 23	Signalling optimisations

| 880049 | Enhancement for the 5G Control Plane Steering of Roaming for UE in Connected mode | eCPSOR_CON |  | CP-210148 | Minokuchi, Atsushi, NTT DOCOMO |
| --- | --- | --- | --- | --- | --- |
| 850039 | Stage 1 of eCPSOR_CON | eCPSOR_CON | S1 | SP-190941 | Minokuchi, Atsushi, NTT DOCOMO |
| 880022 | Stage 3 (CT1) of eCPSOR_CON | eCPSOR_CON | C1 | CP-210148 | Al-Bakri Ban, NTT DOCOMO |
| 900036 | Stage 3 (CT4) of eCPSOR_CON | eCPSOR_CON | C4 | CP-210148 | Al-Bakri Ban, NTT DOCOMO |
| 920061 | Stage 3 (CT6) of eCPSOR_CON | eCPSOR_CON | C6 | CP-210148 | Al-Bakri Ban, NTT DOCOMO |

Summary based on the input provided by NTT DOCOMO in SP-220260.

Steering of roaming (SOR) allows the Home PLMN (HPLMN) operator to steer a UE to a visited PLMN (VPLMN) on which the HPLMN wants the UE to register, when the UE registers on another VPLMN. This capability may be needed for reasons such as reselection to a higher priority PLMN based on business arrangements (TS22.261 – clause 6.30.1).

Steering of Roaming (SOR) is used since GSM, and the requirements are provided in TS22.011.

In 5G, the Control Plane SOR solution (CP-SOR) was introduced using the same requirements in TS22.011.

In Rel-17, additional requirements specific for CP-SOR were provided in TS22.261 that allows the HPLMN to control the timing when a UE registered on a VPLMN, in automatic mode (see clause 3.1 of TS 23.122 [25]) and currently in CONNECTED mode, enters IDLE mode and initiates higher priority PLMN selection based on the type of ongoing communication.

Upon receiving CP-SOR information defined in Rel-15 and Rel-16, while the UE is in CONNECTED mode, the UE waits until it moves to idle mode before performing SOR and attempting to obtain service on a higher priority PLMN.

The UEs in 5G may stay in connected mode for a rather long time, whole day or longer, without going to idle mode. The HPLMN operator may have means to evaluate what is more convenient to provide the service for the user (e.g., based on their subscription profile, more efficient economically from wholesale perspective, allow users to use dedicated "economical" retail plans on specific VPLMN, etc.) and decide which VPLMN is more appropriate for the user to register on. Therefore, new requirements are introduced to allow the HPLMN to enforce the interruption of the ongoing sessions for the sake of performing SOR and moving the UE to another VPLMN to obtain service on a higher priority PLMN.

This feature introduces means to send additional steering of roaming information, called steering of roaming connected mode control information (SOR-CMCI) that enables the HPLMN to control the timing of a UE in connected mode to move to idle mode to perform the steering of roaming. The UE shall support the SOR-CMCI. The support and use of SOR-CMCI by the HPLMN is based on the HPLMN's operator policy.

The SOR-CMCI is provided by the HPLMN operator using different means, a specific application function (SOR-AF) or the information can be stored in the repository system (UDR). This information can be provided to the UE during registration procedure or after the registration (triggered by some parameters in the SOR-AF or the UDR).

The UE's type of ongoing communication / session(s) should be taken into consideration. The high priority services and emergency services are exempted from being interrupted (i.e. this feature is not applicable).

The new SOR related information is to be exchanged between the HPLMN UDM and the UE in a secured manner therefore the HPLMN data management application function ensures the security of the information using the Authentication Server Function (AUSF). The SOR information is transparent for the VPLMN, and if the VPLMN manipulates the information then the security of the information will break. To ensure the security of the information, the HPLMN may request the UE to provide an acknowledgement that the information is received securely.

This feature is optional for the HPLMN. This means that if this optional feature is not selected by the HPLMN operator, then there shall be no impact on the existing SOR functionality.

This feature is applicable only when the UE is in automatic network selection mode (see TS 23.122).

References

List of related CRs: select "TSG Status = Approved" in: https://portal.3gpp.org/ChangeRequests.aspx?q=1&workitem=880049,850039,880022,900036,920061

[1]	TS 22.011: "Service accessibility".

[2]	TS 22.261: "Service requirements for the 5G system; Stage 1".

[3]	TS 23.122: "Non-Access-Stratum (NAS) functions related to Mobile Station (MS) in idle mode".

[4]	TS 24.501: "Non-Access-Stratum (NAS) protocol for 5G System (5GS); Stage 3".

| 910044 | Same PCF Selection For AMF and SMF | TEI17_SPSFAS |  | SP-200447 | Heng, Nie, China Telecom |
| --- | --- | --- | --- | --- | --- |
| 880016 | Stage 2 for TEI17_SPSFAS | TEI17_SPSFAS | S2 | SP-200447 | Heng, Nie, China Telecom |
| 910003 | CT aspects on TEI17_SPSFAS | TEI17_SPSFAS | ct | CP-211184 | Yue Sun, China Telecom |
| 910045 | CT3 aspects on TEI17_SPSFAS | TEI17_SPSFAS | C3 | CP-211184 | Yue Sun, China Telecom |
| 910046 | CT4 aspects on TEI17_SPSFAS | TEI17_SPSFAS | C4 | CP-211184 | Yue Sun, China Telecom |

Summary based on the input provided by China Telecom in SP-220261.

Binding the Access and Mobility Management (AM) and the Session Management (SM) policies provides several benefits, such as the change of  RAT/Frequency Selection Priority (RFSP) values for a UE when the quota for the Data Network Name (DNN) for enhanced Mobile Broadband (eMBB) reaches the limit, or simplify the network deployment. A simple way to achieve such binding is to select the same Policy Control Function (PCF) for the AMF and the SMF.

In Rel 15 and 16, the same PCF selection for the AMF and the SMF is achieved by the mechanism of AMF optionally forwarding the selected PCF to SMF instance(s) during the PDU Session Establishment procedure(s), and the SMF may select the received PCF for SM Policy Control based on local policies. But it is applicable only in the UE initially accessing via 5GS scenario.

This Rel 17 work item specifies a mechanism to achieve the same PCF selection for AMF and SMF in the EPS to 5GS mobility scenario, where the PCF selection for AMF may be performed after the PCF selection for SMF/PGW-C. In this WI, subscription data is expanded to assist the same PCF selection, to offer more flexibility for operation of corresponding service.

The mechanism in this WI is based on the PGW-C+SMF registration of the selected PCF for SM Policy Control into the UDM+HSS. When performing PCF selection, based on subscription data for the PCF selection received from the UDM, the AMF determines that the same PCF for AMF and the SMF serving a specific DNN and/or S-NSSAI needs to be selected, then it selects the PCF ID serving the specific DNN and/or S-NSSAI received from UDM before initiating the Establishment of AM Policy Association. It is applicable for EPS to 5GS Mobility.

For the scenario of pure 5GS, based on subscription data for the PCF selection received from the UDM, the AMF determines that the same PCF for AMF and SMF serving a specific DNN and/or S-NSSAI needs to be selected, then it forwards the PCF ID for AM policies to the SMF and may additionally indicate to the SMF that the same PCF should be selected for SM Policy Control.

References

List of related CRs: https://portal.3gpp.org/ChangeRequests.aspx?q=1&workitem=910044,880016,910003,910045,910046

[1]	TS 23.501: “System architecture for the 5G System (5GS)”

[2]	TS 23.502: “Procedures for the 5G System; Stage 2”

[3]	TS 29.502: “5G System; Session Management Services; Stage 3”

[4]	TS 29.503: “5G System; Unified Data Management Services; Stage 3”

[5]	TS 29.513: “5G System; Policy and Charging Control signalling flows and QoS parameter mapping; Stage 3”

| 900037 | Enhancement of Inter-PLMN Roaming | EoIPR | C4 | CP-203021 | Caixia Qi, Huawei |
| --- | --- | --- | --- | --- | --- |

Summary based on the input provided by Huawei in CP-220152.

The work item defines some enhancements on the interfaces between vNSSF and hNSSF, vNRF and hNRF, vSEPP and hSEPP in stage3 for inter PLMN roaming in 5GC scenario.

Inter PLMN roaming is deployed for a user roaming into a vPLMN which has a roaming agreement with the UE's hPLMN. Some enhancements in stage3 are specifies with regards to Inter PLMN roaming scenarios:

-	The NSSF in the vPLMN can invoke the Nnssf_NSSelection service provided by the NSSF in the hPLMN, TS 29.531 specification for the interface between vNSSF and hNSSF is updated to specify the condition/procedure to support the communication.

-	Multiple NRFs can be deployed in a vPLMN and/or an hPLMN. TS 29.510 specification for the interface between vNRF and hNRF is updated to support the modification or removal of the subscription in the hNRF if multiple NRFs are deployed in hPLMN.

-	TS 29.573 specification for the interface between vSEPP and hSEPP is updated based on the requirements from GSMA and SA3 on the enhancements to the roaming.

References

List of related CRs: select "TSG Status = Approved" in: https://portal.3gpp.org/ChangeRequests.aspx?q=1&workitem=900037

| 940003 | Enhancement on the GTP-U entity restart | EGTPUR | C4 | CP-213097 | YONG YANG, Ericsson |
| --- | --- | --- | --- | --- | --- |

Summary based on the input provided by Ericsson in CP-220212.

A remote GTP-U restart will lead massive amount of signalling in the network (for reporting GTP-U Error Indication and PFCP Session Modification Signalling messages) over Sx/N4 interface.

To avoid that, this work item specifies following enhancements:

-		procedures to detect and report the peer GTP-U entity restart;

-		protocol support in GTP-U to enable to detect peer GTP-U entity restart;

-		protocol support in PFCP to enable User Plane function to report the peer GTP-U entity restart to the Control Plane function.

References

List of related CRs: select "TSG Status = Approved" in: https://portal.3gpp.org/ChangeRequests.aspx?q=1&workitem=940003

| 880032 | Packet Flow Description (PFD) management enhancement | pfdManEnh | C3 | CP-210183 | Xiaoyun Zhou, Huawei |
| --- | --- | --- | --- | --- | --- |

Summary based on the input provided by Huawei in CP-210183.

An optimize solution to reduce the load of signalling for pull mode in EPS and 5GS is defined. A notification push solution for push mode in 5GS is defined.

In stage 3, we define the GET method to retrieve the PFD for the application(s) by the client, the server responds a whole representation of the resource for the requested application identifier(s) even in the case that the client requests the PFDs when the caching timer expires and the PFD(s) is not changed at the server. It brings a large signalling load if the whole set of the PFD(s) are always returned by considering that the number of the PFD(s) is very large in the deployment.

For push mode, in order to protect the PCEF/TDF from overload, CT3 introduces the notification push for the Push mode in EPS, i.e. the PFDF sends a request with notification flag to the PCEF/TDF and then PCFE/TDF initiates separate pull requests to retrieve the PFDs for different application identifier(s) immediately or within the allowed-delay by considering the load of the PCEF/TDF. But this optimized solution is not defined in 5GS.

CT3#110e decided to implement the solution to resolve the above identified issues.

References

List of related CRs: select "TSG Status = Approved" in: https://portal.3gpp.org/ChangeRequests.aspx?q=1&workitem=880032

[1]		3GPP TS 29.251: "Gw and Gwn reference points for sponsored data connectivity".

[2]		3GPP TS 29.551: "5G System; Packet Flow Description Management Service; Stage 3".

[3]		3GPP TS 29.513: "5G System; Policy and Charging Control signalling flows and QoS parameter mapping; Stage 3".

| 940002 | Restoration of Profiles related to UDR | ReP_UDR | C4 | CP-213070 | Ishikawa, Hiroshi, NTT DOCOMO |
| --- | --- | --- | --- | --- | --- |

Summary based on the input provided by NTT DOCOMO in CP-220343.

The "Restoration of Profiles related to UDR" (ReP_UDR) enables restoration of profiles between UDR and related NF consumers, to maintain the status up to date as much as possible even in case data stored in UDR is lost or corrupted.

Data stored in UDR is automatically updated in normal operation. Some data is stored in volatile storage units due to its nature, such as location of UE, subscription data, policy data, and loss or corruption of storage containing such data could negatively impact the service to subscribers. Replication of volatile storage units and periodic back-up of data are used to avoid loss or corruption, however further procedures are required in case the integrity of data in UDR cannot be ensured, especially when relying on replicated data.

Following enhancements are specified to fulfil these required procedures:

-	Preparation of profile in NF consumers

-	Path and contents of notification sent from UDR to NF consumers when loss or corruption at UDR is detected

-	Procedure for synchronization initiated by the NF consumer upon receiving notification of loss or corruption

References

List of related CRs: select "TSG Status = Approved" in: https://portal.3gpp.org/ChangeRequests.aspx?q=1&workitem=940002

| 890070 | PAP/CHAP protocols usage in 5GS | PAP_CHAP | ct | CP-202251 | Mingxue Li, China Telecom |
| --- | --- | --- | --- | --- | --- |
| 890071 | CT1 aspects of PAP/CHAP protocols usage in 5GS | PAP_CHAP | C1 | CP-202251 | Mingxue Li, China Telecom |
| 890072 | CT3 aspects of PAP/CHAP protocols usage in 5GS | PAP_CHAP | C3 | CP-210188 | Mingxue Li, China Telecom |

Summary based on the input provided by China Telecom in CP-202251.

This work item enables the support of the (Extended) Protocol Configuration Options [(e)PCO] parameters related to the Password Authentication Protocol (PAP) / Challenge Handshake Authentication Protocol (CHAP) over 5GS.

This is needed in the following cases:

- migration from EPS to 5GS and potential requirements related to legacy deployments for access to corporate networks, e.g. support of PAP/CHAP in DN-AAA server owned by 3rd parties,

- access to corporate networks by early 5GS deployments (e.g. with standalone SMF), e.g. to support PAP/CHAP in DN-AAA server or LNS owned by 3rd parties.

References

List of related CRs: select "TSG Status = Approved" in: https://portal.3gpp.org/ChangeRequests.aspx?q=1&workitem=890070,890071,890072

[1]	TS 29.561: “Interworking between 5G Network and external Data Networks”

[2]	TS 24.501: “Non-Access-Stratum (NAS) protocol for 5G System (5GS)”

[3]	TS 24.526: “User Equipment (UE) policies for 5G System (5GS)”

# 24	Other Release 17 Features

| tbd | Non-Seamless WLAN offload authentication in 5GS | NSWO_5G | C4 | C4-220349 | WIEHE, Ulrich, Nokia |
| --- | --- | --- | --- | --- | --- |
| 910091 | Study on non-seamless WLAN Offload in 5GS using 3GPP credentials | FS_NSWO_5G | S3 | SP-210262 | Nair, Suresh, Nokia |
| 940011 | Security aspects of Non-Seamless WLAN offload in 5GS | NSWO_5G | S3 | SP-211358 | Ranganathan Mavureddi Dhanasekaran, Nokia |

Summary based on the input provided by Nokia in CP-220150.

The work item defines stage 3 protocol enhancements for Non-seamless WLAN offload (NSWO) authentication in 5GS according to stage 2 requirements from TS 33.501 annex S.

NSWO is an optional capability of a UE supporting WLAN radio access. A UE supporting non-seamless WLAN offload may, while connected to WLAN access, route specific IP flows via the WLAN access without traversing the 3GPP core network.

For authentication, 5G NSWO uses EAP-AKA' as specified in RFC 5448.

A new network function, called NSWOF, supports authentication for NSWO in 5GS. The NSWOF interfaces the WLAN access network via SWa and the AUSF via the Nausf service based interface (SBI). The AUSF retrieves NSWO-specific authentication information from the UDM via the Nudm service based interface.

In addition, the USIM and/or ME can be configured to use 5G NSWO.

References

List of related CRs: select "TSG Status = Approved" in: https://portal.3gpp.org/ChangeRequests.aspx?q=1&workitem=910091,940011

As well as the following CRs:

[1]	CR 29.503 #0790, C4-220141, NSWO authentication

[2]	CR 29.509 #0143r1, C4-221545, NSWO authentication

[3]	CR 29.510 #0669r1, C4-221469, NSWO NF

[4]	CR 29.273 #0530, C4-221155, Non-Seamless WLAN offload in 5GS

[5]	CR 24.302 #0726r1, C1-222069, Add ANID for 5G NSWO

[6]	CR 24.502 #0198r1, C1-222070, Add support of 5G NSWO

[7]	CR 24.501 #4123, C1-222072, Extension of SNN description for NSWO

[8]	CR 31.102 #0946, C6-220154, 5G NSWO (Non-Seamless WLAN Offload) configuration support in the USIM compromised proposal.

[9]	CR 31.111 #0769r2, C6-220174, Addition of the procedure for updating of 5G NSWO configuration in the USIM

| 910007 | Start of Pause of Charging via User Plane | SPOCUP | C4 | CP-210076 | YONG, YANG, Ericsson, frank |
| --- | --- | --- | --- | --- | --- |

Summary based on the input provided by Ericsson in CP-220211.

The work item defines a User Plane (GTP-U) based solution to start pause of charging for a PDN connection or a PDU session to reduce the Charging Data Record discrepancies between SGW or I/V-SMF and PGW or (h)SMF caused by the control plane signalling latency in existing solution.

PGW/SMF Pause of Charging feature is to limit a mismatch between PGW-C/SMF and SGW/I/V-SMF charging volume and packet counts. To further reduce charging discrepancies due to control plane signalling latency, the work item specifies following enhancements to enable start of pause of charging via user plane path:

-		a new feature "Notify Start Pause of Charging via user plane feature" (NSPOC) is introduced in PFCP to enable CP function ((SGW-C and PGW-C for EPC, I/V-SMF and (H-)SMF for 5GC) to instruct UP function (SGW-U and PGW-U for EPC, I/V-UPF and PSA UPF), to trigger pause of charging at the upstream GTP-U entity via sending one or more GTP-U Tunnel Status messages, so that the upstream GTP-U entity can stop usage measurement for those URRs applicable for charging; and

-		a new mechanism to stop/resume the usage measurement in PFCP when NSPOC is supported; and

-	  a new GTP-U message "Tunnel Status" is introduced in GTP-U protocol to be used to notify the upstream GTP-U entity to start pause of charging; and

-		the support of NSPOC feature is exchanged via control plane signalling between PGW-C/SMF and SGW/I/V-SMF respectively.

References

List of related CRs: select "TSG Status = Approved" in: https://portal.3gpp.org/ChangeRequests.aspx?q=1&workitem=910007

| 911037 | Reliable Data Service Serialization Indication | RDSSI |  | SP-190446 | Starsinic, Michael, Convida Wireless LLC |
| --- | --- | --- | --- | --- | --- |
| 840017 | Stage 2 of RDSSI | RDSSI | S2 | SP-190446 | Starsinic, Michael, Convida Wireless LLC |
| 900005 | CT aspects of RDSSI | RDSSI | C1 | CP-203234 | Vivek Gupta, Intel |
| 930002 | IMS voice service support and network usability guarantee for UE s E-UTRA capability disabled scenario in SA 5GS | ING_5GS | ct | CP-212231 | Mingxue Li, China Telecom |
| 930047 | CT1 aspects of ING_5GS | ING_5GS | C1 | CP-212231 | Mingxue Li, China Telecom |
| 930046 | CT6 aspects of ING_5GS | ING_5GS | C6 | CP-212231 | Mingxue Li, China Telecom |
| 930031 | User Consent for 3GPP services | FS_UC3S | S3 | SP-200885 | Rong Wu, Huawei Technologies |
| 890037 | Study on User Consent for 3GPP services | FS_UC3S | S3 | SP-200885 | Rong Wu, Huawei Technologies |
| 930006 | Security aspects on User Consent for 3GPP services | UC3S_SEC | S3 | SP-210836 | Rong Wu, Huawei Technologies |
| 920060 | NPN support of PWS | NPN_PWS | S1 | SP-210585 | Francesco Pica, Qualcomm |
| 910024 | Enhancements of 3GPP profiles for cryptographic algorithms and security protocols | eCryptPr | S3 | SP-210107 | Pinar Comak, Ericsson |
| 920027 | AKMA TLS protocol profiles | AKMA_TLS | S3 | SP-210424 | Escott, Adrian, Qualcomm |
| 910025 | User Plane Integrity Protection for LTE | UPIP_SEC_LTE | S3 | SP-210105 | Evans, Tim, Vodafone |
| 900019 | Adapting BEST for use in 5G networks | BEST_5G | S3 | SP-201020 | Keesmaat, Iko, KPN |
| 840048 | Enhanced Relays for Energy eFficiency and Extensive Coverage | REFEC |  | SP-190307 | Norp, Toon, KPN |
| 810018 | Study on REFEC | FS_REFEC | S1 | SP-180785 | Jose Luis Almodovar Chico, KPN |
| 840034 | Stage 1 of REFEC | REFEC | S1 | SP-190307 | Norp, Toon, KPN |
| 840044 | Network Controlled Interactive Service | NCIS |  | SP-190303 | Ning YANG, oppo |
| 800015 | Study on NCIS | FS_NCIS | S1 | SP-180341 | YANG Ning, Oppo |
| 840030 | Stage 1 of NCIS | NCIS | S1 | SP-191039 | Ning YANG, oppo |
| 840019 | 5G System Enhancement for Advanced Interactive Services | 5G_AIS | S2 | SP-190564 | Lei Yixue, Tencent |
| 890009 | Points for 8K VR 360 Video over 5G | 8K_VR_5G | S4 | SP-200667 | Ozgur Oyman, Company: Intel |
| 880043 | Multi-Device and multi-identity Enhancements (MuDE) | MuDE |  | SP-190823 | Adrian.buckley@vivo.com |
| 850041 | Stage 1 of MuDE | MuDE | S1 | SP-190823 | Adrian.buckley@vivo.com |
| 880018 | Stage 3 of MuDE | MuDe | C1 | CP-201162 | Adrian Buckley |
| 920055 | Best Practice of Packet Forwarding Control Protocol (PFCP) | BEPoP | ct | CP-212024 | Song Yue, China Mobile |
| 920056 | CT3 aspects of BEPoP | BEPoP | C3 | CP-212024 | Song Yue, China Mobile |
| 880014 | CT4 aspects of BEPoP | BEPoP | C4 | CP-212024 | Song Yue, China Mobile |
| 880029 | Enhancement of Handover Optimization | E_HOO | S5 | SP-200466 | Elmdahl, Per, Ericsson |
| 880012 | Extension for headset interface tests of UE | HInT | S4 | SP-200398 | Jan Reimes, HEAD acoustics GmbH, |
| 840038 | MC services support on IOPS mode of operation | MCIOPS | S6 | SP-190944 | Camilo.solano@ericsson.com |
| 840016 | Supporting Flexible Local Area Data Network | FLADN | S2 | SP-190563 | LG Electronics, Hyunsook Kim |
| 830017 | Verification-modified Calling Name Display | VMOD_DISPLAY | S1 | SP-190085 | Hala Mowafy, Ericsson |
| 850038 | Enhanced Calling Name Service Analytics Interworking | eCNAM_An | S1 | SP-190940 | Hala.Mowafy@ericsson.com |
| 910090 | Integration of Generic Bootstrapping Architecture (GBA) into 5GC | GBA_5G |  | SP-190714 | Vlasios Tsiatsis, Ericsson |
| 850023 | Security aspects of Integration of GBA into 5GC | GBA_5G | S3 | SP-190714 | Vlasios Tsiatsis, Ericsson |
| 910004 | CT aspects of Integration of GBA into SBA | GBA_5G | C4 | CP-210283 | de Gregorio, Jesús (Ericsson) |
| 910047 | Integration of Generic Bootstrapping Architecture (GBA) into 5GC | GBA_5G | S3 | SP-190714 | Vlasios Tsiatsis, Ericsson |
| 770024 | EVS Codec Extension for Immersive Voice and Audio Services | IVAS_Codec | S4 | SP-170611 | Bin Wang, Huawei Technologies Co Ltd |
| 860008 | IMS emergency support for Stand-alone Non-Public Network (SNPN) | IESNPN | S1 | SP-191038 | Wong, Curt; |
| 870023 | Management of non-public networks (NPN) | OAM_NPN | S5 | SP-200189 | ZHANG, Kai, Huawei |
| 860016 | Assurance Specification for IMS | SCAS_IMS | S3 | SP-191128 | Bo Zhang, Huawei Technologies |
| 870020 | Security Assurance Specification for 5G (eSCAS_5G) | eSCAS_5G | S3 | SP-200149 | Rong Wu, Huawei Technologies Co. |
| 890013 | eSCAS_5G for Network Slice-Specific Authentication and Authorization Function (NSSAAF) | SCAS_5G_NSSAAF | S3 | SP-200720 | Rong Wu, Huawei Technologies Co., Ltd. |
| 870017 | eSCAS_5G for Non-3GPP InterWorking Function | SCAS_5G_N3IWF | S3 | SP-200146 | Feng Gao, China Unicom |
| 870018 | eSCAS_5G for 5G NWDAF | SCAS_5G_NWDAF | S3 | SP-200147 | QI Minpeng, China Mobile |
| 870019 | eSCAS_5G for Service Communication Proxy | SCAS_5G_SECOP | S3 | SP-200148 | Wei Lu, Nokia |
| 880003 | eSCAS_5Gfor Inter PLMN UP Security | SCAS_5G_IPUPS | S3 | SP-200348 | Jin PENG, ZTE Corporation |
| 860025 | Lawful Interception Rel-17 | LI17 | S3 | SP-190983 | Alex Leadbeater, BT |
| 850047 | (Small) Technical Enhancements and Improvements for Rel-17 | TEI17 |  |  |  |
| 870003 | IMS Optimization for HSS Group ID in an SBA environment | TEI17_IMSGID | S2 | SP-200452 | Foti, George, Ericsson |
| 930036 | System enhancement for Redundant PDU Session | TEI17_SE_RPS |  | SP-200448 | Devaki Chandramouli |
| 880017 | Stage 2 for TEI17_SE_RPS | TEI17_SE_RPS | S2 | SP-200448 | Devaki Chandramouli |
| 930001 | CT aspects of TEI17_SE_RPS | TEI17_SE_RPS | ct | CP-212099 | WON, Sung Hwan, Nokia |
| 930037 | CT1 aspects of TEI17_SE_RPS | TEI17_SE_RPS | C1 | CP-212099 | WON, Sung Hwan, Nokia |
| 930038 | CT3 aspects of TEI17_SE_RPS | TEI17_SE_RPS | C3 | CP-212099 | WON, Sung Hwan, Nokia |
| 930039 | CT4 aspects of TEI17_SE_RPS | TEI17_SE_RPS | C4 | CP-212099 | WON, Sung Hwan, Nokia |
| 910078 | Support for Signed Attestation for Priority and Emergency Sessions | TEI17_SAPES |  | SP-200453 | Foti, George, Ericsson |
| 870004 | Stage 2 for TEI17_SAPES | TEI17_SAPES | S2 | SP-200453 | Foti, George, Ericsson |
| 910020 | CT aspects on TEI17_SAPES | TEI17_SAPES | ct | CP-210272 | Axell, Jörgen, Ericsson |
| 910079 | CT1 aspects on TEI17_SAPES | TEI17_SAPES | C1 | CP-210272 | Axell, Jörgen, Ericsson |
| 910080 | CT3 aspects on TEI17_SAPES | TEI17_SAPES | C3 | CP-210272 | Axell, Jörgen, Ericsson |
| 910081 | Dynamically Changing AM Policies in the 5GC | TEI17_DCAMP |  | SP-200446 | Pancorbo, Belen (Ericsson) |
| 870005 | Stage 2 for TEI17_DCAMP | TEI17_DCAMP | S2 | SP-200446 | Pancorbo, Belen (Ericsson) |
| 910009 | CT aspects on TEI17_DCAMP | TEI17_DCAMP | ct | CP-212163 | Garcia Azorero, Fuencisla. Ericsson |
| 910082 | CT3 aspects on TEI17_DCAMP | TEI17_DCAMP | C3 | CP-212163 | Garcia Azorero, Fuencisla. Ericsson |
| 910083 | CT4 aspects on TEI17_DCAMP | TEI17_DCAMP | C4 | CP-212163 | Garcia Azorero, Fuencisla. Ericsson |
| 870006 | IP address pool information from UDM | TEI17_IPU | S2 | SP-200454 | Stefan Rommer |
| 910084 | Dynamic management of group-based event monitoring | TEI17_GEM |  | SP-200455 | Qian XB Chen, Ericsson |
| 870007 | Stage 2 for TEI17_GEM | TEI17_GEM | S2 | SP-200455 | Qian XB Chen, Ericsson |
| 910010 | CT aspects on TEI17_GEM | TEI17_GEM | ct | CP-212165 | Maria Tianmei, Liang, Ericsson |
| 910085 | CT3 aspects on TEI17_GEM | TEI17_GEM | C3 | CP-212165 | Maria Tianmei, Liang, Ericsson |
| 910086 | CT4 aspects on TEI17_GEM | TEI17_GEM | C4 | CP-212165 | Maria Tianmei, Liang, Ericsson |
| 920041 | Support of different slices over different Non 3GPP access | TEI17_N3SLICE |  | SP-200456 | Laurent Thiebaut, Nokia |
| 870008 | Stage 2 for TEI17_N3SLICE | TEI17_N3SLICE | S2 | SP-200456 | Laurent Thiebaut, Nokia |
| 930099 | CT1 aspects of TEI17_N3SLICE | TEI17_N3SLICE | C1 | CP-211089 | LANDAIS, Bruno, Nokia |
| 920001 | CT4 aspects of TEI17_N3SLICE | TEI17_N3SLICE | C4 | CP-211089 | LANDAIS, Bruno, Nokia |
| 910087 | N7/N40 Interfaces Enhancements to Support GERAN and UTRAN | TEI17_NIESGU |  | SP-200457 | Aihua, Li, China Mobile |
| 870009 | Stage 2 for TEI17_NIESGU | TEI17_NIESGU | S2 | SP-200457 | Aihua, Li, China Mobile |
| 890019 | SA5 aspects of N40 Interface Enhancements to Support GERAN and UTRAN | TEI17_NIESGU | S5 | SP-200854 | Dong, Jia, China Mobile |
| 910011 | CT3 aspects of N7 Interfaces Enhancements to Support GERAN and UTRAN | TEI17_NIESGU | C3 | CP-211194 | Huang Zhenning (China Mobile) |
| 920082 | CT4 aspects of N7 Interfaces Enhancements to Support GERAN and UTRAN | TEI17_NIESGU | C4 | CP-211194 | Huang Zhenning (China Mobile) |

Summary based on the input provided by XXX in xP-22xxxx.

# 25	Management-only Features

This section presents all the Features with no corresponding work in other group than the Operations, administration and management (OAM) group, i.e. SA5. For instance, OAM for "Industrial IoT" is reported in the section on "Industrial IoT" and not here.

| 870022 | Enhancements on EE for 5G networks | EE5GPLUS | S5 | SP-200188 | Jean-Michel CORNILY, ORANGE |
| --- | --- | --- | --- | --- | --- |
| 870024 | Enhancement on Management Aspects of 5G Service-Level Agreement | EMA5SLA | S5 | SP-210860 | SHI, Xiaonan, CMCC |
| 870025 | Management of MDT enhancement in 5G | e_5GMDT | S5 | SP-200191 | Ayani, Zhulia, Ericsson |
| 870026 | Additional NRM features | adNRM | S5 | SP-200192 | Jing Ping, Nokia |
| 870027 | Enhancement of QoE Measurement Collection | eQoE | S5 | SP-200193 | Petersen, Robert, Ericsson |
| 860061 | Study on NR QoE (Quality of Experience) management and optimizations for diverse services | FS_NR_QoE | R3 | RP-193256 | China Unicom |
| 870030 | Enhanced Closed loop SLS Assurance | eCOSLA | S5 | SP-200196 | Groenendijk, Jan, Ericsson |
| 840028 | IMS Charging in 5G System Architecture | 5GSIMSCH | S5 | SP-190367 | Jahangir, Zeeshan, T-Mobile USA |
| 810027 | Intent driven management service for mobile network | IDMS_MN | S5 | SP-180899 | Zou Lan, Huawei |
| 820006 | Study on User Plane Integrity Protection | FS_UP_IP_Sec | S3 | SP-181035 | Evans, Tim, Vodafone |
| 890012 | Enhancements to User Plane Integrity Protection Support in 5GS | eUPIP_SEC | S3 | SP-200719 | Anand Palanigounder, Qualcomm |
| 820035 | Discovery of management services in 5G | 5GDMS | S5 | SP-200770 | Brendan Hassett, Huawei Technologies Sweden AB |
| 860024 | Network policy managementfor 5G mobile networks | NPM | S5 | SP-191211 | China Mobile Shasha Guo |
| 850028 | Study on enhancement of Management Data Analytics Service | FS_eMDAS | S5 | SP-190930 | Yao, Yizhi, Intel |
| 910027 | Enhancements of Management Data Analytics Service | eMDAS | S5 | SP-210132 | Yao, Yizhi, Intel |
| 880028 | Management data collection control and discovery | MADCOL | S5 | SP-200465 | Pollakowski, Olaf, Nokia |
| 880026 | Management of the enhanced tenant concept | eMEMTANE | S5 | SP-200463 | Zhu, Lei, Huawei |
| 880025 | Enhancements of 5G performance measurements and KPIs | ePM_KPI_5G | S5 | SP-200462 | Yizhi Yao, Intel |
| 930032 | Improved support for NSA in the service-based management architecture | NSA_SBMA | S5 | SP-211121 | Lan Zou, Huawei |
| 910031 | Study on Enhancement of service-based management architecture | FS_eSBMA | S5 | SP-210136 | Lan Zou, Huawei |
| 930009 | Improved support for NSA in the service-based management architecture | NSA_SBMA | S5 | SP-210858 | Lan Zou, Huawei |
| 930033 | Access control for management service | MSAC | S5 | SP-210859 | Jing Ping, Nokia |
| 890016 | Study on access control for management service | FS_MNSAC | S5 | SP-200853 | Jing Ping, Nokia |
| 930010 | Access control for management service | MSAC | S5 | SP-210859 | Jing Ping, Nokia |
| 910030 | File Management | FIMA | S5 | SP-210135 | Pollakowski, Olaf, Nokia |
| 900021 | Management Aspects of 5G Network Sharing | MANS | S5 | SP-201080 | Mingrui Sun, China Unicom |
| 920018 | Study on Management Aspects of 5G Network Sharing | FS_MANS | S5 | SP-210387 | Mingrui Sun, China Unicom |

# 26	Features without summary

The Features listed below are not summarised in this document because their output is not significant enough. It can e.g. be some minor protocol enhancements.

The corresponding CR(s), if any, can be found by replacing [UID] by the actual UID in the link below, selecting "TSG Status = Approved" in the page:

https://portal.3gpp.org/ChangeRequests.aspx?q=1&workitem=[UID]

The UID is the left-most number in the table below.

| 880020 | Protocol enhancements for Mission Critical Services | MCProtoc17 | C1 | CP-202193 | AXELL, Jörgen |
| --- | --- | --- | --- | --- | --- |
| 880019 | Stage-3 5GS NAS protocol development 17 | 5GProtoc17 | C1 | CP-201163 | Sedlacek, Ivo, Ericsson |
| 880044 | Stage-3 5GS NAS protocol development 17 general aspects | 5GProtoc17 | C1 | CP-201163 | Sedlacek, Ivo, Ericsson |
| 880045 | Stage-3 5GS NAS protocol development 17 non-IETF aspects | 5GProtoc17-non3GPP | C1 | CP-201163 | Sedlacek, Ivo, Ericsson |
| 880021 | Stage-3 SAE Protocol Development | SAES17 | C1 | CP-201165 | Aghili, Behrouz, InterDigital Communications |
| 880046 | Stage-3 SAE Protocol Development general aspects | SAES17 | C1 | CP-201165 | Aghili, Behrouz, InterDigital Communications |
| 880047 | Stage-3 SAE Protocol Development CSFB aspects | SAES17-CSFB | C1 | CP-201165 | Aghili, Behrouz, InterDigital Communications |
| 880048 | Stage-3 SAE Protocol Development non 3GPP aspects | SAES17-non3GPP | C1 | CP-201165 | Aghili, Behrouz, InterDigital Communications |
| 911034 | IMS Stage-3 IETF Protocol Alignment | IMSProtoc17 | ct | CP-201167 | Gkatzikis, Lazaros(Nokia) |
| 880023 | CT1 aspects of IMS Stage-3 IETF Protocol Alignment | IMSProtoc17 | C1 | CP-201167 | Gkatzikis, Lazaros(Nokia) |
| 911035 | CT3 aspects of IMS Stage-3 IETF Protocol Alignment | IMSProtoc17 | C3 | CP-201167 | Gkatzikis, Lazaros(Nokia) |
| 880013 | Service Based Interface Protocol Improvements Release 17 | SBIProtoc17 |  | CP-211088 | Song Yue, China Mobile |
| 880053 | CT3 aspects of SBIProtoc17 | SBIProtoc17 | C3 | CP-211088 | Song Yue, China Mobile |
| 880054 | CT4 aspects of SBIProtoc17 | SBIProtoc17 | C4 | CP-211088 | Song Yue, China Mobile |

Annex A:
List of expected contributions

Summary needed for highlighted items. Summary to be produced by the Rapporteur as highlighted in the table below.

| UID | Name | A c r o n y m | WG | WID | WI rapporteur name/company |
| --- | --- | --- | --- | --- | --- |
| 0 | Rel-17 on Use of 5G by different sectors ("verticals") | - |  | - | - |
| 840050 | Enhancements for cyber-physical control applications in vertical domains | e C A V |  | SP-190310 | Bahr, Michael, Siemens AG |
| 830020 | Study on eCAV | F S _ e C A V | S1 | SP-190092 | Michael Bahr, Siemens AG |
| 840041 | Stage 1 of eCAV | e C A V | S1 | SP-191043 | Bahr, Michael, Siemens AG |
| 840024 | Study on enhanced support of Non-Public Networks | F S _ e N P N | S2 | SP-200094 | Peter Hedman |
| 910065 | Enhanced support of Non-Public Networks | e N P N |  | SP-200980 | Hedman, Peter, Ericsson |
| 900015 | Stage 2 for eNPN | e N P N | S2 | SP-200980 | Hedman, Peter, Ericsson |
| 910016 | CT aspects of eNPN | e N P N | ct | CP-212103 | Sedlacek, Ivo, Ericsson |
| 910066 | CT1 aspects of eNPN | e N P N | C1 | CP-212103 | Sedlacek, Ivo, Ericsson |
| 910067 | CT3 aspects of eNPN | e N P N | C3 | CP-212103 | Sedlacek, Ivo, Ericsson |
| 910068 | CT4 aspects of eNPN | e N P N | C4 | CP-212103 | Sedlacek, Ivo, Ericsson |
| 920025 | Security Aspects of eNPN | e N P N | S3 | SP-210422 | Jost, Christine, Ericsson |
| 880008 | Study on enhanced security support for Non-Public Networks | F S _ e N P N _ S E C | S3 | SP-200353 | Normann, Henrik, Ericsson |
| 840045 | Audio-Visual Service Production | A V P R O D |  | SP-190304 | Wagdin, Ian, BBC |
| 800014 | Study on AVPROD | F S _ A V P R O D | S1 | SP-181015 | Roland Beutler, EBU |
| 840031 | Stage 1 of AVPROD | A V P R O D | S1 | SP-191041 | Wagdin, Ian, BBC |
| 910001 | Study on Media Production over 5G NPN | F S _ N P N 4 A V P r o d | S4 | SP-210241 | Lohmar, Thorsten, Ericsson |
| 820025 | Study on application layer support for Factories of the Future in 5G network | F S _ F F A P P | S6 | SP-200836 | Shao Weixiang, ZTE Corporation |
| 840025 | Study on enhancement of support for 5G LAN-type service | F S _ 5 G L A N _ e n h | S2 | SP-190626 | Runze Zhou, Huawei |
| 900022 | Study on Charging Aspect of 5G LAN-type Services | F S _ 5 G L A N _ C H | S5 | SP-201081 | CHEN SHAN, Huawei |
| 840042 | Complete Gap Analysis for Railways Mobile Communication System | M O N A S T E R Y E N D | S1 | SP-190312 | Gach, Guillaume, UIC |
| 890035 | Enhancements to Application Architecture for the Mobile Communication System for Railways Phase 2 | e M O N A S T E R Y 2 |  | SP-191104 | Oettl, Martin, Nokia |
| 840037 | Stage 2 of eMONASTERY2 | e M O N A S T E R Y 2 | S6 | SP-191104 | Oettl, Martin, Nokia |
| 890063 | Stage 3 of eMONASTERY2 | e M O N A S T E R Y 2 | C1 | CP-202256 | Gkatzikis, Lazaros, Nokia(Nokia) |
| 840043 | Study on Future Railway Mobile Communication System3 | F S _ F R M C S 3 | S1 | SP-200578 | Gach, Guillaume, UIC, |
| 850046 | Asset Tracking for 5G | A T R A C |  | SP-190816 | Thierry Berisot, NOVAMINT |
| 810017 | Study on ATRAC | F S _ 5 G _ A T R A C | S1 | SP-180922 | Thierry Berisot, NOVAMINT |
| 850037 | Stage 1 of ATRAC | A T R A C | S1 | SP-190931 | Thierry Berisot, NOVAMINT |
| 910052 | Enhancement to the 5GC LoCation Services-Phase 2 | 5 G _ e L C S _ p h 2 |  | SP-200082 | Ming Ai, CATT |
| 870001 | Stage 2 of 5G_eLCS_ph2 | 5 G _ e L C S _ p h 2 | S2 | SP-200082 | Ming Ai, CATT |
| 910006 | CT aspects of 5G_eLCS_ph2 | 5 G _ e L C S _ p h 2 | ct | CP-211090 | Chenxi Bao, CATT |
| 910053 | CT1 aspects of 5G_eLCS_ph2 | 5 G _ e L C S _ p h 2 | C1 | CP-211090 | Chenxi Bao, CATT |
| 910054 | CT3 aspects of 5G_eLCS_ph2 | 5 G _ e L C S _ p h 2 | C3 | CP-211090 | Chenxi Bao, CATT |
| 910055 | CT4 aspects of 5G_eLCS_ph2 | 5 G _ e L C S _ p h 2 | C4 | CP-211090 | Chenxi Bao, CATT |
| 900060 | RAN aspects of NR positioning enhancements | N R _ p o s _ e n h |  | RP-210903 | Intel |
| 860034 | Study on NR positioning enhancements | F S _ N R _ p o s _ e n h | R1 | RP-202094 | CATT |
| 900160 | Core part: NR positioning enhancements | N R _ p o s _ e n h - C o r e | R1 | RP-210903 | Intel |
| 900260 | Perf. part: NR positioning enhancements | N R _ p o s _ e n h - P e r f | R4 | RP-210903 | Intel |
| 840047 | Communication Service Requirements for Critical Medical Applications | C M E D |  | SP-190306 | Lagrange, Mathieu, b<>com |
| 810016 | Study on CMED | F S _ C M E D | S1 | SP-180783 | Mathieu Lagrange, b<>com |
| 840033 | Stage 1 of CMED | C M E D | S1 | SP-190306 | Lagrange, Mathieu, b<>com |
| 0 | Rel-17 drone/UAS/UAV/EAV | - |  | - | - |
| 840083 | 5G Enhancement for UAVs | E A V |  | SP-190308 | Qun Wei, China Unicom |
| 810019 | Study on EAV | F S _ E A V | S1 | SP-180909 | Qun Wei, China Unicom |
| 840039 | Stage 1 of EAV | E A V | S1 | SP-190308 | Qun Wei, China Unicom |
| 820011 | Study on supporting Uncrewed Aerial Systems Connectivity, Identification, and Tracking | F S _ I D _ U A S _ S A 2 | S2 | SP-200097 | Qualcomm, Stefano Faccin |
| 820026 | Study on application layer support for Uncrewed Aerial System (UAS) | F S _ U A S A P P | S6 | SP-200111 | Niranth Amogh, Huawei |
| 920045 | Application layer support for Uncrewed Aerial System (UAS) | U A S A P P |  | SP-200988 | Niranth Amogh, Huawei Telecommunications India |
| 900025 | Stage 2 of Application layer support for Uncrewed Aerial System (UAS) | U A S A P P | S6 | SP-200988 | Niranth Amogh, Huawei Telecommunications India |
| 920004 | CT1 Aspects of Application Layer Support for Uncrewed Aerial Systems (UAS) | U A S A P P | C1 | CP-211111 | Lin Shu, Huawei, |
| 920046 | CT3 Aspects of Application Layer Support for Uncrewed Aerial Systems (UAS) | U A S A P P | C3 | CP-211111 | Lin Shu, Huawei |
| 880007 | Study on security aspects of Uncrewed Aerial Systems (UAS) | F S _ U A S _ S E C | S3 | SP-200352 | Adrian Escott, Qualcomm |
| 810049 | Remote Identification of Uncrewed Aerial Systems | I D _ U A S |  | SP-180771 | Eddy HALL, Qualcomm |
| 810013 | Stage 1 of Remote Identification of Uncrewed Aerial Systems | I D _ U A S | S1 | SP-180771 | Eddy HALL, Qualcomm |
| 900014 | (Stage 2 of) Support of Uncrewed Aerial Systems Connectivity, Identification, and Tracking | I D _ U A S | S2 | SP-200979 | Faccin, Stefano, Qualcomm |
| 910017 | CT aspects for ID_UAS | I D _ U A S | ct | CP-211112 | Sunghoon Kim, Qualcomm |
| 910069 | CT1 aspects for ID_UAS | I D _ U A S | C1 | CP-211112 | Sunghoon Kim, Qualcomm |
| 910070 | CT3 aspects for ID_UAS | I D _ U A S | C3 | CP-211112 | Sunghoon Kim, Qualcomm |
| 910071 | CT4 aspects for ID_UAS | I D _ U A S | C4 | CP-211112 | Sunghoon Kim, Qualcomm |
| 920028 | Security aspects of Uncrewed Aerial Systems | I D _ U A S | S3 | SP-210425 | Escott, Adrian, Qualcomm |
| 0 | Rel-17 Satellite/Non-Terrestrial (NTN) related | - |  | - | - |
| 890034 | Integration of satellite components in the 5G architecture | 5 G S A T _ A R C H |  | SP-191335 | Cyril MICHEL, thales |
| 800048 | Stage 1 of 5GSAT | 5 G S A T | S1 | SP-180326 | MICHEL, Cyril, Thales |
| 800026 | Study on architecture aspects for using satellite access in 5G | F S _ 5 G S A T _ A R C H | S2 | SP-181253 | MICHEL, Cyril |
| 860005 | (Stage 2 of) Integration of satellite components in the 5G architecture | 5 G S A T _ A R C H | S2 | SP-191335 | Cyril MICHEL, thales |
| 911030 | CT aspects of 5GC architecture for satellite networks | 5 G S A T _ A R C H - C T | ct | CP-210149 | Catovic, Amer, Qualcomm |
| 890005 | CT1 aspects of 5GC architecture for satellite networks | 5 G S A T _ A R C H - C T | C1 | CP-210149 | Catovic, Amer, Qualcomm |
| 920057 | CT3 aspects of 5GC architecture for satellite networks | 5 G S A T _ A R C H - C T | C3 | CP-210149 | Catovic, Amer, Qualcomm |
| 911031 | CT4 aspects of 5GC architecture for satellite networks | 5 G S A T _ A R C H - C T | C4 | CP-210149 | Catovic, Amer, Qualcomm |
| 930044 | CT6 aspects of 5GC architecture for satellite networks | 5 G S A T _ A R C H - C T | C6 | CP-210149 | Catovic, Amer, Qualcomm |
| 860046 | Solutions for NR to support non-terrestrial networks (NTN) | N R _ N T N _ s o l u t i o n s |  | RP-210908 | Thales |
| 860146 | Core part: Solutions for NR for NTN | N R _ N T N _ s o l u t i o n s - C o r e | R2 | RP-210908 | Thales |
| 860246 | Perf. Part: Solutions for NR for NTN | N R _ N T N _ s o l u t i o n s - P e r f | R4 | RP-210908 | Thales |
| 920069 | NB-IoT/eMTC support for Non-Terrestrial Networks | L T E _ N B I O T _ e M T C _ N T N | R1 | RP-211601 | MediaTek |
| 920169 | Core part: NB-IoT/eMTC support for Non-Terrestrial Networks | L T E _ N B I O T _ e M T C _ N T N - C o r e | R1 | RP-211601 | MediaTek |
| 930019 | Architecture support for NB-IoT/eMTC Non-Terrestrial Networks in EPS | I o T _ S A T _ A R C H _ E P S | S2 | SP-211124 | SEBIRE, Guillaume, MediaTek Inc. |
| 830025 | Study on management and orchestration aspects with integrated satellite components in a 5G network | F S _ 5 G S A T _ M O | S5 | SP-190138 | Floris Drijver, TNO |
| 860033 | Study on NB-IoT/eMTC support for NTN | F S _ L T E _ N B I O T _ e M T C _ N T N | R1 | RP-202689 | MediaTek |
| 0 | Rel-17 IoT related | - |  | - | - |
| 850012 | Study on enhanced support of Industrial IoT | F S _ I I o T | S2 | SP-200298 | Nokia, Devaki Chandramouli, |
| 880010 | Study on security for enhanced support of Industrial IoT | F S _ I I o T _ S E C | S3 | SP-200355 | Nokia, Anja Jerichow |
| 920024 | Security for enhanced support of Industrial IoT | I I o T _ S E C | S3 | SP-210421 | Nokia, Anja Jerichow |
| 910059 | Support of Enhanced Industrial IIoT | I I o T |  | SP-200973 | Devaki Chandramouli, Nokia |
| 900008 | Stage 2 for IIoT | I I o T | S2 | SP-200973 | Devaki Chandramouli, Nokia |
| 910014 | CT aspects of support of enhanced Industrial IoT | I I o T | ct | CP-212100 | Won, Sung Hwan, Nokia |
| 910060 | CT1 aspects of support of enhanced Industrial IoT | I I o T | C1 | CP-212100 | Won, Sung Hwan, Nokia |
| 910061 | CT3 aspects of support of enhanced Industrial IoT | I I o T | C3 | CP-212100 | Won, Sung Hwan, Nokia |
| 910062 | CT4 aspects of support of enhanced Industrial IoT | I I o T | C4 | CP-212100 | Won, Sung Hwan, Nokia |
| 860044 | Additional enhancements for NB-IoT and LTE-MTC | N B _ I O T e n h 4 _ L T E _ e M T C 6 |  | RP-211340 | Huawei |
| 860144 | Core part: Additional enhancements for NB-IoT and LTE-MTC | N B _ I O T e n h 4 _ L T E _ e M T C 6 - C o r e | R1 | RP-211340 | Huawei |
| 860244 | Perf. part: Additional enhancements for NB-IoT and LTE-MTC | N B _ I O T e n h 4 _ L T E _ e M T C 6 - P e r f | R4 | RP-211340 | Huawei |
| 860051 | NR small data transmissions in INACTIVE state | N R _ S m a l l D a t a _ I N A C T I V E |  | RP-212594 | ZTE |
| 860151 | Core part: NR small data transmissions in INACTIVE state | N R _ S m a l l D a t a _ I N A C T I V E - C o r e | R2 | RP-212594 | ZTE |
| 860251 | Perf. part: NR small data transmissions in INACTIVE state | N R _ S m a l l D a t a _ I N A C T I V E - P e r f | R4 | RP-212594 | ZTE |
| 860045 | Enhanced Industrial Internet of Things (IoT) and ultra-reliable and low latency communication (URLLC) support for NR | N R _ I I O T _ U R L L C _ e n h |  | RP-210854 | Nokia |
| 860145 | Core part: Enhanced IoT and URLLC support for NR | N R _ I I O T _ U R L L C _ e n h - C o r e | R2 | RP-210854 | Nokia |
| 860245 | Perf. Part: Enhanced IoT and URLLC support for NR | N R _ I I O T _ U R L L C _ e n h - P e r f | R4 | RP-210854 | Nokia |
| 0 | Rel-17 V2V/D2D/Sidelink related | - |  | - | - |
| 860042 | NR Sidelink enhancement | N R _ S L _ e n h |  | RP-202846 | LG Electronics |
| 860142 | Core part: NR Sidelink enhancement | N R _ S L _ e n h - C o r e | R1 | RP-202846 | LG Electronics |
| 860242 | Perf. part: NR Sidelink enhancement | N R _ S L _ e n h - P e r f | R4 | RP-202846 | LG Electronics |
| 860038 | Study on NR Sidelink relay | F S _ N R _ S L _ r e l a y | R2 | RP-202208 | OPPO |
| 840035 | Study on enhancements to application layer support for V2X services | F S _ e V 2 X A P P | S6 | SP-200110 | Niranth Amogh, Huawei Telecommunications India |
| 910075 | Enhanced application layer support for V2X services | e V 2 X A P P |  | SP-200831 | Niranth Amogh, Huawei Telecommunications India |
| 890036 | Stage 2 of eV2XAPP | e V 2 X A P P | S6 | SP-200831 | Niranth Amogh, Huawei Telecommunications India |
| 910019 | CT aspects of eV2XAPP | e V 2 X A P P | ct | CP-211109 | Herrero Veron, Christian (Huawei) |
| 910076 | CT1 aspects of eV2XAPP | e V 2 X A P P | C1 | CP-211109 | Herrero Veron, Christian (Huawei) |
| 910077 | CT3 aspects of eV2XAPP | e V 2 X A P P | C3 | CP-211109 | Herrero Veron, Christian (Huawei) |
| 910037 | Support of advanced V2X services - Phase 2 | e V 2 X A R C _ P h 2 |  | SP-210090 | LaeYoung Kim, LG Electronics |
| 850013 | Study on V2X services – Phase 2 | F S _ e V 2 X A R C _ P h 2 | S2 | SP-190631 | LaeYoung Kim, LG Electronics |
| 910021 | Stage 2 of eV2XARC_Ph2 | e V 2 X A R C _ P h 2 | S2 | SP-210090 | LaeYoung Kim, LG Electronics |
| 920005 | CT1 aspects of eV2XARC_Ph2 | e V 2 X A R C _ P h 2 | C1 | CP-211116 | Herrero Veron, Christian (Huawei) |
| 920047 | CT3 aspects of eV2XARC_Ph2 | e V 2 X A R C _ P h 2 | C3 | CP-211116 | Herrero Veron, Christian (Huawei) |
| 920048 | CT6 aspects of eV2XARC_Ph2 | e V 2 X A R C _ P h 2 | C6 | CP-211116 | Herrero Veron, Christian (Huawei) |
| 0 | Rel-17 NR enhancements | - |  | - | - |
| 920070 | UE RF requirements for Transparent Tx Diversity (TxD) for NR | N R _ R F _ T x D | R4 | RP-211940 | Qualcomm |
| 920170 | Core part: UE RF requirements for Transparent Tx Diversity (TxD) for NR | N R _ R F _ T x D - C o r e | R4 | RP-211940 | Qualcomm |
| 920270 | Perf. part: UE RF requirements for Transparent Tx Diversity (TxD) for NR | N R _ R F _ T x D - P e r f | R4 | RP-211940 | Qualcomm |
| 860040 | Further enhancements on MIMO for NR | N R _ f e M I M O |  | RP-212535 | Samsung |
| 860140 | Core part: Further enhancements on MIMO for NR | N R _ f e M I M O - C o r e | R1 | RP-212535 | Samsung |
| 860240 | Perf. part: Further enhancements on MIMO for NR | N R _ f e M I M O - P e r f | R4 | RP-212535 | Samsung |
| 860043 | NR Dynamic spectrum sharing (DSS) | N R _ D S S |  | RP-211345 | Ericsson |
| 860143 | Core part: NR DSS | N R _ D S S - C o r e | R1 | RP-211345 | Ericsson |
| 860047 | UE power saving enhancements for NR | N R _ U E _ p o w _ s a v _ e n h |  | RP-212630 | MediaTek |
| 860147 | Core part: UE power saving enhancements for NR | N R _ U E _ p o w _ s a v _ e n h - C o r e | R2 | RP-212630 | MediaTek |
| 860247 | Perf. part: UE power saving enhancements for NR | N R _ U E _ p o w _ s a v _ e n h - P e r f | R4 | RP-212630 | MediaTek |
| 860049 | Further Multi-RAT Dual-Connectivity enhancements | L T E _ N R _ D C _ e n h 2 |  | RP-201040 | Huawei |
| 860149 | Core part: Further Multi-RAT Dual-Connectivity enhancements | L T E _ N R _ D C _ e n h 2 - C o r e | R2 | RP-201040 | Huawei |
| 860249 | Perf. part: Further Multi-RAT Dual-Connectivity enhancements | L T E _ N R _ D C _ e n h 2 - P e r f | R4 | RP-201040 | Huawei |
| 860050 | Enhancements to Integrated Access and Backhaul (IAB) for NR | N R _ I A B _ e n h |  | RP-211548 | Qualcomm |
| 860150 | Core part: Enhancements to IAB for NR | N R _ I A B _ e n h - C o r e | R2 | RP-211548 | Qualcomm |
| 860250 | Perf. part: Enhancements to IAB for NR | N R _ I A B _ e n h - P e r f | R4 | RP-211548 | Qualcomm |
| 830021 | Study on Security for NR_IAB | F S _ N R _ I A B _ S e c | S3 | SP-201016 | Rajavelsamy Rajadurai, Samsung, |
| 860052 | Enhanced eNB(s) architecture evolution for E-UTRAN and NG-RAN | L T E _ N R _ a r c h _ e v o _ e n h |  | RP-211409 | China Unicom |
| 860152 | Core part: LTE_NR_arch_evo_enh | L T E _ N R _ a r c h _ e v o _ e n h - C o r e | R3 | RP-211409 | China Unicom |
| 890055 | Further enhancement on NR demodulation performance | N R _ d e m o d _ e n h 2 |  | RP-212636 | China Telecom |
| 890255 | Perf. part: NR_demod_enh2 | N R _ d e m o d _ e n h 2 - P e r f | R4 | RP-212636 | China Telecom |
| 890056 | Introduction of DL 1024QAM for NR frequency range 1 (FR1) | N R _ D L 1 0 2 4 Q A M _ F R 1 |  | RP-202886 | Ericsson |
| 890156 | Core part: Introduction of DL 1024QAM for NR FR1 | N R _ D L 1 0 2 4 Q A M _ F R 1 - C o r e | R4 | RP-202886 | Ericsson |
| 890256 | Perf. part: Introduction of DL 1024QAM for NR FR1 | N R _ D L 1 0 2 4 Q A M _ F R 1 - P e r f | R4 | RP-202886 | Ericsson |
| 890057 | NR RRM further enhancement | N R _ R R M _ e n h 2 |  | RP-202874 | Apple |
| 890157 | Core part: Further RRM enhancement for NR and MR-DC | N R _ R R M _ e n h 2 - C o r e | R4 | RP-202874 | Apple |
| 890257 | Perf. part: Further RRM enhancement for NR and MR-DC | N R _ R R M _ e n h 2 - P e r f | R4 | RP-202874 | Apple |
| 890058 | Enhanced NR support for high speed train scenario for frequency range 1 (FR1) | N R _ H S T _ F R 1 _ e n h |  | RP-210833 | CMCC |
| 890158 | Core part: Enhanced NR support for high speed train scenario for FR1 | N R _ H S T _ F R 1 _ e n h - C o r e | R4 | RP-210833 | CMCC |
| 890258 | Perf. part: Enhanced NR support for high speed train scenario for FR1 | N R _ H S T _ F R 1 _ e n h - P e r f | R4 | RP-210833 | CMCC |
| 890059 | Further enhancements of NR RF requirements for frequency range 2 (FR2) | N R _ R F _ F R 2 _ r e q _ e n h 2 |  | RP-212092 | Nokia |
| 890159 | Core part: NR_RF_FR2_req_enh2 | N R _ R F _ F R 2 _ r e q _ e n h 2 - C o r e | R4 | RP-212092 | Nokia |
| 890259 | Perf. part: NR_RF_FR2_req_enh2 | N R _ R F _ F R 2 _ r e q _ e n h 2 - P e r f | R4 | RP-212092 | Nokia |
| 890060 | NR support for high speed train scenario in frequency range 2 (FR2) | N R _ H S T _ F R 2 |  | RP-210800 | Samsung |
| 890160 | Core part: NR support for high speed train scenario in FR2 | N R _ H S T _ F R 2 - C o r e | R4 | RP-210800 | Samsung |
| 890260 | Perf. Part: NR support for high speed train scenario in FR2 | N R _ H S T _ F R 2 - P e r f | R4 | RP-210800 | Samsung |
| 890061 | NR measurement gap enhancements | N R _ M G _ e n h |  | RP-211591 | MediaTek |
| 890161 | Core part: NR and MR-DC measurement gap enhancements | N R _ M G _ e n h - C o r e | R4 | RP-211591 | MediaTek |
| 890261 | Perf. part: NR and MR-DC measurement gap enhancements | N R _ M G _ e n h - P e r f | R4 | RP-211591 | MediaTek |
| 890062 | RF requirements enhancement for NR frequency range 1 (FR1) | N R _ R F _ F R 1 _ e n h |  | RP-212527 | Huawei |
| 890162 | Core part: RF requirements enhancement for NR FR1 | N R _ R F _ F R 1 _ e n h - C o r e | R4 | RP-212527 | Huawei |
| 890262 | Perf. part: RF requirements enhancement for NR FR1 | N R _ R F _ F R 1 _ e n h - P e r f | R4 | RP-212527 | Huawei |
| 0 | Rel-17 Self-Organizing (SON)/Autonomous Networks | - |  | - | - |
| 860053 | Enhancement of data collection for SON (Self-Organising Networks)/MDT (Minimization of Drive Tests) in NR and EN-DC | N R _ E N D C _ S O N _ M D T _ e n h |  | RP-193255 | CMCC |
| 860153 | Core part: Enhancement of data collection for SON (Self-Organising Networks)/MDT (Minimization of Drive Tests) in NR and EN-DC | N R _ E N D C _ S O N _ M D T _ e n h - C o r e | R3 | RP-193255 | CMCC |
| 910029 | Plug and connect support for management of Network Functions | P A C M A N | S5 | SP-210260 | Per Elmdahl, Ericsson |
| 870028 | Enhancements of Self-Organizing Networks (SON) for 5G networks | e S O N _ 5 G | S5 | SP-200194 | Joey Chou, Intel |
| 850032 | Study on autonomous network levels | F S _ A N L | S5 | SP-190928 | China Mobile |
| 880027 | Autonomous network levels | A N L | S5 | SP-200464 | Cao, Xi, China Mobile |
| 0 | Rel-17 Multicast-broadcast Services (MBS) and Mission Critical (MC) | - |  | - | - |
| 900038 | Multicast-broadcast services in 5G | 5 M B S |  | SP-201106 | Meng Li, Huawei |
| 830030 | Study on Architectural enhancements for 5G multicast-broadcast services | F S _ 5 M B S | S2 | SP-200690 | Meng Li, Huawei |
| 900009 | Stage 2 for 5MBS | 5 M B S | S2 | SP-201106 | Meng Li, Huawei |
| 920043 | CT1 aspects of the architectural enhancements for 5G multicast-broadcast services | 5 M B S | C1 | CP-212022 | Gulbani, Giorgi, Huawei |
| 920044 | CT3 aspects of the architectural enhancements for 5G multicast-broadcast services | 5 M B S | C3 | CP-212022 | Gulbani, Giorgi, Huawei |
| 910002 | CT4 aspects of the architectural enhancements for 5G multicast-broadcast services | 5 M B S | C4 | CP-212022 | Gulbani, Giorgi, Huawei |
| 920023 | Security Aspects of Enhancements for 5G Multicast-Broadcast Services | 5 M B S | S3 | SP-210420 | Longhua Guo, Huawei Technologies |
| 870014 | Study on Multicast Architecture Enhancements for 5G Media Streaming | F S _ 5 G M S _ M u l t i c a s t | S4 | SP-200238 | Peng Tan, Telus |
| 810055 | Study on location enhancements for mission critical services | F S _ e n h M C L o c | S6 | SP-190725 | Dom Lazara, Motorola Solutions |
| 850040 | Broadcast / Multicast requirements supporting Mission Critical Services in 5G | 5 M B S _ e M C | S1 | SP-190942 | Toobe, Jens, BDBOS |
| 880006 | Study on Security Aspects of Enhancements for 5G Multicast-Broadcast Services | F S _ 5 M B S _ S E C | S3 | SP-200351 | Longhua Guo, Huawei Technologies |
| 850035 | Study on Mission Critical services over 5G multicast-broadcast system | F S _ M C 5 M B S | S6 | SP-190929 | Val Oprescu, AT&T |
| 860048 | NR multicast and broadcast services | N R _ M B S | R2 | RP-193248 | Huawei |
| 860148 | Core part: NR multicast and broadcast services | N R _ M B S - C o r e | R2 | RP-201038 | Huawei |
| 800023 | Study on Mission Critical services support over 5G System | F S _ M C O v e r 5 G S | S6 | SP-200837 | Verweij, Kees, The Police of the Netherlands |
| 920051 | Mission Critical Services over 5GS | M C O v e r 5 G S |  | SP-200833 | Wendler, Ingo, UIC |
| 890027 | Stage 2 of MCOver5GS | M C O v e r 5 G S | S6 | SP-200833 | Wendler, Ingo, UIC |
| 920007 | CT1 aspects of MCOver5GS | M C O v e r 5 G S | C1 | CP-211118 | Gkatzikis, Lazaros, Nokia |
| 890039 | Mission Critical Data | e M C D a t a 3 |  | SP-191106 | Shih, Jerry, AT&T |
| 860007 | Stage 2 of MCData3 | e M C D a t a 3 | S6 | SP-191106 | Shih, Jerry, AT&T |
| 890038 | CT aspects of eMCData3 | e M C D a t a 3 | C1 | CP-201177 |  |
| 880020 | Protocol enhancements for Mission Critical Services | M C P r o t o c 1 7 | C1 | CP-202193 | AXELL, Jörgen |
| 820040 | Mission Critical system migration and interconnection | M C S M I _ C T | C1 | CP-190143 | Dom Lazara, Motorola Solutions |
| 890003 | CT aspects of Enhanced Mission Critical Communication Interworking with Land Mobile Radio Systems | e M C C I _ C T | C1 | eMCCI_CT | Mike Dolan, FirstNet |
| 0 | Rel-17 Edge computing | - |  | - | - |
| 880042 | Architecture for enabling Edge Applications | E D G E A P P |  | SP-200109 | Gupta, Nishant, Samsung |
| 830008 | Study on Application Architecture for enabling Edge Applications | F S _ E D G E A P P | S6 | SP-190065 | Nishant Gupta, Samsung |
| 860006 | Architecture for enabling Edge Applications | E D G E A P P | S6 | SP-200886 | Gupta, Nishant, Samsung |
| 900006 | CT aspects for Enabling Edge Applications | E D G E A P P |  | CP-211196 | Narendranath Durga Tangudu |
| 900034 | CT1 aspects for Enabling Edge Applications | E D G E A P P | C1 | CP-211196 | Narendranath Durga Tangudu |
| 900035 | CT3 aspects for Enabling Edge Applications | E D G E A P P | C3 | CP-211196 | Narendranath Durga Tangudu |
| 830032 | Study on enhancement of support for Edge Computing in 5GC | F S _ e n h _ E C | S2 | SP-200093 | Hui Ni, Huawei |
| 910048 | Enhancement of support for Edge Computing in 5G Core network | e E D G E _ 5 G C |  | SP-201107 | Hui Ni, Huawei Technologies |
| 900016 | Stage 2 of eEDGE_5GC | e E D G E _ 5 G C | S2 | SP-201107 | Hui Ni, Huawei Technologies |
| 910005 | CT aspects of 5G eEDGE | e E D G E _ 5 G C | ct | CP-212021 | Qi Caixia, Huawei |
| 910049 | CT1 aspects of 5G eEDGE | e E D G E _ 5 G C | C1 | CP-212021 | Qi Caixia, Huawei |
| 910050 | CT3 aspects of 5G eEDGE | e E D G E _ 5 G C | C3 | CP-212021 | Qi Caixia, Huawei |
| 910051 | CT4 aspects of 5G eEDGE | e E D G E _ 5 G C | C4 | CP-212021 | Qi Caixia, Huawei |
| 920026 | Security Aspects of Enhancements of Support for Edge Computing in 5GC | e E D G E _ 5 G C | S3 | SP-210423 | Bo Zhang, Huawei Technologies |
| 920019 | Edge Computing Management | E C M | S5 | SP-210388 | Deepanshu Gautam, Samsung |
| 880002 | Study on Security Aspects of Enhancement of Support for Edge Computing in 5GC | F S _ e E D G E _ S e c | S3 | SP-200347 | Bo Zhang, Huawei Technologies |
| 930034 | Charging aspects of Edge Computing | E D G E _ C H | S5 | SP-210861 | Yao, Yizhi, Intel |
| 880030 | Study on charging aspects of Edge Computing | F S _ E D G E _ C H | S5 | SP-200467 | Yizhi Yao, Intel |
| 930011 | Charging aspects of Edge Computing | E D G E _ C H | S5 | SP-210861 | Yao, Yizhi, Intel |
| 870029 | Study on enhancements of edge computing management | F S _ e E D G E _ M g t | S5 | SP-200195 | Joey Chou, Intel |
| 0 | Rel-17 Slicing | - |  | - | - |
| 920058 | Rel-17 Enhancements of 3GPP Northbound Interfaces and Application Layer APIs | N B I 1 7 |  | CP-211197 | Yali Yan, Huawei |
| 920013 | CT1 aspects of NBI17 | N B I 1 7 | C1 | CP-211197 | Yali Yan, Huawei |
| 920053 | CT3 aspects of NBI17 | N B I 1 7 | C3 | CP-211197 | Yali Yan, Huawei |
| 920054 | CT4 aspects of NBI17 | N B I 1 7 | C4 | CP-211197 | Yali Yan, Huawei |
| 920012 | Enhancement of 5G PCC related services in Rel-17 | e n 5 G P c c S e r 1 7 | C3 | CP-211193 | Xiaoyun Zhou, Huawei |
| 900032 | Enhancement of Network Slicing Phase 2 | e N S _ P h 2 |  | SP-200976 | ZTE, Jinguo Zhu |
| 850010 | Study on Enhancement of Network Slicing Phase 2 | F S _ e N S _ P h 2 | S2 | SP-190931 | So, Tricci, ZTE |
| 900011 | Stage 2 for Enhancement of Network Slicing Phase 2 | e N S _ P h 2 | S2 | SP-200976 | ZTE, Jinguo Zhu |
| 920059 | Stage 3 for Enhancement of Network Slicing Phase 2 | e N S _ P h 2 | ct | CP-211091 | Hannah Wang, ZTE |
| 910041 | CT1 aspects of eNS_Ph2 | e N S _ P h 2 | C1 | CP-211091 | Hannah Wang, ZTE |
| 910042 | CT3 aspects of eNS_Ph2 | e N S _ P h 2 | C3 | CP-211091 | Hannah Wang, ZTE |
| 910043 | CT4 aspects of eNS_Ph2 | e N S _ P h 2 | C4 | CP-211091 | Hannah Wang, ZTE |
| 911007 | Enhancement of RAN slicing for NR | N R _ s l i c e | R2 | RP-212534 | CMCC |
| 911107 | Core part: Enhancement of RAN slicing for NR | N R _ s l i c e - C o r e | R2 | RP-212534 | CMCC |
| 860022 | Study on network slice management enhancement | F S _ N S M E N | S5 | SP-200766 | Brendan Hassett, Huawei Technologies Sweden AB, |
| 900023 | Study on Charging Aspects for Network Slicing Phase 2 | F S _ N E T S L I C E _ C H _ P h 2 | S5 | SP-201082 | Matrixx, Gerald Görmer |
| 910023 | Study on enhanced security for network slicing Phase 2 | F S _ e N S 2 _ S E C | S3 | SP-210106 | Zander Lei, Huawei |
| 910026 | Study on network slice management capability exposure | F S _ N S C E | S5 | SP-210131 | Xiaobo Yu, Alibaba Group |
| 910095 | Study on 5G NR UE full stack testing for Network Slicing | F S _ N R _ S l i c e _ T e s t | R5 | RP-211977 | CMCC |
| 0 | Rel-17 Multi-(U)SIM | - |  | - | - |
| 840049 | Support for Multi-USIM Devices | M U S I M |  | SP-190309 | Liao, Ellen C, Intel |
| 830019 | Study on MUSIM | F S _ M U S I M | S1 | SP-190091 | Liao, Ellen C, Intel |
| 840040 | Stage 1 of MUSIM | M U S I M | S1 | SP-190309 | Liao, Ellen C, Intel |
| 820012 | Study on Stage 2 (System Enablers) for MUSIM | F S _ M U S I M | S2 | SP-200297 | Sašo Stojanovski, Intel |
| 900013 | System enablers for Multi-USIM devices | M U S I M | S2 | SP-210091 | Sašo Stojanovski, Intel |
| 910015 | CT aspects of MUSIM | M U S I M | ct | CP-212102 | Vivek Gupta, Intel |
| 910063 | CT1 aspects of Enabling Multi-USIM Devices | M U S I M | C1 | CP-212102 | Vivek Gupta, Intel |
| 910064 | CT4 aspects of Enabling Multi-USIM Devices | M U S I M | C4 | CP-212102 | Vivek Gupta, Intel |
| 900017 | Study on the security of the system enablers for devices having multiple Universal Subscriber Identity Modules (USIM) | F S _ M U S I M _ S E C | S3 | SP-201018 | Abhijeet Kolekar, Intel Corporation |
| 860063 | Support for Multi-SIM devices for LTE/NR | L T E _ N R _ M U S I M |  | RP-212610 | Vivo |
| 860163 | Core part: Support for Multi-SIM devices for LTE/NR | L T E _ N R _ M U S I M - C o r e | R2 | RP-212610 | Vivo |
| 0 | Rel-17 other cross-TSG Features | - |  | - | - |
| 930030 | Support of the 5GMSG Service | F S _ 5 G M A R C H | S6 | SP-200835 | Liu, Yue, China Mobile |
| 840036 | Study on support of the 5GMSG Service | F S _ 5 G M A R C H | S6 | SP-200835 | Liu, Yue, China Mobile |
| 890010 | Study on security aspects of the 5GMSG Service (St1 in Rel16) | F S _ S E C _ 5 G M S G | S3 | SP-200878 | Xiaoting Huang, China Mobile |
| 930005 | Security aspects of the 5GMSG Service | 5 G M S G | S3 | SP-210835 | Xiaoting Huang, China Mobile |
| 930040 | Application Architecture for MSGin5G Service | 5 G M A R C H |  | SP-200832 | Liu, Yue, China Mobile |
| 890026 | Application Architecture for MSGin5G Service | 5 G M A R C H | S6 | SP-200832 | Liu, Yue, China Mobile |
| 930004 | CT aspects for enabling MSGin5G Service | 5 G M A R C H | ct | CP-212106 | Liu, Yue, China Mobile |
| 930041 | CT1 aspects for enabling MSGin5G Service | 5 G M A R C H | C1 | CP-212106 | Liu, Yue, China Mobile |
| 930042 | CT3 aspects for enabling MSGin5G Service | 5 G M A R C H | C3 | CP-212106 | Liu, Yue, China Mobile |
| 930002 | IMS voice service support and network usability guarantee for UE s E-UTRA capability disabled scenario in SA 5GS | I N G _ 5 G S | ct | CP-212231 | Mingxue Li, China Telecom |
| 930047 | CT1 aspects of ING_5GS | I N G _ 5 G S | C1 | CP-212231 | Mingxue Li, China Telecom |
| 930046 | CT6 aspects of ING_5GS | I N G _ 5 G S | C6 | CP-212231 | Mingxue Li, China Telecom |
| 930031 | User Consent for 3GPP services | F S _ U C 3 S | S3 | SP-200885 | Rong Wu, Huawei Technologies |
| 890037 | Study on User Consent for 3GPP services | F S _ U C 3 S | S3 | SP-200885 | Rong Wu, Huawei Technologies |
| 930006 | Security aspects on User Consent for 3GPP services | U C 3 S _ S E C | S3 | SP-210836 | Rong Wu, Huawei Technologies |
| 930032 | Improved support for NSA in the service-based management architecture | N S A _ S B M A | S5 | SP-211121 | Lan Zou, Huawei |
| 910031 | Study on Enhancement of service-based management architecture | F S _ e S B M A | S5 | SP-210136 | Lan Zou, Huawei |
| 930009 | Improved support for NSA in the service-based management architecture | N S A _ S B M A | S5 | SP-210858 | Lan Zou, Huawei |
| 930033 | Access control for management service | M S A C | S5 | SP-210859 | Jing Ping, Nokia |
| 890016 | Study on access control for management service | F S _ M N S A C | S5 | SP-200853 | Jing Ping, Nokia |
| 930010 | Access control for management service | M S A C | S5 | SP-210859 | Jing Ping, Nokia |
| 920060 | NPN support of PWS | N P N _ P W S | S1 | SP-210585 | Francesco Pica, Qualcomm |
| 910007 | Start of Pause of Charging via User Plane | S P O C U P | C4 | CP-210076 | YONG, YANG, Ericsson, frank |
| 910030 | File Management | F I M A | S5 | SP-210135 | Pollakowski, Olaf, Nokia |
| 910024 | Enhancements of 3GPP profiles for cryptographic algorithms and security protocols | e C r y p t P r | S3 | SP-210107 | Pinar Comak, Ericsson |
| 920027 | AKMA TLS protocol profiles | A K M A _ T L S | S3 | SP-210424 | Escott, Adrian, Qualcomm |
| 910025 | User Plane Integrity Protection for LTE | U P I P _ S E C _ L T E | S3 | SP-210105 | Evans, Tim, Vodafone |
| 900002 | Restoration of PDN Connections in PGW-C/SMF Set | R P C P S E T | C4 | CP-203020 | LANDAIS, Bruno, Nokia |
| 900037 | Enhancement of Inter-PLMN Roaming | E o I P R | C4 | CP-203021 | Caixia Qi, Huawei |
| 911037 | Reliable Data Service Serialization Indication | R D S S I |  | SP-190446 | Starsinic, Michael, Convida Wireless LLC |
| 840017 | Stage 2 of RDSSI | R D S S I | S2 | SP-190446 | Starsinic, Michael, Convida Wireless LLC |
| 900005 | CT aspects of RDSSI | R D S S I | C1 | CP-203234 | Vivek Gupta, Intel |
| 900021 | Management Aspects of 5G Network Sharing | M A N S | S5 | SP-201080 | Mingrui Sun, China Unicom |
| 920018 | Study on Management Aspects of 5G Network Sharing | F S _ M A N S | S5 | SP-210387 | Mingrui Sun, China Unicom |
| 900019 | Adapting BEST for use in 5G networks | B E S T _ 5 G | S3 | SP-201020 | Keesmaat, Iko, KPN |
| 920049 | Enhanced Service Enabler Architecture Layer for Verticals | e S E A L |  | SP-200987 | Basavaraj (Basu) Pattan, Samsung |
| 900024 | Stage 2 of eSEAL | e S E A L | S6 | SP-200987 | Basavaraj (Basu) Pattan, Samsung |
| 920006 | CT1 aspects of eSEAL | e S E A L | C1 | CP-212098 | Sapan Shah (Samsung Electronics) |
| 920050 | CT3 aspects of eSEAL | e S E A L | C3 | CP-212098 | Sapan Shah (Samsung Electronics) |
| 900033 | Access Traffic Steering, Switch and Splitting support in the 5G system architecture; Phase 2 | A T S S S _ P h 2 |  | SP-200977 | Apostolis Salkintzis, Lenovo |
| 840084 | Study on ATSSS_Ph2 | F S _ A T S S S _ P h 2 | S2 | SP-200095 | So, Tricci, ZTE |
| 900012 | Stage 2 of ATSSS_Ph2 | A T S S S _ P h 2 | S2 | SP-200977 | Apostolis Salkintzis, Lenovo |
| 910013 | CT aspects of ATSSS_Ph2 | A T S S S _ P h 2 | ct | CP-210136 | ZHOU Xingyue (Joy), ZTE |
| 910056 | CT1 aspects of ATSSS_Ph2 | A T S S S _ P h 2 | C1 | CP-210136 | ZHOU Xingyue (Joy), ZTE |
| 910057 | CT3 aspects of ATSSS_Ph2 | A T S S S _ P h 2 | C3 | CP-210136 | ZHOU Xingyue (Joy), ZTE |
| 910058 | CT4 aspects of ATSSS_Ph2 | A T S S S _ P h 2 | C4 | CP-210136 | ZHOU Xingyue (Joy), ZTE |
| 900031 | Enablers for Network Automation for 5G - phase 2 | e N A _ P h 2 |  | SP-200975 | TBD |
| 840022 | Study on Enablers for Network Automation for 5G - phase 2 | F S _ e N A _ P h 2 | S2 | SP-200098 | Xiaobo Wu, Huawei Technologies |
| 890015 | Study on security aspects of enablers for Network Automation (eNA) for 5GS Phase 2 | F S _ e N A _ S E C | S3 | SP-200722 | Xiaoting Huang, China Mobile |
| 900010 | Stage 2 of eNA_Ph2 | e N A _ P h 2 | S2 | SP-200975 | TBD |
| 910012 | CT aspects of eNA_Ph2 | e N A _ P h 2 | ct | CP-211191 | Huang Zhenning (China Mobile) |
| 910088 | CT3 aspects of eNA_Ph2 | e N A _ P h 2 | C3 | CP-211191 | Huang Zhenning (China Mobile) |
| 910089 | CT4 aspects of eNA_Ph2 | e N A _ P h 2 | C4 | CP-211191 | Huang Zhenning (China Mobile) |
| 930007 | Security aspects of eNA_Ph2 | e N A _ P h 2 | S3 | SP-210837 | Chang Liu, China Mobile |
| 900030 | Proximity based Services in 5GS | 5 G _ P r o S e |  | SP-200972 | TBD |
| 830033 | Study on System enhancement for Proximity based Services in 5GS | F S _ 5 G _ P r o S e | S2 | SP-190443 | Qiang Deng, CATT |
| 900007 | Stage 2 for Proximity based Services in 5GS | 5 G _ P r o S e | S2 | SP-201096 | Qiang Deng, CATT |
| 910018 | CT aspects of proximity based services in 5GS | 5 G _ P r o S e | ct | CP-212105 | Yong Jiang, CATT |
| 910072 | CT1 aspects of 5G_ProSe | 5 G _ P r o S e | C1 | CP-212105 | Yong Jiang, CATT |
| 910073 | CT3 aspects of 5G_ProSe | 5 G _ P r o S e | C3 | CP-212105 | Yong Jiang, CATT |
| 910074 | CT4 aspects of 5G_ProSe | 5 G _ P r o S e | C4 | CP-212105 | Yong Jiang, CATT |
| 920081 | CT6 aspects of 5G_ProSe | 5 G _ P r o S e | C6 | CP-212105 | Yong Jiang, CATT |
| 890018 | Study on charging aspects of Proximity-based Services in 5GS | F S _ 5 G _ P r o s e _ C H | S5 | SP-200767 | Shu, Min, CATT |
| 880005 | Study on Security Aspects of Enhancement for Proximity Based Services in 5GS | F S _ 5 G _ P r o S e _ S e c | S3 | SP-200350 | Wei Zhou, CATT |
| 930008 | Security Aspects of Proximity based Services in 5GS | 5 G _ P r o S e | S3 | SP-211120 | Wei Zhou, CATT |
| 911005 | NR Sidelink Relay | N R _ S L _ r e l a y |  | RP-212601 | OPPO |
| 911105 | Core part: NR Sidelink Relay | N R _ S L _ r e l a y - C o r e | R2 | RP-211050 | OPPO |
| 911205 | Perf. part: NR Sidelink Relay | N R _ S L _ r e l a y - P e r f | R4 | RP-211050 | OPPO |
| 890030 | Authentication and key management for applications based on 3GPP credential in 5G | A K M A |  | SP-190711 | Xiaoting Huang, China Mobile |
| 850021 | SA3 aspects of AKMA | A K M A | S3 | SP-190711 | Xiaoting Huang, China Mobile |
| 890008 | CT aspects of AKMA | A K M A - C T | ct | CP-203107 | Huang Zhenning (China Mobile) |
| 890031 | CT1 aspects of AKMA | A K M A - C T | C1 | CP-203107 | Huang Zhenning (China Mobile) |
| 890032 | CT3 aspects of AKMA | A K M A - C T | C3 | CP-203107 | Huang Zhenning (China Mobile) |
| 890033 | CT4 aspects of AKMA | A K M A - C T | C4 | CP-203107 | Huang Zhenning (China Mobile) |
| 890049 | Enhancement of Private Network support for NG-RAN | N G _ R A N _ P R N _ e n h | R3 | RP-212585 | China Telecom |
| 890149 | Core part: Enhancement of Private Network support for NG-RAN | N G _ R A N _ P R N _ e n h - C o r e | R3 | RP-212585 | China Telecom |
| 840046 | Multimedia Priority Service (MPS) Phase 2 | M P S 2 |  | SP-190305 | Singh, Ray P; Perspecta Labs |
| 780005 | Study on MPS2 | F S _ M P S 2 | S1 | SP-190315 | Singh, Ray P; Vencore Labs |
| 850011 | Study on MPS2, Stage 2 | F S _ M P S 2 _ S t 2 | S2 | SP-190629 | Don Lukacs; Perspecta Labs |
| 840032 | Stage 1 of MPS2 | M P S 2 | S1 | SP-190561 | Singh, Ray P; Perspecta Labs |
| 870002 | Stage 2 of MPS2 | M P S 2 | S2 | SP-200519 | Streijl, Robert, Perspecta Labs |
| 880031 | Stage 3 of MPS2 | M P S 2 | ct | CP-201207 | Peter Monnes, Perspecta Labs |
| 880050 | CT1 aspects of MPS2 | M P S 2 | C1 | CP-201207 | Peter Monnes, Perspecta Labs |
| 880058 | CT3 aspects of MPS2 | M P S 2 | C3 | CP-201207 | Peter Monnes, Perspecta Labs |
| 880051 | CT4 aspects of MPS2 | M P S 2 | C4 | CP-201207 | Peter Monnes, Perspecta Labs |
| 890011 | Mission critical security enhancements phase 2 | M C X S e c 2 | S3 | SP-200879 | Woodward, Tim, Motorola Solutions, Inc, |
| 890070 | PAP/CHAP protocols usage in 5GS | P A P _ C H A P | ct | CP-202251 | Mingxue Li, China Telecom |
| 890071 | CT1 aspects of PAP/CHAP protocols usage in 5GS | P A P _ C H A P | C1 | CP-202251 | Mingxue Li, China Telecom |
| 890072 | CT3 aspects of PAP/CHAP protocols usage in 5GS | P A P _ C H A P | C3 | CP-210188 | Mingxue Li, China Telecom |
| 890001 | Service-based support for SMS in 5GC | S M S _ S B I |  | CP-212023 | HAOUARI, Wafa, Orange |
| 890028 | CT1 aspects of SMS_SBI | S M S _ S B I | C1 | CP-212023 | HAOUARI, Wafa, Orange |
| 890029 | CT4 aspects of SMS_SBI | S M S _ S B I | C4 | CP-212023 | HAOUARI, Wafa, Orange |
| 840048 | Enhanced Relays for Energy eFficiency and Extensive Coverage | R E F E C |  | SP-190307 | Norp, Toon, KPN |
| 810018 | Study on REFEC | F S _ R E F E C | S1 | SP-180785 | Jose Luis Almodovar Chico, KPN |
| 840034 | Stage 1 of REFEC | R E F E C | S1 | SP-190307 | Norp, Toon, KPN |
| 840044 | Network Controlled Interactive Service | N C I S |  | SP-190303 | Ning YANG, oppo |
| 800015 | Study on NCIS | F S _ N C I S | S1 | SP-180341 | YANG Ning, Oppo |
| 840030 | Stage 1 of NCIS | N C I S | S1 | SP-191039 | Ning YANG, oppo |
| 840019 | 5G System Enhancement for Advanced Interactive Services | 5 G _ A I S | S2 | SP-190564 | Lei Yixue, Tencent |
| 890009 | Points for 8K VR 360 Video over 5G | 8 K _ V R _ 5 G | S4 | SP-200667 | Ozgur Oyman, Company: Intel |
| 880043 | Multi-Device and multi-identity Enhancements (MuDE) | M u D E |  | SP-190823 | Adrian.buckley@vivo.com |
| 850041 | Stage 1 of MuDE | M u D E | S1 | SP-190823 | Adrian.buckley@vivo.com |
| 880018 | Stage 3 of MuDE | M u D e | C1 | CP-201162 | Adrian Buckley |
| 920055 | Best Practice of Packet Forwarding Control Protocol (PFCP) | B E P o P | ct | CP-212024 | Song Yue, China Mobile |
| 920056 | CT3 aspects of BEPoP | B E P o P | C3 | CP-212024 | Song Yue, China Mobile |
| 880014 | CT4 aspects of BEPoP | B E P o P | C4 | CP-212024 | Song Yue, China Mobile |
| 880029 | Enhancement of Handover Optimization | E _ H O O | S5 | SP-200466 | Elmdahl, Per, Ericsson |
| 880028 | Management data collection control and discovery | M A D C O L | S5 | SP-200465 | Pollakowski, Olaf, Nokia |
| 880026 | Management of the enhanced tenant concept | e M E M T A N E | S5 | SP-200463 | Zhu, Lei, Huawei |
| 880025 | Enhancements of 5G performance measurements and KPIs | e P M _ K P I _ 5 G | S5 | SP-200462 | Yizhi Yao, Intel |
| 880012 | Extension for headset interface tests of UE | H I n T | S4 | SP-200398 | Jan Reimes, HEAD acoustics GmbH, |
| 840038 | MC services support on IOPS mode of operation | M C I O P S | S6 | SP-190944 | Camilo.solano@ericsson.com |
| 840016 | Supporting Flexible Local Area Data Network | F L A D N | S2 | SP-190563 | LG Electronics, Hyunsook Kim |
| 820003 | Support of Immersive Teleconferencing and Telepresence for Remote Terminals | I T T 4 R T | S4 | SP-180985 | Ozgur Oyman, Intel |
| 830005 | Terminal Audio quality performance and Test methods for Immersive Audio Services | A T I A S | S4 | SP-190040 | Stéphane Ragot, Orange |
| 860012 | Handsets Featuring Non-Traditional Earpieces | H a N T E | S4 | SP-190989 | Schevciw, Andre, Qualcomm |
| 920008 | 5GMS AF Event Exposure | E V E X | S4 | SP-210374 | Lo, Charles, Qualcomm Inc., |
| 920009 | Edge Extensions to the 5G Media Streaming Architecture | 5 G M S _ E D G E | S4 | SP-210375 | Bouazizi, Imed, Qualcomm Inc. |
| 920010 | 5G Multicast-Broadcast User Service Architecture and related 5GMS Extensions | 5 M B U S A | S4 | SP-210376 | TAN, PENG, TELUS |
| 920011 | 8K Television over 5G | 8 K _ T V _ 5 G | S4 | SP-210381 | Thomas Stockhammer |
| 830017 | Verification-modified Calling Name Display | V M O D _ D I S P L A Y | S1 | SP-190085 | Hala Mowafy, Ericsson |
| 850038 | Enhanced Calling Name Service Analytics Interworking | e C N A M _ A n | S1 | SP-190940 | Hala.Mowafy@ericsson.com |
| 880049 | Enhancement for the 5G Control Plane Steering of Roaming for UE in Connected mode | e C P S O R _ C O N |  | CP-210148 | Minokuchi, Atsushi, NTT DOCOMO |
| 850039 | Stage 1 of eCPSOR_CON | e C P S O R _ C O N | S1 | SP-190941 | Minokuchi, Atsushi, NTT DOCOMO |
| 880022 | Stage 3 (CT1) of eCPSOR_CON | e C P S O R _ C O N | C1 | CP-210148 | Al-Bakri Ban, NTT DOCOMO |
| 900036 | Stage 3 (CT4) of eCPSOR_CON | e C P S O R _ C O N | C4 | CP-210148 | Al-Bakri Ban, NTT DOCOMO |
| 920061 | Stage 3 (CT6) of eCPSOR_CON | e C P S O R _ C O N | C6 | CP-210148 | Al-Bakri Ban, NTT DOCOMO |
| 910090 | Integration of Generic Bootstrapping Architecture (GBA) into 5GC | G B A _ 5 G |  | SP-190714 | Vlasios Tsiatsis, Ericsson |
| 850023 | Security aspects of Integration of GBA into 5GC | G B A _ 5 G | S3 | SP-190714 | Vlasios Tsiatsis, Ericsson |
| 910004 | CT aspects of Integration of GBA into SBA | G B A _ 5 G | C4 | CP-210283 | de Gregorio, Jesús (Ericsson) |
| 910047 | Integration of Generic Bootstrapping Architecture (GBA) into 5GC | G B A _ 5 G | S3 | SP-190714 | Vlasios Tsiatsis, Ericsson |
| 770024 | EVS Codec Extension for Immersive Voice and Audio Services | I V A S _ C o d e c | S4 | SP-170611 | Bin Wang, Huawei Technologies Co Ltd |
| 850045 | Support for Minimization of service Interruption | M I N T |  | SP-190814 | SungDuck Chun; LG Electronics |
| 830018 | Study on MINT | F S _ M I N T | S1 | SP-190090 | SungDuck Chun; LG Electronics |
| 850036 | Stage 1 of MINT | M I N T | S1 | SP-190938 | SungDuck Chun; LG Electronics |
| 920062 | Stage 2 of MINT | M I N T | S2 | SP-210582 | Hyunsook Kim, LG Electronics |
| 900004 | Study on the CT aspects of Support for Minimization of service Interruption | F S _ M I N T - C T | C1 | CP-203232 | Sang Min Park, LG Electronics |
| 930003 | Stage 3 of MINT | M I N T | ct | CP-212166 | Sang Min Park |
| 930049 | CT1 aspects of MINT | M I N T | C1 | CP-212166 | Sang Min Park |
| 930045 | CT4 aspects of MINT | M I N T | C4 | CP-212166 | Sang Min Park |
| 930048 | CT6 aspects of MINT | M I N T | C6 | CP-212166 | Sang Min Park |
| 860008 | IMS emergency support for Stand-alone Non-Public Network (SNPN) | I E S N P N | S1 | SP-191038 | Wong, Curt; |
| 870023 | Management of non-public networks (NPN) | O A M _ N P N | S5 | SP-200189 | ZHANG, Kai, Huawei |
| 860016 | Assurance Specification for IMS | S C A S _ I M S | S3 | SP-191128 | Bo Zhang, Huawei Technologies |
| 870020 | Security Assurance Specification for 5G (eSCAS_5G) | e S C A S _ 5 G | S3 | SP-200149 | Rong Wu, Huawei Technologies Co. |
| 890013 | eSCAS_5G for Network Slice-Specific Authentication and Authorization Function (NSSAAF) | S C A S _ 5 G _ N S S A A F | S3 | SP-200720 | Rong Wu, Huawei Technologies Co., Ltd. |
| 870017 | eSCAS_5G for Non-3GPP InterWorking Function | S C A S _ 5 G _ N 3 I W F | S3 | SP-200146 | Feng Gao, China Unicom |
| 870018 | eSCAS_5G for 5G NWDAF | S C A S _ 5 G _ N W D A F | S3 | SP-200147 | QI Minpeng, China Mobile |
| 870019 | eSCAS_5G for Service Communication Proxy | S C A S _ 5 G _ S E C O P | S3 | SP-200148 | Wei Lu, Nokia |
| 880003 | eSCAS_5Gfor Inter PLMN UP Security | S C A S _ 5 G _ I P U P S | S3 | SP-200348 | Jin PENG, ZTE Corporation |
| 870022 | Enhancements on EE for 5G networks | E E 5 G P L U S | S5 | SP-200188 | Jean-Michel CORNILY, ORANGE |
| 870024 | Enhancement on Management Aspects of 5G Service-Level Agreement | E M A 5 S L A | S5 | SP-210860 | SHI, Xiaonan, CMCC |
| 870025 | Management of MDT enhancement in 5G | e _ 5 G M D T | S5 | SP-200191 | Ayani, Zhulia, Ericsson |
| 870026 | Additional NRM features | a d N R M | S5 | SP-200192 | Jing Ping, Nokia |
| 870027 | Enhancement of QoE Measurement Collection | e Q o E | S5 | SP-200193 | Petersen, Robert, Ericsson |
| 860061 | Study on NR QoE (Quality of Experience) management and optimizations for diverse services | F S _ N R _ Q o E | R3 | RP-193256 | China Unicom |
| 870030 | Enhanced Closed loop SLS Assurance | e C O S L A | S5 | SP-200196 | Groenendijk, Jan, Ericsson |
| 870034 | Enhanced Mission Critical Push-to-talk architecture phase 3 | e n h 3 M C P T T |  | SP-200108 | Dom Lazara; Motorola Solutions |
| 870016 | Stage 2 aspects of enh3MCPTT | e n h 3 M C P T T | S6 | SP-200108 | Dom Lazara; Motorola Solutions |
| 900039 | Stage 3 aspects of enh3MCPTT | e n h 3 M C P T T - C T | C1 | CP-202188 |  |
| 840028 | IMS Charging in 5G System Architecture | 5 G S I M S C H | S5 | SP-190367 | Jahangir, Zeeshan, T-Mobile USA |
| 810027 | Intent driven management service for mobile network | I D M S _ M N | S5 | SP-180899 | Zou Lan, Huawei |
| 820006 | Study on User Plane Integrity Protection | F S _ U P _ I P _ S e c | S3 | SP-181035 | Evans, Tim, Vodafone |
| 890012 | Enhancements to User Plane Integrity Protection Support in 5GS | e U P I P _ S E C | S3 | SP-200719 | Anand Palanigounder, Qualcomm |
| 820035 | Discovery of management services in 5G | 5 G D M S | S5 | SP-200770 | Brendan Hassett, Huawei Technologies Sweden AB |
| 860024 | Network policy managementfor 5G mobile networks | N P M | S5 | SP-191211 | China Mobile Shasha Guo |
| 860025 | Lawful Interception Rel-17 | L I 1 7 | S3 | SP-190983 | Alex Leadbeater, BT |
| 850047 | (Small) Technical Enhancements and Improvements for Rel-17 | T E I 1 7 |  |  |  |
| 870003 | IMS Optimization for HSS Group ID in an SBA environment | T E I 1 7 _ I M S G I D | S2 | SP-200452 | Foti, George, Ericsson |
| 910044 | Same PCF Selection For AMF and SMF | T E I 1 7 _ S P S F A S |  | SP-200447 | Heng, Nie, China Telecom |
| 880016 | Stage 2 for TEI17_SPSFAS | T E I 1 7 _ S P S F A S | S2 | SP-200447 | Heng, Nie, China Telecom |
| 910003 | CT aspects on TEI17_SPSFAS | T E I 1 7 _ S P S F A S | ct | CP-211184 | Yue Sun, China Telecom |
| 910045 | CT3 aspects on TEI17_SPSFAS | T E I 1 7 _ S P S F A S | C3 | CP-211184 | Yue Sun, China Telecom |
| 910046 | CT4 aspects on TEI17_SPSFAS | T E I 1 7 _ S P S F A S | C4 | CP-211184 | Yue Sun, China Telecom |
| 930036 | System enhancement for Redundant PDU Session | T E I 1 7 _ S E _ R P S |  | SP-200448 | Devaki Chandramouli |
| 880017 | Stage 2 for TEI17_SE_RPS | T E I 1 7 _ S E _ R P S | S2 | SP-200448 | Devaki Chandramouli |
| 930001 | CT aspects of TEI17_SE_RPS | T E I 1 7 _ S E _ R P S | ct | CP-212099 | WON, Sung Hwan, Nokia |
| 930037 | CT1 aspects of TEI17_SE_RPS | T E I 1 7 _ S E _ R P S | C1 | CP-212099 | WON, Sung Hwan, Nokia |
| 930038 | CT3 aspects of TEI17_SE_RPS | T E I 1 7 _ S E _ R P S | C3 | CP-212099 | WON, Sung Hwan, Nokia |
| 930039 | CT4 aspects of TEI17_SE_RPS | T E I 1 7 _ S E _ R P S | C4 | CP-212099 | WON, Sung Hwan, Nokia |
| 910078 | Support for Signed Attestation for Priority and Emergency Sessions | T E I 1 7 _ S A P E S |  | SP-200453 | Foti, George, Ericsson |
| 870004 | Stage 2 for TEI17_SAPES | T E I 1 7 _ S A P E S | S2 | SP-200453 | Foti, George, Ericsson |
| 910020 | CT aspects on TEI17_SAPES | T E I 1 7 _ S A P E S | ct | CP-210272 | Axell, Jörgen, Ericsson |
| 910079 | CT1 aspects on TEI17_SAPES | T E I 1 7 _ S A P E S | C1 | CP-210272 | Axell, Jörgen, Ericsson |
| 910080 | CT3 aspects on TEI17_SAPES | T E I 1 7 _ S A P E S | C3 | CP-210272 | Axell, Jörgen, Ericsson |
| 910081 | Dynamically Changing AM Policies in the 5GC | T E I 1 7 _ D C A M P |  | SP-200446 | Pancorbo, Belen (Ericsson) |
| 870005 | Stage 2 for TEI17_DCAMP | T E I 1 7 _ D C A M P | S2 | SP-200446 | Pancorbo, Belen (Ericsson) |
| 910009 | CT aspects on TEI17_DCAMP | T E I 1 7 _ D C A M P | ct | CP-212163 | Garcia Azorero, Fuencisla. Ericsson |
| 910082 | CT3 aspects on TEI17_DCAMP | T E I 1 7 _ D C A M P | C3 | CP-212163 | Garcia Azorero, Fuencisla. Ericsson |
| 910083 | CT4 aspects on TEI17_DCAMP | T E I 1 7 _ D C A M P | C4 | CP-212163 | Garcia Azorero, Fuencisla. Ericsson |
| 870006 | IP address pool information from UDM | T E I 1 7 _ I P U | S2 | SP-200454 | Stefan Rommer |
| 910084 | Dynamic management of group-based event monitoring | T E I 1 7 _ G E M |  | SP-200455 | Qian XB Chen, Ericsson |
| 870007 | Stage 2 for TEI17_GEM | T E I 1 7 _ G E M | S2 | SP-200455 | Qian XB Chen, Ericsson |
| 910010 | CT aspects on TEI17_GEM | T E I 1 7 _ G E M | ct | CP-212165 | Maria Tianmei, Liang, Ericsson |
| 910085 | CT3 aspects on TEI17_GEM | T E I 1 7 _ G E M | C3 | CP-212165 | Maria Tianmei, Liang, Ericsson |
| 910086 | CT4 aspects on TEI17_GEM | T E I 1 7 _ G E M | C4 | CP-212165 | Maria Tianmei, Liang, Ericsson |
| 920041 | Support of different slices over different Non 3GPP access | T E I 1 7 _ N 3 S L I C E |  | SP-200456 | Laurent Thiebaut, Nokia |
| 870008 | Stage 2 for TEI17_N3SLICE | T E I 1 7 _ N 3 S L I C E | S2 | SP-200456 | Laurent Thiebaut, Nokia |
| 930099 | CT1 aspects of TEI17_N3SLICE | T E I 1 7 _ N 3 S L I C E | C1 | CP-211089 | LANDAIS, Bruno, Nokia |
| 920001 | CT4 aspects of TEI17_N3SLICE | T E I 1 7 _ N 3 S L I C E | C4 | CP-211089 | LANDAIS, Bruno, Nokia |
| 910087 | N7/N40 Interfaces Enhancements to Support GERAN and UTRAN | T E I 1 7 _ N I E S G U |  | SP-200457 | Aihua, Li, China Mobile |
| 870009 | Stage 2 for TEI17_NIESGU | T E I 1 7 _ N I E S G U | S2 | SP-200457 | Aihua, Li, China Mobile |
| 890019 | SA5 aspects of N40 Interface Enhancements to Support GERAN and UTRAN | T E I 1 7 _ N I E S G U | S5 | SP-200854 | Dong, Jia, China Mobile |
| 910011 | CT3 aspects of N7 Interfaces Enhancements to Support GERAN and UTRAN | T E I 1 7 _ N I E S G U | C3 | CP-211194 | Huang Zhenning (China Mobile) |
| 920082 | CT4 aspects of N7 Interfaces Enhancements to Support GERAN and UTRAN | T E I 1 7 _ N I E S G U | C4 | CP-211194 | Huang Zhenning (China Mobile) |
| 880019 | Stage-3 5GS NAS protocol development 17 | 5 G P r o t o c 1 7 | C1 | CP-201163 | Sedlacek, Ivo, Ericsson |
| 880044 | Stage-3 5GS NAS protocol development 17 general aspects | 5 G P r o t o c 1 7 | C1 | CP-201163 | Sedlacek, Ivo, Ericsson |
| 880045 | Stage-3 5GS NAS protocol development 17 non-IETF aspects | 5 G P r o t o c 1 7 - n o n 3 G P P | C1 | CP-201163 | Sedlacek, Ivo, Ericsson |
| 880021 | Stage-3 SAE Protocol Development | S A E S 1 7 | C1 | CP-201165 | Aghili, Behrouz, InterDigital Communications |
| 880046 | Stage-3 SAE Protocol Development general aspects | S A E S 1 7 | C1 | CP-201165 | Aghili, Behrouz, InterDigital Communications |
| 880047 | Stage-3 SAE Protocol Development CSFB aspects | S A E S 1 7 - C S F B | C1 | CP-201165 | Aghili, Behrouz, InterDigital Communications |
| 880048 | Stage-3 SAE Protocol Development non 3GPP aspects | S A E S 1 7 - n o n 3 G P P | C1 | CP-201165 | Aghili, Behrouz, InterDigital Communications |
| 911034 | IMS Stage-3 IETF Protocol Alignment | I M S P r o t o c 1 7 | ct | CP-201167 | Gkatzikis, Lazaros(Nokia) |
| 880023 | CT1 aspects of IMS Stage-3 IETF Protocol Alignment | I M S P r o t o c 1 7 | C1 | CP-201167 | Gkatzikis, Lazaros(Nokia) |
| 911035 | CT3 aspects of IMS Stage-3 IETF Protocol Alignment | I M S P r o t o c 1 7 | C3 | CP-201167 | Gkatzikis, Lazaros(Nokia) |
| 880032 | Packet Flow Description (PFD) management enhancement | p f d M a n E n h | C3 | CP-210183 | Xiaoyun Zhou, Huawei |
| 880013 | Service Based Interface Protocol Improvements Release 17 | S B I P r o t o c 1 7 |  | CP-211088 | Song Yue, China Mobile |
| 880053 | CT3 aspects of SBIProtoc17 | S B I P r o t o c 1 7 | C3 | CP-211088 | Song Yue, China Mobile |
| 880054 | CT4 aspects of SBIProtoc17 | S B I P r o t o c 1 7 | C4 | CP-211088 | Song Yue, China Mobile |
| 850028 | Study on enhancement of Management Data Analytics Service | F S _ e M D A S | S5 | SP-190930 | Yao, Yizhi, Intel |
| 910027 | Enhancements of Management Data Analytics Service | e M D A S | S5 | SP-210132 | Yao, Yizhi, Intel |
| 0 | Rel-17 Other NR-only improvements | - |  | - | - |
| 911008 | NR QoE magement and optimizations for diverse services | N R _ Q o E |  | RP-211406 | China Unicom |
| 911108 | Core part: NR QoE management and optimizations for diverse services | N R _ Q o E - C o r e | R3 | RP-211406 | China Unicom |
| 911009 | Rel-17 Power Class 2 UE for NR inter-band CA with or without SUL configurations with x (16>=x>2) bands DL and y (y=1, 2) bands UL | N R _ U E _ P C 2 _ R 1 7 _ C A D C _ S U L _ x B D L _ y B U L |  | RP-212182 | Huawei |
| 911109 | Core part: NR_UE_PC2_R17_CADC_SUL_xBDL_yBUL | N R _ U E _ P C 2 _ R 1 7 _ C A D C _ S U L _ x B D L _ y B U L - C o r e | R4 | RP-212182 | Huawei |
| 911209 | Perf. part: NR_UE_PC2_R17_CADC_SUL_xBDL_yBUL | N R _ U E _ P C 2 _ R 1 7 _ C A D C _ S U L _ x B D L _ y B U L - P e r f | R4 | RP-212182 | Huawei |
| 911010 | Introduction of UE TRP (Total Radiated Power) and TRS (Total Radiated Sensitivity) requirements and test methodologies for FR1 (NR SA and EN-DC) | N R _ F R 1 _ T R P _ T R S |  | RP-212030 | Vivo |
| 911110 | Core part: NR_FR1_TRP_TRS | N R _ F R 1 _ T R P _ T R S - C o r e | R4 | RP-212030 | Vivo |
| 911210 | Perf. part: NR_FR1_TRP_TRS | N R _ F R 1 _ T R P _ T R S - P e r f | R4 | RP-212030 | Vivo |
| 911012 | High power UE (power class 1.5) for NR band n79 | N R _ U E _ P C 1 _ 5 _ n 7 9 |  | RP-210843 | CMCC |
| 911112 | Core part: NR_UE_PC1_5_n79 | N R _ U E _ P C 1 _ 5 _ n 7 9 - C o r e | R4 | RP-210843 | CMCC |
| 911212 | Perf. part: NR_UE_PC1_5_n79 | N R _ U E _ P C 1 _ 5 _ n 7 9 - P e r f | R4 | RP-210843 | CMCC |
| 930052 | UE Conformance - High power UE (power class 1.5) for NR band n79 | N R _ U E _ P C 1 _ 5 _ n 7 9 - U E C o n T e s t | R5 | RP-211979 | CMCC |
| 911013 | High power UE (power class 2) for NR band n34 | N R _ U E _ P C 2 _ n 3 4 |  | RP-210844 | CMCC |
| 911113 | Core part: NR_UE_PC2_n34 | N R _ U E _ P C 2 _ n 3 4 - C o r e | R4 | RP-210844 | CMCC |
| 911213 | Perf. part: NR_UE_PC2_n34 | N R _ U E _ P C 2 _ n 3 4 - P e r f | R4 | RP-210844 | CMCC |
| 930053 | UE Conformance - High power UE (power class 2) for NR band n34 | N R _ U E _ P C 2 _ n 3 4 - U E C o n T e s t | R5 | RP-211980 | CMCC |
| 911014 | High power UE (power class 2) for NR band n39 | N R _ U E _ P C 2 _ n 3 9 |  | RP-210845 | CMCC |
| 911114 | Core part: NR_UE_PC2_n39 | N R _ U E _ P C 2 _ n 3 9 - C o r e | R4 | RP-210845 | CMCC |
| 911214 | Perf. part: NR_UE_PC2_n39 | N R _ U E _ P C 2 _ n 3 9 - P e r f | R4 | RP-210845 | CMCC |
| 930054 | UE Conformance - High power UE (power class 2) for NR band n39 | N R _ U E _ P C 2 _ n 3 9 - U E C o n T e s t | R5 | RP-211981 | CMCC |
| 911015 | Introduction of FR2 FWA (Fixed Wireless Access) UE with maximum TRP of 23dBm for band n259 | N R _ F R 2 _ F W A _ B n 2 5 9 |  | RP-210875 | SoftBank Corp., KDDI, NTT DOCOMO, Rakuten Mobile, Qualcomm |
| 911115 | Core part: NR_FR2_FWA_Bn259 | N R _ F R 2 _ F W A _ B n 2 5 9 - C o r e | R4 | RP-210875 | SoftBank Corp., KDDI, NTT DOCOMO, Rakuten Mobile, Qualcomm |
| 911215 | Perf. part: NR_FR2_FWA_Bn259 | N R _ F R 2 _ F W A _ B n 2 5 9 - P e r f | R4 | RP-210875 | SoftBank Corp., KDDI, NTT DOCOMO, Rakuten Mobile, Qualcomm |
| 911006 | NR Uplink Data Compression (UDC) | N R _ U D C |  | RP-211203 | CATT |
| 911106 | Core part: NR_UDC | N R _ U D C - C o r e | R2 | RP-211203 | CATT |
| 910097 | Study on optimizations of pi/2 BPSK uplink power in NR | F S _ N R _ O p t _ p i 2 B P S K | R4 | RP-210910 | Huawei |
| 910096 | Study on band combination handling in RAN4 | F S _ N R _ E N D C _ c o m b o _ r u l e s | R4 | RP-211146 | ZTE |
| 880078 | Multiple Input Multiple Output (MIMO) Over-the-Air (OTA) requirements for NR UEs | N R _ M I M O _ O T A |  | RP-201800 | CAICT |
| 880178 | Core part: NR_MIMO_OTA | N R _ M I M O _ O T A - C o r e | R4 | RP-201800 | CAICT |
| 880278 | Perf. Part: NR_MIMO_OTA | N R _ M I M O _ O T A - P e r f | R4 | RP-201800 | CAICT |
| 870064 | Introduction of FR2 FWA (Fixed Wireless Access) UE with maximum TRP (Total Radiated Power) of 23dBm for band n257 and n258 | N R _ F R 2 _ F W A _ B n 2 5 7 _ B n 2 5 8 |  | RP-202565 | Softbank |
| 870164 | Core part: NR_FR2_FWA_Bn257_Bn258 | N R _ F R 2 _ F W A _ B n 2 5 7 _ B n 2 5 8 - C o r e | R4 | RP-202565 | Softbank |
| 870264 | Perf. Part: NR_FR2_FWA_Bn257_Bn258 | N R _ F R 2 _ F W A _ B n 2 5 7 _ B n 2 5 8 - P e r f | R4 | RP-202565 | Softbank |
| 880092 | Adding channel bandwidth support to existing NR bands | N R _ b a n d s _ R 1 7 _ B W s |  | RP-212531 | Ericsson |
| 880192 | Core part: NR_bands_R17_BWs | N R _ b a n d s _ R 1 7 _ B W s - C o r e | R4 | RP-212531 | Ericsson |
| 880093 | Introduction of channel bandwidths 35MHz and 45MHz for NR FR1 | N R _ F R 1 _ 3 5 M H z _ 4 5 M H z _ B W |  | RP-211386 | Huawei |
| 880193 | Core part: NR_FR1_35MHz_45MHz_BW | N R _ F R 1 _ 3 5 M H z _ 4 5 M H z _ B W - C o r e | R4 | RP-211386 | Huawei |
| 880293 | Perf. Part: NR_FR1_35MHz_45MHz_BW | N R _ F R 1 _ 3 5 M H z _ 4 5 M H z _ B W - P e r f | R4 | RP-211386 | Huawei |
| 880097 | SAR schemes for UE power class 2 (PC2) for NR inter-band Carrier Aggregation and supplemental uplink (SUL) configurations with 2 bands UL | N R _ S A R _ P C 2 _ i n t e r B _ S U L _ 2 B U L |  | RP-212530 | China Telecom |
| 880197 | Core part: NR_SAR_PC2_interB_SUL_2BUL | N R _ S A R _ P C 2 _ i n t e r B _ S U L _ 2 B U L - C o r e | R4 | RP-212530 | China Telecom |
| 880297 | Perf. Part: NR_SAR_PC2_interB_SUL_2BUL | N R _ S A R _ P C 2 _ i n t e r B _ S U L _ 2 B U L - P e r f | R4 | RP-212530 | China Telecom |
| 920065 | UE Conformance - SAR schemes for UE power class 2 (PC2) for NR inter-band Carrier Aggregation and supplemental uplink (SUL) configurations with 2 bands UL | N R _ S A R _ P C 2 _ i n t e r B _ S U L _ 2 B U L - U E C o n T e s t | R5 | RP-211139 | China Telecom |
| 900061 | NR coverage enhancements | N R _ c o v _ e n h |  | RP-211566 | China Telecom |
| 860036 | Study on on NR coverage enhancements | F S _ N R _ c o v _ e n h | R1 | RP-200861 | China Telecom |
| 900161 | Core part: NR_cov_enh | N R _ c o v _ e n h - C o r e | R1 | RP-211566 | China Telecom |
| 900261 | Perf. part: NR_cov_enh | N R _ c o v _ e n h - P e r f | R4 | RP-211566 | China Telecom |
| 900062 | Support of reduced capability NR devices | N R _ r e d c a p |  | RP-211574 | Ericsson |
| 860035 | Study on support of reduced capability NR devices | F S _ N R _ r e d c a p | R1 | RP-202704 | Ericsson |
| 900162 | Core part: NR_redcap | N R _ r e d c a p - C o r e | R1 | RP-211574 | Ericsson |
| 900262 | Perf. part: NR_redcap | N R _ r e d c a p - P e r f | R4 | RP-211574 | Ericsson |
| 930018 | Architecture Enhancement for NR Reduced Capability Devices | A R C H _ N R _ R E D C A P | S2 | SP-211100 | Aihua Li, China Mobile |
| 900064 | Additional NR bands for UL-MIMO power class 3 (PC3) in Rel-17 | N R _ b a n d s _ U L _ M I M O _ P C 3 _ R 1 7 |  | RP-212184 | Huawei |
| 900164 | Core part: NR_bands_UL_MIMO_PC3_R17 | N R _ b a n d s _ U L _ M I M O _ P C 3 _ R 1 7 - C o r e | R4 | RP-212184 | Huawei |
| 900264 | Perf. part: NR_bands_UL_MIMO_PC3_R17 | N R _ b a n d s _ U L _ M I M O _ P C 3 _ R 1 7 - P e r f | R4 | RP-212184 | Huawei |
| 900167 | Introduction of bandwidth combination set 4 (BCS4) for NR | N R _ B C S 4 - C o r e | R4 | RP-202832 | Ericsson |
| 900068 | Downlink interruption for NR and EN-DC band combinations to conduct dynamic Tx Switching in Uplink | D L _ i n t r p t _ c o m b o s _ T x S W _ R 1 7 |  | RP-210478 | China Telecom |
| 900168 | Core part: DL_intrpt_combos_TxSW_R17 | D L _ i n t r p t _ c o m b o s _ T x S W _ R 1 7 - C o r e | R4 | RP-202885RP-210478 | China Telecom |
| 900268 | Perf. part: DL_intrpt_combos_TxSW_R17 | D L _ i n t r p t _ c o m b o s _ T x S W _ R 1 7 - P e r f | R4 | RP-202885RP-210478 | China Telecom |
| 900069 | High-power UE (power class 1.5) operation in NR bands n77 and n78 | H P U E _ P C 1 _ 5 _ n 7 7 _ n 7 8 |  | RP-202912 | Qualcomm |
| 900169 | Core part: HPUE_PC1_5_n77_n78 | H P U E _ P C 1 _ 5 _ n 7 7 _ n 7 8 - C o r e | R4 | RP-202912 | Qualcomm |
| 930055 | UE Conformance - High-power UE (power class 1.5) operation in NR bands n77 and n78 | H P U E _ P C 1 _ 5 _ n 7 7 _ n 7 8 - U E C o n T e s t | R5 | RP-212581 | Verizon |
| 900070 | NR repeaters | N R _ r e p e a t e r s |  | RP-212129 | Qualcomm |
| 900170 | Core part: HPUE_PC1_5_n77_n78 | N R _ r e p e a t e r s - C o r e | R4 | RP-212129 | Qualcomm |
| 930250 | Perf. part: NR repeaters | N R _ r e p e a t e r s - P e r f | R4 | RP-212129 | Qualcomm |
| 930056 | Increasing UE power high limit for CA and DC | P o w e r _ L i m i t _ C A _ D C | R4 | RP-212622 | Qualcomm |
| 930156 | Core part: Increasing UE power high limit for CA and DC | P o w e r _ L i m i t _ C A _ D C - C o r e | R4 | RP-212622 | Qualcomm |
| 930057 | Introduction of upper 700MHz A block E-UTRA band for the US | L T E _ u p p e r _ 7 0 0 M H z _ A | R4 | RP-212618 | Puloli |
| 930157 | Core part: Introduction of upper 700MHz A block E-UTRA band for the US | L T E _ u p p e r _ 7 0 0 M H z _ A - C o r e | R4 | RP-212618 | Puloli |
| 930257 | Perf. part: Introduction of upper 700MHz A block E-UTRA band for the US | L T E _ u p p e r _ 7 0 0 M H z _ A - P e r f | R4 | RP-212618 | Puloli |
| 930058 | High power UE (power class 2) for one NR FDD band | N R _ P C 2 _ U E _ F D D | R4 | RP-212633 | China Unicom |
| 930158 | Core part: High power UE (power class 2) for one NR FDD band | N R _ P C 2 _ U E _ F D D - C o r e | R4 | RP-212633 | China Unicom |
| 930258 | Perf. part: High power UE (power class 2) for one NR FDD band | N R _ P C 2 _ U E _ F D D - P e r f | R4 | RP-212633 | China Unicom |
| 0 | Rel-17 NR Carrier Aggregation | - |  | - | - |
| 0 | Rel-17 NR intra band Carrier Aggregation (itrabCA) | - |  | - | - |
| 881005 | NR itrabCA for xCC DL/yCC UL including contiguous and non-contiguous spectrum (x>=y) | N R _ C A _ R 1 7 _ I n t r a |  | RP-211757 | Ericsson |
| 881105 | Core part: NR_CA_R17_Intra | N R _ C A _ R 1 7 _ I n t r a - C o r e | R4 | RP-211757 | Ericsson |
| 881205 | Perf. Part: NR_CA_R17_Intra | N R _ C A _ R 1 7 _ I n t r a - P e r f | R4 | RP-211757 | Ericsson |
| 0 | Rel-17 NR inter band Carrier Aggregation (iterbCA) | - |  | - | - |
| 900063 | High power UE for NR TDD intra-band Carrier Aggregation in frequency range FR1 | N R _ i n t r a _ H P U E _ R 1 7 |  | RP-212180 | Huawei |
| 900163 | Core part: NR_intra_HPUE_R17 | N R _ i n t r a _ H P U E _ R 1 7 - C o r e | R4 | RP-212180 | Huawei |
| 900263 | Perf. Part: NR_intra_HPUE_R17 | N R _ i n t r a _ H P U E _ R 1 7 - P e r f | R4 | RP-212180 | Huawei |
| 881006 | NR iterbCA/Dual Connectivity for 2 bands DL with x bands UL (x=1,2) | N R _ C A D C _ R 1 7 _ 2 B D L _ x B U L |  | RP-212511 | ZTE |
| 881106 | Core part: NR_CADC_R17_2BDL_xBUL | N R _ C A D C _ R 1 7 _ 2 B D L _ x B U L - C o r e | R4 | RP-212511 | ZTE |
| 881206 | Perf. part: NR_CADC_R17_2BDL_xBUL | N R _ C A D C _ R 1 7 _ 2 B D L _ x B U L - P e r f | R4 | RP-212511 | ZTE |
| 881007 | NR iterbCA for 3 bands DL with 1 band UL | N R _ C A _ R 1 7 _ 3 B D L _ 1 B U L |  | RP-212239 | CATT |
| 881107 | Core part: NR_CA_R17_3BDL_1BUL | N R _ C A _ R 1 7 _ 3 B D L _ 1 B U L - C o r e | R4 | RP-212239 | CATT |
| 881207 | Perf. part: NR_CA_R17_3BDL_1BUL | N R _ C A _ R 1 7 _ 3 B D L _ 1 B U L - P e r f | R4 | RP-212239 | CATT |
| 881008 | NR iterbCA/Dual Connectivity for 3 bands DL with 2 bands UL | N R _ C A D C _ R 1 7 _ 3 B D L _ 2 B U L |  | RP-212512 | ZTE |
| 881108 | Core part: NR_CADC_R17_3BDL_2BUL | N R _ C A D C _ R 1 7 _ 3 B D L _ 2 B U L - C o r e | R4 | RP-212512 | ZTE |
| 881208 | Perf. part: NR_CADC_R17_3BDL_2BUL | N R _ C A D C _ R 1 7 _ 3 B D L _ 2 B U L - P e r f | R4 | RP-212512 | ZTE |
| 881009 | NR iterbCA for 4 bands DL with 1 band UL | N R _ C A _ R 1 7 _ 4 B D L _ 1 B U L |  | RP-211759 | Ericsson |
| 881109 | Core part: NR_CA_R17_4BDL_1BUL | N R _ C A _ R 1 7 _ 4 B D L _ 1 B U L - C o r e | R4 | RP-211759 | Ericsson |
| 881209 | Perf. part: NR_CA_R17_4BDL_1BUL | N R _ C A _ R 1 7 _ 4 B D L _ 1 B U L - P e r f | R4 | RP-211759 | Ericsson |
| 881010 | NR iterbCA/Dual connectivity for DL 4 bands and 2UL bands | N R _ C A D C _ R 1 7 _ 4 B D L _ 2 B U L |  | RP-212110 | Samsung |
| 881110 | Core part: NR_CADC_R17_4BDL_2BUL | N R _ C A D C _ R 1 7 _ 4 B D L _ 2 B U L - C o r e | R4 | RP-212110 | Samsung |
| 881210 | Perf. part: NR_CADC_R17_4BDL_2BUL | N R _ C A D C _ R 1 7 _ 4 B D L _ 2 B U L - P e r f | R4 | RP-212110 | Samsung |
| 881011 | NR iterbCA for 5 bands DL with x bands UL (x=1, 2) | N R _ C A D C _ R 1 7 _ 5 B D L _ x B U L |  | RP-212176 | Huawei |
| 881111 | Core part: NR_CADC_R17_5BDL_xBUL | N R _ C A D C _ R 1 7 _ 5 B D L _ x B U L - C o r e | R4 | RP-212176 | Huawei |
| 881211 | Perf. part: NR_CADC_R17_5BDL_xBUL | N R _ C A D C _ R 1 7 _ 5 B D L _ x B U L - P e r f | R4 | RP-212176 | Huawei |
| 890054 | Rel-17 High power UE for NR inter-band Carrier Aggregation with 2 bands downlink and x bands uplink (x=1,2) | N R _ P C 2 _ C A _ R 1 7 _ 2 B D L _ 2 B U L |  | RP-211831 | China Telecom |
| 890154 | Core part: NR_PC2_CA_R17_2BDL_2BUL | N R _ P C 2 _ C A _ R 1 7 _ 2 B D L _ 2 B U L - C o r e | R4 | RP-211831 | China Telecom |
| 890254 | Perf. part: NR_PC2_CA_R17_2BDL_2BUL | N R _ P C 2 _ C A _ R 1 7 _ 2 B D L _ 2 B U L - P e r f | R4 | RP-211831 | China Telecom |
| 920066 | UE Conformance - Rel-17 High power UE for NR inter-band Carrier Aggregation with 2 bands downlink and x bands uplink (x=1,2) | N R _ P C 2 _ C A _ R 1 7 _ 2 B D L _ 2 B U L - U E C o n T e s t | R5 | RP-211140 | China Telecom |
| 0 | Rel-17 NR and LTE | - |  | - | - |
| 920072 | LTE/NR spectrum sharing in Band 34/n34 and Band 39/n39 | D S S _ L T E _ B 3 4 _ N R _ B n 3 4 _ L T E _ B 3 9 _ N R _ B n 3 9 | R4 |  | CMCC |
| 920172 | Core part: LTE/NR spectrum sharing in Band 34/n34 and Band 39/n39 | D S S _ L T E _ B 3 4 _ N R _ B n 3 4 _ L T E _ B 3 9 _ N R _ B n 3 9 - C o r e | R4 | RP-211996 | CMCC |
| 920073 | High-power UE operation for fixed-wireless/vehicle-mounted use cases in LTE bands and NR bands | L T E _ N R _ H P U E _ F W V M | R4 | RP-212533 | Nokia |
| 920173 | Core part: High-power UE operation for fixed-wireless/vehicle-mounted use cases in LTE bands and NR bands | L T E _ N R _ H P U E _ F W V M - C o r e | R4 | RP-212533 | Nokia |
| 920273 | Perf. part: High-power UE operation for fixed-wireless/vehicle-mounted use cases in LTE bands and NR bands | L T E _ N R _ H P U E _ F W V M - P e r f | R4 | RP-212533 | Nokia |
| 920074 | Rel-17 Dual Connectivity (DC) of x bands (x=1,2) LTE inter-band CA (xDL1UL) and 4 bands NR inter-band CA (4DL1UL) | D C _ R 1 7 _ x B L T E _ 4 B N R _ y D L 2 U L | R4 | RP-212606 | Huawei |
| 920174 | Core part: Rel-17 Dual Connectivity (DC) of x bands (x=1,2) LTE inter-band CA (xDL1UL) and 4 bands NR inter-band CA (4DL1UL) | D C _ R 1 7 _ x B L T E _ 4 B N R _ y D L 2 U L - C o r e | R4 | RP-212606 | Huawei |
| 920274 | Perf. part: Rel-17 Dual Connectivity (DC) of x bands (x=1,2) LTE inter-band CA (xDL1UL) and 4 bands NR inter-band CA (4DL1UL) | D C _ R 1 7 _ x B L T E _ 4 B N R _ y D L 2 U L - P e r f | R4 | RP-212606 | Huawei |
| 911019 | Addition of MSD (Maximum Sensitivity Degradation) for inter-band EN-DC combinations (1 band LTE+1 band NR FR1) due to added channel bandwidths | E N D C _ R 1 7 _ 1 B L T E _ 1 B N R _ M S |  | RP-212187 | Huawei, HiSilicon |
| 911119 | Core part: ENDC_R17_1BLTE_1BNR_MS | E N D C _ R 1 7 _ 1 B L T E _ 1 B N R _ M S D - C o r e |  | RP-212187 | Huawei, HiSilicon |
| 911011 | Rel-17 Power Class 2 for EN-DC with x LTE bands + y NR band(s) in DL and with 1 LTE band +1 TDD NR band in UL (either x= 2, 3, y=1 or x=1, 2, y=2) | E N D C _ P C 2 _ R 1 7 _ x L T E _ y N R |  | RP-211760 | Ericsson |
| 911111 | Core part: ENDC_PC2_R17_xLTE_yNR | E N D C _ P C 2 _ R 1 7 _ x L T E _ y N R - C o r e | R4 | RP-211760 | Ericsson |
| 911211 | Perf. Part: ENDC_PC2_R17_xLTE_yNR | E N D C _ P C 2 _ R 1 7 _ x L T E _ y N R - P e r f | R4 | RP-211760 | Ericsson |
| 930051 | UE Conformance - Power Class 2 for EN-DC with x LTE bands + y NR band(s) in DL and with 1 LTE band +1 TDD NR band in UL (either x= 2, 3, y=1 or x=1, 2, y=2) | E N D C _ P C 2 _ R 1 7 _ x L T E _ y N R - U E C o n T e s t | R5 | RP-211870 | Ericsson |
| 911018 | Simultaneous Rx/Tx band combinations for NR CA/DC, NR SUL and LTE/NR DC | L T E _ N R _ S i m u l t _ R x T x |  | RP-211382 | Huawei |
| 911118 | Core part: LTE_NR_Simult_RxTx | L T E _ N R _ S i m u l t _ R x T x - C o r e | R4 | RP-211382 | Huawei |
| 911218 | Perf. part: LTE_NR_Simult_RxTx | L T E _ N R _ S i m u l t _ R x T x - P e r f | R4 | RP-211382 | Huawei |
| 890052 | Rel-17 Dual Connectivity (DC) of 5 bands LTE inter-band CA (5DL/1UL) and 1 NR band (1DL/1UL) | D C _ R 1 7 _ 5 B L T E _ 1 B N R _ 6 D L 2 U L |  | RP-211195 | Samsung |
| 890152 | Core part: DC_R17_5BLTE_1BNR_6DL2UL | D C _ R 1 7 _ 5 B L T E _ 1 B N R _ 6 D L 2 U L - C o r e | R4 | RP-211195 | Samsung |
| 890252 | Perf. part: DC_R17_5BLTE_1BNR_6DL2UL | D C _ R 1 7 _ 5 B L T E _ 1 B N R _ 6 D L 2 U L - P e r f | R4 | RP-211195 | Samsung |
| 890053 | Rel-17 Dual Connectivity (DC) of x bands (x=2, 3, 4) LTE inter-band CA (xDL/1UL) and 1 NR FR1 band (1DL/1UL) and 1 NR FR2 band (1DL/1UL) | D C _ R 1 7 _ x B L T E _ 2 B N R _ y D L 3 U L |  | RP-202542 | Samsung |
| 890153 | Core part: DC_R17_xBLTE_2BNR_yDL3UL | D C _ R 1 7 _ x B L T E _ 2 B N R _ y D L 3 U L - C o r e | R4 | RP-202542 | Samsung |
| 890253 | Perf. part: DC_R17_xBLTE_2BNR_yDL3UL | D C _ R 1 7 _ x B L T E _ 2 B N R _ y D L 3 U L - P e r f | R4 | RP-202542 | Samsung |
| 880080 | Band combinations for concurrent operation of NR/LTE Uu bands/band combinations and one NR/LTE V2X PC5 band | N R _ L T E _ V 2 X _ P C 5 _ c o m b o s |  | RP-210588 | CATT |
| 880180 | Core part: NR_LTE_V2X_PC5_combos | N R _ L T E _ V 2 X _ P C 5 _ c o m b o s - C o r e | R4 | RP-210588 | CATT |
| 880280 | Perf. part: NR_LTE_V2X_PC5_combos | N R _ L T E _ V 2 X _ P C 5 _ c o m b o s - P e r f | R4 | RP-210588 | CATT |
| 880094 | High power UE (power class 2) for EN-DC with 1 LTE band + 1 NR TDD band | E N D C _ U E _ P C 2 _ R 1 7 _ N R _ T D D |  | RP-211853 | China Unicom |
| 880194 | Core part: ENDC_UE_PC2_R17_NR_TDD | E N D C _ U E _ P C 2 _ R 1 7 _ N R _ T D D - C o r e | R4 | RP-211853 | China Unicom |
| 880294 | Perf. part: ENDC_UE_PC2_R17_NR_TDD | E N D C _ U E _ P C 2 _ R 1 7 _ N R _ T D D - P e r f | R4 | RP-211853 | China Unicom |
| 911000 | UE Conformance - ENDC_UE_PC2_R17_NR_TDD | E N D C _ U E _ P C 2 _ R 1 7 _ N R _ T D D - U E C o n T e s t | R5 | RP-211420 | China Unicom |
| 880095 | LTE/NR spectrum sharing in Band 38/n38 | D S S _ L T E _ B 3 8 _ N R _ B n 3 8 |  | RP-201314 | Vodafone |
| 880195 | Core part: DSS_LTE_B38_NR_Bn38 | D S S _ L T E _ B 3 8 _ N R _ B n 3 8 - C o r e | R4 | RP-201314 | Vodafone |
| 880096 | LTE/NR spectrum sharing in Band 40/n40 | D S S _ L T E _ B 4 0 _ N R _ B n 4 0 |  | RP-202084 | Reliance Jio |
| 880196 | Core part: DSS_LTE_B40_NR_Bn40 | D S S _ L T E _ B 4 0 _ N R _ B n 4 0 - C o r e | R4 | RP-202084 | Reliance Jio |
| 880296 | Perf. part: DSS_LTE_B40_NR_Bn40 | D S S _ L T E _ B 4 0 _ N R _ B n 4 0 - P e r f | R4 | RP-202084 | Reliance Jio |
| 880098 | Dual Connectivity (DC) of 1 LTE band (1DL/1UL) and 1 NR band (1DL/1UL) | D C _ R 1 7 _ 1 B L T E _ 1 B N R _ 2 D L 2 U L |  | RP-212096 | CHTTL |
| 880198 | Core part: DC_R17_1BLTE_1BNR_2DL2UL | D C _ R 1 7 _ 1 B L T E _ 1 B N R _ 2 D L 2 U L - C o r e | R4 | RP-212096 | CHTTL |
| 880298 | Perf. part: DC_R17_1BLTE_1BNR_2DL2UL | D C _ R 1 7 _ 1 B L T E _ 1 B N R _ 2 D L 2 U L - P e r f | R4 | RP-212096 | CHTTL |
| 880099 | Dual Connectivity (DC) of 2 bands LTE inter-band CA (2DL/1UL) and 1 NR band (1DL/1UL) | D C _ R 1 7 _ 2 B L T E _ 1 B N R _ 3 D L 2 U L |  | RP-212174 | Huawei |
| 880199 | Core part: DC_R17_2BLTE_1BNR_3DL2UL | D C _ R 1 7 _ 2 B L T E _ 1 B N R _ 3 D L 2 U L - C o r e | R4 | RP-212174 | Huawei |
| 890263 | Perf. part: DC_R17_2BLTE_1BNR_3DL2UL | D C _ R 1 7 _ 2 B L T E _ 1 B N R _ 3 D L 2 U L - P e r f | R4 | RP-212174 | Huawei |
| 881000 | Dual Connectivity (DC) of 3 bands LTE inter-band CA (3DL/1UL) and 1 NR band (1DL/1UL) | D C _ R 1 7 _ 3 B L T E _ 1 B N R _ 4 D L 2 U L |  | RP-211758 | Ericsson |
| 881100 | Core part: DC_R17_3BLTE_1BNR_4DL2UL | D C _ R 1 7 _ 3 B L T E _ 1 B N R _ 4 D L 2 U L - C o r e | R4 | RP-211758 | Ericsson |
| 881200 | Perf. part: DC_R17_3BLTE_1BNR_4DL2UL | D C _ R 1 7 _ 3 B L T E _ 1 B N R _ 4 D L 2 U L - P e r f | R4 | RP-211758 | Ericsson |
| 881001 | Dual Connectivity (DC) of 4 bands LTE inter-band CA (4DL/1UL) and 1 NR band (1DL/1UL) | D C _ R 1 7 _ 4 B L T E _ 1 B N R _ 5 D L 2 U L |  | RP-212532 | Nokia |
| 881101 | Core part: DC_R17_4BLTE_1BNR_5DL2UL | D C _ R 1 7 _ 4 B L T E _ 1 B N R _ 5 D L 2 U L - C o r e | R4 | RP-212532 | Nokia |
| 881201 | Perf. part: DC_R17_4BLTE_1BNR_5DL2UL | D C _ R 1 7 _ 4 B L T E _ 1 B N R _ 5 D L 2 U L - P e r f | R4 | RP-212532 | Nokia |
| 881002 | Dual Connectivity (DC) of x bands (x=1,2,3,4) LTE inter-band CA (xDL/1UL) and 2 bands NR inter-band CA (2DL/1UL) | D C _ R 1 7 _ x B L T E _ 2 B N R _ y D L 2 U L |  | RP-211750 | LG Electronics |
| 881102 | Core part: DC_R17_xBLTE_2BNR_yDL2UL | D C _ R 1 7 _ x B L T E _ 2 B N R _ y D L 2 U L - C o r e | R4 | RP-211750 | LG Electronics |
| 881202 | Perf. part: DC_R17_xBLTE_2BNR_yDL2UL | D C _ R 1 7 _ x B L T E _ 2 B N R _ y D L 2 U L - P e r f | R4 | RP-211750 | LG Electronics |
| 881003 | Dual Connectivity (DC) of x bands (x=1,2) LTE inter-band CA (xDL/xUL) and y bands (y=3-x) NR inter-band CA (yDL/yUL) | D C _ R 1 7 _ x B L T E _ y B N R _ 3 D L 3 U L |  | RP-211803 | ZTE |
| 881103 | Core part: DC_R17_xBLTE_yBNR_3DL3UL | D C _ R 1 7 _ x B L T E _ y B N R _ 3 D L 3 U L - C o r e | R4 | RP-211803 | ZTE |
| 881203 | Perf. part: DC_R17_xBLTE_yBNR_3DL3UL | D C _ R 1 7 _ x B L T E _ y B N R _ 3 D L 3 U L - P e r f | R4 | RP-211803 | ZTE |
| 881004 | Dual Connectivity (DC) of x bands (x=1,2,3) LTE inter-band CA (xDL/1UL) and 3 bands NR inter-band CA (3DL/1UL) | D C _ R 1 7 _ x B L T E _ 3 B N R _ y D L 2 U L |  | RP-211805 | ZTE |
| 881104 | Core part: DC_R17_xBLTE_3BNR_yDL2UL | D C _ R 1 7 _ x B L T E _ 3 B N R _ y D L 2 U L - C o r e | R4 | RP-211805 | ZTE |
| 881204 | Perf. part: DC_R17_xBLTE_3BNR_yDL2UL | D C _ R 1 7 _ x B L T E _ 3 B N R _ y D L 2 U L - P e r f | R4 | RP-211805 | ZTE |
| 881012 | band combinations for SA NR supplementary uplink (SUL), NSA NR SUL, NSA NR SUL with UL sharing from the UE perspective (ULSUP) | N R _ S U L _ c o m b o s _ R 1 7 |  | RP-212178 | Huawei |
| 881112 | Core part: NR_SUL_combos_R17 | N R _ S U L _ c o m b o s _ R 1 7 - C o r e | R4 | RP-212178 | Huawei |
| 881212 | Perf. part: NR_SUL_combos_R17 | N R _ S U L _ c o m b o s _ R 1 7 - P e r f | R4 | RP-212178 | Huawei |
| 900056 | UE Conformance - Rel-17 NR CA and DC; and NR and LTE DC Configurations | N R _ C A D C _ N R _ L T E _ D C _ R 1 7 - U E C o n T e s t | R5 | RP-211357 | Huawei |
| 0 | Rel-17 NR new/modified bands | - |  | - | - |
| 911016 | Introduction of 900MHz NR band for Europe for Rail Mobile Radio (RMR) | N R _ R A I L _ E U _ 9 0 0 M H z |  | RP-211495 | UIC |
| 911116 | Core part: NR_RAIL_EU_900MHz | N R _ R A I L _ E U _ 9 0 0 M H z - C o r e | R4 | RP-211495 | UIC |
| 911216 | Perf. part: NR_RAIL_EU_900MHz | N R _ R A I L _ E U _ 9 0 0 M H z - P e r f | R4 | RP-211495 | UIC |
| 911017 | Introduction of 1900MHz NR TDD band for Europe for Rail Mobile Radio (RMR) | N R _ R A I L _ E U _ 1 9 0 0 M H z _ T D D |  | RP-211542 | UIC |
| 911117 | Core part: NR_RAIL_EU_1900MHz_TDD | N R _ R A I L _ E U _ 1 9 0 0 M H z _ T D D - C o r e | R4 | RP-211542 | UIC |
| 911217 | Perf. part: NR_RAIL_EU_1900MHz_TDD | N R _ R A I L _ E U _ 1 9 0 0 M H z _ T D D - P e r f | R4 | RP-211542 | UIC |
| 860041 | Extending current NR operation to 71GHz | N R _ e x t _ t o _ 7 1 G H z | R1 | RP-212637 | Qualcomm |
| 860141 | Core part: Extending current NR operation to 71GHz | N R _ e x t _ t o _ 7 1 G H z - C o r e | R1 | RP-212637 | Qualcomm |
| 860241 | Perf. part: Extending current NR operation to 71GHz | N R _ e x t _ t o _ 7 1 G H z - P e r f | R4 | RP-212637 | Qualcomm |
| 860037 | Study on supporting NR from 52.6 GHz to 71 GHz | F S _ N R _ 5 2 _ t o _ 7 1 G H z | R1 | RP-201838 | Intel |
| 870060 | Study on IMT parameters for 6.425-7.025GHz, 7.025-7.125GHz and 10.0-10.5GHz | F S _ N R _ I M T _ p a r a m | R4 | RP-202276 | Ericsson |
| 870063 | Introduction of NR band n13 | N R _ n 1 3 | R4 | RP-200480 | Huawei |
| 870163 | Core part: Introduction of NR band n13 | N R _ n 1 3 - C o r e | R4 | RP-200480 | Huawei |
| 870263 | Perf. part: Introduction of NR band n13 | N R _ n 1 3 - P e r f | R4 | RP-200480 | Huawei |
| 880082 | Introduction of NR 47GHz band | N R _ 4 7 G H z _ b a n d | R4 | RP-212528 | Ericsson |
| 880182 | Core part: Introduction of NR 47GHz band | N R _ 4 7 G H z _ b a n d - C o r e | R4 | RP-212528 | Ericsson |
| 880282 | Perf. part: Introduction of NR 47GHz band | N R _ 4 7 G H z _ b a n d - P e r f | R4 | RP-212528 | Ericsson |
| 880084 | Introduction of NR band 24 | N R _ b a n d _ n 2 4 | R4 | RP-211505 | Ligado Networks |
| 880184 | Core part: Introduction of NR band 24 | N R _ b a n d _ n 2 4 - C o r e | R4 | RP-211505 | Ligado Networks |
| 880284 | Perf. part: Introduction of NR band 24 | N R _ b a n d _ n 2 4 - P e r f | R4 | RP-211505 | Ligado Networks |
| 890050 | Introduction of 6GHz NR licensed bands | N R _ 6 G H z | R4 | RP-202844 | Huawei |
| 890150 | Core part: Introduction of 6GHz NR licensed bands | N R _ 6 G H z - C o r e | R4 | RP-202844 | Huawei |
| 890250 | Perf. part: Introduction of 6GHz NR licensed bands | N R _ 6 G H z - P e r f | R4 | RP-202844 | Huawei |
| 920075 | Introduction of operation in full unlicensed band 5925-7125MHz for NR | N R _ 6 G H z _ u n l i c _ f u l l | R4 | RP-212302 | Apple |
| 920175 | Core part: Introduction of operation in full unlicensed band 5925-7125MHz for NR | N R _ 6 G H z _ u n l i c _ f u l l - C o r e | R4 | RP-212302 | Apple |
| 920275 | Perf. part: Introduction of operation in full unlicensed band 5925-7125MHz for NR | N R _ 6 G H z _ u n l i c _ f u l l - P e r f | R4 | RP-212302 | Apple |
| 890051 | Introduction of lower 6GHz NR unlicensed operation for Europe | N R _ 6 G H z _ u n l i c _ E U | R4 | RP-212625 | Nokia |
| 890151 | Core part: Introduction of lower 6GHz NR unlicensed operation for Europe | N R _ 6 G H z _ u n l i c _ E U - C o r e | R4 | RP-212625 | Nokia |
| 890251 | Perf. part: Introduction of lower 6GHz NR unlicensed operation for Europe | N R _ 6 G H z _ u n l i c _ E U - P e r f | R4 | RP-212625 | Nokia |
| 900065 | Introduction of NR band n67 | N R _ n 6 7 | R4 | RP-202829 | Ericsson |
| 900165 | Core part: Introduction of NR band n67 | N R _ n 6 7 - C o r e | R4 | RP-202829 | Ericsson |
| 900265 | Perf. Part: Introduction of NR band n67 | N R _ n 6 7 - P e r f | R4 | RP-202829 | Ericsson |
| 900066 | Introduction of NR band n85 | N R _ n 8 5 | R4 | RP-210707 | Ericsson |
| 900166 | Core part: Introduction of NR band n85 | N R _ n 8 5 - C o r e | R4 | RP-210707 | Ericsson |
| 900266 | Perf. part: Introduction of NR band n85 | N R _ n 8 5 - P e r f | R4 | RP-210707 | Ericsson |
| 880083 | Introduction of 1.6 GHz NR supplemental uplink (SUL) band with same uplink frequency range of Band 24 | N R _ S U L _ U L _ n 2 4 |  | RP-210341 | Ligado Networks |
| 880183 | Core part: NR_SUL_UL_n24 | N R _ S U L _ U L _ n 2 4 - C o r e | R4 | RP-210341 | Ligado Networks |
| 880283 | Perf. part: NR_SUL_UL_n24 | N R _ S U L _ U L _ n 2 4 - P e r f | R4 | RP-210341 | Ligado Networks |
| 880085 | Introduction of NR supplemental uplink (SUL) band 1880-1920MHz | N R _ S U L _ b a n d _ 1 8 8 0 _ 1 9 2 0 M H z |  | RP-201363 | CMCC |
| 880185 | Core part: NR_SUL_band_1880_1920MHz | N R _ S U L _ b a n d _ 1 8 8 0 _ 1 9 2 0 M H z - C o r e | R4 | RP-201363 | CMCC |
| 880285 | Perf. part: NR_SUL_band_1880_1920MHz | N R _ S U L _ b a n d _ 1 8 8 0 _ 1 9 2 0 M H z - P e r f | R4 | RP-201363 | CMCC |
| 880086 | Introduction of NR supplemental uplink (SUL) band 2300-2400MHz | N R _ S U L _ b a n d _ 2 3 0 0 _ 2 4 0 0 M H z |  | RP-201364 | CMCC |
| 880186 | Core part: NR_SUL_band_2300_2400MHz | N R _ S U L _ b a n d _ 2 3 0 0 _ 2 4 0 0 M H z - C o r e | R4 | RP-201364 | CMCC |
| 880286 | Perf. part: NR_SUL_band_2300_2400MHz | N R _ S U L _ b a n d _ 2 3 0 0 _ 2 4 0 0 M H z - P e r f | R4 | RP-201364 | CMCC |
| 900055 | UE Conformance - New Rel-17 NR licensed bands and extension of existing NR bands | N R _ l i c _ b a n d s _ B W _ R 1 7 - U E C o n T e s t | R5 | RP-202567 | Huawei |
| 0 | Rel-17 LTE new/modified bands | - |  | - | - |
| 911020 | New bands and bandwidth allocation for 5G terrestrial broadcast - part 1 | L T E _ t e r r _ b c a s t _ b a n d s _ p a r t 1 | R1 | RP-211144 | EBU |
| 911120 | New bands and bandwidth allocation for 5G terrestrial broadcast - part 1 | L T E _ t e r r _ b c a s t _ b a n d s _ p a r t 1 | R1 | RP-211144 | EBU |
| 880079 | Additional LTE bands for UE categories M1/M2/NB1/NB2 | L T E _ b a n d s _ R 1 7 _ M 1 _ M 2 _ N B 1 _ N B 2 |  | RP-211857 | Ericsson |
| 880179 | Core part: LTE_bands_R17_M1_M2_NB1_NB2 | L T E _ b a n d s _ R 1 7 _ M 1 _ M 2 _ N B 1 _ N B 2 - C o r e | R4 | RP-211857 | Ericsson |
| 880279 | Perf. part: LTE_bands_R17_M1_M2_NB1_NB2 | L T E _ b a n d s _ R 1 7 _ M 1 _ M 2 _ N B 1 _ N B 2 - P e r f | R4 | RP-211857 | Ericsson |
| 880081 | Modification of LTE Band 24 Specifications to comply with updated regulatory emission limits | L T E _ B 2 4 _ m o d |  | RP-211507 | Ligado Networks |
| 880181 | Core part: LTE_B24_mod | L T E _ B 2 4 _ m o d - C o r e | R4 | RP-211507 | Ligado Networks |
| 880281 | Perf. part: LTE_B24_mod | L T E _ B 2 4 _ m o d - P e r f | R4 | RP-211507 | Ligado Networks |
| 920067 | UE Conformance - Modification of LTE Band 24 Specifications to comply with updated regulatory emission limits | L T E _ B 2 4 _ m o d - U E C o n T e s t | R5 | RP-212497 | Ligado Networks |
| 0 | Rel-17 LTE inter-band Carrier Aggregation (LTE iterbCA) | - |  | - | - |
| 880087 | LTE iterbCA for 2 bands DL with 1 band UL | L T E _ C A _ R 1 7 _ 2 B D L _ 1 B U L |  | RP-211859 | Qualcomm |
| 880187 | Core part: LTE_CA_R17_2BDL_1BUL | L T E _ C A _ R 1 7 _ 2 B D L _ 1 B U L - C o r e | R4 | RP-211859 | Qualcomm |
| 880287 | Perf. part: LTE_CA_R17_2BDL_1BUL | L T E _ C A _ R 1 7 _ 2 B D L _ 1 B U L - P e r f | R4 | RP-211859 | Qualcomm |
| 880088 | LTE iterbCA for 3 bands DL with 1 band UL | L T E _ C A _ R 1 7 _ 3 B D L _ 1 B U L |  | RP-212192 | Huawei |
| 880188 | Core part: LTE_CA_R17_3BDL_1BUL | L T E _ C A _ R 1 7 _ 3 B D L _ 1 B U L - C o r e | R4 | RP-212192 | Huawei |
| 880288 | Stopped - Perf. part: LTE iterbCA for 3 bands DL with 1 band UL | L T E _ C A _ R 1 7 _ 3 B D L _ 1 B U L - P e r f | R4 | RP-202615 | Huawei |
| 880089 | LTE iterbCA Core for x bands DL (x=4, 5, 6) with 1 band UL | L T E _ C A _ R 1 7 _ x B D L _ 1 B U L |  | RP-211885 | Nokia |
| 880189 | Core part: LTE_CA_R17_xBDL_1BUL | L T E _ C A _ R 1 7 _ x B D L _ 1 B U L - C o r e | R4 | RP-211885 | Nokia |
| 880289 | Perf. part: LTE_CA_R17_xBDL_1BUL | L T E _ C A _ R 1 7 _ x B D L _ 1 B U L - P e r f | R4 | RP-211885 | Nokia |
| 880090 | LTE iterbCA for 2 bands DL with 2 bands UL | L T E _ C A _ R 1 7 _ 2 B D L _ 2 B U L |  | RP-212194 | Huawei |
| 880190 | Core part: LTE_CA_R17_2BDL_2BUL | L T E _ C A _ R 1 7 _ 2 B D L _ 2 B U L - C o r e | R4 | RP-212194 | Huawei |
| 880290 | Perf. part: LTE_CA_R17_2BDL_2BUL | L T E _ C A _ R 1 7 _ 2 B D L _ 2 B U L - P e r f | R4 | RP-212194 | Huawei |
| 880091 | LTE iterbCA for x bands DL (x= 3, 4, 5) with 2 bands UL | L T E _ C A _ R 1 7 _ x B D L _ 2 B U L |  | RP-211748 | LG |
| 880191 | Core part: LTE_CA_R17_xBDL_2BUL | L T E _ C A _ R 1 7 _ x B D L _ 2 B U L - C o r e | R4 | RP-211748 | LG |
| 880291 | Perf. part: LTE_CA_R17_xBDL_2BUL | L T E _ C A _ R 1 7 _ x B D L _ 2 B U L - P e r f | R4 | RP-211748 | LG |

Annex A:
Template for contributing

Editor's note: To contribute to this document, please copy-paste the following template into a document to be submitted to the TSG plenary as "Work Item Summary". This will be further endorsed by the plenary.
Each summary is expected to be one page long maximum. 
The target audience of this document are readers inside and outside the 3GPP community. Contributions should be written as to be understandable by non-specialist. 
The summary should reflect the actual outcome of the Feature - and not the initial intentions.

---

Summary based on the input provided by [company name] in [tdoc numbed].

| Unique_ID | Name | Acronym | WG | WID | WI_rapporteur_name |
| --- | --- | --- | --- | --- | --- |
| AABBCC | Title here | XXX | e.g. S4 | xP-19XXXX | Firstname Familyname, Company |

Introduction

<guidance:
- summarize in one to a few sentences what was achieved by this WI and how: what problems it solves, what new functionality it adds, etc.;

- remember that only what ended up in the specs is of interest;

- clarify what is the relation of this WI to other WIs (e.g. is this an enhancement of a former WI? Is there a related WI?) and what was the status before this WI

Description

<guidance:

- Explain here in a couple of paragraphs the main impacts on the system, mostly: the new and/or impacted entities, and the big lines of the system's behaviour;

- remember that only what ended up in the specs shall be covered here;

- this it not intended to be a copy and paste from the specifications, the WID or status report;

- the reader shall understand how the key functionalities are working without running too far into details;

- use figures, tables, references to illlustrate and  facilitate the understanding;

- remember that you do not write the WI summary for people who contributed to this WI so make sure that you use terminology and abbreviations that can be understood by any engineer or make sure you explain them;

- When the usual Stage 1/Stage 2/Stage 3 process is not followed -and in particular if a full stage is missing- explain why.

- make the following test: give this summary to a colleague who was not at all involved in this topic and let this colleague explain in her/his own words how your WI works after reading the summary>

References

<guidance: It is recommended to add some references where the interested reader can find further documentation, e.g.

- new/impacted TRs and TSs (if any)

- last status report (if any)

- main CRs

NOTE: Please provide references that can be found easily, e.g. indicating Tdoc numbers.>

[1]	TS XX.XXX, Spec title

[2]	TS YY.YYY, Spec title

Annex B:
Process to get further information

# B.1	General

Since the present document is limited to provide an overview of each Feature, this chapter explains how to get additional information, in particular how to retrieve all the Specifications (TSs) and Reports (TRs) as well as all the CRs which relate to a given Work Item.

The Unique Identifier (UID) is the key to get additional information on a given Work Item. It can be found in the table located just below the clause's header. The table has the following format:

Table B-1: table format

| Unique Identifier (UID) | Name | Acronym | Outline Level (1=Feature, 2=Building Block, 3=Work Task) | Responsible Working Group | Work Item Description |
| --- | --- | --- | --- | --- | --- |

For readability reasons, the table headers are omitted in the continuation of the present document.

For instance, for the "Mission Critical Push to Talk over LTE Realignment", the table has to be understood as:

Table B-2: Example of table at the introduction of each Feature

| Unique Identifier (UID) | Name | Acronym | Outline Level | Responsible Working Group | Work Item Description |
| --- | --- | --- | --- | --- | --- |
| 700029 | Mission Critical Push to Talk over LTE Realignment | MCImp-MCPTTR | 2 | S1 | SP-150821 |

Thus, the UID for this Work Item is 700029.

Two methods are now possible to retrieve more information on a given feature: the "Step by step method" and the "Direct method". The "direct method" is faster but implies to know the hierarchical structure of the Work Items. The "step by step method" is slower but is easier to use, in particular when the hierarchical structure is unknown.

For instance, for retrieving all the CRs that relate to "Enhancements for Mission Critical Push To Talk", the search has to be done on UID 740022 but also potentially on its children Work Items (UID 720056, 740023 and 740024).

Table B-3: Example of a hierarchical structure and its consequences on the search procedure

| 740022 | Enhancements for Mission Critical Push To Talk | MCImp-eMCPTT |  | SP-160490 |
| --- | --- | --- | --- | --- |
| 720056 | Stage 2 of Enhancements for Mission Critical Push To Talk | MCImp-eMCPTT | S6 | SP-160490 |
| 740023 | Stage 3 of Enhancements for Mission Critical Push To Talk | MCImp-eMCPTT-CT | CT | CP-160824 |
| 740024 | CT1 aspects of Enhancements for Mission Critical Push To Talk | MCImp-eMCPTT-CT | C1 | CP-160824 |

These two methods are described in the following clauses.

# B.2	Direct method

The links below lead to the pages containing respectively all the Specifications and all the Change Requests (CRs) linked to a given Feature:

https://portal.3gpp.org/Specifications.aspx?q=1&WiUid=[UID]

https://portal.3gpp.org/ChangeRequests.aspx?q=1&workitem=[UID]

where "[UID]" has to be preplaced by the UID value.

Using the example provided in the table 4.1-3, the specification linked to "Stage 2 of Enhancements for Mission Critical Push To Talk" can be found in:

https://portal.3gpp.org/Specifications.aspx?q=1&WiUid=720056

And all the related Change Requests are listed in:

https://portal.3gpp.org/ChangeRequests.aspx?q=1&workitem=720056

# B.3	Step by step method

This method is to be used when the hierarchical structure is not known or when the "direct method" above does not show the expected results.

In this case, the 3GPP Ultimate web site has to be used:

https://portal.3gpp.org

As a preliminary step, it is essential that the "Customized Selection" is set to "All TSGs" (otherwise, a filter would be applied).

Figure B.3-1: Selecting "All TSGs" in "Customized Selection" as to remove any potential filter on the Search

Then select the "Work Plan" tab (upper red arrow in the figure below).

Figure B.3-2: Using the 3GU Portal to retrieve more information on a given Feature

Then the search might be performed by either typing the Acronym (as shown by the left red arrow on the figure above, using the example "MCPTT"), or by the name or UID (right box) then by clicking on the "Search" button. Watch the "Granularity (Level)" field, which is a filter to return only the Items which level is specified here.

In the results, the icon depicting some binoculars has to be hit (lower right red arrow on the figure above).

This will lead to the page shown in the figure below:

Figure B.3-3: Window resulting from a "Search"

On this window, the "Related" tab has to be clicked, as pointed by the red arrow in the figure above. This will lead to the window depicted in the figure below.

Figure B.3-4: "Related" tab in a Work Item search, with links to all related Specifications and Change Requests

The two links pointed by the red arrows in the figure above lead to the pages containing respectively all the Specifications and all the Change Requests (CRs) linked to this Feature.

Annex C:
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2021-11 | TSG#94 |  |  |  |  | Initital draft, call for contributions | 0.1.0 |
| 2022-01 | - |  |  |  |  | Clean-up of skeletton | 0.2.0 |
| 2022-03 | TSG#95e |  |  |  |  | Inputs included in v.0.3.0: SP-220019 WI Summary for Mission critical security enhancements phase 2 (MCXSec2) Motorola Solutions Danmark A/S SP-220020 Summary for Edge Computing Management Samsung R&D Institute UK SP-220075 Summary for Network Slice charging based on 5G Data Connectivity MATRIXX Software SP-220089 EMCData3 WI Summary AT&T GNS Belgium SPRL SP-220260 Summary for Enhancement for the 5G Control Plane Steering of Roaming for UE in CONNECTED mode (eCPSOR_CON) NTT DOCOMO INC. SP-220261 WI Summary for TEI17_SPSFAS China Telecommunications SP-220275 Summary for WI Support of Immersive Teleconferencing and Telepresence for Remote Terminals (ITT4RT) Nokia Corporation (Rapporteur) SP-220277 Summary for eV2XARC_Ph2 LG Electronics SP-220285 Summary for 5GMARCH China Mobile Com. Corporation SP-220289 Summary for Authentication and key management for applications based on 3GPP credential in 5G China Mobile CP-220021 Template for Feature Summary for inclusion in TR 21.917 (Rel-17 Description) MCC/Alain CP-220110 Summary for Stage 3 aspects of eMCCI FirstNet CP-220111 Summary for Stage 3 aspects of enh3MCPTT FirstNet CP-220149 Summary for Restoration of PDN Connections in PGW-C/SMF Set Nokia, Nokia Shanghai Bell CP-220150 Summary for Non-Seamless WLAN offload authentication in 5GS Nokia, Nokia Shanghai Bell CP-220152 Summary for Enhancement of Inter-PLMN Roaming Huawei CP-220211 Summary for Start of Pause of Charging via User Plane Ericsson CP-220212 Summary for Enhancement on the GTP-U entity restart Ericsson CP-220321 Summary for PFD management enhancement Huawei CP-220322 Summary for enhancement of 5G PCC related services in Rel-17 Huawei CP-220343 Summary for Restoration of Profiles related to UDR NTT DOCOMO INC. CP-220347 Summary for Rel-17 Enhancements of 3GPP Northbound Interfaces and Application Layer APIs Huawei CP-220368 Summary for PAP/CHAP protocols usage in 5GS China Telecomunication Corp. Not yet implemented (not available by March 10th 2022): RP-220142 WI Summary for NR Uplink Data Compression (UDC) CATT RP-220154 Summary for WI: NR small data transmissions in INACTIVE state (Rel-17) ZTE Corporation (rapporteur) RP-220165 Summary for SI Study on band combination handling in RAN4 ZTE Corporation RP-220191 WI Summary for Introduction of DL 1024QAM for NR FR1; rapporteur: Ericsson, Nokia Ericsson RP-220211 Summary for WI of NR Sidelink Relay OPPO RP-220316 WI summary for NR support for high speed train scenario in frequency range 2 (FR2) Samsung RP-220408 WI summary: NR Multicast and Broadcast Services Huawei, HiSilicon RP-220410 WI summary: Multi-Radio Dual-Connectivity enhancements Huawei, HiSilicon RP-220445 WI summary for New bands and bandwidth allocation for 5G terrestrial broadcast (part 1) EBU, Qualcomm Incorporated (Rapporteur) RP-220464 Summary of Rel17 WI on NR Dynamic spectrum sharing (DSS) Ericsson RP-220468 WI Summary for UE RF requirements for Transparent Tx Diversity (TxD) for NR Qualcomm Incorporated RP-220519 WI summary: Enhancements to Integrated Access and Backhaul for NR Qualcomm Incorporated, Samsung RP-220521 Summary for WI "NR sidelink enhancement" LG Electronics RP-220530 WI summary: Additional enhancements for NB-IoT and LTE-MTC Huawei, HiSilicon RP-220544 WI summary for NR Repeaters Qualcomm Incorporated RP-220556 WI Summary for NR QoE management and optimizations for diverse services China Unicom RP-220560 WI Summary for Enhanced eNB(s) architecture evolution for E-UTRAN and NG-RAN China Unicom RP-220562 WI Summary for Enhancement of Private Network Support for NG-RAN China Telecom RP-220564 Summary for WI on NR coverage enhancements China Telecom RP-220604 WI summary for WI Core part: Support for Multi-SIM devices for LTE/NR vivo (WI Rapporteur) RP-220606 WI summary for WI: Introduction of UE TRP (Total Radiated Power) and TRS (Total Radiated Sensitivity) requirements and test methodologies for FR1 (NR SA and EN-DC) vivo (WI Rapporteur) RP-220631 Summary for WI: Enhanced NR support for high speed train scenario for frequency range 1 (FR1) CMCC RP-220638 Summary for WI Enhancement of RAN slicing for NR CMCC, ZTE RP-220682 WI summary: RF requirements enhancement for NR frequency range 1 (FR1) Huawei, HiSilicon RP-220752 Summary for WI NR and MR-DC measurement gap enhancements Rapporteur (MediaTek Inc., Intel) | 0.3.0 |