3GPP TSG-RAN Meeting#95-e	RP-220130

# Electronic meeting, 17 – 23 March 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.306 | CR | 0667 | rev | 1 | Current version: | 16.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Pi/2-BPSK specification updates for the merger of 5Gi into 3GPP |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: |  |  |  |  |  |  |  |  |  |  |
| Source to TSG: | Qualcomm, IITH, IITM, CEWiT, Bharti Airtel, Reliance Jio, Vodafone Idea, Tejas Networks, Ericsson, Nokia, Nokia Shanghai Bell, Samsung, Apple, MediaTek, ZTE, Intel, Huawei, HiSilicon, Spreadtrum, NTT DOCOMO, vivo |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core, TEI-17 |  |  |  |  |  | Date: |  |  | 2022-03-01 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Provide UE capability framework for the intended 5Gi merger into 3GPP. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Change the designation from ‘optional’ or ‘FR1 optional’ to ‘mandatory with capability signaling’ for the following UE capabilities: - lowPAPR-DMRS-PUCCH - lowPAPR-DMRS-PUSCHwithPrecoding - powerBoosting-pi2BPSK - pucch-F3-4-HalfPi-BPSK - pusch-HalfPi-BPSK Isolated impact analysis: By utilizing the existing capability signaling framework, full backward compatibility is maintained. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No UE capability framework would be provided for the intended 5Gi merger into 3GPP |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.7.2, 4.2.7.10 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 38.211 CR 0086 |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Note that in the the previous revision of this CR that was submitted in RP-213534, the change described above for lowPAPR-DMRS-PUSCHwithPrecoding-r16 was erronesously implemented in the field for lowPAPR-DMRS-PDSCH-r16 instead. This revision of the CR has corrected this error. |  |  |  |  |  |  |  |  |

< start of changes >

[…]

#### 4.2.7.2	BandNR parameters

| Definitions for parameters | Per | M | FDD-TDD DIFF | FR1-FR2 DIFF |
| --- | --- | --- | --- | --- |
| […] | […] | […] | […] | […] |
| lowPAPR-DMRS-PUCCH-r16 Indicates whether the UE supports low PAPR DMRS for PUCCH format 3 and format 4 with transform precoding and with pi/2 BPSK modulation. UE indicates support of this feature shall indicate support of pucch-F3-4-HalfPi-BPSK and any combination of support of pucch-F3-WithFH, pucch-F4-WithFH and pucch-F1-3-4WithoutFH. It is mandatory with capability signalling. | Band | Yes | N/A | N/A |
| […] | […] | […] | […] | […] |
| lowPAPR-DMRS-PUSCHwithPrecoding-r16 Indicates whether the UE supports low PAPR DMRS for PUSCH with transform precoding and with pi/2 BPSK modulation. It is mandatory with capability signalling. UE indicates support of this feature shall indicate support of pusch-HalfPi-BPSK. | Band | Yes | N/A | N/A |
| […] | […] | […] | […] | […] |
| powerBoosting-pi2BPSK Indicates whether UE supports power boosting for pi/2 BPSK, when applicable as defined in 6.2 of TS 38.101-1 [2] v16.9.0. It is mandatory with capability signalling. This capability is not applicable to IAB-MT. | Band | CY | TDD only | FR1 only |
| […] | […] | […] | […] | […] |

[…]

#### 4.2.7.10	Phy-Parameters

| Definitions for parameters | Per | M | FDD-TDD DIFF | FR1-FR2 DIFF |
| --- | --- | --- | --- | --- |
| […] | […] | […] | […] | […] |
| pucch-F3-4-HalfPi-BPSK Indicates whether the UE supports pi/2-BPSK for PUCCH format 3/4 as defined in 6.3.2.6 of TS 38.211 [6]. It is mandatory with capability signalling for FR1 and FR2. This capability is not applicable to IAB-MT. | UE | Yes | No | Yes |
| […] | […] | […] | […] | […] |
| pusch-HalfPi-BPSK Indicates whether the UE supports pi/2-BPSK modulation scheme for PUSCH as defined in 6.3.1.2 of TS 38.211 [6]. It is mandatory with capability signalling for FR1 and FR2. This capability is not applicable to IAB-MT. | UE | Yes | No | Yes |
| […] | […] | […] | […] | […] |

< end of changes >