3GPP TSG RAN meeting #95e										RP-220141

Electronic Meeting, March 17th - 23rd, 2022

Status Report to TSG

Agenda item:			9.5.2.9

| WI / SI Name | NR Uplink Data Compression (UDC) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: Yes | Performance part: No |  | Testing part: No |
| Acronym | NR_UDC-Core |  |  |  |  |
| Unique ID | 911006 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-211203 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: | Core part: 03/2022 | Performance part: | Testing part: |  |
| Overall Completion level | Study Item: | Core part: 100% | Performance Part: | Testing part: |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | RAN2 |
| --- | --- | --- |
| Rapporteur | Name | Erlin Zeng |
|  | Company | CATT |
|  | Email | erlin.zeng@catt.cn |

1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

2.1	RAN1

2.1.1	Agreements

2.1.2	Remaining Open issues

2.2	RAN2

2.2.1	Agreements

Agreements in R2#116bis-e

The following were agreed based on email discussion summary in R2-2200039

- The parts without TBD in Table 1  are assumed to directly follow LTE UDC mechanism.

- UDC is not applied to the SDAP header and SDAP control PDU.

- The UDC header is located after SDAP header in the UDC PDU format.

- UDC is not applied to DAPS in NR.

- NR UDC is not applied to sidelink DRBs.

- With Figure 4.2.2-1, there is no need to further clarify UDC decompression being performed after PDCP re-ordering in the specification.

- UE shall support number of UDC DRBs 2. FFS if we need to support some additional UE capability.

- Continue by email, can include tech proposals from tdocs below (proponents are expected to request), continue on the non-agreed parts, review CRs.

The following were agreed based on offline report R2-2201914

- FFS whether UE data rate limitation with UDC need to be supported with a UE capability.

- UDC continuity can be configured for the same cases as ROHC continuity

- Assume that P2 and P5 can be supported, CRs for review to next meeting anyway. If issues are found R2 can revert this assumption (at next meeting).

- P2: UDC is supported for non-split bearer type in NR-DC. It is supported that MN sends to SN the maximum number of UDC DRBs that can be configured by SN. FFS if any other coordination is needed.

- P5: Support NR UDC for MR-DC and split bearer type, with the following restrictions

- - Only include NR-DC, NGEN-DC, and NE-DC (i.e., EN-DC is not supported)

- - No enhancements supported for potential data loss for split bearer case.

- Send an LS to RAN3 to inform of NR UDC potential impact to CU-CP/UP splitting scenario. R2 understands that decisions as well as the required specification work are up to RAN3.

An LS to RAN3 was agreed in R2-2202033.

Agreements in R2#117-e

The following were agreed based on offline report in R2-2203838

- No additional inter-node coordination between MN and SN is needed on top of what has been specified in the current RRC CR in R2-2203108.

- Related to drb-ContinueUDC, assume that we modify the normative text, but the modification should be simpler than Option 2, details in the CR discussion.

- RAN2 recognizes that the UE is not expected to handle peak data rates with UDC, but No additional UE capability restriction is introduced in terms of maximum uplink data rate for NR UDC. UE is allowed to not compress, in case data rate is higher than what the UE is capable of. FFS if any of this need to be captured in a TS (can be discussed in the CR discussion)

- No enhancement is introduced to handle the potential issue of PDCP PDUs to be discarded by the network side

All 4 CRs (including CRs to 38.300, 38.331, 38.323 and 37.340) for NR UDC introduction (not including UE capability) were agreed, and two UE capability related CRs (draft CRs to 38.306 and 38.331) were endorsed.

2.2.2	Remaining Open issues

None.

2.3	RAN3

2.3.1	Agreements

There were two RAN3 meetings from last RAN plenary, but only RAN3#115 meeting treated NR UDC related issue since there was an LS received from RAN2 to let RAN3 discuss the scenario of CU-CP/UP splitting for NR UDC. RAN3 agreed that NR UDC can be applied for CU-CP/UP splitting scenario in both LTE and NR and corresponding CRs (CRs to 38.460 and 38.463) were agreed.

2.3.2	Remaining Open issues

None.

2.4	RAN4

2.4.1	Agreements

2.4.2	Remaining Open issues

2.5	RAN5

2.5.1	Agreements

2.5.2	Remaining Open issues

2.5.3	Remaining Open issues with cross-WG dependencies

2.6	RAN6

2.6.1	Agreements

2.6.2	Remaining Open issues

3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

NOTE: This section only needs to be filled in for WI/SIs where there is a corresponding relevant WI/SI in SA/CT.

3.1	SAx/CTs

3.1.1	Agreements with cross-TSG impacts

3.1.2	Remaining Open issues with cross-TSG impacts

NOTE: This section should also flag any critical dependencies that need TSG attention.

4.	References

NOTE:	This can be e.g. a list of all related Tdocs in the affected WGs since last TSG, references to LSs, produced TRs/TSs, the work/study item description or status reports of previous TSGs.

[1]	RP-211203 Revised WID on NR Uplink Data Compression (UDC)	CATT, CMCC

[2]	RP-220141 Status Report on NR Uplink Data Compression (UDC)	 CATT

[3]	R2-2201276	 Work plan for NR UDC	CATT

[4]	R2-2201277 Introduction of the support for UDC in NR	CATT, CMCC, Huawei, HiSilicon, MediaTek, Ericsson, China Unicom, China Telecom, OPPO, ZTE, Samsung

[5]	R2-2201278 Introduction of the support for UDC in NR	CATT, CMCC, Huawei, HiSilicon, MediaTek, Ericsson, China Unicom, China Telecom, OPPO, ZTE, Samsung

[6]	R2-2201279 Introduction of the support for UDC in NR	CATT, CMCC, Huawei, HiSilicon, MediaTek, Ericsson, China Unicom, China Telecom, OPPO, ZTE, Samsung

[7]	R2-2201280	 Introduction of the support for UDC in NR	CATT, CMCC, Huawei, HiSilicon, MediaTek, Ericsson, China Unicom, China Telecom, OPPO, ZTE, Samsung

[8]	R2-2201281	 Introduction of the support for UDC in NR	CATT, CMCC, Huawei, HiSilicon, MediaTek, Ericsson, China Unicom, China Telecom, OPPO, Samsung

[9]	R2-2200039 Report of [Post116-e][088][UDC] UDC initial discussion (CATT)	CATT

[10]	R2-2201914	 Report of [AT116bis-e][053][UDC] General (CATT)	CATT

[11]	R2-2200977 Discussion on UDC support in NR	Huawei, HiSilicon

[12]	R2-2200495 Limit UL data rate for UDC in UE capability	MediaTek Inc.

[13]	R2-2200581	 Issue on UDC continuation	Samsung Electronics Polska

[14]	R2-2200724	 Remaining issues on NR UDC	Qualcomm Incorporated

[15]	R2-2200932	 Consideration on NR UDC	OPPO

[16]	R2-2201129 Open topics on UDC functionality	Apple

[17]	R2-2201227	 Furhter Consideration on  UDC in NR	ZTE Corporation,Sanechips

[18]	R2-2201282	 Clarifications on NR UDC applicable scenarios	CATT, CMCC

[19]	R2-2201650	 Discussion on remaining issues for UDC	LG Electronics, Ericsson

[20]	R2-2203838 Report of [AT117-e][051][UDC] Open Issues and CRs (CATT) – Phase 1	CATT

[21]	R2-2204116 Report of [AT117-e][051][UDC] Open Issues and CRs (CATT) – Phase 2	CATT

[22]	R2-2203107 Introduction of the support for UDC in NR	CATT, CMCC, Huawei, HiSilicon, MediaTek Inc., Ericsson, China Unicom, China Telecom, OPPO, ZTE, Samsung, Apple, Nokia, Nokia Shanghai Bell

[23]	R2-2203108 Introduction of the support for UDC in NR	CATT, CMCC, Huawei, HiSilicon, MediaTek Inc., Ericsson, China Unicom, China Telecom, OPPO, ZTE, Samsung, Apple, Nokia, Nokia Shanghai Bell

[24]	R2-2203109 Introduction of the support for UDC in NR	CATT, CMCC, Huawei, HiSilicon, MediaTek Inc., Ericsson, China Unicom, China Telecom, OPPO, ZTE, Samsung, Apple, Nokia, Nokia Shanghai Bell

[25]	R2-2204094 Introduction of the support for UDC in NR	CATT, CMCC, Huawei, HiSilicon, MediaTek Inc., Ericsson, China Unicom, China Telecom, OPPO, ZTE, Samsung, Apple, Nokia, Nokia Shanghai Bell

[26]	R2-2203111 Introduction of the support for UDC in NR	CATT, CMCC, Huawei, HiSilicon, MediaTek Inc., Ericsson, China Unicom, China Telecom, OPPO, Samsung, Apple, Nokia, Nokia Shanghai Bell

[27]	R2-2203110 Introduction of UE capabilities for NR UDC	CATT, CMCC, Huawei, HiSilicon, MediaTek Inc., Ericsson, China Unicom, China Telecom, OPPO, ZTE, Samsung, Apple, Nokia, Nokia Shanghai Bell

[28]	R2-2204095	 Introduction of UE capabilities for NR UDC	CATT, CMCC, Huawei, HiSilicon, MediaTek Inc., Ericsson, China Unicom, China Telecom, OPPO, ZTE, Samsung, Apple, Nokia, Nokia Shanghai Bell

[29]	R2-2203112 Introduction of UE capabilities for NR UDC	CATT, CMCC, Huawei, HiSilicon, MediaTek Inc., Ericsson, China Unicom, China Telecom, OPPO, ZTE, Samsung, Apple, Nokia, Nokia Shanghai Bell

[30]	R2-2202367 Limit UL data rate for UDC in UE capability	MediaTek Inc., Samsung

[31]	R2-2202442 Consideration on NR UDC	OPPO

[32]	R2-2202520 UDC constraints and limitations	Apple

[33]	R2-2202678 Clarification on PDCP SDU for UDC continuity	Samsung Electronics

[34]	R2-2202961 Remaining issues on NR UDC	Qualcomm Incorporated

[35]	R2-2203023 Discussion on remaining issues for UDC	Huawei, HiSilicon

[36]	R2-2203106 Considerations on NR UDC open issues	CATT

[37]	R2-2203164 Discussion on UDC	LG Electronics Inc.

[38]	R2-2203249 Further Consideration on UDC in NR	ZTE Corporation, Sanechips

[39]	R3-221999 Discussion on support of UDC in CPUP separation scenario CATT, ZTE

[40]	R3-222000 (CR for 38 460) Support of UDC in E1 CATT, ZTE

[41]	R3-222001 (CR for 38 463) Support of UDC in E1 CATT, ZTE

[42]	R3-222334 Support of UDC in NR ZTE, CATT

[43]	R3-222133 UDC for CU-CP/UP splitting scenario Huawei

[44]	R3-222134 UDC for CU-CP/UP splitting scenario Huawei

[45]	R3-222135 UDC for CU-CP/UP splitting scenario Huawei

[46]	R3-222613  UDC for CU-CP/UP splitting scenario Huawei, CATT, ZTE, Nokia, Nokia Shanghai Bell, China Telecom

[47]	R3-222614 UDC for CU-CP/UP splitting scenario ZTE, CATT, Huawei, Nokia, Nokia Shanghai Bell, China Telecom

17.05.2021		minor adaptations for RAN #92e

28.01.2021		minor adaptations for RAN #91e

09.11.2020		minor adaptations for RAN #90e

31.08.2020		minor adaptations for RAN #89e

20.04.2020		minor adaptations for RAN #88e

18.02.2020		minor adaptations for RAN #87e

14.11.2019		minor adaptations for RAN #86

18.08.2019		minor adaptations for RAN #85

12.05.2019		minor adaptations for RAN #84

27.02.2019		minor adaptations for RAN #83

21.11.2018		completion levels with colours added (for RAN #82)

v04.81	31.07.2018		simplification of template and addition of cross-TSG aspects (for RAN #81)

v04.80	21.05.2018		minor adaptations for RAN #80

v04.79	26.02.2018		minor adaptations for RAN #79

v04.78	18.11.2017		minor adaptations for RAN #78

v04.77	06.08.2017		minor adaptations for RAN #77

v04.76	15.05.2017		minor adaptations for RAN #76

v04.75	31.01.2017		minor adaptations for RAN #75

v04.74	28.10.2016		minor adaptations for RAN #74

v04.73	01.09.2016		adaptations for RAN #73 (time units in extra Excel table, RAN6 reporting included)

v04.72	26.05.2016		adaptations for RAN #72 (introduction of NR & GERAN TUs)

v04.71	10.02.2016		minor adaptations for RAN #71

v04.70	30.10.2015		minor adaptations for RAN #70

v04.69	12.08.2015		minor adaptations for RAN #69

v04.68	21.05.2015		minor adaptations for RAN #68

v04.67	01.02.2015		minor adaptations for RAN #67

v04.66	16.11.2014		minor adaptations for RAN #66

v04.65	16.08.2014		minor adaptations for RAN #65

v04.64	22.05.2014		minor adaptations for RAN #64

v04.63	24.01.2014		restructuring for RAN #63 to cover Core & Perf. in one doc file

v03.62	11.11.2013		section 1.2.3 adapted for RAN #62

v03	11.08.2013		section 1.2.3 added on time budget

v02	07.05.2010		history added, some spelling corrections

v01	13.11.2009		First version of the template