3GPP TSG RAN meeting #95e										RP-220190

Electronic Meeting, March 17-23, 2022

## Status Report to TSG

Agenda item:			9.5.4.4

| WI / SI Name |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: Yes | Performance part: Yes |  | Testing part: No |
| Acronym | NR_DL1024QAM_FR1 |  |  |  |  |
| Unique ID | 890156 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-213654 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: mm/yyyy | Core part: 03/2022 | Performance part: 06/2022 | Testing part: mm/yyyy |  |
| Overall Completion level | Study Item: xx % | Core part: 100% | Performance Part: 70% | Testing part: xx% |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | RAN4 |
| --- | --- | --- |
| Rapporteur | Name | Wanuga, Kevin (RAN1); Uesaka, Kazuyoshi (RAN4) |
|  | Company | Nokia, Ericsson |
|  | Email | kevin (dot) wanuga (at) nokia (dot) com; kazuyoshi (dot) uesaka (at) ericsson (dot) com |

## 1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

## 2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

## 2.1	RAN1

#### 2.1.1	Agreements

Agreed to update the field description for existing capability supportedModulationOrderDL, allowing applying the existing UE reported parameter value (supportedModulationOrderDL) with 1024-QAM scaling factor.

#### 2.1.2	Remaining Open issues

## 2.2	RAN2

#### 2.2.1	Agreements

Agreed with the Stage-2 CR for DL 1024QAM in FR1.

Agreed with the CRs on the RRC signalling to support DL 1024QAM in FR1.

#### 2.2.2	Remaining Open issues

## 2.3	RAN3

#### 2.3.1	Agreements

#### 2.3.2	Remaining Open issues

## 2.4	RAN4

#### 2.4.1	Agreements

RAN4#101-bis-e

The following agreements are made on RF performance part in RAN4#101-bis-e (R4-2203103):

- For total power dynamic range (TPDR), test 1024QAM if no power backoff. In the case of 1024QAM with power backoff, 256QAM shall be tested if supported with full power. In case also 256QAM is with power backoff then 64QAM shall be tested (at full output power)

- For error vector magnitude (EVM), test 1024QAM if no power backoff. In the case of 1024QAM with power backoff, 256QAM shall in addition be tested. In case also 256QAM is with power backoff then in addition 64QAM shall be tested (at full output power).

- Endorsed the draft CRs for BS RF performance parts to TS38.141-1, TS38.141-2, TS37.145-1, TS37.145-2, and TS37.141.

The following agreements are made on UE demodulation performance part in RAN4#101-bis-e (R4-2203044).

- PDSCH demodulation

  - Assume Tx EVM 2.5% to derive UE demodulation requirements for DL 1024QAM.

  - Set MCS23 Rank 1.

  - Schedule PDSCH in the special slots for TDD SCS 30kHz.

- SDR tests

  - Discuss further the set MCS indexes for SDR tests.

- CQI reporting tests

  - Assume Tx EVM 2.5% to derive SNR test points for CQI reporting requirements for DL 1024QAM.

  - Discuss further the SNR test points corresponding to CQI index 14.

RAN4#102-e

The following agreements are made on RF performance part in RAN4#102-e (R4-2207430):

- Agreed with the CRs for BS RF performance parts to TS38.141-1, TS38.141-2, TS37.145-1, TS37.145-2, and TS37.141.

The following agreements are made on UE demodulation performance part in RAN4#102-e (R4-2207254).

- PDSCH demodulation

  - Set TDLD30-5

  - Add 1dB additional margin on top of the average of companies’ impairment results for PDSCH 1024QAM.

- SDR tests

  - Set the practical MCS indexes with table 4 (TS38.214 Table 5.1.3.1-4) for SDR test for 2Rx as follows:

    - Rank 1: MCS23

    - Rank 2: Not define SDR test

  - Set the practical MCS indexes with table 4 (TS38.214 Table 5.1.3.1-4) for SDR test for 4Rx as follows:

    - Rank 1: MCS24

    - Rank 2: MCS23

- CQI reporting tests

  - Discuss the final CQI definition requirements with CQI table 4 considering the impairment margin and SNR levels provided by TE with the assumption of Tx EVM 2.5%.

    - This is the common understanding SNR corresponding to CQI index 14 for 1024QAM table is around 28-30dB for 2Rx UE without impairment margin.

    - This is the common understanding SNR corresponding to CQI index 14 for 1024QAM table is around 26-28dB for 4Rx UE without impairment margin.

#### 2.4.2	Remaining Open issues

RF performance part

- CR for RMC for UE RF tests

UE demodulation requirement part

- Agree with the final SNR test points for PDSCH demodulation and CSI reporting requirements.

- CR for UE demodulation and CQI reporting requirements.

## 2.5	RAN5

#### 2.5.1	Agreements

#### 2.5.2	Remaining Open issues

#### 2.5.3	Remaining Open issues with cross-WG dependencies

## 2.6	RAN6

#### 2.6.1	Agreements

#### 2.6.2	Remaining Open issues

## 3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

## 3.1	SAx/CTs

#### 3.1.1	Agreements with cross-TSG impacts

#### 3.1.2	Remaining Open issues with cross-TSG impacts

## 4.	References

RAN1#108-e

- R1-2202927	“LS on updated Rel-17 RAN1 UE features list for NR”, RAN1.

RAN2#117-e

- R2-2203322	"Introduction of DL 1024QAM for NR”, Ericsson, Nokia, Nokia Shanghai Bell.

- R2-2203962	"Introduction of DL 1024QAM for NR”, Ericsson, Nokia, Nokia Shanghai Bell.

- R2-2203323	"Introduction of DL 1024QAM for NR”, Ericsson, Nokia, Nokia Shanghai Bell.

- R2-2203963	"Introduction of DL 1024QAM for NR”, Ericsson, Nokia, Nokia Shanghai Bell.

RAN4#101-bis-e

- R4-2200134	"Discussion on BS RF conformance testing for 1024QAM for NR FR1", CATT.

- R4-2200135	"Draft CR for 37.145-1 on BS RF conformance testing for 1024QAM for NR FR1", CATT.

- R4-2201458	"Draft CR to TS 38.141-1: Introduction of 1024 QAM in FR1", ZTE Corporation.

- R4-2201489	"CR to TS 38.141-2: Introduction of 1024 QAM in FR1", Ericsson.

- R4-2201514	"Test procedure for support of 1024QAM", Huawei, HiSilicon.

- R4-2201664	"On 1024QAM for NR BS conformance tests", Nokia, Nokia Shanghai Bell.

- R4-2201665	"Draft CR to TS 37.145-2 with 1024QAM introduction", Nokia, Nokia Shanghai Bell.

- R4-2202995	"Draft CR for 37.145-1 on BS RF conformance testing for 1024QAM for NR FR1", CATT.

- R4-2202996	"Draft CR to TS 38.141-1: Introduction of 1024 QAM in FR1", ZTE.

- R4-2202998	"draftCR to TS 38.141-2: Introduction of 1024 QAM in FR1", Ericsson.

- R4-2202999	"Draft CR to TS 37.145-2 with 1024QAM introduction", Nokia.

- R4-2203000	"Draft CR for 37.141 on BS RF conformance testing for 1024QAM for NR FR1", CATT.

- R4-2202961	"Email discussion summary for [101-bis-e][305] NR_DL1024QAM_RF", Moderator (Ericsson).

- R4-2203103	"Email discussion summary for [101-bis-e][305] NR_DL1024QAM_RF", Moderator (Ericsson).

- R4-2202962	"Email discussion summary for [101-bis-e][312] NR_DL1024QAM_Demod_NWM", Moderator (Ericsson).

- R4-2203104	"Email discussion summary for [101-bis-e][312] NR_DL1024QAM_Demod_NWM", Moderator (Ericsson).

- R4-2201427	"Summary of PDSCH simulation results for DL 1024QAM in FR1", Ericsson.

- R4-2201719	"Simulation Results and views on 1024QAM FR1 UE Demod Tests", Qualcomm Incorporated.

- R4-2203044	"Way forward for NR DL1024QAM demodulation and CQI reporting requirements", Ericsson.

- R4-2200265	"Discussion on PDSCH demod requirements with 1KQAM", Apple.

- R4-2200376	"Discussion on the PDSCH requirements for 1024QAM", MediaTek inc.

- R4-2201011	"Discussion and simulation results on 1024QAM PDSCH", Huawei, HiSilicon.

- R4-2201428	"UE demodulation requirements for DL 1024QAM", Ericsson.

- R4-2201783	"Discussion on PDSCH requirements for DL 1024QAM in FR1", Intel Corporation.

- R4-2202980	"UE demodulation requirements for DL 1024QAM", Ericsson.

- R4-2200266	"Discussion on SDR requirements with 1KQAM", Apple.

- R4-2201012	"Discussion and simulation results on 1024QAM SDR", Huawei, HiSilicon.

- R4-2201429	"SDR requirements for DL 1024QAM", Ericsson.

- R4-2201784	"Discussion on SDR requirements for DL 1024QAM in FR1", Intel Corporation.

- R4-2201013	"Discussion and simulation results on 1024QAM CSI", Huawei, HiSilicon.

- R4-2201430	"CQI reporting requirements for DL 1024QAM", Ericsson.

RAN4#102-e

- R4-2203969	“CR for 37.145-1 on BS RF conformance testing for 1024QAM for NR FR1”, CATT.

- R4-2203970	“CR for 37.141 on BS RF conformance testing for 1024QAM for NR FR1”, CATT.

- R4-2205019	“CR to TS 38.141-2: Introduction of 1024 QAM in FR1”, Ericsson.

- R4-2205195	“CR to TS 37.145-2 with 1024QAM introduction”, Nokia, Nokia Shanghai Bell.

- R4-2205463	“CR to TS 38.141-1: Introduction of 1024 QAM in FR1”, ZTE Corporation.

- R4-2207189	“CR to TS 38.141-2: Introduction of 1024 QAM in FR1”, Ericsson.

- R4-2207157	“Email discussion summary for [102-e][307] NR_DL1024QAM_RF”, Moderator (Ericsson).

- R4-2207430	“Email discussion summary for [102-e][307] NR_DL1024QAM_RF”, Moderator (Ericsson).

- R4-2205085	“Update work plan for UE demodulation requirements for DL 1024QAM for NR FR1”, Ericsson.

- R4-2206075	“PDSCH Impairment results and other requirements for 1024QAM FR1 UE Demod Tests”, Qualcomm Incorporated.

- R4-2203760	“Discussion on PDSCH demod requirements with 1KQAM”, Apple.

- R4-2205086	“Summary of PDSCH simulation results for DL 1024QAM in FR1”, Ericsson.

- R4-2205087	“UE demodulation requirements for DL 1024QAM”, Ericsson.

- R4-2205748	“Discussion and simulation results on 1024QAM PDSCH”, Huawei, HiSilicon.

- R4-2205904	“Draft CR to TS38.101-4, PDSCH requirements for 1024QAM in FR1 FDD”, MediaTek inc.

- R4-2205905	“Discussion on the PDSCH requirements for 1024QAM”, MediaTek inc.

- R4-2206001	“Simulation results for 1024QAM PDSCH”, Intel Corporation.

- R4-2203761	“Discussion on SDR requirements with 1KQAM”, Apple.

- R4-2205088	“SDR requirements for DL 1024QAM”, Ericsson.

- R4-2205749	“Discussion and simulation results on 1024QAM SDR”, Huawei, HiSilicon.

- R4-2206002	“Discussion on SDR requirements for 1024QAM”, Intel Corporation.

- R4-2205089	“CQI reporting requirements for DL 1024QAM”, Ericsson.

- R4-2205750	“Discussion and simulation results on 1024QAM CSI”, Huawei, HiSilicon.

- R4-2205751	“Draft CR on FDD CQI reporting cases for 1024QAM and CSI RMC (TS38.101-4)”, Huawei, HiSilicon

- R4-2205906	“Discussion on the CQI requirements for 1024QAM”, MediaTek inc.

- R4-2207158	“Email discussion summary for [102-e][318] NR_DL1024QAM_Demod”, Moderator (Ericsson).

- R4-2207431	“Email discussion summary for [102-e][318] NR_DL1024QAM_Demod”, Moderator (Ericsson).

- R4-2207254	“Way forward for NR DL1024QAM demodulation and CQI reporting”, Ericsson.

10.01.2022		minor adaptations for RAN #95e

04.10.2021		minor adaptations for RAN #94e

08.08.2021		minor adaptations for RAN #93e

17.05.2021		minor adaptations for RAN #92e

28.01.2021		minor adaptations for RAN #91e

09.11.2020		minor adaptations for RAN #90e

31.08.2020		minor adaptations for RAN #89e

20.04.2020		minor adaptations for RAN #88e

18.02.2020		minor adaptations for RAN #87e

14.11.2019		minor adaptations for RAN #86

18.08.2019		minor adaptations for RAN #85

12.05.2019		minor adaptations for RAN #84

27.02.2019		minor adaptations for RAN #83

21.11.2018		completion levels with colours added (for RAN #82)

v04.81	31.07.2018		simplification of template and addition of cross-TSG aspects (for RAN #81)

v04.80	21.05.2018		minor adaptations for RAN #80

v04.79	26.02.2018		minor adaptations for RAN #79

v04.78	18.11.2017		minor adaptations for RAN #78

v04.77	06.08.2017		minor adaptations for RAN #77

v04.76	15.05.2017		minor adaptations for RAN #76

v04.75	31.01.2017		minor adaptations for RAN #75

v04.74	28.10.2016		minor adaptations for RAN #74

v04.73	01.09.2016		adaptations for RAN #73 (time units in extra Excel table, RAN6 reporting included)

v04.72	26.05.2016		adaptations for RAN #72 (introduction of NR & GERAN TUs)

v04.71	10.02.2016		minor adaptations for RAN #71

v04.70	30.10.2015		minor adaptations for RAN #70

v04.69	12.08.2015		minor adaptations for RAN #69

v04.68	21.05.2015		minor adaptations for RAN #68

v04.67	01.02.2015		minor adaptations for RAN #67

v04.66	16.11.2014		minor adaptations for RAN #66

v04.65	16.08.2014		minor adaptations for RAN #65

v04.64	22.05.2014		minor adaptations for RAN #64

v04.63	24.01.2014		restructuring for RAN #63 to cover Core & Perf. in one doc file

v03.62	11.11.2013		section 1.2.3 adapted for RAN #62

v03	11.08.2013		section 1.2.3 added on time budget

v02	07.05.2010		history added, some spelling corrections

v01	13.11.2009		First version of the template