3GPP TSG RAN Meeting #95e								RP-220191

Electronic Meeting, March 17-23, 2022

Agenda Item:		9.5.4.4

Source:			Ericsson

Title:				WI Summary for Introduction of DL 1024QAM for NR FR1

WI code(s):			NR_DL1024QAM_FR1

leading WG:		RAN4

Release:			Rel-17

### 1	Introduction

This work item [1] has specified downlink 1024QAM for NR PDSCH operation in FR1, which provides higher downlink peak rate compared with Release-15 NR, with the high downlink SINR and better channel condition (e.g., LOS or LOS-like channel), and with no mobility or very low mobility environment.

This WI specified the downlink 1024QAM mapping function, and the corresponding MCS/CQI tables and RRC signalling. This WI also specified the corresponding BS/UE RF requirements to transmit/receive the signals with 1024QAM [2].

### 2	Description

Introduction of the modulation mapping function of 1024QAM

It was introduced the modulation mapping function of 1024QAM for PDSCH, where 10 tuples of bits, bi, …, bi+9 are mapped to complex-valued modulation symbols d(i), according to

di=16821-2bi16-1-2bi+28-1-2bi+44-1-2bi+62-1-2bi+8+j16821-2bi+116-1-2bi+38-1-2bi+54-1-2bi+72-1-2bi+9

Introduction of MCS table supporting 1024QAM

For supporting 1024QAM for PDSCH, a new five-bit MCS table with 1024QAM entries was introduced by removing 5 MCS entries and adding 5 new entries for 1024QAM from the existing MCS index table 2. The removed MCS indexes are 2, 4, 6, 8, and 10. The added MCS entries for 1024QAM are given as follows:

| MCS index | Modulation | Target code rate R x [1024] | Spectrum efficiency |
| --- | --- | --- | --- |
| 23 | 10 | 805.5 | 7.8662 |
| 24 | 10 | 853 | 8.3301 |
| 25 | 10 | 900.5 | 8.7939 |
| 26 | 10 | 948 | 9.2578 |
| 31 | 10 | reserved |  |

New RRC signalling was also introduced to indicate use of 1024QAM MCS table.

Introduction of CQI table supporting 1024QAM

For supporting 1024QAM for PDSCH, a new CQI table with 1024QAM entries was introduced by reusing LTE CQI table with 1024QAM entries as follows:

| CQI index | Modulation | Code rate x 1024 | Efficiency |
| --- | --- | --- | --- |
| 0 | Out of range |  |  |
| 1 | QPSK | 78 | 0.1523 |
| 2 | QPSK | 193 | 0.377 |
| 3 | QPSK | 449 | 0.877 |
| 4 | 16QAM | 378 | 1.4766 |
| 5 | 16QAM | 616 | 2.4063 |
| 6 | 64QAM | 567 | 3.3223 |
| 7 | 64QAM | 666 | 3.9023 |
| 8 | 64QAM | 772 | 4.5234 |
| 9 | 64QAM | 873 | 5.1152 |
| 10 | 256QAM | 711 | 5.5547 |
| 11 | 256QAM | 797 | 6.2266 |
| 12 | 256QAM | 885 | 6.9141 |
| 13 | 256QAM | 948 | 7.4063 |
| 14 | 1024QAM | 853 | 8.3301 |
| 15 | 1024QAM | 948 | 9.2578 |

New RRC signalling was also introduced to indicate use of 1024QAM CQI table.

BS Tx EVM for DL 1024QAM

The Error Vector Magnitude (EVM) is a measure of the difference between the ideal symbols and the measured symbols after the equalization. The required gNB Tx EVM for 1024QAM is set to 2.5% for frequencies equal to or below 4.2GHz and 2.8% for frequencies above 4.2GHz in FR1.

### 3	References

- RP-213654	“Revised WID on Introduction of DL 1024QAM for NR FR1”, Nokia.

- RP-220190	“Status report for WI Introduction of DL 1024QAM for NR FR1; rapporteur: Ericsson, Nokia”, Ericsson.