3GPP TSG RAN Meeting #95-e							RP-220201

Electronic Meeting, March 17 - 23, 2022

Source:	Samsung

Title:	On the status of Rel-17 NR sidelink enhancement work in RAN WG1

Agenda item:	9.5.1.3

Document for:	Discussion and Decision

- Introduction

RAN1 had worked for Rel-17 NR sidelink enhancement until RAN1#108-e [1] in order to complete remaining issues by RAN guidance made in RAN#94-e [2] as:

- RAN1 is tasked to complete the remaining normative work for Rel-17 NR sidelink enhancement by Q1 of 2022

  - All RAN1 decisions that impact other WGs should be finalized in RAN1#107bis-e

- Use the list of open issues provided RP-212880 (status report of WI: NR sidelink enhancement) as a starting point for technical discussions in RAN1.

  - This does not mean that all the issues included in the list are considered essential or the list is complete

  - RAN1 should not spend additional effort to further refine the list

According to the outcome achieved in RAN1#108-e meeting [1], this contribution provides our view on the status of Rel-17 NR sidelink enhancement in RAN1 point of view.

- Discussion

Rel-17 NR sidelink enhancement includes two sub-agenda items. One is on inter-UE coordination for Mode 2 enhancement and the other is about resource allocation for power saving [3]. For the inter-UE coordination, all remaining issues for Scheme 2 captured in the status report [4] were completed in RAN1#108-e [1]. On the other hand, as shown in the Appendix, Scheme 1 has still a couple of open issues which need to be discussed in further RAN1 meeting(s). For the resource allocation for power saving, all other issues captured in the status report [4] were completed except for the proposal to specify a mechanism to report resource(s) within sidelink DRX active time. It was intensively discussed in RAN1#108-e [1] but could not be endorsed due to sustained objections.

One way to move forward in RAN1 is to take the majority, i.e., specifying the mechanism in Rel-17 NR sidelink but it requires further discussion on the details in future RAN1 meeting(s). The other way is to leave it to UE implementation but system performance of Rel-17 sidelink communication with DRX might be degraded because the probability of resource collision may increase.

- Conclusion

This contribution presents the current status of the Rel-17 NR sidelink enhancement work item in RAN1 point of view. Based on discussion, we provide the following observations.

Observation 1: There are a few open issues that need to be handled, but Rel-17 NR sidelink enhancement WI can be declared complete from RAN1 point of view. The open issues can be resolved in future meetings as part of maintenance.

Observation 2: One issue that might be discussed in RAN#95-e is the proposal to specify a mechanism to report resource(s) within sidelink DRX active time. It was intensively discussed in RAN1#108e but was not endorsed due to sustained objections.

Proposal: RAN guidance may be necessary for RAN1 to determine whether or not to specify a resource reporting mechanism within sidelink DRX active time in Rel-17 NR sidelink enhancement.

- Appendix

In this Section, we provide the list of FFS (marked in yellow) from RAN1#108-e [1].

Agreement

- For UE-B’s behavior when UE-B receives multiple preferred resource sets from the same UE-A

  - It is up to UE-B implementation to use one or multiple of them in its resource (re)selection

- Conclusion: UE-B’s behavior when UE-B receives multiple non-preferred resource sets from the same UE-A

  - No RAN1 specification change to TS38.214 is deemed necessary in RAN1#108-e

- For UE-B’s behavior when UE-B receives both a single preferred resource set and a single non-preferred resource set from the same UE-A

  - FFS: It is up to UE-B implementation to use one or multiple of them in its resource (re)selection

Agreement

- For UE-B’s behavior when UE-B receives multiple preferred resource sets from the different UE-As,

  - UE-B uses each received preferred resource set for its resource selection for each TB to be transmitted to each UE-A providing the preferred resource set.

- Conclusion: UE-B’s behavior when UE-B receives multiple non-preferred resource sets from the different UE-As.

  - No RAN1 specification change to TS38.214 is deemed necessary in RAN1#108-e (except for the processing timeline)

- For UE-B’s behavior when UE-B receives both a single preferred resource set and a single non-preferred resource set from the different UE-As,

  - FFS: It is up to UE-B implementation to use one or multiple of them in its resource (re)selection

Agreement

- Notations:

  - (n+T_1) – Start slot of resource selection window for determining the set of resources

    - For inter-UE coordination information triggered by UE-B’s explicit request, this value of (n+T_1) is provided by UE-B’s request as per the existing agreement

    - For inter-UE coordination information triggered by a condition other than explicit request reception, this value of (n+T_1) is determined by UE-A’s implementation as per the existing agreement

  - (n+T_2) – End slot of resource selection window for determining the set of resources

    - For inter-UE coordination information triggered by UE-B’s explicit request, this value of (n+T_2) is provided by UE-B’s request as per the existing agreement

    - For inter-UE coordination information triggered by a condition other than explicit request reception, this value of (n+T_2) is determined by UE-A’s implementation as per the existing agreement

  - (n’+T’_1) – Start slot of resource selection window used for sidelink transmission carrying inter-UE coordination information

  - (n’+T’_2) – End slot of resource selection window used for sidelink transmission carrying inter-UE coordination information

  - n' is the slot where UE procedure of determining TX resources of sidelink transmission carrying inter-UE coordination information is triggered

- For inter-UE coordination information triggered by UE-B’s explicit request

  - Alt 1-1:

    - X1 ≤ (n’+T’_1)

    - (n’+T’_2) ≤ X2

- For inter-UE coordination information triggered by a condition other than explicit request reception,

  - Alt 2-2:

    - (n’+T’_2) < X3

- FFS: Values for X1, X2, X3

- References

- 3GPP TSG RAN1#108-e, Final Report of 3GPP TSG RAN.

- 3GPP TSG RAN#94-e, Final Report of 3GPP TSG RAN.

- RP‑202846, “WID revision: NR Sidelink enhancement,” RAN#90-e, Dec., 2020.

- RP-212880, Status Report of WI: NR sidelink enhancement, LG Electronics.