3GPP TSG RAN Meeting #95-e	RP-220227

# e-Meeting, March 17 - 23, 2022

Source:	TCL Communication

Title:	Discussion on XR enhancements for NR

Agenda item:	9.2.8

Document for:	Discussion

- Introduction

At RAN #94-e, the Rel-18 Study Item on XR Enhancements for NR was approved. The objectives of this study item are as follows:

Objectives on XR-awareness in RAN (RAN2): - Study and identify the XR traffic (both UL and DL) characteristics, QoS metrics, and application layer attributes beneficial for the gNB to be aware of. - Study how the above information aids XR-specific traffic handling. Objectives on XR-specific Power Saving (RAN1, RAN2): - Study XR specific power saving techniques to accommodate XR service characteristics (periodicity, multiple flows, jitter, latency, reliability, etc...). Focus is on the following techniques: - C-DRX enhancement. - PDCCH monitoring enhancement. Objectives on XR-specific capacity improvements (RAN1, RAN2): - Study mechanisms that provide more efficient resource allocation and scheduling for XR service characteristics (periodicity, multiple flows, jitter, latency, reliability, etc…). Focus is on the following mechanisms: - SPS and CG enhancements; - Dynamic scheduling/grant enhancements.

In this contribution we will discuss the objective in more detail and provide our considerations on them.

- Discussion

  - XR service characteristics and XR awareness in RAN

It is well known that XR service has some special characteristics, such as high data rate, low latency, real-time interaction. Modelling of the XR service characteristics can facilitate identifying XR-awareness parameters for RAN and 5GC to support XR service better and efficiently.

During the discussion of the SI establishment in RAN, many models of the characteristics for the XR service were proposed, some of them are in common and included in the objective and the scope of the SI, and others need further study and discussion due to different views from companies, such as non-integer periodicity with jitter, multiple flows with association (e.g. multiple flow synchronization), variable packet size, latency sensitive, high reliability but unequal importance inter/intra flows, etc.

In the recent SA2 meeting, some models of XR service characteristics were also discussed and some conclusions were reached as frame and GOP, periodic and jitter, and frame importance, PDU set granularity, traffic correlation.

Based on modelling the characteristics of XR service, some XR-awareness parameters could be defined. Generally, according to the means used to provide those parameters to RAN, the XR-awareness parameters could be divided into two categories, i.e., control plane parameters and user plane parameters which are provided to RAN over control plane and user plane respectively. And the control plane parameters could further comprise QoS parameters or service configuration parameters which are provide by means of QoS configuration or service configuration information in the XR service session establishment or reconfiguration procedure.

- QoS parameters

Obviously, the XR service characteristics of jitter requirement, latency requirement, reliability requirement and packet size should be expressed as QoS parameters. However, compared with 5QI QoS parameters which are based on QoS Flow granularity and packet, the QoS parameters for XR service should be based on PDU set granularity (e.g., video/audio frame/tile, Application Data Units etc.) and packet and some new QoS parameters may be needed.

- Service configuration parameters

The XR service characteristics of periodicity, number of flows, importance and correlation inter flows are service specific and could be stable during the XR service session. All those parameters should be configured in the XR service session establishment or reconfiguration procedure.

- User plane parameters

For XR services, the characteristic of PDU Set is a very important since a group of packets would be used to carry payloads of a PDU Set (e.g. a frame, video frame/slice/tile). In application layer, packets in such a PDU Set should be handled as a whole, i.e., the groups of packets within the PDU Set have inherent dependency on each other. For example, the frame/video/tile slice could only be decoded and decompressed in case all of the packets carrying the frame/video slice are successfully received. P-frame could only be decoded and decompressed in case of the previous I-frame or P-fame are decoded and decompressed successfully.

Since XR service is characterized by high data rate, low latency, it’s more possible to cause congestion in RAN. RAN should schedule radio resources for the packets transmission efficiently taking the inherent dependency of the packets of a PDU Set. For example, RAN only drops the obsolete packets or the least important packet in congestion state. To meet this kind or requirement and improve the resource efficiency, one potential solution is to provide the PDU Set SN and the packet SN within a PDU Set to RAN. Obviously, it is more suitable to provide those SN by user plane since the SN varies at every packet and PDU Set.

Moreover, the PDU Set SN and packet SN information are also helpful for the function to support the requirement of multiple flows for the XR service.

Another fact need to be noticed is that the source point of the PDU Set SN and the packet SN usually locates in the application layer while the destination is at the transport layer, so the delivery for those SN is not peer to peer and is cross-layer.

Observation 1: PDU Set and its SN information based scheduling is important for RAN to transmit the packet of XR service efficiently

  - XR-specific Power Saving

From the perspective of packet transmission in RAN, the potential issues identified during Rel-17 SI on XR include:

- non-integer periodicity: For XR service video stream, video frames are always periodic with variable packet size. For example, video stream can have different frame rate, e.g., 60/90/120 fps. As a consequence, the corresponding periodicity is 16.77/11.11/8.33ms and is non-integer;

- unpredictable jitter: due to the different delays caused by network transportation, every packet will arrive at RAN with different jitter and not exactly periodically.

As mentioned above, a fact is that a group of packets would be used to carry payloads of a PDU Set. For example, each compressed video frame is divided into a number of transport units, each of which is encapsulated into a transport packet. That is to say, a group of packets for a frame arrive at RAN and then are transmitted by RAN one by one successively rather than are transmitted as a whole until all packets arrives at RAN. Another fact is that in application layer, the decoding and decompression also treat the group of packets as a whole, so at the client of the service, buffering previously arrived packets for a PDU Set is necessary and some delayed packets could be tolerable.

Based on the issues and facts, the C-DRX configuration and XR-specific C-DRX enhancement in RAN should take periodicity of packet burst of a PDU Set and the random jitter for each packets into consideration. To some extent, extra delay for some packet may be inevitable but is tolerable for XR service.

Observation 2: XR-specific C-DRX enhancement should take periodicity of packet burst of a PDU Set and the random jitter for each packet into consideration, and extra delay for some packets may be inevitable but is tolerable for XR service.

  - XR-specific capacity improvements

Due to the periodic characteristic of XR service, SPS and CG is assumed as the basic transmission solution in RAN for XR service. However, the same potential issues and facts as that for XR-specific Power Saving are also applied to XR-specific capacity improvements. What’s more, the variable frame size of the video frame for XR service could cause either or both variable size packets and variable number of packets for PUD Set. Obviously the current CG and SPS approaches based on semi-static configuration of fixed periodicities and grant sizes are not adequate to support XR traffic with variable packet burst of a PDU set. So the XR-specific capacity improvement based on SPS and CG enhancement should consider not only the periodicity of packet burst of a PDU Set and the random jitter for each packet, but also the variable packet burst of a PDU Set.

Observation 3: The SPS/CG enhancement for XR-specific capacity improvements should take periodicity of variable packet burst of a PDU Set and the random jitter for each packet into consideration, the variable packet burst of a PDU Set may be variable size and/or number of packets.

- Conclusion

In this contribution, we provide our considerations on enhancement for XR and then we have:

Observation 1: PDU Set and its SN information based scheduling is important for RAN to transmit the packet of XR service efficiently

Observation 2: XR-specific C-DRX enhancement should take periodicity of packet burst of a PDU Set and the random jitter for each packet into consideration, and extra delay for some packets may be inevitable but is tolerable for XR service.

Observation 3: The SPS/CG enhancement for XR-specific capacity improvements should take periodicity of variable packet burst of a PDU Set and the random jitter for each packet into consideration, the variable packet burst of a PDU Set may be variable size and/or number of packets.

- References

- RP-213587, “Study on XR Enhancements for NR”, Nokia, RAN#94-e

- S2-2201087, “Key issue on traffic characteristics of XR and media service”, Lenovo, Motorola Mobility, Huawei, HiSilicon, Vivo, CATT，SA2#149e

- S2-2201808, “New Key Issue: PDU Set integrated packet handling”，Huawei, HiSilicon, KDDI, CATT, Ericsson, Futurewei, Intel, InterDigital, Lenovo, Motorola Mobility, Nokia, Nokia Shanghai Bell, OPPO, Tencent, Tencent Cloud, vivo, Deutsche Telekom, China Mobile, SA2#149e

- S2-2201809, “New Key Issue: The differentiated QoS handling considering different importance of media units”, CATT, Huawei, Hisilicon, Nokia, OPPO, Futurewei, Intel, Tencent, Lenovo, vivo, Ericsson, Interdigital Inc., SA2#149e

- S4-220220, 3GPP TR 26.926 V1.1.0 (2022-02)