3GPP TSG-RAN WG3 Meeting #114b-e	R3-221196

# E-meeting, 17-26 Jan 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.413 | CR | 0691 | rev | 2 | Current version: | 16.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Direct data forwarding for 4G to 5G handover |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Samsung, China Telecom, Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | Direct_data_fw_NR-Core |  |  |  |  |  | Date: |  |  | 2022-01-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The following scenario for EPS to 5GS was agreeed for direct data forwarding. - Agree to consider solutions on direct data forwarding from EPS to 5GS in case one DRB in target gNB contains QoS flows mapped to different E-RABs in the source eNB The following solution was agreed at RAN3#114bis: - Solution 1: CU-CP requests one data forwarding address from the CU-UP using the existing signalling. CU-CP feedback the tunnel address to the two E-RABs in Handover Request Ack message to 5GC. With this, the data from the two E-RABs in the source node will be sent to one DRB buffer in the target (ref R3-211957/R3-212545/R3-212356) |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add semantic description that the same DL Forwarding UP TNL Information may correspond to multiple E-RAB IDs. Impact Analysis: Impact assessment towards the previous version of the specification (same release): This CR has isolated impact with the previous version of the specification (same release) because it only impacts the EPC to SA handover with direct data forwarding for dis-aggregated node case. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It remains unclear whether/how the scenario (QoS flows forwarded over multiple E-RAB tunnels will be mapped to a single DRB) is supported in case of EPC to 5GC with remapping when the target NG-RAN node is disaggregated case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.3.1.121 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev0: R3-215362 Rev1: R3-220673 Update the cover page. Rev2: R3-221196 Update the semantic descriptions. |  |  |  |  |  |  |  |  |

Change Begins

9.3.1.121	Data Forwarding Response E-RAB List

This IE is used at inter-system HO to provide DL data forwarding address information, if direct data forwarding is applied.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Data Forwarding Response E-RAB List |  | 1..<maxnoofERABs> |  | The list may include the same DL Forwarding UP TNL Information for multiple E-RABs. |
| >E-RAB ID | M |  | 9.3.2.3 |  |
| >DL Forwarding UP TNL Information | M |  | UP Transport Layer Information 9.3.2.2 |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofE-RABs | Maximum no. of E-RABs. Value is 256. |

<Unchanged Text Omitted>

Change Ends