3GPP TSG-RAN WG1 Meeting #108-e	R1-2202722

# E-meeting, February 21st – March 22nd, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.211 | CR | 0088 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on corrections on SL timing |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Moderator (Nokia), Nokia Shanghai Bell, vivo, ZTE, Sanechips, Ericsson, Huawei, HiSilicon, Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_V2X_NRSL-Core |  |  |  |  |  | Date: |  |  | 2022-02-25 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Clause 8.5 refers to 38.133 and 38.304 but those documents are missing in the list of references in Clause 2. Documents referred in the clause 8.5 do not include running numbers. Statement on obtaining the reference radio frame implicitly based on 4.2 of 38.213 should be replaced with reference to 38.133 12.2.x that contains definitions of all the reference radio frames (GNSS, eNb, gNb or SyncRef UE) specified for SL. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The missing specifications are added to the clause 2. Running numbers are added to the references in clause 8.5. The reference document that specifies SL timing reference radio frames is changed to 38.133. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | There can be ambiguity in timing reference radio frame definition. References to other specifications are unclear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 8.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Isolated impact analysis: This CR clarifies the documents that are referred when SL timing is defined. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 38.201: "NR; Physical Layer – General Description"

[3]	3GPP TS 38.202: "NR; Services provided by the physical layer"

[4]	3GPP TS 38.212: "NR; Multiplexing and channel coding"

[5]	3GPP TS 38.213: "NR; Physical layer procedures for control "

[6]	3GPP TS 38.214: "NR; Physical layer procedures for data "

[7]	3GPP TS 38.215: "NR; Physical layer measurements"

[8]	3GPP TS 38.104: "NR; Base Station (BS) radio transmission and reception"

[9]	void

[10]	3GPP TS 38.306: "NR; User Equipment (UE) radio access capabilities"

[11]	3GPP TS 38.321: "NR; Medium Access Control (MAC) protocol specification"

[12]	3GPP TS 38.133: "NR; Requirements for support of radio resource management”

[13]	3GPP TS 38.304: “NR; User Equipment (UE) procedures in Idle mode and RRC Inactive state”

< Unchanged parts omitted >

## 8.5	Timing

Transmission of a sidelink radio frame number i from the UE shall start (NTA,SL+NTA,offset)∙Tc seconds before the start of the corresponding timing reference frame at the UE. The UE is not required to receive sidelink or downlink transmissions earlier than the value of NTA,offset, which is given in [12, TS 38.133], after the end of a sidelink transmission.

For sidelink transmissions:

If the UE has a serving cell fulfilling the S criterion according to clause 8.2 of [13, TS 38.304]

-	The timing of reference radio frame i equals that of downlink radio frame i in the cell with the same uplink carrier frequency as the sidelink and

-	NTA,offset is given by clause 4.3.1 of [TS 38.211],

Otherwise

-	The timing of reference radio frame i isgiven by clause 12.2.2, 12.2.3, 12.2.4 or 12.2.5 of [12, TS 38.133] and

-	NTA,offset=0.

Figure 8.5-1: Sidelink timing relation

The quantity NTA,SL equals to 0.