3GPP TSG-RAN WG1 Meeting #108-e	R1-2202965

e-Meeting, February 21 – March 3, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.212 | CR | 0099 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections on enhanced IIoT and URLLC in 38.212 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_IIOT_URLLC_enh-Core |  |  |  |  |  | Date: |  |  | 2022-03-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Capture endorsed TP and agreements on HARQ-ACK enhancements and intra-UE multipelxing from RAN1#107b-e meeting and RAN1#108-e meeting. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Reflect agreement on multiplexing high-priority HARQ-ACK and low-priority HARQ-ACK on PUCCH Format 2. - Reflect agreement on multiplexing high-priority SR, high-priority HARQ-ACK and low-priority HARQ-ACK on PUCCH. - Reflect agreement on UCI with different priorities on PUSCH without UL-SCH. - Reflect agreement on multiplexing high-priority HARQ-ACK and low-priority HARQ-ACK on PUSCH with CSI including a single part. - Reflect agreement on HARQ-ACK enhancement related to DCI fields. - Capture text proposal 3-2 in section 1.1.1 of R1-2200692 for Clauses 7.3.1.1. - Reflect agreement on multiplexing HARQ-ACK of a single priority onto a PUSCH of a different priority. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The specification for Rel-17 enhanced IIoT and URLLC is incomplete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3.3.1, 6.3.1, 6.3.1.1.4, 6.3.1.4.3, 6.3.1.4.4, 6.3.1.6, 6.3.2, 6.3.2.1.5, 6.3.2.4.1.6, 6.3.2.4.2.6, 6.3.2.7, 7.3.1.1.1, 7.3.1.2.2, 7.3.1.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 38.213, TS 37.213 |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

#### 5.3.3.1	Encoding of 1-bit information

For , the code block is encoded according to Table 5.3.3.1-1, where  and  is the modulation order for the code block.

Table 5.3.3.1-1: Encoding of 1-bit information

|  | Encoded bits |
| --- | --- |
| 1 |  |
| 2 |  |
| 4 |  |
| 6 |  |
| 8 |  |

The "x" and "y" in Table 5.3.3.1-1 are placeholders for Clause 6.3.1.1, 6.3.2.5.1, 6.3.2.6.1 of [4, TS 38.211] to scramble the information bits in a way that maximizes the Euclidean distance of the modulation symbols carrying the information bits.

< Unchanged parts are omitted >

## 6.3	Uplink control information

### 6.3.1	Uplink control information on PUCCH

The procedure in this clause applies to PUCCH formats 2/3/4.

The following clauses 6.3.1.2, 6.3.1.3 and 6.3.1.5 apply regardless of whether the higher layer parameter UCI-MuxWithDifferentPriority is configured or not. The following clauses 6.3.1.1, 6.3.1.4 and 6.3.1.6 apply by assuming UCI-MuxWithDifferentPriority is not configured, or UCI-MuxWithDifferentPriority is configured and the UCIs for transmission on a PUCCH are of the same priority index, unless stated otherwise.

If the UE is configured with a PUCCH-SCell, UCI-MuxWithDifferentPriority is replaced by UCI-MuxWithDifferentPriority-secondaryPUCCHgroup for the secondary PUCCH group in this clause.

#### 6.3.1.1	UCI bit sequence generation

< Unchanged parts are omitted >

##### 6.3.1.1.4	UCI with different priority indexes

If UCI-MuxWithDifferentPriority is configured, and HARQ-ACK bits associated with priority index 0, HARQ-ACK bits associated with priority index 1, and SR associated with prioritiy index 1 if any are transmitted on a PUCCH, two UCI bit sequences are generated, a0(1), a1(1), a2(1), a3(1), …,aA(1)-11 and a0(2), a1(2), a2(2), a3(2), …,aA(2)-12, according to the following, where A(1)=OACK-HP+OSR-HP  and A(2)=OACK-LP:

-	the HARQ-ACK bits associated with priority index 1 are mapped to the UCI bit sequence a0(1), a1(1), a2(1), a3(1), …,aOACK-HP-11, where ai(1)=oiACK-HP  for i=0,1,…,OACK-HP-1, the HARQ-ACK bit sequence o0ACK-HP, o1ACK-HP, …, oOACK-HP-1ACK-HP is given by Clause 9.1 of [5, TS 38.213], and OACK-HP is the number of HARQ-ACK bits associated with priority index 1;

-	if there is SR associated with priority index 1 for transmission on the PUCCH, set ai(1)=oi-OACK-HPSR-HP for  i=OACK-HP,OACK-HP+1,…,OACK-HP+OSR-HP-1, where the SR bit sequence o0SR-HP, o1SR-HP, …,oOSR-HP-1SR-HP  is given by Clause 9.2.5.1 of [5, TS 38.213]; if there is no SR associated with priority index 1 for transmission on the PUCCH, set OSR-HP=0;

-	the HARQ-ACK bits associated with priority index 0 are mapped to the UCI bit sequence a0(2), a1(2), a2(2), a3(2), …,aOACK-LP-12, where ai(2)=oiACK-LP  for i=0,1,…,OACK-LP-1, the HARQ-ACK bit sequence o0ACK-LP, o1ACK-LP, …, oOACK-LP-1ACK-LP is given by Clause 9.1 of [5, TS 38.213], and OACK-LP is the number of HARQ-ACK bits associated with priority index 0.

< Unchanged parts are omitted >

#### 6.3.1.4	Rate matching

< Unchanged parts are omitted >

##### 6.3.1.4.3	UCI with different priority indexes encoded by Polar code

The following procedure in this clause 6.3.1.4.3 applies if UCI-MuxWithDifferentPriority is configured, and HARQ-ACK bits associated with priority index 0, HARQ-ACK bits associated with priority index 1, and SR associated with prioritiy index 1 if any are transmitted on a PUCCH.

The input bit sequence to rate matching is dr0,dr1,dr2,dr3,…,dr(Nr-1) where r is the code block number, and Nr is the number of coded bits in code block number r.

Table 6.3.1.4.3-1: Rate matching output sequence length EUCI for UCIs with different priority indexes

| UCIs for transmission on a PUCCH | UCI for encoding | Value of EUCI |
| --- | --- | --- |
| HARQ-ACK of priority index 1, HARQ-ACK of priority index 0 | HARQ-ACK of priority index 1 | EUCI=minEtot,OACK-HP+L/RUCImax-HP/Qm∙Qm |
|  | HARQ-ACK of priority index 0 | EUCI=Etot-minEtot,OACK-HP+L/RUCImax-HP/Qm∙Qm |
| HARQ-ACK of priority index 1, SR of priority index 1, HARQ-ACK of priority index 0 | HARQ-ACK of priority index 1, SR of priority index 1 | EUCI=minEtot,OACK-HP+OSR-HP+L/RUCImax-HP/Qm∙Qm |
|  | HARQ-ACK of priority index 0 | EUCI=Etot-minEtot,OACK-HP+OSR-HP+L/RUCImax-HP/Qm∙Qm |

Rate matching is performed according to Clause 5.4.1 by setting IBIL=1 and the rate matching output sequence length to Er=EUCICUCI, where CUCI is the number of code blocks for UCI determined according to Clause 6.3.1.2.1 and the value of EUCI is given by Table 6.3.1.4.3-1:

-	OACK-HP is the number of bits for HARQ-ACK associated with priority index 1 for transmission on the current PUCCH;

-	OSR-HP is the number of bits for SR associated with priority index 1 for transmission on the current PUCCH;

-	if A≥360, L=11; otherwise, L is the number of CRC bits determined according to clause 6.3.1.2.1, where A equals OACK-HP for the case of "HARQ-ACK of priority index 1, HARQ-ACK of priority index 0", and equals OACK-HP+OSR-HP for the case of "HARQ-ACK of priority index 1, SR of priority index 1, HARQ-ACK of priority index 0" respectively in Table 6.3.1.4.3-1;

-	RUCImax-HP is the configured maximum PUCCH coding rate of priority index 1;

-	Etot is given by Table 6.3.1.4-1.

The output bit sequence after rate matching is denoted as fr0,fr1,fr2,…,fr(Er-1) where Er is the length of rate matching output sequence in code block number r.

##### 6.3.1.4.4	UCI with different priority indexes encoded by channel coding of small block lengths

The following procedure in this clause 6.3.1.4.4 applies if UCI-MuxWithDifferentPriority is configured, and HARQ-ACK bits associated with priority index 0, HARQ-ACK bits associated with priority index 1, and SR associated with prioritiy index 1 if any are transmitted on a PUCCH.

The input bit sequence to rate matching is d0,d1,d2,…,dN-1.

The value of EUCI is determined according to Table 6.3.1.4.3-1 by setting L=0.

Rate matching is performed according to Clause 5.4.3 by setting the rate matching output sequence length E=EUCI.

The output bit sequence after rate matching is denoted as f0,f1,f2,…,fE-1.

< Unchanged parts are omitted >

#### 6.3.1.6	Multiplexing of coded UCI bits to PUCCH

If CSI of two parts or UCIs with different priority indexes are transmitted on a PUCCH, the coded bits corresponding to UCI bit sequence  is denoted by and the coded bits corresponding to UCI bit sequence  is denoted by .

For PUCCH format 2 when UCI-MuxWithDifferentPriority is configured, the coded bit sequence g0,g1,g2,g3,…,gG-1 is generated for UCIs with different priority indexes by setting gi=gi(1) for i=0,1,…,G1-1, and setting gi=gi-G1(2) for i=G1,G1+1,…,G1+G2-1.

For PUCCH format 3/4, the coded bit sequence , where , is generated according to the following.

Table 6.3.1.6-1: PUCCH DMRS and UCI symbols

| PUCCH duration (symbols) | PUCCH DMRS symbol indices | Number of UCI symbol indices sets | 1st UCI symbol indices set | 2nd UCI symbol indices set | 3rd UCI symbol indices set |
| --- | --- | --- | --- | --- | --- |
| 4 | {1} | 2 | {0,2} | {3} | - |
| 4 | {0,2} | 1 | {1,3} | - | - |
| 5 | {0, 3} | 1 | {1, 2, 4} | - | - |
| 6 | {1, 4} | 1 | {0, 2, 3, 5} | - | - |
| 7 | {1, 4} | 2 | {0, 2, 3, 5} | {6} | - |
| 8 | {1, 5} | 2 | {0, 2, 4, 6} | {3, 7} | - |
| 9 | {1, 6} | 2 | {0, 2, 5, 7} | {3, 4, 8} | - |
| 10 | {2, 7} | 2 | {1, 3, 6, 8} | {0, 4, 5, 9} | - |
| 10 | {1, 3, 6, 8} | 1 | {0,2,4,5,7,9} | - | - |
| 11 | {2, 7} | 3 | {1,3,6,8} | {0,4,5,9} | {10} |
| 11 | {1,3,6,9} | 1 | {0,2,4,5,7,8,10} | - | - |
| 12 | {2, 8} | 3 | {1,3,7,9} | {0,4,6,10} | {5, 11} |
| 12 | {1,4,7,10} | 1 | {0,2,3,5,6,8,9,11} | - | - |
| 13 | {2, 9} | 3 | {1,3,8,10} | {0,4,7,11} | {5,6,12} |
| 13 | {1,4,7,11} | 2 | {0,2,3,5,6,8,10,12} | {9} | - |
| 14 | {3, 10} | 3 | {2,4,9,11} | {1,5,8,12} | {0,6,7,13} |
| 14 | {1,5,8,12} | 2 | {0,2,4,6,7,9,11,13} | {3, 10} | - |

Denote  as UCI OFDM symbol index. Denote  as the number of elements in UCI symbol indices set  for , where  and  are given by Table 6.3.1.6-1 according to the PUCCH duration and the PUCCH DMRS configuration. Denote  as the number of OFDM symbols carrying UCI in the PUCCH. Denote  as the modulation order of the PUCCH.

For PUCCH format 3, set NUCIsymbol=12⋅NPRBPUCCH,3/NSFPUCCH,3 , where  is the number of PRBs that is determined by the UE for PUCCH format 3 transmission according to Clause 9.2 of [5, TS 38.213], and NSFPUCCH,3 is the spreading factor for PUCCH format 3 [4, TS 38.211].

For PUCCH format 4, set , where  is the spreading factor for PUCCH format 4.

Find the smallest such that .

Set ;

Set ;

Set ;

Set ;

for  to

if

for  to

for  to

;

;

end for

end for

elseif

if

;

else

;

end if

;

for  to

for  to

;

;

end for

end for

for  to

for  to

;

;

end for

end for

else

for  to

for  to

;

;

end for

end for

end if

end for

Set

for  to

for  to

for  to

;

;

end for

end for

end for

< Unchanged parts are omitted >

### 6.3.2	Uplink control information on PUSCH

The following clauses 6.3.2.2, 6.3.2.3, and 6.3.2.5 apply regardless of whether the higher layer parameter UCI-MuxWithDifferentPriority is configured or not. The following clauses 6.3.2.1, 6.3.2.4, and 6.3.2.6 apply by assuming UCI-MuxWithDifferentPriority is not configured, or UCI-MuxWithDifferentPriority is configured and the UCIs for transmission on a PUSCH are of the same priority index, unless stated otherwise. In addition, clauses 6.3.2.1.4, 6.3.2.4.1.5, 6.3.2.4.2.5 and 6.3.2.6 also apply if UCI-MuxWithDifferentPriority is configured and CG-UCI is of a different priority index with HARQ-ACK.

If the UE is configured with a PUCCH-SCell, UCI-MuxWithDifferentPriority is replaced by UCI-MuxWithDifferentPriority-secondaryPUCCHgroup for the secondary PUCCH group in this clause.

#### 6.3.2.1	UCI bit sequence generation

< Unchanged parts are omitted >

##### 6.3.2.1.5	UCI with different priority indexes

If UCI-MuxWithDifferentPriority is configured, and HARQ-ACK bits associated with priority index 0, and CSI part 1 if any are transmitted on a PUSCH associated with priority index 1, the following UCI bit sequences are generated, a0(1), a1(1), a2(1), a3(1), …,aA(1)-11, and a0(2), a1(2), a2(2), a3(2), …,aA(2)-12  if any, according to the following:

-	If CSI part 1 is also transmitted on the PUSCH,

-	Set ai(1) for i=0,1,…,A(1)-1 as the bit sequence of CSI part 1, where the CSI fields of all CSI reports, in the order from upper part to lower part in Table 6.3.2.1.2-6, are mapped to the UCI bit sequence a0(1), a1(1), a2(1), a3(1), …,aA(1)-11 starting with a0(1).

-	Set  ai(2)=oiACK-LP  for i=0,1,…,OACK-LP-1 and A(2)=OACK-LP, where the HARQ-ACK bit sequence o0ACK-LP, o1ACK-LP, …, oOACK-LP-1ACK-LP associated with priority index 0 is given by Clause 9.1 of [5, TS 38.213].

-	Otherwise, set  ai(1)=oiACK-LP  for i=0,1,…,OACK-LP-1 and A(1)=OACK-LP, where the HARQ-ACK bit sequence o0ACK-LP, o1ACK-LP, …, oOACK-LP-1ACK-LP associated with priority index 0 is given by Clause 9.1 of [5, TS 38.213].

If UCI-MuxWithDifferentPriority is configured, and HARQ-ACK bits associated with priority index 1, and CSI if any are transmitted on a PUSCH associated with priority index 0, the following UCI bit sequences are generated, a0, a1, a2, a3, …,aA-1, a0(1), a1(1), a2(1), a3(1), …,aA(1)-11 if any, and a0(2), a1(2), a2(2), a3(2), …,aA(2)-12  if any, according to the following:

-	If HARQ-ACK bits associated with priority index 1 and CSI are transmitted on the PUSCH without UL-SCH and the CSI includes CSI part 1 without CSI part 2, and there is only one HARQ-ACK bit associated with priority index 1 given by Clause 9.1 of [5, TS 38.213], set a0=o0ACK-HP, a1=0, and A=2; otherwise, set ai=oiACK-HP  for i=0,1,…,OACK-HP-1 and A=OACK-HP, where the HARQ-ACK bit sequence o0ACK-HP, o1ACK-HP, …, oOACK-HP-1ACK-HP associated with priority index 1 is given by Clause 9.1 of [5, TS 38.213];

-	Set ai(1) for i=0,1,…,A(1)-1 as the bit sequence of CSI part 1, if CSI part 1 is also transmitted on the PUSCH, where the CSI fields of all CSI reports, in the order from upper part to lower part in Table 6.3.2.1.2-6, are mapped to the UCI bit sequence a0(1), a1(1), a2(1), a3(1), …,aA(1)-11 starting with a0(1);

-	Set ai(2) for i=0,1,…,A(2)-1 as the bit sequence of CSI part 2, if CSI part 2 is also transmitted on the PUSCH, where the CSI fields of all CSI reports, in the order from upper part to lower part in Table 6.3.2.1.2-7, are mapped to the UCI bit sequence a0(2), a1(2), a2(2), a3(2), …,aA(1)-12 starting with a0(2).

If UCI-MuxWithDifferentPriority is configured, and HARQ-ACK bits associated with priority index 0, HARQ-ACK bits associated with priority index 1, and CSI part 1 if any are transmitted on a PUSCH, the following UCI bit sequences are generated, a0, a1, a2, a3, …,aA-1, a0(1), a1(1), a2(1), a3(1), …,aA(1)-11, and a0(2), a1(2), a2(2), a3(2), …,aA(2)-12  if any, according to the following:

-	Set ai=oiACK-HP  for i=0,1,…,OACK-HP-1 and A=OACK-HP, where the HARQ-ACK bit sequence o0ACK-HP, o1ACK-HP, …, oOACK-HP-1ACK-HP associated with priority index 1 is given by Clause 9.1 of [5, TS 38.213];

-	If CSI part 1 is also transmitted on the PUSCH and the PUSCH is associated with priority index 1,

-	Set ai(1) for i=0,1,…,A(1)-1 as the bit sequence of CSI part 1, where the CSI fields of all CSI reports, in the order from upper part to lower part in Table 6.3.2.1.2-6, are mapped to the UCI bit sequence a0(1), a1(1), a2(1), a3(1), …,aA(1)-11 starting with a0(1).

-	Set  ai(2)=oiACK-LP  for i=0,1,…,OACK-LP-1 and A(2)=OACK-LP, where the HARQ-ACK bit sequence o0ACK-LP, o1ACK-LP, …, oOACK-LP-1ACK-LP associated with priority index 0 is given by Clause 9.1 of [5, TS 38.213].

-	Otherwise,

-	Set  ai(1)=oiACK-LP  for i=0,1,…,OACK-LP-1 and A(1)=OACK-LP, where the HARQ-ACK bit sequence o0ACK-LP, o1ACK-LP, …, oOACK-LP-1ACK-LP associated with priority index 0 is given by Clause 9.1 of [5, TS 38.213].

-	Set  ai(2)=ai(1)  for i=0,1,…,A1-1 and A(2)=A(1), if CSI part 1 is also transmitted on the PUSCH and the PUSCH is associated with priority index 0, where the CSI part 1 sequence a0(1), a1(1), a2(1), a3(1), …,aA(1)-1(1) is given by Table 6.3.2.1.2-6 by replacing a0(1), a1(1), a2(1), a3(1), …,aA(1)-11, and the CSI fields of all CSI reports, in the order from upper part to lower part in Table 6.3.2.1.2-6, are mapped to the CSI part 1 sequence a0(1), a1(1), a2(1), a3(1), …,aA(1)-1(1) starting with a0(1).

< Unchanged parts are omitted >

#### 6.3.2.4	Rate matching

##### 6.3.2.4.1	UCI encoded by Polar code

< Unchanged parts are omitted >

###### 6.3.2.4.1.6	UCI with different priority indexes

If UCI-MuxWithDifferentPriority is configured, and HARQ-ACK bits associated with priority index 0, and CSI part 1 if any are transmitted on a PUSCH associated with priority index 1:

-	If CSI part 1 is also transmitted on the PUSCH,

-	Perform rate matching for CSI part 1 according to clause 6.3.2.4.1.2, by assuming the number of HARQ-ACK information bits to be transmitted on PUSCH in clause 6.3.2.4.1.2 is 0 bit.

-	Perform rate matching for HARQ-ACK with priority index 0 according to clause 6.3.2.4.1.3, by taking HARQ-ACK with priority index 0 as CSI part 2 and replacing βoffsetPUSCH by βoffsetHARQ-ACK-LP, and assuming the number of HARQ-ACK information bits to be transmitted on PUSCH in clause 6.3.2.4.1.3 is 0 bit.

-	Otherwise, perform rate matching for HARQ-ACK with priority index 0 according to clause 6.3.2.4.1.2, by taking HARQ-ACK with priority index 0 as CSI-part 1 and replacing βoffsetPUSCH by βoffsetHARQ-ACK-LP, and assuming the number of HARQ-ACK information bits to be transmitted on PUSCH in clause 6.3.2.4.1.2 is 0 bit.

If UCI-MuxWithDifferentPriority is configured, and HARQ-ACK bits associated with priority index 1, and CSI if any are transmitted on a PUSCH associated with priority index 0:

-	Perform rate matching for HARQ-ACK with priority index 1 according to clause 6.3.2.4.1.1, by taking HARQ-ACK with priority index 1 as HARQ-ACK and replacing βoffsetPUSCH by βoffsetHARQ-ACK-HP.

-	Perform rate matching for CSI part 1 according to clause 6.3.2.4.1.2, by taking HARQ-ACK with priority index 1 as HARQ-ACK, if CSI part 1 is also transmitted on the PUSCH.

-	Perform rate matching for CSI part 2 according to clause 6.3.2.4.1.3, by taking HARQ-ACK with priority index 1 as HARQ-ACK, if CSI part 2 is also transmitted on the PUSCH.

If UCI-MuxWithDifferentPriority is configured, and HARQ-ACK bits associated with priority index 0, HARQ-ACK bits associated with priority index 1, and CSI part 1 if any are transmitted on a PUSCH:

-	Perform rate matching for HARQ-ACK with priority index 1 according to clause 6.3.2.4.1.1, by taking HARQ-ACK with priority index 1 as HARQ-ACK and replacing βoffsetPUSCH by βoffsetHARQ-ACK-HP.

-	If CSI part 1 is also transmitted on the PUSCH and the PUSCH is associated with priority index 1,

-	Perform rate matching for CSI part 1 according to clause 6.3.2.4.1.2, by taking HARQ-ACK with priority index 1 as HARQ-ACK.

-	Perform rate matching for HARQ-ACK with priority index 0 according to clause 6.3.2.4.1.3, by taking HARQ-ACK with priority index 0 as CSI part 2 and replacing βoffsetPUSCH by βoffsetHARQ-ACK-LP, and taking HARQ-ACK with priority index 1 as HARQ-ACK.

-	Otherwise,

-	Perform rate matching for HARQ-ACK with priority index 0 according to clause 6.3.2.4.1.2, by taking HARQ-ACK with priority index 0 as CSI-part 1 and replacing βoffsetPUSCH by βoffsetHARQ-ACK-LP, and taking HARQ-ACK with priority index 1 as HARQ-ACK.

-	Perform rate matching for CSI part 1 according to clause 6.3.2.4.1.3, by taking CSI part 1 as CSI part 2 and replacing βoffsetPUSCH by βoffsetCSI-part1, taking HARQ-ACK with priority index 0 as CSI-part 1 and taking HARQ-ACK with priority index 1 as HARQ-ACK, if CSI part 1 is also transmitted on the PUSCH and the PUSCH is associated with priority index 0.

< Unchanged parts are omitted >

##### 6.3.2.4.2	UCI encoded by channel coding of small block lengths

< Unchanged parts are omitted >

###### 6.3.2.4.2.6	UCI with different priority indexes

If UCI-MuxWithDifferentPriority is configured, and HARQ-ACK bits associated with priority index 0, and CSI part 1 if any are transmitted on a PUSCH associated with priority index 1:

-	If CSI part 1 is also transmitted on the PUSCH,

-	Perform rate matching for CSI part 1 according to clause 6.3.2.4.2.2, by assuming the number of HARQ-ACK information bits to be transmitted on PUSCH in clause 6.3.2.4.2.2 is 0 bit.

-	Perform rate matching for HARQ-ACK with priority index 0 according to clause 6.3.2.4.2.3, by taking HARQ-ACK with priority index 0 as CSI part 2 and replacing βoffsetPUSCH by βoffsetHARQ-ACK-LP, and assuming the number of HARQ-ACK information bits to be transmitted on PUSCH in clause 6.3.2.4.2.3 is 0 bit.

-	Otherwise, perform rate matching for HARQ-ACK with priority index 0 according to clause 6.3.2.4.2.2, by taking HARQ-ACK with priority index 0 as CSI-part 1 and replacing βoffsetPUSCH by βoffsetHARQ-ACK-LP, and assuming the number of HARQ-ACK information bits to be transmitted on PUSCH in clause 6.3.2.4.2.2 is 0 bit.

If UCI-MuxWithDifferentPriority is configured, and HARQ-ACK bits associated with priority index 1, and CSI if any are transmitted on a PUSCH associated with priority index 0:

-	Perform rate matching for HARQ-ACK with priority index 1 according to clause 6.3.2.4.2.1, by taking HARQ-ACK with priority index 1 as HARQ-ACK and replacing βoffsetPUSCH by βoffsetHARQ-ACK-HP.

-	Perform rate matching for CSI part 1 according to clause 6.3.2.4.2.2, by taking HARQ-ACK with priority index 1 as HARQ-ACK, if CSI part 1 is also transmitted on the PUSCH.

-	Perform rate matching for CSI part 2 according to clause 6.3.2.4.2.3, by taking HARQ-ACK with priority index 1 as HARQ-ACK, if CSI part 2 is also transmitted on the PUSCH.

If UCI-MuxWithDifferentPriority is configured, and HARQ-ACK bits associated with priority index 0, HARQ-ACK bits associated with priority index 1, and CSI part 1 if any are transmitted on a PUSCH:

-	Perform rate matching for HARQ-ACK with priority index 1 according to clause 6.3.2.4.2.1, by taking HARQ-ACK with priority index 1 as HARQ-ACK and replacing βoffsetPUSCH by βoffsetHARQ-ACK-HP.

-	If CSI part 1 is also transmitted on the PUSCH and the PUSCH is associated with priority index 1,

-	Perform rate matching for CSI part 1 according to clause 6.3.2.4.2.2, by taking HARQ-ACK with priority index 1 as HARQ-ACK.

-	Perform rate matching for HARQ-ACK with priority index 0 according to clause 6.3.2.4.2.3, by taking HARQ-ACK with priority index 0 as CSI part 2 and replacing βoffsetPUSCH by βoffsetHARQ-ACK-LP, and taking HARQ-ACK with priority index 1 as HARQ-ACK.

-	Otherwise,

-	Perform rate matching for HARQ-ACK with priority index 0 according to clause 6.3.2.4.2.2, by taking HARQ-ACK with priority index 0 as CSI-part 1 and replacing βoffsetPUSCH by βoffsetHARQ-ACK-LP, and taking HARQ-ACK with priority index 1 as HARQ-ACK.

-	Perform rate matching for CSI part 1 according to clause 6.3.2.4.2.3, by taking CSI part 1 as CSI part 2 and replacing βoffsetPUSCH by βoffsetCSI-part1, taking HARQ-ACK with priority index 0 as CSI-part 1 and taking HARQ-ACK with priority index 1 as HARQ-ACK, if CSI part 1 is also transmitted on the PUSCH and the PUSCH is associated with priority index 0.

< Unchanged parts are omitted >

#### 6.3.2.7	Multiplexing of coded UCI bits with different priority indexes to PUSCH

If UCI-MuxWithDifferentPriority is configured, and HARQ-ACK bits associated with priority index 0, and CSI part 1 if any are transmitted on a PUSCH associated with priority index 1,

-	If CSI part 1 is also transmitted on the PUSCH, the coded UCI bits are multiplexed onto PUSCH according to the procedures in Clause 6.2.7 by taking HARQ-ACK with priority index 0 as CSI part 2, and assuming the number of HARQ-ACK information in Clause 6.2.7 is 0 bit;

-	Otherwise, the coded UCI bits are multiplexed onto PUSCH according to the procedures in Clause 6.2.7 by taking HARQ-ACK with priority index 0 as CSI-part 1, and assuming the number of HARQ-ACK information in Clause 6.2.7 is 0 bit.

If UCI-MuxWithDifferentPriority is configured, and HARQ-ACK bits associated with priority index 1, and CSI if any are transmitted on a PUSCH associated with priority index 0, the coded UCI bits are multiplexed onto PUSCH according to the procedures in Clause 6.2.7 by taking HARQ-ACK with priority index 1 as HARQ-ACK.

If UCI-MuxWithDifferentPriority is configured, and HARQ-ACK bits associated with priority index 0, HARQ-ACK bits associated with priority index 1, and CSI part 1 if any are transmitted on a PUSCH,

-	if CSI part 1 is also transmitted on the PUSCH and the PUSCH is associated with priority index 1, the coded UCI bits are multiplexed onto PUSCH according to the procedures in Clause 6.2.7 by taking HARQ-ACK with priority index 1 as HARQ-ACK, and taking HARQ-ACK with priority index 0 as CSI part 2;

-	otherwise, the coded UCI bits are multiplexed onto PUSCH according to the procedures in Clause 6.2.7 by taking HARQ-ACK with priority index 1 as HARQ-ACK and taking HARQ-ACK with priority index 0 as CSI part 1, and taking CSI part 1 as CSI part 2 if CSI part 1 is also transmitted on the PUSCH and the PUSCH is associated with priority index 0.

< Unchanged parts are omitted >

### 7.3.1	DCI formats

< Unchanged parts are omitted >

#### 7.3.1.1	DCI formats for scheduling of PUSCH

##### 7.3.1.1.1	Format 0_0

< Unchanged parts are omitted >

Table 7.3.1.1.1-4A: Channel access type & CP extension if ChannelAccessMode-r16 = "semistatic" is provided

| Bit field mapped to index | Channel Access Type | The CP extension T_"ext" index defined in Clause 5.3.1 of [4, TS 38.211] | Initiator of the channel occupancy associated with the UL transmission as described in Clause x.x in TS 37.213 |
| --- | --- | --- | --- |
| 0 | No sensing as defined in Clause 4.3 in TS 37.213 | 0 | gNB |
| 1 | No sensing as defined in Clause 4.3 in TS 37.213 | 2 | gNB |
| 2 | Sensing within a 25us interval as defined in Clause 4.3 in TS 37.213 | 0 | gNB |
| 3 | Sensing as defined in Clause 4.3.1.2 in TS 37.213 | 0 | UE |
| Note: Row index 3 is only applicable if ue-SemiStaticChannelAccessConfig is provided. Otherwise, the row is reserved. |  |  |  |

< Unchanged parts are omitted >

#### 7.3.1.2	DCI formats for scheduling of PDSCH

< Unchanged parts are omitted >

##### 7.3.1.2.2	Format 1_1

DCI format 1_1 is used for the scheduling of one or multiple PDSCH in one cell.

The following information is transmitted by means of the DCI format 1_1 with CRC scrambled by C-RNTI or CS-RNTI or MCS-C-RNTI:

-	Identifier for DCI formats – 1 bits

-	The value of this bit field is always set to 1, indicating a DL DCI format

< Unchanged parts are omitted >

-	PDSCH-to-HARQ_feedback timing indicator – 0, 1, 2, or 3 bits as defined in Clause 9.2.3 of [5, TS 38.213]. The bitwidth for this field is determined as bits, where I is the number of entries in the higher layer parameter dl-DataToUL-ACK.

If higher layer parameter priorityIndicatorDCI-1-1 is configured, if the bit width of the PDSCH-to-HARQ_feedback timing indicator in DCI format 1_1 for one HARQ-ACK codebook is not equal to that of the PDSCH-to-HARQ_feedback timing indicator in DCI format 1_1 for the other HARQ-ACK codebook on the same cell for PUCCH transmission, a number of most significant bits with value set to '0' are inserted to smaller PDSCH-to-HARQ_feedback timing indicator until the bit width of the PDSCH-to-HARQ_feedback timing indicator in DCI format 1_1 for the two HARQ-ACK codebooks are the same.

If higher layer parameter pucch-sSCellDyn is configured, if the bit width of the PDSCH-to-HARQ_feedback timing indicator in DCI format 1_1 associated with one cell for PUCCH transmission is not equal to that of the PDSCH-to-HARQ_feedback timing indicator in DCI format 1_1 associated with the other cell for PUCCH transmision, a number of most significant bits with value set to '0' are inserted to smaller PDSCH-to-HARQ_feedback timing indicator until the bit width of the PDSCH-to-HARQ_feedback timing indicator in DCI format 1_1 associated with the two cells are the same.

If the UE is configured with a PUCCH-SCell, pucch-sSCellDyn is replaced by pucch-sSCellDyn-secondaryPUCCHgroup for the secondary PUCCH group.

-	One-shot HARQ-ACK request – 0 or 1 bit.

-	1 bit if higher layer parameter pdsch-HARQ-ACK-OneShotFeedback-r16 or pdsch-HARQ-ACK-enhType3List is configured;

-	0 bit otherwise.

-	Enhanced Type 3 codebook indicator - 0, 1, 2, or 3 bits.

-	0 bit if pdsch-HARQ-ACK-enhType3DCIfield is not configured;

-	log2(nCB) bits otherwise, where nCB is the number of entries in the higher layer parameter pdsch-HARQ-ACK-enhType3List.

If the UE is configured with a PUCCH-SCell, pdsch-HARQ-ACK-enhType3DCIfield is replaced by pdsch-HARQ-ACK-enhType3DCIfield-secondaryPUCCHgroup for the secondary PUCCH group, and pdsch-HARQ-ACK-enhType3List is replaced by pdsch-HARQ-ACK-enhType3List-secondaryPUCCHgroup for the secondary PUCCH group.

-	PDSCH group index – 0 or 1 bit.

-	1 bit if the higher layer parameter pdsch-HARQ-ACK-Codebook-r16= enhancedDynamic;

-	0 bit otherwise.

-	New feedback indicator – 0, 1 or 2 bits.

-	1 bit if the higher layer parameter pdsch-HARQ-ACK-Codebook-r16= enhancedDynamic and the higher layer parameter nfi-TotalDAI-Included is not configured;

-	2 bits if the higher layer parameter pdsch-HARQ-ACK-Codebook-r16= enhancedDynamic and the higher layer parameter nfi-TotalDAI-Included=true; the MSB corresponds to the scheduled PDSCH group, and the LSB corresponds to the non-scheduled PDSCH group, as defined in [TS38.213] clause 9.1.3.3

-	0 bit otherwise.

-	Number of requested PDSCH group(s) – 0 or 1 bit.

-	1 bit if the higher layer parameter pdsch-HARQ-ACK-Codebook-r16= enhancedDynamic;

-	0 bit otherwise.

-	HARQ-ACK retransmission indicator – 0 or 1 bit.

-	1 bit if higher layer parameter pdsch-HARQ-ACK-retx is configured.

-	0 bit otherwise.

If the UE is configured with a PUCCH-SCell, pdsch-HARQ-ACK-retx is replaced by pdsch-HARQ-ACK-retx-secondaryPUCCHgroup for the secondary PUCCH group.

< Unchanged parts are omitted >

-	PUCCH Cell indicator – 0 or 1 bit.

-	1 bit if higher layer parameter pucch-sSCellDyn is configured.

-	0 bit otherwise.

If DCI formats 1_1 are monitored in multiple search spaces associated with multiple CORESETs in a BWP for scheduling the same serving cell, zeros shall be appended until the payload size of the DCI formats 1_1 monitored in the multiple search spaces equal to the maximum payload size of the DCI format 1_1 monitored in the multiple search spaces.

< Unchanged parts are omitted >

##### 7.3.1.2.3	Format 1_2

DCI format 1_2 is used for the scheduling of PDSCH in one cell.

The following information is transmitted by means of the DCI format 1_2 with CRC scrambled by C-RNTI or CS-RNTI or MCS-C-RNTI:

-	Identifier for DCI formats – 1 bits

-	The value of this bit field is always set to 1, indicating a DL DCI format.

< Unchanged parts are omitted >

-	PDSCH-to-HARQ_feedback timing indicator – 0, 1, 2, or 3 bits as defined in Clause 9.2.3 of [5, TS 38.213]. The bitwidth for this field is determined as log2(I) bits, where I is the number of entries in the higher layer parameter DL-DataToUL-ACK-DCI-1-2.

If higher layer parameter priorityIndicatorDCI-1-2 is configured, if the bit width of the PDSCH-to-HARQ_feedback timing indicator in DCI format 1_2 for one HARQ-ACK codebook is not equal to that of the PDSCH-to-HARQ_feedback timing indicator in DCI format 1_2 for the other HARQ-ACK codebook on the same cell for PUCCH transmission, a number of most significant bits with value set to '0' are inserted to smaller PDSCH-to-HARQ_feedback timing indicator until the bit width of the PDSCH-to-HARQ_feedback timing indicator in DCI format 1_2 for the two HARQ-ACK codebooks are the same.

If higher layer parameter pucch-sSCellDynDCI-1-2 is configured, if the bit width of the PDSCH-to-HARQ_feedback timing indicator in DCI format 1_2 associated with one cell for PUCCH transmission is not equal to that of the PDSCH-to-HARQ_feedback timing indicator in DCI format 1_2 associated with the other cell for PUCCH transmision, a number of most significant bits with value set to '0' are inserted to smaller PDSCH-to-HARQ_feedback timing indicator until the bit width of the PDSCH-to-HARQ_feedback timing indicator in DCI format 1_2 associated with the two cells are the same.

-	One-shot HARQ-ACK request –0 or 1 bit.

-	1 bit if higher layer parameter pdsch-HARQ-ACK-OneShotFeedbackDCI-1-2 or pdsch-HARQ-ACK-enhType3DCI-1-2 is configured;

-	0 bit otherwise.

-	Enhanced Type 3 codebook indicator - 0, 1, 2, or 3 bits.

-	0 bit if pdsch-HARQ-ACK-enhType3DCIfieldDCI-1-2 is not configured;

-	log2(nCB) bits otherwise, where nCB is the number of entries in the higher layer parameter pdsch-HARQ-ACK-enhType3List.

If the UE is configured with a PUCCH-SCell, pdsch-HARQ-ACK-enhType3DCIfield is replaced by pdsch-HARQ-ACK-enhType3DCIfield-secondaryPUCCHgroup for the secondary PUCCH group, and pdsch-HARQ-ACK-enhType3List is replaced by pdsch-HARQ-ACK-enhType3List-secondaryPUCCHgroup for the secondary PUCCH group.

-	HARQ-ACK retransmission indicator – 0 or 1 bit.

-	1 bit if higher layer parameter pdsch-HARQ-ACK-retxDCI-1-2 is configured.

-	0 bit otherwise.

If the UE is configured with a PUCCH-SCell, pdsch-HARQ-ACK-retxDCI-1-2 is replaced by pdsch-HARQ-ACK-retxDCI-1-2-secondaryPUCCHgroup for the secondary PUCCH group.

-	Antenna port(s) – 0, 4, 5, or 6 bits

-	0 bit if higher layer parameter antennaPortsFieldPresenceDCI-1-2 is not configured;

-	Otherwise 4, 5 or 6 bits as defined by Tables 7.3.1.2.2-1/2/3/4, where the number of CDM groups without data of values 1, 2, and 3 refers to CDM groups {0}, {0,1}, and {0, 1,2} respectively. The antenna ports p0,…,pv-1 shall be determined according to the ordering of DMRS port(s) given by Tables 7.3.1.2.2-1/2/3/4. If a UE is configured with both dmrs-DownlinkForPDSCH-MappingTypeA-DCI-1-2 and dmrs-DownlinkForPDSCH-MappingTypeB-DCI-1-2 and is configured with higher layer parameter antennaPortsFieldPresenceDCI-1-2, the bitwidth of this field equals maxxA,xB, where xA is the "Antenna ports" bitwidth derived according to dmrs-DownlinkForPDSCH-MappingTypeA-DCI-1-2 and xB is the "Antenna ports" bitwidth derived according to dmrs-DownlinkForPDSCH-MappingTypeB-DCI-1-2. A number of xA-xB zeros are padded in the MSB of this field, if the mapping type of the PDSCH corresponds to the smaller value of xA and xB.

If a UE is not configured with higher layer parameter antennaPortsFieldPresenceDCI-1-2, antenna port(s) are defined assuming bit field index value 0 in Tables 7.3.1.2.2-1/2/3/4.

< Unchanged parts are omitted >