3GPP TSG-RAN1 WG1 Meeting #108-e	R1-2202801

# e-meeting, February 21st – March 3rd, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.212 | CR | 0095 | rev | - | Current version: | 16.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of NZC partitioning in eType II CSI |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Moderator (Qualcomm Incorporated), Nokia, Nokia Shanghai Bell, ZTE, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_eMIMO-Core |  |  |  |  |  | Date: |  |  | 2022-03-02 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In Table 6.3.2.1.2-5A of 38.212, the number of nonzero coefficient amplitudes and phases mapped in Group 1 and Group 2 for eType II CSI is incorrect in a small number of cases, when ν≤KNZ≤2(ν-1), i.e., for reported rank ν=2 and KNZ=2, rank ν=3 and KNZ=3,4, rank ν=4 and KNZ=4,5,6. For these cases the current formula for Group 1, KNZ/2-υ, yields a negative value whereas the formula for Group 2, KNZ/2, yields a number larger than the total number, KNZ-ν, of reported coefficients. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The formulae for the number of nonzero coefficient mapped in Group 1 and 2 are corrected such that the size of Group 1 is 0 and that of Group 2 is KNZ-ν when ν≤KNZ≤2(ν-1). The size of the two groups are unchanged for all other values of KNZ>2(ν-1) |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UEs would determine unnecessarily large number of NZC coefficients in order to comply with PMI partitioning requirement, resulting in unacceptable overhead. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.2.1.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | The CR is expected to have isolated impact. The CR captures the common understanding. UEs implementing an earlier version of the specification are expected to determine a larger number of NZC coefficients in order to comply with PMI partitioning requirement, resulting in unnecessary feedback overhead. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<omitted text>

#### 6.3.2.1.2	CSI

<omitted text>

Table 6.3.2.1.2-5A: Mapping order of CSI fields of one CSI report, CSI part 2 of codebookType=typeII-r16 or typeII-PortSelection-r16

| CSI report number | CSI fields |
| --- | --- |
| CSI report #n CSI part 2, group 0 | PMI fields X1, from left to right as in Tables 6.3.2.1.2-1A/2A, if reported |
| CSI report #n CSI part 2, group 1 | The following PMI fields X2, from left to right, as in Tables 6.3.2.1.2-1A/2A: i2,3,l:l=1,…,υ, i1,5, i1,6,l:l=1,…,υ and max⁡(0,KNZ2-υ)KNZ2-υ×3 highest priority bits of i2,4,l:l=1,…,υ,max⁡(0,KNZ2-υ)KNZ/2-υ×4 highest priority bits of {i2,5,l:l=1,…,υ} and ν*2LMυ-KNZ/2 highest priority bits ofi1,7,l:l=1,…,υ, in decreasing order of priority based on function Pril,i,f defined in clause 5.2.3 of TS38.214, if reported |
| CSI report #n CSI part 2, group 2 | The following PMI fields X2, from left to right, as in Tables 6.3.2.1.2-1A/2A: minKNZ-v, KNZ2KNZ/2×3 lowest priority bits of i2,4,l:l=1,…,υ, minKNZ-v, KNZ2KNZ/2×4 lowest priority bits of i2,5,l:l=1,…,υ and KNZ/2 lowest priority bits of i1,7,l:l=1,…,υ, in decreasing order of priority based on function Pril,i,f defined in clause 5.2.3 of TS38.214, if reported |

<omitted text>