3GPP TSG-RAN WG3 Meeting #114b-e	R3-221189

E-meeting, 17-26 Jan 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.455 | CR | 0053 | rev | 1 | Current version: | 16.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on PRS Beam Information |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_POS-Core |  |  |  |  |  | Date: |  |  | 2022-01-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In the current NR-PRS Beam Information IE, the PRS Resource Set ID is included, but there is no resource ID: IE/Group Name; Presence; Range; IE type and reference; Semantics description. NR-PRS Beam Information; 1 .. < maxPRS-ResourceSets >. >PRS Resource Set ID; M; INTEGER (0..7); The resource set in which the resources are associated with the angle.. >PRS Angle Item; 1..< maxPRS-ResourcesPerSet >. >>NR PRS Azimuth; M; INTEGER (0..359). >>NR PRS Azimuth fine; O; INTEGER (0..9); Fine angles. >>NR PRS Elevation; O; INTEGER (0..180). >>NR PRS Elevation fine; O; INTEGER (0..9); Fine angles. IE ommitted. The NR-PRS Beam Informatrion provides the beam angle information for each PRS resource. The UE can measure the PRS and report the measurement results for each PRS resource to the LMF for the DL-AoD positioning. For example, the UE can report the following information in the LPP message Provide Location Information. NR-DL-AoD-MeasElement-r16 ::= SEQUENCE { dl-PRS-ID-r16 INTEGER (0..255), nr-PhysCellID-r16 NR-PhysCellID-r16 OPTIONAL, nr-CellGlobalID-r16 NCGI-r15 OPTIONAL, nr-ARFCN-r16 ARFCN-ValueNR-r15 OPTIONAL, nr-DL-PRS-ResourceID-r16 NR-DL-PRS-ResourceID-r16 OPTIONAL, nr-DL-PRS-ResourceSetID-r16 NR-DL-PRS-ResourceSetID-r16 OPTIONAL, nr-TimeStamp-r16 NR-TimeStamp-r16, nr-DL-PRS-RSRP-Result-r16 INTEGER (0..126), nr-DL-PRS-RxBeamIndex-r16 INTEGER (1..8) OPTIONAL, nr-DL-AoD-AdditionalMeasurements-r16 NR-DL-AoD-AdditionalMeasurements-r16 OPTIONAL, ... } Then LMF can then match the measurements to the PRS angle information based on the NR-PRS Beam Information IE of the TRP. The LMF can then calculate the AoD information and further calculate the UE location. In order the enable the LMF to match the measurements per PRS resource to the PRS angle, the “PRS Angle Item” should include the PRS resource ID information. Otherwise, an easier way can have the clarification in the semantics description, such as, “The PRS angle items are associated with the PRS resource in the ascending order in each resource set”. Thus, two options can be considered: Option 1: Add the “Resource ID” IE in each “PRS Angle Item” IE. Option 2: Add the semantics description for the “PRS Angle Item” IE: “The PRS angle items are associated with the PRS resource in the ascending order in each resource set”. This CR provides the solution of Option 2. Rev1: group as preference for option 1, Option 1 is implemented |
| R3-20 |  |  |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the “Resource ID” IE in each “PRS Angle Item” IE. Impact assessment towards the previous version of the specification (same release): This CR has isolated impact with the previous version of the specification (same release), the impact is functional. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The LMF cannot know the angle information for the measurements related of each PRS resource. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.2.58, 9.3.5,9.3.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS/TR 38.473 CR 0849 |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev1: group as preference for option 1, Option 1 is implemented |  |  |  |  |  |  |  |  |

<<<<<<<<<<<<<<<<<<<< Changes Begin >>>>>>>>>>>>>>>>>>>>

### 9.2.58	NR-PRS Beam Information

This IE contains spatial direction information of the DL-PRS Resources.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| NR-PRS Beam Information |  | 1 .. < maxPRS-ResourceSets > |  |  |  |  |
| >PRS Resource Set ID | M |  | INTEGER (0..7) | The resource set in which the resources are associated with the angle. |  |  |
| >PRS Angle Item |  | 1..< maxPRS-ResourcesPerSet > |  |  |  |  |
| >>NR PRS Azimuth | M |  | INTEGER (0..359) |  |  |  |
| >>NR PRS Azimuth fine | O |  | INTEGER (0..9) | Fine angles |  |  |
| >>NR PRS Elevation | O |  | INTEGER (0..180) |  |  |  |
| >>NR PRS Elevation fine | O |  | INTEGER (0..9) | Fine angles |  |  |
| >>PRS Resource ID | O |  | INTEGER(0..63) |  | YES | ignore |
| LCS to GCS Translation |  | 0 .. <maxnolcs-gcs-translation> |  | If absent, the azimuth and elevation are provided in GCS. |  |  |
| >Alpha | M |  | INTEGER (0..359) |  |  |  |
| >Alpha-fine | O |  | INTEGER (0..9) | Fine angles |  |  |
| >Beta | M |  | INTEGER (0..359) |  |  |  |
| >Beta-fine | O |  | INTEGER (0..9) | Fine angles |  |  |
| >Gamma | M |  | INTEGER (0..359) |  |  |  |
| >Gamma-fine | O |  | INTEGER (0..9) | Fine angles |  |  |

| Range bound | Explanation |
| --- | --- |
| maxPRS-ResourceSets | Maximum no of DL-PRS resource sets per TRP. Value is 2. |
| maxPRS-ResourcesPerSet | Maximum no of DL-PRS resources of the DL-PRS resource set of the TRP. Value is 64. |
| maxnolcs-gcs-translation | Maximum no. of LCS-GS-Translation-Parameters that can reported with one message. Value is 3. The current version of the specification supports 1. |

<<<<<<<<<<<<<<<<<<<< Next Changes >>>>>>>>>>>>>>>>>>>>

### 9.3.5	Information Element definitions

-- ASN1START

-- **************************************************************

--

#### -- Information Element Definitions

--

-- **************************************************************

<<<<<<<<<<<<<<<<<<<< Ommitted part>>>>>>>>>>>>>>>>>>>>

maxNoSSBs,

maxnoofPRSresourceSet,

maxnoofPRSresource,

id-Cell-ID,

id-TRPInformationTypeItem,

id-SrsFrequency,

id-TRPType,

id-SRSSpatialRelationPerSRSResource,

id-PRS-Resource-ID

FROM NRPPA-Constants

Criticality,

NRPPATransactionID,

ProcedureCode,

ProtocolIE-ID,

TriggeringMessage

<<<<<<<<<<<<<<<<<<<< Ommitted part>>>>>>>>>>>>>>>>>>>>

PRSAngleItem  ::= SEQUENCE {

nRPRSAzimuth			INTEGER (0..359),

nRPRSAzimuthFine		INTEGER (0..9) OPTIONAL,

nRPRSElevation			INTEGER (0..180) OPTIONAL,

nRPRSElevationFine		INTEGER (0..9) OPTIONAL,

iE-Extensions			ProtocolExtensionContainer { { PRSAngleItem-ExtIEs} } OPTIONAL,

...

}

PRSAngleItem-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

{ ID id-PRS-Resource-ID		CRITICALITY ignore EXTENSION PRS-Resource-ID		PRESENCE OPTIONAL },

...

}

<<<<<<<<<<<<<<<<<<<< Ommitted part>>>>>>>>>>>>>>>>>>>>

id-Cell-ID														ProtocolIE-ID ::= 60

id-SrsFrequency													ProtocolIE-ID ::= 61

id-TRPType														ProtocolIE-ID ::= 62

id-SRSSpatialRelationPerSRSResource								ProtocolIE-ID ::= 63

id-PRS-Resource-ID												ProtocolIE-ID ::= xx

END

-- ASN1STOP

<<<<<<<<<<<<<<<<<<<< Changes End >>>>>>>>>>>>>>>>>>>>